3GPP TSG RAN WG1 #107-e	R1-2111147

e-Meeting, November 11th – 19th, 2021

Agenda Item:	8.2.5

Source:		Fujitsu

Title:	Considerations on multi-PDSCH/PUSCH with a single DCI and HARQ for NR from 52.6 GHz to 71 GHz

Document for:	Discussion/Decision

- Introduction

In this contribution, we would like to present some discussions about multi-PDSCH/PUSCH scheduling with a single DCI and HARQ for extending NR operation to 71 GHz.

Some relevant agreements are listed in the Appendix.

- Discussion

2.1 Time domain bundling for Type-1 HARQ-ACK codebook

In the last meeting, time domain bundling for Type-1 HARQ-ACK codebook was discussed. In general, we think it is beneficial to reduce codebook size and should be supported. For example, it can be enabled by RRC signaling. The details of how to support the time domain bundling are clarified as below based on the example of TDRA table and set of K1 given in Figure 1.

For each K1, the corresponding candidate PDSCH reception occasion is determined based on all the SLIVs of each row in the TDRA table. More specifically, for a specific K1, if at least one of SLIVs in a row in the TDRA table is not colliding with UL symbols configured by RRC signaling, it has a candidate PDSCH reception occasion. In Figure 1, the set of K1 is assumed to be {1, 2}. For  K1=1, since SLIVs of row 2 in slot N-3 and slot N-4 are not colliding with UL symbols, it has a candidate PDSCH reception occasion. Similarly, for K1=2, it also has a candidate PDSCH reception occasion. That is, there are 2 determined candidate PDSCH reception occasions respectively corresponding to K1=1 and K1=2.

For each determined candidate PDSCH reception occasion, HARQ-ACK information for all PDSCHs in slots that include SLIV(s) not colliding with UL symbols can be bundled (by binary AND operation) as 1 bit. For example, assume that PDSCHs are scheduled only in slot N-3 and slot N-4 with K1=1, and the PDSCHs are successfully received. Then for the candidate PDSCH reception occasions corresponding to K1=1, it should report ACK, since PDSCHs in slot N-3 and slot N-4, since the PDSCHs are scheduled and successfully received. And for the candidate PDSCH reception occasions corresponding to K1=2, it should report NACK corresponding to PDSCHs in slot N-3, slot N-4 and slot N-5, since there is no PDSCH scheduled with K1=2 and the corresponding HARQ-ACK information should be assumed to be NACK.

| Set of K1; 1, 2. R (TDRA table). Index = 0; {SLIV R0_0}. Index = 1; {SLIV R1_0, SLIV R1_1}. Index = 2; {SLIV R2_0, SLIV R2_1, SLIV R2_2, SLIV R2_3}. |  |
| --- | --- |

Figure 1. Example of TDRA table and set of K1

Proposal 1: For Type-1 HARQ-ACK codebook, support time domain bundling.

- For each K1, the corresponding candidate PDSCH reception occasion can be determined based on all the SLIVs of each row in the TDRA table. If at least one of SLIVs in a row in the TDRA table is not colliding with UL symbols configured by RRC signaling, it corresponds to one candidate PDSCH reception occasion.

- For each determined candidate PDSCH reception occasion, HARQ-ACK information for all PDSCHs in slots that include SLIV(s) not colliding with UL symbols can be bundled as 1 bit.

2.3 SPS activation/retransmission

As agreed, DCI format 1_1 can support multi-PDSCH scheduling. While in the legacy, DCI format 1_1 can be used for SPS activation/retransmission. In the last meeting, how to support SPS activation/retransmission by a DCI that can schedule multiple PDSCHs was discussed. However, there is still no conclusion yet.

3 options were proposed.

- Option 1: Allow only single SLIV-based (de)activation

- Option 2: Based on the last configured SLIV

- Option 3: Based on the first (valid) SLIV

Among the 3 options, Option 2 and Option 3 can provide more flexibility than option 1. Therefore, Option 2 and option 3 should be preferred if they will not require much standardization effort.

Firstly, as agreed, for a DCI scheduling multiple PDSCHs, the K1 indicated by the DCI (or provided by RRC signaling) indicates the slot offset between the last PDSCH scheduled by the DCI (i.e. the last configured SLIV as clarified in the last meeting) and slot carrying the corresponding HARQ-ACK information.

Secondly, in Rel15/16, TS38.213 specifies that “For a SPS PDSCH reception ending in slot, the UE transmits the PUCCH in slot n+k where k is provided by the PDSCH-to-HARQ_feedback timing indicator field, if present, in a DCI format activating the SPS PDSCH reception.”

Considering the two aspects above, if the last configured SLIV (Option 2) is used for SPS activation, there will be no further specification impact. While if the first (valid) SLIV (Option 1) is used, there will be further specification impact. Assuming Option 1 is used, for example, the definition of k1 indicated by DCI for SPS activation, or the definition of k for HARQ-ACK feedback timing for SPS PDSCH need to be further discussed, with consideration on how to generate the HARQ-ACK information for SPS PDSCH in Type-1 HARQ-ACK codebook. Therefore, from the perspective of standardization workload, Option 2 is slightly preferred over Option 3.

As a summary, Option 2 with can achieve a well trade-off between flexibility and standardization effort. We slightly prefer Option 2.

Proposal 2: For SPS activation/retransmission via DCI format 1_1 when multi-PDSCH scheduling is supported, the following 3 options can be considered, and Option 2 is slightly preferred for a well trade-off between flexibility and standardization effort.

- Option 1: Allow only single SLIV-based (de)activation

- Option 2: Based on the last configured SLIV

- Option 3: Based on the first (valid) SLIV

- Conclusion

According to the discussions above, we have the following observations and proposals for multi-PDSCH/PUSCH scheduling via a single DCI.

Proposal 1: For Type-1 HARQ-ACK codebook, support time domain bundling.

- For each K1, the corresponding candidate PDSCH reception occasion can be determined based on all the SLIVs of each row in the TDRA table. If at least one of SLIVs in a row in the TDRA table is not colliding with UL symbols configured by RRC signaling, it corresponds to one candidate PDSCH reception occasion.

- For each determined candidate PDSCH reception occasion, HARQ-ACK information for all PDSCHs in slots that include SLIV(s) not colliding with UL symbols can be bundled as 1 bit.

Proposal 2: For SPS activation/retransmission via DCI format 1_1 when multi-PDSCH scheduling is supported, the following 3 options can be considered, and Option 2 is slightly preferred for a well trade-off between flexibility and standardization effort.

- Option 1: Allow only single SLIV-based (de)activation

- Option 2: Based on the last configured SLIV

- Option 3: Based on the first (valid) SLIV

Appendix: relevant agreements

| RAN1#104-e[1] | Agreement: - For a UE and for a serving cell, scheduling multiple PDSCHs by single DL DCI and scheduling multiple PUSCHs by single UL DCI are supported. - Each PDSCH or PUSCH has individual/separate TB(s) and each PDSCH/PUSCH is confined within a slot. - FFS: The maximum number of PDSCHs or PUSCHs that can be scheduled with a single DCI - FFS: Whether multiple PDSCH scheduling applies to 120 kHz in addition to 480 and 960 kHz - At least for 120 kHz SCS, single-slot scheduling with slot-based monitoring will still be supported as specified in Rel-15/Rel-16 - The followings will not be considered in this WI. - Single DCI to schedule both PDSCH(s) and PUSCH(s) - Single DCI to schedule one or multiple TBs where any single TB can be mapped over multiple slots, where mapping is not by repetition - Single DCI to schedule N TBs (N>1) where a TB can be repeated over multiple slots (or mini-slots) - Note: This does not imply that existing slot aggregation and/or repetition for PDSCH and PUSCH by single DCI is precluded for the serving cell. Agreement: - For a DCI scheduling multiple PDSCHs, HARQ-ACK information corresponding to PDSCHs scheduled by the DCI is multiplexed with a single PUCCH in a slot that is determined based on K1, - where K1 (indicated by the PDSCH-to-HARQ_feedback timing indicator field in the DCI or provided by dl-DataToUL-ACK if the PDSCH-to-HARQ_feedback timing indicator field is not present in the DCI) indicates the slot offset between the slot of the last PDSCH scheduled by the DCI and the slot carrying the HARQ-ACK information corresponding to the scheduled PDSCHs. - It is noted that granularity of K1 can be separately discussed. - FFS: If needed, further discuss whether or not HARQ-ACK information corresponding to different PDSCHs scheduled by the DCI can be carried by different PUCCH(s) Agreement: For generating type-2 HARQ-ACK codebook corresponding to DCI that can schedule multiple PDSCHs, the following alternatives can be considered to DAI counting and will be down-selected in RAN1#104bis-e. - Alt 1: C-DAI/T-DAI is counted per DCI. - Alt 2: C-DAI/T-DAI is counted per PDSCH. - Alt 3: C-DAI/T-DAI is counted per M scheduled PDSCH(s), where M is configurable (e.g., 1, 2, 4, …). - FFS: Codebook generation details - FFS: How to signal DAI values (e.g., increase of DAI bits for Alt 2 and Alt 3) - FFS: Whether to apply time domain bundling of HARQ-ACK feedback Agreement: The multi-PUSCH scheduling defined in Rel-16 NR-U is the baseline for multi-PUSCH scheduling in Rel-17. - FFS: Applicability to multi-PDSCH scheduling. Agreement: - For the multi-PUSCH scheduling in Rel-17, study the enhancement of the following in addition to Rel-16 multi-PUSCH scheduling. - CBGTI: Whether or not CBG (re)transmission is supported when more than one PUSCHs are scheduled (Already supported when only one PUSCH is scheduled). - CSI-request: Whether to apply same or different rule compared to Rel-16 (e.g., the PUSCH that carries the AP-CSI feedback is the first PUSCH that satisfies the multiplexing timeline). - TDRA: Down-select among - Alt 1: TDRA table is extended such that each row indicates up to [X, FFS for X] multiple PUSCHs (continuous in time-domain). Each PUSCH has a separate SLIV and mapping type. The number of scheduled PUSCHs is signalled by the number of indicated valid SLIVs in the row of the TDRA table signalled in DCI. - Alt 2: TDRA table is extended such that each row indicates up to [X, FFS for X] multiple PUSCHs (that can be non-continuous in time-domain). Each PUSCH has a separate SLIV and mapping type. The number of scheduled PUSCHs is signalled by the number of indicated valid SLIVs in the row of the TDRA table signalled in DCI. - Alt 3: TDRA table is extended such that each row indicates up to 8 multiple PUSCH groups (that can be non-continuous between PUSCH groups). Each PUSCH group has a separate SLIV, mapping type and number of slots/PUSCHs N. Within each PUSCH group, N PUSCHs occupy the same OFDM symbols indicated by the SLIV and mapping type. The number of scheduled PUSCHs is the sum of number of PUSCHs in all PUSCH groups in the row of the TDRA table signalled in DCI. - FDRA: Whether/how to enhance FDRA e.g., by increasing RBG size or changing allocation granularity - Frequency hopping: Whether/how to support frequency hopping for scheduled PUSCHs, e.g., inter-PUSCH/intra-PUSCH hopping - URLLC related fields such as priority indicator and open-loop power control parameter set indication: Whether/how to apply URLLC related fields for scheduled PUSCHs - Applicability to multi-PDSCH scheduling in Rel-17. - Note: Other enhancements are not precluded. |
| --- | --- |
| RAN1#104bis-e[2] | Agreement: - The maximum number of PDSCHs that can be scheduled with a single DCI in Rel-17 is 8 for SCS of 480 and 960 kHz. - FFS: Further restrictions for 480 kHz to 4 - FFS: A UE capability to select between 4 and 8 for 480 kHz SCS - Note: Multi-PDSCH scheduling for the case of 120 kHz SCS is still FFS as per prior agreement. This case can be addressed after this FFS has been decided. - The maximum number of PUSCHs that can be scheduled with a single DCI in Rel-17 is 8. - FFS: Further restrictions for 120 kHz and 480 kHz SCS - FFS: A UE capability to select between different values for 120 kHz and 480 kHz SCS Agreement: For a DCI that can schedule multiple PDSCHs, - MCS for the 1st TB: This appears only once in the DCI and applies commonly to the first TB of each PDSCH - NDI for the 1st TB: This is signaled per PDSCH and applies to the first TB of each PDSCH - RV for the 1st TB: This is signaled per PDSCH, with 2 bits if only a single PDSCH is scheduled or 1 bit for each PDSCH otherwise and applies to the first TB of each PDSCH - HARQ process number: This applies to the first scheduled PDSCH and is incremented by 1 for subsequent PDSCHs (with modulo operation, if needed) - FFS: - MCS/NDI/RV for the 2nd TB for each PDSCH, including whether scheduling of the 2nd TB for each PDSCH can be supported or not - Details of resource allocation related fields such as VRB-to-PRB mapping, PRB bundling size indicator, rate matching indicator, and ZP CSI-RS trigger - Whether/how to signal CBGFI/CBGTI if CBGFI/CBGTI is supported for multi-PDSCH scheduling - Details of fields that are common with multi-PUSCH scheduling, e.g., TDRA, FDRA, priority indicator, including potential enhancements Agreement: - For a DCI that can schedule multiple PUSCHs, - TDRA: Alt 2 (TDRA table is extended such that each row indicates up to 8 multiple PUSCHs (that can be non-continuous in time-domain). Each PUSCH has a separate SLIV and mapping type. The number of scheduled PUSCHs is implicitly indicated by the number of indicated valid SLIVs in the row of the TDRA table signalled in DCI.), as per agreement made in RAN1#104-e - FFS: signaling details - Note: Alt 2 does not preclude continuous resource allocation in time-domain. - For a DCI that can schedule multiple PDSCHs, - TDRA: TDRA table is extended such that each row indicates up to 8 multiple PDSCHs (that can be non-continuous in time-domain). Each PDSCH has a separate SLIV and mapping type. The number of scheduled PDSCHs is implicitly indicated by the number of indicated valid SLIVs in the row of the TDRA table signalled in DCI. - FFS: signaling details - Note: This does not preclude continuous resource allocation in time-domain. - Note: Multi-PDSCH scheduling for the case of 120 kHz SCS is still FFS as per prior agreement. This case can be addressed after this FFS has been decided. Agreement: For enhancements of generating type-1 HARQ-ACK codebook corresponding to DCI that can schedule multiple PDSCHs, the following options can be considered, - Option 1: The set of candidate PDSCH reception occasions is determined according to each SLIV of each row in the TDRA table and based on extension of K1 set - Option 1a: The set of candidate PDSCH reception occasions is determined according to each SLIV of each row in the TDRA table - Option 2: The set of candidate PDSCH reception occasions is determined according to the last SLIV of each row in the TDRA table - FFS: Codebook generation details, including how to handle the collision with TDD DL/UL configuration and whether/how to extend K1 set based on K1 and slot offset between last PDSCH and other PDSCHs in a row in the TDRA table Conclusion: The following is observed for alternative 1 from prior agreement. - For Alt 1 (C-DAI/T-DAI is counted per DCI) of generating type-2 HARQ-ACK codebook corresponding to DCI that can schedule multiple PDSCHs, - C-DAI/T-DAI in DL DCI: Same DAI overhead with Rel-16 single-PDSCH DCI - T-DAI in UL DCI: - In case of single codebook handling feedback for both single and multi-PDSCH scheduling, same DAI overhead with Rel-16 UL DCI - In case of separate sub-codebooks, need additional DAI field (with same bit-width of DAI with Rel-16 UL DCI), in UL DCI for all serving cells including a serving cell not configured with multi-PDSCH DCI - Note that DAI field increment for this case is similar for the case in Rel-15 where CBG is configured - HARQ-ACK codebook generation: - A separate sub-codebook can be generated when multi-PDSCH DCI is configured for a serving cell, similar to the way as 2nd sub-codebook is defined to handle CBG-based scheduling - FFS: whether single codebook or separate sub-codebooks is(are) generated when multi-PDSCH DCI is configured for a serving cell - FFS: how many sub-codebooks are generated when multi-PDSCH DCI is configured for a serving cell and CBG is configured for the serving cell and/or the other serving cell(s) - HARQ-ACK payload size is increased compared to single PDSCH scheduling only, since the number of HARQ-ACK bits corresponding to each DAI of the (sub-)codebook for multi-PDSCH DCI in case of separate sub-codebooks (or for all DL DCIs in case of single codebook) depends on the maximum configured number of PDSCHs for multi-PDSCH DCI across serving cells belonging to the same PUCCH cell group. - The number of HARQ-ACK bits for multi-PDSCH DCI in case of separate sub-codebooks, or for all DL DCIs in case of single codebook, does not depend on the number of actually scheduled PDSCHs, rather, it is fixed as the maximum configured number of PDSCHs. - FFS: time domain bundling of HARQ-ACK feedback, as per agreement in RAN1#104-e - Note that multi-PDSCH DCI refers to a DL DCI where at least one entry of the TDRA table allows scheduling more than one PDSCH Conclusion: The following is observed for alternative 2 from prior agreement. - For Alt 2a (C-DAI/T-DAI is counted per PDSCH with a single codebook) of generating type-2 HARQ-ACK codebook corresponding to DCI that can schedule multiple PDSCHs, - C-DAI/T-DAI in DL DCI: Bit-width can be increased (FFS: by how much), in DL DCI not only for multi-PDSCH DCI but also for single-PDSCH DCI for all serving cells including a serving cell not configured with multi-PDSCH DCI - T-DAI in UL DCI: Bit-width can be increased (FFS: by how much), in UL DCI for all serving cells including a serving cell not configured with multi-PDSCH DCI - C-DAI/T-DAI in DL DCI and T-DAI in UL DCI shall be designed such that at most 3 consecutive DCI missing can be resolved, same as in Rel-15/16 NR. - FFS: details on increment of DAI field size - FFS: whether/how to handle the case where different DCI formats (e.g., DCI format 1_0 and DCI format 1_1) have different field sizes for C-DAI/T-DAI - HARQ-ACK codebook generation: - The number of HARQ-ACK bits depends on the number of scheduled PDSCHs. - FFS: ordering of the PDSCHs for DAI counting - FFS: time domain bundling of HARQ-ACK feedback, as per agreement in RAN1#104-e - Note that multi-PDSCH DCI refers to a DL DCI where at least one entry of the TDRA table allows scheduling more than one PDSCH Conclusion: The following is observed for alternative 3 from prior agreement. - For Alt 3 (C-DAI/T-DAI is counted per M scheduled PDSCH(s), where M is configurable) of generating type-2 HARQ-ACK codebook corresponding to DCI that can schedule multiple PDSCHs, - If M equals to the maximum configured number of PDSCHs, Alt 3 is the same with Alt 1, if the same number of codebooks is assumed. - Else if M equals to 1, Alt 3 is the same with Alt 2. - Otherwise (i.e., 1<M<the maximum configured number of PDSCHs), Alt 3 is similar to Alt 2, except that - The number of HARQ-ACK bits corresponding to each DAI increases by M times. - NACK bits may be padded if the number of scheduled PDSCHs is not an integer multiple of M. - FFS: details on DAI field size - FFS: whether single codebook or separate sub-codebooks is(are) generated when multi-PDSCH DCI is configured for a serving cell - In addition, new RRC parameter to configure M needs to be introduced. - Note that multi-PDSCH DCI refers to a DL DCI where at least one entry of the TDRA table allows scheduling more than one PDSCH |
| RAN1#105-e[3] | Agreement: - Do not use fallback DCI (i.e., DCI formats 0_0 and 1_0) for multi-PDSCH/PUSCH scheduling. - Use DCI format 0_1 to schedule multiple PUSCHs with a single DCI. - Use DCI format 1_1 to schedule multiple PDSCHs with a single DCI. Conclusion: For a DCI that can schedule multiple PUSCHs, - CSI-request: When the DCI schedules M PUSCHs, the PUSCH that carries the aperiodic CSI feedback is M-th scheduled PUSCH for M <= 2, or (M-1)-th scheduled PUSCH for M > 2. Agreement: - If a PDSCH among multiple PDSCHs that are scheduled by a single DCI is collided with uplink symbol(s) indicated by tdd-UL-DL-ConfigurationCommon or tdd-UL-DL-ConfigurationDedicated, the UE does not receive the PDSCH. - FFS on how to handle HARQ-related issue for the PDSCH (e.g., HARQ process numbering) - The UE does not expect to be scheduled with multiple PDSCHs by a single DCI, where every PDSCH is collided with uplink symbol(s) indicated by tdd-UL-DL-ConfigurationCommon or tdd-UL-DL-ConfigurationDedicated. - If a PUSCH among multiple PUSCHs that are scheduled by a single DCI is collided with downlink symbol(s) indicated by tdd-UL-DL-ConfigurationCommon or tdd-UL-DL-ConfigurationDedicated, the UE does not transmit the PUSCH. - FFS on how to handle HARQ-related issue for the PUSCH (e.g., HARQ process numbering) - The UE does not expect to be scheduled with multiple PUSCHs by a single DCI, where every PUSCH is collided with downlink symbol(s) indicated by tdd-UL-DL-ConfigurationCommon or tdd-UL-DL-ConfigurationDedicated. Agreement: For TDRA in a DCI that can schedule multiple PDSCHs (or PUSCHs), - A row of the TDRA table can indicate PDSCHs (or PUSCHs) that are in consecutive or non-consecutive slots. - FFS: The maximum value of the gap between two consecutively scheduled PDSCHs or between two consecutively scheduled PUSCHs - FFS: The maximum value of the gap between the first scheduled PDSCH and the last scheduled PDSCH or between the first scheduled PUSCH and the last scheduled PUSCH - FFS: Details to introduce the gap between PDSCHs or between PUSCHs Agreement: For enhancements of generating type-1 HARQ-ACK codebook corresponding to DCI that can schedule multiple PDSCHs, the set of candidate PDSCH reception occasions corresponding to a UL slot with HARQ-ACK transmission is determined based on a set of DL slots and a set of SLIVs corresponding to each DL slot belonging to the set of DL slots. - The set of DL slots includes all the unique DL slots that can be scheduled by any row index r of TDRA table in DCI indicating the UL slot as HARQ-ACK feedback timing. - The set of SLIVs corresponding to a DL slot (belonging to the set of DL slots) at least include all the SLIVs that can be scheduled within the DL slot by any row index r of TDRA table in DCI indicating the UL slot as HARQ-ACK feedback timing. - FFS: details of further pruning of the set of SLIVs - FFS: impact if receiving more than one PDSCH in a slot is allowed, e.g., handling of overlapped SLIVs from different rows in the same and different DL slot - FFS impact of time domain bundling, if supported Agreement: - At least for 120 kHz SCS, for a DCI that can schedule multiple PUSCHs and is configured with the TDRA table containing at least one row with multiple SLIVs, - If CBG-based (re)transmission is configured, CBGTI field is not present when more than one PUSCHs are scheduled, but is present when a single PUSCH is scheduled, as in Rel-16. - FFS: - For 480/960 kHz SCS, whether to apply the same behavior with 120 kHz SCS or not to support CBGTI field configuration in the DCI that can schedule multiple PUSCHs - For a DCI that can schedule multiple PDSCHs and is configured with the TDRA table containing at least one row with multiple SLIVs, whether/how to configure CBGTI/CBGFI fields Agreement: If Alt 1 (C-DAI/T-DAI is counted per DCI) is adopted for generating type-2 HARQ-ACK codebook corresponding to a DCI that can schedule multiple PDSCHs, - At least two sub-codebooks are generated for a PUCCH cell group where - The first sub-codebook is for the following cases: - Any DCI that is not configured with CBG-based scheduling and is configured with TDRA table containing rows each with a single SLIV - Any DCI that is not configured with CBG-based scheduling and is configured with TDRA table containing at least one row with multiple SLIVs and schedules only a single PDSCH - The second sub-codebook is for the following case: - Any DCI that is configured with TDRA table containing at least one row with multiple SLIVs and schedules multiple PDSCHs - FFS: Methods (if needed) to align the size of HARQ-ACK feedback corresponding to different DCIs - FFS: Whether HARQ-ACK bits for 2 PDSCHs scheduled by this DCI can be included in the first sub-codebook in some cases - FFS: SPS PDSCH release, SCell dormancy indication without scheduled PDSCH - FFS: 2 or 3 sub-codebooks if CBG is configured for a serving cell in the PUCCH cell group - FFS: impact of time domain bundling, if supported, e.g., the number of sub-codebooks including single codebook if all A/N bits are bundled into a single bit per DCI Agreement: If Alt 2 (C-DAI/T-DAI is counted per PDSCH) is adopted for generating type-2 HARQ-ACK codebook corresponding to a DCI that can schedule multiple PDSCHs, - PDSCH(s) scheduled by a single DCI is counted firstly, serving cell(s) in the same PUCCH cell group and same PDCCH monitoring occasion is counted secondly, and PDCCH monitoring occasion(s) is counted thirdly. - The bit width of counter DAI field in fallback DCI (i.e., DCI formats 0_0 and 1_0) remains the same as in Rel-15 NR. - Note: The DAI bit width and number of sub-codebooks shall ensure that at most 3 consecutive missed DCIs can be resolved, same as in Rel-15/16 NR - This shall not impose additional gNB’s scheduling restriction. - In case where CBG retransmission is not configured for any serving cell in a same PUCCH cell group, the number of bits for each of counter DAI and total DAI in non-fallback DCI is extended (if needed) at least based on - The number of SLIVs associated with the row indexes in TDRA table - FFS: details - FFS: the case with configuration of CBG retransmission - FFS: the number of sub-codebooks - FFS: for the UE indicating by type2-HARQ-ACK-Codebook support for more than one PDSCH reception on a serving cell that are scheduled from a same PDCCH monitoring occasion |
| RAN1#106-e[4] | Agreement: - For single TRP operation, for 480/960 kHz SCS, - FFS: A UE does not expect to be scheduled with more than one PDSCH in a slot, by a single DCI or multiple DCIs. - FFS: A UE does not expect to be scheduled with more than one PUSCH in a slot, by a single DCI or multiple DCIs. - For single TRP operation, for 120 kHz SCS (same as current specification for FR2-1 for PUSCH), - Subject to UE capability, a UE can be scheduled with more than one PDSCH in a slot, by a single DCI or multiple DCIs. - Subject to UE capability, a UE can be scheduled with more than one PUSCH in a slot, by a single DCI or multiple DCIs. - FFS for multi-TRP operation - Note: The optimization of HARQ codebook size for Type 1 or Type 2 codebook design is considered as a low priority in Rel-17 (this does not preclude HARQ ACK bundling in time domain). - The agreement made in RAN1#105-e is revised as follows. Agreement: (RAN1#105-e) For enhancements of generating type-1 HARQ-ACK codebook corresponding to DCI that can schedule multiple PDSCHs, the set of candidate PDSCH reception occasions corresponding to a UL slot with HARQ-ACK transmission is determined based on a set of DL slots and a set of SLIVs corresponding to each DL slot belonging to the set of DL slots. - The set of DL slots contains all the unique DL slots determined by considering all combinations of the configured K1 values and the configured rows of the TDRA table. - The set of SLIVs corresponding to a DL slot (belonging to the set of DL slots) contains all the SLIVs for that slot determined by considering all combinations of the configured K1 values and the configured rows of the TDRA table. - The Rel-16 procedure is reused for determining the candidate PDSCH reception occasions for the set of SLIVs corresponding to each DL slot belonging to the set of DL slots - Note: The Rel-16 procedure already handles pruning of multiple SLIVs corresponding to a DL slot, for both UEs that are and are not capable of receiving multiple PDSCHs per slot - FFS impact of time domain bundling, if supported. Agreement: Consider the following options to construct type-2 HARQ-ACK codebook when CBG operation is configured, and down-select to one of the following options in RAN1#106bis-e. - Option 1: HARQ-ACK bits corresponding to CBG-based PDSCH reception and multi-PDSCH reception are merged into the same sub-codebook. - Option 2: HARQ-ACK bits corresponding to CBG-based PDSCH reception and HARQ-ACK bits corresponding to multi-PDSCH reception are contained in separate sub-codebooks. - Option 3: UE does not expect to be configured with both of CBG operation and multi-PDSCH scheduling in the same PUCCH cell group. - Note: Multi-PDSCH reception refers to the case where multiple PDSCHs are scheduled by a DCI that is configured with TDRA table containing at least one row with multiple SLIVs. |
| RAN1#106bis-e[5] | Agreement: For a PDSCH that is scheduled by multi-PDSCH scheduling DCI and is skipped due to collision with semi-static UL symbol(s), - For Type-1 HARQ-ACK codebook generation, the PDSCH is not considered and the HARQ-ACK bit corresponding to the PDSCH is not reported by UE. - Note: Rel-16 procedure can be reused to handle this case. - For Type-2 HARQ-ACK codebook generation, UE reports NACK for the PDSCH. - FFS on HARQ-ACK bit ordering - Note: Codebook generation in case time domain bundling is enabled can be separately discussed if time domain bundling is supported. |

References

- Draft Report of 3GPP TSG RAN WG1 #104-e v0.3.0, Online meeting, 25th January - 5th February 2021)

- Draft Report of 3GPP TSG RAN WG1 #104bis-e v0.2.0, online meeting, 12th - 20th April 2021

- Draft Report of 3GPP TSG RAN WG1 #105-e v0.2.0, online meeting, 10th - 27th May 2021

- Draft Report of 3GPP TSG RAN WG1 #106-e v0.2.0, online meeting, 16th - 27th August 2021

- Draft Report of 3GPP TSG RAN WG1 #106b-e v0.2.0, online meeting, 11th - 19th October 2021