3GPP TSG RAN WG1 #107	R1-2111153

e-Meeting, 11 – 19 November, 2021

Agenda item:		8.16.2

Source:	Nokia, Nokia Shanghai Bell

Title:	On UE features for supporting NR from 52.6 GHz to 71 GHz

Document for:		Discussion and Decision

# Introduction

In RAN1#106bis-e the first baseline has been agreed for a number of UE features. In this contribution we present our views on the UE features for supporting NR from 52.6 GHz to 71 GHz.

# Discussion

Detailed comments on UE features are listed below. For reference, the baseline agreements from RAN1#106bis are available in the Annex.

- 24-1:

  - SSB support for non-initial access should be mentioned in this FG as well

  - Confirm components 2-6

- 24-2:

  - FG name mentions stand-alone support, but the functionality is relevat to PSCell operation as well.

  - It would be better to merge this FG with FG 24-1.

- 24-3:

  - It would be better to merge this FG with FG 24-4.

- 26-5:

  - Confirm the FG, details to be defined later.

Proposal: Consider the observations and modifications proposed above for the next version of the corresponding RAN1 UE features list.

# Conclusion

In this contribution we have presented our views on UE features for supporting NR from 52.6 GHz to 71 GHz, with the following proposal:

Proposal: Consider the observations and modifications proposed above for the next version of the corresponding RAN1 UE features list.

# References

- Chair’s Notes RAN1#106bis-e, Oct. 2021

# Annex

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (Sidelink WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 24. NR_ext_to_71GHz | 24-1 | General Basic FR2-2 [DL] support | 1. Support 120KHz SCS [transmission and] reception for [initial/non-initial access] [2. Support multi-RB PUCCH format 0/1/4 for 120 kHz] [3. PRACH with 120KHz SCS and length 139[/571/1151]] [4. Support 120kHz subcarrier spacing for DL data and control channels and reference signals in FR2-2] [5. Support 120kHz subcarrier spacing for UL data and control channels and reference signals in FR2-2] [6. Support multi-PUSCH[/PDSCH] scheduling by single DCI for the operation with 120 kHz SCS] |  |  |  | FR2-2 is not supported | [per UE][per band] |  |  |  |  | Optional with capability signalling [A UE that supports FR2-2 must indicate this FG is supported] |
| 24. NR_ext_to_71GHz | 24-1a | FR2-2 PRACH support | 1. PRACH with 120KHz SCS and length 139[/571/1151] | 24-1 |  |  |  | [per UE][per band] |  |  |  |  | Optional with capability signalling |
| 24. NR_ext_to_71GHz | 24-1b | Basic FR2-2 UL support | 1. Support multi-RB PUCCH format 0/1/4 for 120 kHz [2. Support 120kHz subcarrier spacing for UL data and control channels and reference signals in FR2-2] | [24-1] |  |  |  | [per UE][per band] |  |  |  |  | Optional with capability signalling |
| 24. NR_ext_to_71GHz | 24-1c | Multi-PDSCH scheduling | 1. Support multi-PDSCH scheduling by single DCI for the operation with 120 kHz SCS | 24-1 |  |  |  | [per UE][per band] |  |  |  |  | Optional with capability signalling |
| 24. NR_ext_to_71GHz | 24-1d | Multi-PUSCH scheduling | 1. Support multi-PUSCH scheduling by single DCI for the operation with 120 kHz SCS | 24-1b |  |  |  | [per UE][per band] |  |  |  |  | Optional with capability signalling |
| 24. NR_ext_to_71GHz | 24-2 | 120KHz SSB based stand-alone support in FR2-2 | 1. Support 480120KHz SSB for initial access in FR2-2 | 24-1 |  |  |  |  |  |  |  |  | Optional with capability signalling |
| 24. NR_ext_to_71GHz | 24-3 | 480KHz SSB based stand-alone support for initial access in FR2-2 | 1. Support 480KHz SSB for initial access in FR2-2 | 24-1[, 24-2, 24-4] | Yes FFS |  |  | [Per UE/band] |  |  |  | From WID: - In addition to 120kHz, support 480 kHz SSB for initial access with support of CORESET#0/Type0-PDCCH configuration in the MIB with following constraints: - Note: 480 kHz is an optional SSB numerology for initial access for the UE. A UE supporting a band in 52.6-71 GHz must at least support 120 kHz SCS (for initial access and after initial access) - [only 480kHz CORESET#0/Type0-PDCCH SCS supported for 480 kHz SSB SCS] | Optional with capability signalling |
| 24. NR_ext_to_71GHz | 24-4 | 480KHz SCS support | 1. 480KHz SCS for UL data and control channels and reference signal transmission in FR2-2 2. 480KHz SCS for DL data and control channels and reference signal reception in FR2-2 3. 480KHz for SSB monitoring [for non-initial access] 4. Multiple-slot PDCCH monitoring for 480KHz with X=[4] slots [FFS: Component description to be updated once further details of multi-slot monitoring capability are known, e.g., definition of Y] 5. PRACH with 480KHz and length 139/[571] FFS: 6. Support multi-RB PUCCH format 0/1/4 for 480 kHz FFS: 7. Multi-PUSCH/PDSCH scheduling by single DCI for the operation with 480 kHz SCS | 24-1 | Yes |  |  | [Per UE/band] |  |  |  | From WID: In addition to 120kHz SCS, specify new SCS, 480kHz and 960kHz, and define maximum bandwidth(s), for operation in this frequency range for data and control channels and reference signals, only NCP supported. [Agreement: A UE supporting 480 kHz SCS supports multi-slot PDCCH monitoring for 480 kHz SCS. Agreement: Do not support PRACH length L=571, 1151 for 960kHz PRACH and at least L =1151 for 480kHz PRACH] Note: • Resolve the issues of wideband PRACH, multi-RB PUCCH format 0/1/4, and multi-PUSCH/PDSCH scheduling by single DCI, i.e., whether to have components of a single FG or separate FGs, for 120 kHz first, then use the same structure for 480 kHz • Resolve the issue of having separate capabilities for DL and UL (data and control channels as well as reference signals) for 120 kHz first, then use the same structure for 480 kHz | Optional with capability signalling |
| 24. NR_ext_to_71GHz | 24-5 | 960KHz SCS support | 1. 960KHz SCS for UL data and control channels and reference signal transmission in FR2-2 2. 960KHz SCS for DL data and control channels and reference signal reception in FR2-2 3. 960KHz for SSB monitoring 4. Multiple-slot PDCCH monitoring for 960KHz with X=[8] slots [FFS: Component description to be updated once further details of multi-slot monitoring capability are known, e.g., definition of Y] 5. PRACH with 960KHz and length 139 FFS: 6. Support multi-RB PUCCH format 0/1/4 for 960 kHz FFS: 7. Multi-PUSCH/PDSCH scheduling by single DCI for the operation with 960 kHz SCS | 24-1 | Yes |  |  | [Per UE/band] |  |  |  | From WID - In addition to 120kHz SCS, specify new SCS, 480kHz and 960kHz, and define maximum bandwidth(s), for operation in this frequency range for data and control channels and reference signals, only NCP supported. [Agreement: A UE supporting 960 kHz SCS supports multi-slot PDCCH monitoring for 960 kHz SCS. Agreement: Do not support PRACH length L=571, 1151 for 960kHz PRACH and at least L =1151 for 480kHz PRACH] Note: • Resolve the issues of wideband PRACH, multi-RB PUCCH format 0/1/4, and multi-PUSCH/PDSCH scheduling by single DCI, i.e., whether to have components of a single FG or separate FGs, for 120 kHz first, then use the same structure for 480 kHz • Resolve the issue of having separate capabilities for DL and UL (data and control channels as well as reference signals) for 120 kHz first, then use the same structure for 480 kHz | Optional with capability signalling |
| 24. NR_ext_to_71GHz | 24-?6 | Cat 3 or Cat 4 LBT support Uplink channel access procedure for FR2-2 unlicensed operation | - Cat 3 [or Cat 4] LBT support [(not agreed yet if CW is supported, so it can be either Cat 3 or Cat 4 LBT for now. Will update when we have agreement)] - [Support LBT performed per carrier/BWP bandwidth] | 24-1 |  |  |  | [Per band] |  |  |  |  | Optional with capability signalling [A UE that supports FR2-2 must indicate this FG is supported when required by regulation] |
| 24. NR_ext_to_71GHz | 24-?7 | Cat 2 LBT support for uplink channel access procedure for FR2-2 unlicensed operation | 1. Support Cat 2 LBT | 24-1[, 24-6] |  |  |  | [Per band] |  |  |  |  | Optional with capability signalling [A UE that supports FR2-2 must indicate this FG is supported when required by regulation] |
| 24. NR_ext_to_71GHz | 24-8 | 32 DL HARQ processes [for FR 2-2] | Support 32 HARQ processes in DL [for 480/960 kHz] |  |  |  |  |  |  |  |  |  | Optional with capability signalling |
| 24. NR_ext_to_71GHz | 24-9 | 32 UL HARQ processes [for FR 2-2] | Support 32 HARQ processes in UL [for 480/960 kHz] |  |  |  |  |  |  |  |  |  | Optional with capability signalling |