3GPP TSG RAN WG1 #107	R1-2111154

e-Meeting, 11 – 19 November, 2021

Agenda item:		8.16.3

Source:	Nokia, Nokia Shanghai Bell

Title:	On UE features for enhanced IIoT and and URLLC

Document for:		Discussion and Decision

# Introduction

In RAN1#106bis-e the first baseline has been agreed for a number of UE features. In this contribution we present our views on the UE features for enhanced IIoT and and URLLC.

# Discussion

Detailed comments on UE features are listed below. For reference, the baseline agreements from RAN1#106bis are available in the Annex.

- 25-1:

  - No need for separate capability for handling of the collision for the same HARQ process due to deferred SPS HARQ-ACK

- 25-3:

  - No need for separate capabilities per UCI type

- 25-3a:

  - The pre-requisite feature group from Cov. Enh. WI of the dynamic PUCCH repetition indication should be 30-5.

  - No need for separate capabilities per UCI type

- 25-5

  - RAN1 agreed to support UE capability signaling of {1,2,4,8} CBs, and therefore the description of component 2 in ’Components’ and ’Note’ would need to be changed accordingly (in green):

| 25. NR_IIOT_URLLC_enh | 25-6 | Enhanced type 3 HARQ-ACK codebook feedback | 1. Support feedback of enhanced type 3 HARQ-ACK codebook, triggered by a DCI 1_1 and DCI format 1_2 (for a UE supporting DCI format 1_2, and 1-1) 2. Support configuration of up to 8X enhanced type 3 HARQ-ACK codebooks. 3. Support feedback of a dynamically selected enhanced type 3 HARQ-ACK codebook based on triggering information in DCI 1_1 and DCI 1_2 (for a UE supporting DCI format 1_2, 11-1) 4. Support transmission of enhanced type 3 HARQ-ACK codebook using the first or second PUCCH configuration based on PHY priority indication in the triggering DCI (for a UE supporting two HARQ-ACK codebooks / PUCCH config in 11-4) | 10-16 | For component 2, the UE indicates its capability in the number of enhanced type 3 HARQ-ACK codebooks: {1,2,4,8...,X} For component 3, the dynamic indication is only supported if the UE for component 2 supports more than one enhanced type 3 HARQ-ACK codebook to be configured | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- |

- 25-9:

  - No need for separate capabilities for different numerologies

  - Minor editorial suggestion to remove the ’carrier’ in the component description (in green)

| 25-9 | Semi-static PUCCH cell (FFS or PUCCH carrier) switching | Semi-static PUCCH cell (FFS or PUCCH carrier)switching using configured time-domain pattern of applicable PUCCH cell / carrier FFS whether to separate the capability for different numerologies |
| --- | --- | --- |

- 25-10:

  - No need for separate capabilities for different numerologies

- 25-15:

  - Square brackets can be removed, i.e. UE feature is needed, as RAN1 finalized the details of this feature during RAN1#106bis-e:

Agreement For collision between HP CG PUSCH and LP DG PUSCH, if MAC delivers two MAC PDUs to PHY, PHY layer can make the prioritization so that the UE is expected to transmit the CG PUSCH and cancel the DG PUSCH at latest from the first symbol that is overlapping with the CG PUSCH. - Note: For the DG PUSCH, it is up to UE implementation to handle OFDM symbols of the DG PUSCH before the start of HP CG PUSCH which are nonoverlapping with the HP CG PUSCH. - FFS: How to handle the collision when there is repetition for CG and/or DG PUSCH

- 25-17:

  - Merge with FG 25-16, or at least add FG 25-16 as pre-requisite. It is not reasonable to assume a UE would be able to do multiplex HARQ-ACK of different priorities only when piggybacking on PUSCH but would be unable to do the same with PUCCH.

  - No need for separate capabilities per UCI type

Proposal: Consider the observations and modifications proposed above for the next version of the corresponding RAN1 UE features list.

# Conclusion

In this contribution we have presented our views on UE features for enhanced IIoT and and URLLC, with the following proposal:

Proposal: Consider the observations and modifications proposed above for the next version of the corresponding RAN1 UE features list.

# References

- Chair’s Notes RAN1#106bis-e, Oct. 2021

# Annex

Agreement

FG 25-1 is kept as “SPS HARQ-ACK deferral in case of TDD collision” as follows.

| 25.NR_IIOT_URLLC_enh | 25-1 | SPS HARQ-ACK deferral in case of TDD collision | 1. Idenfify HARQ-ACK bits of active SPS configurations for deferral in the initial PUCCH slot 2. Determination of the target PUCCH slot for SPS HARQ-ACK deferral 3. Multiplexing and transmission of deferred SPS HARQ-ACK information in the target PUCCH slot FFS whether to separate capability for handling of the collision for the same HARQ process due to deferred SPS HARQ-ACK | 5-18 | Yes | N/A |  | Per UE | No | No | N/A |  | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Note that yellow highlight means FFS and to be discussed further. These parts are provides as placeholders.

Agreement

FG 25-2 is kept as “Repetitions for PUCCH format 0, and 2 over multiple slots with K = 2, 4, 8” as follows.

| 25.NR_IIOT_URLLC_enh | 25-2 | Repetitions for PUCCH format 0, and 2 over multiple slots with K = 2, 4, 8 | Repetitions for PUCCH format 0 and 2 over multiple slots with K = 2, 4, 8 | 4-23 | Yes | N/A |  | Per UE | No | No | N/A |  | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Note that yellow highlight means FFS and to be discussed further. These parts are provides as placeholders.

Agreement

FG 25-3 is kept as “Repetitions for PUCCH format 0, 1, 2, 3 and 4 over multiple PUCCH subslots with configured K = 2, 4, 8” as follows.

| 25.NR_IIOT_URLLC_enh | 25-3 | Repetitions for PUCCH format 0, 1, 2, 3 and 4 over multiple PUCCH subslots with configured K = 2, 4, 8 | Repetitions for PUCCH format 0, 1, 2, 3 and 4 over multiple PUCCH subslots with RRC configured repetition factor K = 2, 4, 8 FFS whether to separate the capability per UCI type | 4-23 11-3 | Yes | N/A |  | Per UE | No | No | N/A |  | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Note that yellow highlight means FFS and to be discussed further. These parts are provides as placeholders.

Agreement

FG 25-3a is kept as “Repetitions for PUCCH format 0, 1, 2, 3 and 4 over multiple PUCCH subslots using dynamic repetition indication” as follows.

| 25.NR_IIOT_URLLC_enh | 25-3a | Repetitions for PUCCH format 0, 1, 2, 3 and 4 over multiple PUCCH subslots using dynamic repetition indication | Repetitions for PUCCH format 0, 1, 2, 3 and 4 over multiple PUCCH subslots based on dynamic repetition indication. FFS whether to separate the capability per UCI type | 25-3 30-5 | Yes | N/A |  | Per UE | No | No | N/A |  | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Note that yellow highlight means FFS and to be discussed further. These parts are provides as placeholders.

Agreement

FG 25-4 is kept as “One-shot HARQ ACK feedback triggered by DCI format 1_2” as follows.

FG 25-5 is kept as “PHY priority handling for one-shot HARQ ACK feedback” as follows.

FG 25-6 is kept as “Enhanced type 3 HARQ-ACK codebook feedback” as follows.

| 25.NR_IIOT_URLLC_enh | 25-4 | One-shot HARQ ACK feedbacktriggered by DCI format 1_2 | 1. Support feedback of type 3 HARQ-ACK codebook, triggered by a DCI 1_2 scheduling a PDSCH 2. Support feedback of type 3 HARQ-ACK codebook, triggered by a DCI 1_2 without scheduling a PDSCH using a reserved FDRA value | 10-16 11-1 | Yes | N/A |  | Per UE | No | No | N/A |  | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 25. NR_IIOT_URLLC_enh | 25-5 | PHY priority handling for one-shot HARQ ACK feedback | Support transmission of type 3 HARQ-ACK codebook using the first or second PUCCH configuration based on PHY priority indication in the triggering DCI | 10-16 11-4 | Yes | N/A |  | Per UE | No | No | N/A |  | Optional with capability signaling |
| 25. NR_IIOT_URLLC_enh | 25-6 | Enhanced type 3 HARQ-ACK codebook feedback | 1. Support feedback of enhanced type 3 HARQ-ACK codebook, triggered by a DCI 1_1 and DCI format 1_2 (for a UE supporting DCI format 1_2, and 1-1) 2. Support configuration of up to X enhanced type 3 HARQ-ACK codebooks. 3. Support feedback of a dynamically selected enhanced type 3 HARQ-ACK codebook based on triggering information in DCI 1_1 and DCI 1_2 (for a UE supporting DCI format 1_2, 11-1) 4. Support transmission of enhanced type 3 HARQ-ACK codebook using the first or second PUCCH configuration based on PHY priority indication in the triggering DCI (for a UE supporting two HARQ-ACK codebooks / PUCCH config in 11-4) | 10-16 | Yes | N/A |  | Per UE | No | No | N/A | For component 2, the UE indicates its capability in the number of enhanced type 3 HARQ-ACK codebooks: {1,...,X} For component 3, the dynamic indication is only supported if the UE for component 2 supports more than one enhanced type 3 HARQ-ACK codebook to be configured | Optional with capability signaling |

Note that yellow highlight means FFS and to be discussed further. These parts are provides as placeholders.

Agreement

FG 25-7 is kept as “Triggered HARQ-ACK codebook re-transmission” as follows.

| 25. NR_IIOT_URLLC_enh | 25-7 | Triggered HARQ-ACK codebook re-transmission | 1. Support HARQ-ACK re-transmission from an earlier PUCCH slot based on the triggering information in DCI format 1_1 and DCI format 1_2 (for a UE supporting DCI format 1_2, 11-1) 2. Support the related PHY priority handling in terms of HARQ-ACK codebook selection and the applicable PUCCH configuration (for a UE supporting two HARQ-ACK codebooks / PUCCH config in 11-4) |  | Yes | N/A |  | Per UE | No | No | N/A |  | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Note that yellow highlight means FFS and to be discussed further. These parts are provides as placeholders.

Agreement

FG 25-8 is kept as “Semi-static HARQ-ACK codebook for sub-slot PUCCH” as follows

| 25.NR_IIOT_URLLC_enh | 25-8 | Semi-static HARQ-ACK codebook for sub-slot PUCCH | Semi-static (Type 1) HARQ-ACK codebook for sub-slot based PUCCH configuration | 4-11 11-3 | Yes | N/A |  | Per UE | No | No | N/A |  | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Note that yellow highlight means FFS and to be discussed further. These parts are provides as placeholders.

Agreement

FG 25-9 is kept as “Semi-static PUCCH cell switching” as follows.

FG 25-10 is kept as “PUCCH cell switching based on dynamic indication” as follows.

| 25.NR_IIOT_URLLC_enh | 25-9 | Semi-static PUCCH cell (FFS or PUCCH carrier) switching | Semi-static PUCCH cell (FFS or PUCCH carrier)switching using configured time-domain pattern of applicable PUCCH cell / carrier FFS whether to separate the capability for different numerologies |  | Yes | N/A |  | Per UE | No | No | N/A |  | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 25.NR_IIOT_URLLC_enh | 25-10 | PUCCH cell (FFS or PUCCH carrier)switching based on dynamic indication | PUCCH cell (FFS or PUCCH carrier)switching based on dynamic indication in the DCI scheduling the PUCCH FFS whether to separate the capability for different numerologies |  | Yes | N/A |  | Per UE | No | No | N/A |  | Optional with capability signaling |

Note that yellow highlight means FFS and to be discussed further. These parts are provides as placeholders.

Agreement

FG 25-11 is kept as “4-bits subband CQI” as follows.

| 25.NR_IIOT_URLLC_enh | 25-11 | 4-bits subband CQI | Subband CQI reporting with 4 bits per subband |  | Yes | N/A |  | Per UE | No | No | N/A |  | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Note that yellow highlight means FFS and to be discussed further. These parts are provides as placeholders.

Agreement

FG 25-12 is kept as “UE initiating a semi-static channel occupancy with configurations dependent on gNB semi-static channel access configurations” as follows.

| 25.NR_IIOT_URLLC_enh | 25-12 | UE initiating a semi-static channel occupancy with configurations dependent on gNB semi-static channel access configurations | Support initiating a semi-static channel access occupancy by the UE where the corresponding period is the same as, integer multiple of, or inter-factor of the period configured for a semi-static channel occupancy that can be initiated by gNB. | 10-1a | Yes | N/A |  | Per band | N/A | N/A | N/A | The signaling is per band but is only expected for a band where shared spectrum channel access must be used | Optional with capability signaling [This FG is a part of basic operation for following scenarios defined in TS38.300 Scenario A2, B, C, D and E with semi-static channel access mode] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Note that yellow highlight means FFS and to be discussed further. These parts are provides as placeholders.

Agreement

FG 25-13 is kept as “UE initiating a semi-static channel occupancy with independent configurations from gNB semi-static channel access configurations” as follows.

| 25.NR_IIOT_URLLC_enh | 25-13 | UE initiating a semi-static channel occupancy with independent configurations from gNB semi-static channel access configurations | Support initiating a semi-static channel access occupancy by the UE where the corresponding period is independently configured from the period configured for a semi-static channel occupancy that can be initiated by gNB. | 10-1a | Yes | N/A |  | Per band | N/A | N/A | N/A | The signaling is per band but is only expected for a band where shared spectrum channel access must be used | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Note that yellow highlight means FFS and to be discussed further. These parts are provides as placeholders.

Agreement

FG 25-14 is kept with square brackets as “PHY prioritization of overlapping low-priority DG-PUSCH and high-priority CG-PUSCH” for further discussion as follows.

FG 25-15 is kept with square brackets as “PHY prioritization of overlapping high-priority DG-PUSCH and low-priority CG-PUSCH” for further discussion as follows.

| 25. NR_IIOT_URLLC_enh | [25-14] | PHY prioritization of overlapping low-priority DG-PUSCH and high-priority CG-PUSCH | Support PHY prioritization for the case where low-priority DG-PUSCH collides with high-priority CG-PUSCH | 12-1 | Yes | N/A |  | Per band | N/A | N/A | N/A |  | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 25. NR_IIOT_URLLC_enh | [25-15] | PHY prioritization of overlapping high-priority DG-PUSCH and low-priority CG-PUSCH | Support PHY prioritization of overlapping high-priority dynamic grant PUSCH and low-priority configured grant PUSCH on a BWP of a serving cell | 12-1 | Yes | N/A |  | Per band | N/A | N/A | N/A |  | Optional with capability signaling |

Note that yellow highlight means FFS and to be discussed further. These parts are provides as placeholders.

Agreement

FG 25-16 is kept with as “HARQ-ACK with different priorities multiplexing using a PUCCH” as follows.

FG 25-17 is kept with as “HARQ-ACK piggybacked on a PUSCH of a different priority” as follows.

| 25.NR_IIOT_URLLC_enh | 25-16 | HARQ-ACK with different priorities multiplexing using a PUCCH | 1. Support multiplexing a high-priority HARQ-ACK and a low-priority HARQ-ACK into a PUCCH. Support separate coding for the two HARQ-ACKs. 2. [Support multiplexing a low-priority HARQ-ACK and a high-priority SR into a PUCCH for some HARQ-ACK/SR PF combinations (FFS applicable combinations).] 3. [Support multiplexing a low-priority HARQ-ACK, a high-priority HARQ-ACK and a high-priority SR into a PUCCH.] FFS whether to merge with FG 25-17 FFS whether to separate capability for different UCI type | 11-3 | Yes | N/A |  | Per UE | No | No | N/A |  | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 25.NR_IIOT_URLLC_enh | 25-17 | HARQ-ACK piggybacked on a PUSCH of a different priority | 1. Support multiplexing a low-priority HARQ-ACK in a high-priority PUSCH (conveying UL-SCH only). Support separate beta_offset values for this priority combination. 2. Support multiplexing a high-priority HARQ-ACK in a low-priority PUSCH (conveying UL-SCH only). Support separate beta_offset values for this priority combination. 3. Support multiplexing a low-priority HARQ-ACK, a high-priority PUSCH conveying UL-SCH, a high-priority HARQ-ACK and/or CSI. 4. Support multiplexing a high-priority HARQ-ACK, a low-priority PUSCH conveying UL-SCH, a low-priority HARQ-ACK and/or CSI. FFS whether to merge into FG 25-16 FFS whether to separate capability for different UCI type | 11-3 12-1 | Yes | N/A |  | Per UE | No | No | N/A |  | Optional with capability signaling |

Note that yellow highlight means FFS and to be discussed further. These parts are provides as placeholders.

Agreement

FG 25-18 is kept as “Parallel PUCCH and PUSCH transmission across CCs in inter-band CA” as follows.

| 25.NR_IIOT_URLLC_enh | 25-18 | Parallel PUCCH and PUSCH transmission across CCs in inter-band CA | Support simultaneous PUCCH/PUSCH transmissions on different cells [at least] for inter-band CA. |  | Yes | N/A |  | Per UE | No | No | N/A |  | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Note that yellow highlight means FFS and to be discussed further. These parts are provides as placeholders.

Agreement

Rel-16 capability signaling for PUSCH repetition Type B (FG 11-5) is reused for unlicensed spectrum.

Agreement

- FG 25-1 is applicable to TDD only

  - It is clarified in the column of “Need of FDD/TDD differentiation”

Agreement

- FGs 25-9 and 25-10 are applicable to TDD only

  - It is clarified in the column of “Need of FDD/TDD differentiation”, FFS details.

Agreement

- Remove “[This FG is a part of basic operation for following scenarios defined in TS38.300 Scenario A2, B, C, D and E with semi-static channel access mode]” from the column of “Mandatory/Optional” in FG 25-12

Agreement

- FG 25-12 is added as prerequisite feature group for FG 25-13