3GPP TSG RAN WG1 #107	R1-2111156

e-Meeting, 11 – 19 November, 2021

Agenda item:		8.16.5

Source:	Nokia, Nokia Shanghai Bell

Title:	On UE features for NR positioning enhancements

Document for:		Discussion and Decision

# Introduction

In RAN1#106bis-e the first baseline has been agreed for a number of UE features. In this contribution we present our views on the UE features for NR positioning enhancements.

# Discussion

Detailed comments on UE features are listed below. For reference, the baseline agreements from RAN1#106bis are available in the Annex.

- 27-1:

  - Add FG 13-3 as pre-requisite, as this is an enhancement to Rel-16 DL-TDOA.

  - Rx TEG is not depending on the positioning technique (DL-TDOA, Multi-RTT), hence no need for separate row.

- 27-1-5

  - Add new FG to cover the case of phase center offset compensation for TEGs. See details for proposed FG in the Annex.

- 27-2:

  - Add FG 13-4, 13-8 as pre-requisite, as this requires SRS resources for positioning

Add value 24, as given by RAN1 agreement from RAN1#106bis-e:

Agreement:

The agreement from RAN1#106e on the number of DL PRS RSRP measurements per TRP is extended as follows:

- For UE-A DL-AOD, support reporting more than 8 up to 16 N DL PRS RSRP measurements per TRP, where N is UE capability and candidate values include {16,24}.

- For UE-A DL-AOD, support reporting more than 8 up to 16 M first path PRS RSRP measurements per TRP, where M is a UE capability

  - FFS: Values of M. Candidate values include {2,4,8,16,24}.

  - FFS: Whether M is always equal to N

- Note: Multiple RSRPs corresponding to same or different Rx Beam index should be able to be reported for a given PRS resource for same or different timestamps.

- Note: the maximum number of DL PRS RSRP associated with the same Rx beam index is up to the UE implementation

- 27-3:

  - Add FG 13-4, 13-8 as pre-requisite, as this requires SRS resources for positioning

- 27-2-1:

  - Add ”13-2 or 13-3” as pre-requisite, as it requires PRS reception for DL-AoD or DL-TDOA

- 27-2-2:

  - Add FG 13-5 as pre-requisite as this extends the Rel-16 parameter space.

- 27-3-1:

  - Add FG 13-1 as pre-requisite

- 27-3-2:

  - Confirm the FG, details to be decided later.

  - Add FG 13-1 as pre-requisite

  - Per UE capability, the processing itself can be a per band indication

- 27-3-3:

  - Confirm the FG, details to be decided later.

- 27-4-1:

  - Need to add corresponding Rel-16 FGs on measurement reports as pre-requisites, as this is an add-on.

  - No need for separate capability for soft and hard indication. OK to list as component in this capability though.

  - No need to signal candidate values.

- 27-5-1:

  - Add FG 13-1 as pre-requisite

Proposal: Consider the observations and modifications proposed above for the next version of the corresponding RAN1 UE features list.

# Conclusion

In this contribution we have presented our views on UE features for NR positioning enhancements, with the following proposal:

Proposal: Consider the observations and modifications proposed above for the next version of the corresponding RAN1 UE features list.

# References

- Chair’s Notes RAN1#106bis-e, Oct. 2021

# Annex

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (Sidelink WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 27. NR_pos_enh | 27-1-x1 | Mitigation of UE Rx timing delays Maximum number of UE-RxTEGs [for UE-assisted DL TDOA and/or Multi-RTT positioning] | The maximum number of UE-RxTEG per UE, which is supported and reported by UE [for DL TDOA [and/or Multi-RTT positioning] FFS: the values (>1). FFS: whether to have a value=1 to indicate UE Rx timing errors is well calibrated FFS: whether to have separate values/FGs for DL TDOA and/or Multi-RTT positioning [If UE supports this capability with the values > 1, the UE supports including one UE Rx TEG ID for the RSTD reference time and one UE Rx TEG ID for each DL RSTD measurement (including each additional DL RSTD measurement), in a DL TDOA measurement report] | [13-3] | No |  | Mitigation of UE Rx timing delays is not supported | FFS: Per UE or per band | n/a | n/a | n/a | [The candidate values are {1,2,4,6,8,12,16,24,32}] Need for location server to know if the feature is supported. FFS: Separate row for “Support of UE-RxTEG reporting for DL-TDOA”, and “Support of UE-RxTEG reporting for M-RTT” | Optional with capability signaling |
| 27. NR_pos_enh | 27-x1-2 | Mitigation of UE Tx timing delays Maximum number of UE-TxTEGs [for UL TDOA and/or Multi-RTT positioning] | The maximum number of UE-TxTEG per UE, which is supported and reported by UE [for UL TDOA and/or Multi-RTT positioning] FFS: the values (>1). FFS: whether to have a value=1 to indicate UE Tx timing errors is well calibrated FFS: whether a UE supports to have different values/FGs for UL TDOA and/or Multi-RTT positioning FFS: Separate row for “Support of UE-TxTEG reporting for MRTT”, and a separate row for the “maximum number of TxTEGs for RTT” [If UE supports this capability with the values > 1, the UE supports to provide the association information of UL SRS resources for positioning with Tx TEGs to the LMF. - FFS: Whether the association information is sent directly from UE to LMF, or is first provided to gNB and then forwarded to LMF] | [13-4, 13-8] | FFS |  | Mitigation of UE Tx timing delays is not supported | FFS: Per UE or per band or per FS | n/a | n/a | n/a | [The candidate values are {1, 2, 4, 8, 16, 32}] Need for location server to know if the feature is supported. FFS: whether gNB needs to know if the feature is supported. | Optional with capability signaling |
| 27. NR_pos_enh | 27-x1-3 | Mitigation of UE RxTx timing delays Maximum number of UE-RxTxTEGs for Multi-RTT | The maximum number of UE-RxTxTEG per UE, which is supported and reported by UE for Multi-RTT positioning FFS: the values (>1) FFS: whether to have a value=1 to indicate UE RxTx timing errors is well calibrated [If a UE support this capability with the values > 1, the UE supports reporting of UE RxTx TEG ID with UE Rx-Tx time difference measurements for Multi-RTT positioning] | [13-4, 13-8] | No |  | Mitigation of UE RxTx timing delays is not supported | FFS: Per UE or per band | n/a | n/a | n/a | [The candidate values are {1, 2, 4, 6, 8, 12, 16, 24, 32, 64, [128]}] Need for location server to know if the feature is supported. FFS: Separate row for “Support of UE-RxTxTEG reporting for Multi-RTT” | Optional with capability signaling |
| 27. NR_pos_enh | 27-x1-4 | The maximum Number of UE Rx TEGs for measuring the same DL PRS resource | The maximum number of different UE-RxTEGs that a UE can support to measure the same DL PRS of a TRP. FFS: The values (>1) | 27-x1 | No |  | Mitigation of UE Rx timing delays by using different Rx TEGs are not supported | FFS: Per UE or per band | n/a | n/a | n/a | [The candidate values are {1, 2, 4, 8}] Need for location server to know if the feature is supported. [Note: UE measures the same instance of the DL PRS with multiple RX TEGs] FFS: Separate row for “Support measuring the same DL PRS of a TRP with different UE-RxTEGs” FFS: Separate row for “The maximum Number of UE Rx TEGs for measuring the same DL PRS resource simultaneously” | Optional with capability signaling |
| 27. NR_pos_enh | 27-z2-1 | Support of [UE-assisted] DL PRS RSRP of the first [or additional] path [for DL-AoD] | UE’s capability to Support providing the of [measuring and reporting the] PRS RSRP of the first [or additional] path [for DL-AoD positioning method] [Note: Applicable for DL-TDOA and Multi-RTT] | [13-2 or 13-3] | No |  |  | FFS: Per UE or per band | n/a | n/a | n/a | Need for location server to know if the feature is supported. | Optional with capability signaling |
| 27. NR_pos_enh | 27-z2-2 | Support of DL PRS RSRP reporting for more than 8 measurements [for UE-assisted DL-AoD positioning] | UE capability to Support reporting K> 8 DL PRS RSRP measurements per TRP. FFS: the values of K •Note: Multiple RSRPs corresponding to same or different Rx Beam index should be able to be reported for a given PRS resource for different timestamps. Note: Additional capability may be added: •FFS: Additional capability may be added to limit the maximum number of DL PRS RSRP associated with the same Rx beam index | [13-5] | No |  | UE report of more than 8 DL PRS-RSRP is not supported. | FFS: Per UE or per band | n/a | n/a FFS | n/a | [The candidate values are {12, 16, 32, 64}] Need for location server to know if the feature is supported. | Optional with capability signaling |
| 27. NR_pos_enh | 27-u3-1 | Support of M-sample measurements [of DL PRS measurement on single DL PRS period/occasion] | The capability to support providing reporting a measurement based on measuring M samples (instances) of a DL PRS resource set M= [1[,2-34]. FFS: other values. If the UE does not provide the capability, the UE [is assumed to] support M=4 only. | [13-1] | No |  |  | FFS: Per UE or per band | n/a | n/a | n/a | [The candidate values are {1, 2, 3, 4}] Need for location server to know if the feature is supported. | Optional with capability signaling |
| 27. NR_pos_enh | 27-u53-2 | DL PRS measurement outside MG [and in a PRS processing priority window] - processing types | 1. Supported PRS processing types subject to the UE determining that DL PRS to be higher priority for PRS measurement outside MG [and in a PRS processing priority window]. Candidate values: {Type 1A, Type 1B, Type 2} Note: - Type 1A refers to DL PRS being prioritized over other DL signals/channels in all OFDM symbols within the PRS processing priority window. The DL signals/channels from all DL CCs (per UE) are affected. - Type 21B refers to DL PRS being prioritized over other DL signals/channels in all OFDM symbols within the PRS processing priority window. The DL signals/channels from certain DL CCs are affected. - Type 2C refers to DL PRS being prioritized over other DL signals/channels only in DL PRS symbols within the PRS processing priority window. Note: When the UE determines higher priority for other DL signals/channels over the PRS measurement/processing, the UE is not expected to measure/process DL PRS which is applicable to all of the above capability options. [Note: Within a PRS processing window, UE measurement is inside the active DL BWP with PRS having the same numerology as the active DL BWP] | [13-1] | No FFS |  |  | FFS: Per UE or per band | n/a | n/a | n/a | Need for location server to know if the feature is supported. FFS: Separate feature group for a UE to declare support of each of the Type-1A, Type-1B, Type-2” capabilities | Optional with capability signaling |
| 27. NR_pos_enh | 27-u63-3 | DL PRS Processing Capability outside MG - buffering capability | 1. DL PRS buffering capability: Type 1 or Type 2 a) Type 1 – sub-slot/symbol level buffering T: [{8, 16, 20, 30, 40, 80, 160, 320, 640, 1280}] ms b) Type 2 – slot level buffering 2. Duration of DL PRS symbols N in units of ms a UE can process every T ms assuming maximum DL PRS bandwidth in MHz, which is supported and reported by UE. a) Type 1 – sub-slot/symbol level buffering b) N: {0.125, 0.25, 0.5, 1, 2, 4, 6, 8, 12, 16, 20, 25, 30, 32, 35, 40, 45, 50} ms 3. Max number of DL PRS resources that UE can process in a slot under it a) FR1 bands: {1, 2, 4, 6, 8, 12, 16, 24, 32, 48, 64} for each SCS: 15kHz, 30kHz, 60kHz b) FR2 bands: {1, 2, 4, 6, 8, 12, 16, 24, 32, 48, 64} for each SCS: 60kHz, 120kHz | 27-u5 | No |  |  | Per band | n/a | n/a | n/a | FFS: Separate feature group for a UE to declare PRS processing capabilities of each of the Type-1A, Type-1B, Type-2” capabilities | Optional with capability signaling |
| 27. NR_pos_enh | 27-v4-1 | LOS/NLOS Indicator [for UE-assisted positioning] | UE’s capability to Support reporting LoS/NLoS indicator to LMF [for RSTD and UE Rx-Tx time difference measurements to LMF for DL and DL+UL positioning] FFS: whether to have separate capability component/FG for RSTD and UE Rx-Tx time difference measurements. FFS: whether to have separate capability component for hard and soft indication |  | No |  |  | FFS: Per UE or per band | n/a | n/a | n/a | [The candidate value are [0:0.1:1]] Need for location server to know if the feature is supported. | Optional with capability signaling |
| 27. NR_pos_enh | 27-w5-1 | Support of [UE-initiated] on-demand PRS | UE’s capability to support UE-initiated on-demand DL PRS [request signalling] | [13-1] | No |  |  | Per UE | n/a | n/a | n/a | FFS: Need for location server to know if the feature is supported. | Optional with capability signaling |
| 27. NR_pos_enh | 27-t1 | Multipath reporting for DL-TDOA | UE’s capability to support up to N>2 additional path relative timing reporting for DL-TDOA and multi RTT FFS: value of N FFS: whether to have separate capability component for DL-TDOA and UE multi-RTT additional path relative timing reporting time difference measurements. FFS: additional capability to also include power reporting. |  | No |  |  | FFS | n/a | n/a | n/a | FFS: Need for location server to know if the feature is supported. | Optional with capability signaling |
| 27. NR_pos_enh | 27-t2 | Multipath reporting for Multi-RTT | UE’s capability to support up to N>2 additional path relative timing reporting for DL-TDOA and multi RTT FFS: value of N FFS: whether to have separate capability component for DL-TDOA and UE multi-RTT additional path relative timing reportingtime difference measurements. FFS: additional capability to also include power reporting. |  | No |  |  | FFS | n/a | n/a | n/a | FFS: Need for location server to know if the feature is supported. | Optional with capability signaling |

New FG proposed:

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (Sidelink WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 27.NR_pos_enh | 27-1-5 | Phase center offset compensation for TEGs | Indicates if the UE supports PCO compensation as part of the TEG margin calculation. FFS: Separate capabilities for UE Rx TEG, UE Tx TEG, UE RxTx TEG. | [13-3,13-41,3-8] | No |  | PCO errors are not calibrated as part of TEGs | [Per UE] | n/a | n/a | n/a | Need for location server to know if the feature is supported. | Optional with capability signaling |