3GPP TSG RAN WG1 #107	R1-2111160

e-Meeting, 11 – 19 November, 2021

Agenda item:		8.16.9

Source:	Nokia, Nokia Shanghai Bell

Title:	On UE features for NB-IoT and LTE-MTC enhancements

Document for:		Discussion and Decision

# Introduction

In RAN1#106bis-e the first baseline has been agreed for a number of UE features. In this contribution we present our views on the UE features for NB-IoT and LTE-MTC enhancements.

# Discussion

Detailed comments on UE features are listed below. For reference, the baseline agreements from RAN1#106bis are available in the Annex.

- FGs 1-1, 1-2, 1-3, 1-4: per UE

- For component 3 in FG 1-3, select option 2, i.e. UE supports both Alt-1 and Alt-2e

Proposal: Consider the observations and modifications proposed above for the next version of the corresponding RAN1 UE features list.

# Conclusion

In this contribution we have presented our views on UE features for NB-IoT and LTE-MTC enhancements, with the following proposal:

Proposal: Consider the observations and modifications proposed above for the next version of the corresponding RAN1 UE features list.

# References

- Chair’s Notes RAN1#106bis-e, Oct. 2021

# Annex

Agreement

FG 1-3 is kept as “14 HARQ processes for PDSCH for HD-FDD Cat. M1 UEs” as follows.

| 1. NB_IOTenh4_LTE_eMTC6 | 1-3 | 14 HARQ processes for PDSCH for HD-FDD Cat. M1 UEs | 1. Support of 14 DL HARQ processes for unicast in HD-FDD in CE mode A in RRC_CONNECTED 2. PDSCH scheduling delay 3. HARQ-ACK delay solution with Alt-1 and Alt-2e | 1. Category M1 2. HD-FDD 3. CE Mode A 4. Support of 10 DL HARQ processes in HD-FDD in CE mode A 5. HARQ-ACK bundling support in HD-FDD in CE mode A | Yes | N/A | The network cannot enable 14 HARQ processes for the UE | Per UE | FDD only | N/A | - PDSCH scheduling delay: - 2 BL/CE DL subframes. - 1 BL/CE DL subframe + 1 subframe + 3 BL/CE UL subframes + 1 subframe + 1 BL/CE DL subframe. - 1 subframe + 3 BL/CE UL subframes + 1 subframe + 2 BL/CE DL subframes. - HARQ-ACK delay: - Alt-1: The HARQ-ACK delay is determined through an expression consisting of different subframe types (Using a similar principle as the PDSCH scheduling delay). - Alt-2e: The HARQ-ACK delay is determined following the legacy approach. That is, the “HARQ-ACK delay” is kept expressed in terms of “absolute subframes”. | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Note that yellow highlight means FFS and to be discussed further. These parts are provides as placeholders.

Agreement

FG 1-1 is kept as “16-QAM for unicast NPDSCH” as follows.

| 1. NB_IOTenh4_LTE_eMTC6 | 1-1 | 16-QAM for unicast NPDSCH | 1. Reception of unicast NPDSCH modulated with 16-QAM 2. CQI report to support 16-QAM modulation 3. Downlink power allocation for 16-QAM | Category NB-2 | Yes | N/A | The network cannot schedule a unicast NPDSCH modulated with 16-QAM for the UE | Per UE | [Yes] | N/A |  | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Note that yellow highlight means FFS and to be discussed further. These parts are provides as placeholders.

Agreement

FG 1-2 is kept as “16-QAM for unicast NPUSCH” as follows.

| 1. NB_IOTenh4_LTE_eMTC6 | 1-2 | 16-QAM for unicast NPUSCH | 1. Transmission of unicast NPUSCH modulated with 16-QAM 2. New term in the UE’s transmit power control equation. | Category NB-2 | Yes | N/A | The network cannot schedule a unicast NPUSCH modulated with 16-QAM for the UE | Per UE | [Yes] | N/A |  | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Note that yellow highlight means FFS and to be discussed further. These parts are provides as placeholders.

Agreement

FG 1-4 is kept as “A maximum DL TBS of 1736 bits for HD-FDD Cat. M1 Ues in CE mode A only” as follows.

| 1. NB_IOTenh4_LTE_eMTC6 | 1-4 | A maximum DL TBS of 1736 bits for HD-FDD Cat. M1 Ues in CE mode A only | 1. Support of 1736 bits max DL TBS for unicast in HD-FDD in CE mode A in RRC_CONNECTED | 1. Category M1 2. HD-FDD 3. CE Mode A | Yes | N/A | The network cannot schedule a PDSCH with TBS larger than 1000 bits for Cat. M1 Ues | Per UE | FDD only | N/A |  | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Note that yellow highlight means FFS and to be discussed further. These parts are provides as placeholders.

Agreement

- For component 3 in FG 1-3, select one of the following options:

  - Option 1: UE reports one of {alt1, alt2-e, both}

  - Option 2: UE supports both Alt-1 and Alt-2e

Agreement

- Following features are removed from the prerequisite feature groups for FG 1-3

  - CE Mode A

  - Support of 10 DL HARQ processes in HD-FDD in CE mode A

  - HARQ-ACK bundling support in HD-FDD in CE mode A

Agreement

- CE Mode A is removed from the prerequisite feature groups for FG 1-4