3GPP TSG RAN WG1 #107	R1-2111163

e-Meeting, 11 – 19 November, 2021

Agenda item:		8.16.13

Source:	Nokia, Nokia Shanghai Bell

Title:	On UE features for DSS

Document for:		Discussion and Decision

# Introduction

In RAN1#106bis-e the first baseline has been agreed for a number of UE features. In this contribution we present our views on the UE features for DSS.

# Discussion

Detailed comments on UE features are listed below. For reference, the baseline agreements from RAN1#106bis are available in the Annex.

- 34-1:

  - Type is per BC

- 34-2:

  - Type is per BC

- 35-1:

  - Type can be confirmed as “per UE”.

  - Need for FR1/FR2 differentiation should be ”yes”, since testability needs have shown that in cases like this FR differentiation has been needed.

  - Add a note stating that The NZP-CSI-RS configured as temporary RS for fast SCell activation are not considered when counting the maximum NZP-CSI-RS configurations of FG2-33.

Proposal: Consider the observations and modifications proposed above for the next version of the corresponding RAN1 UE features list.

# Conclusion

In this contribution we have presented our views on UE features for DSS, with the following proposal:

Proposal: Consider the observations and modifications proposed above for the next version of the corresponding RAN1 UE features list.

# References

- Chair’s Notes RAN1#106bis-e, Oct. 2021

# Annex

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (Sidelink WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 34. NR_DSS | 34-12 | Cross-carrier scheduling from SCell to PCell/PSCell (Type B) | [Support of Cross-carrier scheduling (CCS) from sSCell to PCell/PSCell (Type B)] - CIF configured on sSCell for CCS Cross-carrier scheduling from sSCell to PCell/PSCell with CIF - sSCell USS set(s) (for CCS from sSCell to PCell/PSCell) and search space sets on PCell/PSCell can be configured so that the UE monitors them in overlapping [slot/symbol] of PCell/PSCell and sSCell - FFS: BD limit handling between [Option A/C] or [Option C] and any configuration of associated parameters and UE reporting of any associated parameters - FFS: #unicast DCI limits for PCell/PSCell scheduling - FFS: PDCCH overbooking on sSCell USS set(s) is not allowed - FFS: Same numerology or different numerologies between sSCell and P(S)Cell - FFS: USS set(s) for DCI format 0_1,1_1,0_2,1_2 (if supported) configured on sSCell for CCS from sSCell to PCell/PSCell - FFS: Support of sSCell deactivation/activation when sSCell cross carrier scheduling to PCell/PSCell is configured - FFS: Support of sSCell dormancy when sSCell cross carrier scheduling to PCell/PSCell is configured - FFS: PDCCH monitoring occasion(s) is within the first 3 OFDM symbols of a PCell/PSCell slot - FFS: Numbers of CORESET configurations and search space sets on sSCell (for PCell/PSCell cross-carrier scheduling) - FFS: frame boundary alignment between PCell/PSCell and sSCell - FFS: Precoder granularity of CORESET size when CCS from sSCell to PCell/PSCell is configured - FFS: Whether the PCell/PSCell SCS other than 15kHz can be allowed Note: The SCell configured with Cross-carrier scheduling to PCell/PSCell is referred to as ‘sSCell’ | FFS 6-5[, 6-10, 18-5, 18-5b, 34-1] | Yes |  |  | Per BC FFS | No | N Applicable to FR1 only FFS |  | sSCell can only be configured in FR1 FFS: UE should be able to indicate following two as part of this per-BC capability: (1) subcarrier spacing for sSCell and/or subcarrier spacing for Pcell, (2) frequency band pair(s) for {PCell/PSCell, sSCell} | Optional with capability signalling |
| 34. NR_DSS | 34-21 | Cross-carrier scheduling from SCell to PCell/PSCell [with search space restrictions] (Type A) | Support of Cross-carrier scheduling from sSCell to PCell/PSCell [with search space restrictions] (Type A) - CIF configured on sSCell for CCS Cross-carrier scheduling from sSCell to PCell/PSCell with CIF - FFS: sSCell USS set(s) (for CCS from sSCell to PCell/PSCell) and at least following search space sets on PCell/PSCell can only be configured such that UE does not monitor them in same [slot/symbol] of PCell/PSCell and sSCell - USS sets for DCI formats 0_1,1_1,0_2,1_2 (if supported) - USS sets for DCI formats 0_0,1_0 - Type3-CSS set(s) for DCI formats 1_0/0_0 with C-RNTI/CS-RNTI/MCS-C-RNTI - FFS: BD limit handling and any configuration of associated parameters and UE reporting of any associated parameters - FFS: #unicast DCI limits for PCell/PSCell scheduling - FFS: PDCCH overbooking on sSCell USS set(s) is not allowed - FFS: Same numerology or different numerologies between sSCell and P(S)Cell - FFS: USS set(s) for DCI format 0_1,1_1,0_2,1_2 (if supported) configured on sSCell for CCS from sSCell to Pcell/PSCell - FFS: sSCell USS set(s) (for CCS from sSCell to Pcell/PSCell) and Type0/0A/1/2 CSS sets on Pcell/PSCell can be configured so that the UE monitors them in overlapping [slot/symbol] of Pcell/PSCell and sSCell. FFS overlap handling - FFS: Support of monitoring DCI formats 0_1,1_1,0_2,1_2 on PCell/PSCell USS set(s) - FFS: Support of sSCell deactivation/activation when sSCell cross carrier scheduling to PCell/PSCell is configured - FFS: Support of sSCell dormancy when sSCell cross carrier scheduling to PCell/PSCell is configured - FFS: PDCCH monitoring occasion(s) is within the first 3 OFDM symbols of a PCell/PSCell slot - FFS: Numbers of CORESET configurations and search space sets on sSCell (for PCell/PSCell cross-carrier scheduling) - FFS: frame boundary alignment between PCell/PSCell and sSCell - FFS: Precoder granularity of CORESET size when CCS from sSCell to PCell/PSCell is configured - FFS: Whether the PCell/PSCell SCS other than 15kHz can be allowed | FFS 6-5[, 6-10, 18-5, 18-5b] | Yes |  |  | Per BC FFS | No | N Applicable to FR1 only FFS |  | sSCell can only be configured in FR1 FFS: UE should be able to indicate following two as part of this per-BC capability: (1) subcarrier spacing for sSCell and/or subcarrier spacing for Pcell, (2) frequency band pair(s) for {PCell/PSCell, sSCell} | Optional with capability signalling |

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between Ues (Sidelink WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 35. LTE_NR_DC_enh2 | 35-1 | [TRS\Temporary] RS [based/for] Scell activation | [Scell activation with the assistance of temporary RS/TRS] - [TRS\Temporary RS] is aperiodic and triggered by MAC CE - FFS: Temporary RS is based on aperiodic TRS and FFS other signals - FFS: Temporary RS is only configured within the BWP indicated by firstActiveDownlinkBWP-Id for the sSCell - FFS: A SSB of the to-be-activated Scell can be indicated as a QCL source for the temporary RS in case of known Scell - FFS: Maximum number of temporary RS resource sets that can be configured to UE per CC {1 … 16} - FFS: Maximum number of temporary RS resource sets that can be configured to UE across CCs {1 … 256} - FFS: Maximum number of triggering states for temporary RS based Scell activation by a MAC-CE {1 … 64} - FFS: Maximum number of temporary RS resource sets that can be associated with a triggering state {1 … 16} [Note: following are reported via the legacy feature, FG2-33 - Maximum number of configured NZP-CSI-RS resources per CC - Maximum total number of simultaneous NZP-CSI-RS resources in active BWPs across all CCs - Maximum number simultaneous NZP-CSI-RS resources per CC - Maximum total number of CSI-RS ports in simultaneous NZP-CSI-RS resources in active BWPs across all CCs] | 6-5 | Yes | N/A |  | [Per UE[/Per BC/per band] | [No/Yes] | [No/Yes] |  |  | Optional with capability signalling |