3GPP TSG-RAN WG1 Meeting #107-e              	R1-2111186

e-Meeting, November 11th – 19th, 2021

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DRAFT CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.213 | CR | DRAFT | rev | - | Current version: | 16.7.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Draft CR on Intra-UE Multiplexing/Prioritization and UL Skipping |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: |  |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_L1enh_URLLC-Core |  |  |  |  |  | Date: |  |  | 2021-11-05 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | In Rel-16, there are several open issues that RAN1 and RAN2 do not have common understanding how MAC and PHY procedure work consistently, when lch-basedPrioritization is configured, and Rel-16 UL skipping is enabled. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - When lch-basedPrioritization is configured, Rel-16 UL skipping is disabled in Rel-16 for both DG and CG. - Reflect the behaviour described in the LS R2-2008599: in the collision scenario between CG and DG with same/different PHY-priority index, and only one transport block is delivered to PHY, PHY transmit on the grant for which a transport block is delivered and skip the transmission on the other grant. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The MAC and PHY procedures are unclear if lch-basedPrioritization is configured, and Rel-16 UL skipping is enabled. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

9	UE procedure for reporting control information

<Unchanged parts are omitted>

If the MAC entity is configured with lch-basedPrioritization, the MAC entity is not expected to be configured with enhancedSkipUplinkTxDynamic with value true, and the MAC entity is not expected to be configured with

enhancedSkipUplinkTxConfigured with value true.

If a transmission of a PUSCH scheduled by a DCI format would overlap in time with a configured grant PUSCH, and only one transport block is delivered as described in [10, TS38.321], the UE shall apply the procedures described in this clause only to the PUSCH for which a transport block is delivered. The PUSCH for which a transport block is not delivered is ignored.

When a UE determines overlapping for PUCCH and/or PUSCH transmissions of different priority indexes other than PUCCH transmissions with SL HARQ-ACK reports before considering limitations for UE transmission as described in clause 11.1, including repetitions if any, the UE first resolves the overlapping for PUCCH and/or PUSCH transmissions of smaller priority index as described in clauses 9.2.5 and 9.2.6. Then,

<Unchanged parts are omitted>