3GPP TSG RAN WG1 #107-e	R1-2111200

e-Meeting, November 11th – 19th, 2021

Agenda item:		8.2.6

Source:	Nokia, Nokia Shanghai Bell

Title:	Channel access mechanism

Document for:		Discussion and Decision

# Introduction

The revised work item on supporting NR from 52.6 GHz to 71 GHz [1] was approved at RAN#92-e. Before that, 3GPP  carried out a study on required changes to NR using existing DL/UL waveform to support operation between 52.6 GHz and 71GHz, reported in [2]. This contribution deals with the following objective of the WID:

- Channel access mechanism assuming beam based operation in order to comply with the regulatory requirements applicable to unlicensed spectrum for frequencies between 52.6GHz and 71GHz.

  - Specify both LBT and No-LBT related procedures, and for No-LBT case no additional sensing mechanism is specified.

  - Study, and if needed specify, omni-directional LBT, directional LBT and receiver assistance in channel access

  - Study, and if needed specify, energy detection threshold enhancement

We consider first the LBT channel access mechanism and related procedures, focusing on the transmitter LBT procedure in Section 2, LBT beamforming in Section 3, channel access within COT in Section 4, receiver assistance in Section 5. Then we discuss the no-LBT channel access in Section 6.

# Transmitter LBT procedure design

In this section, we consider LBT channel access mode and address basic design aspects for LBT procedure at the transmitter. We consider first in Section 2.1 the LBT procedure used to initiate the COT, that is, Cat-3 or Cat-4 LBT procedure. We discuss Cat-2 LBT in Section 2.2, energy detection threshold in Section 2.3, LBT bandwidth in Section 2.3, and multi-channel LBT procedure in Section 2.4. We also consider short control signalling and SSB related channel access in Sections 2.5 and 2.6, respectively.

## LBT procedure initiating COT

In TR38.808 [2], it is stated on the LBT procedure:

Use the CCA check procedure in EN 302 567 as the baseline for channel access for 60GHz band when LBT is applied. The following can be discussed further during normative work:

-	whether CAPC and contention window adjustment mechanisms are introduced,

-	whether contention window range needs to be adjusted.

EN 302 567 [3] presents only a minor restriction to the LBT contention window size: the contention window for random back-off must be at least 3. In TR36.889 [4], channel access schemes were categorized into 4 categories. From those categories, both Cat-3 LBT: LBT with random back-off with a contention window of fixed size and Cat-4 LBT: LBT with random back-off with a contention window of variable size can meet the EN 302 567 LBT requirements. It was concluded in RAN1#106bis-e [16] that there is no consensus to introduce CWS Adjustment for unlicensed operation in FR2-2, meaning that Cat-3 LBT is used for initiating COT.

The actual size of the random back-off contention window is one of the open items that is essential to agree. As said, EN 302 567 [3] sets a limit that the contention window for random back-off must be at least 3. On the contention window size, it can be noted that a longer contention window is needed when there is a considerable number of nodes contending for channel access, and there is a risk that multiple nodes would occupy the channel simultaneously and interfere each other's transmissions. However, interference and channel access contention are expected to be less severe on 60 GHz than on 5 GHz unlicensed band due to more directive transmissions and stronger signal attenuation. The presented coexistence results do not indicate that high contention would be faced frequently. Relatively short contention window limits also the LBT overhead. A contention window size of 4 would allow to perform LBT on a vacant channel in 28 us or less, meaning that LBT could be performed in 1 slot with 480 kHz SCS, in 2 slots in case of 960 kHz SCS, or in the 4-symbol gap between consecutive SSB pairs in case of 120 kHz SCS. Hence, we propose a short contention window size of e.g. 4.

Proposal 1: A short contention window size of [4] is used in the LBT procedure

## On Cat-2 LBT procedure

During RAN1#106-e [9], it was agreed for COT sharing that the responding device may need to perform Cat-2 LBT if the preceding transmission gap exceeds a maximum value Y. It is left for gNB to decide, e.g., based on local regulations, whether the Cat-2 LBT needs to be performed by the responding device in such situation. Further, Cat-2 LBT is agreed to be a UE capability.

It was also noted in RAN1#106-e that other potential use cases for Cat-2 LBT will be discussed separately. The other potential uses cases include:

- Initiating device resuming transmission after a gap

- Prior to switching to a new transmission beam or resuming transmission on a previously used beam.

- Multi-channel LBT

- Channel sensing at a receiving device for providing Rx-assistance

We discuss these use cases in more detail in Sections 3, 4, and 5. It can be noted that each use case relates to either UE or gNB but not both: UE is not expected to resume transmission after a gap as an initiating device but instead it is acting as responding device sharing a COT or providing Rx-assistance for gNB. On the other hand, gNB may be switching it’s transmission beams several times within a COT.

Regarding the gNB side, the results in Figures 5 and 6 do not show any tangible performance improvement from gNB performing Cat-2 LBT when switching the transmission beam as discussed in Section 3.4. With the lack of clear performance improvement, we are not convinced that Cat-2 LBT procedure should be introduced for beam switching.

CAT-2 LBT based channel access does not fulfil the requirements of EN 302 567, limiting its possible use to outside Europe. Hence, multi-channel LBT using Cat-2 LBT is unnecessary as a different multi-channel LBT mechanism is anyway needed for fulfilling EN 302 567 requirements in Europe, as discussed in Section 2.5

Cat-2 LBT has been proposed also for initiating device resuming transmission after a gap exceeding a maximum value. However, no tangible performance improvements from such LBT is shown. Further, when considering long transmission gaps, gNB may also initiate a new COT with Cat-3 LBT. It should be noted that the difference in minimum LBT duration is reduced between Cat-2 LBT and Cat-3 LBT with short contention window size, while Cat-3 LBT facilitates more flexible channel access when there is contention for channel. As long as the Cat-3 LBT design remains open, it is not possible to assess whether Cat-2 LBT could provide any benefits in such situations.

Therefore, we observe and propose:

Observation 1: For initiating device resuming transmission after a long transmission gap, Cat-2 LBT performance cannot be compared against Cat-3 LBT as long as Cat-3 LBT design remains open.

Proposal 2: Do not support Cat-2 LBT in beam switching or in multi-channel LBT.

## Energy Detection

In RAN1 #104-e meeting [13], energy detection threshold was discussed with the following agreement.

Agreement: The baseline ED threshold can be computed as EDT=-80 dBm+10*log10PmaxPout+10*log10(Operating Channel BW in MHz) Where Pout is RF output power (EIRP) and Pmax is the RF output power limit, Pout≤Pmax. - FFS: Further adjustment on ED threshold based on the sensing beam and the transmission beam (further adjustment should not violate EDT requirements as per regulations) - FFS: If Pout is max output EIRP of the device or instantaneous output EIRP - FFS definition of Operating Channel BW - FFS: Whether ED threshold for NR-U and NR-U coexistence scenarios (eg, at regulation level) can be appropriately relaxed compared with the threshold of coexistence between NR-U and Wi-Fi. - FFS: EDT when the COT has time varying transmission beams and varying EIRP

In RAN1#104bis-e [14], a related working assumption was made:

Working assumption: For Pout in EDT determination, define Pout as the maximum EIRP of the node determining EDT during a COT.

It can be noted that the working assumption leads to unnecessarily large EDT and potentially conservative channel access, as on EN 302 567 Pout is defined as RF output power, which is further defined as “The RF output power is the mean equivalent isotropically radiated power (EIRP) for the equipment during a transmission burst.” However, to progress on this matter, we support confirming the working assumption with the clarifications discussed during RAN1#106bis-e [12].

Proposal 3: For Pout in EDT determination, define Pout as the maximum EIRP of the intended transmissions by the node determining EDT during a COT.

One of the controversial open items on EDT is the potential EDT adjustment based on the sensing beam and transmission beam. As essential step for the discussion, it was seen necessary to clarify on which point of receiver the energy in channel is measured. Two views were identified [12]:

- Energy is measured after antenna and antenna gain is included in the energy measurement.

- Energy in front of the antenna assembly is estimated and does not include antenna gain. For the energy estimate, the energy is measured after antenna and estimated antenna gain is reduced from the energy measured.

ETSI EN 302 567 [3] specifies the energy detection by “The Operating Channel shall be considered occupied for a slot time of 5 μs if the energy level in the channel exceeds the threshold corresponding to the power level given in step 7) below.” where step 7 provides formula for EDT determination. EN 302 567 discusses about energy and corresponding power, without any indication that antenna gain should be removed from measured energy. Further, it is hard to considered the energy as energy in front of the antenna assembly as antenna or at least some assumptions on antenna properties are needed to convert received radiation to energy or power. Hence we see that the energy is measured after antenna and as no specific antenna properties are specified, antenna gain is included in the energy measurement.

Observation 2: In LBT, energy is measured after antenna and antenna gain is included in the energy measurement.

When comparing directive and omni-directional sensing beam for LBT with the energy measured after antenna, it is clear that interference in intended transmission direction results in higher measured energy with directive sensing beam that with omni-directional beam. The difference is frequently significant and sensing with directive beam would determine channel occupied more easily than with omni-directional beam. Directive sensing beam is also less sensitive for interference outside the beam. This has motivated proposals that the sensing beam gain should impact either EDT or energy measurement.

It should be noted that EN 302 567 considers beamforming and smart antenna systems in several places, e.g. in the description of test method for LBT. However, antenna gain is not incorporated into the determination of EDT. It is hazardous to assume that this is unintentional and risk compliance with EN 302 567 by increasing EDT with increasing sensing beam gain. Decreasing EDT with increasing sensing beam gain on other hand is not sensible. Hence, we see that EDT should not depend on sensing beam gain.

EDT is already a function of transmitted EIRP. When EIRP, and range of resulting interference, increases, the EDT decreases making LBT more sensitive for channel occupancy. However, the increase in EIRP does not differentiate between increase in Tx power or in beamforming gain, although EIRP increase by beamforming can be expected to cause less interference to other systems than an increase by Tx power. Further, any difference on the sensing beam and transmission beam is not reflected on the EDT determination.

However, the potential EDT adjustment shall not result in EDT that would violate EN 302 567 requirement. That means that any modification e.g. based on transmit beam gain could only reduce the current EDT and penalize the affected equipment in the channel access. For example, we would further restrict channel access for NR equipment with insufficient transmit beamforming gain. This raises difficult questions like how to determine insufficient beamforming gain – what beamforming gain is required so that equipment is not penalized for that? Hence, specifying EDT dependence on beamforming is far from a trivial task. It should be also noted that estimating beamforming gain is also a challenging task for implementation. Given the tight schedule of WI and considerable required standardization and implementation efforts, we propose that EDT is not adjusted based on the transmit or sensing beam gain.

Proposal 4: Further adjustment of EDT based on the transmit or sensing beam gain is not specified.

During RAN1#106bis-e, it was agreed that the exact location of energy measurement during the 5 µs observation slot is left for implemention. One item related to energy measurement remaining open is the minimum duration for the energy measurement. Determination of minimum measurement duration has been proposed as well as leaving it for implementation to decide. Main concern against leaving it for implementation has been potentially inaccurate measurements and intentional biasing of measurements. However, minimum measurement duration cannot be easily verified and does not alone ensure sufficiently accurate measurement. In Europe, sufficient measurement accuracy is verified via ETSI EN 302 567 adaptivity test, but such tests are not required in all regions. To address the concern on the measurement accuracy, RAN4 may consider defining a requirement on measurement or channel access accuracy, while the minimum measurement duration is left for implementation.

Proposal 5: Minimum duration for energy measurement during the 5 µs observation slot is not defined. LS is sent to RAN4 to consider defining a requirement on measurement or channel access accuracy.

## Multi-channel LBT

In RAN1 #104-e meeting [13], the following agreement has been reached about multi-channel LBT:

Agreement: Define Type A and Type B multi-channel channel access as: - Type A: Perform independent eCCA for each channel - Type B: Identify a primary channel and perform eCCA on the primary channel, while perform Cat 2 LBT for other channels in the last observation slot Down-selection between - Alt1: Support Type A multi-channel channel access only - Alt2: Support both Type A and Type B multi-channel channel access. Note: How eCCA is performed on each channel, and the BW of the channels over which eCCAs are performed are separately discussed

Furthermore, the following conclusion has been reached on CWS adjustment at RAN1 #106bis-e [16]:

Conclusion: There is no consensus to introduce CWS Adjustment for unlicensed operation in FR2-2.

For operation in the 52.6-71 GHz band, it is beneficial to support multi-channel LBT to facilitate wideband operation as already supported in Rel-15/16. Therefore, NR-U at 60 GHz should support multiple-channel access operation at least for the case of wideband carrier, e.g., in the range of 400 MHz to 2.16 GHz.

ETSI EN 302 567 specifies support for the Type A channel access operation on 60GHz unlicensed band. This kind of operation was also used for Rel-16 NR-U. Hence, the Type A channel access operation should definitely be supported for multi-channel operation.

Type B operation corresponds to the channel bonding case for multiple-channel access as defined in 5 GHz specifications. However, ETSI regulation in EN 302 567 doesn’t specify the channel bonding-based multiple-channel access on 60GHz unlicensed band. Furthermore, Type B channel access further relies on an assumption on pre-determined or fixed channelization. For example, in NR-U on 5GHz spectrum, a fixed channelization and LBT BW of 20 MHz are defined for channel bonding based multi-channel access. However, on 60GHz, there is no fixed channelization or nominal channel BW defined in ETSI BRAN regulation where different channel bandwidths are supported. Support of Type B channel access would impose unnecessary restrictions on the design of supported channel bandwidths for NR-U on 60GHz unlicensed band and cause extra specification efforts in 3GPP RAN1 as well as RAN4.

At the current state, the benefits from Type B channel access are also unclear. As only one eCCA is required on the primary channel, Type B channel access might in principle speed up the multi-channel LBT procedure in certain cases (e.g. in low traffic environments). However,  fixed and relatively small CWSs will be adapted for eCCA according to the discussion at RAN1#106bis-e [16], the advantages of Type B channel access will become insignificant in comparison with Type A channel access. Furthermore, Type B channel access decisions mainly rely on the outcome of eCCA on the primary channel. Once the eCCA check is blocked on the primary channel, the transmitter can not access any of the channels for transmission. In contrast, Type A channel access performs independent eCCA on each channel, which enables more flexible channel usage.

Based on the considerations above, we have the following proposal:

Proposal 6: Only Type A multi-channel access procedure (i.e. Alt.1 defined in RAN1#104-e meeting) shall be supported in NR-U on 60GHz band.

## Short Control Signalling

Short Control Signals (SCS) were discussed during the Study Item on supporting NR from 52.6 GHz to 71 GHz, as captured into the TR 38.808:

Support contention-exempt short control signalling transmission in 60GHz band for regions where LBT is required 	and short control signaling without LBT is allowed. It should be noted that if regulations do not allow short control 	signaling exemption in a region when operating with LBT, operation with LBT for these short control signals should 	be supported. Restrictions to the transmission, such as, on duty cycle (airtime measured over a relatively long period 	of time), content, TX power, etc. can be discussed when specifications are developed.

As for the definition of Short Control Signals, there has been a recent effort to introduce their support into EN 302 567. In the ETSI BRAN 108 meeting, text as copied below was introduced to the latest draft EN 302 567 v2.2.0 [3]. Short control signalling transmission, as defined by ETSI, are control and management transmission, that are not required to undergo LBT procedure, but can instead be transmitted without channel sensing, as long as the total duration of SCS transmissions over a 100-ms observation interval does not exceed 10%, as the excerpt from EN 302 567 v2.2.0 below shows.

### 4.2.6 Short Control Signalling Transmissions #### 4.2.6.1 Applicability The present requirement applies to all equipment within the scope of the present document. #### 4.2.6.2 Definition Short Control Signalling Transmissions are transmissions used by the equipment to send management and control frames without sensing the channel for the presence of other signals. #### 4.2.6.3 Limits The use of Short Control Signalling Transmissions is constrained as follows: - within an observation period of 100 ms, - the total duration of the equipment's Short Control Signalling Transmissions shall be less than 10 ms within said observation period. #### 4.2.6.4 Conformance The conformance tests as defined in clause 5.3.8 shall be carried out.

Following observation can be made based on the EN 302 567 :

- SCS allowance of 10% within a 100 ms observation period can be used for various (unspecified) types of control and management transmissions

- SCS transmissions do not need to be periodic, as long as the 10 % allowance within 100 ms period is not exceeded

- multiple SCS transmissions are allowed within the 100 ms observation interval, as long as the 10% limit is not exceeded

- SCS can be transmitted by both gNB, as well as UE(s)

The SCS definition in EN 302 567 is written from a single device (UE or gNB) point of view, i.e. in principle there could be multiple devices operating in a cell, each transmitting control transmissions without LBT for up to 10 % of the time.

Observation 3: EN 302 567, v2.2.0 allows for Short Control Signalling transmissions for up to 10% of time within an observation period of 100 ms.

In the context of NR operation on 60 GHz unlicensed spectrum, there are a few clear candidates for signals and channels that can be considered as short control signalling. At RAN1#104bis-e [14] and RAN1#105-e [15], it was agreed to support short control signalling for transmission of SS/PBCH and RACH Msg1 and MsgA, while applicability of on other signals and channels was left open. Candidate signals include at least:

Downlink:

- SS/PBCH blocks (already agreed)

- PDCCH

- CSI-RS and other reference signals, e.g., for beam management

- SIBs

- Paging

Uplink:

- HARQ-ACK feedback on either PUCCH or PUSCH

- Scheduling Request

- CSI feedback

- Sounding RS, e.g., for beam management

- RACH related transmissions (Msg1 and MsgA already agreed)

Transmitting selected signals/channels as SCS transmissions is highly attractive as deterministic transmission time of the signals critical control and managements transmissions can be maintained, when the channel access uncertainty is removed. This can simplify the design a lot, as the procedures mitigating the impact of blocked channel access for control signals are not needed. Additionally, even for PUSCH data, short control signaling framework could facilitate transmission of SR or a similar indication, when actual PUSCH transmission is not possible due to LBT failure. Furthermore, there is no need to modify e.g. signal design to facilitate the time gap for LBT measurements, making TDM easier. Therefore, we see that NR design for 60 GHz bands should make most of the short control signaling allowance, such that unnecessary modifications to L1 design can be minimized.

Proposal 7: NR-U design for 60 GHz bands supports transmission of the following DL and UL control and management signals as short control signalling without LBT:

- Downlink: SS/PBCH blocks (already agreed), PDCCH, CSI-RS and other reference signals, e.g., for beam management, SIBs, Paging

- Uplink: HARQ-ACK feedback on either PUCCH or PUSCH, Scheduling Request, CSI feedback, Sounding RS, e.g., for beam management, RACH related transmissions

Even if it is not possible to transmit all the signals listed above within the 10 % allowance, it is still beneficial to use short control signaling as often as possible, at least part of transmission instances. Furthermore, especially for the UL transmissions, one needs to consider means for gNB to control that the 10% maximum limit for SCS transmissions of 100 ms observation period is not exceeded. Moreover, it should be discussed if the maximum amount of SCS transmissions in a cell should be limited somehow, in cases where gNB and multiple UEs are all transmitting significant proportion of their transmissions without LBT. Related to this point, at RAN1#105-e [15], in the context of RACH the following was agreed:

Agreement: - Contention Exempt Short Control Signaling rules apply to the transmission of msg1 for the 4 step RACH and MsgA for the 2-step RACH for all supported SCS. - Note restriction for short control signalling transmissions apply (10% over any 100ms intervals) - Alt 1: The 10% over any 100ms interval restriction is applicable to all available msg1/msgA resources configured (not limited to the resources actually used) in a cell - Alt 2: The 10% over any 100ms interval restriction is applicable to the msg1/msgA transmission from one UE perspective - FFS: Other UL signals/channels can be transmitted with Contention Exempt Short Control Signaling rule, such as msg3, SRS, PUCCH, PUSCH without user plain data, etc

In our view, it is reasonable to apply the 10% limit for short control signaling per link direction in a cell, such that all UEs in a cell shall share the same 10% short control signaling allowance. The network should have sufficient means (i.e. signaling) for controlling that the UEs will not exceed this limit at any point in time. For RACH related transmissions this corresponds to Alt 1 above. The same rule can also be generalized to cover other UL signals and channels.

Proposal 8: For the UL transmissions, the 10% short control signalling allowance is shared by all the UEs in the cell.

## LBT for SSB

In RAN1 #104-e meeting [13], channel access for transmission of SSBs was discussed with the following agreement.

Agreement: - SSB transmission with LBT is supported, at least when the conditions for contention exempt short control signalling based SSB transmission is not met - Note the channel access for SSB with LBT may not be different from a normal COT with multiple beams - FFS: If any difference from a multi-beam COT LBT needs to be introduced

When gNB transmits SSBs as part of normal COT, there is no need for any specific channel access considerations. When gNB transmits SSBs alone, that is, not as part of a normal COT, we can recognize three channel access cases:

- SSBs are transmitted as short control signals without LBT. We cover this case as part of Section 2.6.

- Only part of SSBs can be transmitted as short control signals without LBT while channel access for the remaining part of SSBs is acquired with LBT. This case can be faced in Europe when gNB is compliant with EN 302 567 and overall transmission time for SSBs exceeds 10%. This depends on the SSB sub-carrier spacing, applied number of SSBs and the period. For example, assuming all 64 SSBs, SSB period would need to be at least 40ms, 20 ms or 10 ms, for 120 kHz, 240 kHz, and 480 kHz SCS respectively so that all SSBs could be transmitted under short control signalling exemption.

- Short control signalling contention exemption cannot be used at all and channel access for all SSBs is acquired with LBT. This case can be faced in Japan.

The SSB symbol locations for 120 kHz SCS were agreed to follow the baseline of existing FR2 pattern, and SSB symbol locations for 480 kHz and 960 kHz SCS were agreed as follows:

Agreement: For SSB with 120kHz SCS for NR 52.6 GHz to 71 GHz, - 120 kHz SCS: the first symbols of the candidate SS/PBCH blocks have indexes {4, 8,16, 20} + 28×n, where index 0 corresponds to the first symbol of the first slot in a half-frame. - For carrier frequencies within 52.6 GHz to 71GHz, support at least 𝑛 = 0, 1, 2, 3, 5, 6, 7, 8, 10, 11, 12, 13, 15, 16, 17, 18. - Other values of n (if any) are FFS, and support of additional n values are subject to support of DBTW for 120kHz SSB Agreement: For 480kHz and 960kHz sub-carrier spacing, first symbols of the candidate SSB have index {2, 9} + 14*n, where index 0 corresponds to the first symbol of the first slot in a half-frame.

The symbol locations are presented in Figure 1. When gNB is transmitting SSBs alone, Cat-3 LBT could be performed prior to the half-frame burst of SSBs or “sub-sweep block of SSBs”. In the case of 480 kHz or 960 kHz SCS, the number of slots between these SSB bursts could range from 2 to 4 slots, although the design is still open. This is enough to perform Cat-3 LBT with reasonably sized contention window.

Figure 1. SSB candidate symbol locations.

In case of 120 kHz SCS, there is also possibility to perform Cat-3 LBT within the 4 symbols prior the block of two SSBs This would provide additional flexibility in time when LBT can be performed. However, rather short contention window of e.g. 4 observation slots would be required. This would mean a LBT gap of at least 8 us + 4*5us = 28 us, while 4 symbols with 120 kHz SCS has duration of 35.7 us. Hence, in the case of 120 kHz SCS, LBT can be performed during the 4 empty symbols preceding the SSBs, but it is clear that there is space for much longer contention window.

Observation 4: Short contention window of [4] observation slots facilitates flexible LBT timing for SSB transmissions.

It has been proposed that gNB performs Cat-2 LBT at the beam switching. It can be seen from Figure 1 that for 120 kHz SCS, two SSBs are located back-to-back without any gap between them. For 480 kHz and 960 kHz SCS, there is a gap of 3 symbols between the SSBs, which is insufficient gap for Cat-2 LBT requiring 8 us. Hence, with the agreed SSB symbol locations, it is not possible to require Cat-2 LBT at the gNB beam switching for consecutive SSBs. Introduction of LBT gap would require a re-design of SSB symbol locations, which would mean a considerable additional specification effort and need to revert the earlier agreements.

As said, there are SSB configurations that exceed the 10% transmission time allowance for short control signalling. For example, overall transmission time for SSBs would be 11.4% with periods of 20ms, 10 ms, and 5 ms for 120 kHz, 240 kHz, and 480 kHz SCS respectively. Alternatively, 56 SSBs from the 64 SSBs could be transmitted under short control signalling exemption (SCSe).

Observation 5: Depending on SSB sub-carrier spacings and SSB periodicity, only sub-set of total SSBs can be covered by short control signalling exemption.

When only a part of SSBs can be transmitted as short control signals, different approaches can be considered to handle this. Firstly, if all SSBs cannot be covered by SCSe, the SSBs that do not fall under the SCSe would need to follow the LBT procedure to initiate COT. It is not necessary or sensible to mandate that all SSBs need to fall under SCSe to be able to apply SCSe at all.

Proposal 9: It is possible to apply SCSe to one part of actually transmitted SSBs and LBT procedure for other/rest of the SSBs.

Short control signalling is not required to have any fixed periodicity but only that the 10ms transmission time is not exceeded in any 100ms window. When transmission of a portion of SSBs is conditioned with LBT, it makes sense that the same SSBs are not constantly conditioned with LBT. Instead, use of SCS contention exemption and use of LBT could be cycled over the SSBs to evenly distribute the channel access uncertainty over the SSBs. There are different ways to achieve this; in a fully flexible manner where gNB could choose dynamically in every SSB burst/discovery window that which SSBs are sent based on SCS contention exemption and which are sent (or not) based on LBT, or in a more static or predetermined manner where choise for SSB ‘grouping’ to {SCS or LBT} is done in advance e.g. in a periodic manner. In latter case the expected pattern could be informed to the UE, while in fully flexible case UE would need to assume both options for each SSB.

Proposal 10: Use of short control signal contention exemption and use of LBT for SSBs is predetermined or semi-statically determined, distributing the channel access uncertainty over the SSBs.

# Beamforming for LBT

## Relation between LBT sensing beam and transmission beam

Regarding LBT operation in 60 GHz unlicensed band, one of the major issues to clarify is the beamforming for LBT or, particularly, LBT beam relation to Tx beam. Here, setting the LBT beam equal to the gNB transmission beam may appear at first sight as a straightforward solution especially when serving single UE within a COT. However, serving single UE per COT can be inefficient due to UE and gNB processing times and increased LBT overhead and can increase latencies for other UEs. When directional LBT sensing is performed for multiple UEs simultaneously or consecutively at beginning of COT (i.e., using a wider LBT beam or more complex LBT beamforming procedures), transmissions in different beams can be multiplexed in the same COT as agreed in RAN1#103e [11]. This highlights that preferred LBT sensing beam may depend on the multiplexing strategy used by gNB.

Further, the exact beamforming operations are subject to the details of the transceiver and antenna array system utilized at the gNB that may vary a lot between different vendors. The LBT beamforming operations may also vary from a COT to COT depending on the UEs scheduled to be served during the COT. Moreover, the whole NR beam based operation at gNB is defined so that the design of exact beam patterns is left for implementation. It is hard to see that those should be now specified for the sake of LBT. RAN1 has discussed this issue and identified the following alternatives in RAN1#106e:

Agreement: 3GPP specification consider defining at least the relative relationship between all applicable sensing beam(s) and the transmission beam(s) to define sensing beam for LBT, where at least sensing beam(s) “covers” the transmission beam(s), considering following alternatives. Target down-selection by RAN1 #106bis-e - Alt 1: Specify necessary requirement/test procedure to guarantee sensing beam “covers” the transmission beam - Some methods to define “cover” have been discussed in RAN1 (may further down select the list) and are considered as acceptable from RAN1 perspective - Alt-1A: the angle included in the [3] dB beamwidth of the transmission beam is included in the [X, FFS] dB beamwidth of the sensing beam. - Alt-1B: the sensing beam gain measured along the direction of peak transmission direction is at least X [FFS] dB of the transmission beam gain - Alt-1C: The sensing beam gain is measured in one or more directions where the transmission beam EIRP is within A [FFS] dB of the peak EIRP. The sensing beam gain measured along the chosen directions is at least X [FFS] dB of the transmission beam gain in those directions. - Alt-1D: The sensing beam gain is measured in one or more directions where the transmission beam EIRP is within A [FFS] dB of the peak EIRP and the sensing beam gain measured along the chosen directions is at least X [FFS] dB of the peak sensing beam gain - Alt-1E: Sensing beam has the minimum [3] dB beamwidth which at least contains all beam peak directions of transmission beams. - Sending LS to RAN4 and inform them the above and request them to make the final choice - RAN4 choice may not be limited by the list above, but if different method is selected, RAN1 would like to have an opportunity to check as well - Alt 2. Extending the beam correspondence framework and QCL/TCI/SpatialRelationInfo framework to define “cover” and to indicate sensing beam(s) associated with a transmission beam(s) - On gNB side sensing beam selection for a DL transmission beam, - Option 1: The selection of eligible sensing beam for a transmission beam is left for gNB implementation - No testing or enforcement introduced in 3GPP spec for this option - Option 2: Beam correspondence at gNB side is assumed. Supporting one or more of the following behaviors - A1. For a gNB transmission beam corresponding to TCI state A for a certain UE, the gNB can use the same beam for sensing - A2. If TCI B is used as QCL source (Type D) for TCI A for a certain UE, then gNB transmission beam corresponding to TCI B can be used as the sensing beam for transmission with TCI A. - A3. If TCI C is NOT used as QCL source (Type D) for TCI A for any UE, then gNB cannot use the transmission beam corresponds to TCI C as the sensing beam for transmission with TCI A. - FFS: How and if to support sensing with a beam without corresponding RS sent? For example, how to use quasi-Omni beam for sensing if there is no SSB transmitted with quasi-omni beam - On UE side sensing beam selection for a UL transmission beam - Beam correspondence is assumed at UE - FFS: What if beam correspondence is not supported at UE. - Supporting one or more of the following behaviors - If the UE is indicated to transmit with a beam corresponding to a certain SRI, the UE can use the same beam for sensing - Assuming Rel.17 unified TCI framework, if the UE is indicated to transmit with a beam corresponding to a certain unified TCI, the UE can use the reception beam corresponding to the TCI for sensing - FFS: How and if to support a wider sensing beam (such as pseudo-omni beam, which is supported in WiFi) to be used for a narrower transmission beam under QCL/TCI framework - Option 0: Not supported - Option 1: UE implementation. - No testing or enforcement introduced in 3GPP spec for this option - Option 2: gNB indication. - FFS details. - FFS: How and if to support a multiple sensing beams to be used for a transmission beam under QCL/TCI framework - Note: Supporting both alternatives or a combination of the two alternatives is not precluded

In RAN1 #106-bis-e there was further agreement:

Agreement: - When UE indicates a capability for beam correspondence with beamCorrespondenceWithoutUL-BeamSweeping ={1}, support the following behaviors - If the UE is indicated to transmit with a beam corresponding to a certain SRI, the UE can use the same beam for sensing - Assuming Rel.17 unified TCI framework, if the UE is indicated to transmit with a beam corresponding to a certain unified TCI, the UE can use the reception beam corresponding to the TCI for sensing - FFS: The case when UE does not indicate a capability for beam correspondence - Note: The UE should meet local regulatory requirements

The agreement does not cover gNB, when UE does not indicate capability for beam correspondence without UL beam sweeping or when UE uses different beam for sensing than for transmission, e.g., quasi-omni beam for sensing. When considering these cases, it can be noted that:

- Beam correspondence and QCL/TCI/SpatialRelationInformation/unified TCI framework have been proposed to be used also for gNB. However beam correspondence has been defined only for UEs but not for gNBs. Hence definition and testing of beam correspondence for gNBs would require considerable additional efforts. QCL is used to describe dependencies of downlink reference signals. A proposed behaviour how to utilize QCL for defining association between sensing beams and transmission beams has been shown in Option 2. Again effort is needed to specify such mechanism.

- Another important question still open would be how to specify quasi-omni sensing beams. Specifying quasi-omni sensing beams using QCL framework is not straightforward as there is typically no RS sent on the quasi-omni beam. It is also a bit questionable approach, why use QCL framework if there is no intention to associate known beams? In general there is no need to do the effort and specify such mechanisms, it is enough that the gNBs fullfill the requirements set by the regulations.

- Until now beam correspondence without UL beam sweep has not been mandatory for UE. For unlicensed bands it would be useful to facilitate reasonable UE cost, which the usage of beam correspondence is assumed to increase.

Therefore, it is proposed to leave the relationship between LBT sensing beam and transmission beam mostly up to implementation for these cases, as long as the relevant region and deployment specific requirements (e.g. ETSI EN 302 567) are fulfilled and tests defined by regulations are passed. In FL summary [12], it was proposed that:

Proposal 2.9.2-3 For the following situations - gNB - UE does not indicate a capability for beam correspondence with beamCorrespondenceWithoutUL-BeamSweeping ={1} - UE uses a different beam for sensing than the beam used for transmission, Specify necessary requirement/test procedure to guarantee sensing beam(s) “covers” the transmission beam(s) - Some methods to define “cover” have been discussed in RAN1 (may further down select the list) and are considered as acceptable from RAN1 perspective - Alt-1A: the angle included in the [3] dB beamwidth of the transmission beam is included in the [X, FFS] dB beamwidth of the sensing beam. - Alt-1B: the sensing beam gain measured along the direction of peak transmission direction is at least X [FFS] dB of the transmission beam gain - Alt-1C: The sensing beam gain is measured in one or more directions where the transmission beam EIRP is within A [FFS] dB of the peak EIRP. The sensing beam gain measured along the chosen directions is at least X [FFS] dB of the transmission beam gain in those directions. - Alt-1D: The sensing beam gain is measured in one or more directions where the transmission beam EIRP is within A [FFS] dB of the peak EIRP and the sensing beam gain measured along the chosen directions is at least X [FFS] dB of the peak sensing beam gain - Alt-1E: Sensing beam has the minimum [3] dB beamwidth which at least contains all beam peak directions of transmission beams. - Sending LS to RAN4 and inform them the above and request them to make the final choice - RAN4 choice may not be limited by the list above, but if different method is selected, RAN1 would like to have an opportunity to check as well - RAN4 can further decide for gNB or UE separately if such test or requirement is not needed or not practical and leave it for UE implementation

Generic requirements as proposed in Proposal 2.9.2-3 or in Alt 1 with more detailed alternatives outlined in Alt-1A – Alt-1E can be supported. As done in Proposal 2.9.2-3, it should be left for RAN4 to ultimately decide on how to define such beam relationship, the requirement and corresponding test case. Hence we propose to agree Proposal 2.9.2-3 and send LS to RAN4 so they can do the decision. Also there is no need to discuss and try to reduce number of the alternatives (Alt-1A-Alt-1E) further in RAN1 but the discussion should be continued in RAN4.

Observation 6: Generic requirements for relative relationship between LBT sensing beam(s) and transmission beam(s) should be done in RAN4, not in RAN1.

Proposal 11: Proposal 2.9.2-3 from RAN1#106bis-e FL summary is agreed for defining the relative relationship between applicable LBT sensing beam(s) and the transmission beam(s) for gNB, when UE does not indicate capability for beam correspondence without UL beam sweeping or when UE uses different beam for sensing than for transmission.

## LBT beam at the start of the COT

One of the objectives identified for the WI in RP-202925 is to [1]:

Study, and if needed specify, omni-directional LBT, directional LBT and receiver assistance in channel access

Supporting directional LBT is complicated, the progress of the topic has been slow at least for gNB LBT, and the companies are still having very differing positions. Directional LBT is not strictly essential for channel access operation but access with wide beam covering all Tx beams can be used instead. Considering the situation, it would be beneficial to deprioritize the directional LBT until baseline channel access methodology is agreed.

Proposal 12: COT initiating LBT with multiple independent per-beam LBT sensing should be deprioritised while completing the design for baseline channel access procedures.

In the following, we present our views on directional LBT in case that RAN 1 reaches common view on the relationship between applicable LBT sensing beam(s) and the transmission beam(s) for gNB and directional LBT is discussed in RAN1#107.

In RAN1#104-e [13], it was agreed that:

Agreement: For a COT with MU-MIMO (SDM) transmission, further consider the follow alternatives (down-select or support both) - Alt 1: Single LBT sensing at the start of the COT with wide beam ‘cover’ all beams to be used in the COT with appropriate ED threshold - Alt 2: Independent per-beam LBT sensing at the start of COT is performed for beams used in the COT Agreement: Within a COT with TDM of beams with beam switching, down-select one or more of the following LBT operations - Alt 1: Single LBT sensing with wide beam ‘cover’ all beams to be used in the COT with appropriate ED threshold - FFS: Details on the definition of “cover” - Alt 2: Independent per-beam LBT sensing at the start of COT is performed for beams used in the COT - Alt 3: Independent per-beam LBT sensing at the start of COT is performed for beams used in the COT with additional requirement on Cat 2 LBT before beam switch

As discussed earlier, the exact beamforming operations are subject to the details of the transceiver and antenna array system utilized at the gNB that may vary a lot between different vendors. Hence, we propose to support both Alt 1 and Alt 2, allowing room for different implementations.

Proposal 13: For a COT with MU-MIMO (SDM) or within a COT with TDM of beams with beam switching, support both Alt 1 and Alt 2.

## eCCA with multiple LBT beams

As discussed above, gNB may use Alt. 1 or Alt. 2 when gNB acquires channel access for multiple transmission beams to be served in the same COT. In the case of Alt. 2 and analog beamforming, a question to be addressed is how the CCAs for different beams are carried out in time. Some alternatives illustrated in Figure 2 are:

- gNB carries out separate eCCA check procedure for each beam with independent back-off values. Once the CCA check procedure is completed, gNB enters self-deferral until the CCA check procedure for other beams is completed. The CCA check procedures can be time multiplexed as blocks or interlaced in time, as shown in Figure 2a and 2b, respectively.

- gNB carries out single eCCA check procedure during which each beam is sensed as shown in Figure 2c. It may be determined that each beam is sensed at least N times (gNB may increase drawn back-off value to meet the requirement.).

The point of random back-off is to randomize the channel access times of multiple contending devices after a channel becomes vacant. Multiple beam-specific eCCA procedures would mean in practice that the gNB experiences a longer back-off given by the sum of the individual random back-off values. This would at least increase the LBT overhead and lead to long LBT durations, which may be too long e.g for SSB structure. In the case that gNB would actually experience channel contention, it would also reduce the gNB’s probability to gain channel access on all scheduled beams due to long LBT duration. On other hand, single occupied beam would not prevent gNB to obtain channel access on the other beams. The benefits and drawbacks for a single eCCA procedure over all beams are basically the opposite. The shorter LBT gap is important especially when LBT is performed for block of two SSBs during the gap of 4 symbols. Based on the above, we see that the LBT requirements need to be defined so that different solutions for LBT beamforming and eCCA procedures are equally supported.

Figure 2. CCA check procedures for gNB using 2 LBT beams: separate CCA check procedure for each beam multiplexed in blocks in time (Fig 2a) or interlaced (Fig 2b), single CCA check procedure during which beams are sensed (Fig 2c), and single wide LBT beam covering both transmit beams (Fig 2d).

In RAN1#104bis-e [14], this was considered further and it was agreed for the alternatives in the earlier agreement that:

Agreement: For a COT with MU-MIMO (SDM) transmission, when independent per-beam LBT sensing at the start of COT is performed for beams used in the COT (Alt 2 in earlier agreement) is considered, the following alternatives are further considered - Alt A: The per-beam LBT for different beams is performed in TDM fashion - Alt A-1: The node completes one eCCA on one beam, and directly move on to the eCCA on the other beam, with no transmission in the middle - Alt A-2: The node completes one eCCA on one beam, start transmission with the beam to occupy the COT, then move on to the eCCA on the other beam - Alt A-3: The node performs eCCA of the different beams simultaneous, round robin between different beams - Alt B: The per-beam LBT for different beams is performed simultaneously in parallel, assuming the node has the capability to simultaneously sense in different beams Agreement: Within a COT with TDM of beams with beam switching, when independent per-beam LBT sensing at the start of COT is performed for beams used in the COT (Alt 2 or Alt 3 in earlier agreement) is considered, the following alternatives are further considered - Alt A: The per-beam LBT for different beams is performed one after another in time domain - Alt A-1: The node completes one eCCA on one beam, and directly move on to the eCCA on the other beam, with no transmission in the middle - Alt A-2: The node completes one eCCA on one beam, start transmission with the beam to occupy the COT, then move on to the eCCA on the other beam - Alt A-3: The node performs eCCA of the different beams simultaneous, round robin between different beams - Alt B: The per-beam LBT for different beams is performed simultaneously in parallel, assuming the node has the capability to simultaneously sense in different beams

Related to these agreements what comes to MU-MIMO (SDM) case the gNB should be able to do sensing simultaneously and hence alternative B should be supported.

Proposal 14: For a COT with MU-MIMO (SDM) transmission, support Alt B.

For TDM case the gNB may not have capability for simultaneous processing, meaning that the sensing of beams needs to be arranged in time domain. In general implementation flexibility should be maintained and the options should not be restricted too much, especially if relationship between gNB LBT sensing beam(s) and the transmission beam(s) is left largely for gNB implementation. However, some further restrictions and downselections should be done for the alternatives listed above.

Observation 7: It is important to maintain flexibility of gNB implementation for multi-beam COT.

For alternative A-1 it is not clear what happens on beam(s) for which eCCA is completed and are in self-deferral while waiting eCCA completion on the last beam. We see that the delay between the last channel sensing on the beam and the actually channel occupation may become too large and, hence, would propose that a further CCA check is performed on all self-deferred beams after completing the last eCCA and before starting the COT as shown in Figure 2a. This would ensure that the gap between sensing and channel occupancy does not get too high. Hence, we suggest following modification to Alt A-1

Observation 8: Alt A-1 should be modified as: The node completes one eCCA on one beam, and directly moves on to the eCCA on the other beam, with no transmission in the middle. After completing eCCA on all beams, a further round robin CCA check is carried out in all beams (except the last beam).

Alternative A-2 does not look like a true multi-beam COT but more like combination of shorter single beam COTs and hence does not need to be supported.

In alternative A-3 it should be clarified if a single contention window is shared by beams or there are separate contention windows for each beam. In first case the CCA check procedure would be like in Figure 2c, whereas in latter case like in Figure 2b. Both options should be supported as discussed above. In the case that single CW is shared by the beams, a further restriction is needed that the last CCA checks on all beams indicate a vacant channel. Otherwise it would be possible to start COT on a beam that is measured to be occupied. Hence, we suggest following modification to Alt A-3

Observation 9: Alt A-3 should be modified as: The node performs eCCA of the different beams simultaneous, round robin between different beams.

- single contention window is shared by beams or each beam has a separate contention window.

- the last CCAs shall indicate vacant channel on all beams that are part of the COT

Proposal 15: For a COT with TDM transmission, support the modified Alt A-1 and Alt A-3:

- Alt A-1: The node completes one eCCA on one beam, and directly moves on to the eCCA on the other beam, with no transmission in the middle. After completing eCCA on all beams, a further round robin CCA check is carried out in all beams (except the last beam).

- Alt A-3: The node performs eCCA of the different beams simultaneous, round robin between different beams.

  - single contention window is shared by beams or each beam has a separate contention window.

  - the last CCAs shall indicate vacant channel on all beams that are part of the COT

## Evaluation results for Cat 2 LBT at gNB beam switch

In this section, we consider the requirement of performing Cat 2 LBT before beam switch. Cat 2 LBT before beam switch may in worst case cause considerable overhead. The gNB needs to frequently serve multiple directions in a single COT in a time multiplexed manner due to use of analog or hybrid beamforming. Transmissions on different beams are also heavily interleaved in time. For example, gNB may transmit PDSCH for a first UE, UL grant for a second UE and while the UE is preparing the PUSCH, gNB may transmit a DL assignment and PDSCH for a third UE, receive HARQ-ACK from a the first UE, and then receive PUSCH from the second UE, and receive HARQ-ACK from the third UE, etc, as shown in Figure 3. Cat 2 LBT of 8 us prior to every change of Tx beam, consuming 1-8 symbol durations depending on subcarrier spacing, would induce considerable overhead. Further, Cat 2 LBT at gNB beam switch cannot be done between the two consecutive SSBs with the SSB symbol locations agreed for 120 kHz, 480 kHz and 960 kHz SCS. Requiring Cat-2 LBT at every beam switch would mean that SSB symbol locations should be re-designed to facilitate LBT gap in case that short control signalling exemption is not used for SSBs.

Observation 10: Cat-2 LBT at every gNB beam switch would cause significant increase in overhead and is not even possible between the SSBs in the agreed SSB time locations.

Figure 3. gNB transmissions on 3 beams during a shared COT

In following, we provide simulation results for use of Cat 2 LBT at gNB beam switch within COT as discussed above. Results are for half size office scenario and 50%/50% DL/UL. Simulation parameters are found in Appendix 2. LBT mode alternatives used in simulations are shown in Figure 4. As seen in figure the simulations cover both omni-LBT and directional per-beam LBT with and without additional Cat 2 LBT at gNB beam switch. In simulations, the LBT modes are compared against the case without LBT, i.e. use of beamforming as coexistence method.

Figure 4. Various LBT modes used in simulations.

Median and 5th percentile throughputs are shown in Figure 5 and Figure 6 for DL and UL, respectively. Compared to the results without LBT, the LBT seems to cause a clear loss in most cases. There is gain in DL median throughput due to per-beam LBT in high load, but at price of loss in DL and UL 5 percentile throughput.

Observation 11: Use of LBT provides mostly loss in median throughput compared to no-LBT mode and reduces throughput for cell edge UEs

Results for beam switching within COT both without additional Cat-2 LBT (Alt 1a, Alt2) and with additional Cat-2 LBT (Alt1b, Alt3) are provided for both the omni LBT and per beam LBT. Results show practically no difference in performance with or without Cat-2 LBT and hence no gain from introducing the additional Cat-2 LBT. These results support our discussion related to Cat-2 LBT in Section 2.2.

Observation 12: Simulation results do not show any gain from introduction of additional Cat-2 LBT at gNB beam switch during COT.

Figure 5. FTP model 3 DL throughput median and 5th percentile for low, medium and high load

Figure 6. FTP model 3 UL throughput median and 5th percentile for low, medium and high load

# Channel access within COT

Related to the UE sharing of gNB initiated COT, following agreement was reached in RAN1#106-e [9]:

Agreement: On COT sharing from an initiating device transmission to responding device transmission, support both of the following two alternatives - Alt 1: No maximum gap defined between the initiating device transmission and responding device transmission. A responding device transmission can occur without LBT with any gap within the maximum COT duration - Alt 3: Define a maximum gap Y, such that a responding device transmission can occur without LBT only if the transmission starts within Y from the end of the initiating device transmission. If the responding device transmission starts after Y from the end of the initiating device transmission, a Cat 2 LBT is needed before the responding device transmission. - The Cat 2 LBT uses the same sensing structure as the 8 us initial deferral period as in eCCA - Further down-select between the following options: - Option 1: Y=8 us (motivated by need to operate in all regions) - Option 2: Y=a multiple number of OFDM symbols - Option 3: gNB determines Y (for example, according to local regulation) - Cat. 2 LBT is a UE capability - The usage of the two alternatives is a gNB choice and depends at least on local regulations. Note: Alt. 3 is motivated by the regulations in Japan, but use of Cat. 3 LBT is also an option for operation in Japan and Cat. 2 LBT is not restricted for use only in Japan. Note: Maximum gap allowed without Cat 2 LBT between two initiating device transmissions is to be separately discussed Note: Other use cases of Cat 2 LBT will be separately discussed

At 60 GHz band, the benefits from the transmitter LBT are rather unclear. As discussed in previous section, we did not observe tangible benefits from one-shot LBT at the gNB beam switch within COT. Hence, Alt. 1 appears as a reasonable approach in a large variety of deployments.

The agreement reached in RAN1#106-e determines the maximum gap in Alt 3 between the initiating device and responding device transmissions. In the case that gNB schedules UL transmissions for different UEs in TDM fashion to consecutive time resources, the maximum gap between gNB and UE transmissions may be exceeded for the latter UE(s). We do not see such limitation as reasonable or necessary from channel access or coexistence viewpoint. Instead, we see that a maximum gap should be allowed also between consecutive responding device transmissions. For the sake of simplicity, this gap can be the same as the gap Y allowed between the initiating device and responding device transmissions.

When considering the identified options for Y in Alt 3, a strict Y=8 us is needed to fulfill regulations in all regions. On the other hand, in most regions there is no definition for a maximum gap before the UE’s response without LBT, including EN 302 567. In those regions, a short Y value presents significant yet unnecessary restrictions for the use of Cat-1 channel access: Cat-1 channel access cannot be used for PUSCH or for PDSCH HARQ-ACK unless there are some other transmissions between the UL grant or the PDSCH because of the time required for UE processing. As Alt 3 is agreed as an alternative, we see it necessary to support Y values that allow for reasonable use of Alt 3, without complicated restrictions on UL scheduling. The supported Y values should include also values that are equal to or longer than PDSCH processing time and PUSCH preparation time. The PUSCH preparation time was agreed in RAN1#106-e [9] to be roughly 320 us for 120 kHz, 480 kHz, and 960 kHz SCS.

As a wide range of Y values would be needed and as the necessity of Cat-2 LBT is very much dependent on the particular setup the network is operating (local regulations, network load, etc.), we support Option 3 for Alt 3., as it gives the maximum flexibility to operate and mandate Cat-2 LBT from a responding device only if needed. It should be also noted that UE cannot detect the start of the gap on its own, especially with beam based operation where gNB may transmit on a beam not received by UE just before the gap. Hence, UE does not have any use for value Y alone and the value of Y can be left for gNB implementation.

Observation 13: In case of Alt. 3 for COT sharing, there is need for a wide range of time gap Y values to facilitate efficient scheduling while fulfilling local regulations having a wide range in requirements.

Proposal 16: Support Option 3, i.e., gNB determining maximum gap Y in Alt. 3. There is no need to signal the value Y to the UEs.

In general, when the UE is acting as a responding device within gNB initiated COT, there is a need to provide indication of the required channel access type (Cat-1, Cat-2 or Cat-3) to the UE. Hence, an appropriate signaling and procedure needs to be detailed and specified:

- For dynamically scheduled UL transmissions, Rel-16 DCI indication can be adopted with appropriate modifications on the indicated channel access types. In case of Alt 1, the indicated channel access types would include Cat-1 and Cat-3 while in case of Alt 3, Cat-1, Cat-2 or Cat-3 channel access type could be indicated. Unlike in Rel-16, there is no need to indicate CAPC or CP extension.

- For configured UL transmissions like scheduling request and CG-PUSCH, there are several options to consider, including the straightforward solution of configuring UE to perform always LBT initiating COT, or solutions using COT related signalling on a common DCI from gNB, e.g., UE always performing Cat-2 LBT within COT, or UE always performing Cat-2 LBT after a pre-configured time interval from the latest PDCCH but within indicated COT duration (so no additional dynamic signaling is needed), or delivering a COT specific configuration in a common DCI separating Cat-1 LBT slots from Cat-2 LBT slots, etc. Further, advanced dependencies may be used by the gNB, making Cat-2 LBT after a gap conditional to the UE Tx power, EIRP, etc. Hence, there is a need to define and agree on the preferable signaling options for semi-static UL transmissions indicating an appropriate channel access type for the UE.

Proposal 17: For dynamically scheduled UL transmissions, adopt Rel-16 DCI indication with appropriate modifications on the indicated channel access types. There is no need for an indication of CAPC or CP extension.

Proposal 18: For configured UL transmissions like scheduling request and CG-PUSCH, consider and agree on the necessary signalling indicating appropriate channel access type for the UE.

The determination of Pout based on the EIRP of the following transmission means that the EDT and channel access probability can depend considerably on the characteristics of the transmission. For example, CG PUSCH configurations of different number of RBs can have a considerable difference in EDT. If UE supports LBT and is configured with CG PUSCH, it would be attractive to use a configuration with high EDT outside of gNB initiated COT and other one with higher Tx power within gNB initiated COT, allowing for higher MCS and larger TBS. Hence, the configuration and operation of CG PUSCH should take the advantage of EDT dependency on Pout but without imposing considerable impacts on UE or gNB processing. This can be simply achieved by adding to the CG PUSCH configuration an indication whether the configuration is to be used inside or outside of a gNB initiated COT.

Proposal 19: CG PUSCH configuration shall include indication of whether the CG PUSCH configuration is used inside or outside of a gNB initiated COT, or both.

The results of the ED or interference level measurements at the gNB side are also informative to adjust the UE behavior (Tx power, MCS, etc.) when performing configured UL transmissions within the COT. Hence, gNB should be able to indicate the measured energy level to the UEs i.e., via GC-PDCCH with measured energy indication.

Proposal 20: Study the benefits of sharing the ED measurements results at gNB to the UEs.

Implementation of LBT procedure can have considerable impact on UE, especially if Rel-16 NR-U is not supported by the UE. With the use of short control signalling and Cat-1 channel access within COT, UEs without LBT can be supported from the channel access viewpoint. However, gNB should be aware of that so that gNB does not schedule UE with allocations requiring LBT.

Proposal 21: Support for Cat-3 LBT is UE capability.

# Receiver assistance in channel access

Following TR 38.808, in this WI we further discuss Class A receiver assistance channel access scheme – where Rx provides assistance information (signalling) to transmitter only. It was agreed that Class B, B1, B2, and C will not be investigated as a receiver assistance channel access schemes in the scope of Release 17.

Therefore, the Work Item studies only Class A receiver assistance, as defined in TR38.808 [2]:

The following receiver assisted channel access and interference management schemes have been considered and can be further investigated when specifications are developed.

-	Class A) Receiver provides assistance information (signalling) to transmitter only. The following aspects of Class A can be further discussed when specifications are developed.

-	Applicability in the following potential channel access modes:

-	LBT is performed prior to transmission,

-	No LBT is performed prior to transmission.

-	Details of assistance information (e.g., type, timing, content, how the assistance information is obtained etc.).

-	Whether the assistance information can be obtained by LBT performed at the receiver prior to transmission.

-	Whether the assistance information can be obtained by existing layer 1 and layer 3 measurements with enhancements if needed.

-	If any specification changes are needed to support Class A.

Ideal LBT would defer a transmission when the intended receiver is considerably interfered by an on-going transmission, or when the transmission would considerably interfere the reception of already on-going transmission. The basic approximation of transmitter ED-LBT is that channel energy measured on the transmitter is highly correlated with the channel energy at the receiver. While this is a reasonable approximation with omni-directional antennas, the measured channel energy may be very different at transmitter and receiver with beam-based operation.

Further, from the RAN1 106-e agreements [9] we have the following tools to be considered.

Agreement: For receiver to provide assistance in channel access, channel sensing and reporting need to be performed. The following schemes can be further considered. Target down-selection by RAN1 #106bis-e - Scheme 1: L1-RSSI based receiver assistance - Resource used for RSSI measurement - Alt 1: RSSI measurement is based on the time/frequency resources configured for ZP-CSI-RS - FFS: any enhancement needed for ZP-CSI-RS for this purpose (eg., ZP-CSI-RS over all REs in BWP over one or more symbols). - Alt 2: Energy measurement on operating BW over indicated or specified number of symbols or time interval - L1-RSSI is reported in an AP-CSI report - L1-RSSI trigger in UL grant - FFS if L1-RSSI trigger can also be carried in DL grant - Timeline for L1-RSSI reporting is at least equal to AP-CSI reporting and RAN1 strives to tighten the timeline - Note: If L1-RSSI reporting timeline cannot be tighter than AP-CSI reporting timeline, this scheme is not needed - FFS: How to indicate the measurement beam for L1-RSSI - FFS: What is included in the L1-RSSI report, such as the value of RSSI measurement, comparison outcome with Energy Detection threshold, etc - Scheme 2: CCA or eCCA based receiver assistance with existing phy channel/signals - Scheme 2-1: gNB schedules/triggers UL PUCCH/SRS transmission with the DL assignment DCI and indicates CCA or eCCA in the DCI. UE performs CCA or eCCA for the scheduled/triggered UL transmission and if LBT passes, transmits the Receiver-assistance information (implicitly or explicitly) in the PUCCH (or SRS in the case of 1-bit Rx-assistance) to indicate the LBT outcome. gNB detects the scheduled UL transmission to tell if UE passes the CCA or eCCA. After detecting the Receiver-assistance information, the downlink data transmission happens. - FFS if the downlink data transmission can be granted with the same DL DCI that schedules/triggers the first UL PUCCH/SRS transmission, in which case, the CCA or eCCA is performed for at least the first UL PUCCH/SRS transmission - Scheme 2-2: gNB schedules/triggers UL transmission PUSCH with the UL assignment DCI and indicates CCA or eCCA in the DCI. UE performs CCA or eCCA for the scheduled/triggered UL transmission and if LBT passes, transmits the Receiver-assistance information (implicitly or explicitly) in the PUSCH to indicate the LBT outcome. gNB detects the scheduled UL transmission to tell if UE passes the CCA or eCCA. After detecting the Receiver-assistance information, the downlink data transmission happens. - Scheme 3: CCA or eCCA based receiver assistance with new RTS/CTS type transmission - New RTS/CTS-like signaling introduced. - gNB sends RTS-like signaling to UE. UE performs CCA or eCCA and if LBT passes, transmits CTS-like signaling to explicitly indicate the LBT outcome. gNB detects the CTS-like signaling to identify if the UE passed CCA or eCCA. After detecting the CTS-like signal, the data transmission happens - Scheme 4: Legacy L3-RSSI with potential enhancements - FFS potential enhancements, e.g., supporting gNB indicating the beam used for UE RSSI measurement, supporting gNB indicating new reference SCS and measurement bandwidths - Note: The schemes listed above are not mutually exclusive and should be discussed separately.

In RAN1 #106-bis-e following decisions were made:

Conclusion: There is no consensus to support CCA or eCCA based receiver assistance with new RTS/CTS type transmission Agreement: Support extending Rel.16 L3-RSSI to unlicensed operation in FR2-2 - Introduce RRC configuration for reference SCS, measurement duration, and measurement bandwidth - Extend the reference SCS/CP field (ref-SCS-CP-r16) and measurement duration field (measDurationSymbols-r16) in RMTC-Config - FFS value range and valid combinations for ref-SCS-CP-r16 and measDurationSymbols-r16 - Introduce parameter in RMTC-Config to indicate the measurement bandwidth - FFS: Value range for measurement bandwidth - For the QCL Type-D of L3-RSSI measurement, down-select one or both of the following alternatives - Alt 1: gNB configures the beam when configures the L3-RSSI measurement - Alt 2: Use the QCL type-D of the latest received PDSCH and the latest monitored CORESET

Since the LBT at the receiver introduces additional delay and complexity, it is reasonable to utilize the existing measurements and reports as a part of receiver assistance. Therefore, Scheme 1 and Scheme 4 from the above agreement can be chosen as options of interest. For Scheme 1 the Alt 2 of using simple energy measurement is more natural in the context of RSSI measurement. Scheme 4 without changes is obviously supported already now, however support for the new SCSs for the 60GHz band need to be added.

Proposal 22: For L1-RSSI use energy measurement on operating BW over indicated or specified number of symbols or time interval as resource used for RSSI measurement (Alt 2)

There has been agreement shown above in RAN1#106bis-e related to supporting scheme 4. Agreement leaves bunch of parameters for further study including SCS. We propose to extend the reference SCS/CP field and add the SCS values 120, 480 and 960 kHz. Another FFS is measurement duration. Currently for normal CP there are values 1, 14, 28, 42 and 70 symbols. In our view the same values can be kept also with new subcarrier spacings. For measurement bandwidth signalling the channel bandwidths defined by RAN4 must be supported. RAN4 is discussing channelization aspects, however the discussion is not completed yet. So far only the following agreements on minimum and maximum channel bandwidths exist [17]:

- 120kHz: Min. 100MHz, Max. 400MHz

- 480kHz: Min. 400MHz, Max. 1600MHz

- 960kHz: Min. 400MHz

Hence at least 100MHz, 400MHz and 1600MHz need to be supported.

Proposal 23: Add 120, 480 and 960 kHz as SCS options to ref-SCS-CP-r16.

Proposal 24: Use the Rel-16 values for measurement duration (measDurationSymbols-r16).

Proposal 25: Channel bandwidths defined by RAN4 are used as measurement bandwidths.

Also QCL Type-D of L3-RSSI measurement has been left for further study with two alternatives. Either the beam is configured using RRC configuration or it follows latest received PDSCH and the latest monitored CORESET. In some case it may be beneficial to be able to configure the beam for measurement before transmitting so we favour RRC configuration.

Proposal 26: For the QCL Type-D of L3-RSSI measurement, gNB configures the beam when it configures the L3-RSSI measurement (Alt 2)

Scheme 2 with implicit signalling is already supported due to the decision to introduce Cat 2 LBT, with Cat 2 LBT agreed to be optional for the UE. Adding explicit signalling of assistance information requires some standardization work. If indication of Cat 1, 2 or 3 channel access is included into scheduling DCI as in Rel-16 NR-U, operation according to scheme 2 will be possible without any further specification effort.

Observation 14: The network can operate scheme 2 in a fully standards transparent manner. There is no need to define further mechanisms to support scheme 2.

# Channel access without LBT

60 GHz unlicensed band can be used for a wide variety of NR use cases, some of which, e.g. relaying and IAB backhaul, are relevant for scenarios where transmissions are highly directional. In such scenarios, the need for and benefits from LBT are unclear. However, LBT procedure can have considerable implementation impact for the very high data rate devices to be used on 60 GHz band. Correspondingly, it is stated in WID [1] that channel access mechanism without LBT is supported:

- Physical layer procedure(s) including [RAN1]:

  - Channel access mechanism assuming beam based operation in order to comply with the regulatory requirements applicable to unlicensed spectrum for frequencies between 52.6GHz and 71GHz.

    - Specify both LBT and No-LBT related procedures, and for No-LBT case no additional sensing mechanism is specified.

As LBT procedure involves considerable implementation impact, the mechanism without LBT should cause only limited additional implementation efforts. Hence it is reasonable that additional sensing mechanisms are not required as also stated by the WID.

The variety of relevant use cases may be seen also in ETSI BRAN work. In addition to the work item on harmonized standard EN 302 567 [3] covering the requirements of at least c1, ETSI BRAN has an ongoing work item developing harmonized standard EN 303 722 for fixed network equipment according to the requirements of at least c3. Channel access mechanisms chosen in this work item are Automatic Transmit Power Control (ATPC) and Automatic Link Adaptation (ALA). Power control range in EN 303 722 specification draft approved for European Norm Approval Process (ENAP) seems to be settling to minimum of 3dB [10]. Proposed requirements for ALA are 10% reduction in duty cycle for a given amount of payload or 3dB reduction in power.

A work item for a new harmonized standard (EN 303 753) to cover devices not in scope of the other two standards was adopted by BRAN in July 2020 [5], and the intended harmonized standard is likely suitable for 3GPP NR-U systems. Work on EN 303 753 is expected to continue until late 2021, and the decisions on coexistence mechanisms would take into account latest developments in 3GPP and other relevant SDOs. We see that channel access mechanism without LBT should fulfil the expected requirements of EN 303 753.

Observation 15: Channel access mechanism without LBT should fulfil the requirements of EN 303 722 as well as the expected requirements of EN 303 753.

## Beamforming impact

Beamforming impacts significantly interference as well as co-channel coexistence. Narrow beams or large antenna gain will limit the transmitted signal to a certain direction (from the transmitter). If EIRP is kept constant, the spatial area interfered by a transmitted signal is reduced with more narrow beams. Additionally, narrow Rx beams can suppress interference coming from directions outside the Rx beam.

In [6] we have presented simulation results for single and two operators in the Scenario Indoor-A [7]. Both full load and half-load cases were evaluated. The offered load as well as the number of served UEs was doubled on the indoor office area when the second network was introduced. Nevertheless, drop in mean throughput remained below 30% for DL and below 40% for UL. 30% to 60% throughput losses were observed at 5%ile cell edge. The results show that beamforming alone without LBT, even with a modest number of antenna elements, can provide enough spatial separation for efficient coexistence of two networks. Hence when beamforming is used, simultaneous use of LBT is not necessary.

In [8] we have provided simulation results for single and two operators in both Indoor and Outdoor scenarios. Conclusions of these simulations are very similar to the previous paragraph. BRAN has discussed the coexistence for EN 303 753 based on the beamforming as the primary mechanism and will continue the discussion in next meetings. In Section 3 we have provided simulation results with no-LBT and several LBT modes. Also these results show clearly that use of LBT is not required and beamforming performs similar or even better than LBT as channel access method.

It would be useful to have possibility to implement NR-U UE without mandatory LBT support. In this case the channel access could be based on beamforming. Beamforming gain for a typical UE is lower than for a typical base station and hence UE requirements for channel access could be different. Possibly lower beamforming requirements could be tied e.g. to use of some other limitation e.g. lower EIRP, limited duty cycle or limited response time for base station transmissions.

## Conditions for the use of no-LBT channel access mode

It is stated in TR 38.808 [2]:

For operation where LBT is not required, the following can be further discussed when specifications are developed:

-	whether to introduce additional conditions/mechanisms for no-LBT to be used, or whether to leave it for gNB implementation,

-	when no-LBT mode is used, whether to introduce additional restrictions, such as DFS needs to be applied, ATPC needs to be applied, long term sensing needs to be applied, certain duty cycle limitation, certain transmit power limitation, MCOT limits, etc, or leave the restriction for gNB implementation,

-	when no-LBT mode is used, whether to introduce mechanism for the system to fallback to LBT mode, or whether to leave it for gNB implementation.

There have been proposals of restrictions to use of no-LBT mode, or fallback to another mechanism when using no-LBT mode, e.g. DFS, ATPC, automatic link adaptation, duty cycle limitation (MCOT limitation, stay away time) or fallback to LBT e.g. using timer.

It has been shown in the paragraphs above that beamforming works adequately well as channel access method. Also there are regulations and ETSI harmonized standards that do not require additional conditions/mechanisms to be used such as EN 303 753 [5]. Actually for EN 303 753 duty cycle limitation is being discussed for responding devices which do not meet the beamforming gain requirement. Some other like EN 303 722 [10] require methods such as ATPC and automatic link adaptation. Implementation of gNBs are required to follow the regulations applied for the location and the use case. ATPC and automatic link adaptation are such methods that are supported in NR already now, although it is not clear if regulations require some changes to the existing functionality. If 3GPP would also mandate use of additional mechanisms there would be a risk that there are several redundant mechanisms implemented, which would introduce unnecessary complexity. LBT has much higher impact to implementation and hence implementing LBT should not be mandatory for products following regulations that do not require LBT. Hence we propose to leave additional conditions or restrictions on the use of no-LBT mode or additional coexistence mechanisms or fallbacks for no-LBT mode for gNB implementation which anyway has to be compliant with the regulations.

Proposal 27: Leave any additional conditions/mechanisms/restriction/fallback modes on the no-LBT channel access mode for gNB implementation.

# Conclusion

In this contribution, we considered both LBT and no-LBT channel access mechanisms for NR on 60 GHz unlicensed band. We made following observations and proposals:

Proposal 1: A short contention window size of [4] is used in the LBT procedure

Observation 1: For initiating device resuming transmission after a long transmission gap, Cat-2 LBT performance cannot be compared against Cat-3 LBT as long as Cat-3 LBT design remains open.

Proposal 2: Do not support Cat-2 LBT in beam switching or in multi-channel LBT.

Proposal 3: For Pout in EDT determination, define Pout as the maximum EIRP of the intended transmissions by the node determining EDT during a COT.

Observation 2: In LBT, energy is measured after antenna and antenna gain is included in the energy measurement.

Proposal 4: Further adjustment of EDT based on the transmit or sensing beam gain is not specified.

Proposal 5: Minimum duration for energy measurement during the 5 µs observation slot is not defined. LS is sent to RAN4 to consider defining a requirement on measurement or channel access accuracy.

Proposal 6: Only Type A multi-channel access procedure (i.e. Alt.1 defined in RAN1#104-e meeting) shall be supported in NR-U on 60GHz band.

Observation 3: EN 302 567, v2.2.0 allows for Short Control Signalling transmissions for up to 10% of time within an observation period of 100 ms.

Proposal 7: NR-U design for 60 GHz bands supports transmission of the following DL and UL control and management signals as short control signalling without LBT:

- Downlink: SS/PBCH blocks (already agreed), PDCCH, CSI-RS and other reference signals, e.g., for beam management, SIBs, Paging

- Uplink: HARQ-ACK feedback on either PUCCH or PUSCH, Scheduling Request, CSI feedback, Sounding RS, e.g., for beam management, RACH related transmissions

Proposal 8: For the UL transmissions, the 10% short control signalling allowance is shared by all the UEs in the cell.

Observation 4: Short contention window of [4] observation slots facilitates flexible LBT timing for SSB transmissions.

Observation 5: Depending on SSB sub-carrier spacings and SSB periodicity, only sub-set of total SSBs can be covered by short control signalling exemption

Proposal 9: It is possible to apply SCSe to one part of actually transmitted SSBs and LBT procedure for other/rest of the SSBs.

Proposal 10: Use of short control signal contention exemption and use of LBT for SSBs is predetermined or semi-statically determined, distributing the channel access uncertainty over the SSBs.

Observation 6: Generic requirements for relative relationship between LBT sensing beam(s) and transmission beam(s) should be done in RAN4, not in RAN1.

Proposal 11: Proposal 2.9.2-3 from RAN1#106bis-e FL summary is agreed for defining the relative relationship between applicable LBT sensing beam(s) and the transmission beam(s) for gNB, when UE does not indicate capability for beam correspondence without UL beam sweeping or when UE uses different beam for sensing than for transmission.

Proposal 12: COT initiating LBT with multiple independent per-beam LBT sensing should be deprioritised while completing the design for baseline channel access procedures.

Proposal 13: For a COT with MU-MIMO (SDM) or within a COT with TDM of beams with beam switching, support both Alt 1 and Alt 2.

Proposal 14: For a COT with MU-MIMO (SDM) transmission, support Alt B.

Observation 7: It is important to maintain flexibility of gNB implementation for multi-beam COT.

Observation 8: Alt A-1 should be modified as: The node completes one eCCA on one beam, and directly moves on to the eCCA on the other beam, with no transmission in the middle. After completing eCCA on all beams, a further round robin CCA check is carried out in all beams (except the last beam).

Observation 9: Alt A-3 should be modified as: The node performs eCCA of the different beams simultaneous, round robin between different beams.

- single contention window is shared by beams or each beam has a separate contention window.

- the last CCAs shall indicate vacant channel on all beams that are part of the COT

Proposal 15: For a COT with TDM transmission, support the modified Alt A-1 and Alt A-3:

- Alt A-1: The node completes one eCCA on one beam, and directly moves on to the eCCA on the other beam, with no transmission in the middle. After completing eCCA on all beams, a further round robin CCA check is carried out in all beams (except the last beam).

- Alt A-3: The node performs eCCA of the different beams simultaneous, round robin between different beams.

  - single contention window is shared by beams or each beam has a separate contention window.

  - the last CCAs shall indicate vacant channel on all beams that are part of the COT

Observation 10: Cat-2 LBT at every gNB beam switch would cause significant increase in overhead and is not even possible between the SSBs in the agreed SSB time locations.

Observation 11: Use of LBT provides mostly loss in median throughput compared to no-LBT mode and reduces throughput for cell edge UEs

Observation 12: Simulation results do not show any gain from introduction of additional Cat-2 LBT at gNB beam switch during COT.

Observation 13: In case of Alt. 3 for COT sharing, there is need for a wide range of time gap Y values to facilitate efficient scheduling while fulfilling local regulations having a wide range in requirements.

Proposal 16: Support Option 3 i.e., gNB determining maximum gap Y in Alt. 3. There is no need to signal the value Y to the UEs.

Proposal 17: For dynamically scheduled UL transmissions, adopt Rel-16 DCI indication with appropriate modifications on the indicated channel access types. There is no need for an indication of CAPC or CP extension.

Proposal 18: For configured UL transmissions like scheduling request and CG-PUSCH, consider and agree on the necessary signalling indicating appropriate channel access type for the UE.

Proposal 19: CG PUSCH configuration shall include indication of whether the CG PUSCH configuration is used inside or outside of a gNB initiated COT, or both.

Proposal 20: Study the benefits of sharing the ED measurements results at gNB to the UEs.

Proposal 21: Support for Cat-3 LBT is UE capability.

Proposal 22: For L1-RSSI use energy measurement on operating BW over indicated or specified number of symbols or time interval as resource used for RSSI measurement (Alt 2)

Proposal 23: Add 120, 480 and 960 kHz as SCS options to ref-SCS-CP-r16.

Proposal 24: Use the Rel-16 values for measurement duration (measDurationSymbols-r16).

Proposal 25: Channel bandwidths defined by RAN4 are used as measurement bandwidths.

Proposal 26: For the QCL Type-D of L3-RSSI measurement, gNB configures the beam when it configures the L3-RSSI measurement (Alt 2)

Observation 14: The network can operate scheme 2 in a fully standards transparent manner. There is no need to define further mechanisms to support scheme 2.

Observation 15: Channel access mechanism without LBT should fulfil the requirements of EN 303 722 as well as the expected requirements of EN 303 753.

Proposal 27: Leave any additional conditions/mechanisms/restriction/fallback modes on the no-LBT channel access mode for gNB implementation.

# References

- RP-211584, “Revised WID: Study on supporting NR from 52.6GHz to 71 GHz”, Qualcomm, Intel

- TR 38.808, “Study on supporting NR from 52.6GHz to 71 GHz", 3GPP

- EN 302 567 v2.2.0, "Multiple-Gigabit/s radio equipment operating in the 60 GHz band; Harmonised Standard covering the essential requirements of article 3.2 of Directive 2014/53/EU", ETSI

- TR 36.889, “Study on Licensed-Assisted Access to Unlicensed Spectrum", 3GPP

- BRAN230026v003, “Wideband Data Transmission Systems (WDTS) for Mobile and Fixed Radio Equipment operating in the 57 - 71 GHz band; Harmonised Standard for access to radio spectrum”

- R1-2006909, “Simulation Results for NR from 52.6 GHz to 71 GHz”, Nokia, Nokia Shanghai Bell

- R1-2005193, “Summary #3 of email discussions for [101-e-Post-NR-52_71_GHz]”, Intel

- BRAN(20)108042, “60GHz beamforming coexistence”, Nokia

- “Draft Report of 3GPP TSG RAN WG1 #106-e v0.2.0”, 3GPP

- EN 303 722 v0.0.11, “Wideband Data Transmission System (WDTS) for Fixed Network Radio Equipment operating in the 57 - 71 GHz band; Harmonised Standard for access to radio spectrum”, ETSI

- “Draft Report of 3GPP TSG RAN WG1 #103-e v0.2.0”, 3GPP

- R1-2110540, “FL summary of channel access mechanism for 52.6GHz-71GHz band, ver2”, Qualcomm

- “Draft Report of 3GPP TSG RAN WG1 #104-e v0.3.0”, 3GPP

- “Draft Report of 3GPP TSG RAN WG1 #104bis-e v0.1.0”, 3GPP

- “Draft Report of 3GPP TSG RAN WG1 #105-e v0.2.0”, 3GPP

- “Draft Report of 3GPP TSG RAN WG1 #106bis-e v0.1.9”, 3GPP

- R1-2104173, “Reply LS on the maximum/minimum channel bandwidths and channelization for NR operation in 52.6 to 71GHz”, RAN4

# Appendix

Simulation parameters are provided in this appendix.

Figure 7. Half size office scenario.

Table 1. Simulation Parameters

|  | Base Station - BS | User Equipment - UE |
| --- | --- | --- |
| Carrier Frequency [GHz] | 60 GHz |  |
| Subcarrier Spacing [kHz] | 960 kHz |  |
| Bandwidth [MHz] | 2000 MHz |  |
| Number of RB | 160 |  |
| Deployment Scenario | Scenario indoor-A, half size |  |
| UE distribution | 100% indoor |  |
| Channel Model | - gNB-to-gNB and gNB-to-UE links: InH – office channel & PL model from TR38.901 - UE-to-UE links: [InH – office channel & PL model from TR38.901] |  |
| Mobility | 3 km/hr |  |
| Antenna Configuration | (Mg,Ng,M,N,P) = (1,1,4,8,2) with (0.5 dv, 0.5 dH) | (Mg,Ng,M,N,P) = (1,2,2,2,2) with (0.5 dv, 0.5 dH) |
| Antenna Pattern | Table A.2.1-7 of TR38.802 for ceiling mount | Table A.2.1-8 of TR38.802 |
| Antenna Element Gain | 5 dBi | 5 dBi |
| Power Limitation | 40 dBm EIRP | 25 dBm EIRP |
| Noise figure | 7 dB | 10 dB |
| Transmission Rank | Rank adaptative transmission between Rank 1 and 2 |  |
| TDD DL/UL Ratio | Dynamic TDD |  |
| CSI Feedback | Ideal feedback |  |
| Traffic Model | FTP Model 3 (27 Mbyte file) Num packets per second per user: 2 (low), 3.5 (medium), 5 (high) (100% DL) Num packets per second per user: 1.5 (low), 2.75 (medium), 4 (high) (50/50%, split between UL/DL) |  |
| UE Receiver | MMSE-IRC |  |
| Cell selection criteria | Random select from strongest RSRP with 1 dB HO Margin |  |
| DL/UL Traffic Ratio | 100% DL, 50/50% DL/UL |  |