3GPP TSG-RAN WG1 Meeting #107-e	R1-2111211

# e-meeting, November 11th – 19th, 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DRAFT CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.214 | CR |  | rev | - | Current version: | 15.14.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction on determination of TDRA table to be used for PUSCH |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CATT |  |  |  |  |  |  |  |  |  |
| Source to TSG: |  |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Core |  |  |  |  |  | Date: |  |  | 2021-11-05 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | According to current specification, dedicated TDRA table configured by pusch-TimeDomainAllocationList provided in pusch-Config may be used for PUSCH scheduled by DCI scrambled by TC-RNTI (i.e. Msg 3 retransmission). To be more specific, for CBRA in RRC CONNECTED, if pusch-TimeDomainAllocationList is provided in pusch-Config and DCI scheduling Msg 3 retransmission is transmitted in Type1-PDCCH CSS not associated with CORESET 0, the dedicated TDRA table is used for Msg 3 retransmissions. However, gNB cannot differentiate UEs during Msg 3 reception so common TDRA table instead of dedicated TDRA table should be used. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Correct Table 6.1.2.1.1-1 to specify that Default A or pusch- TimeDomainAllocationList provided in pusch-ConfigCommon is used for PUSCH scheduled by DCI scrambled by TC-RNTI. - Correct the RRC parameter name in Table 6.1.2.1.1-1 - Add “-” in the blank entries in Table 6.1.2.1.1-1 to be aligned with other entries. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | - Misalignment on TDRA table determination for PUSCH scheduled by DCI scrambled by TC-RNTI transmitted in Type1-PDCCH CSS not associated with CORESET 0 in case dedicated TDRA table is configured. - Misalignment on RRC parameter name with TS38.331. - Potential confusion on the difference between a blank entry and an entry with “-”. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.1.2.1.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | Isolated Impact Analysis: The CR impact is isolated to PUSCH scheduled by DCI scrambled by TC-RNTI transmitted in Type1-PDCCH CSS not associated with CORESET 0 in case dedicated TDRA table is configured. The correction is common understanding in RAN1 and should not impact gNB and UE implementations. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

##### 6.1.2.1.1	Determination of the resource allocation table to be used for PUSCH

Table 6.1.2.1.1-1 defines which PUSCH time domain resource allocation configuration to apply. Either a default PUSCH time domain allocation A according to table 6.1.2.1.1-2, is applied, or the higher layer configured pusch-TimeDomainAllocationList in either pusch-ConfigCommon or pusch-Config is applied.

Table 6.1.2.1.1-4 defines the subcarrier spacing specific values j. j is used in determination of K2 in conjunction to table 6.1.2.1.1-2, for normal CP or table 6.1.2.1.1.-3 for extended CP, where µPUSCH is the subcarrier spacing configurations for PUSCH.

Table 6.1.2.1.1-5 defines the additional subcarrier spacing specific slot delay value for the first transmission of PUSCH scheduled by the RAR. When the UE transmits a PUSCH scheduled by RAR, the Δ value specific to the PUSCH subcarrier spacing µPUSCH is applied in addition to the K2 value.

Table 6.1.2.1.1-1: Applicable PUSCH time domain resource allocation

| RNTI | PDCCH search space | pusch-ConfigCommon includes pusch-TimeDomainAllocationList | pusch-Config includes pusch-TimeDomainAllocationList | PUSCH time domain resource allocation to apply |
| --- | --- | --- | --- | --- |
| PUSCH scheduled by MAC RAR as described in clause 8.2 of [6, TS 38.213] |  | No | - | Default A |
|  |  | Yes | - | pusch-TimeDomainAllocationList provided in pusch-ConfigCommon |
| TC-RNTI | Type1 common | No | - | Default A |
|  |  | Yes | - | pusch- TimeDomainAllocationList provided in pusch-ConfigCommon |
| C-RNTI, MCS-C-RNTI, CS-RNTI | Any common search space associated with CORESET 0 | No | - | Default A |
|  |  | Yes | - | pusch-TimeDomainAllocationList provided in pusch-ConfigCommon |
| C-RNTI, MCS-C-RNTI, CS-RNTI, SP-CSI-RNTI | Any common search space not associated with CORESET 0, UE specific search space | No | No | Default A |
|  |  | Yes | No | pusch-TimeDomainAllocationList provided in pusch-ConfigCommon |
|  |  | No/Yes | Yes | pusch-TimeDomainAllocationList provided in pusch-Config |

Table 6.1.2.1.1-2: Default PUSCH time domain resource allocation A for normal CP

| Row index | PUSCH mapping type | K2 | S | L |
| --- | --- | --- | --- | --- |
| 1 | Type A | j | 0 | 14 |
| 2 | Type A | j | 0 | 12 |
| 3 | Type A | j | 0 | 10 |
| 4 | Type B | j | 2 | 10 |
| 5 | Type B | j | 4 | 10 |
| 6 | Type B | j | 4 | 8 |
| 7 | Type B | j | 4 | 6 |
| 8 | Type A | j+1 | 0 | 14 |
| 9 | Type A | j+1 | 0 | 12 |
| 10 | Type A | j+1 | 0 | 10 |
| 11 | Type A | j+2 | 0 | 14 |
| 12 | Type A | j+2 | 0 | 12 |
| 13 | Type A | j+2 | 0 | 10 |
| 14 | Type B | j | 8 | 6 |
| 15 | Type A | j+3 | 0 | 14 |
| 16 | Type A | j+3 | 0 | 10 |

Table 6.1.2.1.1-3: Default PUSCH time domain resource allocation A for extended CP

| Row index | PUSCH mapping type | K2 | S | L |
| --- | --- | --- | --- | --- |
| 1 | Type A | j | 0 | 8 |
| 2 | Type A | j | 0 | 12 |
| 3 | Type A | j | 0 | 10 |
| 4 | Type B | j | 2 | 10 |
| 5 | Type B | j | 4 | 4 |
| 6 | Type B | j | 4 | 8 |
| 7 | Type B | j | 4 | 6 |
| 8 | Type A | j+1 | 0 | 8 |
| 9 | Type A | j+1 | 0 | 12 |
| 10 | Type A | j+1 | 0 | 10 |
| 11 | Type A | j+2 | 0 | 6 |
| 12 | Type A | j+2 | 0 | 12 |
| 13 | Type A | j+2 | 0 | 10 |
| 14 | Type B | j | 8 | 4 |
| 15 | Type A | j+3 | 0 | 8 |
| 16 | Type A | j+3 | 0 | 10 |

Table 6.1.2.1.1-4: Definition of value j

| µPUSCH | j |
| --- | --- |
| 0 | 1 |
| 1 | 1 |
| 2 | 2 |
| 3 | 3 |

Table 6.1.2.1.1-5: Definition of value Δ

| µPUSCH | Δ |
| --- | --- |
| 0 | 2 |
| 1 | 3 |
| 2 | 4 |
| 3 | 6 |