3GPP TSG RAN WG1 #107-e                                                                                  R1-2111223

e-Meeting, November 11th – 19th, 2021

Source:	CATT

Title:                        Remaining issues on inter-cell operation for multi-TRP/panel

Agenda Item:	8.1.2.2

Document for:	Discussion and Decision

Introduction

In RAN1#106-e meeting, the following agreement on inter-cell mTRP operation was achieved [1]:

Agreement

- For inter-cell mTRP , one PCI associated with one or more of activated TCI states for PDSCH/PDCCH is associated with one CORESETPoolIndex , another PCI associated with one or more of activated TCI states for PDSCH/PDCCH is associated with another CORESETPoolIndex

- FFS : The association between PCI and CORESETPoolIndex when switching between intra-cell mTRP and inter-cell mTRP

In RAN1#106bis-e meeting, the following agreements on inter-cell mTRP operation were further achieved [2]:

Agreement

- Center frequency, SCS, SFN offset are assumed to be the same for SSBs from the serving cell and the configured  SSBs with PCI different from the serving cell for inter-cell multi TRP operation.

- The information related to “SSB time domain position” for SSB with PCI different from the serving cell consists of [halfFrameIndex and] ssb-PositionsInBurst

Agreement

Support two independent X values (X1, X2) are reported as a UE capability for two different assumptions on additional SSB time domain position and periodicity with respect to serving cell SSB.

- X1 (Case 1)= The maximum number of configured additional PCIs when each configuration of SSB time domain positions and periodicity of the additional PCIs is the same as SSB time domain positions and periodicity of the serving cell PCI

- X2 (Case 2)= The maximum number of configured additional PCIs when the configurations of SSB time domain positions and periodicity of the additional PCIs is not according to Case 1

- Note: By definition, Case 1 and Case 2 cannot be enabled simultaneously

- Supported values for X1 and X2 includes at least 0,1,2,3 and 7. FFS on other values

- This UE capability has FR1 and FR2 differentiation (FFS : Whether this UE capability is per UE or per band)

In this contribution, our views on PCI association with CORESETPoolIndex and rate matching would be presented.

Discussions

PCI association with CORESETPoolIndex

According to the agreement of RAN1#106-e meeting, there is a following FFS point remained on PCI association with CORESETPoolIndex:

- FFS : The association between PCI and CORESETPoolIndex when switching between intra-cell mTRP and inter-cell mTRP

The association between CORESET and CORESETPoolIndex is configured by RRC. For example, CORESET0 and CORESET1 are both associated with CORESETPoolIndex0. CORESET2 and CORESET3 are both associated with CORESETPoolIndex1. In the case of inter-cell mTRP, there is a restriction that for PDSCH/PDCCH associated with one CORESETPoolIndex, MAC CE activates one or more TCI states associated with only one PCI at a time. Therefore, the activated TCI states for CORESET0 and CORESET1 are associated with PCI0. The activated TCI states for CORESET2 and CORESET3 are associated with PCI1. Then, inter-cell mTRP could be achieved by CORESET0 (CORESET1) and CORESET2 (CORESET3). For intra-cell mTRP as supported in Rel-16, all the activated TCI states associated with two values of CORESETPoolIndex should be associated with one cell. Consequently, if a new TCI associated with PCI0 is activated for CORESET2, intra-cell mTRP (CORESET0 and CORESET2) would be switched to. Therefore, switching between intra-cell and inter-cell mTRP has already been supported using MAC-CE without additional spec impact.

Proposal-1: MAC CE based switching between intra-cell and inter-cell mTRP has already been supported without additional spec impact.

Rate matching

In RAN1#104-e, for non-serving cell information, the following agreement was achieved [3]:

Agreement

Agree on scheme1

- Scheme1: PDSCH/PDCCH from non-serving cell (PCI) associated with TCI state and/or QCL-info is rate matched around non-serving cell SSB with the same PCI

- FFS: whether PDSCH /PDCCH from serving cell (PCI) is rate matched around non-serving cell SSB

- FFS: whether PDSCH/PDCCH from non-serving cell (PCI) associated with TCI state and/or QCL-info is rate matched around serving cell SSB

In current specs, there are rate matching mechanisms for SSB and PDSCH in one cell. For Inter-cell operation, despite of possible non-ideal backhaul between serving cell and non-serving cell, the semi-static coordination of PDSCH resource allocation between coordinated cells is still feasible. To avoid interference between SSB and PDSCH from different cells, the UE shall assume that the PRBs containing SSB resources are not available for PDSCH in the OFDM symbols where SSB is transmitted in the other coordinated cell.

Proposal-2: PDSCH/PDCCH from serving cell is rate matched around non-serving cell SSB. PDSCH/PDCCH from non-serving cell is rate matched around serving cell SSB.

Conclusions

In this contribution, we focus on the remaining issues of PCI association with CORESETPoolIndex and rate matching for inter-cell mTRP. We have the following proposals:

Proposal-1: MAC CE based switching between intra-cell and inter-cell mTRP has already been supported without additional spec impact.

Proposal-2: PDSCH/PDCCH from serving cell is rate matched around non-serving cell SSB. PDSCH/PDCCH from non-serving cell is rate matched around serving cell SSB.

References

[1] 3GPP RAN1#106-e meeting, Chairman’s notes.

[2] 3GPP RAN1#106bis-e meeting, Chairman’s notes.

[3] 3GPP RAN1#104-e meeting, Chairman’s notes.