3GPP TSG RAN WG1 #107-e		                                                                 R1-2111208

e-Meeting, November 11th – 19th, 2021

Title:	[Draft] Reply LS on initial state of elements controlled by MAC CEs

Response to:	R2-2109098

Release:	Rel-15, Rel-16

Work Item:	NR_newRAT-Core

Source:

To:	RAN2

Cc:	RAN4

Attachment:	-

Contact Person:

Name:	Min Zhu

E-mail Address:	zhumin@catt.cn

Overall description

RAN1 thanks RAN2 for the LS on initial state of elements controlled by MAC CEs, and would provide responses to the following questions from RAN2.

Firstly, since a handover only refers to PCell change, RAN2 respectfully asks RAN1 to clarify whether the initial deactivation when using handover should be applied for both PCell change and PSCell change/addition in case of DC.

Response to RAN2:

Yes. Either PCell change or PSCell change/addition would result in CSI-RS/SRS resources or CSI reportings being updated/reconfigured. The same UE behavior should be assumed for these two scenarios. Therefore, the handover mentioned should include both PCell change and PSCell change/addition.

Secondly, RAN2 respectfully asks RAN1 to clarify whether the initial deactivation when using configuration should be applied for both “initial configuration by RRC” and “reconfiguration by RRC”.

Response to RAN2:

Yes. Similar as the first question, new CSI-RS/SRS resources or CSI reportings would be configured to the UE whenever initial RRC configuration or RRC reconfiguration happens. The initial deactivation should be applied to both cases.

Thirdly, RAN2 respectfully asks RAN1 to clarify whether the UE behavior relevant to (Enhanced) PUCCH spatial relation Activation/Deactivation MAC CE should be aligned with the other MAC CEs.

Response to RAN2:

Yes. Spatial relation info for (Enhanced) PUCCH are configured by RRC and then activated by the MAC CE. It has similar activation mechanism to that of CSI-RS/SRS or CSI reporting. Therefore, the UE behavior should be aligned with other MAC CEs.

Actions:

To RAN2:

RAN1 respectfully requests RAN2 to take the above responses into account in the future work.

Date of Next TSG-RAN1 Meetings:

RAN1#108-e			17-25 January 2022   			E-Meeting