3GPP TSG RAN WG1 #107-e                                                   R1-2111240

e-Meeting, November 11th – 19th, 2021

Source:              CATT

Title:                   Discussion on UE feature of UE Power saving enhancements for NR

Agenda Item:     8.16.7

Document for:   Discussion and Decision

# Introduction

The UE feature of UE power saving enhancements for NR [1] was discussed in RAN1#106b-e.  Feature Groups 29-1 Paging Enhancement and 29-2 TRS resources for IDLE/Inactive UEs were agreed as the place holder for further discussion as follows,

Agreement

FG 29-1 is kept as placeholder as “Paging enhancement” as follows

| 29. NR_UE_pow_sav_enh | 29-1 | Paging enhancement | 1. Support paging early indication 2. Support UE subgroup indication |  | N |  | High idle/inactive mode UE power consumption if NR SA networks | Per UE | N | N | N | For component 2, it is up to RAN2 whether/how to separate the capability for UE subgroup indication | Optional without capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Note that yellow highlight means FFS and to be discussed further. These parts are provides as placeholders.

Agreement

FG 29-2 is kept as “TRS resources for idle/inactive UEs” as follows.

| 29. NR_UE_pow_sav_enh | 29-2 | TRS resources for idle/inactive UEs | TRS eceiving for idle/inactive Ues 1. Support reading TRS configuration from SIB 2. Support eceiving L1 indication for TRS availability FFS whether to separate the capability for receiving L1 indication for TRS availability |  | N |  | Lose of power saving gain on AGC, time/frequency tracking in idle/inactive mode | Per UE | N | N | N |  | Optional without capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Note that yellow highlight means FFS and to be discussed further. These parts are provides as placeholders.

This paper further discusses the UE features of UE power saving enhancements for NR.

# UE Features of UE Power Saving Enhancements for NR

In [1], the objectives of UE power saving enhancements are as follows,

- Specify enhancements for idle/inactive-mode UE power saving, considering system performance aspects [RAN2, RAN1]

  - Study and specify paging enhancement(s) to reduce unnecessary UE paging receptions, subject to no impact to legacy UEs [RAN2, RAN1]

- NOTE: RAN1 to check and update, if needed, evaluation methodology in RAN1 #102-e meeting

  - Specify means to provide potential TRS/CSI-RS occasion(s) available in connected mode to idle/inactive-mode UEs, minimizing system overhead impact [RAN1]

- NOTE: Always-on TRS/CSI-RS transmission by gNodeB is not required

- Study and specify, if agreed, enhancements on power saving techniques for connected-mode UE, subject to minimized system performance impact [RAN1, RAN4]

  - Study and specify, if agreed, extension(s) to Rel-16 DCI-based power saving adaptation during DRX Active Time for an active BWP, including PDCCH monitoring reduction when C-DRX is configured [RAN1]

- NOTE: Rel-15 and Rel-16 available power saving solutions should be supported by the UE and included in the evaluation. RAN1 will ask the confirmation from RAN2 that Rel-15 and Rel-16 available power saving solutions are properly utilized.

  - Study the feasibility and performance impact of relaxing UE measurements for RLM and/or BFD, particularly for low mobility UE with short DRX periodicity/cycle, and specify, if agreed, relaxation in the corresponding requirements [RAN4]

- NOTE: Supplementary RAN2 work, if needed, can be triggered by RAN4 LS

The UE feature of UE power saving enhancement for NR includes paging enhancement for IDLE/Inactive UEs, PDCCH monitoring adaptation for CONNECTED mode UEs, and RLM measurement relaxation.   The UE features for CONNECTED mode UEs would be critical to the network configuration and gNB scheduling since network will receive the feedback of UE capability to indicate whether UE supports the UE features.   However, network might not know whether IDLE/Inactive UE supports the IDLE/Inactive UE features since the UE capability inquiry by network and UE response through RRC signaling only when UE is in RRC_CONNECTED mode.   The UE feature for IDLE/Inactive mode UE should be designed as the feature indication is transparent to the network since the UE capability of UE support of this feature would not be completely known by the network.  Thus, UE features of power saving enhancement for IDLE/Inactive UEs should be “optional without capability signaling” since these features should be optional and capability would not be conveyed to the network by IDLE/Inactive UEs.

Proposal 1:  UE features of power saving enhancement for IDLE/Inactive UEs should be optional without capability signaling

For objective of NR enhancements for IDLE/Inactive UE power saving, the paging subgrouping and PDCCH-based PEI are supported for reducing the unnecessary paging reception.  The paging subgrouping was assigned by the CORE network through NAS signaling or derived from UE ID for randomization as agreed in RAN2.   It was agreed in RAN1#106b-e that paging subgroup is indicated by PEI only.   The configuration of PDCCH-based PEI and monitoring occasions for paging subgroup indication needs to be broadcasted by RRC and/or NAS signaling to IDLE/Inactive UEs, The UE capability of paging enhancement should include the UE support of both paging subgrouping and PDCCH-based PEI.   The configuration of PDCCH-based PEI and the contents in the new DCI formats for PEI would be specified with broadcast parameters and derived by IDLE/Inactive UEs regardless UE capability in support of paging subgrouping for decoding L1 signaling in the PEI or paging DCI is fed back to the network.

Proposal 2: UE capability of paging enhancement for IDLE/Inactive UE power saving should be based on the support of both PDCCH-based PEI with new DCI format and paging subgroup indication.

For IDLE/Inactive UE power saving by the additional TRS/CSI-RS configuration, SIB-X is used to broadcast the configuration of TRS/CSI-RS resource with L1 signaling for the indication of TRS availability dynamically.  The UE capability of TRS should be the UE obtaining the TRS configuration from the SIB and the L1 signaling from DCI formats in either Paging DCI or PDCCH-based PEI.   Since TRS/CSI-RS would not persistently available with availability indicated by L1 signaling dynamically, it is UE implementation whether to use TRS/CSI-RS for AGC or channel tracking even it is configured and available.  Thus, it would not have any consequence of not able to perform AGC and channel tracking since TRS/CSI-RS is not a reliable source for channel tracking due to its nonpersistent existence.   The consequence of UE not supporting this feature should be potential loss of power saving gain with TRS/CSI-RS.

Proposal 3:  The UE capability of TRS should be the UE obtaining the TRS configuration from the SIB and the L1 signaling from DCI formats in either Paging DCI or PEI.  Consequence if the feature is not supported by the UE IDLE/Inactive should only be “UE would not use the configured TRS resource to achieve power saving”.

The objective of CONNECTED UE power saving with reducing PDCCH monitoring reduction is achieved by dynamic adaptation of PDCCH monitoring interval.  It was agreed in RAN1#106-e that up to 2 bits are include in the scheduling DCI format 1_1, 1_2, 0_1, and 0_2 to indicate PDCCH monitoring adaptation.     The UE feature of PDCCH adaptation discussion in RAN1#106b-e would like to partition the PDCCH adaptation to 4 different UE feature subgroups with PDCCH skipping, SSSG switching with 2 SSSGs, and SSSG switching with 3 SSSGs, and PDCCH skipping and 2 SSSG switching.   If UE supports SSSG switching, it is not necessary to partition the support of SSSG switching with 2 SSSGs or 3SSSGs to 2 different feature subgroups.   Thus, the UE feature is for UE to support the configured bits of PDCCH monitoring adaptation in the scheduling DCI.   It was also agreed in RAN1#105-e that SSSG switching is supported by scheduling DCI.   The UE feature for PDCCH monitoring adaptation would also include the indication from scheduling DCI for SSSG switching.

Proposal 4:  The UE capability of PDCCH monitoring adaptation for CONNECTED mode UE is to indicate the support of up to 2-bit indication in the scheduling DCI formats 1_1, 1_2, 0_1 and 0_2 for PDCCH skipping, SSSG switching up to 3 SSSGs and combined PDCCH skipping and SSSG switching of SSSGs.

The UE capability for UE power saving enhancements is proposed in Table 1

Table 1:  UE capability for UE power saving enhancements

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (Sidelink WI only)”. | Consequence if the feature is not supported by the UE | Type 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory / Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR_UE_ pow_sav_enh | 29-1 | Paging enhancement | Paging subgroup indication for IDLE/Inactive UE 1. Support of paging subgroup configuration. 2, Support of L1 signaling of paging subgroup indication 3. Support of new DCI format for paging subgroup indication (PEI) 4. Support of PEI monitoring occasion(s) |  | N |  | IDLE/Inactive UE follows legacy paging procedure at each Paging Occasion. | Per UE | N | N | N |  | Optional without capability signaling |
| NR_UE_ pow_sav_enh | 29-2 | TRS for IDLE/Inactive UEs | TRS resource configuration for IDLE/Inactive UEs - Support of SIB decoding for the configuration of TRS resource and L1 signaling for availability indication - Support paging DCI and new DCI format for PEI with additional bit for TRS availability indication |  | N |  | IDLE/Inactive UE would not use the configured TRS resource to achieve power saving | Per UE | N | N | N |  | Optional without capability signaling |
| NR_UE_ pow_sav_enh | 29-3a | PDCCH monitoring adaptation | Support of up to 2-bit indication of PDCCH skipping by scheduling DCI format 1_1, 1_2, 0_1 and 0_2 |  | Y |  | UE could not reduce PDCCH monitoring configured by the given search space. | Per UE | N | N | N |  | Optional with capability signaling |
| NR_UE_ pow_sav_enh | 29-3b | PDCCH monitoring adaptation | Support of up to 2-bit indication of SSSG switching up to 3 SSSGs by scheduling DCI format 1_1, 1_2, 0_1, and 0_2 |  | Y |  | UE could not reduce the PDCCH monitoring with the SSSG switching | Per UE | N | N | N |  | Optional with capability signaling |
| NR_UE_ pow_sav_enh | 29-3 | PDCCH monitoring adaptation | Support of 2-bit indication of PDCCH skipping and SSSG switching of 2 SSSGs by scheduling DCI format 1_1, 1_2, 0_1 and 0_2 |  | Y |  | UE could not reduce the PDCCH monitoring by PDCCH skipping and/or SSSG switching | Per UE | N | N | N |  | Optional with capability signaling |

# Conclusion

In this contribution, we have discussed the UE features for UE power saving enhancements for NR.  We have the following proposals

- Proposal 1:  UE features of power saving enhancement for IDLE/Inactive UEs should be optional without capability signaling

- Proposal 2: UE capability of paging enhancement for IDLE/Inactive UE power saving should be based on the support of both PDCCH-based PEI with new DCI format and paging subgroup indication.

- Proposal 3:  The UE capability of TRS should be the UE decoding of the TRS configuration in the SIB and the L1 signaling from DCI formats in either Paging DCI or PEI.  Consequence if the feature is not supported by the UE IDLE/Inactive should only be “UE would not use the configured TRS resource to achieve power saving”.

- Proposal 4:  The UE capability of PDCCH monitoring adaptation for CONNECTED mode UE is to indicate the support of up to 2-bit indication in the scheduling DCI formats 1_1, 1_2, 0_1 and 0_2 for PDCCH skipping, SSSG switching up to 3 SSSGs and combined PDCCH skipping and SSSG switching of SSSGs.

We proposed to use Table 1for the UE feature for UE power saving enhancements for NR.

# References

- RP-200938, “UE Power Saving Enhancements for NR”, MediaTek