3GPP TSG RAN WG1 #107-e 	                                                         	   R1-2111342

e-Meeting, Nov. 11th – 19th, 2021

Source:	OPPO

Title:	Enhancements for unlicensed band URLLC/IIoT

Agenda Item:	8.3.2

Document for:	Discussion and Decision

Introduction

In RAN1 #106b e-meeting, some agreements were achieved. In this contribution, we show our views on the enhancements on Rel-17 unlicensed band URLLC/IIoT[1].

Agreement

In semi-static channel access mode, for PUSCH repetition Type B, orphan symbol(s) are dropped as in Rel-16

Agreement

In semi-static channel access mode, the configuration of energy detection threshold to perform sensing at UE is based on maxEnergyDetectionThreshold.

- That means that in semi-static channel access mode, configuration of ul-toDL-COT-SharingED-Threshold is not applicable.

- As the consequence, energy detection threshold to perform sensing at UE is based on maxEnergyDetectionThreshold if maxEnergyDetectionThreshold is configured. Otherwise (i.e., if maxEnergyDetectionThreshold is not configured), energy detection threshold to perform sensing at UE is based on the UE maximum transmit power.

Agreement

Support configuration of harq-ProcID-Offset2 for operation in unlicensed spectrum when the cg-RetransmissionTimer-r16 is not configured.

Agreement

The following RRC parameters are NOT needed when cg-RetransmissionTimer is configured for CG operation with shared spectrum channel access.

- pusch-RepTypeIndicator

- startingFromRV0

Agreement

The RRC parameter of phy-PriorityIndex is applicable for CG operation in unlicensed band.

Agreement

Introduce new RRC parameters ul-AccessConfigListDCI-0-2 and ul-AccessConfigListDCI-1-2 to support indication of CP extension, LBT type, and CAPC with DCI 0_2 and 1_2 with dynamic channel access.

Agreement

In semi-static channel access mode, a DL transmission burst based on sharing of a UE initiated COT corresponding to a UE FFP, shall include scheduled DL transmission or a DCI intended for the UE that initiated that FFP.

- A DL transmission to any other UE in the cell than the COT initiating UE and/or a broadcast transmission can be additionally included in the DL transmission burst if the gNB fulfils the following condition:

- It is gNB‘s responsibility to ensure that other UEs do not assume gNB-initiated COT based transmission for a UL transmission based on the detection of any transmission in the DL transmission burst.

Agreement

In semi-static channel access mode, for PUSCH repetition Type B: If a nominal repetition overlaps with a set of symbols in an idle period associated to gNB’s FFP in case UE shares gNB-initiated COT for the nominal repetition or associated to UE’s FFP in case UE assumes UE-initiated COT for the nominal repetition, all the symbols in the idle period should be considered as invalid symbols which are not considered for an actual repetition as in Rel-16.

- Segmentation before and/or after the idle period is applied when applicable.

- FFS on impact of processing timeline for PUSCH on the UE behaviour

Agreement

In semi-static channel access mode for a UE which is allowed to operate as an initiating device, CG-StartingOffsets is not applicable.

- Note: That is, CG-StaringOffsets is not applicable at all for a UE configured with UE FFP parameters (e.g. period, offset) regardless whether the UE would initiate its own COT or would share gNB’s COT.

Agreement

When performing Intra-UE multiplexing procedure, if a PUCCH with HARQ-ACK overlaps with a CG-PUSCH and the cg-RetransmissionTimer is configured:

- If the HARQ-ACK and the CG-PUSCH have the same priority and the CG-PUSCH is selected for HARQ-ACK multiplexing:

- If cg-UCI-Multiplexing is enabled for that CG-PUSCH, HARQ-ACK would be multiplexed in CG-PUSCH.

- Otherwise, CG-PUSCH would be dropped.

- If the HARQ-ACK and the CG-PUSCH have different priority and the CG-PUSCH is selected for HARQ-ACK multiplexing:

- If multiplexing HARQ-ACK on the CG-PUSCH with different priroity is not indicated,

- The LP channel between PUCCH or CG-PUSCH would be dropped as in Rel-16.

- If multiplexing HARQ-ACK on the CG-PUSCH with different priroity is indicated,

- If cg-UCI-Multiplexing is enabled for that CG-PUSCH, HARQ-ACK would be multiplexed in CG-PUSCH.

- Otherwise, the LP channel would be dropped.

Discussion

Harmonizing configured grant

cg-RetransmissionTimer Configuration

At least for FBE, configuration of (cg-RetransmissionTimer) should not be mandated when configured grant Type 1 or Type 2 are configured on unlicensed spectrum. And cg-RetransmissionTimer is a configured grant specific parameter and can be configured for each configured grant independently. For multiple configured grant cases, parts of configured grants can be configured with cg-RetransmissionTimer and parts of configured grants cannot be configured with cg-RetransmissionTimer.

Proposal 1:  cg-RetransmissionTimer can be configured for each configured grant independently.

Channel access related aspects

In RAN#106e meeting the following agreement was drawn for dynamic scheduled PUSCH [2].

Agreement

In semi-static channel access mode, the content of the channel access field in a DCI scheduling a UL transmission for a UE determines an index to a row in Table 1 with Alt-1 (Option 1)

TABLE 1

| Bit field mapped to index | Channel Access Type | The CP extension T_"ext" index defined in Clause 5.3.1 of [4, TS 38.211] | Initiator of a channel occupancy associated to UL transmission described in Clause x.x in TS 37.213 |
| --- | --- | --- | --- |
| 0 | No sensing as defined in Clause 4.3 in TS 37.213 | 0 | Alt-1:gNB Alt-2: UE-initiated COT if condition A, otherwise gNB’s COT |
| 1 | No sensing as defined in Clause 4.3 in TS 37.213 | 2 | Alt-1:gNB Alt-2: UE-initiated COT if condition A, otherwise gNB’s COT |
| 2 | 9us sensing within a 25us interval as defined in Clause 4.3 in TS 37.213 | 0 | gNB |
| 3 | 9us sensing [within a 25us interval] as defined in Clause 4.3 in TS 37.213 | 0 | UE |

Note: The last row in Table 1 is only applicable when the UE can operate as an initiating device as configured by gNB.

For Alt-1, it is clear and there is no ambiguity based on gNB and UE. While Alt-2, the UE will first check the condition A, which depends on whether the UE has obtained already an UE initiated COT. This determination may trigger further discussions on whether gNB and UE may obtain different observations or draw different conclusions. If so, how to resolve this mis-understanding. We think that such discussions need to be avoided and a simple Alt-1 solution can be adopted.

Proposal 2: For the DCI content to determine gNB CO or UE CO, adopt Alt-1.

Another leftover issue is how a UE determines if there is a gNB initiated CO. In RAN1#105-e meeting, the following agreement was drawn for CG-based UL transmission. Thus, the UE will have to determine whether there is an gNB initiated CO. In legacy specification, the UE determines it by detecting any DL transmission in the gNB FFP. However, this may not work properly in R17 as the DL transmission may also occur in UE initiated CO.  A straightforward solution is to explicitly broadcast a signal to inform that the gNB has initiated a CO.

- Alt-a is taken in the following agreement:

Agreement:

In semi-static channel access mode when a UE can operate as UE-initiated COT,

- Select one of the following alternatives to determine whether a configured UL transmission that is aligned with a UE FFP boundary and ends before the idle period of that UE FFP, is based on UE-initiated COT or sharing a gNB-initiated COT:

  - Alt-a: If the transmission is confined within a gNB FFP before the idle period of that gNB FFP, and the UE has already determined that gNB is initiated that gNB FFP, UE assumes that the configured UL transmission corresponds to gNB-initiated COT. Otherwise, UE assumes that the configured UL transmission corresponds to UE-initiated COT

  - Alt-b: The UE assumes that the configured UL transmission corresponds to UE-initiated COT..

Proposal 3: gNB can explicitly broadcast a signal to inform the UEs that the gNB initiated CO has been created.

Conclusions

In this contribution, we show our views on the enhancements on unlicensed band URLLC/IIoT with following proposals:

Proposal 1:  cg-RetransmissionTimer can be configured for each configured grant independently.

Proposal 2: For the DCI content to determine gNB CO or UE CO, adopt Alt-1.

proposal 3: gNB can explicitly broadcast a signal to inform the UEs that the gNB initiated CO has been created.

References

- RAN1#106b-e chairman notes

- RAN1#106-e chairman notes