3GPP TSG RAN WG1#107e      	       	                                R1-2111388    E-meeting November 11th-19th, 2021

Agenda Item:					    8.2.4

Source:	Sony

Title:	Beam management enhancement for NR from 52.6GHz to 71GHz

Document for:	Discussion

# Introduction

In RANP#90e, the WID of from 52.6GHz to 71GHz frequency has been established in [1] and the potential beam management enhancement was captured as below under RAN1’s working scope

According to the outcome of the study item on Supporting NR above 52.6GHz and leveraging FR2 design to the extent possible, this WI extends NR operation up to 71GHz considering, both, licensed and unlicensed operation, with the following objectives: - Physical layer aspects including [RAN1]: - Specify timing associated with beam-based operation to new SCS (i.e., 480kHz and/or 960kHz), study, and specify if needed, potential enhancement for shared spectrum operation - Study which beam management will be used as a basis: R15/16 or R17 in RAN #91-e

Moreover, during previous RAN1 e-meeting(s), an agreement on beam management was achieved as well

Agreement: Consider at least the following aspects in system operations with beams - Study of BFR mechanism enhancements, if supported - e.g., the use of aperiodic CSI-RS for BFR, increased number of RSs for monitoring/candidates and efficient utilization of the increased number of RSs, enhanced reliability to cope with narrower beamwidth - Study of UE capabilities on beam switch timing in beam management procedure - Study of enhancements for beam management and corresponding RS(s) in DL and UL are needed further considering at least the following aspects, if supported: - beam switching time, beam alignment delay (including initial access), LBT failure, and potential coverage loss (if large SCS is supported) - Study of beam switching gap handling for signals/channels (e.g. CSI-RS, PDSCH, SRS, PUSCH) for higher subcarriers spacing, if supported

In this contribution, we would like to present our view on beam management enhancement for NR operating in the frequency band of 52.6-71GHz.

# Beam management enhancement

During RANP#90e, companies noticed that there is ongoing enhancement for multi-beam operation under Rel.17 FeMIMO and discussed on which release i.e. Rel.15/16 or Rel.17 of beam management should be the baseline of Rel.17 beam management enhancement for above 52.6GHz. Fortunately, in RAN1#104e, it was agreed that both Rel.15/16 and Rel.17 can be considered as baseline for 52.6-71GHz. Since Rel.17 FeMIMO is now an ongoing topic, which feature(s) will be finally specified is not fixed yet. Therefore, more discussion will be needed to check whether such features specified in Rel.17 FeMIMO beam management are applicable to the frequency band of 52.6-71GHz.

## Beam failure recovery based on aperiodic CSI-RS

In Rel.15 and 16, the procedures of SpCell and SCell BFR were specified based on different mechanisms respectively. Particularly, the DL RS in set q0 (for beam failure detection, BFD) and the DL RS in set q1 (for new candidate beam determination, CBD) have been specified as periodic RS, i.e. SSB or periodic CSI-RS. With such restrictive RS settings, UE could periodically measure and evaluate the link quality and determine whether to send beam failure recovery request (BFRQ) to NW when all monitored links fail in the metric of PDCCH hypothetical BLER.

However, when operating at unlicensed spectrum, NW may not be allowed to transmit periodic BFR RS (including both BFD RS and CBD RS) for every periodic occasion, due to channel access mechanism, e.g. LBT failure if any. In such a case, UE would have no chance to measure the DL quality associated with the monitored CORESET(s) in a timely manner and thus may not be able to measure the quality of CBD RS neither. Without indicated COT, the UE might do nothing for DL reception during a period when beam failure may happen.

For more details, we depict the scenario of Rel.15/16 BFD in Figure 1. Specifically, a UE could only measure periodic RS, e.g. CSI-RS for BFD and/or CBD, when there is indicated COT from NW. If there is no COT covering the 2nd P-CSI-RS, the UE knows the situation that NW cannot transmit periodic CSI-RS due to LBT failure or any other issue, but anyway it misses an occasion to measure the P-CSI-RS for DL quality assessment.

- : Depending on periodic BFD RS and CBD RS, UE would not be able to evaluate DL quality at the periodic occasion(s), due to no indicated COT.

- : Illustration of BFD and CBD by using both periodic and aperiodic CSI-RS

To address such issue, only simple standard effort is needed that aperiodic CSI-RS dynamically triggered by NW could be considered as a supplementary tool for BFR. For instance, when NW finds no COT to send periodic BFR RS or CBD RS on a periodic occasion, it may find the next available COT to trigger aperiodic CSI-RS for UE to carry out DL measurement. Note that the aperiodic CSI-RS triggered by DCI should have the same QCL-TypeD with the DMRS of the monitored CORESET. To evaluate the performance of beam pair link(s), the UE needs to merge the result of periodic BFR measurement with that of aperiodic BFR measurement, if aperiodic CSI-RS is dynamically triggered. Hence, we have

- : Support aperiodic CSI-RS for beam failure detection (BFD) and candidate beam determination (CBD) at least for unlicensed band operation.

## Beam management for multiple scheduled PDSCH/PUSCH

In RAN1#104e session 8.2.5, RAN1 agreed to support the function that single DCI schedules multiple PDSCH/PUSCH. Afterwards corresponding default QCL assumption and multiple beam issues were introduced and discussed in session 8.2.4 accordingly. In this section, we would like to present our views on related issues.

### Multi-beam operation for M-TRP

Agreement: For multiple PDSCHs/PUSCHs scheduled by a single DCI, at least for single TRP, support indication of only a single TCI state/SRI in DCI - FFS: number of TCI states/SRIs in a single DCI scheduling multiple PDSCHs/PUSCHs for multi-TRP Working assumption: For multi-PDSCH scheduling for multi-TRPs, support a single DCI field ‘Transmission Configuration Indication’ as in Rel-16 TCI state indication mechanism for multi-TRPs - The single DCI field ‘Transmission Configuration Indication’ indicates one or two TCI states associated with a code point for single DCI based multi-TRP mechanism - The single DCI field ‘Transmission Configuration Indication’ indicates only one TCI state associated with a code point for multi-DCI based multi-TRP mechanism - Reuse Rel-16 RRC configuration and MAC CE activation/deactivation methods for the one or two TCI states - FFS: Details of multiple TCI state association with multiple PDSCHs

In RAN1#104bis-e, RAN1 agreed to only support single beam operation for single DCI scheduled PDSCH/PUSCH under single TRP scenario. Following the same mechanism, we think it can be extended to multi-TRP operation with each beam from/to each TRP.

In RAN1#106e, the working assumption on S-DCI based multi-PDSCH was achieved by reusing Rel.16 multi-PDSCH beam indication mechanism. In fact, there is parallel discussion in session 8.1.1 for multi-beam operation using unified TCI state (defined in Rel.17). Specifically, whether M > 1 DL TCI states and/or N > 1 UL TCI states has been concluded as not supported in Rel.17. A few of companies believe that M = 2 DL TCI states and/or N = 2 UL TCI states for two TRPs are the only use cases, where each TCI state is from/to each TRP.

Given above information with multi-PDSCH settled, we would like to propose for the multi-PUSCH as

- : For single-DCI scheduled multi-PUSCH for multi-TRP, support up to 2 SRIs and each SRI is pointed to each TRP.

### Default beam operation

Since single-DCI scheduled multi-PDSCH was supported in RAN1#104e, two default beam issues raised which was captured in following agreement. To clearly illustrate the scenario, we have depicted Figure 2 and Figure 3.

Agreement: Further study the following: - For multi-PDSCH scheduling with a single DCI, study the QCL assumption(s) the UE should apply for each PDSCH for the case when some of the scheduled PDSCHs have scheduling offset less than timeDurationForQCL while some have scheduling offset equal to or greater than timeDurationForQCL. - For multi-PDSCH scheduling with a single DCI, study the QCL assumption(s) the UE should apply for each PDSCH for the case when all of the scheduled PDSCHs have scheduling offset less than timeDurationForQCL - Note: If the current Rel-16 behavior would be extended to multiple-PDSCH scheduling, it could result in a different QCL assumption for each PDSCH due to the fact the that the CORESET with the lowest ID can be different for different slots, resulting in a potentially different TCI state for each slot Note: Applicability to multi-TRP can be discussed further Agreement: For the single TRP case, for multi-PDSCHs scheduled by a single DCI with a single DCI field ‘Transmission Configuration Indication’ that indicates a single TCI state (if the DCI field is present), - Case 1: PDSCH scheduling offset for all PDSCHs ≥ timeDurationForQCL - Case 1-1: tci-PresentInDCI enabled - Single QCL assumption based on the indicated codepoint of the single DCI field ‘Transmission Configuration Indication’ is applied for all scheduled PDSCHs - Case 1-2: tci-PresentInDCI not present - Single QCL assumption of the single scheduling DCI scheduled multi-PDSCHs is applied for all scheduled PDSCHs - Case 2: PDSCH scheduling offset for any scheduled PDSCH < timeDurationForQCL - Down select one of the following alternatives - Alt 1: Single QCL assumption is applied for all scheduled PDSCHs - FFS: Details of single QCL assumption - Alt 2: multiple QCL assumptions are applied - FFS: Details of multiple QCL assumptions - FFS: When some of PDSCHs are collided with semi-static UL symbols and then skipped - FFS: The multi-TRP case

In Figure 2, some PDSCH are scheduled with time gap between scheduling DCI and PDSCH smaller than timeDurationForQCL. Considering UE capability on decoding PDCCH and then preparing indicated Rx beam, the default beam should be applied at UE side, i.e. the CORESET with lowest ID in the latest slot the UE monitored. But for the other PDSCH which are scheduled later than the default beam threshold, there are two options on the table. First, it is to apply indicated Rx beam but problem is that UE has to switch Rx beam in a CP duration. It might be infeasible for some UE implementation. Hence, we have

- : default beam operation when some PDSCH are earlier than timeDurationForQCL whereas other PDSCH is later than the default beam threshold

When all scheduled PDSCH are within timeDurationForQCL, the default beam may vary from slot to slot, since a UE always has to follow the default beam from the lowest CORESET ID in most recent slot. To avoid such case, we have similar thought.

- : default beam operation when all scheduled PDSCH are within the default beam threshold

- : For the case of single DCI scheduled multiple PDSCH of single-TRP, when any of the scheduled PDSCHs < timeDurationForQCL, UE applies the same default Rx beam from the 1st PDSCH to the last PDSCH.

## Aperiodic RS triggered across multiple slots

As single-DCI scheduled multiple PDSCH/PUSCH has been agreed in RAN1#104e, the DL overhead in control channel can be greatly reduced. When it comes to aperiodic CSI-RS in DL and aperiodic SRS in UL, it is a nature question whether single-DCI triggered AP-CSI-RS or AP-SRS across multiple slots are valuable or not to the newly introduced SCS.

Let’s take the DL beam sweeping procedure via aperiodic CSI-RS as an example. As for P1/P3 DL beam sweeping procedure, a UE has to retune multiple times its Rx beams in a slot. From implementation aspects of UE Rx beam retuning, it normally costs the order of 100ns for UE to retune an analog beam in RF. Assuming AP-CSI-RS resource set with Repetition ON, a UE needs to refine its Rx beams within a slot (very short duration) multiple times. For such hash requirement, some UE implementation might be infeasible at all. Hence for SCS-480kHz or SCS-960kHz, it is better to allow more time for UE to retune its Rx beams, e.g. Rx beam change on a per slot level, rather than per symbols level.

In Rel.15/16, multiple AP-CSI-RS can be scheduled across multiple slots. For better illustration, we capture it in Figure 4. Specifically, single DCI could only dynamically indicate one TriggerState which may contain multiple CSI-ReportConfigIDs. Each CSI-ReportConfigID at least associates with one ResourceSetting for UE to carry out DL measurement and the UE reports according to the CSI-ReportConfig. For one beam reporting instance, multiple CSI-ReportConfig are associated which seems undesirable from overhead perspective.

- : Rel.15/16 aperiodic CSI-RS triggering

As for UL aperiodic SRS, the SRS request field in either DCI format 0_1 or 1_1 could only trigger one SRS resource set within one slot. As for UL beam sweeping, i.e. SRS resource set configured with usage as ‘Beam Management’, please see Figure 5 where a UE needs to prepare multiple SRS resources with spatial relations within a slot. If high SCS is configured for the UL BWP, it might be infeasible for UE to carry out such UL beam sweeping.

- : Rel.15/16 aperiodic SRS triggering

- : Study and specify if needed single DCI scheduled multiple aperiodic CSI-RS and/or aperiodic SRS across multiple slots.

## Beam alignment during initial access

For the operation above 52.6 GHz, the use of narrower beams, even during initial access, would be beneficial in order to overcome extremely high propagation loss. However, the connection tends to be lost when using SSB with narrower beams, due to the change of environment or movement of the UE. In addition, enhancement to more than 64 SSBs would have a large specification impact.

Even in legacy NR, narrower beams are provided by CSI-RS. Therefore, narrower beams using CSI-RS during initial access could be considered. Figure 5 shows beam refinement using CSI-RS. At first, the CSI-RS configuration is broadcast via SIB1. From the CSI-RS configuration, the UE measures the best narrower beam among the CSI-RS set that is QCLed with the best SSB. Finally, the UE can report the best CSI-RS information in Msg3. From Msg4 transmission, the gNB can use the reported narrower beam to the UE. This would contribute to early and fine beam alignment.

- : Beam refinement using CSI-RS

- : Beam alignment during initial access procedure should be considered for NR above 52.6 GHz.

# Conclusions

Finally, allow us to repeat our proposals to draw attention.

- : Support aperiodic CSI-RS for beam failure detection (BFD) and candidate beam determination (CBD) at least for unlicensed band operation.

- : For single-DCI scheduled multi-PUSCH for multi-TRP, support up to 2 SRIs and each SRI is pointed to each TRP.

- : For the case of single DCI scheduled multiple PDSCH of single-TRP, when any of the scheduled PDSCHs < timeDurationForQCL, UE applies the same default Rx beam from the 1st PDSCH to the last PDSCH.

- : Study and specify if needed single DCI scheduled multiple aperiodic CSI-RS and/or aperiodic SRS across multiple slots.

- : Beam alignment during initial access procedure should be considered for NR above 52.6 GHz.

# Reference

- RP-202925: Revised WID: Extending current NR operation to 71 GHz

- 3GPP TS 38.214, “Physical layer procedures for data”