3GPP TSG RAN WG1 Meeting #107-e                                                                 	 R1-2111395

E-meeting, November 11th – 19th, 2021

Source:	Sony

Title:	Enhancements on HARQ for NTN

Agenda Item:	8.4.3

Document for: 	Discussion

- Introduction

In RAN#86, a new WI “Solutions for NR to support non-terrestrial networks (NTN)” was approved [1]. The work item aims to specify the enhancements identified for NR NTN (non-terrestrial networks) especially LEO and GEO with implicit compatibility to support HAPS (high altitude platform station) and ATG (air to ground) scenarios. For RAN1, the detailed objectives are to specify enhancing features to Rel-15 & Rel-16’s NR radio interface as follows [1]:

4.1.1 RAN1 Enhancing features to address the identified issues due to long propagation delays, large Doppler effects, and moving cells in NTN, the following should be specified (see TR 38.821): - Timing relationship enhancements[RAN1,RAN2] - Enhancements on UL time and frequency synchronization [RAN1,RAN2] - HARQ - Number of HARQ process [RAN1] - Enabling / disabling of HARQ feedback as described in the TR 38.821 [RAN1&2] In addition, the following topics should be specified if beneficial and needed - Enhancement on the PRACH sequence and/or format and extension of the ra-ResponseWindow duration (in the case of UE with GNSS capability but without pre-compensation of timing and frequency offset capabilities) [RAN1/2]. - Feeder link switch [RAN2,RAN1] - Beam management and Bandwidth Parts (BWP) operation for NTN with frequency reuse [RAN1/2] - Including signalling of polarization mode

In this contribution, we share our considerations on enhancement of HARQ for NTN.

- Support of enabling / disabling of HARQ feedback

In this section, we discuss the impact of  disabling  HARQ feedback.

2.1 Restriction on HARQ feedback disabling

In NR, a 3ms delay after ACK feedback is allowed for MAC CE activation/deactivation timing in order to synchronize the action between gNB and UE. There is an issue that the gNB would not know whether the MAC CE is received correctly by UE when HARQ feedback is disabled. Important MAC CEs such as SP CSI reporting on PUCCH activation/deactivation by MAC CE, SP SRS activation/deactivation by MAC CE and SCell activation/deactivation by MAC CEs may cause continuous errors if they are missing or decoded incorrectly.

In that sense, it is beneficial to enable HARQ feedback for at least one HARQ process so that the reliability of MAC CEs can be ensured by transmitting them using the HARQ process whose HARQ feedback is enabled.

Proposal 1: UE expects that any PDSCH carrying a MAC CE command, whose activation/deactivation time is coupled to the transmission time of the associated HARQ-ACK, is scheduled via a HARQ process with HARQ feedback enabled.

2.2 Impact on HARQ codebook

For dynamic scheduling, the HARQ process ID is included in the DCI, for DL semi-persistent scheduling the HARQ process ID is derived from the DCI scrambled by CS-RNTI. In that sense, the UE does not know whether the HARQ feedback is enabled or disabled for a given PDSCH until its scheduling or activation DCI is received for either dynamic PDSCH or SPS PDSCH. As the enabling / disabling of HARQ feedback for downlink transmission is configured on a per HARQ process basis via RRC signalling and the existing HARQ codebooks are designed based on the principle that all PDSCHs need HARQ-ACK feedback, it is necessary to identify enhancements for HARQ-ACK codebook construction when feedback for some HARQ processes is disabled. In RAN1#105e and 106e meeting, the following agreements were achieved for HARQ codebook enhancement,

Agreement: - For Type-1 HARQ codebook, if DCIs carrying the feedback-disabled and feedback-enabled HARQ processes are detected by UE, one of following options should be supported: - Option-1: The UE will report NACK only for the feedback-disabled HARQ process regardless of decoding results of corresponding PDSCH - Option-2: The UE will report NACK/ACK for the feedback-disabled HARQ process depending on the decoding results of corresponding PDSCH - FFS: Other cases, e.g., if only DCI carrying feedback-disabled HARQ process is detected by UE Agreement: For Type-1 HARQ codebook, if only DCI carrying feedback-disabled HARQ process is detected by UE, one of following options should be supported: - Option-1: The UE’s behavior is same as the case if DCIs carrying the feedback-disabled and feedback-enabled HARQ processes are detected by UE - Option-2: The UE should skip the codebook feedback at least when the feedback is carried by PUCCH - FFS: the case that feedback is carried by PUSCH. Agreement: For Type-2 HARQ codebook in NTN, - For the DCI of PDSCH with feedback-enabled HARQ processes, the C-DAI and T-DAI are the count of only feedback-enabled processes - FFS: Whether DCI for SPS release and any other DCIs are included in counting of C-DAI and T-DAI Agreement: For the DCI of PDSCH with feedback-disabled HARQ processes, only one of following is supported for Type-2 codebook: - Option-1: The C-DAI and T-DAI are the count of feedback-enabled processes, despite they are not incremented, and are taken into account by the UE for type 2 codebook generation. - Option-2: The C-DAI and T-DAI are ignored by the UE regardless of the value for Type 2 codebook generation.

In the following part, we will discuss each solution for necessary enhancement for HARQ-ACK codebook construction.

Type 1 HARQ codebook

Type-1 HARQ-ACK codebook/semi-static HARQ-ACK codebook can provide robustness against missed DCI transmissions, whose length depends on the RRC configured parameters such as TDRA table, serving cell number and K1 value/dl_DataToUL-ACK. On current codebook design rules, the number of redundant feedback bits of the semi-static HARQ-ACK codebook would be too large when some of the HARQ processes are feedback disabled. This is because the UE assumes that all PDSCH occasions, i.e., all HARQ processes need HARQ feedback and so constructs the codebook based on pre-configured parameters via RRC signalling. For example, if a UE is configured with 32 HARQ processes in the serving cell, then assuming HARQ process ID#1 is configured for HARQ feedback enabled and the other HARQ process IDs are configured for HARQ feedback disabled, the UE still has to reserve the ACK/NACK bits for all 32HARQ processes in the feedback occasion. When multiple serving cells are configured, the UE needs to reserve the ACK/NACK bits even though all HARQ processes are feedback disabled in one serving cell. The reserved redundancy bits represent a large overhead. To keep the semi-static feature of Type-1 HARQ codebook and reduce the codebook overhead, one possible solution is using different RRC configured TDRA tables for HARQ processes with HARQ-ACK feedback disabling and enabling. When more than 1 PDSCH can be scheduled in one slot, the UE needs to reserve the HARQ ACK/NACK bits for maximum possible PDSCH transmission occasions. As Fig.1 shows, the length of TDRA 1 is 4 symbols and the starting symbol is 3, and so on to TDRA 2, TDRA 3 and TDRA 4. UE needs to reserve 3 HARQ ACK/NACK bits for each K1 value when it configures SLIV table in Fig.1. If only HARQ processes with HARQ-ACK feedback enabled are configured with the TDRA 1 and the maximum possible PDSCH transmission number is 1, then the UE only needs to reserve 1 HARQ ACK/NACK bit for each K1 value. In the case when all the HARQ processes are feedback disabled in one serving cell, the UE does not need to reserve ACK/NACK bits for that serving cell. Note that it does not conflict with Proposal 2 that MAC CE whose activation/deactivation time is coupled to the transmission time of the associated HARQ-ACK must be scheduled with a feedback enabled HARQ process, because these MAC CEs can be scheduled with a feedback enabled HARQ process in the primary cell.

Fig.1 Example of SLIV table

Type 2 HARQ codebook

For dynamic HARQ-ACK codebook, the UE feeds back ACK/NACK based on the actual schedule condition without redundant HARQ feedback bits reservation. In DCI for DL scheduling, a value of the counter DAI field denotes the accumulative number of {serving cell, PDCCH monitoring occasion}-pair(s) and a value of the total DAI field denoting the total number of {serving cell, PDCCH monitoring occasion}-pair(s) are used to avoid DCI missed detection of dynamic scheduling and SPS release. In DCI for UL scheduling, the value of DAI in DCI format 0_1 denotes total number of {serving cell, PDCCH monitoring occasion}-pair(s) in which PDSCH transmission(s) associated with PDCCH or PDCCH indicating SPS PDSCH release or DCI format 1_1 indicating SCell dormancy is used for rate matching when HARQ-ACK information is multiplexed in the PUSCH transmission.

When some HARQ processes are feedback disabled, enhancement on DAI increment is necessary. To reduce the UE complexity, it is preferred that UE assumes the same value of counter DAI and total DAI in DCI formats including DCI of PDSCH with feedback-enabled HARQ process and DCI of PDSCH with feedback-disabled HARQ process. Therefore, for dynamic scheduling PDSCH, we propose that the counter DAI and total DAI should count only the DCI scheduling PDSCH with HARQ-ACK enabled amongst DCI of PDSCH with feedback-enabled HARQ processes and DCI of PDSCH with feedback-disabled HARQ processes, and the value of DAI in DCI format 0_1 count only DCI scheduling PDSCH with HARQ-ACK enabled. SPS PDSCH release is a special case as the HARQ process number field is invalid, which will be discussed in Section 3 in detail.

Observation 1: When HARQ feedback is disabled for some HARQ processes, the redundant feedback bits of Type-1 / semi-static HARQ-ACK codebook would be large based on current HARQ-ACK codebook design

Proposal 2: HARQ codebook enhancement is supported as:

- For Type-1 HARQ codebook, reduce codebook size :.

  - When all HARQ processes are feedback disabled for one secondary cell, the UE does not reserve HARQ ACK/NACK bits for that secondary cell.

  - Reduce the Type-1 HARQ codebook size by restricting the scheduling occasion, e.g., restricted TDRA table of feedback enabled HARQ process.

- For Type-2 HARQ codebook, the value of the counter/total DAI field in the DCI of PDSCH with feedback-disabled HARQ process are given the count value of {serving cell, PDCCH monitoring occasion}-pair(s) whose associated PDSCH is HARQ feedback enabled.

2.3 Impact of SPS PDSCH

In RAN1#103-e meeting [2] and 106b-e meeting [3], the following agreements were achieved for UE behaviour of HARQ process with feedback disabled.

Agreement: For a DL HARQ process with disabled HARQ feedback, the UE is not expected to receive another PDSCH or set of slot-aggregated PDSCH scheduled for the given HARQ process that starts until [X] after the end of the reception of the last PDSCH or slot-aggregated PDSCH for that HARQ process. - FFS: value of X and units in which it is defined. - FFS: Whether TB of the two PDSCHs needs to be different Agreement: For DCI indicating SPS PDSCH release, HARQ-ACK report is as in Rel-16.

Different from dynamic scheduling, semi-persistent scheduling does not need dynamic signalling therefore can reduce the scheduling latency especially in NTN with its large RTT. For DL semi-persistent scheduling the HARQ process ID is derived from the following formula:

HARQ Process ID = [floor (CURRENT_slot × 10 / (numberOfSlotsPerFrame × periodicity))] modulo nrofHARQ-Processes

Where CURRENT_slot=[(SFN × numberOfSlotsPerFrame) + slot number in the frame] and the nrofHARQ-Processes is the number of configured HARQ process for SPS, the SPS HARQ process ID is calculated by the PDSCH location in time domain, if two HARQ processes are configured for SPS to UE, there are three possibilities:

- Case 1: Both HARQ process are feedback enabled.

- Case 2: One HARQ process is feedback enabled, the other is feedback disabled.

- Case 3: Both HARQ process are feedback disabled.

For case 1, as the UE is not expected to receive another PDSCH for a given HARQ process with feedback enabled until after the end of the expected transmission of HARQ-ACK for that HARQ process, the gNB needs to configure large period for SPS PDSCH to compensate the large delay. When the round trip delay (RTD) between satellite and UE are changing with time, the SPS periodicity should be capable of compensating the largest RTD.

For case 2, the UE behaviour of HARQ process with feedback enabled and feedback disabled are different. The UE is not expected to receive another PDSCH for a given HARQ process with feedback enabled until after the end of the expected transmission of HARQ-ACK for that HARQ process, while the UE is not expected to receive another PDSCH for a given HARQ process with feedback disabled until after the end of the reception of the last PDSCH for that HARQ process. Therefore, the scheduling behaviour may be complicated to the UE and similar with case 1 the SPS periodicity should be large enough to compensate the RTD.

For case 3, the gNB can configure the SPS periodicity without considering the RTD as both HARQ processes are feedback disabled. However, the gNB will not receive any feedback from UE during the whole SPS procedure. In NR, whether the SPS PDSCH activation is received correctly is indicated implicitly by HARQ feedback for SPS PDSCH, while the gNB cannot know whether the activation is received based on HARQ feedback for any SPS PDSCH for which HARQ feedback is disabled. In that sense, it is beneficial for the UE to feedback for the SPS PDSCH activation to ensure the reliability of SPS PDSCH transmission. And the counter DAI and total DAI field should be incremented with PDCCH for PDSCH scheduling or PDCCH indicating SPS PDSCH release or PDCCH indicating SPS PDSCH activation for HARQ process with HARQ feedback disabling.

Proposal 3: Support UE reporting HARQ feedback information for the SPS PDSCH activation when the related SPS PDSCH is HARQ feedback disabled.

Proposal 4: The counter DAI, total DAI and DAI in DCI format 0_1 should count for PDCCH indicating SPS PDSCH activation when the related SPS PDSCH is HARQ feedback disabled.

Proposal 5: The counter DAI, total DAI and DAI in DCI format 0_1 should always count for PDCCH indicating SPS PDSCH release whether the related SPS PDSCH is HARQ feedback disabled or enabled.

- Summary

In this contribution, we provide considerations on more delay tolerant HARQ for NTN. Following is our proposal:

Proposal 1: UE expects that any PDSCH carrying a MAC CE command, whose activation/deactivation time is coupled to the transmission time of the associated HARQ-ACK, is scheduled via a HARQ process with HARQ feedback enabled.

Observation 1: When HARQ feedback is disabled for some HARQ processes, the redundant feedback bits of Type-1 / semi-static HARQ-ACK codebook would be large based on current HARQ-ACK codebook design

Proposal 2: HARQ codebook enhancement is supported as:

- For Type-1 HARQ codebook, reduce codebook size with keeping the codebook size semi-static.

  - When all HARQ processes are feedback disabled for one secondary cell, the UE does not reserve HARQ ACK/NACK bits for that secondary cell.

  - Reduce the Type-1 HARQ codebook size by restricting the scheduling occasion, e.g., restricted TDRA table of feedback enabled HARQ process.

- For Type-2 HARQ codebook, the value of the counter/total DAI field in the DCI of PDSCH with feedback-disabled HARQ process are given the count value of {serving cell, PDCCH monitoring occasion}-pair(s) whose associated PDSCH is HARQ feedback enabled.

Proposal 3: Support UE reporting HARQ feedback information for the SPS PDSCH activation when the related SPS PDSCH is HARQ feedback disabled.

Proposal 4: The counter DAI, total DAI and DAI in DCI format 0_1 should count for PDCCH indicating SPS PDSCH activation when the related SPS PDSCH is HARQ feedback disabled.

Proposal 5: The counter DAI, total DAI and DAI in DCI format 0_1 should always count for PDCCH indicating SPS PDSCH release whether the related SPS PDSCH is HARQ feedback disabled or enabled.

References

- 3GPP, RAN #86, RP-193234, “NR-NTN: Solutions for NR to support non-terrestrial networks (NTN).

- Chairman's Notes RAN1 #103-e, October-November 2020.

- Chairman's Notes RAN1 #106b-e, October 2021.