3GPP TSG-RAN WG1 Meeting #107-e	R1- 2111417

e-Meeting, November 11th – 19th, 2021

Agenda Item:	8.16.4

Source:	Ericsson

Title:	On UE features for NR NTN

Document for:	Discussion

# 1	Introduction

The work item on solutions for NR to support non-terrestrial networks (NTN) was approved at RAN#86 and the work item description is updated in [1]. An overview of 3GPP non-terrestrial networks can be found in [2] and an overview of state of the art in LEO satellite access can be found in [3].

In this contribution, we discuss the UE features for NR NTN.

# 2	Guidelines for UE capability definitions

- The discussion on defining UE features parameters for Rel-17 has now started. It is beneficial to recall RAN2’s guidelines for UE capability definitions, as documented in the RAN2 LS [4] and copied below:

1	Avoid defining “incapability” bits as they may cause interpretation issues

The definition of the capability should not say that “a UE setting the bit does not support Rel-16 feature X”. Such statements caused a lot of problems in Rel-15. One example was the pucch-F0-2WithoutFH that indicates that “the UE does not support PUCCH formats 0 and 2 without frequency hopping”.

2	Absence of a capability bit shall not imply support for something that is not yet defined

One example is “absence of channelBW means that the UE supports all BWs”. Since new BWs might be added later (and in a release independent manner) and maybe in some other specification, “all BWs” can become ambiguous. Any legacy UE would suddenly be expected to support the new bandwidth as well.

Another example is the default capability for UE power class and UL duty cycle. From signalling point of view, reducing the overhead is marginal (unless it is reported per-BandCombination or Per-Band-of-a-BandCombination as discussed in bullet 4 below). Instead, gNB faces difficulty in comprehending the implicit capabilities amongst UEs compliant with different versions in a release.

3 	Define baseline feature bits that covers the minimum parts of a feature and use additional bits for parts that can be used together with the baseline feature

If a feature has a baseline feature, the baseline feature must be clearly defined. It is not enough to write that “if the UE does not set the bit it does not support feature group x-y”.  If a capability bit for a certain “feature X” has been defined, all UEs that support “feature X” shall set the bit, and additional parts of the feature may not be supported by all UEs.

Capability signalling and rules to determine whether a UE supports a feature should not be mixed. For example, avoid to say

“In TDD band combinations, only UEs that set the featureX capability bit support feature. In FDD-only band combinations all UEs support feature X even if they do not set the bit.”

If later FDD-only band combinations are defined, where UEs are not required to support the feature, band combinations will not be release independent. An example where this caused problem in Rel-15 was the capability simultaneousRx-Tx for EN-DC.

4 	Minimize features “per-BandCombination” and “per-Band-of-a-BandCombination”

The number of features that require capability signaling with “per-BandCombination” and “per-Band-of-a-BandCombination” (FeatureSetPerCC) differentiation should be minimized. Such features cause an explosion in the size of the capability signaling and are even more severe when it comes to the number of possible configurations, making it impossible for the gNB to take all combinations into account.

It would be of help for RAN2 to be informed of the reasons why “per-BC” or “per-band-of-a-BC” was considered necessary, and whether other alternatives were considered.

5 	Avoid defining functionality that has no RRC configuration but is dependent on capability bits.

The specification should not be written so that the network determines what configuration it can use for a UE implicitly by the reported UE capabilities. Instead, the gNB should always configure the UE explicitly by DL RRC signalling, respecting  the reported capabilities.

A problematic case in Rel-15 was the UL/DL MIMO layers, which resulted in a late-stage introduction of explicit MIMO signalling support by RAN2 (maxLayersMIMO-Indication).

6	UE capabilities defined in specifications must be self-contained

RAN2 would also like to point out that once the “snapshot” of defined Rel-16 capabilities have been captured in RAN2 TR 38.822, RAN2 does not plan to update this TR. Discussions (on any WG capabilities) should be based on the actual specification text. RAN2 will document the capabilities in TS 3x.306, preferably with references to RAN1/RAN4 specifications and to corresponding RRC configurations (fields and information elements).

RAN2 understands that RAN1 uses the “feature IDs”, since they are the only available definitions during the initial stage of discussions. RAN2 would like to request that once the RRC signalling is in place and can be mapped to each “feature ID”, the actual field names used in the signalling are also utilized in the RAN1/RAN4 capability discussions (to avoid inter-WG confusion e.g. in LSs).

7	Rationale for necessity of both xDD and FRx differentiations for per-UE capability

RAN2 did not discuss the RAN1 LS on XDD-FRX Differentiation (R1-1913579/R2-2000013) at the RAN2#109-e meeting, but RAN2 would anyway appreciate to be provided with rationale in case Per-UE capability with both FDD/TDD and FR1/FR2 differentiations is deemed as necessary for a feature.

# 3	Detailed comments on the initial list of UE features for NR NTN

Our detailed comments on the initial list of UE features for NR NTN are in the table below (with Track Changes).

In particular, we propose to remove the misleading text “support mixture of FDD/TDD (for HAPS and/or ATG) and/or FR1/FR2” in the column “Capability interpretation for mixture of FDD/TDD and/or FR1/FR2”.

- The issue related to “The capability interpretation for mixture of FDD/TDD and/or FR1/FR2” was asked in RAN2 LS [5], to which RAN1 answered in [6].

- Specifically, the issue occurs when there are multiple serving cells. This is however not the case for the NTN UE features defined in the table below. Therefore, the misleading text “support mixture of FDD/TDD (for HAPS) and/or FR1/FR2” in the column “Capability interpretation for mixture of FDD/TDD and/or FR1/FR2” should be removed.

Remove the misleading text “support mixture of FDD/TDD (for HAPS and/or ATG) and/or FR1/FR2” in the column “Capability interpretation for mixture of FDD/TDD and/or FR1/FR2”.

Adopt the changes proposed in the column “Ericsson comments”.

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (Sidelink WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional | Ericsson comments |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 26. NR_NTN_solutions | 26-1 | Uplink Time pre-compensation | - UE specific TA calculation in RRC_IDLE and RRC_INACTIVE states based on its GNSS-acquired position and the serving satellite ephemeris. - UE specific TA calculation in RRC_CONNECTED state based on its GNSS-acquired position and the serving satellite ephemeris. - UE applies common TA according to the parameters provided by the network (if any) - For TA update in RRC_CONNECTED state, combination of both open (i.e. UE autonomous TA estimation, and common TA estimation) and closed (i.e., received TA commands) control loops - FFS: UE (re)acquire the serving satellite ephemeris and common TA upon the expiration of the corresponding validity timer |  | No | No | Release 17 UE cannot access [NTN/ satellite/HAPS/ATG] | [Per UE/per band] | No | No |  | An NTN UE is required to at least support UE specific TA calculation based at least on its GNSS-acquired position and the serving satellite ephemeris [Note: Serving satellite ephemeris Epoch time is implicitly known as a reference time defined by the starting time of a DL slot and/or frame] FFS: Whether this starting time is given by predefined rule or it is indicated by the Network | Optional with capability signalling For UE supports NR [NTN/ satellite/HAPS/ATG], UE must indicate this FG is supported. [Note: This UE feature group is applicable only for NR NTN cell, for terrestrial cell this feature is not supported] |  |
| 26. NR_NTN_solutions | 26-2 | Uplink Frequency/Doppler pre-compensation | - [In RRC_IDLE and RRC_INACTIVE states calculate frequency pre-compensation to counter shift the Doppler experienced on the service link] - In RRC_CONNECTED states, calculate frequency pre-compensation to counter shift the Doppler experienced on the service link |  | No | No | Release 17 UE cannot access [NTN/ satellite/HAPS/ATG] | [Per UE/per band] | No | No |  | An NR NTN UE shall be capable of at least using its acquired GNSS position and satellite ephemeris to calculate frequency pre-compensation to counter shift the Doppler experienced on the service link FFS: whether this FG gets merged with FG 26-1 | Optional with capability signalling For UE supports NR [NTN/ satellite/HAPS/ATG], UE must indicate this FG is supported. [Note: This UE feature group is applicable only for NR NTN cell, for terrestrial cell this feature is not supported] |  |
| 26. NR_NTN_solutions | 26-3 | Enhancement on the timing relationship | - estimate of UE-gNB RTT - delaying the start of ra-ResponseWindow [by UE-gNB RTT] - delaying the start of msgB-ResponseWindow [by UE-gNB RTT] - FFS: delaying the scheduling of PUSCH, PUCCH and PDCCH ordered PRACH, CSI reference resource, transmission of aperiodic SRS with cell-specific K_offset - FFS: delaying the UE action and assumption on a downlink configuration carried by MAC CE command by K_mac if it is indicated - FFS: separate FGs for cell specific Koffset and Kmac/UE-gNB RTT estimation/delay of RAR/MsgB response window | [26-1][26-2] | No | No | Release 17 UE cannot access [NTN/ satellite/HAPS/ATG] | [Per UE/per band] | No | No |  | FFS: whether this FG gets merged with FG 26-1 | Optional with capability signalling For UE supports NR [NTN/ satellite/HAPS/ATG], UE must indicate this FG is supported [Note: This UE feature group is applicable only for NR NTN cell, for terrestrial cell this feature is not supported] |  |
| 26. NR_NTN_solutions | 26-4 | UE reporting of information about the UE specific TA pre-compensation | - [The exact content of UE reporting of information about the UE specific TA pre-compensation] - FFS: frequency of the reports - FFS: granularity of the reported content | [26-1][26-2] | Yes | No |  | [Per UE/Per band] | No | No |  | FFS: whether this FG gets merged with FG 26-1 if the note “For UE supports NR [NTN/ satellite/HAPS/ATG], UE must indicate this FG is supported” is confirmed in the positive | Optionalwith capability signalling [For UE supports NR [NTN/ satellite/HAPS/ATG], UE must indicate this FG is supported] [Note: This UE feature group is applicable only for NR NTN cell, for terrestrial cell this feature is not supported] |  |
| 26. NR_NTN_solutions | 26-5 | Increasing the number of HARQ processes | - The maximal supported HARQ process number is 32 for both UL and DL - FFS: Support on the maximal HARQ process number is up to UE capability - FFS: separate features for DL and UL |  | Yes | No | [Increasing the number of HARQ processes avoids HARQ stalling] | FFS | No | No |  | FFS: whether this FG gets merged with FG 26-1 if the note “For UE supports NR [NTN/ satellite/HAPS/ATG], UE must indicate this FG is supported” is confirmed in the positive | Optional with capability signalling [For UE supports NR [NTN/ satellite/HAPS/ATG], UE must indicate this FG is supported] [Note: This UE feature group is applicable only for NR NTN cell, for terrestrial cell this feature is not supported] |  |
| 26. NR_NTN_solutions | 26-6 | Type-2 HARQ codebook Enhancement | - Enhancement on Type-2 HARQ codebook in NTN - FFS: HARQ disabling |  | Yes | No |  | [Per UE/per band] | No | No |  | FFS: whether this FG gets merged with FG 26-1 if the note “For UE supports NR [NTN/ satellite/HAPS/ATG], UE must indicate this FG is supported” is confirmed in the positive | Optional with capability signalling [For UE supports NR [NTN/ satellite/HAPS/ATG], UE must indicate this FG is supported] [Note: This UE feature group is applicable only for NR NTN cell, for terrestrial cell this feature is not supported] | There is no need to have 26-1/2 as prerequisite for this UE feature, b/c it can be implemented and tested as a standalone one. |
| 26. NR_NTN_solutions | [26-6a] | [Type-1 HARQ codebook enhancement] | - Enhancement on Type-1 HARQ codebook in NTN - FFS: HARQ disabling |  | Yes | No |  | [Per UE/per band] | No | No |  | FFS: whether this FG gets merged with FG 26-1 if the note “For UE supports NR [NTN/ satellite/HAPS/ATG], UE must indicate this FG is supported” is confirmed in the positive | Optional with capability signalling [For UE supports NR [NTN/ satellite/HAPS/ATG], UE must indicate this FG is supported] [Note: This UE feature group is applicable only for NR NTN cell, for terrestrial cell this feature is not supported] | There is no need to have 26-1/2 as prerequisite for this UE feature, b/c it can be implemented and tested as a standalone one. |
| 26. NR_NTN_solutions | [26-7] | [NTN Performance enhancement] | - The maximum number of supported aggregation factor (i.e., pdsch-AggregationFactor) for DL PDSCH is [X] FFS: X = 16 or 32 |  | Yes | No |  | [Per UE/per band] | [No] | No |  |  | Optional with capability signalling [Note: This UE feature group is applicable only for NR NTN cell, for terrestrial cell this feature is not supported] | This is per UE capability from Rel-15 (see TS 38.306). It shouldn’t be per BC. |
| 26. NR_NTN_solutions | 26-8 | Support of polarization [signalling/ information/reception] in NR NTN | - Support polarization indication reception in SIB indicating DL and/or UL polarization information using respective polarization type parameters to indicate: RHCP or LHCP or linear - FFS: polarization information for DL is indicated in SIB by the network |  | No | No | [It is assumed by the network that UE supports at least linear polarization] | [Per UE/Per band] | No | No |  | [Value range for component 1: {(RHCP, LHCP, Linear), (RHCP, LHCP), (RHCP), (LHCP), (Linear)}] FFS: whether this FG gets merged with FG 26-1 if the note “For UE supports NR [NTN/ satellite/HAPS/ATG], UE must indicate this FG is supported” is confirmed in the positive | Optional with capability signalling [For UE supports NR [NTN/ satellite/HAPS/ATG], UE must indicate this FG is supported] [Note: This UE feature group is applicable only for NR NTN cell, for terrestrial cell this feature is not supported] |  |
| 26. NR_NTN_solutions | 26-9 | UE-specific K_offset | - Reception of UE-specific K_offset via MAC-CE - delaying the scheduling of PUSCH, PUCCH and PDCCH ordered PRACH, CSI reference resource, transmission of aperiodic SRS with UE-specific Koffset | 26-3, 26-4 | Yes | No |  | FFS | No | No |  | FFS: whether this FG gets merged with FG 26-1 if the note “For UE supports NR [NTN/ satellite/HAPS/ATG], UE must indicate this FG is supported” is confirmed in the positive | Optional with capability signalling [For UE supports NR [NTN/ satellite/HAPS/ATG], UE must indicate this FG is supported] [Note: This UE feature group is applicable only for NR NTN cell, for terrestrial cell this feature is not supported] |  |

# Conclusion

Based on the discussion in the previous sections we propose the following:

Proposal 1	Remove the misleading text “support mixture of FDD/TDD (for HAPS) and/or FR1/FR2” in the column “Capability interpretation for mixture of FDD/TDD and/or FR1/FR2”.

Proposal 2	Adopt the changes proposed in the column “Ericsson comments”.

# References

RP-211784, “Solutions for NR to support non-terrestrial networks (NTN),” 3GPP TSG RAN #93-e, September 2021.

X. Lin, S. Rommer, S. Euler, E. A. Yavuz, and R. S. Karlsson, “5G from space: An overview of 3GPP non-terrestrial networks,” to appear in IEEE Communications Standards Magazine, March 2021. Available at arXiv preprint arXiv:2103.09156.

X. Lin, S. Cioni, G. Charbit, N. Chuberre, S. Hellsten, and J.-F. Boutillon, “On the path to 6G: Embracing the next wave of low Earth orbit satellite access,” to appear in IEEE Communications Magazine, April 2021. Available at arXiv preprint arXiv:2104.10533.

R1-2001513, Guidelines for UE capability definitions.

R1-1909953, LS on ambiguity of UE L1 FDD/TDD FR1/FR2 capabilities.

R1-1911741, Reply LS on ambiguity of UE L1 FDD/TDD FR1/FR2 capabilities.