3GPP TSG RAN WG1 #107-e                                                              	       R1-2111479

e-Meeting, November 11th – 19th, 2021

Agenda Item:	8.1.2.3

Source: 	Intel Corporation

Title:	Multi-TRP enhancements for beam management

Document for: 	Discussion/Decision

# Introduction

According to NR MIMO WID RP-193133, the following is the scope of work in this AI.

2. Enhancement on the support for multi-TRP deployment, targeting both FR1 and FR2: - Evaluate and, if needed, specify beam-management-related enhancements for simultaneous multi-TRP transmission with multi-panel reception

In this contribution we provide our views on the features that should be considered for beam-management-related enhancements for simultaneous multi-TRP transmission with multi-panel reception.

# Enhancements for beam measurement and reporting

## UE panel/antenna related feedback

FL Proposal 1.1 RAN1#106b-e: gNB configures/UE indicates if reported beams are associated to different RX spatial filters, or maximum number of supported layers corresponding to DL RS in a group, or whether two beams in a beam pair can be used for spatial multiplexing or diversity: - Alt-1: whether beams are associated to different Rx filters/panels - Alt-1a: gNB configures UE to report beams are associated with same and/or different RX spatial filters - Alt-2: whether beams are received with spatial multiplexing or diversity - Alt-2a : gNB configures UE to report beams for spatial multiplexing or diversity. - Alt-3: maximum number of supported layer per DL RS in a group

In our view Alt-2/2a is beneficial to support the agreed reporting option-2 of beam-pairs. We think it is better for gNB to request UE to report beam-pairs that can be used for spatial multiplexing or diversity because UE will likely have a choice to report either types of beam pairs but will not have information to choose one type over the other. Alt-1 can also work but it is still unclear to the gNB if different Rx filters mean diversity or spatial multiplexing for the purposes of L1-RSRP calculation – this ambiguity is not present in Alt-2.

Proposal-1: Support gNB configuration of whether the reported beam-pair is based on spatial multiplexing or diversity because UE may not have information to choose one criteria over the other. This does not require UE indication feedback.

## L1-SINR and interference measurement

FL Proposal 1.2 RAN1#106b-e: Support L1-SINR for beam reporting option 2 - IMR resource assumptions: - Alt-1: reuse CMR of other beam in the beam group - Alt-2: explicit IMR configuration, including ZP and/or NZP IMR

In order to obtain accurate beam-pairing information at the gNB, we think L1-SINR reporting should be an integral part of group based beam reporting framework. Given that a UE identifies a beam-pair for reporting as part of group-based beam reporting framework, it can also calculate L1-SINR for such candidate beam-pairs. We are supportive of both Alt-1 and Alt-2. Alt-1 is aligned with the idea that UE is responsible for identifying potential beam-pairs. Alt-2 is aligned with the idea that the gNB could configure beam-pairs.

Proposal-2: Support L1-SINR reporting as part of enhanced group based beam reporting and for interference measurement include interference due to the paired beam for L1-SINR determination

## UE reception during beam switching

UE panel architecture 1

Figure 1: UE with 2-TRP Rx panel combinations {0,1},{0,2},{0,3},{1,2},{1, 3},{2, 3}

Let us assume that each panel for the example UE above comprises of 2 Rx antennas and the UE is able to receive downlink transmissions from any 2 of the 4 Rx panels (4 Rx ports in base-band). Let us also assume that the UE reports beam pairs corresponding to panel-ids {0, 2} and {1, 3}. The gNB may consider a joint/time-synchronous beam switch (TRP-1 and TRP-2) leading to switching the UE panels from {0, 2} → {1, 3} or an independent/asynchronous beam switch (switching one beam at a time) leading to switching the UE panels from {0, 2} → {1, 2} → {1, 3}. In this situation, UE reception due to panel combination {1, 2}, which was not reported as a beam-pair) may lead to poor downlink reception quality during the transition.

UE panel architecture 2

Figure 2: UE with 2-TRP Rx panel combinations {0,2},{1,3} only

Let us assume that each panel for the example UE above comprises of 2 Rx antennas and the UE is able to receive downlink transmissions from only 2 combinations of the 4 Rx panels {0,2} and {1,3}. Let us also assume that the UE reports beam pairs corresponding to panel-ids {0, 2} and {1, 3}. The gNB may consider a joint/time-synchronous beam switch (TRP-1 and TRP-2) leading to switching the UE panels from {0, 2} → {1, 3} or an independent/time-asynchronous beam switch leading to switching the UE panels from {0, 2} → {1, 2} → {1, 3}. In this situation, UE reception due to panel combination {1, 2} during the transition (which was not reported) may not be possible due to UE architecture constraints (and may trigger BFR).

There could be different ways of resolving this, we propose a scheduling restriction:

Proposal-3: A UE is not expected to receive downlink signals/channels or transmit uplink signals/channels in a time overlapping manner with a beam pair combination that it has not reported within a certain interval of time.

# Beam failure recovery for multi-TRP

## Cell-specific and TRP specific BFR

Agreement - Support simultaneous configuration of cell-specific BFR and TRP-specific BFR in different CCs. - FFS: whether cell-specific and TRP-specific BFR can be configured in the same CC.

We do not see a use-case for the gNB to configure cell-specific and TRP-specific BFR in the same CC. Generally we think TRP specific BFR should be a standalone and complete procedure including RACH-based fall-back. Also it is not expected that gNB would not take into account certain TRPs (in a serving cell) while configuring TRP specific BFR. It would also be additional work to define interaction between both these procedures running in parallel.

Proposal-4: Cell-specific and TRP-specific BFR configuration in the same CC is not supported

## PUCCH-SR for single DCI

Agreement Support to configure an association between a BFD-RS set on SpCell and a PUCCH-SR resource / SR configuration for per TRP BFR. - FFS: Configure an association between a BFD-RS set on SCell and a PUCCH-SR resource / SR configuration for per TRP BFR A UE capability signaling is introduced for indicating the support of this association. Above applies only for multi-DCI case.

We think that association between a BFD-RS set on SpCell and a PUCCH-SR resource/SR configuration is also beneficial for single-DCI case.

Figure 3:Example scenario for MTRP BFR where some SCells have 1 BFD/NBI-RS set and some other SCells have 2 BFD/NBI RS sets. This is valid for both single-DCI and multi-DCI operation.

In the above figure we show an example of a general use-case where there is a mix of SCells configured with 2 BFD/NBI-RS sets and 1 BFD/NBI RS sets. Even in a single TRP case, a UE can be scheduled from either TRP – therefore, NW should know which TRP should schedule an uplink grant after reception of LRR.

Proposal-5: For single-DCI case also support association between a TRP (e.g., BFD-RS set) on a SpCell and a PUCCH-SR resource on the same SpCell

## RACH-based fall-back

Agreement RACH-based transmission can be triggered on a SpCell at least in the following scenarios - Scenario 1: When beam failure is detected on all BFD-RS sets on the SpCell - FFS: other scenarios - Scenario 2: at least one TRP fails on SpCell - Scenario 3: at least one pre-defined TRP fails on SpCell - Scenario 4: at least one TRP fails and no PUCCH-SR is configured, and no UL grant is available - Scenario 5: If MAC-CE based reporting does not work (details FFS) - Scenario 6: When no PUCCH-SR is configured Agreement For RACH-based transmission, at least when all BFD-RS sets fail in SPCell, CBRA is supported

Following our above proposal of no simultaneous configuration of cell-specific and TRP specific BFR in a CC, we propose to support CFRA based fallback for multi-TRP BFR when all BFD-RS sets fail in SpCell. It is up to the gNB to decide on the trade-off between overhead and latency due to PRACH transmission. Our view is that multi-TRP BFR should have full functionality of Rel-15 BFR procedure.

Proposal-6: For RACH based transmission, at least when all BFD-RS sets fail in SpCell, CFRA is supported

## Resetting beam for recovery

Agreement For the case of all CORESETs with 1 activated TCI state per CORESET , after 28 symbols from receiving the BFR response, the QCL assumption of all CORESETs associated with CORESETPoolIndex k (k=0,1) is updated by the RS resource associated with the latest reported new candidate beam (if found) associated with the failed BFD -RS set k (k=0,1) in the MAC-CE for TRP -specific BFR - The above applies to Scell and SpCell - The above applies for the multi-DCI case

In the last RAN1 meeting, association between BFD-RS set and CORESETPoolIndex is been established to enable recovery from a single TRP failure for mTRP BFR procedure.

The problem of resetting the beam for recovery also applies to single-DCI case. In this case, we propose to associate a CORESET and an explicitly configured BFD-RS set.

Proposal-7: Support association between a CORESET and an explicitly configured BFD-RS set for single DCI operation to allow resetting beam for recovery.

## BFRA MAC-CE content

The information carried by MAC-CE in Step 2 of the BFRQ process can be defined on a per CC basis and depending on whether 1 TRP or 2 TRP failure has occurred on that CC.

When a SCell or PCell is configured with 2 BFD/NBI-RS sets and 1 TRP failure is observed, we believe it is beneficial to include NBI set index in the MAC-CE. This supports the situation where only 1 PUCCH-SR is configured (e.g. PCell in FR1), or no PUCCH-SR is configured. This also allows the UE to select either PUCCH-SR resources or serve as a confirmation of the failed TRP to the gNB.

When a SCell is configured with 2 BFD/NBI-RS sets and 2 TRP failure is observed, then NBI information corresponding to 1 or both TRPs can be sent. Although sending information for 1 TRP could be sufficient in most cases, we propose to send NBI information for both TRPs to support use-cases where recovery latency is critical.

In summary our proposal is follows:

Proposal-8: The MAC-CE contents for SCell/PCell for 1 and 2 TRP failure cases are summarized below:

|  | 1 TRP failure | 2 TRP failure |
| --- | --- | --- |
| SCell configured with 1 BFD/NBI set | (a new beam index or no new beam identified) + failed CC index [Rel-16 behaviour] | Not applicable |
| SCell configured with 2 BFD/NBI set | ((a new beam index + NBI set index) or (no new beam identified)) and failed CC index | ((a new beam index + NBI set index) or (no new beam identified)) for TRP1, TRP2 and failed CC index |
| PCell configured with 1 BFD/NBI set | No MAC-CE, RACH fall-back [Rel-15 behaviour] | Not applicable |
| PCell configured with 2 BFD/NBI set | ((a new beam index + NBI set index) or (no new beam identified)) and failed CC index | No MAC-CE, RACH fall-back [Rel-15] |

# Conclusions

Proposal-1: Support gNB configuration of whether the reported beam-pair is based on spatial multiplexing or diversity because UE may not have information to choose one criteria over the other. This does not require UE indication feedback.

Proposal-2: Support L1-SINR reporting as part of enhanced group based beam reporting and for interference measurement include interference due to the paired beam for L1-SINR determination

Proposal-3: A UE is not expected to receive downlink signals/channels or transmit uplink signals/channels in a time overlapping manner with a beam pair combination that it has not reported within a certain interval of time.

Proposal-4: Cell-specific and TRP-specific BFR configuration in the same CC is not supported

Proposal-5: For single-DCI case also support association between a TRP (e.g., BFD-RS set) on a SpCell and a PUCCH-SR resource on the same SpCell.

Proposal-6: For RACH based transmission, at least when all BFD-RS sets fail in SpCell, CFRA is supported

Proposal-7: Support association between a CORESET and an explicitly configured BFD-RS set for single DCI operation to allow resetting beam for recovery.

Proposal-8: The MAC-CE contents for SCell/PCell for 1 and 2 TRP failure cases are summarized below

|  | 1 TRP failure | 2 TRP failure |
| --- | --- | --- |
| SCell configured with 1 BFD/NBI set | (a new beam index or no new beam identified) + failed CC index [Rel-16 behaviour] | Not applicable |
| SCell configured with 2 BFD/NBI set | ((a new beam index + NBI set index) or (no new beam identified)) and failed CC index | ((a new beam index + NBI set index) or (no new beam identified)) for TRP1, TRP2 and failed CC index |
| PCell configured with 1 BFD/NBI set | No MAC-CE, RACH fall-back [Rel-15 behaviour] | Not applicable |
| PCell configured with 2 BFD/NBI set | ((a new beam index + NBI set index) or (no new beam identified)) and failed CC index | No MAC-CE, RACH fall-back [Rel-15] |

# References

[1] RP-193133, New WID: Further enhancements on MIMO for NR, Samsung, RAN#86