3GPP TSG RAN WG1 #107-e		    	R1-2111517

e-Meeting, November 11th – 19th, 2021

Source:	Intel Corporation

Title:	Mechanisms to Improve Reliability of NR MBS for

RRC_CONNECTED UEs

Agenda item:	8.12.2

Document for:	Discussion and Decision

# Introduction

In this paper, reliability aspects of NR multicast and broadcast services (MBS) for RRC_CONECTED UEs are discussed. The new work item on NR Support of Multicast and Broadcast Services [1] has the following objectives for normative work on NR MBS related to improving the reliability of multicast/broadcast service:

- Specify RAN basic functions for broadcast/multicast for UEs in RRC_CONNECTED state [RAN1, RAN2, RAN3]: - Specify required changes to improve reliability of Broadcast/Multicast service, e.g. by UL feedback. The level of reliability should be based on the requirements of the application/service provided. [ RAN1, RAN2]

In this paper, we discuss mechanisms for improving the reliability of NR based multicast and broadcast services within a single cell for RRC_CONNECTED UEs.

# Discussion

In this section, design considerations for reliability mechanisms for NR MBS focused on RRC_CONNECTED UEs is provided.

## 2.1. Improving Reliability based on UL Feedback

### Support of HARQ/ACK Schemes

Since NR MBS supports FDM of MBS and unicast PDSCH, as well as HARQ feedback, the case of out-of-order HARQ feedback needs to be considered i.e., if the last OFDM symbol of the MBS PDSCH occurs before the last OFDM symbol of the unicast PDSCH, is the UE allowed to transmit MBS HARQ feedback after unicast? In some cases, allowing out of order HARQ feedback may be beneficial where unicast and HARQ may have independently configured set of K1 values.

- For the case when unicast and MBS PDSCH partially overlap in time on different PRBs, out-of-order HARQ feedback may be supported wherein the HARQ feedback corresponding to the PDSCH ending earlier in time may be transmitted after the HARQ feedback for the PDSCH ending later in time

### Configurability of HARQ/ACK Feedback

The following working assumption was reached in RAN1#105-e meeting:

Agreement: Update the WA made in RAN1#105-e meeting regarding enabling/disabling HARQ-ACK feedback as follows: Working assumption: For enabling/disabling ACK/NACK-based HARQ-ACK feedback for RRC_CONNECTED UE receiving multicast via dynamic group-common PDSCH: - RRC signaling configures the enabling/ disabling function of group-common DCI indicating the enabling /disabling ACK/NACK based HARQ-ACK feedback. - If RRC signaling configures the function of group-common DCI based indication, group-common DCI indicates (explicitly or implicitly) whether ACK/NACK based HARQ-ACK feedback is enabled/disabled - Otherwise, enabling/disabling ACK/NACK based HARQ-ACK feedback is configured by RRC signaling. - FFS details on RRC signaling and group-common DCI indicating. - FFS whether/how this option is extended to apply to NACK-only based feedback and multiple G-RNTI cases. - FFS the relation to the HARQ-ACK codebook types and HARQ-ACK codebook construction. - FFS the relation to the enabling/disabling ACK/NACK based HARQ-ACK feedback for retransmission. - FFS whether/how to allow UE not to react to the DCI signaling, but instead follow UE-specific RRC configuration for HARQ feedback. - FFS whether/how to apply it to SPS group-common PDSCH. - UE capability for enabling/ disabling function of group-common DCI indicating the enabling /disabling ACK/NACK based HARQ-ACK feedback is introduced and FFS details. - Note: It is up to network implementation to avoid any potential HARQ ACK mismatch between different UEs in the same multicast group

- For the above agreement, there is an FFS on “whether/how to allow UE not to react to the DCI signaling, but instead follow UE-specific RRC configuration for HARQ feedback”. The mechanism for enabling/disabling HARQ is already very complicated. Additional complications can be introduced if functionality to ignore DCI feedback is introduced. To this end, if RRC enables the feature of dynamic enabling/disabling of HARQ, the UE should always expect to be signaled by the corresponding scheduling DCI if HARQ is enabled or disabled. There should be no provision to ignore or not indicate this via DCI.

- If RRC enables dynamic indication of HARQ on/off, UE always expects to be signaled with HARQ on/off in DCI i.e., there is no case where UE can ignore DCI based indication

- For the case of NACK-only feedback, only RRC based configuration of HARQ ON/OFF would suffice and complicated DCI based indication is not necessary.

- For NACK-only feedback RRC configures ON/OFF with no DCI based indication

- In RAN1#106bis-e meeting the following agreement was reached.

Agreement: For group-common DCI indicating whether ACK/NACK based HARQ-ACK feedback is enabled/disabled, down-select from the following alternatives: - Alt1: Reuse one existing field in the group-common DCI. - Alt2: Introduce a new field in the group-common DCI.

- To this end, the MBS DCI formats have quite a few fields which are not used. Any of these fields can be re-used for indication of HARQ-ACK feedback on/off. Therefore Alt-1 is preferred.

- Reuse one existing field in the group-common DCI for HARQ/ACK feedback enabled/disabled indication

### HARQ Codebook Design and PUCCH Configuration

#### Type 1 HARQ Codebook Design:

Agreement: When UE is configured Type-1 codebooks for unicast and multicast with different priorities, respectively, the UE separately generates each of the Type-1 codebooks. - FFS: How UE is configured one codebook for unicast and one codebook for multicast and the two codebooks are of different priorities

In order to address the FFS point in the agreement above, the pdsch-HARQ-ACK-Codebook for MBS configuration may have a separate parameter which indicates the priority level of the codebook. For example, the Type 1 MBS codebooks can all be of low priority compared to unicast or all be of high priority compared to unicast.

- If UE is configured one CB for multicast and one CB for unicast of different priorities, the pdsch-HARQ-ACK-Codebook configuration can include a priority indicator which applies to all multicast codebooks.

As discussed in Section 2.1.2, ACK/NACK-based or NACK-only-based feedback can be dynamically disabled by DCI. However, this may have some non-trivial impact on codebook construction.  For Type 1 HARQ Codebook disabling the HARQ-ACK for a GC-PDSCH does not change the Type 1 HARQ-ACK codebook generation. For the GC-PDSCHs whose ACK/NACK feedback is disabled, padding NACK can be used such that the CB size is same as in the case if HARQ feedback was enabled.

- For Type 1 Codebook if HARQ is disabled by DCI for a GC-PDSCH, Type 1 codebook generation does not change, and padding NACK can be used such that CB size is same as in the case if HARQ-ACK feedback was enabled.

#### Type 2 HARQ Codebook Design:

For the case when HARQ-ACK feedback for GC-PDSCH is dynamically disabled by DCI, for Type 2 HARQ Codebook, the C-DAI for the GC-PDSCH which has HARQ disabled can be ignored. On the other hand, the T-DAI can still be updated since it may be beneficial to have the T-DAI indicating the total number of PDSCHs needing HARQ-ACK. Furthermore, other HARQ-ACK feedback related fields in the DCI scheduling GC-PDSCH with HARQ disabled may indicate valid information. For example, the PUCCH resource indicator field in the DCI may indicate a valid PUCCH resource for HARQ-ACK transmission. Therefore, although HARQ-ACK is not reported for the GC-PDSCH, the DCI can be used for triggering HARQ-ACK transmission for other GC-PDSCHs with HARQ enabled.

- For Type 2 Codebook if HARQ is disabled by DCI for a GC-PDSCH, the C-DAI can for the GC-PDSCH can be ignored but the T-DAI is still indicated.

Agreement: When UE supports and is configured with more than one G-RNTI, - for Type-2 codebook construction, DAI is separately counted per G-RNTI. - Type-2 codebook is constructed by concatenating Type-2 sub-codebook of each RNTI following the ascending order of the G-RNTI value.

In light of the above agreement if different priorities are supported and configured, then the DAI should be counted per G-RNTI per priority index. Similarly, concatenation should also be for each priority index.

- When UE is configured with more than one G-RNTI as well as more than one priority index, for Type 2 codebooks, DAI is separately counter per G-RNTI per priority index and the codebook concatenation is also per priority index. Same principle is also followed for NACK only feedback.

#### PUCCH Configuration:

Agreement: For UE supporting both unicast and multicast, the pdsch-HARQ-ACK-Codebook/pdsch-HARQ-ACK-CodebookList can be separately configured for multicast from that for unicast. Agreement: For the separate PUCCH-ConfigurationList that is optionally configured to UE for NACK-only based HARQ-ACK feedback for multicast, - The separate PUCCH-ConfigurationList for multicast configuration can be a list which includes up to 2 PUCCH-Config configurations corresponding low priority feedback and high priority feedback, respectively. - FFS: how to handle the case when separate PUCCH-ConfigurationList is not configured to UE for NACK-only based HARQ-ACK feedback for multicast

For the FFS point in the second agreement above, when separate PUCCH-ConfigurationList is not configured for NACK-only feedback for multicast, the UE can re-use the PUCCH configuration for ACK/NACK based feedback for multicast. Alternatively, the configuration for unicast can be reused if PUCCH-ConfigurationList is not provided for multicast. Additionally, if high priority NACK-only feedback is configured, the UE expects to be configured with a separate PUCCH-ConfigurationList for NACK-only feedback.

- If separate PUCCH-ConfigurationList is not configured for NACK-only feedback for multicast, the UE can re-use the PUCCH configuration for ACK/NACK based feedback for multicast

- If high priority NACK-only feedback is configured, the UE expects to be configured with a separate PUCCH-ConfigurationList for NACK-only feedback

For PUCCH transmission which overlap in time, a dropping rule is used to drop the low priority HARQ. Additionally, multiplexing across priorities is not allowed in a single PUCCH occasion.

- Multiplexing across priorities is not allowed in a single PUCCH occasion. For overlapping PUCCH transmissions in time, dropping rule is used.

- For PUCCH resource selection the following rules can be considered for overlap involving different cases:

- If MBS and unicast have separate PUCCH-Configuration but with the same priority index and the PUCCH overlap in time, the PUCCH carrying the multiplexed HARQ-ACK feedback is from the PUCCH-Configuration of unicast for the corresponding priority.

- If multicast ACK/NACK based feedback and NACK-only feedback have separate PUCCH-Configuration but with the same priority index and overlap in time for the PUCCH resources carrying the feedback, then the PUCCH resource carrying the multiplexed feedback is from the PUCCH-Configuration of the ACK/NACK based feedback.

- If unicast ACK/NACK based feedback and multicast NACK-only feedback have separate PUCCH-Configuration but with the same priority index and overlap in time for the PUCCH resources carrying the feedback, then the PUCCH resource carrying the multiplexed feedback is from the PUCCH-Configuration of the ACK/NACK based feedback for unicast

HARQ priority is also applicable to the feedback for MBS SPS. If feedback for one MBS SPS overlaps with that of another MBS SPS or another unicast, similar priority handling rule is needed i.e., same priority feedback is handled together. SPS configuration of priority 0/1 maps to HARQ codebook of priority 0/1 respectively.

- HARQ priority rules are also applicable to feedback for MBS SPS wherein SPS configuration of priority 0/1 maps to HARQ codebook priority 0/1 respectively.

### HARQ Retransmission

- One major feature in NR is the introduction of CBG based retransmissions wherein UEs can signal which CBs failed and only those CBs need to be retransmitted instead of the entire TB. However, in the case of groupcast, each UE in the group might face independent CB failures and the entire TB will need to be retransmitted. Furthermore, retransmissions are also limited by the worst UEs in the group. To counter this, in case of ACK/NACK based HARQ, UE specific retransmission may be supported if PDCCH monitoring on USS is supported for NR MBS. In this case, gNB can retransmit failed CBGs to a single or smaller sub-group of UEs instead of retransmitting the entire TB.

- For ACK/NACK based HARQ operation, support UE specific CBG based retransmission. Other advanced retransmission schemes are not precluded.

# Conclusions

In this paper, design options were proposed for reliability improvements for NR MBS. To this end, the following proposals were made:

- For the case when unicast and MBS PDSCH partially overlap in time on different PRBs, out-of-order HARQ feedback may be supported wherein the HARQ feedback corresponding to the PDSCH ending earlier in time may be transmitted after the HARQ feedback for the PDSCH ending later in time

- If RRC enables dynamic indication of HARQ on/off, UE always expects to be signaled with HARQ on/off in DCI i.e., there is no case where UE can ignore DCI based indication

- For NACK-only feedback RRC configures ON/OFF with no DCI based indication

- For NACK-only feedback RRC configures ON/OFF with no DCI based indication

- If UE is configured one CB for multicast and one CB for unicast of different priorities, the pdsch-HARQ-ACK-Codebook configuration can include a priority indicator which applies to all multicast codebooks.

- For Type 1 Codebook if HARQ is disabled by DCI for a GC-PDSCH, Type 1 codebook generation does not change, and padding NACK can be used such that CB size is same as in the case if HARQ-ACK feedback was enabled.

- For Type 2 Codebook if HARQ is disabled by DCI for a GC-PDSCH, the C-DAI can for the GC-PDSCH can be ignored but the T-DAI is still indicated

- When UE is configured with more than one G-RNTI as well as more than one priority index, for Type 2 codebooks, DAI is separately counter per G-RNTI per priority index and the codebook concatenation is also per priority index. Same principle is also followed for NACK only feedback

- If separate PUCCH-ConfigurationList is not configured for NACK-only feedback for multicast, the UE can re-use the PUCCH configuration for ACK/NACK based feedback for multicast

- If high priority NACK-only feedback is configured, the UE expects to be configured with a separate PUCCH-ConfigurationList for NACK-only feedback

- Multiplexing across priorities is not allowed in a single PUCCH occasion. For overlapping PUCCH transmissions in time, dropping rule is used

- For PUCCH resource selection the following rules can be considered for overlap involving different cases:

- If MBS and unicast have separate PUCCH-Configuration but with the same priority index and the PUCCH overlap in time, the PUCCH carrying the multiplexed HARQ-ACK feedback is from the PUCCH-Configuration of unicast for the corresponding priority.

- If multicast ACK/NACK based feedback and NACK-only feedback have separate PUCCH-Configuration but with the same priority index and overlap in time for the PUCCH resources carrying the feedback, then the PUCCH resource carrying the multiplexed feedback is from the PUCCH-Configuration of the ACK/NACK based feedback.

- If unicast ACK/NACK based feedback and multicast NACK-only feedback have separate PUCCH-Configuration but with the same priority index and overlap in time for the PUCCH resources carrying the feedback, then the PUCCH resource carrying the multiplexed feedback is from the PUCCH-Configuration of the ACK/NACK based feedback for unicast

- HARQ priority rules are also applicable to feedback for MBS SPS wherein SPS configuration of priority 0/1 maps to HARQ codebook priority 0/1 respectively

- For ACK/NACK based HARQ operation, support UE specific CBG based retransmission. Other advanced retransmission schemes are not precluded

# References

- RP-193248, New Work Item on NR Support of Multicast and Broadcast Services, Huawei, RAN#86, Sitges, Spain, December 2019.

- Chairman’s Notes, 3GPP RAN1 106-e Meeting, August 2021

- Chairman’s Notes, 3GPP RAN1 105-e Meeting, May 2021

- Chairman’s Notes, 3GPP RAN1 104bis-e Meeting, April 2021

- 3GPP TR 36.890 v13.0.0, Study on Single Cell Point-to-Multipoint Transmission (Release 13)

- 3GPP TS 38.211 v16.2.0, NR Physical Channels and Modulation (Release 16)

- 3GPP TS 38.212 v16.2.0, NR Multiplexing and Channel Coding (Release 16)

- 3GPP TS 38.213 v16.2.0, NR Physical Layer Procedures for Control (Release 16)

- 3GPP TS 38.214 v16.2.0, NR Physical Layer Procedures for Data (Release 16)