3GPP TSG RAN WG1 #107-e					        													R1-2111528

e-Meeting, November 11th – 19th, 2021

Source:	Intel Corporation

Title:	On UE features for NR NTN

Agenda item:	8.16.4

Document for:	Discussion and Decision

# Introduction

In this contribution, we provide discussion on UE features for NR NTN based on the latest proposal in [1]. Table with the details on UE features for NR NTN is captured in the Appendix (copied from [1] with proposed changes highlighted by red font).

# Discussion

Several UE feature groups can be considered as essential for NR NTN since it is not possible to do initial access for a UE if the corresponding features are not supported. In our view such features correspond to at least FG 26-1 and 26-3. FG 26-2 is also important, but it may not be needed for some bands, for example for GEO satellites Doppler pre-compensation may not be needed.

So, in our view there is no need to support such case where FG 26-1 is supported but FG 26-3 is not supported and vice versa. Thus, we propose to merge FG 26-1 and 26-3. Also, in our view FFS is not needed for “delaying the scheduling of PUSCH, PUCCH and PDCCH ordered PRACH, CSI reference resource, transmission of aperiodic SRS with cell-specific K_offset” and “delaying the UE action and assumption on a downlink configuration carried by MAC CE command by K_mac if it is indicated”. If the FFS are deleted for the above components, bullet “FFS: separate FGs for cell specific Koffset and Kmac/UE-gNB RTT estimation/delay of RAR/MsgB response window” can be deleted.

For FG 26-4, component 2 and component 3 corresponding to frequency and granularity of the TA report are not needed since it is not clear which RAN1 features are assumed for these components. In our view it is better to discuss RAN1 features first (i.e. periodical reports with some periodicity or reports with different granularities) before listing it in the UE feature list.

For FG 26-5, components 2 and 3 should be deleted since component 1 is enough to indicate that 32 HARQ processes are supported for DL and UL. In our view there is no need to have additional flexibility for indication of the corresponding UE features since it will lead to market fragmentation.

For FG 26-8, component “polarization information for DL is indicated in SIB by the network” doesn’t make sense since polarisation information is just a recommendation for the UE.

# Conclusion

In this contribution, we provide discussion on UE features for NR NTN. The following proposal is made.

Proposal:

- Consider changes to the UE feature list for NR NTN marked below using red font

# References

- R1-2110587, Updated RAN1 UE features list for Rel-17 NR after RAN1 #106bis-e

# Appendix

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (Sidelink WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 26. NR_NTN_solutions | 26-1 | Uplink Time pre-compensation and Enhancement on the timing relationship | - UE specific TA calculation in RRC_IDLE and RRC_INACTIVE states based on its GNSS-acquired position and the serving satellite ephemeris. - UE specific TA calculation in RRC_CONNECTED state based on its GNSS-acquired position and the serving satellite ephemeris. - UE applies common TA according to the parameters provided by the network (if any) - For TA update in RRC_CONNECTED state, combination of both open (i.e. UE autonomous TA estimation, and common TA estimation) and closed (i.e., received TA commands) control loops - FFS: UE (re)acquire the serving satellite ephemeris and common TA upon the expiration of the corresponding validity timer - estimate of UE-gNB RTT - delaying the start of ra-ResponseWindow [by UE-gNB RTT] - delaying the start of msgB-ResponseWindow [by UE-gNB RTT] - FFS: delaying the scheduling of PUSCH, PUCCH and PDCCH ordered PRACH, CSI reference resource, transmission of aperiodic SRS with cell-specific K_offset - FFS: delaying the UE action and assumption on a downlink configuration carried by MAC CE command by K_mac if it is indicated - FFS: separate FGs for cell specific Koffset and Kmac/UE-gNB RTT estimation/delay of RAR/MsgB response window |  | No | No | Release 17 UE cannot access [NTN/ satellite/HAPS/ATG] | [Per UE/per band] | No | No | [support mixture of FDD/TDD (for HAPS and/or ATG) and/or FR1/FR2] | An NTN UE is required to at least support UE specific TA calculation based at least on its GNSS-acquired position and the serving satellite ephemeris [Note: Serving satellite ephemeris Epoch time is implicitly known as a reference time defined by the starting time of a DL slot and/or frame] FFS: Whether this starting time is given by predefined rule or it is indicated by the Network | Optional with capability signalling For UE supports NR [NTN/ satellite/HAPS/ATG], UE must indicate this FG is supported. [Note: This UE feature group is applicable only for NR NTN cell, for terrestrial cell this feature is not supported] |
| 26. NR_NTN_solutions | 26-2 | Uplink Frequency/Doppler pre-compensation | - [In RRC_IDLE and RRC_INACTIVE states calculate frequency pre-compensation to counter shift the Doppler experienced on the service link] - In RRC_CONNECTED states, calculate frequency pre-compensation to counter shift the Doppler experienced on the service link |  | No | No | Release 17 UE cannot access [NTN/ satellite/HAPS/ATG] | [Per UE/per band] | No | No | [support mixture of FDD/TDD (for HAPS and/or ATG) and/or FR1/FR2] | An NR NTN UE shall be capable of at least using its acquired GNSS position and satellite ephemeris to calculate frequency pre-compensation to counter shift the Doppler experienced on the service link FFS: whether this FG gets merged with FG 26-1 | Optional with capability signalling For UE supports NR [NTN/ satellite/HAPS/ATG], UE must indicate this FG is supported. [Note: This UE feature group is applicable only for NR NTN cell, for terrestrial cell this feature is not supported] |
| 26. NR_NTN_solutions | 26-3 | Enhancement on the timing relationship | - estimate of UE-gNB RTT - delaying the start of ra-ResponseWindow [by UE-gNB RTT] - delaying the start of msgB-ResponseWindow [by UE-gNB RTT] - FFS: delaying the scheduling of PUSCH, PUCCH and PDCCH ordered PRACH, CSI reference resource, transmission of aperiodic SRS with cell-specific K_offset - FFS: delaying the UE action and assumption on a downlink configuration carried by MAC CE command by K_mac if it is indicated - FFS: separate FGs for cell specific Koffset and Kmac/UE-gNB RTT estimation/delay of RAR/MsgB response window | [26-1][26-2] | No | No | Release 17 UE cannot access [NTN/ satellite/HAPS/ATG] | [Per UE/per band] | No | No | [support mixture of FDD/TDD (for HAPS and/or ATG) and/or FR1/FR2] | FFS: whether this FG gets merged with FG 26-1 | Optional with capability signalling For UE supports NR [NTN/ satellite/HAPS/ATG], UE must indicate this FG is supported [Note: This UE feature group is applicable only for NR NTN cell, for terrestrial cell this feature is not supported] |
| 26. NR_NTN_solutions | 26-4 | UE reporting of information about the UE specific TA pre-compensation | - [The exact content of UE reporting of information about the UE specific TA pre-compensation] - FFS: frequency of the reports - FFS: granularity of the reported content | [26-1][26-2] | Yes | No |  | [Per UE/Per band] | No | No | [support mixture of FDD/TDD (for HAPS and/or ATG) and/or FR1/FR2] | FFS: whether this FG gets merged with FG 26-1 if the note “For UE supports NR [NTN/ satellite/HAPS/ATG], UE must indicate this FG is supported” is confirmed in the positive | Optionalwith capability signalling [For UE supports NR [NTN/ satellite/HAPS/ATG], UE must indicate this FG is supported] [Note: This UE feature group is applicable only for NR NTN cell, for terrestrial cell this feature is not supported] |
| 26. NR_NTN_solutions | 26-5 | Increasing the number of HARQ processes | - The maximal supported HARQ process number is 32 for both UL and DL - FFS: Support on the maximal HARQ process number is up to UE capability - FFS: separate features for DL and UL |  | Yes | No | [Increasing the number of HARQ processes avoids HARQ stalling] | FFS | No | No | [support mixture of FDD/TDD (for HAPS and/or STG) and/or FR1/FR] | FFS: whether this FG gets merged with FG 26-1 if the note “For UE supports NR [NTN/ satellite/HAPS/ATG], UE must indicate this FG is supported” is confirmed in the positive | Optional with capability signalling [For UE supports NR [NTN/ satellite/HAPS/ATG], UE must indicate this FG is supported] [Note: This UE feature group is applicable only for NR NTN cell, for terrestrial cell this feature is not supported] |
| 26. NR_NTN_solutions | 26-6 | Type-2 HARQ codebook Enhancement | - Enhancement on Type-2 HARQ codebook in NTN - FFS: HARQ disabling | [26-1, 26-2] | Yes | No |  | [Per UE/per band] | No | No | [support mixture of FDD/TDD (for HAPS and/or ATG) and/or FR1/FR2] | FFS: whether this FG gets merged with FG 26-1 if the note “For UE supports NR [NTN/ satellite/HAPS/ATG], UE must indicate this FG is supported” is confirmed in the positive | Optional with capability signalling [For UE supports NR [NTN/ satellite/HAPS/ATG], UE must indicate this FG is supported] [Note: This UE feature group is applicable only for NR NTN cell, for terrestrial cell this feature is not supported] |
| 26. NR_NTN_solutions | [26-6a] | [Type-1 HARQ codebook enhancement] | - Enhancement on Type-1 HARQ codebook in NTN - FFS: HARQ disabling | [26-1, 26-2] | Yes | No |  | [Per UE/per band] | No | No | [support mixture of FDD/TDD (for HAPS and/or ATG) and/or FR1/FR2] | FFS: whether this FG gets merged with FG 26-1 if the note “For UE supports NR [NTN/ satellite/HAPS/ATG], UE must indicate this FG is supported” is confirmed in the positive | Optional with capability signalling [For UE supports NR [NTN/ satellite/HAPS/ATG], UE must indicate this FG is supported] [Note: This UE feature group is applicable only for NR NTN cell, for terrestrial cell this feature is not supported] |
| 26. NR_NTN_solutions | [26-7] | [NTN Performance enhancement] | - The maximum number of supported aggregation factor (i.e., pdsch-AggregationFactor) for DL PDSCH is [X] - FFS: X = 16 or 32 |  | Yes | No |  | [Per UE/per band/Per BC] | [No] | No | [support mixture of FDD/TDD (for HAPS and/or ATG) and/or FR1/FR2] |  | Optional with capability signalling [Note: This UE feature group is applicable only for NR NTN cell, for terrestrial cell this feature is not supported] |
| 26. NR_NTN_solutions | 26-8 | Support of polarization [signalling/ information/reception] in NR NTN | - Support polarization indication reception in SIB indicating DL and/or UL polarization information using respective polarization type parameters to indicate: RHCP or LHCP or linear - FFS: polarization information for DL is indicated in SIB by the network |  | No | No | [It is assumed by the network that UE supports at least linear polarization] | [Per UE/Per band] | No | No | [support mixture of FDD/TDD (for HAPS and/or ATG) and/or FR1/FR2] | [Value range for component 1: {(RHCP, LHCP, Linear), (RHCP, LHCP), (RHCP), (LHCP), (Linear)}] FFS: whether this FG gets merged with FG 26-1 if the note “For UE supports NR [NTN/ satellite/HAPS/ATG], UE must indicate this FG is supported” is confirmed in the positive | Optional with capability signalling [For UE supports NR [NTN/ satellite/HAPS/ATG], UE must indicate this FG is supported] [Note: This UE feature group is applicable only for NR NTN cell, for terrestrial cell this feature is not supported] |
| 26. NR_NTN_solutions | 26-9 | UE-specific K_offset | - Reception of UE-specific K_offset via MAC-CE delaying the scheduling of PUSCH, PUCCH and PDCCH ordered PRACH, CSI reference resource, transmission of aperiodic SRS with UE-specific Koffset | 26-3, 26-4 | Yes | No |  | FFS | No | No | [support mixture of FDD/TDD (for HAPS and/or ATG) and/or FR1/FR2] | FFS: whether this FG gets merged with FG 26-1 if the note “For UE supports NR [NTN/ satellite/HAPS/ATG], UE must indicate this FG is supported” is confirmed in the positive | Optional with capability signalling [For UE supports NR [NTN/ satellite/HAPS/ATG], UE must indicate this FG is supported] [Note: This UE feature group is applicable only for NR NTN cell, for terrestrial cell this feature is not supported] |