3GPP TSG RAN WG1 #107-e			    R1-2111535

e-Meeting, November 11th – 19th, 2021

Source:	Intel Corporation

Title:	Discussion on UE Features for NR MBS

Agenda item:	8.16.12

Document for:	Discussion and Decision

# Introduction

In this paper, we provide our views on initial version of RAN1 UE features for Rel-17 NR MBS [2].

# Discussion

The initial version of NR MBS UE feature list provides a set of feature groups based on current design of NR MBS. We have provided our initial feedback in Table 1. The proposed changes are marked in red in the table. In summary we propose the following updates to the initial version of the UE Feature list:

- FG 33-1: This group should be renamed to “Broadcast for RRC_CONNECTED and RRC_IDLE/INACTIVE UEs” to capture the fact that low QoS broadcast transmission is supported by both connected and idle/inactive mode UEs

- FG 33-2:

  - This group should be renamed to “Multicast for RRC_CONNECTED UEs” to capture the fact that Multicast is supported only for connected mode UEs in Rel-17.

  - The support of ACK/NACK based HARQ feedback should be separated into a separate dependent feature group since the default mode is HARQ disabled and RRC is used to enable it. UEs may not support HARQ feedback.

- FG 33-4:

  - Add support of enabling/disabling NACK-only feedback by RRC signaling

- FG 33-5-1:

  - HARQ/ACK feedback support should be separated from this feature group similar to 33-2

- FG 33-6:

  - 33-6-1 should be renamed HARQ-ACK priority indication for multicast in DCI

  - 33-6-2/3 should be combined into a single FG since these two features are inter-related

- FG 33-7:

  - NACK-only feedback should also be included in this FG

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) |
| --- | --- | --- | --- | --- | --- | --- |
| 33. NR_MBS | 33-1 | Broadcast for RRC_CONNECTED and RRC_IDLE/INACTIVE UEs | - Support of group-common PDCCH/PDSCH with CRC scrambled by MCCH-RNTI. - Support of group-common PDCCH/PDSCH with CRC scrambled by G-RNTI. - Support of CFR configuration for broadcast. - Support of CORESET and common search space for broadcast. - Support of DCI format 1_0 with CRC scrambled with G-RNTI/MCCH-RNTI for broadcast. - Support of inter-slot TDM between unicast PDSCH and group-common PDSCH in different slots. - Support MCCH change notification indication via DCI. |  | Yes | Per UE |
| 33. NR_MBS | 33-2 | Dynamic scheduling for multicast Multicast for RRC_CONNECTED UEs | - Support of group-common PDCCH/PDSCH with CRC scrambled by G-RNTI. - Support of CFR configuration for multicast. - Support of CORESET and common search space configuration for multicast. - Support of DCI format 1_0 / 1_1 with CRC scrambled with G-RNTI for multicast. - Support of inter-slot TDM between unicast PDSCH and group-common PDSCH in different slots. - Support ACK/NACK based HARQ-ACK feedback, and support enabling/disabling ACK/NACK based HARQ-ACK feedback configured by RRC signaling. - Support PTM retransmission for multicast. - Support PTP retransmission for multicast. FFS whether/how to separate the above capabilities from FG 33-2 |  | Yes | Per UE |
| 33. NR_MBS | 33-2-x | Multiple G-RNTIs for group-common PDSCHs | Capability on number of G-RNTI for groupcast FFS details. | 33-2 | Yes | Per UE |
| 33. NR_MBS | 33-3-1 | Slot-level repetition for group-common PDSCH | - Support slot-level repetition for group-common PDSCH for multicast. FFS whether to split FG 33-1-1 into two FGs for semi-static slot-level repetition and dynamic slot-level repetition | 33-2 | Yes | Per UE |
| 33. NR_MBS | 33-3-2 | FDM-ed unicast PDSCH and group-common PDSCH | - Support FDM between one unicast PDSCH and one group-common PDSCH in a slot. - Support FDM-ed Type-1 HARQ-ACK codebook for multicast. - Support FDM-ed Type-2 HARQ-ACK codebook for multicast. FFS whether/how to separate the capability for HARQ-ACK codebook | 33-1, 33-2 | Yes | Per UE |
| 33. NR_MBS | 33-3-3 | Intra-slot TDM-ed unicast PDSCH and group-common PDSCH | - Support TDM between one unicast PDSCH and one group-common PDSCH in a slot. - Support TDM between M (M>1) TDMed unicast PDSCHs and one group-common PDSCH in a slot per CC - Support TDM among N (N>1) group-common PDSCHs in a slot per CC - Support TDM between K (K>1) TDMed unicast PDSCHs and L (L>1) TDMed group-common PDSCHs in a slot per CC - The UE maximum number of TDMed PDSCH receptions capability in a slot per CC is kept as for Rel-15/Rel-16, i.e., {2/4/7} based on UE FG5-11/5-11a/5-11b. - Note: Group-common PDSCH(s) are counted as unicast PDSCH(s). - Support TDM-ed Type-1 HARQ-ACK codebook for multicast. - Support TDM-ed Type-2 HARQ-ACK codebook for multicast. FFS whether/how to separate the capability for HARQ-ACK codebook | 33-1, 33-2 | Yes | Per UE |
| 33. NR_MBS | 33-x | ACK/NACK based HARQ-ACK feedback for multicast | - Support ACK/NACK based HARQ-ACK feedback, - Support enabling/disabling ACK/NACK based HARQ-ACK feedback configured by RRC signaling | 33-2 | Yes | Per UE |
| 33. NR_MBS | 33-4 | NACK-only based HARQ-ACK feedback for multicast | - Support NACK-only based HARQ-ACK feedback. - Support enabling/disabling NACK-only based HARQ-ACK feedback configured by RRC signaling | 33-2 | Yes | Per UE |
| 33. NR_MBS | 33-5-1 | SPS group-common PDSCH for multicast | - Support one SPS group-common PDSCH configuration for multicast - Support ACK/NACK based HARQ-ACK feedback for SPS group-common PDSCH without PDCCH scheduling. - Support NACK-only based HARQ-ACK feedback for SPS group-common PDSCH without PDCCH scheduling. - FFS: HARQ-ACK feedback for SPS group-common with PDCCH scheduling and SPS release PDCCH. - Support slot-level repetition for SPS group-common PDSCH FFS whether/how to separate the above capabilities from FG 33-5-1 | 33-2 | Yes | Per UE |
| 33. NR_MBS | 33-5-2 | Multiple SPS group-common PDSCH configuration | - Support [N>1] SPS group-common PDSCH configuration for multicast - Support [M>=1] activated SPS group-common PDSCH configurations among the N SPS group-common PDSCH configurations per CFR for multicast | 33-2 | Yes | Per UE |
| 33. NR_MBS | 33-6-1 | HARQ/ACK DL priority indication for multicast in DCI | 1. Support of priority indicator field configured in DCI formats 1_1 with CRC scrambled with G-RNTI for multicast. FFS whether/how to separate the above capability from FG 33-6-1 | 33-2 | Yes | Per UE |
| 33. NR_MBS | 33-6-2 | Two HARQ-ACK codebooks simultaneously constructed for supporting HARQ-ACK codebooks with different priorities for multicast or for unicast and multicast at a UE. | 1. Supports two HARQ-ACK codebooks with different priorities to be simultaneously constructed different priorities for multicast or for unicast and multicast at a UE. | 33-6-1 | Yes | Per UE |
| 33. NR_MBS | 33-6-3 | More than one PUCCH for HARQ-ACK transmission for multicast or for unicast and multicast within a slot | 1. Supports two non-overlapping slot-based PUCCHs for ACK/NACK based HARQ-ACK feedback for multicast or for unicast and multicast with different priorities in a slot. | 33-6-1, 33-6-2 | Yes | Per UE |
| 33. NR_MBS | 33-7 | Supporting group-common DCI indicating the enabling/disabling [ACK/NACK based] HARQ-ACK feedback | 1. Supports the function of group-common DCI indicating the enabling/disabling [ACK/NACK based] HARQ-ACK feedback | 33-2 | Yes | Per UE |

# Conclusions

In this paper, we provided feedback on the initial version of the RAN1 UE feature list for NR MBS.

# References

- RP-193248, New Work Item on NR Support of Multicast and Broadcast Services, Huawei, RAN#86, Sitges, Spain, December 2019.

- R1-2110587, Preliminary RAN1 UE features list for Rel-17 NR

- 3GPP TS 38.211 v16.2.0, NR Physical Channels and Modulation (Release 16)

- 3GPP TS 38.212 v16.2.0, NR Multiplexing and Channel Coding (Release 16)

- 3GPP TS 38.213 v16.2.0, NR Physical Layer Procedures for Control (Release 16)

- 3GPP TS 38.214 v16.2.0, NR Physical Layer Procedures for Data (Release 16)