3GPP TSG RAN WG1 Meeting #107-e	R1-2111536

e-Meeting, November 11 – 19, 2021

Source:	Intel Corporation

Title:	UE features for DSS

Agenda item:	8.16.13

Document for:	Discussion/Decision

# Introduction

In this contribution, we discuss aspects related to UE capability signaling for NR DSS & NR DC.

# Discussion

## NR_DSS

In last RAN1 meeting, updated FG 34-1/34-2 for NR DSS were agreed for further discussion in RAN1#107-e [1], which are provided in Annex A.

For FG 34-2 for Type B UE, there are multiple FFSs which captures potential points that may be defined as UE capability.

- For item 3), it could be updated based on the agreements on max BD/CCE handling Option A in last RAN1 meeting.

- The item 5) is aligned with the agreements. Therefore, it should be included.

- The item 6) and 14) are related. Since it was agreed in last meeting that PCell SCS is not higher than the sSCell SCS, the allowed combinations of (PCell SCS, sSCell SCS) could be (15,15), (15,30) and (15,60) kHz. Since DSS is targeting co-existence with LTE, SCS for PCell can be limited to 15kHz.

- The item 7) can be included as one aspect of 34-2.

- The item 8), 9) may be defined as separate FGs, since the basic feature CCS from sSCell to PCell/PSCell works without support of sSCell dormancy/deactivation.

- Regarding item 12), it was agreed in last meeting unaligned CA can be supported for CCS from sSCell to PCell/PSCell. We prefer to differnet a separate FG for unaligned CA.

For FG 34-1 for Type A UE, there are no new agreements for elaboration. The multiple FFS items can be handled in same way FG 34-2.

- The item 2) could be included since it is aligned with agreed behavior.

- The item 5) is aligned with the agreements. Therefore, it should be included.

- The item 6) and 16) are related. Since it was agreed in last meeting that PCell SCS is not higher than the sSCell SCS, the allowed combinations of (PCell SCS, sSCell SCS) could be (15,15), (15,30) and (15,60) kHz. Since DSS is targeting co-existence with LTE, SCS for PCell can be limited to 15kHz.

- The item 7) can be included as one aspect of 34-1.

- The item 10), 11) may be defined as separate FGs, since the basic feature CCS from sSCell to PCell/PSCell works without support of sSCell dormancy/deactivation. It can be merged with the one for Type B UE.

- Regarding item 14), it was agreed in last meeting unaligned CA can be supported for CCS from sSCell to PCell/PSCell. We prefer to differnet a separate FG for unaligned CA. It can be merged with the one for Type B UE.

Proposal 1: For FG 34-2, it is proposed that

- Revise item 3) based on agreements on max BD/CCE handling Option A in last RAN1 meeting.

- The item 5) & 7) can be included

- For item 6) & 14), it can be merged and the supported combinations of (PCell SCS, sSCell SCS) could be (15,15), (15,30) and (15,60) kHz.

Proposal 2: For FG 34-1, it is proposed that

- The item 2, 5) & 7) can be included

- For item 6) & 16), it can be merged and the supported combinations of (PCell SCS, sSCell SCS) could be (15,15), (15,30) and (15,60) kHz.

Proposal 3:

- Add separate FG for sSCell dormancy with prerequisite FGs 34-1 or 34-2. FFS details

- Add separate FG for sSCell deacitvation with prerequisite FGs 34-1 or 34-2. FFS details

- Add separate FG for unaligned CA with prerequisite FGs 34-1 or 34-2

## LTE_NR_DC_enh2

In last RAN1 meeting, updated FG 35-1 for LTE_NR_DC_enh2 was agreed for further discussion in RAN1#107-e [1], which are provided in Annex B. Since no additional agreement was made in last RAN1 meeting, we may wait for further RAN1/2 progress, including at least any potential down-selection between temporary RS triggering Alt1 or Alt 2.

# Conclusions

In this contribution, we discussed issues related to UE capability signaling for DSS and NR DC. The following is a summary of proposals:

Proposal 1: For FG 34-2, it is proposed that

- Revise item 3) based on agreements on max BD/CCE handling Option A in last RAN1 meeting.

- The item 5) & 7) can be included

- For item 6) & 14), it can be merged and the supported combinations of (PCell SCS, sSCell SCS) could be (15,15), (15,30) and (15,60).

Proposal 2: For FG 34-1, it is proposed that

- The item 2, 5) & 7) can be included

- For item 6) & 16), it can be merged and the supported combinations of (PCell SCS, sSCell SCS) could be (15,15), (15,30) and (15,60).

Proposal 3:

- Add separate FG for sSCell dormancy with prerequisite FGs 34-1 or 34-2. FFS details

- Add separate FG for sSCell deacitvation with prerequisite FGs 34-1 or 34-2. FFS details

- Add separate FG for unaligned CA with prerequisite FGs 34-1 or 34-2

# References

- R1-2109917, “Summary of UE features for DSS” Moderator (AT&T)

- R1-2109636, “On efficient activation/de-activation for SCells” Intel Corporation

- R1-2109637, “On efficient activation/de-activation for SCells” Intel Corporation

# ANNEX A: UE features on NR_DSS [1]

| 34-12 | Cross-carrier scheduling from SCell to PCell/PSCell (Type B) | [Support of Cross-carrier scheduling (CCS) from sSCell to PCell/PSCell (Type B)] - CIF configured on sSCell for CCS Cross-carrier scheduling from sSCell to PCell/PSCell with CIF - sSCell USS set(s) (for CCS from sSCell to PCell/PSCell) and search space sets on PCell/PSCell can be configured so that the UE monitors them in overlapping [slot/symbol] of PCell/PSCell and sSCell - FFS: BD limit handling between [Option A/C] or [Option C] and any configuration of associated parameters and UE reporting of any associated parameters - FFS: #unicast DCI limits for PCell/PSCell scheduling - FFS: PDCCH overbooking on sSCell USS set(s) is not allowed - FFS: Same numerology or different numerologies between sSCell and P(S)Cell - FFS: USS set(s) for DCI format 0_1,1_1,0_2,1_2 (if supported) configured on sSCell for CCS from sSCell to PCell/PSCell - FFS: Support of sSCell deactivation/activation when sSCell cross carrier scheduling to PCell/PSCell is configured - FFS: Support of sSCell dormancy when sSCell cross carrier scheduling to PCell/PSCell is configured - FFS: PDCCH monitoring occasion(s) is within the first 3 OFDM symbols of a PCell/PSCell slot - FFS: Numbers of CORESET configurations and search space sets on sSCell (for PCell/PSCell cross-carrier scheduling) - FFS: frame boundary alignment between PCell/PSCell and sSCell - FFS: Precoder granularity of CORESET size when CCS from sSCell to PCell/PSCell is configured - FFS: Whether the PCell/PSCell SCS other than 15kHz can be allowed Note: The SCell configured with Cross-carrier scheduling to PCell/PSCell is referred to as ‘sSCell’ | FFS 6-5[, 6-10, 18-5, 18-5b, 34-1] | Yes |  |  | Per BC FFS | No | N Applicable to FR1 only FFS |  | sSCell can only be configured in FR1 FFS: UE should be able to indicate following two as part of this per-BC capability: (1) subcarrier spacing for sSCell and/or subcarrier spacing for Pcell, (2) frequency band pair(s) for {PCell/PSCell, sSCell} | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 34-21 | Cross-carrier scheduling from SCell to PCell/PSCell [with search space restrictions] (Type A) | Support of Cross-carrier scheduling from sSCell to PCell/PSCell [with search space restrictions] (Type A) - CIF configured on sSCell for CCS Cross-carrier scheduling from sSCell to PCell/PSCell with CIF - FFS: sSCell USS set(s) (for CCS from sSCell to PCell/PSCell) and at least following search space sets on PCell/PSCell can only be configured such that UE does not monitor them in same [slot/symbol] of PCell/PSCell and sSCell - USS sets for DCI formats 0_1,1_1,0_2,1_2 (if supported) - USS sets for DCI formats 0_0,1_0 - Type3-CSS set(s) for DCI formats 1_0/0_0 with C-RNTI/CS-RNTI/MCS-C-RNTI - FFS: BD limit handling and any configuration of associated parameters and UE reporting of any associated parameters - FFS: #unicast DCI limits for PCell/PSCell scheduling - FFS: PDCCH overbooking on sSCell USS set(s) is not allowed - FFS: Same numerology or different numerologies between sSCell and P(S)Cell - FFS: USS set(s) for DCI format 0_1,1_1,0_2,1_2 (if supported) configured on sSCell for CCS from sSCell to Pcell/PSCell - FFS: sSCell USS set(s) (for CCS from sSCell to Pcell/PSCell) and Type0/0A/1/2 CSS sets on Pcell/PSCell can be configured so that the UE monitors them in overlapping [slot/symbol] of Pcell/PSCell and sSCell. FFS overlap handling - FFS: Support of monitoring DCI formats 0_1,1_1,0_2,1_2 on PCell/PSCell USS set(s) - FFS: Support of sSCell deactivation/activation when sSCell cross carrier scheduling to PCell/PSCell is configured - FFS: Support of sSCell dormancy when sSCell cross carrier scheduling to PCell/PSCell is configured - FFS: PDCCH monitoring occasion(s) is within the first 3 OFDM symbols of a PCell/PSCell slot - FFS: Numbers of CORESET configurations and search space sets on sSCell (for PCell/PSCell cross-carrier scheduling) - FFS: frame boundary alignment between PCell/PSCell and sSCell - FFS: Precoder granularity of CORESET size when CCS from sSCell to PCell/PSCell is configured - FFS: Whether the PCell/PSCell SCS other than 15kHz can be allowed | FFS 6-5[, 6-10, 18-5, 18-5b] | Yes |  |  | Per BC FFS | No | N Applicable to FR1 only FFS |  | sSCell can only be configured in FR1 FFS: UE should be able to indicate following two as part of this per-BC capability: (1) subcarrier spacing for sSCell and/or subcarrier spacing for Pcell, (2) frequency band pair(s) for {PCell/PSCell, sSCell} | Optional with capability signalling |

# ANNEX B: UE features on LTE_NR_DC_enh2 [1]

| 35-1 | [TRS\Temporary] RS [based/for] Scell activation | [Scell activation with the assistance of temporary RS/TRS] - [TRS\Temporary RS] is aperiodic and triggered by MAC CE - FFS: Temporary RS is based on aperiodic TRS and FFS other signals - FFS: Temporary RS is only configured within the BWP indicated by firstActiveDownlinkBWP-Id for the sSCell - FFS: A SSB of the to-be-activated Scell can be indicated as a QCL source for the temporary RS in case of known Scell - FFS: Maximum number of temporary RS resource sets that can be configured to UE per CC {1 … 16} - FFS: Maximum number of temporary RS resource sets that can be configured to UE across CCs {1 … 256} - FFS: Maximum number of triggering states for temporary RS based Scell activation by a MAC-CE {1 … 64} - FFS: Maximum number of temporary RS resource sets that can be associated with a triggering state {1 … 16} [Note: following are reported via the legacy feature, FG2-33 - Maximum number of configured NZP-CSI-RS resources per CC - Maximum total number of simultaneous NZP-CSI-RS resources in active BWPs across all CCs - Maximum number simultaneous NZP-CSI-RS resources per CC - Maximum total number of CSI-RS ports in simultaneous NZP-CSI-RS resources in active BWPs across all CCs] | 6-5 | Yes | N/A |  | [Per UE[/Per BC/per band] | [No/Yes] | [No/Yes] |  |  | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |