3GPP TSG RAN WG1 #107-e       																	    	R1-2111537

e-Meeting, November 11th – 19th, 2021

Source:	Intel Corporation

Title:	On UE features for IOT over NTN

Agenda item:	8.16.14

Document for:	Discussion and Decision

# Introduction

In this contribution, we provide discussion on UE features for IoT-NTN based on the latest proposal in [1]. Table with the details on UE features for IoT-NTN is captured in the Appendix (copied from [1] with proposed changes highlighted by red font).

# Discussion

We have the following comments on the UE feature list proposed in [1].

- For 2-1, component 2-1 “Derive epoch time of serving satellite ephemeris from implicit epoch time” is not needed since this feature shall be implicitly considered as support if UE can receive satellite ephemeris. So, this component is redundant.

- For component 11 in FG 2-1 there is no need to capture the text in brackets “[and updating time and frequency pre-compensation between segments but not within segments during UL repetition]”. This text shall be specified in RAN1 rather than in UE feature list since it defines UE behavior.

- For FG 2-3 components 2 and 3 are not needed since there is no RAN1 feature which corresponds to the frequency and granularity of the report. If such features are needed it shall be discussed in the IoT NTN agenda item rather than in UE feature list discussion.

# Conclusion

In this contribution, we provide discussion on UE features for IoT-NTN. The following proposal is made.

Proposal:

- Consider changes to the UE feature list for IoT-NTN marked below using red font

# References

- R1-2110588, Updated RAN1 UE features list for Rel-17 LTE after RAN1 #106bis-e

# Appendix

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the eNB to know if the feature is supported | [Need for the UE to know if the feature is supported (only for V2X WI, where the PC5-RRC capability signalling is delivered between the UEs)] | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Capability interpretation for mixture of FDD/TDD | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 2. LTE_NBIOT_eMTC_NTN | 2-1 | Basic IoT over NTN support | 1. UE derives its position based on its GNSS implementation [1-1. UE report the validity duration of GNSS] 2. [Indicate/receive] serving Satellite position state vector X,Y,Z in ECEF (m) and serving Satellite velocity state vector VX,VY,VZ in ECEF (m/s) [2-1. Derive epoch time of serving satellite ephemeris from implicit epoch time] 3. Receive ephemeris orbital parameter for the serving satellite 4. UE specific TA calculation in RRC_IDLE state based on its GNSS-acquired position and the serving satellite ephemeris. 5. UE specific TA calculation [in [RRC_IDLE and RRC_CONNECTED state] based on its GNSS-acquired position and the serving satellite ephemeris. 6. UE applies common TA [in RRC_IDLE and RRC_CONNECTED] according to the parameters provided by the network (if any) [7. For TA update in RRC_CONNECTED state, combination of both open (i.e. UE autonomous TA estimation, and common TA estimation) and closed (i.e., received TA commands) control loops X. [Common TA estimation is based on indication of common TA drift rate] 8. In RRC_IDLE state calculate frequency pre-compensation to counter shift the Doppler experienced on the service link. 9. in RRC_CONNECTED state, calculate frequency pre-compensation to counter shift the Doppler experienced on the service link. 10. Support a validity timer of UL synchronization is configured by the network 11. Support segment duration configuration is by the network [and updating time and frequency pre-compensation between segments but not within segments during UL repetition] 12. UE receives cell specific K_offset 13. UE applies K_offset in timing relationship enhancements 14.UE estimates of UE-gNB RTT [15. delaying the starts of ra-ResponseWindow] |  | [No] | N/A | Release 17 UE cannot access [NTN/satellite] |  | No | No | FFS: whether this feature group needs to be separate for eMTC and NB-IoT FFS: Serving satellite ephemeris Epoch time is implicitly known as a reference time defined by the starting time of a DL slot and/or frame. FFS: Whether this starting time is given by predefined rule or it is indicated by the network FFS: differentiation based on orbits such as LEO/MEO/GEO | Optional with capability signalling For UEs supporting NB-IoT/eMTC NTN, it must indicate this FG is supported [Note: This UE feature group is applicable only for IoT-NTN cell, for terrestrial cell this feature is not supported] |
| 2. LTE_NBIOT_eMTC_NTN | 2-2 | Enhancing timing relationships using a time offset | - UE receives UE specific K_offset - 2. UE applies K_offset in timing relationship enhancements | 2-1 [, 2-3] | [No] | N/A | UE does not know the offset to apply for UL transmission |  | No | No | The K_offset is a scheduling offset used for the identified timing relationships that need to be modified for IoT NTN. For IoT NTN, support cell-specific Koffset configuration for use during initial access. For IoT NTN, support the use of UE-specific Koffset in CONNECTED mode. FFS: whether this feature group needs to be separate for eMTC and NB-IoT FFS: differentiation based on orbits such as LEO/MEO/GEO | Optional with capability signalling FFS: For UEs supporting NB-IoT/eMTC NTN, it must indicate this FG is supported |
| 2. LTE_NBIOT_eMTC_NTN | [2-3] | [UE reporting of information about the UE specific TA pre-compensation] | - [The exact content of UE reporting of information about the UE specific TA pre-compensation] - [frequency of the reports] - [granularity of the reported content] | [2-1] | No | No | UL scheduling for FDD-HD: Use of UE-specific TA and/or K_offset to avoid UL-DL collisions in FDD-HD |  | No | No | UE-specific TA reporting is supported in IoT-NTN FFS: Detailed contents of report FFS: whether this feature group needs to be separate for eMTC and NB-IoT FFS: differentiation based on orbits such as LEO/MEO/GEO | Optional with capability signalling FFS: For UEs supporting NB-IoT/eMTC NTN, it must indicate this FG is supported |