3GPP TSG RAN WG1 Meeting #107-e	R1-2111710

e-Meeting, November 11th – 19th, 2021

Title:	Draft reply LS on initial state of elements controlled by MAC CEs

Response to:	LS on initial state of elements controlled by MAC CEs (R2-2109098)

Release:	Rel-15, Rel-16

Work Item:	NR_newRAT-Core

Source:	RAN1

To:	RAN2

Cc:

Contact Person:

Name:

E-mail Address:

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments: 	-

RAN2 has discussed the handling of the radio resources/states which are de-/activated by MAC CEs. RAN2 has identified some unclarities in 38.321. Since the corresponding text was based on the MAC CEs from RAN1, RAN2 would like to check the following UE behaviours with RAN1.

--------------------------------------------------------------From 38.321 ----------------------------------------------------------------

..

5.18.2 The configured Semi-persistent CSI-RS/CSI-IM resource sets are initially deactivated upon configuration and after a handover.

..

5.18.4 The configured TCI states for PDSCH are initially deactivated upon configuration and after a handover.

..

5.18.6 The configured Semi-persistent CSI reporting on PUCCH is initially deactivated upon configuration and after a handover.

..

5.18.7 The configured Semi-persistent SRS resource sets are initially deactivated upon configuration and after a handover.

..

5.18.9 The configured Semi-persistent ZP CSI-RS resource sets are initially deactivated upon configuration and after a handover.

5.18.17 The configured resource sets Semi-persistent Positioning SRS are initially deactivated upon configuration and after a handover.

--------------------------------------------------------------From 38.321 ----------------------------------------------------------------

Firstly, since a handover only refers to PCell change, RAN2 respectfully asks RAN1 to clarify whether the initial deactivation when using handover should be applied for both PCell change and PSCell change/addition in case of DC.

A1: Since the thing is whether the configured elements are initially deactivated or not, our view is that the initial deactivation can be applied for PSCell change/addition in case of DC as well as for PCell change.

Secondly, RAN2 respectfully asks RAN1 to clarify whether the initial deactivation when using configuration should be applied for both “initial configuration by RRC” and “reconfiguration by RRC”.

A2: Regarding reconfiguration, if related RRC parameters are changed by reconfiguration, then it should be also applied when using configuration.

Thirdly, RAN2 respectfully asks RAN1 to clarify whether the UE behavior relevant to (Enhanced) PUCCH spatial relation Activation/Deactivation MAC CE should be aligned with the other MAC CEs.

A3: It seems that there is no difference between (Enhanced) PUCCH spatial relation Activation/Deactivation MAC CE and other MAC CEs.

2. Actions:

To RAN2

ACTION: 	RAN1 respectfully asks RAN2 to take the above information into account in their further work

3. Date of Next Meetings:

TSG-RAN WG1 Meeting #107bis-e 	17 – 25, January, 2022			e-Meeting

TSG-RAN WG1 Meeting #108-e 	21, February – 4, March, 2022			e-Meeting