3GPP TSG RAN WG1 #107-e			R1-2111721

e-Meeting, November 11th – 19th, 2021

Agenda item:	8.1.2.4

Source: 	Samsung

Title: 	Enhancements on HST-SFN

Document for:	Discussion and Decision

Introduction

In Rel.17 NR FeMIMO WID, multi-TRP enhancements for HST-SFN are given as follows [1]:

Enhancement on the support for multi-TRP deployment, targeting both FR1 and FR2: - Identify and specify features to improve reliability and robustness for channels other than PDSCH (that is, PDCCH, PUSCH, and PUCCH) using multi-TRP and/or multi-panel, with Rel.16 reliability features as the baseline - Identify and specify QCL/TCI-related enhancements to enable inter-cell multi-TRP operations, assuming multi-DCI based multi-PDSCH reception - Evaluate and, if needed, specify beam-management-related enhancements for simultaneous multi-TRP transmission with multi-panel reception - Enhancement to support HST-SFN deployment scenario: - Identify and specify solution(s) on QCL assumption for DMRS, e.g. multiple QCL assumptions for the same DMRS port(s), targeting DL-only transmission - Evaluate and, if the benefit over Rel.16 HST enhancement baseline is demonstrated, specify QCL/QCL-like relation (including applicable type(s) and the associated requirement) between DL and UL signal by reusing the unified TCI framework

This contribution provides Samsung’s view on the highlighted topic above.

SFN transmission of PDCCH/PDSCH from multi-TRP

  - Default spatial relation and PL-RS for PUCCH, PUSCH, and SRS

In RAN1#106-e [3], the following agreement was made about the default spatial relation and PL-RS for PUCCH, PUSCH and SRS considering SFN PDCCH transmission.

Agreement If enhanced SFN PDCCH transmission scheme (scheme 1 or if TRP-based pre-compensation is supported in FR2) is configured, and if the CORESET with the lowest ID in the active DL BWP is indicated with two TCI states - If PL-RS and spatial relation information are not configured for PUCCH and enableDefaultBeamPL-ForPUCCH is configured in FR2 - For single-TRP PUCCH transmission, select the first TCI state of the CORESET as default beam and PL RS - If PUSCH scheduled by DCI format 0_0 and enableDefaultBeamPL-ForPUSCH0-0 is configured in FR2, and if PUCCH resource is not configured on active UL BWP in the cell or if spatial relation is not configured in any PUCCH resource on active UL BWP in the cell, - For single-TRP PUSCH transmission scheduled by DCI format 0_0, select the first TCI state of the CORESET as default beam and PL RS - If PL-RS and spatial relation information are not configured for SRS and enableDefaultBeamPL-ForSRS is configured in FR2 - For single-TRP SRS resource, select the first TCI state of the CORESET as default beam and PL RS - FFS other details, if any - These are UE optional features

Regarding SFN transmission of PDCCH from multi-TRP, the current default spatial relation and pathloss reference RS (PL-RS) for PUCCH, PUSCH, and SRS would be affected. As a result of the possibility of SFN transmission of PDCCH from multi-TRP, the existing method based on current specification to derive a default spatial setting and PL-RS for PUCCH, PUSCH, and SRS cannot directly be applied. This is a direct consequence of the existence of multiple TCI states associated to the SFNed CORESET with the lowest ID. Therefore, it is required to resolve this ambiguity.

However, the current specification considers the only case of a single default spatial relation and PL-RS for PUCCH, PUSCH, and SRS. In Rel-17, the multiple default spatial relations and PL-RSs can be also considered at least for multi-TRP based PUCCH and PUSCH repetition. For PUCCH or PUSCH repetition intended towards multi-TRP, if certain conditions are fulfilled, it is natural to select multiple default spatial relations so that each spatial relation can be mapped to the PUCCH or PUSCH repetitions towards a corresponding TRP similar with two default TCI states in Rel-16 PDSCH TDMed repetition. Otherwise, for example, if a single default PL-RS is selected based on only one TCI state (spatial relation from a certain TRP), then the computed PL may not be accurate to compensate PL in PUCCH or PUSCH repetitions towards the remaining TRPs. Similarly, if a spatial setting is selected based on a single TCI states, then the PUCCH transmissions towards the remaining TRPs may not be correctly beamformed. Therefore, it is essential to consider multiple default beams for multi-TRP based PUCCH or PUSCH repetitions.

Proposal 1. Support multiple default spatial relations and PL-RSs at least for PUCCH and PUSCH for both SFN transmission of PDCCH and multi-TRP PUCCH/PUSCH repetition.

  - Default TCI for single-TRP PDSCH reception

When we consider default TCI for single-TRP PDSCH reception, the case when CORESET and PDSCH is overlapped can be also considered, which is captured at Clause 5.1.5 in TS38.214 highlighted as follows.

Independent of the configuration of tci-PresentInDCI and tci-PresentDCI-1-2 in RRC connected mode, if the offset between the reception of the DL DCI and the corresponding PDSCH is less than the threshold timeDurationForQCL and at least one configured TCI state for the serving cell of scheduled PDSCH contains qcl-Type set to 'typeD', - the UE may assume that the DM-RS ports of PDSCH(s) of a serving cell are quasi co-located with the RS(s) with respect to the QCL parameter(s) used for PDCCH quasi co-location indication of the CORESET associated with a monitored search space with the lowest controlResourceSetId in the latest slot in which one or more CORESETs within the active BWP of the serving cell are monitored by the UE. In this case, if the qcl-Type is set to 'typeD' of the PDSCH DM-RS is different from that of the PDCCH DM-RS with which they overlap in at least one symbol, the UE is expected to prioritize the reception of PDCCH associated with that CORESET. This also applies to the intra-band CA case (when PDSCH and the CORESET are in different component carriers).

Similar with the highlighted sentence above, we can consider the case when the CORESET, which is activated with one or two TCI states, is overlapped with the scheduled single-TRP PDSCH. This can be extended to the intra-band CA case as well. To make the specification as a complete one, our view is to consider the overlapped CORESET which is activated one or two TCI states and is located in same or different component carrier with the scheduled single-TRP PDSCH.

Proposal 2. Support to consider the case when the CORESET, which is overlapped with the scheduled single-TRP PDSCH reception in same carrier or intra-band CA, is activated one or two TCI states.

  - Default TCI for Rel-17 SFN PDSCH reception

In RAN1#106-e [3], the following agreement was made to re-use the existing rule onto the default TCI for Rel-17 SFN PDSCH reception.

Agreement If enableTwoDefaultTCI-States is configured and at least one TCI codepoint indicates two TCI states and time offset between the reception of the DL DCI and the PDSCH is less than the threshold timeDurationForQCL, default beam(s) for Rel-17 enhanced SFN PDSCH (scheme 1 or if supported TRP-based pre-compensation) reception: - Alt 1: Reuse rule to determine TCI states as defined for Rel-16 PDSCH scheme-1a This is a UE optional feature

Similar with determining the default TCI for single-TRP PDSCH reception as in the current specification, we also consider the case when a CORESET and the scheduled PDSCH is overlapped, which is captured at Clause 5.1.5 in TS38.214 highlighted as follows. Even in the reception for Rel-17 SFN PDSCH, the case for the CORESET, which is activated one or two TCI states, is overlapped and located in same or different component carrier with the scheduled Rel-17 SFN PDSCH should be also considered.

Independent of the configuration of tci-PresentInDCI and tci-PresentDCI-1-2 in RRC connected mode, if the offset between the reception of the DL DCI and the corresponding PDSCH is less than the threshold timeDurationForQCL and at least one configured TCI state for the serving cell of scheduled PDSCH contains qcl-Type set to 'typeD', … - If a UE is configured with enableTwoDefaultTCI-States, and at least one TCI codepoint indicates two TCI states, the UE may assume that the DM-RS ports of PDSCH or PDSCH transmission occasions of a serving cell are quasi co-located with the RS(s) with respect to the QCL parameter(s) associated with the TCI states corresponding to the lowest codepoint among the TCI codepoints containing two different TCI states. When the UE is configured by higher layer parameter repetitionScheme set to 'tdmSchemeA' or is configured with higher layer parameter repetitionNumber, and the offset between the reception of the DL DCI and the first PDSCH transmission occasion is less than the threshold timeDurationForQCL, the mapping of the TCI states to PDSCH transmission occasions is determined according to clause 5.1.2.1 by replacing the indicated TCI states with the TCI states corresponding to the lowest codepoint among the TCI codepoints containing two different TCI states based on the activated TCI states in the slot with the first PDSCH transmission occasion. In this case, if the 'QCL-TypeD' in both of the TCI states corresponding to the lowest codepoint among the TCI codepoints containing two different TCI states is different from that of the PDCCH DM-RS with which they overlap in at least one symbol, the UE is expected to prioritize the reception of PDCCH associated with that CORESET. This also applies to the intra-band CA case (when PDSCH and the CORESET are in different component carriers)

Proposal 3. Support to consider the case when the CORESET, which is overlapped with the scheduled Rel-17 SFN PDSCH reception in same carrier or intra-band CA, is activated one or two TCI states.

  - Remaining issues on default TCI for Rel-17 SFN PDCCH/PDSCH

We would like to deal with some remaining issues on default TCI considering Rel-17 SFN PDCCH/PDSCH.

The following two agreements were made in RAN1#106-e and RAN1#106b-e, respectively, which are related to default beam operation for PDSCH when scheduling offset is less than the threshold timeDurationForQCL.

Agreement If enableTwoDefaultTCI-States is configured and at least one TCI codepoint indicates two TCI states and time offset between the reception of the DL DCI and the PDSCH is less than the threshold timeDurationForQCL, default beam(s) for Rel-17 enhanced SFN PDSCH (scheme 1 or if supported TRP-based pre-compensation) reception: - Alt 1: Reuse rule to determine TCI states as defined for Rel-16 PDSCH scheme-1a This is a UE optional feature Agreement When SFN PDSCH is not configured by RRC, for PDSCH reception scheduled by DCI format 1_0, 1_1, 1_2, if the time offset between the reception of the DL DCI and the corresponding PDSCH is smaller than the threshold timeDurationForQCL, - For DCI format 1_1/1_2, support both configurations with and without TCI state field. - [If enableTwoDefaultTCIStates is not configured,] for both cases with and without TCI state field, - If enhanced SFN PDCCH transmission scheme 1 is configured and the lowest CORESET ID in the latest slot is indicated with two TCI states, select the 1st TCI state of the two TCI states of the CORESET as default beam for the PDSCH reception - FFS: Whether above applies for TRP-based pre-compensation if TRP-based pre-compensation is agreed to be support in FR2 - Otherwise, UE applies the one active TCI state of the CORESET with the lowest controlResourceSetId in the latest slot when receiving the PDSCH

Regarding the second agreement above, we support to remove “[If enableTwoDefaultTCIStates is not configured]” since the agreed default beam behavior can be applied, i.e., using a single TCI as a default beam, regardless of configuration of enableTwoDefaultTCIStates.

Proposal 4. Support to remove “[If enableTwoDefaultTCIStates is not configured]” in previous agreement.

Also, it seems that above agreements do not cover the scenario when PDSCH is RRC configured with SFN scheme and enableTwoDefaultTCIStates is not configured. In order to make complete specification, in such scenario, we support the similar default beam behavior with the second agreement above.

Proposal 5. When SFN PDSCH is configured by RRC, for PDSCH reception scheduled by DCI format 1_0, 1_1, 1_2, if the time offset between the reception of the DL DCI and the corresponding PDSCH is smaller than the threshold timeDurationForQCL

- For DCI format 1_1/1_2, support both configurations with and without TCI state field.

- If enableTwoDefaultTCIStates is not configured, for both cases with and without TCI state field,

  - If enhanced SFN PDCCH transmission scheme 1 is configured and the lowest CORESET ID in the latest slot is indicated with two TCI states, select the 1st TCI state of the two TCI states of the CORESET as default beam for the PDSCH reception

    - FFS: Whether above applies for TRP-based pre-compensation if TRP-based pre-compensation is agreed to be support in FR2

  - Otherwise, UE applies the one active TCI state of the CORESET with the lowest controlResourceSetId in the latest slot when receiving the PDSCH

Further, there was a possible agreement discussed in RAN1#106b-e as follows:

Possible Agreement For PDSCH reception scheduled by DCI format 1_0, [1_1 and 1_2] and if applicable the time offset between the reception of the DL DCI and the corresponding PDSCH is equal or larger than the threshold timeDurationForQCL - Support configuration when there is no TCI field in the DCI scheduling PDSCH, when SFN is configured for both PDCCH and PDSCH by RRC - UE applies the TCI state(s) of the scheduling CORESET when receiving the PDSCH - if there are two active TCI states for the CORESET, UE applies the both QCL assumptions of the CORESET that schedules the PDSCH when receiving the PDSCH - otherwise, if there is one active TCI states for the CORESET, UE applies the one active TCI state of the CORESET when receiving the PDSCH FFS : UE behavior if the UE does not support the above

Generally, we support the above possible agreement, and also fine for removing FFS. Regarding UE behavior if the UE does not support above, we would like to support that TCI field is always present in DCI format 1_1 and 1_2.

Moreover, we would like to discuss additionally about the configuration if PDCCH is not SFN configured but PDSCH is RRC configured with SFN scheme. In this case, when time offset is equal to or larger than the threshold timeDurationForQCL, UE supports the configuration which there is no TCI field in the DCI scheduling PDSCH. Then, UE applies the one active TCI state of the scheduling CORESET when receiving the PDSCH, which is the same solution as the above possible agreement if there is one active TCI state for the CORESET although PDCCH is SFN configured.

Proposal 6. Support the possible agreement in principle and prefer to discuss additionally the following.

- Removing FFS and add UE behavior “TCI field is always present in DCI format 1_1 and 1_2” if the UE does not support the above.

- Consider the configuration if PDCCH is not SFN configured but PDSCH is RRC configured with SFN scheme. In this case, support the configuration which there is no TCI field in the DCI scheduling PDSCH when time offset is equal to or larger than the threshold timeDurationForQCL, and UE applies the one active TCI state of the scheduling CORESET when receiving the PDSCH.

Further, there was a proposal#4-7 discussed in RAN1#106b-e as follows:

Proposal #4-7: Support configuration of DCI format 1_0/1_1/1_2 without TCI state field for PDSCH transmission using SFN scheme in FR1 - Reuse default TCI states agreed for FR2, i.e., UE applies TCI state(s) of the scheduling CORESET when receiving the PDSCH - if there are two active TCI states for the CORESET, UE applies both QCL assumption of the CORESET that schedules the PDSCH when receiving the PDSCH - otherwise, UE applies the one active TCI state of the CORESET when receiving the PDSCH

The intention of this proposal is to enable the configuration which there is no TCI field in the DCI scheduling PDSCH in FR1 as well. It seems that the above proposal #4-7 includes the case when PDSCH is SFN configured and both when PDCCH is SFN configured or not. Hence, in order to make complete specification, we also would like to discuss the case when PDSCH is not SFN configured. In this case, if there are two active TCI states for the scheduling CORESET, the UE applies the 1st TCI state of the two TCI states of the CORESET for the PDSCH reception. Also, if there is a single TCI state for the CORESET, the UE applies the one TCI state of the CORESET for the PDSCH reception, which is current specification.

Proposal 7. Support the proposal #4-7 in previous meeting, and prefer to discuss additionally the case when PDSCH is not SFN configured.

- In this case, if there are two active TCI states for the scheduling CORESET, the UE applies the 1st TCI state of the two TCI states of the CORESET for the PDSCH reception.

  - PDCCH monitoring with different QCL-TypeD

In RAN1#106-e [3], the following agreement was made for identifying two QCL-TypeD properties for multiple overlapping CORESETs which is an extension of the existing QCL prioritization rule for PDCCH monitoring.

Agreement When a CORESET is activated with two TCI states which overlaps with another CORESET, support extension of Rel-15 prioritization rule for PDCCH monitoring of PDCCH candidates in overlapping monitoring occasions with different QCL-TypeD - FFS: Prioritization rule considers CORESETs indicated with 1 and/or 2 TCI states - Supports identifying two QCL-TypeD properties for multiple overlapping CORESETs - UE capability is introduced - FFS other details - FFS: Strive to have same / similar solution as discussed under AI 8.1.2.1

This enhancement is necessary since UE monitors PDCCH only in a CORESET having the same QCL-TypeD as the CORESET determined from the priority rule defined in TS38.213 as follows.

- the CORESET corresponds to the CSS set with the lowest index in the cell with the lowest index containing CSS, if any; otherwise, to the USS set with the lowest index in the cell with lowest index

- the lowest USS set index is determined over all USS sets with at least one PDCCH candidate in overlapping PDCCH monitoring occasions

We realized that if we adopt a priority order as SS type > the number of TCI states for CORESET > serving cell index > SS set ID, i.e., prioritize a SFN CORESET to a single TCI CORESET, this may make a new priority rule that a CORESET associated with Type 3 CSS may have higher priority than a CORESET associated with CSS other than Type 3. This will happen regardless of the decision for the possible CSS types which can be associated with SFN CORESET, since CSSs other than Type 3 do not use the second QCL-TypeD. This is not aligned with legacy operation and the agreed QCL prioritization rule from AI 8.1.2.1 PDCCH repetition. Hence, in order to keep the same priority among CSS types and also align with the agreed QCL prioritization rule from AI 8.1.2.1 PDCCH repetition, if at least one SFN CORESET exists among multiple overlapping CORESETs, we support the priority order as SS type > serving cell index > SS set ID > the number of TCI states for CORESET. If prioritized CORESET has two TCI states (i.e., SFN CORESET), then two TCI states of the CORESET are used to monitor. However, if prioritized CORESET has a single TCI state, then the second QCL-TypeD is identified by the first TCI state of a CORESET with the second highest priority. In case of no SFN CORESET among multiple overlapping CORESETs, then a second QCL-TypeD is not determined.

Proposal 8. When a CORESET is activated with two TCI states which overlaps with another CORESET, support PDCCH monitoring of PDCCH candidates in overlapping monitoring occasions with QCL-TypeD properties identified according to prioritization rule

- SS type > serving cell index > SS set ID > the number of TCI states for CORESET

- If prioritized CORESET has one TCI state, the second QCL type D is identified by the first TCI state of a CORESET with the second highest priority

- Note: SS type with CSS has higher priority than SS type with USS, SS set with lower index has higher priority than SS set with higher index, serving cell with lower index has higher priority than serving cell index with higher index, two TCI states for CORESET has higher priority than one TCI state

- In case of no SFN CORESET among multiple overlapping CORESETs, then a second QCL-TypeD is not determined.

If the priority considering two activated TCI states for a CORESET is defined and the corresponding CORESET with the highest priority (i.e., reference CORESET) is activated with two TCI states, it should be determined which CORESETs other than the reference CORESET can be monitored considering the highlighted word “same” in the following part of the Clause 10.1 in TS 38.213.

If a UE - is configured for single cell operation or for operation with carrier aggregation in a same frequency band, and - monitors PDCCH candidates in overlapping PDCCH monitoring occasions in multiple CORESETs that have been configured with same or different qcl-Type set to 'typeD' properties on active DL BWP(s) of one or more cells the UE monitors PDCCHs only in a CORESET, and in any other CORESET from the multiple CORESETs that have been configured with qcl-Type set to same 'typeD' properties as the CORESET, on the active DL BWP of a cell from the one or more cells

If the reference CORESET is activated with TCI states, e.g., TCI states (a, b), our view is that the UE can monitor PDCCHs in any other CORESET from the multiple CORESETs that have been configured with TCI state(s) (a, b), a only, or b only on the active DL BWP of a cell from the one or more cells. That means, the word in current specification “same” should include all the subset (i.e., (a,b), a, and b) of all TCI states of the reference CORESET.

Proposal 9. For the QCL prioritization rule for PDCCH reception, if a UE monitors a first CORESET with TCI states (a, b) from multiple CORESETs, the UE also monitors any other CORESETs with TCI state(s) (a,b), a, or b which is activated with the subset of TCI states from the reference CORESET.

  - CSS associated with SFN CORESET

In RAN1#106b-e, the following agreement was agreed to down-select among two alternatives for CSS associated with SFN CORESET.

Agreement For CSS associated with SFN CORESET, study the following alternatives and down-select in RAN1#107e: - Alt 2: UE doesn’t expect PDCCH candidates in CSS to be associated with CORESET activated with two TCI states, except for CSS type 3 associated with CORESET configured with scheme 1 - Alt 3: If PDCCH candidates in CSS 0/0A/1/2/3 are associated with CORESET that activated with two TCI states, the first TCI state is applied for the CSS reception, except for CSS type 3 associated with CORESET configured with scheme 1. - For CSS type 3 associated with CORESET configured with scheme 1, both TCI states can be applied for the CSS reception.

Since RAN1 has already agreed that Type 3 CSS is only supported for PDCCH repetition among all types, i.e., linkage between CSSs are only valid for Type 3 CSS. In order to make consistent design for PDCCH repetition and SFN PDCCH, we support Alt.2.

Proposal 10. Support Alt.2 for CSS associated with SFN CORESET (i.e., only Type 3 CSS).

  - RLM RS selection considering CORESET activated with two TCI states

Based on the enhancement on Rel-17 HST-SFN, the number of activated TCI states can be extended as 2. The motivation on some enhancements, e.g., defining new default beam behaviors (for PUCCH, PUSCH, SRS, and PDSCH) BFD RS, and QCL prioritization rule, is to clarify whether the existing rule considering CORESET with a single TCI state is extended or not when two TCI states are activated in CORESET.

One more possible enhancement is RLM RS selection rule considering two activated TCI states for CORESET. In current specification, When RadioLinkMonitoringRS is not configured, there is a rule to select RLM RSs among the source RS of TCI state for CORESETs which have a single TCI. The selection priority depends on the periodicity of search space set first (the shortest periodicity has the highest selection priority) and if periodicities are same, then the index of CORESET can be used to determine a selection priority (the highest CORESET ID has the highest selection priority). Hence, if two TCI states are activated for all CORESETs or some CORESETs among all CORESETs, it should be studied whether/how to enhance RLM RS selection rule (e.g., the number of activated TCI states can be considered as an additional standard for selection priority).

Proposal 11. Study whether/how to enhance RLM RS selection rule considering CORESET activated with two TCI states.

Conclusion

In this contribution, the following observations and proposals are given:

Proposal 1. Support multiple default spatial relations and PL-RSs at least for PUCCH and PUSCH for both SFN transmission of PDCCH and multi-TRP PUCCH/PUSCH repetition.

Proposal 2. Support to consider the case when the CORESET, which is overlapped with the scheduled single-TRP PDSCH reception in same carrier or intra-band CA, is activated one or two TCI states.

Proposal 3. Support to consider the case when the CORESET, which is overlapped with the scheduled Rel-17 SFN PDSCH reception in same carrier or intra-band CA, is activated one or two TCI states.

Proposal 4. Support to remove “[If enableTwoDefaultTCIStates is not configured]” in previous agreement.

Proposal 5. When SFN PDSCH is configured by RRC, for PDSCH reception scheduled by DCI format 1_0, 1_1, 1_2, if the time offset between the reception of the DL DCI and the corresponding PDSCH is smaller than the threshold timeDurationForQCL

- For DCI format 1_1/1_2, support both configurations with and without TCI state field.

- If enableTwoDefaultTCIStates is not configured, for both cases with and without TCI state field,

  - If enhanced SFN PDCCH transmission scheme 1 is configured and the lowest CORESET ID in the latest slot is indicated with two TCI states, select the 1st TCI state of the two TCI states of the CORESET as default beam for the PDSCH reception

    - FFS: Whether above applies for TRP-based pre-compensation if TRP-based pre-compensation is agreed to be support in FR2

  - Otherwise, UE applies the one active TCI state of the CORESET with the lowest controlResourceSetId in the latest slot when receiving the PDSCH

Proposal 6. Support the possible agreement in principle and prefer to discuss additionally the following.

- Removing FFS and add UE behavior “TCI field is always present in DCI format 1_1 and 1_2” if the UE does not support the above.

- Consider the configuration if PDCCH is not SFN configured but PDSCH is RRC configured with SFN scheme. In this case, support the configuration which there is no TCI field in the DCI scheduling PDSCH when time offset is equal to or larger than the threshold timeDurationForQCL, and UE applies the one active TCI state of the scheduling CORESET when receiving the PDSCH.

Proposal 7. Support the proposal #4-7 in previous meeting, and prefer to discuss additionally the case when PDSCH is not SFN configured.

- In this case, if there are two active TCI states for the scheduling CORESET, the UE applies the 1st TCI state of the two TCI states of the CORESET for the PDSCH reception.

Proposal 8. When a CORESET is activated with two TCI states which overlaps with another CORESET, support PDCCH monitoring of PDCCH candidates in overlapping monitoring occasions with QCL-TypeD properties identified according to prioritization rule

- SS type > serving cell index > SS set ID > the number of TCI states for CORESET

- If prioritized CORESET has one TCI state, the second QCL type D is identified by the first TCI state of a CORESET with the second highest priority

- Note: SS type with CSS has higher priority than SS type with USS, SS set with lower index has higher priority than SS set with higher index, serving cell with lower index has higher priority than serving cell index with higher index, two TCI states for CORESET has higher priority than one TCI state

- In case of no SFN CORESET among multiple overlapping CORESETs, then a second QCL-TypeD is not determined.

Proposal 9. For the QCL prioritization rule for PDCCH reception, if a UE monitors a first CORESET with TCI states (a, b) from multiple CORESETs, the UE also monitors any other CORESETs with TCI state(s) (a,b), a, or b which is activated with the subset of TCI states from the reference CORESET.

Proposal 10. Support Alt.2 for CSS associated with SFN CORESET (i.e., only Type 3 CSS).

Proposal 11. Study whether/how to enhance RLM RS selection rule considering CORESET activated with two TCI states.

References

- 3GPP RAN, RP-193133, Samsung, New WID: further enhancements on MIMO for NR

- Chairman’s notes, RAN1#105-e

- Chairman’s notes, RAN1#106-e