3GPP TSG RAN WG1 #107-e			R1-2111738

e-Meeting, November 11th – 19th, 2021

Agenda Item:		8.5.1

Source:				Samsung

Title:					Discussion on accuracy improvements by mitigating UE Rx/Tx and/or gNB Rx/Tx timing delays

Document for:	Discussion

# Introduction

- Specify methods, measurements, signalling, and procedures for improving positioning accuracy of the Rel-16 NR positioning methods by mitigating UE Rx/Tx and/or gNB Rx/Tx timing delays, including [RAN1]

- DL, UL and DL+UL positioning methods

- UE-based and UE-assisted positioning solutions

- Specify methods, measurements, signalling, and procedures for improving positioning accuracy of the Rel-16 NR positioning methods by mitigating UE Rx/Tx and/or gNB Rx/Tx timing delays, including [RAN1]

- DL, UL and DL+UL positioning methods

- UE-based and UE-assisted positioning solutions

In the approved new WI for positioning enhancement [1], one important direction is to refine the timing based estimation.

This contribution discusses the accuracy improvements on timing based positioning solutions especially for the timing error and timing error group.

# Handling of Timing error

At RAN1#104 meeting, UE Rx/Tx errors were discussed. RAN1 agreed on a definition of timing errors:

- Tx timing error: From a signal transmission perspective, there will be a time delay from the time when the digital signal is generated at baseband to the time when the RF signal is transmitted from the Tx antenna. For supporting positioning, the UE/TRP may implement an internal calibration/compensation of the Tx time delay for the transmission of the DL PRS/UL SRS signals, which may also include the calibration/compensation of the relative time delay between different RF chains in the same TRP/UE. The compensation may also possibly consider the offset of the Tx antenna phase center to the physical antenna center. However, the calibration may not be perfect. The remaining Tx time delay after the calibration, or the uncalibrated Tx time delay is defined as Tx timing error.

- Rx timing error: From a signal reception perspective, there will be a time delay from the time when the RF signal arrives at the Rx antenna to the time when the signal is digitized and time-stamped at the baseband. For supporting positioning, the UE/TRP may implement an internal calibration/compensation of the Rx time delay before it reports the measurements that are obtained from the DL PRS/UL SRS signals, which may also include the calibration/compensation of the relative time delay between different RF chains in the same TRP/UE. The compensation may also possibly consider the offset of the Rx antenna phase center to the physical antenna center. However, the calibration may not be perfect. The remaining Rx time delay after the calibration, or the uncalibrated Rx time delay is defined as Rx timing error.

In addition, RAN1 introduced the concept of Timing Error Group (TEG):

- UE Tx ‘timing error group’ (UE Tx TEG): A UE Tx TEG is associated with the transmissions of one or more UL SRS resources for the positioning purpose, which have the Tx timing errors within a certain margin.

- TRP Tx ‘timing error group’ (TRP Tx TEG): A TRP Tx TEG is associated with the transmissions of one or more DL PRS resources, which have the Tx timing errors within a certain margin.

- UE Rx ‘timing error group’ (UE Rx TEG): A UE Rx TEG is associated with one or more DL measurements, which have the Rx timing errors within a certain margin.

- TRP Rx ‘timing error group’ (TRP Rx TEG): A TRP Rx TEG is associated with one or more UL measurements, which have the Rx timing errors within a margin.

- UE RxTx ‘timing error group’ (UE RxTx TEG): A UE RxTx TEG is associated with one or more UE Rx-Tx time difference measurements, and one or more UL SRS resources for the positioning purpose, which have the ‘Rx timing errors+Tx timing errors’ within a certain margin.

- TRP RxTx ‘timing error group’ (TRP RxTx TEG): A TRP RxTx TEG is associated with one or more gNB Rx-Tx time difference measurements and one or more DL PRS resources, which have the ‘Rx timing errors+Tx timing errors’ within a certain margin.

In this section, we discuss TEG for DL TDOA, UL TDOA and Multi-RTT with the corresponding impacts on the measurement or the signaling for positioning.

## UE RxTx TEG for Multi-RTT

### Reporting of UE Rx/Tx TEG IDs

In RAN1#105 meeting, there were agreements related to Rx/Tx TEG for Multi-RTT. The agreements were further revised in RAN1#106 as follows:

Agreement: Make the following modification of the previous agreement: For mitigating UE Tx/Rx timing errors for DL+UL positioning, a UE may should support, up to UE capability, either one or both of the following options: - Option 1: Reporting of UE RxTx TEG ID is supported by the UE - FFS: Further details on how the UE RxTx TEG IDs are related/associated to UE Tx TEG IDs and/or UE Rx TEG IDs and to the UE Rx-Tx measurements. - Option 2: Reporting of UE RxTx TEG ID is not supported by the UE; reporting of UE Rx TEG ID and UE Tx TEG ID is supported. - In either option, a UE Tx TEG ID is associated with (downselection needed) - Alt. 1: an UL SRS resource for positioning corresponding to the Tx timing of the UE Rx-Tx measurement - Alt. 2: the Tx timing of the UE Rx-Tx measurement - Alt. 3: one or more UL SRS resources for positioning - Note: An UE Rx TEG ID is associated with one DL PRS resource (or more DL PRS resources) corresponding to the Rx time of the measurement - FFS: How to resolve potential mismatch between UE and gNB Rx-Tx time difference measurements (e.g. UE provides the UE Rx-Tx measurements associated with a Tx TEG with SRS1, while gNB provides the gNB Rx-Tx measurements with a Rx TEG associated with SRS2). - FFS: The potential impact and modification on the definition of Rx-Tx time difference measurements

For the issue of indicating the Rx/Tx TEG in multi-RTT method, from our understanding, both options can work. However, they might be suitable for different use cases or UE capability. For instance, if the UE is capable of generating the overall TEG, known as the RxTx TEG, the RxTx TEG ID could be provided. However, if the UE is unable to do it, it can generate the Tx TEG and Rx TEG separately. Thus, it is beneficial to support both reporting options.

Proposal 1: Both options for UE TEG reporting (i.e., reporting the UE RxTx TEG ID or reporting both UE Rx TEG ID and UE Tx TEG ID) are supported for DL+UL positioning subject to the UE capability.

Another point to discuss is to what information the Tx TEG ID is associated with. , First of all, in our understanding,  Alt.2 was excluded based on the following agreement related to UE Rx-Tx time difference measurement reached in RAN1#106:

Agreement: - If a Tx TEG ID is reported with a UE Rx-Tx time difference measurement, the UE should also report the association of the Tx TEG ID to the UL SRS resource(s) - FFS: how the the association of the Tx TEG ID to the UL SRS resource(s) is determined by UE. - FFS: details of the signalling Agreement: If a RxTx TEG ID is reported with a UE Rx-Tx time difference measurement, the UE may optionally also report a Tx TEG ID.

Given the above two agreements, if a UE reports RxTx TEG, the UE can then ignore the reporting of Tx TEG for DL+UL positioning methods, and if the Tx TEG ID is reported, it should be associated with the UL SRS resources. This thus excludes Alt 2 in which Tx TEG ID is associated with the Tx timing of the UE Rx-Tx measurement.

Regarding Alt.1 vs. Alt. 3, Alt.1 is more suitable because it provides the association between the TEG and a SRS resource for a Tx timing for one measurement. In our understanding, the TEG could vary due to some hardware specificities as well as the transmission configuration/settings. Thus, even with same resource or same timing, the TEG might be different. For accurately associating the UE Tx TEG ID, both factors need to be considered.

Proposal 2: For the reporting of UE Tx TEG in DL+UL positioning, a Tx TEG ID is associated with an UL SRS resource for positioning corresponding to the Tx timing of the Rx-Tx measurement.

### Definition of UE Rx-Tx time difference

In RAN1#106, the following agreements related to UE Rx-Tx time difference measurement were reached:

Agreement: - Consider supporting one of the following alternatives related to the UE Rx-Tx time difference (decision to be made in RAN1#106b): - Option 1: - Subject to UE capability, the UE may report an additional UL Timestamp associated to a UE Rx-Tx measurement, corresponding to the timing of the uplink subframe of a positioning SRS. - Add the following to the UE Rx-Tx time difference definition (similar to the definition for HD-FDD UE in TS 36.214): - If the UE does not transmit SRS in subframe #j, and if the UE reports an additional timestamp for the positioning SRS associated to the measurement, it shall compensate for the difference in the transmit timing of uplink subframe #j and the transmission timing of the subframe containing positioning SRS. - Option 2: - Subject to a UE capability, a UE may optionally report Timing Adjustment (TA) change information - Option 3A: The TA change information is included in the UE Tx TEG report - Option 3B: The TA change information is included in the Rx-Tx measurement report - Note: TA change information corresponds to: Tx Timing change with a timestamp that this change occurred. - Option 3: - Subject to UE capability, the UE may report an additional UL Timestamp associated to a UE Rx-Tx measurement, corresponding to the timing of the uplink subframe of a positioning SRS. - Add the following to the UE Rx-Tx time difference definition (similar to the definition for HD-FDD UE in TS 36.214): - If the UE does not transmit SRS in subframe #j, and if the UE reports an additional timestamp for the positioning SRS associated to the measurement, it is up to UE to compensate for the difference in the transmit timing of uplink subframe #j and the transmission timing of the subframe containing positioning SRS, or include the difference (Timing Adjustment change) without compensation within the report - Other options are not precluded.

In addition,  a related discussion in RAN1#106bis occurred:

Proposal 3.3-2a(H)

- Consider supporting one of the following alternatives related to the UE Rx-Tx time difference (decision to be made in RAN1#106b):

  - Option 1:

    - Subject to UE capability, the UE may report an additional UL Timestamp associated to a UE Rx-Tx measurement, corresponding to the timing of the uplink subframe of a positioning SRS.

    - Add the following to the UE Rx-Tx time difference definition (similar to the definition for HD-FDD UE in TS 36.214):

      - If the UE does not transmit SRS in subframe #j, and if the UE reports an additional timestamp for the positioning SRS associated to the measurement, it shall compensate for the difference in the transmit timing of uplink subframe #j and the transmission timing of the subframe containing positioning SRS.

  - Option 2:

    - Subject to a UE capability, a UE may optionally report Timing Adjustment (TA) change information

      - Option 2A: The TA change information is included in the UE Tx TEG report

      - Option 2B: The TA change information is included in the Rx-Tx measurement report

      - Note: TA change information corresponds to: Tx Timing change with a timestamp that this change occurred.

  - Option 3:

    - Send an LS to RAN4, requesting RAN4 to make the decision to select Option 1 or Option 2

- If RAN1 makes the decision to adopt either Option 1 or Option 2, send an LS to RAN4 to check if RAN4 has any issue to support the option.

The main intention of the above discussion is to address the potential UL timing changes between the time of the SRS transmission and the time of the UE performing Rx-Tx time difference measurement. For Option 1 vs Option 2, the difference is whether the UE compensates the UL timing change by itself or report the TA change information to LMF. Due to the large values of TA adjustment errors in 38.133 (i.e., at least 32Tc which is roughly 16ns), the TA change information reported by UE cannot be accurate enough to satisfy the positioning accuracy requirements. Option 3 kicks the ball to another working, this is not necessary, and which we think it’s not necessary and such an issue should preferably be decided by RAN1. Thus, we prefer Option 1 among the above three options.

Proposal 3:

- UE may report an additional UL Timestamp associated to a UE Rx-Tx measurement, corresponding to the timing of the uplink subframe of a positioning SRS

- Add the following to the UE Rx-Tx time difference definition:

  - If the UE does not transmit SRS in subframe #j, and if the UE reports an additional timestamp for the positioning SRS associated to the measurement, it shall compensate for the difference in the transmit timing of uplink subframe #j and the transmission timing of the subframe containing positioning SRS.

# RSTD measurement enhancements

In RAN1#106 meeting, the following agreements related to the measurement time windows were reached:

Agreement: Consider the following options (both could be selected) until RAN1#106b-e - Option 1: Support LMF to optionally indicate the measurement time window (MTW) for a UE for the measurement instances included in a measurement report. - Option 2: Support LMF to optionally indicate the measurement time window for a gNB for the measurement instances included in a measurement report. - FFS: the details of the MTW configuration. - Any requirements can be discussed by RAN4 after decision on the options is made.

One of the purposes for the measurement time windows for both the UE and the gNB is to enable the UE/gNB to provide the measurement instances in the same time window to avoid miss-match between the UL and DL measurements. Based on the agreements in previous meetings, one measurement report could include multiple measurement instances and each measurement instance has its own timestamp. Thus there might be a case where UE reports two measurement instances with same timestamp due to, e.g., the UL timing change or these instances are in the same time unit from which the time stamp is derived. In this case, the measurement time window can help LMF identify these two measurement instances.

Proposal 4:

- Support LMF to optionally configure the measurement time window (MTW) for a UE for the measurement instances included in a single measurement report.

- Support LMF to optionally indicate the measurement time window for a gNB for the measurement instances included in a single measurement report.

# Conclusion

This contribution discusses the accuracy improvements on timing based positioning solutions. Observations and proposals are summarized as follows:

Proposal 1: Both options for UE TEG reporting (i.e., reporting the UE RxTx TEG ID or reporting both UE Rx TEG ID and UE Tx TEG ID) are supported for DL+UL positioning subject to the UE capability.

Proposal 2: For the reporting of UE Tx TEG in DL+UL positioning, a Tx TEG ID is associated with an UL SRS resource for positioning corresponding to the Tx timing of the Rx-Tx measurement.

Proposal 3:

- UE may report an additional UL Timestamp associated to a UE Rx-Tx measurement, corresponding to the timing of the uplink subframe of a positioning SRS

- Add the following to the UE Rx-Tx time difference definition:

  - If the UE does not transmit SRS in subframe #j, and if the UE reports an additional timestamp for the positioning SRS associated to the measurement, it shall compensate for the difference in the transmit timing of uplink subframe #j and the transmission timing of the subframe containing positioning SRS.

Proposal 4:

- Support LMF to optionally configure the measurement time window (MTW) for a UE for the measurement instances included in a single measurement report.

- Support LMF to optionally indicate the measurement time window for a gNB for the measurement instances included in a single measurement report.

# References

- RP-210903, New WID on NR Positioning Enhancements, RAN#91-e, Mar, 2021.