3GPP TSG RAN WG1 Meeting #107-e		             R1-2111758

e-Meeting, November 11th – 19th, 2021

Agenda item:	8.11.1.1

Source: 	Samsung

Title: 	On resource allocation for power saving

Document for:	Discussion and Decision

# Introduction

UE-to-UE direct communication is one important scenario developed by 3GPP with a series of techniques including LTE D2D, LTE V2X and NR sidelink. The first standard of NR sidelink was completed in Rel-16 Work Item “5G V2X with NR sidelink”, in which the sidelink technique focusing on vehicle-to-everything (V2X) was specified. In Rel-17, a wider scope of sidelink applications is expected, and a new Rel-17 Work Item on “NR sidelink enhancement” was approved in RAN#86 [1] to support various use cases in both public safety and commercial scenarios. The WID was further updated in RAN#90-e [2] to include potential impact of sidelink DRX in power saving scope.

Rel-16 NR sidelink mainly focus on vehicle UEs which have sufficient battery supply. In Rel-17, power sensitive UEs such as pedestrian UE and UE in public safety and commercial use cases are involved in the typical use cases. Therefore, power saving for NR sidelink was introduced in Rel-17 WI to allow SL UE to operate in a power efficient manner with the following objective:

- Specify resource allocation to reduce power consumption of the UEs [RAN1, RAN2] - Baseline is to introduce the principle of Rel-14 LTE sidelink random resource selection and partial sensing to Rel-16 NR sidelink resource allocation mode 2. - Note: Taking Rel-14 as the baseline does not preclude introducing a new solution to reduce power consumption for the cases where the baseline cannot work properly. - This work should consider the impact of sidelink DRX, if any.

In this contribution, possible solutions for NR sidelink power saving, including how to reuse LTE technique and necessary enhancements are provided.

# Resource allocation schemes

In Rel-16 NR sidelink mode 2 operation, long term sensing window was specified to detect resource reservations and avoid collision as much as possible. However, the sensing procedure can hardly be supported by power sensitive UEs with limited battery capacity. Random selection and partial sensing were agreed as NR SL power saving schemes to achieve a trade-off between power consumption and reliability.

## Random selection

The following agreements regarding random selection was made in RAN1#106-e meeting [3]:

Agreement For random resource selection in a resource pool (pre-)configured with full/partial sensing and random resource selection, down-select to one of the followings in RAN1#106bis-e - Option 1: A priority threshold value or a range of priority levels is (pre-)configured for the resource pool, below or within which random resource selection is allowed - Note, lower value means higher priority - FFS whether resource pool partitioning can be additionally applied - Option 2: Increase the priority for the transmission based on random selection and indicate the new priority value in the priority field in the 1st-stage SCI - FFS: An extra field is added in SCI for indicating the original priority value associated with QoS requirement, - FFS: A 1-bit field in the SCI indicates that the UE is performing random resource selection, or - FFS: An extra field is added in SCI for indicating the mapping to the original priority value associated with QoS requirement. - Option 7: Exclude resources reserved by UE performing random selection without re-evaluation / pre-emption checking, regardless of their priorities. E.g. a 1-bit field in the SCI indicates that the UE is performing random resource selection and not performing re-evaluation and pre-emption checking - Option 12: No special consideration

Random selection significantly reduces power consumption by skipping sensing procedure, but cannot avoid collision by selecting resource randomly, thus leading to a degradation in PRR performance of legacy sidelink UEs. In addition, the collision between Rel-17 UEs using random selection may become non-negligible as the density of UEs using random selection increases. Therefore, some enhancements on random selection, especially for resource pool mixed with full/partial sensing schemes, was discussed to reduce potential reliability degradation.

Considering the pre-emption and re-evaluation check procedures in legacy Rel-16 NR SL, introducing a priority threshold to only allow high priority traffic using random selection can efficiently reduce collisions introduced by random selection. Therefore, option 1 should be supported. Other options could result in actual increase of priority of traffic using random selection, which introduces an unfair penalty to sensing-based traffic thus should not be supported.

In addition, resource partitioning can be introduced to improve reliability. For example, the partitioning can be based on priority of SL data. The partitioning of resources can be done in such a way so as to minimize the probability of collision and make the probability of collision related to the priority of SL data, higher priority SL data having lower probability of collision and vice versa.

Figure 1 (a) shows the impact of increasing the number of user groups on PRR. As the number of groups that users are partitioned into increases, the randomness within each group is reduced leading to less collision and higher PRR. Another benefit of partitioning users into groups is to control the collision probability as a function of the traffic priority. The simulation results in Figure 1 (b), show the benefit of improving the PRR of high priority traffic, albeit at the expense of a lower PRR for low priority traffic, by allocating more resources to the high priority traffic. In this simulation it is assumed that half the users have low priority traffic and half the users have high priority traffic. Figure 1 (b) shows three different scenarios:

- The 3 bars on the left show the PRR when all users share the same resources. The bar on the left of the group is the PRR for low priority users. The two bars on the right of the group is the PRR for high priority users. As expected, the PRR is the same in all cases (within the tolerance of the simulation).

- The 3 bars in the middle show the PRR when the resources are partitioned into three groups. The low priority users (half of the total users) are allocated one group, while the high priority users (the other half of the total users) are allocated two groups. The bar on the left of the group is the PRR for low priority users. The bar in the middle of the group is the PRR for the high priority users when the high priority users share the resources of the two allocated groups. The bar on the right of the group is the PRR for high priority users, when the high priority users are further partitioned between the two groups allocated to high priority users. As expected, the PRR of the low priority users goes down, while the PRR of the high priority users improves. Furthermore, by partitioning the high priority users between the high priority groups, there is a further slight improvement in the PRR.

- The 3 bars on the right show the PRR when the resources are partitioned into four groups. The low priority users (half of the total users) are allocated one group, while the high priority users (the other half of the total users) are allocated three groups. The bar on the left of the group is the PRR for low priority users. The bar in the middle of the group is the PRR for the high priority users when the high priority users share the resources of the three allocated groups. The bar on the right of the group is the PRR for high priority users, when the high priority users are further partitioned between the three groups allocated to high priority users. As expected, the PRR of the low priority users goes down, while the PRR of the high priority users improves. Furthermore, by partitioning the high priority users between the high priority groups, there is a further slight improvement in the PRR.

(a) 											   (b)

Figure 1. Impact of user grouping on PRR for random resource selection.

When the resource pool is shared with users using random resource selection, partial sensing and full sensing the resources are partitioned, for example into two groups of resources as shown in Figure 2. For users performing no sensing and using random resource selection in resource allocation procedure:

- High priority users, e.g. users with priority value below a threshold use resources A (a lower priority value indicates a higher priority).

- Low priority users, e.g. users with priority value equal to or larger than a threshold use resources B (a higher priority value indicates a lower priority).

Typically, the system is designed such that the probability of collision on resources A is less than the probability of collision on resources B.

For users that use partial sensing and full sensing, these users can sense resources B and determine an available resource there to use. There is a still a possibility that the transmission from a user with partial sensing or full sensing collides with a transmission from a user with random resource selection. When a user with partial sensing or full sensing retransmits the data it can chose between resources A (with lower collision probability) or resources B (with higher collision probability) based on the transmission number m, wherein, m=0 for the initial transmission, m=1 for the first re-transmission, m=2 for the second retransmission, … When the transmission number is less than M, sensing-based data uses resources B, when the transmission number is greater than or equal to M, sensing-based data uses resources A. The threshold M can depend on the priority of the data. A single value of M can be configured for all priority levels, or a priority-specific value of Mpriority can be configured for each priority level. The decision to use A or B can be based on the priority of traffic and re-transmission number for example,

- For high priority traffic (with sensing), the initial transmission uses resources B, and starting from the first retransmission resources A are used (M=1).

- For medium priority traffic (with sensing), the initial transmission and first re-transmission use resource B and starting from the second retransmission resources A are used (M=2).

- For low priority traffic (with sensing), the initial transmission, and first and second re-transmissions use resource B and starting from the third retransmission resources A are used (M=3).

- …

Figure 2. Resource pool partitioning for a resource pool shared with random resource selection, partial sensing and full sensing.

Having a single priority threshold for all resources of a resource pool for random resource selection traffic will not work well. By having single priority threshold for all resources, traffic that has a priority value higher than the threshold (a high priority value indicates low priority traffic) will not be transmitted by the resource pool. This could lead to one of two outcomes both of which are undesirable:

- Low priority traffic is precluded from being transmitted from users that only support random resource selection.

- Higher layers artificially decrease the priority value of low priority traffic below the threshold, in case of random sensing, to transmit in the resource pool. This creates a disadvantage for traffic using partial of full sensing.

Proposal 1: For random resource selection in a resource pool (pre-)configured with full/partial sensing and random selection,

- Support option 1 with resource partitioning that is a priority threshold value or a range of priority levels is (pre-)configured for resources within the resource pool, below or within which random resource selection is allowed

- If no resource partitioning is introduced, support Option 12 with no special enhancement

## Resource selection based on at least CPS

The following agreement regarding contiguous partial sensing was made in RAN1#106bis-e meeting [3]:

Agreement When UE performs at least contiguous partial sensing in a mode 2 Tx pool for a resource (re)selection procedure triggered by aperiodic transmission (Prsvp_TX=0) in slot n, TA and TB for CPS monitoring window and a candidate resource set (SA) is initialized according to potentially one of the following approaches (final decision in RAN1#107-e). Other approaches are not precluded and the details in each approach can still be updated. - Approach 1: (SA is initialized based on at least slots with PBPS and/or CPS results and guarantee a minimum of M slots for CPS) - The UE selects a set of Y’ candidate slots with corresponding PBPS and/or CPS results (if available) within the RSW. - FFS how to handle the case if the total number of Y’ candidate slots is less than a (pre-)configured threshold Y’min without dropping the aperiodic transmission - FFS whether the Y’ candidate slots for aperiodic transmission is the same as the Y candidate slots in PBPS for periodic transmission of another TB(s) - FFS whether/how to prioritize/select resources based on partial sensing results. - FFS: How to select Y’ in case of CPS only - Candidate resource set (SA) is initialized to the set of all single-slot candidate resources in the selected Y’ candidate slots. - For the CPS monitoring window [n+TA, n+TB]: - TA and TB are both selected such that UE has sensing results for a minimum of M consecutive logical slots before ty0, where ty0 is the first slot of the selected Y’ candidate slots. - FFS: By default, M is 31 unless (pre-)configured with another value, or M is (pre-)configured based on transmission priority - FFS the range of (pre-)configured M from a TBD lowest value up to 30 - FFS: how to handle the case when the minimum M slots for CPS cannot be guaranteed - FFS: RSW in case of CPS only - Approach 2: (SA is initialized based on all candidate single-slot resources and guarantee a minimum of M slots for CPS) - Candidate resource set (SA) is initialized to the set of all candidate single-slot resources in [n+TB+Tproc,0+Tproc,1, n+T2], where TB is selected by the UE such that length of [n+TB+Tproc,0+Tproc,1, n+T2] ≥ T2min. - Tproc,0, Tproc,1 are in units of physical time/slots - FFS whether/how to prioritize/select resources based on partial sensing results (if PBPS is performed). - For the CPS monitoring window [n+TA, n+TB]: - TA = X - FFS value X for TA including X=1 and negative value - TB is selected such that UE has sensing results for a minimum of M consecutive logical slots before the start of (n+TB+Tproc,0+Tproc,1). - FFS: By default, M is 31 unless (pre-)configured with another value, or M is (pre-)configured based on transmission priority - FFS the range of (pre-) configured M from a TBD lowest value up to 30 - FFS: how to handle the case when the minimum M slots for CPS cannot be guaranteed - Approach 3: (independent approach for different case) - When UE additionally performs periodic-based partial sensing in the resource pool, the above Approach 1 applies. - When UE does not perform periodic-based partial sensing in a resource pool that does not allow resource reservation for another TB, the above Approach 2 applies.

In a resource pool with periodic reservation for another TB enabled, it was agreed to support PBPS and CPS for resource selection procedure triggered by aperiodic transmissions. Different than periodic transmissions, before the triggering slot n, UE cannot anticipate the PBPS/CPS window for the given aperiodic transmission, thus reusing of existing sensing results (of another TB), if exist, and triggering new CPS after slot n can be adopted in this scenario. Therefore, selecting candidate slot with PBPS/CPS result is a power-efficient way to improve reliability by utilizing existing sensing result (of another TB) for resource exclusion, and Approach 1 above should be used for the scenario.

For CPS-only case in a resource pool with periodic reservation for another TB disabled, since candidate slots are mainly designed for PBPS which does not exist in the pool, there is no need to further select candidate slots, and UE can simply initialize candidate resource set as all resources within RSW after sensing window, thus Approach 2 should be used for the scenario.

Proposal 2: When UE performs both CPS and PBPS or CPS only in a mode 2 Tx pool for a resource (re)selection procedure triggered by aperiodic transmission (Prsvp_TX=0) in slot n, TA and TB for CPS monitoring window and a candidate resource set (SA) is initialized according to Approach 3:

- Approach 3: (independent approach for different case)

  - When UE additionally performs periodic-based partial sensing in the resource pool, the above Approach 1 applies.

  - At least when UE does not perform periodic-based partial sensing in a resource pool that does not allow resource reservation for another TB, the above Approach 2 applies. Other conditions of applying Approach 2 above is not excluded.

In approach 1, how to determine the availability of PBPS and/or CPS results corresponding to candidate slots needs to be further defined. For example, a candidate slot Y1 can correspond to M PBPS occasions and a CPS monitoring window with length of N slots, and if UE only monitors M1<M PBPS occasions and/or N1<N slots within CPS window, UE needs to determine whether slot Y1 can be selected as one of the Y' candidate slots.

Consider the existence of PBPS/CPS results is hard to be guaranteed, it is preferred not to restrict available PBPS/CPS results as all corresponding occasions/window being monitored, and determine Y' candidate slots in a best effort way. Therefore, a slot can defined as with available PBPS and/or CPS results if the number of corresponding PBPS occasions or slots in CPS window being monitored is larger than a given threshold. The threshold can be defined as one slot to utilize existing PBPS/CPS result.

The definition of PBPS occasions and CPS window for this case was not specifically defined and needs clarification. To keep consistency between different scenarios, the PBPS occasions for approach 1 can reuse the definition of PBPS occasions in the case of UE performing new PBPS for a given periodic transmission, as agreed in RAN1#104-e. The CPS window can be defined as start from M logical slots earlier than slot ty0SL until slot n, subject to processing time (i.e. Tproc,0) restriction, wherein the parameter M can be similar as in other cases, e.g. by default M is 31 unless (pre-)configured with another value, and the another values can be configured per priority.

In addition, if the remaining PDB is large enough, it is possible that UE start a new CPS window [n+TA, n+TB) between the triggering slot n and the first slot of selected Y candidate slots. Therefore, for any resources after slot n+TB, it can be assumed as with available CPS result thus can be considered into candidate slot set.

Proposal 3: For approach 1, the following candidate slots can be considered as with available PBPS and/or CPS results.

- Candidate slot with at least one corresponding PBPS occasion being monitored (e.g. for other TBs). The definition of PBPS occasions is the same as agreed in RAN1#104-e.

- Slots with CPS result start from M logical slots earlier than slot ty0SL until slot n, subject to processing time (Tproc,0) restriction, if there already exists any CPS result (e.g. for other TBs) within the interval, and if the M logical slots earlier than slot ty0SL is earlier than Tproc,0 physical slots before slot n (ty0SL- M <n-Tproc,0)

- Slots after n+TB, subject to processing time restriction, to allow UE performing CPS between slot n and starting of the selected candidate slots as available CPS results

A minimum threshold of Y’min is defined in approach 1 to ensure sufficient randomization during resource selection procedure. If the total number of Y’ candidate slots with PBPS/CPS result is less than a (pre-)configured threshold Y’min, approach 1 is no longer appropriate and approach 2 can be adopted to generate SA. In order to still allow utilization of existing PBPS/CPS results in the case, some prioritization rule during resource selection can be introduced. After sensing-based candidate resource exclusion, candidate resources on any of the Y’ candidate slots can be prioritized over on other slots without sensing result. Furthermore, since each of the Y’ candidate slots may correspond to different number of already monitored occasions/slots, UE can further support prioritization of slots with more sensing result, e.g. slots correspond to larger number of PBPS occasions/CPS slots being monitored, or more type of sensing results e.g. candidate slots with PBPS+CPS results is prioritized over candidate slots with only CPS results. Consider the randomization of resource selection procedure, the prioritization can be not absolute but based on probability, e.g. one resource prioritized over other resources means that the one resource have higher probability of being selected. Since the final decision of transmission resource is performed by MAC layer, the prioritization can be leave up to MAC layer if PHY layer additionally provided relevant information, or PHY layer can perform prioritization before reporting SA to MAC layer. For example, if PHY layer generates SA with more than X⋅Mtotal resources after Step 7), PHY layer can perform the prioritization and report a smaller set to MAC layer.

Proposal 4: For approach 1, if the total number of Y' candidate slots is less than a (pre-)configured threshold Y'min, approach 2 is applied with the following:

- After sensing-based candidate resource exclusion, if any remaining candidate resource in SA is located on the Y' candidate slots, the candidate resource is prioritized for resource selection.

Proposal 5: For approach 1 and approach 2, support prioritization of resources based on partial sensing result during resource selection procedure.

- Candidate slot with more sensing result have higher priority to be selected.

  - The more sensing result includes: larger number of monitored PBPS occasions, larger number of monitored CPS slots, more type of sensing results e.g. candidate slots with PBPS+CPS results is prioritized over candidate slots with only CPS results

- Prioritization is performed by higher probability of being selected within candidate resource set SA

In both approach 1 and approach 2, a new CPS window [n+TA, n+TB) for given transmission is defined. The new CPS window can be similarly defined with up to 31 contiguous slots to cover intra-period resource reservation. The starting point of sensing window should be no earlier than slot n to ensure the feasibility of contiguous partial sensing for aperiodic transmission with no predictability of trigger slot. Therefore, n+TA can be defined as M' (up to 31) logical slots before ty0SL and is restricted as no earlier than the next logical slot after slot n (i.e. max (n+1, ty0SL-M') with necessary converting from logical slots to physical slots) where ty0SL is the slot index of earliest candidate resource in approach 1, and can be defined as TA≥1 for approach 2.

For both approaches, TB can be configured with at least M consecutive logical slots after n+TA to cover intra-period reservations in SCI. Support a smaller TB value than n+TA+31, i.e. support M<31, may also be necessary to satisfy latency requirement and ensure UE to have sufficient candidate slots after slot n+TB. Therefore, the value of M can be configured according to priority and remaining PDB to balance the size of CPS window and remaining RSW in which candidate slots can be selected according to traffic requirement. Moreover, specific entry e.g. TA=TB =0 can also be considered to allow UE skipping sensing procedure e.g. using periodic-based partial sensing if configured, or using random selection instead.

Proposal 6: For with CPS window [n+TA, n+TB] in approach 1 and approach 2, not support negative value for TA.

Proposal 7: For sensing window [n+TA, n+TB) in approach 1, n+TA is up to M' logical slots before ty0SL and is no earlier than the next logical slot after slot n, and n+TB is Tproc,0+Tproc,1 slots earlier than ty0SL, and Tproc,0+Tproc,1 are in units of physical time/slots.

- The value of M' should allow a minimum of M logical slots within CPS window, e.g. M' = M+ Tproc,0+Tproc,1.

Proposal 8: For approach 1 and 2, support that by default, M is 31 unless (pre-)configured with another value, or M is (pre-)configured based on transmission priority.

- The range of M is (pre-)configured with a lowest value of 0 up to 30.

Considering the restriction of small remaining PDB for latency-sensitive traffic, it may not always possible to guarantee the minimum M slots for CPS. For approach 1, since the value of TA, TB can be selected according to candidate slots, UE should at first try if re-selection of a later ty0SL is possible that satisfy both Y'min candidate slots and minimum M slots of CPS window. If not, then UE can perform resource exclusion based on the limited number of CPS slots as best-effort resource selection, because utilizing limited CPS result is anyway better than pure random selection. Similarly, for approach 2, UE can perform resource exclusion based on limited CPS result as for approach 1.

Proposal 9: For approach 1 and 2, when the minimum M slots for CPS cannot be guaranteed,

- For approach 1, if possible, UE can delay re-select the Y' candidate slots with later ty0SL to satisfy both Y'min candidate slots and minimum M slots of CPS window; otherwise, UE still monitors the CPS window [n+TA, n+TB], and perform resource exclusion and reporting according to the limited sensing results

- For approach 2, UE still monitors the CPS window [n+TA, n+TB], and perform resource exclusion and reporting according to the limited sensing results

Compared with new CPS window, introducing a new set of PBPS occasions after trigger slot n corresponding to a new set of Y candidate slots within RSW is not preferred. Consider the typical values of traffic periods within a resource pool, and also typical values of PDB, only the smallest values of periods are possible to be monitored within RSW. Even for the smallest values, UE needs to wait and sense after triggering slot n for at least one period, which will strongly delay the total transmission and impact traffic latency.

Proposal 10: When UE performs both CPS and PBPS in a mode 2 Tx pool with periodic reservation for another TB (sl-MultiReserveResource) enabled, for a resource (re)selection procedure triggered by aperiodic transmission (Prsvp_TX=0) in slot n, not support UE determining a new set of Y candidate slots for the purpose of triggering new PBPS between slot n and the first slot of the new Y candidate slots.

Considering that even for aperiodic transmissions, the packet might arrive as a burst, e.g. similarly as event-triggered packets in LTE V2X, it may be possible to have some predictability for aperiodic transmissions. The scenario can be further studied and if possible, contiguous partial sensing window before slot n can also be adopted for aperiodic transmissions.

If higher layers (e.g. the application layer) can anticipate the arrival of SL data in advance, it provides an indication to physical layer in slot m that SL data is expected to arrive in slot n, the UE can start CPS before slot n. For example the sensing window in the range [n+TA,n+TB). Where,

- n+TA=maxm+1,n-31

- n+TB=m-Tproc0

## Re-evaluation & pre-emption

The following agreement regarding contiguous partial sensing was made in RAN1#106-e and RAN1#106bis-e meetings [3, 4]:

Agreement For a resource pool (pre-)configured with at least partial sensing and UE is configured by its higher layer for partial sensing, - Periodic-based partial sensing and contiguous partial sensing schemes are supported for resource re-evaluation and pre-emption checking - FFS details of partial sensing for re-evaluation and pre-emption checking, including any restrictions / conditions on performing PBPS and CPS, subset of resources, timing, candidate resource set (SA) and etc - Same as in Rel-16, the higher layer indicates a set of resources (r0,r1,r2,…) and/or a set of resources (r0',r1',r2',…) for re-evaluation and/or pre-emption checking, respectively - Pre-emption checking is enabled according to the Release-16 interpretation of sl-PreemptionEnable. - FFS: If additional enhancements are needed for enabling/disabling - The triggering of re-evaluation and pre-emption checking is as in R16. Working Assumption In a resource pool (pre-)configured to enable partial sensing, when UE is configured with partial sensing by its higher layer, the resources for which the UE performs re-evaluation and/or pre-emption checking are for the initial transmission and retransmissions of every TB according to Rel-16 specification based on partial sensing results. - Same as in Rel-16, for periodic transmission, re-evaluation check is not applied to the resources that have been signalled in current period or previous periods, except that it is up to UE implementation whether to apply re-evaluation check to the resources in non-initial reservation period that have been signalled neither in the immediate last nor in the current period. - The resource in the main bullet is the set of resources (r0,r1,r2,…) and/or the set of resources (r0',r1',r2',…) for re-evaluation and/or pre-emption checking, respectively, which has been agreed in RAN1 #106-e.

The legacy procedure can be reused for Rel-17 power saving mode as a baseline. It was agreed that both PBPS and CPS are supported for re-evaluation and pre-emption checking, and the enabling/disabling and triggering timing was also agreed. The remaining details include sensing occasions/window of PBPS and CPS for re-evaluation and pre-emption checking that still need to be discussed.

For CPS-based pre-emption check and re-evaluation checking, the corresponding sensing window could be defined with a limited size W, e.g. up to 31 slots, to efficiently detect potential collisions and keep consistency with the principle of contiguous partial sensing for resource (re-)selection. For a reserved sidelink transmission in slot m, the corresponding re-evaluation check or a pre-emption check is performed in slot m-T3, thus the end of the sensing window for a pre-emption check or a re-evaluation check can be m-T3-Tproc,0. The start of the sensing window should be W slots prior to slot m, where W is the furthest slot that can be reserved by SCI, i.e. 31 slots in Rel-16. Therefore, the sensing window is: [m-31,m-T3-Tproc,0). In addition, considering that the starting slot m-31 may be earlier than the timing point at which resource (re-)selection is performed by the UE, whether the time duration between m-31 and resource (re-)selection performed by the UE can be handled as re-evaluation/pre-emption checking or resource (re-)selection needs further discussion.

For PBPS-based pre-emption check and re-evaluation, the determination of periodic sensing occasions could reuse the principle of PBPS for resource (re-)selection. However, further conditions/enhancements should be discussed to ensure the sensing window of PBPS-based pre-emption check and re-evaluation is feasible to be monitored by UE.

Proposal 11: For a UE configured with enabled pre-emption check or enabled re-evaluation check, if UE reserves a sidelink transmission in slot m, the UE start monitoring at M logical slots earlier than slot m until T3+Tproc,0 slots earlier than slot m, where T3 and Tproc,0 are in units of physical time/slots

- T3 and Tproc,0 are defined as the same value in Rel-16 NR V2X

- If slot m-M is before the time of transmission resource being selected, the UE start monitoring for pre-emption/re-evaluation check after the time of transmission resource being selected, subject to processing delay

- PBPS-based sensing for pre-emption check and re-evaluation check can be supported by reusing existing PBPS results, if any

  - Not support UE determining a new set of Y'' candidate slots for the purpose of triggering new PBPS for pre-emption check and re-evaluation check

# Sidelink DRX

The following agreements regarding sidelink DRX (SL DRX) were made in RAN1#106-e meeting [4]:

Agreement A UE can perform SL reception of PSCCH and RSRP measurement for sensing during its SL DRX inactive time. - FFS: When such reception and measurement is performed, whether it is subject to specification, or is up to UE implementation - FFS: Other details

According to the agreements above, the remaining issue is whether UE sensing behavior within SL DRX inactive time needs to be specified or it is up to UE implementation. In our view, leaving sensing behavior in SL DRX inactive time as UE implementation will cause system performance degradation since some UE cannot perform sensing within DRX inactive time and that will cause interference to other UEs. Also, we also think that specifying different UE sensing behavior during SL DRX active and inactive time cannot be justified at this stage since Rel-17 UE sensing behavior was not completed yet and the performance gain for the different UE sensing behavior was not verified. In this aspect, UE sensing behavior should be the same regardless of whether the UE is in or not in SL DRX inactive time. In other word, if we specify UE sensing behavior without SL DRX operation, this can be applied in the same manner within SL DRX inactive time.

Proposal 12: A UE shall perform the same sensing behavior regardless of whether the sensing window/occasion is in or not in SL DRX inactive time.

In addition, the following working assumption for SL DRX was made in RAN1#106bis-e meeting [3]:

Working Assumption When PHY layer is indicated with an active time of RX UE from MAC layer for candidate resource selection, a restriction is applied in PHY layer so that at least a subset of candidate resources reported to MAC layer is located within the indicated active time of the RX UE. The following options will be further discussed in RAN1 to restrict resources for candidate resource selection taking into account the indicated active time from MAC layer: - Option 1: PHY layer selects and reports candidate resources only within the indicated active time of the RX UE - Option 2: PHY layer selects and reports candidate resources in which at least a subset of the candidate resources is within the indicated active time of the RX UE - Option 3: PHY layer selects and reports an additional candidate resource set of candidate resources within the indicated active time of the RX UE

For the listed options above, the simplest solution is Option 1. If the clear advantage and motivation of other Options are not verified, we support Option 1.

Proposal 13: PHY layer selects and reports candidate resources only within the indicated SL DRX active time of the RX UE.

# Conclusions

In this contribution, the issues in random selection and partial sensing procedures are discussed with the following proposals:

Proposal 1: For random resource selection in a resource pool (pre-)configured with full/partial sensing and random selection,

- Support option 1 with resource partitioning that is a priority threshold value or a range of priority levels is (pre-)configured for resources within the resource pool, below or within which random resource selection is allowed

- If no resource partitioning is introduced, support Option 12 with no special enhancement

Proposal 2: When UE performs both CPS and PBPS or CPS only in a mode 2 Tx pool for a resource (re)selection procedure triggered by aperiodic transmission (Prsvp_TX=0) in slot n, TA and TB for CPS monitoring window and a candidate resource set (SA) is initialized according to Approach 3:

- Approach 3: (independent approach for different case)

  - When UE additionally performs periodic-based partial sensing in the resource pool, the above Approach 1 applies.

  - At least when UE does not perform periodic-based partial sensing in a resource pool that does not allow resource reservation for another TB, the above Approach 2 applies. Other conditions of applying Approach 2 above is not excluded.

Proposal 3: For approach 1, the following candidate slots can be considered as with available PBPS and/or CPS results.

- Candidate slot with at least one corresponding PBPS occasion being monitored (e.g. for other TBs). The definition of PBPS occasions is the same as agreed in RAN1#104-e.

- Slots with CPS result start from M logical slots earlier than slot ty0SL until slot n, subject to processing time (Tproc,0) restriction, if there already exists any CPS result (e.g. for other TBs) within the interval, and if the M logical slots earlier than slot ty0SL is earlier than Tproc,0 physical slots before slot n (ty0SL- M <n-Tproc,0)

- Slots after n+TB, subject to processing time restriction, to allow UE performing CPS between slot n and starting of the selected candidate slots as available CPS results

Proposal 4: For approach 1, if the total number of Y' candidate slots is less than a (pre-)configured threshold Y'min, approach 2 is applied with the following:

- After sensing-based candidate resource exclusion, if any remaining candidate resource in SA is located on the Y' candidate slots, the candidate resource is prioritized for resource selection.

Proposal 5: For approach 1 and approach 2, support prioritization of resources based on partial sensing result during resource selection procedure.

- Candidate slot with more sensing result have higher priority to be selected.

  - The more sensing result includes: larger number of monitored PBPS occasions, larger number of monitored CPS slots, more type of sensing results e.g. candidate slots with PBPS+CPS results is prioritized over candidate slots with only CPS results

- Prioritization is performed by higher probability of being selected within candidate resource set SA

Proposal 6: For with CPS window [n+TA, n+TB] in approach 1 and approach 2, not support negative value for TA.

Proposal 7: For sensing window [n+TA, n+TB) in approach 1, n+TA is up to M' logical slots before ty0SL and is no earlier than the next logical slot after slot n, and n+TB is Tproc,0+Tproc,1 slots earlier than ty0SL, and Tproc,0+Tproc,1 are in units of physical time/slots.

- The value of M' should allow a minimum of M logical slots within CPS window, e.g. M' = M+ Tproc,0+Tproc,1.

Proposal 8: For approach 1 and 2, support that by default, M is 31 unless (pre-)configured with another value, or M is (pre-)configured based on transmission priority.

- The range of M is (pre-)configured with a lowest value of 0 up to 30.

Proposal 9: For approach 1 and 2, when the minimum M slots for CPS cannot be guaranteed,

- For approach 1, if possible, UE can delay re-select the Y' candidate slots with later ty0SL to satisfy both Y'min candidate slots and minimum M slots of CPS window; otherwise, UE still monitors the CPS window [n+TA, n+TB], and perform resource exclusion and reporting according to the limited sensing results

- For approach 2, UE still monitors the CPS window [n+TA, n+TB], and perform resource exclusion and reporting according to the limited sensing results

Proposal 10: When UE performs both CPS and PBPS in a mode 2 Tx pool with periodic reservation for another TB (sl-MultiReserveResource) enabled, for a resource (re)selection procedure triggered by aperiodic transmission (Prsvp_TX=0) in slot n, not support UE determining a new set of Y candidate slots for the purpose of triggering new PBPS between slot n and the first slot of the new Y candidate slots.

Proposal 11: For a UE configured with enabled pre-emption check or enabled re-evaluation check, if UE reserves a sidelink transmission in slot m, the UE start monitoring at M logical slots earlier than slot m until T3+Tproc,0 slots earlier than slot m, where T3 and Tproc,0 are in units of physical time/slots

- T3 and Tproc,0 are defined as the same value in Rel-16 NR V2X

- If slot m-M is before the time of transmission resource being selected, the UE start monitoring for pre-emption/re-evaluation check after the time of transmission resource being selected, subject to processing delay

- PBPS-based sensing for pre-emption check and re-evaluation check can be supported by reusing existing PBPS results, if any

  - Not support UE determining a new set of Y'' candidate slots for the purpose of triggering new PBPS for pre-emption check and re-evaluation check

Proposal 12: A UE shall perform the same sensing behavior regardless of whether the sensing window/occasion is in or not in SL DRX inactive time.

Proposal 13: PHY layer selects and reports candidate resources only within the indicated SL DRX active time of the RX UE.

# References

[1] RP-193231, “NR Sidelink enhancement,” RAN#86, Stiges, Spain, December, 2019.

[2] RP-202846, “Revised WID on NR Sidelink enhancement,” RAN#90-e, e-meeting, Dec, 2020.

[3] 3GPP TSG RAN1#106bis-e, Final Report of 3GPP TSG RAN

[4] 3GPP TSG RAN1#106-e, Final Report of 3GPP TSG RAN

[5] R1-2109514, “Discussion on Sidelink Enhancement”, Samsung, RAN1#106bis-e, e-meeting, October, 2021.