3GPP TSG RAN WG1 #107-e	        		             R1-2111766

e-Meeting, November 11th – 19th, 2021

Agenda item:	8.14.2

Source: 	Samsung

Title: 					Potential enhancements for XR

Document for:	Discussion and Decision

# Introduction

This contribution considers proposed possible enhancements to XR in Rel-18 [1][3].

# Considerations on XR Operation

## Capacity

Capacity is clearly a key metric for any application and particularly for XR due to the relatively high data rate requirements. Regardless of any evaluations, it may be argued that capacity can always be met given sufficient bandwidth. For example, taking the ITU 2020 average SE target of 7.8 bits/s/Hz for dense urban and an average source data rate of 50 Mbps, ~15 UEs can be supported over a BW of 100 MHz. The number of UEs can be scaled according to available BW, target data rate, TDD UL-DL configuration (e.g. ~10 UEs for DDDSU), and is further reduced for a strict PDB.

It is however important to maximize spectral efficiency (SE) for XR, particularly in FR1, and exploiting the characteristics of XR traffic is the only way to do that as there is little room to improve the physical layer design in NR. As XR video traffic includes frames of variable importance, with I-frames being more important than P-frames (there also B-frames that can be discarded without creating material visual artifacts although their sizes are smaller in comparison), it would be important to (a) have IP packets that contain specific type(s) of video frames (I-frames or P-frames) and (b) identify at RAN which IP packet contains which type(s) of video frame. The first step is possible (SA4 can be consulted) but the second step is currently not possible (SA2 can be consulted). Once that information is provided at the RAN, different QoS flows can be established and XR can be supported based on Rel-17 mechanisms while optimizing SE based on the characteristics of the XR traffic as categorized by the different QoS flows. The meaning/interpretation of PDB may also need to be concluded if it is the I-frames that are most important and is acceptable to lose P-frames. In the UL, NR supports SR configurations for multiple traffic priorities/QoS flows and no further specification support is expected.

There have also been proposals to drop IP packets of a video frame when a corresponding PDB cannot be achieved in order to improve system SE. Such approach assumes at least that IP packets of a video frame can be generated by the video codec and delivered to RAN in order, that the order of the IP packets can be identified at RAN (e.g. first/last IP packet), and that there is no harm from dropping IP packets if the network can predict that the PDB will be exceeded. Those assumptions are currently invalid and also need input from SA for feasibility/accuracy. Moreover, failing to meet PDB is expected to be an infrequent event (otherwise, XR is non-functioning) and also, when such event occurs, remaining IP packets would not typically be many. Therefore, unlike SE gains due to I-frame/P-frame differentiation, a benefit from dropping few remaining IP packets when a respective PDB cannot be met is expected to be marginal, and whether or not such benefit can even exist may depend on the codec.

Observation 1: Material DL spectral efficiency gains for XR can be achieved by identifying separate QoS flows for I-frames and P-frames at the RAN.

Observation 2: With QoS flow differentiation, KPIs should be per QoS flow (separate KPIs for I-frames and P-frames).

Observation 3: RAN can process separate QoS flows for I-frames and Q-frames as in Rel-16.

Observation 4: Required conditions for RAN to drop IP packets of a video frame that cannot meet PDB are currently not supported, a user-experience impact from dropping such IP packets is unclear, and a SE gain is expected to be marginal.

Proposal 1: The Rel-18 SI/WI on XR should include requests to SA2 and SA4 for mechanisms to provide separate QoS flows for I-frames and P/B-frames at RAN in order for RAN to identify IP packets with only respective frames.

XR KPIs (especially the PDB) will also be limited by the 5%-10% of the UEs with the smallest geometries (“cell-edge” UEs). RAN1 evaluations for XR capacity did not differentiate UEs based on SINR but that is not a realistic assumption in actual network deployments that implement data rate control as otherwise that small fraction of UEs would consume most resources. LTE supports inter-cell interference coordination (ICIC) to enhance the geometry of “cell edge” UEs and that functionality is also essential to support XR. The specification impact is modest, mostly in RAN3, and the LTE mechanisms can largely be re-used.

Proposal 2: The Rel-18 XR SI phase should include RAN3 and consideration of ICIC mechanisms.

## Latency

Most NR bands are TDD, and TDD operation is challenging for latency.

For the DL, without I-frame/P-frame differentiation at RAN, latency is generally not an issue as there are enough HARQ processes and the XR KPIs are not meaningful to support is scenarios requiring DL coverage enhancements. However, with I-frame/P-frame differentiation at RAN, it is beneficial to retransmit TBs associated with I-frames as fast as possible in order to satisfy the PDB and therefore it would be beneficial to have minimum latency for associated HARQ-ACK reports. Scheduling on a (primary) cell from more than one scheduling cells is to be specified in Rel-17 DSS to offload PDCCH from the primary cell but having multiple scheduling cells for a scheduled cell can also be considered for latency reductions or for reliability enhancements in XR. The same holds for Rel-17 M-TRP support for operation in FR2 and CBG-based retransmissions that can also reduce latency and improve system SE/capacity.

In Rel-17 URLLC, it has been agreed to support carrier switching for PUCCH transmissions in inter-band CA. Although reliance on CA would not be an unreasonable assumption for XR, there is no support for intra-band CA as then all cells currently use same UL-DL configurations to avoid DL-to-UL interference. There are some proposals for Rel-18 SIs aiming to extend support of carrier switching to intra-band CA, or to support simultaneous transmissions/receptions in different BWPs of a same cell, but corresponding designs are unlikely to be available early enough. A same issue may be considered for PUSCH transmissions although they would be more complex to support and corresponding carrier switching is not motivated by the UL traffic characteristics of XR.

In general, the Rel-18 XR SI phase should consider applicability of all Rel-17 specifications for reducing latency and potentially also consider additional mechanisms for reducing latency, at least if there is no Rel-18 SI/WI on URLLC.

Proposal 3: The Rel-18 XR SI phase should include latency reduction for at least DL scheduling.

## RRC-based Scheduling

## SPS PDSCH

Enhancements to SPS PDSCH configurations have been considered in Tdocs for XR to address (a) non-integer periodicity and jitter, (b) variable data rates and large TBS, and (c) need for link adaptation. It is rather clear that those characteristics make use of SPS for XR PDSCH inappropriate and are typical reasons for use of DCI-based PDSCH scheduling. For example, for XR periodicities of 8.33 msec or 16.67 msec and with ±4 msec jitter, it is rather pointless to rely on SPS PDSCH. For example, for variable data rates due to adaptive video encoding, it is again rather pointless to rely on SPS PDSCH and aim to adapt corresponding configurations (that will not be without cost). The same obviously applies for any link adaptation. For the large TBS of XR PDSCH, using DCI for scheduling is even more appropriate as the relative control overhead is marginal while any SPS PDSCH enhancement cannot possibly offer better link adaptation or scheduling flexibility. Moreover, DCI-based scheduling is preferable for UE power consumption as it is far less costly to decode few PDCCH candidates within few symbols than to decode non-existent SPS PDSCHs over slots (and transmit corresponding NACKs on PUCCH). Therefore, DCI-based PDSCH scheduling is optimal with respect to XR traffic characteristics and metrics such as capacity, latency, and flexible resource utilization, while avoiding a substantial RAN1/RAN2 specification impact from adaptations to SPS PDSCH configurations.

Observation 5: PDSCH transmissions for XR are best supported by DCI and there is no need/motivation to enhance SPS.

## CG PUSCH

UL traffic for XR is generally periodic but the TBS can be variable. This basically enables a UE to transmit CG-PUSCH as in Rel-16 and, if needed, provide a BSR. SRS can also be transmitted for link adaptation. Then, subsequent scheduling, if needed, can be DCI-based. That approach is also favorable to a network and it enables easy management of UL resources (e.g. as in Rel-15). The latency budget is not an issue in the UL as multiple pose information receptions are filtered (including potentially discarding some) in order to determine characteristics for a next DL transmission.

Observation 6: PUSCH transmissions for XR can be based on Rel-16 CG-PUSCH and DCI-based PUSCH.

## UE Power Savings

UE power savings for XR devices, such as AR glasses, are an important consideration as the modem would consume a significant portion of the total power (e.g. ~40%). Rel-17 provides a sound foundation for UE power savings and is currently unclear what additional UE power savings can be provided for XR applications and at a level that can make a meaningful impact to the overall UE power consumption.

Characteristics of XR traffic that were suggested as motivations for enhancements to Rel-17 UE power savings include the non-integer periodicities of XR traffic and the presence of jitter. With jitter, whether or not C-DRX periodicities are extended to include non-integer values is of little importance – i.e. existing C-DRX periodicities suffice. To address the jitter, either Rel-17 power savings mechanisms can be used, or C-DRX can start earlier (by an amount determined by the NW). For example, for a jitter of ±4 msec, starting C-DRX earlier by ~4 msec avoids any impact on PDB at the expense of the UE having its RF on for a longer time and unnecessarily decoding a few PDCCH candidates for an average of ~4 msec (the time of ~4 msec may be reduced by ignoring the tails of the Gaussian distribution below a certain probability as they will be a secondary factor). For a XR traffic periodicity of 8.33 msec or 16.67 msec, the percentage of time the UE can be in sleep mode is then significantly reduced. However, such analysis can be misleading as it does not capture the full UE operation for XR or potential enhancements on other aspects.

A UE supporting XR will need to transmit pose information every ~4 msec and therefore the time for having the RF on is not affected by the jitter. The same would apply considering UL-DL configurations and the UE having to provide HARQ-ACK/CSI reports before or after DL XR traffic. Further, the UE would have to process XR video traffic and the baseband power consumption will be dominated by corresponding data rates of tens of Mbps instead of the few kbps corresponding to decoding a few PDCCH candidates prior to arrival of DL traffic. Additionally, even if the UE wakes up before XR traffic arrives, that is not necessarily detrimental for UE power consumption as the UE may be able to provide timely CSI reports, or perform beam management, that can result to more accurate link adaptation, shorter time to deliver XR traffic, and therefore compensate or even provide gains in terms of UE power consumption and system SE.

Design of a “low power” WUS has been proposed to possibly improve UE power savings for XR. However, for UEs in RRC connected mode, benefits from an LP-WUS are unclear. A UE-specific WUS design, similar to WUS provided by DCI format 2_6, that is as reliable as DCI format 2_6 and does impact UE design and cost, may require a separate SI. A study would also need to evaluate relative power savings from not monitoring PDCCH over a few msec, i.e. from avoiding receptions of few kbps, with respect to the power consumption required to receive data rates of ~50 Mbps and transmit pose information or UCI every ~4-5 msec. It is also noted that Rel-17 SSSG switching/skipping is expected to be able to effectively provide dynamic adaptation of C-DRX parameters such as for drx-inactivity timer (e.g. adjust the length of a long drx-inactivity timer by indicating a corresponding PDCCH skipping duration) and even avoid the need for C-DRX configuration. Moreover, a differentiation of I-frames and P-frames at RAN (different QoS flows) should make it easier to meet the PDB and re-use of Rel-17 UE power saving designs is then default while minor enhancements, such as having separate indication for PDCCH monitoring for DL/UL scheduling may be considered.

Observation 7: There is no apparent need to introduce new periodicities to C-DRX operation for XR.

Observation 8: Power consumption is an important consideration for XR but a Rel-18 SI on XR need not become an SI on UE power savings for new designs, models, and associated evaluations and complexity assessments. Rel-17 mechanisms should serve as baseline for consideration of possible gains from new mechanisms.

# Conclusions

This contribution considered proposed possible enhancements to Rel-17 for XR for inclusion to a Rel-18 SI on XR and proposes the following.

Proposal 1: The Rel-18 SI/WI on XR should include requests to SA2 and SA4 for mechanisms to provide separate QoS flows for I-frames and P/B-frames at RAN in order for RAN to identify IP packets with only respective frames.

Proposal 2: The Rel-18 XR SI phase should include RAN3 and consideration of ICIC mechanisms.

Proposal 3: The Rel-18 XR SI phase should include latency reduction for at least DL scheduling.

In addition, the following observations are made.

Observation 1: Material DL spectral efficiency gains for XR can be achieved by identifying separate QoS flows for I-frames and P-frames at the RAN.

Observation 2: With QoS flow differentiation, KPIs should be per QoS flow (separate KPIs for I-frames and P-frames).

Observation 3: RAN can process separate QoS flows for I-frames and Q-frames as in Rel-16.

Observation 4: Required conditions for RAN to drop IP packets of a video frame that cannot meet PDB are currently not supported, a user-experience impact from dropping such IP packets is unclear, and a SE gain is expected to be marginal.

Observation 5: PDSCH transmissions for XR are best supported by DCI and there is no need/motivation to enhance SPS.

Observation 6: PUSCH transmissions for XR can be based on Rel-16 CG-PUSCH and DCI-based PUSCH.

Observation 7: There is no apparent need to introduce new periodicities to C-DRX operation for XR.

Observation 8: Power consumption is an important consideration for XR but a Rel-18 SI on XR need not become an SI on UE power savings for new designs, models, and associated evaluations and complexity assessments. Rel-17 mechanisms should serve as baseline for consideration of possible gains from new mechanisms.

# References

- RP-210460 “Revised SID on XR Evaluations for NR”; RANP

- R1-2109521 “Potential enhancements for XR”; Samsung

- RP-212671 “[RAN94e-R18Prep-11] Enhancements for XR”; Moderator (Nokia)