3GPP TSG RAN WG1 #107-e			R1-2111769

e-Meeting, November 11th – 19th, 2021

Agenda item:	8.17.1

Source: 	Samsung

Title: 	UE features for Rel-17 NR FeMIMO

Document for:	Discussion and Decision

Introduction

This contribution provides Samsung’s views on Rel-17 UE features for FeNR-MIMO.

Enhancements on Multi-beam Operation

Based on the discussion in RAN1#106b-e, there are 12 components in FG 23-1-1 which is a basic feature that enables the unified TCI framework for both intra-cell and inter-cell beam management. Hence, we support to remove bracket for “[intra- and inter-cell]” in the name of FG and “[For both intra- and inter-cell beam management:]” to confirm that this FG can enable basic functionalities for both intra- and inter-cell beam management for Unified TCI.

Proposal 1: Remove the bracket for “[intra- and inter-cell]” in the name of FG and “[For both intra- and inter-cell beam management:]” to support functionalities for both intra- and inter-cell beam management for Unified TCI based on FG 23-1-1.

1. Support of joint DL/UL TCI update [and separate DL/UL TCI update] with their components (configuration mechanism, QCL rules, applicable source and target signals) [and Common cross-CC TCI update and activation (involving RRC common TCI state pool)]

Regarding the description of Component 1 for FG 23-1-1 as above, we are fine for both joint DL/UL TCI update as well as separate DL/UL TCI update. To avoid unnecessary complication/fragmentation, all Rel-17 UEs support both joint TCI and separate DL/UL TCI (i.e. there shouldn’t be separate UE capabilities for the two).

Proposal 2: Remove the bracket for “[and separate DL/UL TCI update]” to support both joint and separate DL/UL TCI update based on FG 23-1-1.

Proposal 3: Support “and Common cross-CC TCI update and activation (involving RRC common TCI state pool)”.

2. Support of association between TCI state and UL PC settings [for PUCCH, PUSCH, and SRS] [(PLRS and other, including handling of beam [alignment /misalignment] for PLRS)] [9. Support of association between a TCI state and PL-RS, where the “beam alignment” between the DL source RS in the UL or (if applicable) joint TCI state to provide spatial relation indication and the PL-RS is assumed by the UE

Regarding the description of Component 2 and 9 for FG 23-1-1 as above, we do not support to include beam alignment related parameters/proposals for PLRS, at least for now. It is too early to add this. Hence, in Component 2, we would like to remove “including handling of beam [alignment /misalignment] for PLRS)]”. Similarly, in Component 9, we prefer to remove “where the “beam alignment” between the DL source RS in the UL or (if applicable) joint TCI state to provide spatial relation indication and the PL-RS is assumed by the UE” and only support “Support of association between a TCI state and PL-RS”.

Proposal 4: Do not support to include beam alignment related parameters/proposals for PLRS. Remove “including handling of beam [alignment /misalignment] for PLRS)]” in Component 2 and “where the “beam alignment” between the DL source RS in the UL or (if applicable) joint TCI state to provide spatial relation indication and the PL-RS is assumed by the UE” in Component 9.

3. Supported mode of [MAC-CE/MAC-CE+DCI]-based beam TCI state indication [(including TCI state activation, use of DCI formats 1_1/1_2 with and without DL assignment)]

Regarding the description of Component 3 for FG 23-1-1 as above, we support MAC-CE/DCI based TCI state indication including all valid DCI formats (1_1/1_2 with and without DL assignment). We are against separating different types of beam indication mechanisms into different FGs, i.e. they should all be supported together. Hence, we prefer to revise Component 3 as “Supported mode of MAC-CE+DCI-based TCI state indication (including TCI state activation, use of DCI formats 1_1/1_2 with and without DL assignment)”.

Proposal 5: Revise the description of Component 3 as “Supported mode of MAC-CE+DCI-based TCI state indication (including TCI state activation, use of DCI formats 1_1/1_2 with and without DL assignment)”

[11. The minimum beam application time in Y symbols]

Regarding Component 11 for FG 23-1-1 as above, we support the component for minimum beam activation latency (Y symbols). The candidate values for the minimum Y can be discussed further. Note that Y is configurable by the NW.

Proposal 6: Support Component 11, i.e., “The minimum beam application time in Y symbols”, and the candidate values can be discussed further.

Regarding FG 23-1-2, this feature should cover both inter-cell beam measurement and reporting and inter-cell beam indication. We should rename this feature to support of inter-cell beam management.

Proposal 7: Rename FG 23-1-2 to “support of inter-cell beam management”.

1. Support of [L1-RSRP/beam] measurement and report on SSB(s) with PCI(s) different from serving cell PCI 2. Support of [L1-RSRP/beam] reporting of up to Kmax=4 SSBRI-RSRP[,SS-RSRP] [pairs/beams], with at least one [pair/beam] associated with a PCI different from serving cell PCI [in one report instance]

Regarding Components 1 and 2 for FG 23-1-2 as above, we support beam measurement and reporting on SSBs of cells with PCI different from that of the serving cell. Include a UE capability on maximum number of cells with PCI(s) different from that of the serving cell the UE can measure. Also, we support beam reporting for up to Kmax=4 beams (SSBRI-RSRP pairs) including beams with cells with PCI(s) different from that of the serving cell.

Proposal 8: Support Components 1 and 2 in FG 23-1-2.

In addition, the following should be cover by FG 23-1-2:

- Activation of TCI states associated, through indirect QCL Type-D relation, with SSBs of cells with PCI different from that of the serving cell.

- Include a Component on the maximum number of cells with different PCIs (including the serving cell) the activated TCI states can be associated with.

Proposal 9: Include support of activation of TCI states associated, through indirect QCL Type-D relation, with SSBs of cells with PCI different from that of the serving cell in FG 23-1-2.

Proposal 10: Include a component on the maximum number of cells with different PCIs (including the serving cell) the activated TCI states can be associated with.

Pending progress in AI 8.1.1 in RAN1, we can consider FG 23-1-4 for enhanced UE multi-panel support and discuss further after finalizing the feature.

Proposal 11: Consider FG 23-1-4 for enhanced UE multi-panel support.

Enhancements for Multi-TRP Deployment

  - Enhancements on Multi-TRP for PDCCH, PUCCH and PUSCH

PDCCH enhancement

Based on the discussion in RAN1#106b-e, there are 4 components including FFS point in FG 23-2-1 which is a basic feature for multi-TRP PDCCH repetition.

Regarding FG 23-2-1 for supporting PDCCH repetition, the name of the FG would be PDCCH repetition (i.e., removing “Multi-TRP”) since our view is that single-TRP based PDCCH repetition can be supported based on the definition of PDCCH repetition (two linked SS sets associated with corresponding CORESET(s) and the corresponding CORESET(s) can be both same and different CORESETs).

Proposal 12: Modify the name of FG 23-2-1 as PDCCH repetition (i.e., removing “Multi-TRP”).

1. Support of PDCCH repetition (based on two linked SS sets associated with corresponding CORESETs) [with non-SFN scheme TDM and FDM] [including PDCCH repetition for Type 3 CSS] Component1: details FFS

Regarding the description of Component 1 for FG 23-2-1 as above, we are fine the wording only “with TDM and FDM” since it seems “non-SFN scheme” and “TDM and FDM” are duplication. Also, we are fine “including PDCCH repetition for Type 3 CSS” and prefer to remove FG 23-2-3 which defines support of PDCCH repetition for Type 3 CSS.

Proposal 13: Support “with TDM and FDM” and “including PDCCH repetition for Type 3 CSS” in the description of Component 1 in FG 23-2-1 and remove FG 23-2-3.

2. Support of reporting one [or more] number(s) as required number of BDs for the two PDCCH candidates Component2: details FFS

Regarding the description and the candidate values of Component 2 for FG 23-2-1 as above, we support to delete “[or more]” since there is no consensus to introduce RRC configuration for the number of BDs, which means that gNB should follow UE capability reporting, i.e., one of 2 or 3. Hence, reporting more than 1 value for the number of BDs is not needed. As a consequence, the candidate values of Component 2 can be only 2 or 3.

Proposal 14: Support to delete “[or more]” in the description and support the candidate values 2 or 3 for Component 2 in FG 23-2-1.

3. [If 3 or {2, 3} is reported in component 2, support of whether the individual candidate is monitored or not when one of the linked PDCCH candidates uses the same set of CCEs as an individual (unlinked) PDCCH candidate, and they both are associated with the same DCI size, scrambling, and CORESET for the case that the linked PDCCH candidates is 3 BDs] [If 2 or {2,3} is reported in component 2, the individual candidate is monitored when one of the linked PDCCH candidates uses the same set of CCEs as an individual (unlinked) PDCCH candidate, and they both are associated with the same DCI size, scrambling, and CORESET for the case that the linked PDCCH candidates is 2 BDs.] Component3: details FFS

Regarding the description of Component 3 for FG 23-2-1 as above, we support to delete “or {2, 3}” due to the reason mentioned in Proposal 14. Also, during the discussion in AI 8.1.2.1 PDCCH enhancement, if a UE reports the number of BDs as 3, an individual PDCCH candidate can be monitored in case of overlapping (when one of the linked PDCCH candidates uses the same set of CCEs as an individual (unlinked) PDCCH candidate, and they both are associated with the same DCI size, scrambling, and CORESET) since 3 BDs may include at least two separate decoding of each linked PDCCH candidate. but for the case of 2 BDs, whether an individual PDCCH candidate can be monitored or not depends on UE capability since there can be a case when 2 BDs may not include two separate decoding of each linked PDCCH candidate. However, the current description for Component 3 seems opposite way, i.e., whether individual PDCCH candidate is monitored can be determined by UE capability in case of 3 BDs but it is monitored in case of 2 BDs.

Proposal 15: Support to delete “or {2, 3}” in the description for Component 3 in FG 23-2-1.

Regarding the granularity of UE capability reporting, the reporting level can be per FSPC due to the complex functionality and buffer burden of multi-TRP PDCCH repetition for UE.

Proposal 16: Support FG 23-2-1 per FSPC.

PUCCH/PUSCH enhancement

In our view, similar to single-DCI based mTRP PDSCH repetition (TDM schemes) in Rel-16, UE capability signalling for single-DCI based PUCCH/PUSCH repetition can be supported per band level.

Proposal 17: Support FG 23-3-2 for multi-TRP PUCCH repetition per band level.

Proposal 18: Support FG 23-3-1 and FG 23-3-1-1 multi-TRP PUSCH repetition type A and B, respectively, per band level.

In RAN1#106-e, per TRP PHR reporting is UE optional capability in Rel-17. Therefore, we can split component#4 in FG 23-3-1 as optional capability and add new UE capability signalling for that. And also prerequisite feature group of new UE capability signalling for enhanced PHR can be FG 23-3-1.

Proposal 19: Split Component 4 (per TRP PHR reporting) of FG 23-3-1 as separate FG which is per UE signaling.

  - Enhancements on Multi-TRP inter-cell operation

It was agreed to relate X1 to the case where the SSB time domain positions or periodicity of additional PCI(s) are not exactly the same as those of the serving cell PCI, and X2 to the case where the SSB time domain positions or periodicity of additional PCI(s). From our understanding, X1 and X2 are not relevant to FR1 and FR2, respectively. Hence, we suggest to remove “FR1/case1” and “FR2/case2” from components 3 and 4, respectively.

Proposal 20: Support to delete “FR1/case1” in the description for Component 3, and “FR2/case2” in the description for Component 4 in FG 23-4.

  - Enhancements on beam management for multi-TRP

For component 6 in FG 23-5-1, group-based beam reporting enhancement(s) was not discussed for inter-cell mTRP; we suggest to put bracket only on this part. Furthermore, the bracket on [in one report instance] should be removed, which is the essence of group based reporting.

Proposal 21: Support FG 23-5-1 in principle; support to put bracket only on [inter-cell] in Component 6 in FG 23-5-1; support to remove the bracket on [in one report instance] in Component 1 in FG 23-5-1.

Proposal 22: Support FG 23-5-2 for MTRP BFR enhancements.

  - Enhancements on HST-SFN deployment

Regarding a new required UE feature identifying two QCL-TypeD properties for SFNed PDCCH, since there are ongoing separate discussions for PDCCH repetition and SFNed PDCCH in PDCCH enhancement and HST-SFN agendas, respectively, but it is expected to make an aligned solution for both, our view is that FG 23-2-2 in PDCCH enhancement can be reused for the purpose of identifying two QCL-TypeD properties to receive SFNed PDCCH which two TCIs have been activated on a CORESET.

Proposal 23: Re-use FG 23-2-2 as identifying two QCL-TypeD properties for SFNed PDCCH.

Regarding FG 23-6-4, since all new default beam behaviors are included in a single FG so far, we would like to suggest to split this FG as three separate feature groups and each feature group can have their own purpose to support how many numbers of default beams and which direction of default beam.

Proposal 24: Split FG 23-6-4 as three separate feature groups and all three feature groups are FR2 only:

- Components 3 as a single feature group to require a new default beam behavior with a single default DL beam

- Components 1 and 2 to require a new default beam behavior with two default DL beams

- Components 4, 5, and 6 to require a new default beam behavior with a single default UL beam.

CSI enhancements: MTRP and FR1 FDD reciprocity

Multi-TRP CSI

Regarding FG 23-7-1, we would like to include the following components of UE basic feature

Proposal 25: Support 23-7-1, i.e., basic features of CSI enhancement for Multi-TRP.

- Support Nmax=1 as a component of UE basic feature for Multi-TRP CSI

- Support of a tuple: {the max # of ports per a resource, the max # of resources in a resource set}.

  - Include candidate value 16 Tx ports per resource, i.e., the tuple {16, Ks}.

- Include CSI report modes, i.e., Mode 1 (Option 1) with X=0 and Mode 2 (Option 2), as a component of UE basic feature for mTRP CSI

- For FR1 include CMR sharing between NCJT and sTRP measurement hypotheses as a component of UE basic feature

Regarding 23-7-2, similar to one of the components of FG 23-7-1, it can be reported as tuple, i.e., {the max # of ports per a resource, the max # of resources in a resource set}. The main objective of this feature group would be to indicate, as optional feature, the support of >16 ports per a CMR, i.e., >32 ports across two CMRs in a CMR pair.

Proposal 26: Support 23-7-2 to be reported as tuples, i.e., {the max # of ports per a resource, the max # of resources in a resource set}.

- Support candidate values of the maximum # ports as 24 and 32, i.e., the tuples {24, Ks} and {32, Ks}, as UE optional feature.

- Default Ks,max can be set as:

  - Ks,max=8 when CMR sharing is not enabled.

  - Ks,max=4 when CMR sharing is enabled

Regarding 23-7-3 it can be merged with 23-7-2.

Proposal 27: Merge 23-7-3 with 23-7-2.

Proposal 28: Support 23-7-4, i.e., the maximum number of CMR pairs (Nmax), as UE optional feature

- Support Nmax=2 as UE optional feature

Regarding 23-7-5, we support it for FR2.

Proposal 29: Support 23-7-5, i.e., whether two CMRs from a CMR pair configured for a NCJT measurement hypothesis can be used for Single-TRP measurement hypotheses, as UE optional feature for FR2.

Additional UE feature group is needed to indicate support of CSI report based on Mode 1 (Option 1) with X=1, 2 as UE feature.

Proposal 30: Support CSI report based on Mode 1 (Option 1) with X=1, 2 as UE optional feature.

FDD CSI

Proposal 31: add the following components

- FG 23-9-1: basis FG on Rel. 17 codebook

  - Support for rank 1-2, and

  - Support parameter combinations with M=1

- FG 23-9-2: support of M=2

  - We only support Component 2 as “Support parameter combinations with M=2” and delete other Components.

  - Support max value of #CSI-RS ports for which M=2 is supported by the UE

Conclusion

In this contribution, the following proposals are given:

Proposal 1: Remove the bracket for “[intra- and inter-cell]” in the name of FG and “[For both intra- and inter-cell beam management:]” to support functionalities for both intra- and inter-cell beam management for Unified TCI based on FG 23-1-1.

Proposal 2: Remove the bracket for “[and separate DL/UL TCI update]” to support both joint and separate DL/UL TCI update based on FG 23-1-1.

Proposal 3: Support “and Common cross-CC TCI update and activation (involving RRC common TCI state pool)”.

Proposal 4: Do not support to include beam alignment related parameters/proposals for PLRS. Remove “including handling of beam [alignment /misalignment] for PLRS)]” in Component 2 and “where the “beam alignment” between the DL source RS in the UL or (if applicable) joint TCI state to provide spatial relation indication and the PL-RS is assumed by the UE” in Component 9.

Proposal 5: Revise the description of Component 3 as “Supported mode of MAC-CE+DCI-based TCI state indication (including TCI state activation, use of DCI formats 1_1/1_2 with and without DL assignment)”

Proposal 6: Support Component 11, i.e., “The minimum beam application time in Y symbols”, and the candidate values can be discussed further.

Proposal 7: Rename FG 23-1-2 to “support of inter-cell beam management”.

Proposal 8: Support Components 1 and 2 in FG 23-1-2.

Proposal 9: Include support of activation of TCI states associated, through indirect QCL Type-D relation, with SSBs of cells with PCI different from that of the serving cell in FG 23-1-2.

Proposal 10: Include a component on the maximum number of cells with different PCIs (including the serving cell) the activated TCI states can be associated with.

Proposal 11: Consider FG 23-1-4 for enhanced UE multi-panel support.

Proposal 12: Modify the name of FG 23-2-1 as PDCCH repetition (i.e., removing “Multi-TRP”).

Proposal 13: Support “with TDM and FDM” and “including PDCCH repetition for Type 3 CSS” in the description of Component 1 in FG 23-2-1 and remove FG 23-2-3.

Proposal 14: Support to delete “[or more]” in the description and support the candidate values 2 or 3 for Component 2 in FG 23-2-1.

Proposal 15: Support to delete “or {2, 3}” in the description for Component 3 in FG 23-2-1.

Proposal 16: Support FG 23-2-1 per FSPC.

Proposal 17: Support FG 23-3-2 for multi-TRP PUCCH repetition per band level.

Proposal 18: Support FG 23-3-1 and FG 23-3-1-1 multi-TRP PUSCH repetition type A and B, respectively, per band level.

Proposal 19: Split Component 4 (per TRP PHR reporting) of FG 23-3-1 as separate FG which is per UE signaling.

Proposal 20: Support to delete “FR1/case1” in the description for Component 3, and “FR2/case2” in the description for Component 4 in FG 23-4.

Proposal 21: Support FG 23-5-1 in principle; support to put bracket only on [inter-cell] in Component 6 in FG 23-5-1; support to remove the bracket on [in one report instance] in Component 1 in FG 23-5-1.

Proposal 22: Support FG 23-5-2 for MTRP BFR enhancements.

Proposal 23: Re-use FG 23-2-2 as identifying two QCL-TypeD properties for SFNed PDCCH.

Proposal 24: Split FG 23-6-4 as three separate feature groups and all three feature groups are FR2 only:

- Components 3 as a single feature group to require a new default beam behavior with a single default DL beam

- Components 1 and 2 to require a new default beam behavior with two default DL beams

- Components 4, 5, and 6 to require a new default beam behavior with a single default UL beam.

Proposal 25: Support 23-7-1, i.e., basic features of CSI enhancement for Multi-TRP.

- Support Nmax=1 as a component of UE basic feature for Multi-TRP CSI

- Support of a tuple: {the max # of ports per a resource, the max # of resources in a resource set}.

  - Include candidate value 16 Tx ports per resource, i.e., the tuple {16, Ks}.

- Include CSI report modes, i.e., Mode 1 (Option 1) with X=0 and Mode 2 (Option 2), as a component of UE basic feature for mTRP CSI

- For FR1 include CMR sharing between NCJT and sTRP measurement hypotheses as a component of UE basic feature

Proposal 26: Support 23-7-2 to be reported as tuples, i.e., {the max # of ports per a resource, the max # of resources in a resource set}.

- Support candidate values of the maximum # ports as 24 and 32, i.e., the tuples {24, Ks} and {32, Ks}, as UE optional feature.

- Default Ks,max can be set as:

  - Ks,max=8 when CMR sharing is not enabled.

  - Ks,max=4 when CMR sharing is enabled.

Proposal 27: Merge 23-7-3 with 23-7-2.

Proposal 28: Support 23-7-4, i.e., the maximum number of CMR pairs (Nmax), as UE optional feature

- Support Nmax=2 as UE optional feature

Proposal 29: Support 23-7-5, i.e., whether two CMRs from a CMR pair configured for a NCJT measurement hypothesis can be used for Single-TRP measurement hypotheses, as UE optional feature for FR2.

Proposal 30: Support CSI report based on Mode 1 (Option 1) with X=1, 2 as UE optional feature.

Proposal 31: add the following components

- FG 23-9-1: basis FG on Rel. 17 codebook

  - Support for rank 1-2, and

  - Support parameter combinations with M=1

- FG 23-9-2: support of M=2

  - We only support Component 2 as “Support parameter combinations with M=2” and delete other Components.

  - Support max value of #CSI-RS ports for which M=2 is supported by the UE

References

- 3GPP RAN, RP-193133, Samsung, New WID: further enhancements on MIMO for NR

- R1-2108679, “Preliminary RAN1 UE features list for Rel-17 NR”, AT&T, NTT DOCOMO, INC.

- R1-2109474, “Views on Rel-17 CSI enhancements”, Samsung