3GPP TSG RAN WG1 #107-e	R1-2111771

e-Meeting, November 11th – 19th, 2021

Agenda item:	8.16.3

Source: 	Samsung

Title: 				     UE features for IIoT and URLLC

Document for:	Discussion and Decision

Introduction

This contribution discussed UE features for Rel-17 IIoT/URLLC.

Discussion

In [1], following agreements were made in RAN1#106bis-e.

Agreement

FG 25-1 is kept as “SPS HARQ-ACK deferral in case of TDD collision” as follows.

| 25.NR_IIOT_URLLC_enh | 25-1 | SPS HARQ-ACK deferral in case of TDD collision | 1. Idenfify HARQ-ACK bits of active SPS configurations for deferral in the initial PUCCH slot 2. Determination of the target PUCCH slot for SPS HARQ-ACK deferral 3. Multiplexing and transmission of deferred SPS HARQ-ACK information in the target PUCCH slot FFS whether to separate capability for handling of the collision for the same HARQ process due to deferred SPS HARQ-ACK | 5-18 | Yes | N/A |  | Per UE | No | No | N/A |  | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Note that yellow highlight means FFS and to be discussed further. These parts are provides as placeholders.

| Samsung’s view | It is not clear what main motivation separate capability provides for the case of handling of the collision due to same HARQ process. This is because similar UE behavior is already considered under 25-1 where HARQ-ACK information for a certain HARQ process ID should be dropped if deferral timing exceeds maximum configured deferral value. |
| --- | --- |

Agreement

FG 25-2 is kept as “Repetitions for PUCCH format 0, and 2 over multiple slots with K = 2, 4, 8” as follows.

| 25.NR_IIOT_URLLC_enh | 25-2 | Repetitions for PUCCH format 0, and 2 over multiple slots with K = 2, 4, 8 | Repetitions for PUCCH format 0 and 2 over multiple slots with K = 2, 4, 8 | 4-23 | Yes | N/A |  | Per UE | No | No | N/A |  | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Note that yellow highlight means FFS and to be discussed further. These parts are provides as placeholders.

| Samsung’s view |  |
| --- | --- |

Agreement

FG 25-3 is kept as “Repetitions for PUCCH format 0, 1, 2, 3 and 4 over multiple PUCCH subslots with configured K = 2, 4, 8” as follows.

| 25.NR_IIOT_URLLC_enh | 25-3 | Repetitions for PUCCH format 0, 1, 2, 3 and 4 over multiple PUCCH subslots with configured K = 2, 4, 8 | Repetitions for PUCCH format 0, 1, 2, 3 and 4 over multiple PUCCH subslots with RRC configured repetition factor K = 2, 4, 8 FFS whether to separate the capability per UCI type | 4-23 11-3 | Yes | N/A |  | Per UE | No | No | N/A |  | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Note that yellow highlight means FFS and to be discussed further. These parts are provides as placeholders.

| Samsung’s view | It is preferable that 25-3 supports all UCI types without separating. |
| --- | --- |

Agreement

FG 25-3a is kept as “Repetitions for PUCCH format 0, 1, 2, 3 and 4 over multiple PUCCH subslots using dynamic repetition indication” as follows.

| 25.NR_IIOT_URLLC_enh | 25-3a | Repetitions for PUCCH format 0, 1, 2, 3 and 4 over multiple PUCCH subslots using dynamic repetition indication | Repetitions for PUCCH format 0, 1, 2, 3 and 4 over multiple PUCCH subslots based on dynamic repetition indication. FFS whether to separate the capability per UCI type | 25-3 30-5 | Yes | N/A |  | Per UE | No | No | N/A |  | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Note that yellow highlight means FFS and to be discussed further. These parts are provides as placeholders.

| Samsung’s view | It is preferable that 25-3a supports all UCI types without separating. |
| --- | --- |

Agreement

FG 25-4 is kept as “One-shot HARQ ACK feedback triggered by DCI format 1_2” as follows.

FG 25-5 is kept as “PHY priority handling for one-shot HARQ ACK feedback” as follows.

FG 25-6 is kept as “Enhanced type 3 HARQ-ACK codebook feedback” as follows.

| 25.NR_IIOT_URLLC_enh | 25-4 | One-shot HARQ ACK feedbacktriggered by DCI format 1_2 | 1. Support feedback of type 3 HARQ-ACK codebook, triggered by a DCI 1_2 scheduling a PDSCH 2. Support feedback of type 3 HARQ-ACK codebook, triggered by a DCI 1_2 without scheduling a PDSCH using a reserved FDRA value | 10-16 11-1 | Yes | N/A |  | Per UE | No | No | N/A |  | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 25. NR_IIOT_URLLC_enh | 25-5 | PHY priority handling for one-shot HARQ ACK feedback | Support transmission of type 3 HARQ-ACK codebook using the first or second PUCCH configuration based on PHY priority indication in the triggering DCI | 10-16 11-4 | Yes | N/A |  | Per UE | No | No | N/A |  | Optional with capability signaling |
| 25. NR_IIOT_URLLC_enh | 25-6 | Enhanced type 3 HARQ-ACK codebook feedback | 1. Support feedback of enhanced type 3 HARQ-ACK codebook, triggered by a DCI 1_1 and DCI format 1_2 (for a UE supporting DCI format 1_2, and 1-1) 2. Support configuration of up to X enhanced type 3 HARQ-ACK codebooks. 3. Support feedback of a dynamically selected enhanced type 3 HARQ-ACK codebook based on triggering information in DCI 1_1 and DCI 1_2 (for a UE supporting DCI format 1_2, 11-1) 4. Support transmission of enhanced type 3 HARQ-ACK codebook using the first or second PUCCH configuration based on PHY priority indication in the triggering DCI (for a UE supporting two HARQ-ACK codebooks / PUCCH config in 11-4) | 10-16 | Yes | N/A |  | Per UE | No | No | N/A | For component 2, the UE indicates its capability in the number of enhanced type 3 HARQ-ACK codebooks: {1,...,X} For component 3, the dynamic indication is only supported if the UE for component 2 supports more than one enhanced type 3 HARQ-ACK codebook to be configured | Optional with capability signaling |

Note that yellow highlight means FFS and to be discussed further. These parts are provides as placeholders.

| Samsung’s view | In TR 38.822 v16.0.0, there are three 11-1s; 11-1 (Basic BAP procedures) for NR_IAB-Core, 11-1 (Additional measurement gap patterns for PRS measurements) for NR positioning and 11-1(Monitoring DCI format 1_2 and DCI format 0_2) for NR_L1enh_URLLC. So, this ambiguity should be clarified. |
| --- | --- |

Agreement

FG 25-7 is kept as “Triggered HARQ-ACK codebook re-transmission” as follows.

| 25. NR_IIOT_URLLC_enh | 25-7 | Triggered HARQ-ACK codebook re-transmission | 1. Support HARQ-ACK re-transmission from an earlier PUCCH slot based on the triggering information in DCI format 1_1 and DCI format 1_2 (for a UE supporting DCI format 1_2, 11-1) 2. Support the related PHY priority handling in terms of HARQ-ACK codebook selection and the applicable PUCCH configuration (for a UE supporting two HARQ-ACK codebooks / PUCCH config in 11-4) |  | Yes | N/A |  | Per UE | No | No | N/A |  | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Note that yellow highlight means FFS and to be discussed further. These parts are provides as placeholders.

| Samsung’s view | Similar to 25-4, it is likely that some of UEs doesn’t support 11-1. So, this capability should be separated depending on DCI format like 10-16 (DCI format 1_1) and 25-4(DCI format 1_2) Similar to 25-5, it is likely that some of UEs doesn’t support 11-4. So, second component should independent UE capability which is conditioned with 11-4. |
| --- | --- |

Agreement

FG 25-8 is kept as “Semi-static HARQ-ACK codebook for sub-slot PUCCH” as follows

| 25.NR_IIOT_URLLC_enh | 25-8 | Semi-static HARQ-ACK codebook for sub-slot PUCCH | Semi-static (Type 1) HARQ-ACK codebook for sub-slot based PUCCH configuration | 4-11 11-3 | Yes | N/A |  | Per UE | No | No | N/A |  | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Note that yellow highlight means FFS and to be discussed further. These parts are provides as placeholders.

| Samsung’s view |  |
| --- | --- |

Agreement

FG 25-9 is kept as “Semi-static PUCCH cell switching” as follows.

FG 25-10 is kept as “PUCCH cell switching based on dynamic indication” as follows.

| 25.NR_IIOT_URLLC_enh | 25-9 | Semi-static PUCCH cell (FFS or PUCCH carrier) switching | Semi-static PUCCH cell (FFS or PUCCH carrier)switching using configured time-domain pattern of applicable PUCCH cell / carrier FFS whether to separate the capability for different numerologies |  | Yes | N/A |  | Per UE | No | No | N/A |  | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 25.NR_IIOT_URLLC_enh | 25-10 | PUCCH cell (FFS or PUCCH carrier)switching based on dynamic indication | PUCCH cell (FFS or PUCCH carrier)switching based on dynamic indication in the DCI scheduling the PUCCH FFS whether to separate the capability for different numerologies |  | Yes | N/A |  | Per UE | No | No | N/A |  | Optional with capability signaling |

Note that yellow highlight means FFS and to be discussed further. These parts are provides as placeholders.

| Samsung’s view | - It should be separated depending on same or different numerologies since a UE should operate PUCCH transmission in slot basis. So, if different numerologies is considered, additional UE complexity should be required to process PUCCH cell switching over different numerologies. - In RAN1#106bis-e, it was agreed that PUCCH cell switching is only supported in TDD case. So, this UE capability should be only for TDD. |
| --- | --- |

Agreement

FG 25-11 is kept as “4-bits subband CQI” as follows.

| 25.NR_IIOT_URLLC_enh | 25-11 | 4-bits subband CQI | Subband CQI reporting with 4 bits per subband |  | Yes | N/A |  | Per UE | No | No | N/A |  | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Note that yellow highlight means FFS and to be discussed further. These parts are provides as placeholders.

| Samsung’s view |  |
| --- | --- |

Agreement

FG 25-12 is kept as “UE initiating a semi-static channel occupancy with configurations dependent on gNB semi-static channel access configurations” as follows.

| 25.NR_IIOT_URLLC_enh | 25-12 | UE initiating a semi-static channel occupancy with configurations dependent on gNB semi-static channel access configurations | Support initiating a semi-static channel access occupancy by the UE where the corresponding period is the same as, integer multiple of, or inter-factor of the period configured for a semi-static channel occupancy that can be initiated by gNB. | 10-1a | Yes | N/A |  | Per band | N/A | N/A | N/A | The signaling is per band but is only expected for a band where shared spectrum channel access must be used | Optional with capability signaling [This FG is a part of basic operation for following scenarios defined in TS38.300 Scenario A2, B, C, D and E with semi-static channel access mode] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Note that yellow highlight means FFS and to be discussed further. These parts are provides as placeholders.

| Samsung’s view | The sentences within bracket are not necessary since 10-1a has already the note. |
| --- | --- |

Agreement

FG 25-13 is kept as “UE initiating a semi-static channel occupancy with independent configurations from gNB semi-static channel access configurations” as follows.

| 25.NR_IIOT_URLLC_enh | 25-13 | UE initiating a semi-static channel occupancy with independent configurations from gNB semi-static channel access configurations | Support initiating a semi-static channel access occupancy by the UE where the corresponding period is independently configured from the period configured for a semi-static channel occupancy that can be initiated by gNB. | 10-1a | Yes | N/A |  | Per band | N/A | N/A | N/A | The signaling is per band but is only expected for a band where shared spectrum channel access must be used | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Note that yellow highlight means FFS and to be discussed further. These parts are provides as placeholders.

| Samsung’s view | We think that 25-12 should be Prerequisite feature groups instead of 10-1a since 25-12 is subset of 25-13. |
| --- | --- |

Agreement

FG 25-14 is kept with square brackets as “PHY prioritization of overlapping low-priority DG-PUSCH and high-priority CG-PUSCH” for further discussion as follows.

FG 25-15 is kept with square brackets as “PHY prioritization of overlapping high-priority DG-PUSCH and low-priority CG-PUSCH” for further discussion as follows.

| 25. NR_IIOT_URLLC_enh | [25-14] | PHY prioritization of overlapping low-priority DG-PUSCH and high-priority CG-PUSCH | Support PHY prioritization for the case where low-priority DG-PUSCH collides with high-priority CG-PUSCH | 12-1 | Yes | N/A |  | Per band | N/A | N/A | N/A |  | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 25. NR_IIOT_URLLC_enh | [25-15] | PHY prioritization of overlapping high-priority DG-PUSCH and low-priority CG-PUSCH | Support PHY prioritization of overlapping high-priority dynamic grant PUSCH and low-priority configured grant PUSCH on a BWP of a serving cell | 12-1 | Yes | N/A |  | Per band | N/A | N/A | N/A |  | Optional with capability signaling |

Note that yellow highlight means FFS and to be discussed further. These parts are provides as placeholders.

| Samsung’s view | We are fine to remove brackets for 25-14 and 25-15. |
| --- | --- |

Agreement

FG 25-16 is kept with as “HARQ-ACK with different priorities multiplexing using a PUCCH” as follows.

FG 25-17 is kept with as “HARQ-ACK piggybacked on a PUSCH of a different priority” as follows.

| 25.NR_IIOT_URLLC_enh | 25-16 | HARQ-ACK with different priorities multiplexing using a PUCCH | 1. Support multiplexing a high-priority HARQ-ACK and a low-priority HARQ-ACK into a PUCCH. Support separate coding for the two HARQ-ACKs. 2. [Support multiplexing a low-priority HARQ-ACK and a high-priority SR into a PUCCH for some HARQ-ACK/SR PF combinations (FFS applicable combinations).] 3. [Support multiplexing a low-priority HARQ-ACK, a high-priority HARQ-ACK and a high-priority SR into a PUCCH.] FFS whether to merge with FG 25-17 FFS whether to separate capability for different UCI type | 11-3 | Yes | N/A |  | Per UE | No | No | N/A |  | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 25.NR_IIOT_URLLC_enh | 25-17 | HARQ-ACK piggybacked on a PUSCH of a different priority | 1. Support multiplexing a low-priority HARQ-ACK in a high-priority PUSCH (conveying UL-SCH only). Support separate beta_offset values for this priority combination. 2. Support multiplexing a high-priority HARQ-ACK in a low-priority PUSCH (conveying UL-SCH only). Support separate beta_offset values for this priority combination. 3. Support multiplexing a low-priority HARQ-ACK, a high-priority PUSCH conveying UL-SCH, a high-priority HARQ-ACK and/or CSI. 4. Support multiplexing a high-priority HARQ-ACK, a low-priority PUSCH conveying UL-SCH, a low-priority HARQ-ACK and/or CSI. FFS whether to merge into FG 25-16 FFS whether to separate capability for different UCI type | 11-3 12-1 | Yes | N/A |  | Per UE | No | No | N/A |  | Optional with capability signaling |

Note that yellow highlight means FFS and to be discussed further. These parts are provides as placeholders.

| Samsung’s view | 25-16 and 25-17 should be merged together since both design principles are the same to UE and no implementation difference is identified. Furthermore, it may need additional huge specification efforts since RAN1 should consider the case where a UE supports only one of UE capabilities. This case may affect Intra-UE multiplexing procedures that has already a lot of important pending issues to be addressed. |
| --- | --- |

Agreement

FG 25-18 is kept as “Parallel PUCCH and PUSCH transmission across CCs in inter-band CA” as follows.

| 25.NR_IIOT_URLLC_enh | 25-18 | Parallel PUCCH and PUSCH transmission across CCs in inter-band CA | Support simultaneous PUCCH/PUSCH transmissions on different cells [at least] for inter-band CA. |  | Yes | N/A |  | Per UE | No | No | N/A |  | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Note that yellow highlight means FFS and to be discussed further. These parts are provides as placeholders.

| Samsung’s view | From at least RAN1 agreement, inter-band CA is only scope of simultaneous PUCCH/PUSCH transmission. So, [at least] should be removed and then can be revisited if intra-band case is also supported. |
| --- | --- |

Agreement

Rel-16 capability signaling for PUSCH repetition Type B (FG 11-5) is reused for unlicensed spectrum.

Agreement

- FG 25-1 is applicable to TDD only

  - It is clarified in the column of “Need of FDD/TDD differentiation”

Agreement

- FGs 25-9 and 25-10 are applicable to TDD only

  - It is clarified in the column of “Need of FDD/TDD differentiation”, FFS details.

Agreement

- Remove “[This FG is a part of basic operation for following scenarios defined in TS38.300 Scenario A2, B, C, D and E with semi-static channel access mode]” from the column of “Mandatory/Optional” in FG 25-12

Agreement

- FG 25-12 is added as prerequisite feature group for FG 25-13

Conclusion

This contribution considered the UE features list for Rel-17 URLLC/IIoT. Proposals are included in above table.

Reference

[1] R1-2109709, Summary on UE features for enhanced IIoT and URLLC