3GPP TSG RAN WG1 #107-e	        		             R1-2111772

e-Meeting, November 11th – 19th, 2021

Agenda item:	8.16.4

Source: 	Samsung

Title: 	UE features for NR NTN

Document for:	Discussion and Decision

Introduction

This contribution considers UE features list for Rel-17 NTN.

Discussions

In [1], the RAN1 UE features list for Rel-17 was discussed.

Mandatory/optional

The current version has mandatory for FG26-1/26-2/26-3. Since Rel-16, all features are basically optional. Instead, the following changes can be made. Also, FG26-4 “UE reporting of information about the UE specific TA pre-compensation” should be one of the basic FG because the gNB needs to know its value in order to schedule with proper offset.

| Features | Index | Feature group | Components | Mandatory/Optional |
| --- | --- | --- | --- | --- |
| 26. NR_NTN_solutions | 26-1 | Uplink Time pre-compensation | - UE specific TA calculation in RRC_IDLE and RRC_INACTIVE states based on its GNSS-acquired position and the serving satellite ephemeris. - UE specific TA calculation in RRC_CONNECTED state based on its GNSS-acquired position and the serving satellite ephemeris. - UE applies common TA according to the parameters provided by the network (if any) - For TA update in RRC_CONNECTED state, combination of both open (i.e. UE autonomous TA estimation, and common TA estimation) and closed (i.e., received TA commands) control loops - FFS: UE (re)acquire the serving satellite ephemeris and common TA upon the expiration of the corresponding validity timer | Mandatory Optional with capability signalling For UE supports NR [NTN/ satellite/HAPS/ATG], UE must indicate this FG is supported. [Note: This UE feature group is applicable only for NR NTN cell, for terrestrial cell this feature is not supported] This is the basic FG for NTN. |
| 26. NR_NTN_solutions | 26-2 | Uplink Frequency/Doppler pre-compensation | - [In RRC_IDLE and RRC_INACTIVE states calculate frequency pre-compensation to counter shift the Doppler experienced on the service link] - in RRC_CONNECTED states, calculate frequency pre-compensation to counter shift the Doppler experienced on the service link. | Mandatory Optional with capability signalling For UE supports NR [NTN/ satellite/HAPS/ATG], UE must indicate this FG is supported. [Note: This UE feature group is applicable only for NR NTN cell, for terrestrial cell this feature is not supported] This is the basic FG for NTN. |
| 26. NR_NTN_solutions | 26-3 | Enhancement on the timing relationship | - estimate of UE-gNB RTT - delaying the start of ra-ResponseWindow [by UE-gNB RTT] - delaying the start of msgB-ResponseWindow [by UE-gNB RTT] | Mandatory Optional with capability signalling For UE supports NR [NTN/ satellite/HAPS/ATG], UE must indicate this FG is supported. [Note: This UE feature group is applicable only for NR NTN cell, for terrestrial cell this feature is not supported] This is the basic FG for NTN. |
| 26. NR_NTN_solutions | 26-4 | UE reporting of information about the UE specific TA pre-compensation | - [The exact content of UE reporting of information about the UE specific TA pre-compensation] - FFS: frequency of the reports - FFS: granularity of the reported content | [Optional] with capability signalling [For UE supports NR [NTN/ satellite/HAPS/ATG], UE must indicate this FG is supported] [Note: This UE feature group is applicable only for NR NTN cell, for terrestrial cell this feature is not supported] This is the basic FG for NTN. |

Rename FG 26-7

In order to clarify, the following changes are proposed. Also, it does not need value 8 since it is already supported.

| Features | Index | Feature group | Components |
| --- | --- | --- | --- |
| 26. NR_NTN_solutions | [26-7] | [NTN Performance enhancement] Increased PDSCH aggregation factor | 1. The maximum number of supported aggregation factor (i.e., pdsch-AggregationFactor) for DL PDSCH is [X] FFS: X = 16 or 32 |

FG 26-5

It is proposed that the type of FG 26-5 should be per FSPC.

| Features | Index | Feature group | Components | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) |
| --- | --- | --- | --- | --- |
| 26. NR_NTN_solutions | 26-5 | Increasing the number of HARQ processes | - The maximal supported HARQ process number is 32 for both UL and DL - FFS: Support on the maximal HARQ process number is up to UE capability - FFS: separate features for DL and UL | Per FSPC |

Conclusions

This contribution considered the UE features list for Rel-17 NTN. The above changes are proposed.

- References

[1] R1-2109914, “Summary of UE features for NR NTN”, Moderators (AT&T, NTT DOCOMO), RAN1#106b-e, Oct., 2021.