3GPP TSG RAN WG1 #107-e	R1-2111776

# e-Meeting, November 11th – 19th, 2021

Agenda item:	8.16.8

Source:	Samsung

Title:	UE features for NR Coverage Enhancement

Document for:	Discussion and decision

- Introduction

This contribution discusses UE features for Rel-17 Coverage Enhancement WI.

- Feature Group for Enhancements on PUSCH Repetition Type A

Current FGs for enhanced PUSCH repetition Type A are separated for dynamic grant (DG, FG 30-1/30-2) and for configured grant (CG, FG 30-1a/30-2a). It is the approach taken in Rel-15 for PUSCH repetition, e.g., FG 5-16/5-17. On the other hand, in Rel-16, a single capability (FG 11-5) consists of DG and CG. In our view, the Rel-16 approach simplifies the capability signalling. Even for UE implementation perspective, we do not see the value of differentiating DG and CG from UE capability.

In this regard, we are fine with moderator’s latest proposal 2-1 from RAN1#106bis-e by removing the FFS [1].

Proposal 1: Combine DG and CG into a single feature group, i.e., merge FGs 30-1 and 30-1a and merge FGs 30-2 and 30-2a.

| Index | Feature group | Components |
| --- | --- | --- |
| 30-1 | Increased maximum number of PUSCH Type A repetitions | 1. K = 1, 2, 3, 4, 7, 8, 12, 16, 20, 24, 28, 32 times repetitions for DG-PUSCH. The number of repetitions is jointly coded with SLIV in TDRA list. A row index of the TDRA list is indicated by a DCI. 2. K = 1, 2, 3, 4, 7, 8, 12, 16, 20, 24, 28, 32 times repetitions for Type 2 CG-PUSCH. The number of repetitions is jointly coded with SLIV in TDRA list. A row index of the TDRA list is indicated by a Type 2 configured grant configuration. |
| 30-1a | Increased maximum number of Type 2 configurecd grant PUSCH Type A repetitions | K = 1, 2, 3, 4, 7, 8, 12, 16, 20, 24, 28, 32 times repetitions. The number of repetitions is jointly coded with SLIV in TDRA list. A row index of the TDRA list is indicated by a Type 1 configured grant configuration. |
| 30-2 | PUSCH Type A repetitions based on available slots | 1. Transmission occasions for K repetitions for DG-PUSCH are determined on the basis of available slots. RV is cycled across transmission occasions. 2. Transmission occasions for K repetitions for CG-PUSCH are determined on the basis of available slots. RV is cycled across transmission occasions. |
| 30-2a | Configurecd grant PUSCH Type A repetitions based on available slots | Transmission occasions for K repetitions for configured grant PUSCH are determined on the basis of available slots. RV is cycled across transmission occasions. |

- Feature Group for TBoMS

In RAN1#106bis-e, it was discussed whether/how to separate FG 30-3, e.g., split based on DG and CG. Similarly with the above discussion for PUSCH repetition Type A enhancement, we do not see the need to split current FG 30-3.

Another discussion point raised in previous RAN1 meeting was whether/how to include the capability for the repetition of TB processing over multi-slot PUSCH. In our view, if a UE supports TBoMS and PUSCH repetition Type A respectively, it means the UE also supports the repetition of TBoMS. That is, no need to introduce a separate capability for the repetition of TBoMS.

Proposal 2: FG 30-3 is kept as “TB processing over multi-slot PUSCH”

| 30-3 | TB processing over multi-slot PUSCH | Support of TB processing over multi-slot PUSCH in RRC connected mode. |
| --- | --- | --- |

- Feature Group for DM-RS bundling

It is understood that FGs 30-4/30-4a/30-4b/30-4c/30-4d/30-4e/30-4f/30-4g are within square bracket because the corresponding agreement is still working assumption or FFS. It can be further addressed based on RAN1 progress.

FG 30-4a/30-4b/30-4c/30-4d define specific DM-RS bundling capability combined with PUSCH Type A repetition, PUSCH Type B repetition, PUSCH TBoMS, and PUCCH, respectively. Similar with above discussion on FG 30-1/30-1a, a single capability for DM-RS bundling would be enough. For example, if a UE supports DM-RS bundling and PUSCH TBoMS respectively, it means the UE also supports DM-RS bundling for TBoMS.

Proposal 3: Merge FGs 30-4b, 30-4c, and 30-4d into FG 30-4a.

| 30-4a | [DM-RS bundling for PUSCH repetition type A] | Support DM-RS bundling for PUSCH repetition type A and PUCCH |
| --- | --- | --- |
| 30-4b | [DM-RS bundling for PUSCH repetition type B] | Support DM-RS bundling for PUSCH repetition type B |
| 30-4c | [DM-RS bundling for TB processing over multi-slot PUSCH] | Support DM-RS bundling for TB processing over multi-slot PUSCH |
| 30-4d | [DMRS bunding for PUCCH repetitions] | Support DM-RS bundling for PUCCH repetitions |

In case of frequency hopping with DM-RS bundling (FG 30-4e/30-4f), there would be no functional difference between PUSCH and PUCCH. Note that RAN1 agreement states “to strive for common design for PUSCH/PUCCH with DMRS bundling as much as possible”. Therefore, FG 30-4e and 30-4f can be consolidated.

Proposal 4: Merge FG 30-4f into FG 30-4e.

| 30-4e | [Inter-slot frequency hopping with inter-slot bundling for PUSCH/PUCCH] | Support inter-slot frequency hopping with inter-slot bundling for PUSCH and PUCCH |
| --- | --- | --- |
| 30-4f | [Enhanced inter-slot frequency hopping for PUCCH repetitions with DMRS bundling] | Enhanced inter-slot frequency hopping for PUCCH repetitions with DMRS bundling |

- Feature group for Msg3 repetition

Regarding FG 30-6 (Msg3 repetition), company views have been divided at RAN1#106bis-e [1]. We are still of the opinion that FG 30-6 is not needed because a UE can implicitly indicate its capability via PRACH transmission thanks to the RACH resource partition for msg3 repetition.

Proposal 5: Remove FG 30-6 from the UE feature for Rel-17 Coverage Enhancement.

| 30-6 | Msg3 repetition | Support of Msg3 repetition for Msg3 initial transmission and re-transmission in RRC connected mode. |
| --- | --- | --- |

- Mandatory/optional

All Rel-17 FGs should be “Optional with capability signaling”. We may need to decide which FG is the basic FG for Rel-17 Coverage Enhancement.

Proposal 6: Set “Optional with capability signaling” for all feature groups under Rel-17 Coverage Enhancement.

- Conclusions

We propose the following based on above discussion. Basically, the underlying assumption is to avoid introducing too specific/fragmented UE capability to support Rel-17 Coverage Enhancement:

Proposal 1: Combine DG and CG into a single feature group, i.e., merge FGs 30-1 and 30-1a and merge FGs 30-2 and 30-2a.

| Index | Feature group | Components |
| --- | --- | --- |
| 30-1 | Increased maximum number of PUSCH Type A repetitions | 1. K = 1, 2, 3, 4, 7, 8, 12, 16, 20, 24, 28, 32 times repetitions for DG-PUSCH. The number of repetitions is jointly coded with SLIV in TDRA list. A row index of the TDRA list is indicated by a DCI. 2. K = 1, 2, 3, 4, 7, 8, 12, 16, 20, 24, 28, 32 times repetitions for Type 2 CG-PUSCH. The number of repetitions is jointly coded with SLIV in TDRA list. A row index of the TDRA list is indicated by a Type 2 configured grant configuration. |
| 30-1a | Increased maximum number of Type 2 configurecd grant PUSCH Type A repetitions | K = 1, 2, 3, 4, 7, 8, 12, 16, 20, 24, 28, 32 times repetitions. The number of repetitions is jointly coded with SLIV in TDRA list. A row index of the TDRA list is indicated by a Type 1 configured grant configuration. |
| 30-2 | PUSCH Type A repetitions based on available slots | 1. Transmission occasions for K repetitions for DG-PUSCH are determined on the basis of available slots. RV is cycled across transmission occasions. 2. Transmission occasions for K repetitions for CG-PUSCH are determined on the basis of available slots. RV is cycled across transmission occasions. |
| 30-2a | Configurecd grant PUSCH Type A repetitions based on available slots | Transmission occasions for K repetitions for configured grant PUSCH are determined on the basis of available slots. RV is cycled across transmission occasions. |

Proposal 2: FG 30-3 is kept as “TB processing over multi-slot PUSCH”

| 30-3 | TB processing over multi-slot PUSCH | Support of TB processing over multi-slot PUSCH in RRC connected mode. |
| --- | --- | --- |

Proposal 3: Merge FGs 30-4b, 30-4c, and 30-4d into FG 30-4a.

| 30-4a | [DM-RS bundling for PUSCH repetition type A] | Support DM-RS bundling for PUSCH repetition type A and PUCCH |
| --- | --- | --- |
| 30-4b | [DM-RS bundling for PUSCH repetition type B] | Support DM-RS bundling for PUSCH repetition type B |
| 30-4c | [DM-RS bundling for TB processing over multi-slot PUSCH] | Support DM-RS bundling for TB processing over multi-slot PUSCH |
| 30-4d | [DMRS bunding for PUCCH repetitions] | Support DM-RS bundling for PUCCH repetitions |

Proposal 4: Merge FG 30-4f into FG 30-4e.

| 30-4e | [Inter-slot frequency hopping with inter-slot bundling for PUSCH/PUCCH] | Support inter-slot frequency hopping with inter-slot bundling for PUSCH and PUCCH |
| --- | --- | --- |
| 30-4f | [Enhanced inter-slot frequency hopping for PUCCH repetitions with DMRS bundling] | Enhanced inter-slot frequency hopping for PUCCH repetitions with DMRS bundling |

Proposal 5: Remove FG 30-6 from the UE feature for Rel-17 Coverage Enhancement.

| 30-6 | Msg3 repetition | Support of Msg3 repetition for Msg3 initial transmission and re-transmission in RRC connected mode. |
| --- | --- | --- |

Proposal 6: Set “Optional with capability signaling” for all feature groups under Rel-17 Coverage Enhancement.

References:

- R1-2109714, Summary on UE features for NR coverage enhancement, Moderator (NTT DOCOMO, INC.)

- R1-2110587, Updated RAN1 UE features list for Rel-17 NR after RAN1 #106bis-e, Moderators (AT&T, NTT DOCOMO, INC.)

# Annex: Updated RAN1 UE features list for Rel-17 NR after RAN1 #106bis-e [2]

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (Sidelink WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 30. NR_cov_enh | 30-1 | Increased maximum number of PUSCH Type A repetitions | K = 1, 2, 3, 4, 7, 8, 12, 16, 20, 24, 28, 32 times repetitions. The number of repetitions is jointly coded with SLIV in TDRA list. A row index of the TDRA list is indicated by a DCI. | [5-17] | Yes | N/A | UE does not support more than 16 repetitions. | [Per UE] | No | No | N/A |  | [Optional with capability signalling] |
| 30. NR_cov_enh | 30-1a | Increased maximum number of Type 2 configurecd grant PUSCH Type A repetitions | K = 1, 2, 3, 4, 7, 8, 12, 16, 20, 24, 28, 32 times repetitions. The number of repetitions is jointly coded with SLIV in TDRA list. A row index of the TDRA list is indicated by a Type 1 configured grant configuration. | [5-16], [30-1] | Yes | N/A | UE does not support more than 16 repetitions for Type 2configurecd grant PUSCH. | [Per UE] | No | No | N/A |  | [Optional with capability signalling] |
| 30. NR_cov_enh | 30-2 | PUSCH Type A repetitions based on available slots | Transmission occasions for K repetitions are determined on the basis of available slots. RV is cycled across transmission occasions. | [5-17] | Yes | N/A | UE does not support PUSCH repetitions counted on the basis of available slots. | [Per UE] | FFS | No | N/A |  | [Optional with capability signalling] |
| 30. NR_cov_enh | 30-2a | Configurecd grant PUSCH Type A repetitions based on available slots | Transmission occasions for K repetitions for configured grant PUSCH are determined on the basis of available slots. RV is cycled across transmission occasions. | [5-14 or 5-16], [30-2] | Yes | N/A | UE does not support configured grant PUSCH repetitions counted on the basis of available slots. | [Per UE] | FFS | No | N/A |  | [Optional with capability signalling] |
| 30. NR_cov_enh | 30-3 | TB processing over multi-slot PUSCH | Support of TB processing over multi-slot PUSCH in RRC connected mode. | [11-6] | Yes | N/A | UE does not support TB processing over multi-slot PUSCH. | [Per UE] | FFS | No | N/A |  | [Optional with capability signalling] |
| 30. NR_cov_enh | 30-4 | [The maximum duration for DM-RS bundling] | The maximum duration during which UE is able to maintain power consisitency and phase continuity to support DM-RS bundling for PUSCH/PUCCH |  | Yes | N/A | UE does not support DM-RS bundling for PUSCH/PUCCH | [Per UE] | FFS | No | N/A |  | [Optional with capability signalling] |
| 30. NR_cov_enh | 30-4a | [DM-RS bundling for PUSCH repetition type A] | Support DM-RS bundling for PUSCH repetition type A | [30-4], [30-1] or [30-2] | Yes | N/A | UE does not Support DM-RS bundling for PUSCH repetition type A | [Per UE] | FFS | No | N/A |  | [Optional with capability signalling] |
| 30. NR_cov_enh | 30-4b | [DM-RS bundling for PUSCH repetition type B] | Support DM-RS bundling for PUSCH repetition type B | [30-4], [11-5] [30-1] | Yes | N/A | UE does not Support DM-RS bundling for PUSCH repetition type B | [Per UE] | FFS | No | N/A |  | [Optional with capability signalling] |
| 30. NR_cov_enh | 30-4c | [DM-RS bundling for TB processing over multi-slot PUSCH] | Support DM-RS bundling for TB processing over multi-slot PUSCH | [30-4], [30-3] | Yes | N/A | UE does not Support DM-RS bundling for TB processing over multi-slot PUSCH | [Per UE] | FFS | No | N/A |  | [Optional with capability signalling] |
| 30. NR_cov_enh | 30-4d | [DMRS bunding for PUCCH repetitions] | Support DM-RS bundling for PUCCH repetitions | [30-4], [4-23] | Yes | N/A | UE does not support DMRS bunding for PUCCH repetitions | [Per UE] | FFS | No | N/A |  | [Optional with capability signalling] |
| 30. NR_cov_enh | 30-4e | [Inter-slot frequency hopping with inter-slot bundling for PUSCH] | Support inter-slot frequency hopping with inter-slot bundling for PUSCH | [30-4a] or [30-4b] or [30-4c] | Yes | N/A | UE does not support inter-slot frequency hopping with inter-slot bundling for PUSCH | [Per UE] | FFS | No | N/A |  | [Optional with capability signalling] |
| 30. NR_cov_enh | 30-4f | [Enhanced inter-slot frequency hopping for PUCCH repetitions with DMRS bundling] | Enhanced inter-slot frequency hopping for PUCCH repetitions with DMRS bundling | [30-4d] | Yes | N/A | UE does not support Enhanced inter-slot frequency hopping for PUCCH repetitions with DMRS bundling | [Per UE] | FFS | No | N/A |  | [Optional with capability signalling] |
| 30. NR_cov_enh | 30-4g | [Restart DM-RS bundling after the events that violate power consistency and phase continuity] | Support restarting DM-RS bundling after the events that violate power consistency and phase continuity | [30-4] | Yes | N/A | UE does not support restarting DM-RS bundling after the events that violate power consistency and phase continuity | [Per UE] | FFS | No | N/A |  | [Optional with capability signalling] |
| 30. NR_cov_enh | 30-5 | Slot based dynamic PUCCH repetition indication | Support dynamic PUCCH repetition indication | [4-23] | Yes | N/A | UE does not support Dynamic PUCCH repetition indication | [Per UE] | FFS | No | N/A |  | Optional with capability signalling |
| 30. NR_cov_enh | 30-6 | Msg3 repetition | Support of Msg3 repetition for Msg3 initial transmission and re-transmission in RRC connected mode. |  | Yes | N/A | UE does not support Msg3 repetition for Msg3 initial transmission and re-transmission in RRC connected mode. | [Per UE] | No | No | N/A |  | [Optional with capability signalling] |