3GPP TSG-RAN WG1 e-Meeting #107	R1- 2111790

Nov. 11th – Nov. 19th, 2021

Title:	Discussion on UL Time Synchronization for NTN

Source: 	Fraunhofer IIS, Fraunhofer HHI

Agenda Item:	8.4.2–Enhancements on UL Time and Frequency Synchronization

TDoc Type:	Discussion

Document for:	Decision

# Introduction

In RAN1#106b-e, the following agreements captured with respect to common timing advance (TA) signaling [3]:

RAN1#106b-e:

Agreement:

Confirm the working assumption:

Common TA may include parameter(s) indicating timing drift.

- The UE will apply common TA according to the parameters provided by the network (if any). No offset between the common TA according to the parameters provided by the network and the actual feeder link RTT is considered when defining UE UL timing error requirements.

Agreement:

Common TA Epoch time is implicitly known as a reference time defined by the starting time of a DL slot and/or frame.

- FFS: Whether this starting time is given by predefined rule or it is indicated by the Network

  - Note: “implicitly known” means that UTC is not provided to define the Common TA epoch time.

Agreement:

In NTN, the Network may optionally indicate one or more of the following parameters:

- Common TA , Common TA drift rate and Common TA drift rate variation.

- FFS: Common TA third order derivative.

- FFS: Details of combination of Common TA parameters

The main focus of this contribution is to address the FFS bullets in the above mentioned agreements.

# Discussion

In our previous contribution [4], we provided simulation results for the signaling of common TA in NTN taking into account the signaling of three parameters namely, the common TA drift rate, the common TA 2nd order drift rate, and common TA 3rd order drift rate. The summary of the simulation results are presented below. For all simulation results, we considered maximum elevation angle of 90 and minimum elevation angle of 10 degrees [1]-[2]. Two sample scenarios are considered for SIB update rate, i.e., 5 second and 28 second. The simulation results summary are provided in the following tables. Every entry in the table shows the maximum estimation error.

Table 1 - SIB update rate 5 [s]

| Scenario / SIB update | CommonTA drift rate | 2nd order drift rate | 3rd order drift rate |
| --- | --- | --- | --- |
| LEO 600 / 5 [s] | 1.0527 [µs] | 0.0051 [µs] | 7.1149×10-5 [µs] |
| LEO 1200 / 5 [s] | 0.4381 [µs] | 9.742×10-4 [µs] | 7.3032×10-6 [µs] |

Table 2 - SIB update rate 28 [s]

| Scenario / SIB update | Delay drift rate | 2nd order drift rate | 3rd order drift rate |
| --- | --- | --- | --- |
| LEO 600 / 28 [s] | 32.0996 [µs] | 0.8865 [µs] | 0.0597 [µs] |
| LEO 1200 / 28 [s] | 13.6574 [µs] | 0.1707 [µs] | 0.0059 [µs] |

Observation 1: Signaling the 3rd order drift rate parameter improves the common TA estimation at the UE side substantially.

Proposal 1: Network may optionally indicate the common TA 3rd order derivative parameter.

After providing the common TA parameters to the UE by the network, the UE can calculate the common TA, denote by TAcommon at time t as follows

TAcommont= TAcommonref(t0)+t-t0× TAcommon_drfit+ t-t02× TAcommon_2nd_drfit+ t-t03× TAcommon_3rd_drfit,

where

- t0 is a reference time for calculation of common TA, e.g. an implicit epoch time.

- TAcommonref(t0) is the value of common TA at reference time.

- TAcommon_drfit is the value of common TA drift.

- TAcommon_2nd_drfit is the value of common TA 2nd order drift.

- TAcommon_3rd_drfit is the value of common TA 3rd order drift.

Proposal 2: NTN UE calculates the common TA according to the common TA broadcast parameters describing a polynomial function given by

TAcommont= TAcommonref(t0)+t-t0× TAcommon_drfit+ t-t02× TAcommon_2nd_drfit+ t-t03× TAcommon_3rd_drfit,

where

- t0 is a reference time for calculation of common TA, e.g. an implicit epoch time.

- TAcommonref(t0) is the value of common TA at reference time.

- TAcommon_drfit is the value of common TA drift.

- TAcommon_2nd_drfit is the value of common TA 2nd order drift.

- TAcommon_3rd_drfit is the value of common TA 3rd order drift.

Furthermore, it can be observed from the equation above that common TA epoch time can be implicitly signaled and set to be the DL slot where common TA parameters broadcast to the UE.

Proposal 3: RAN1 to consider the DL slot where the common TA parameters broadcast to the NTN UE as the common TA epoch time.

# Conclusions

Observation 1: Signaling the 3rd order drift rate parameter improves the common TA estimation at the UE side substantially.

Proposal 1: Network may optionally indicate the common TA 3rd order derivative parameter.

Proposal 2: NTN UE calculates the common TA according to the common TA broadcast parameters describing a polynomial function given by

TAcommont= TAcommonref(t0)+t-t0× TAcommon_drfit+ t-t02× TAcommon_2nd_drfit+ t-t03× TAcommon_3rd_drfit,

where

- t0 is a reference time for calculation of common TA, e.g. an implicit epoch time.

- TAcommonref(t0) is the value of common TA at reference time.

- TAcommon_drfit is the value of common TA drift.

- TAcommon_2nd_drfit is the value of common TA 2nd order drift.

- TAcommon_3rd_drfit is the value of common TA 3rd order drift.

Proposal 3: RAN1 to consider the DL slot where the common TA parameters broadcast to the NTN UE as the common TA epoch time.

# References

[1] 3GPP TR 38.811, “Study on New Radio (NR) to support non terrestrial networks (Release 15),” 3rd Generation Partnership Project; Technical Specification Group Radio Access Network, Version 15.1.0, June 2019.

[2] 3GPP TR 38.821 v16.0.0 (2019-12): 3rd Generation Partnership Project; Technical Specification Group Radio Access Network; Solutions for NR to support non-terrestrial networks (NTN) (Release 16)

[3] 3GPP TSG RAN WG1 Meeting #106b-e, “RAN1 Chairman’s Notes 8.4 eom”, Oct. 2021.

[4] R1-2110292 “Discussion on UL Time Synchronization for NTN”, Fraunhofer IIS – Fraunhofer HHI, Oct. 2021.