3GPP TSG RAN WG1 #107-e		                                    		   R1-2111796

e-Meeting, Nov. 11th – Nov. 19th, 2021

Agenda Item:	8.8.3

Source:	InterDigital, Inc.

Title:	Type A PUSCH repetitions for Msg3

Document for:	Discussion and Decision

# Introduction

This contribution addresses the following issues for Type A PUSCH repetitions for Msg3:

- Indication of the number of repetitions for msg3 initial transmission and retransmission

- Use of flexible slots/symbols indicated via TDD-UL-DL-ConfigCommon

The contribution proposes to indicate the number of repetitions of msg3 initial transmission using TDRA field and same mechanism for retransmission.

# Indication of the number of repetitions for msg3

In RAN1#106b-e, RAN1 made a working assumption to down-select only one of 2 methods for indication of the number of repetitions of msg3 initial transmission. The two methods are to use TDRA-based indication (Alt. 1) or MCS information field (Alt. 2). RAN1 discussed extensively pros and cons of each method at RAN1#106-e and RAN1#106b-e [1].

It has been observed that Alt. 1 impacts the flexibility of time domain resource indication while Alt. 2 impacts the flexibility of MCS indication. However, the nature of the “flexibility impact” is different, and much worse for Alt. 2. In the case of Alt. 1, there is only impact on the number of possible combinations for existing entries of the TDRA table (e.g. K2, S, L) but there is no impact on the possible range of values that these parameters can take. The network still has possibility to configure any value for K2, S and L. On the other hand, in the case of Alt. 2, there is impact on the possible values of MCS that can be indicated. If Alt. 2 is adopted, it is not possible to indicate more than 4 values of MCS. Proponents of Alt. 2 argue that these values are not required for power-limited UEs, and that large number of PRBs may not have significant impact on the link budget. However, requiring large number of PRBs is a concern not only for link budget but also from system capacity perspective since other UEs are served by the cell.

Observation: Restricting msg3 transmission to lowest MCS values increases required number of PRBs for the UE and reduces resources available to other UEs.

Relying on a parameter (MCS) to not take certain values in a certain target scenario to repurpose a field for a completely unrelated parameter (number of repetitions) may be tempting but is not a clean design and is also limiting in terms of forward-compatibility, e.g. if a future feature would require use of larger MCS values. Another strong point in favour of Alt. 1 is that it is aligned with existing mechanism defined for PUSCH repetition.

Proposal 1: Support TDRA-based indication for the number of repetitions of msg3 initial transmission.

The same mechanism can also be used for msg3 retransmission:

Proposal 2: Support TDRA-based indication for the number of repetitions of msg3 retransmission.

RAN1 also made an agreement (in RAN1#106) to down-select between two options for the interpretation of the information field. In the first Option, the UE interprets msg2 assuming that msg3 repetition is enabled if it sent a preamble requesting msg3 repetition. In the second Option, there is an additional indication from the network of whether the UE should interpret msg2 assuming legacy behaviour or msg3 repetitions. The first Option is much simpler and does not require any signalling, and there does not seem to be any strong need to adopt Option 2 since the value K=1 can still be configured for the number of repetitions in Option 1.

Proposal 3: If a UE selects a PRACH resource associate with msg3 repetition for transmission of msg1, the UE obtains the number of repetitions from the repurposed information field or new TDRA table (Option 1).

RAN1 agreed to support repetition factors K from a set comprising at least {2, 3, 4, 7, 8} and values {1, 12, 16} are still under discussion. If proposal 2 is agreed, the value K=1 is to be indicated from the field that RAN1 will select for the indication of the number of repetitions. Values 12 and 16 can also be useful in more extreme coverage scenarios and are preferably supported as well.

Proposal 4: Support repetition factors {1, 2, 3, 4, 7, 8, 12, 16}.

# Use of flexible symbols

The network may want to configure flexible symbols in tdd-UL-DL-ConfigurationCommon. Forbidding the use of such symbols for msg3 repetitions in a fixed way would severely hamper the ability of the network to use these features at the same time. At the same time, it should be possible for the network to control whether such symbols can be used, preferably on a symbol basis. In R16, an RRC parameter InvalidSymbolPattern was introduced for the same purpose in support of PUSCH repetition Type B. A straightforward solution for the msg3 repetition case would be to allow configuration of this parameter in system information.

Proposal 5: Support configuration of InvalidSymbolPattern parameter in system information. Invalid symbols within InvalidSymbolPattern and DL symbols of tdd-UL-DL-ConfigurationCommon are not available for msg3 repetition.

# Conclusion.

This contribution proposes the following:

Proposal 1: Support TDRA-based indication for the number of repetitions of msg3 initial transmission.

Proposal 2: Support TDRA-based indication for the number of repetitions of msg3 retransmission.

Proposal 3: If a UE selects a PRACH resource associate with msg3 repetition for transmission of msg1, the UE obtains the number of repetitions from the repurposed information field or new TDRA table (Option 1).

Proposal 4: Support repetition factors {1, 2, 3, 4, 7, 8, 12, 16}.

Proposal 5: Support configuration of InvalidSymbolPattern parameter in system information. Invalid symbols within InvalidSymbolPattern and DL symbols of tdd-UL-DL-ConfigurationCommon are not available for msg3 repetition.

# References

[1] R1-2108584,	“Feature lead summary #3 on support of Type A PUSCH repetitions for Msg3”, Moderator (ZTE Corporation).

[2] R2-2110419, “Feature lead summary #3 on support of Type A PUSCH repetitions for Msg3”, Moderator (ZTE Corporation).

# Appendix

Agreements from RAN1#106b-e

NR_cov_enh – Msg3 repetition (From AI 5)

R1-2108712	LS on Msg3 repetition in coverage enhancement	RAN2, ZTE

Response LS needed. To be handled under agenda item 8.8.3.

The Draft LS of v01 is endorsed in principle.

Final LS R1-2110585 is endorsed.

Working Assumption

Down-select only one from the following methods for indication of the number of repetitions of Msg3 initial transmission.

- Alt 1: If TDRA information field is chosen, Option 2 is supported.

  - The candidate values for repetition factor could be chosen from {[1], 2, 3, 4, 7, 8, [12], [16]}

- Alt 2: If MCS information field is chosen, repurpose the MCS information field as follows.

  - 2 MSB bits of the MCS information field are used for selecting one repetition factor from a SIB1 configured set with 4 candidate values.

    - The set of candidate values for repetition factor could be chosen from {[1], 2, 3, 4, 7, 8, [12], [16]}

Note: Whether ‘1’ is included depends on the outcome of interpretation of the selected information field.

Agreement

Include the following into the reply LS to R1-2108712(R2-2109195).

RAN1 thinks at least the number of preambles per SSB per RO for request of Msg3 repetition, i.e., CB-PreamblesPerSSB, is needed. It’s up to RAN2 whether to indicate the start of preamble index for request of Msg3 repetition with shared RO.

Agreement

Include the following into the reply LS to R1-2108712(R2-2109195).

- From RAN1 perspective, there is no need to separately configure the following legacy RACH parameters configured in RACH-ConfigCommon for requesting Msg3 PUSCH repetition with shared RO on a given UL carrier.

- prach-ConfigurationIndex

- msg1-FDM

- msg1-FrequencyStart

- zeroCorrelationZoneConfig

- totalNumberOfRA-Preambles

- ssb-perRACH-OccasionAndCB-PreamblesPerSSB

- FFS: rsrp-ThresholdSSB

- rsrp-ThresholdSSB-SUL

- prach-RootSequenceIndex

- msg1-SubcarrierSpacing

- restrictedSetConfig

- msg3-transformPrecoder

Conclusion

There is no consensus to additionally support intra-slot frequency hopping for Msg3 PUSCH with repetition in Rel-17.

Note: intra-slot FH is supported when a UE is scheduled Msg3 PUSCH without repetition.

Agreement

Include the following into the reply LS to R1-2108712(R2-2109195)

 From RAN1 perspective, it can be beneficial to separately configure rsrp-ThresholdSSB for requesting Msg3 PUSCH repetition with shared RO on a given UL carrier.

Agreement

If UE is indicated with Msg3 PUSCH with repetition, the frequency hopping flag information field in UL RAR grant or DCI format 0_0 with CRC scrambled by TC-RNTI is reused to enable/disable inter-slot frequency hopping.

Agreement

The Rel-15/16 Msg3 PUSCH collision handling rules are reused for transmission of Msg3 PUSCH repetition in an available slot.

 FFS whether collision with downlink symbols indicated by tdd-UL-DL-ConfigurationDedicated is an exceptional case, i.e., Msg3 PUSCH repetition cannot be canceled by downlink symbols indicated by tdd-UL-DL-ConfigurationDedicated in Rel-17.

 FFS: Rel-17 Msg3 PUSCH collision rules are also applied if introduced in other WI(s)

Agreements from RAN1#106-e

Agreement

Do NOT support fallback RAR UL grant in 2-step RACH for indicating Msg3 repetition.

Agreement

The separate preambles for requesting Msg3 repetition could be configured only in an RO configured with 4-step RACH preambles not for requesting Msg3 repetition.

Working Assumption

Down-select only one from the following methods for indication of the number of repetition of Msg3 initial transmission.

- Alt 1: If TDRA information field is chosen, introducing a new configurable TDRA table including the repetition factors.

  - The new TDRA table is configured by SIB1, with selecting one of the two options below.

    - Option 1: The new TDRA table includes separate new indication for K2, mapping type, SLIV and repetition factor.

    - Option 2: The new TDRA table includes legacy indication for K2, mapping type and SLIV from legacy TDRA table, and new indication for repetition factor.

  - If a new TDRA table is not configured, the legacy default TDRA table is used, and repetition factor K=1 is applied.

    - K=1.

- Alt 2: If MCS information field is chosen, repurpose the MCS information field as follows.

  - X MSB bits of the MCS information field are used for repetition indication.

    - FFS the value of X.

    - FFS whether the X bits are directly used for indicating the repetition factor (i.e., the decimal value of X is equal to the repetition factor) or used for selecting one repetition factor from a predefined/SIB1 configured set.

- Alt 3: If TPC information field is chosen, repurpose the TPC information field by selecting one of the two options below.

  - Option 1: X LSB bits of the TPC information field are used for repetition indication.

    - FFS the value of X.

    - FFS whether the X bits are directly used for indicating the repetition factor (i.e., the decimal value of X is equal to the repetition factor) or used for selecting one repetition factor from a predefined/SIB1 configured set.

  - Option 2: A predefined TPC command table with including repetition factor K is introduced.

    - FFS details.

Agreement

Down-select one of the two options on how a UE should interpret the selected information field for indication of the number of repetitions.

- Option 1:

- When a UE requests Msg3 repetition, the new TDRA table or repurposed information field is applied. gNB schedules Msg3 with or without repetition for the UE requesting Msg3 repetition.

  - Repetition factor K=1 is included in the TDRA table or one entry/codepoint of the repurposed information field.

- When the UE doesn’t request Msg3 repetition (including legacy UE), the legacy TDRA table or legacy information field is applied. gNB schedules Msg3 without repetition for the UE not requesting Msg3 repetition.

- Option 2:

- When a UE requests Msg3 repetition, gNB schedules Msg3 with or without repetition by respectively using the new TDRA table or legacy TDRA table; or gNB schedules Msg3 with or without repetition by respectively using repurposed information field or legacy interpretation of information field. Whether the UE should apply the new or the legacy TDRA table, or apply repurposed or legacy interpretation of the information field, is indicated by gNB.

  - FFS details, e.g. implicit or explicit indication or predefined.

  - Repetition factor K=1 is NOT included in the TDRA table or one entry/codepoint of the repurposed information field.

- When the UE doesn't request Msg3 repetition (including legacy UE), gNB schedules Msg3 without repetition. The UE applies the legacy TDRA table, or the legacy interpretation of the information field.

Agreement

- Support at least repetition factor K = {2, 4} for Msg3 PUSCH repetition.

- FFS whether to support other values, e.g., 8.

- Note: K=1 is supported and how to support K=1 is FFS.

Agreement

- The available slot of Msg3 PUSCH repetition is only determined by the tdd-UL-DL-ConfigurationCommon and ssb-PositionsInBurst, no other additional Rel-16 signals/signalings will be considered.

- If a symbol for Msg3 repetition in a slot overlaps with SSB transmission [FFS:N Gap symbols after SSB], the slot is determined as not available during the counting of repetitions. As there is no Msg3 repetition in the slot, no Msg3 repetition omission applies to the slot.

Agreement

Do not support TBoMS for Msg3 in Rel-17 coverage enhancement WI.

Agreements from RAN1#105-e

R1-2106007

Update on 5/22:

Agreement: A UE requests Msg3 PUSCH repetition at least when the RSRP of the downlink pathloss reference is lower than an RSRP threshold.

- FFS the determination of the RSRP threshold.

Agreement: For repetition indication of Msg3 re-transmission, select one options from the following two options.

- Option 1: Use the same mechanism as supported for Msg3 initial transmission.

- Option2: Use HARQ process number bit field in DCI format 0_0 with CRC scrambled by TC-RNTI.

Agreement: Available slot for Msg3 PUSCH repetition doesn’t depend on dynamic SFI in DCI format 2-0.

Agreement: Available slot for Msg3 PUSCH repetition doesn’t depend on UL CI.

Agreement: Use a fixed RV sequence [0 2 3 1] for repetition of Msg3 initial and re-transmission.

- The RV cycling for Msg3 initial transmission follows the rule specified in the first row in Table 6.1.2.1-2 in TS38.214.

- The RV cycling for Msg3 re-transmission follows the rules specified in Table 6.1.2.1-2 in TS38.214.

- FFS: The RV cycling for Msg3 is based on transmission occasions on available slot.

Agreement:

- For requesting Msg3 PUSCH repetition, support the following:

  - Use separate preamble with shared RO configured by the same PRACH configuration index with legacy UEs.

    - FFS whether to introduce a PRACH mask to indicate a sub-set of ROs associated with a same SSB index within an SSB-RO mapping cycle for requesting Msg3 repetition for a UE.

    - FFS definition of shared RO (e.g., whether the shared RO can be an RO with preamble(s) for 4-step RACH only or with preambles for both 4-step RACH and 2-step RACH).

  - FFS whether or not to additionally support one (& only one) more option:

    - E.g., option 2: Use separate RO configured by a separate PRACH configuration index from legacy UEs

    - E.g., Option 3: Use separate RO, which include

      - the separate RO configured by a separate RACH configuration index from legacy UE, and

      - the remaining RO (if any) configured, by the same PRACH configuration index with legacy UEs, that cannot be used by legacy rules for PRACH transmission.

Agreement: Available slots for Msg3 PUSCH repetition do not depend on tdd-UL-DL-ConfigurationDedicated.

Agreement: Available slot for Msg3 PUSCH repetition depends on TDD-UL-DL-Configcommon.

- A slot is determined as available for Msg3 repetition only if the consecutive symbols allocated for Msg3 repetition in the slot are all available symbols.

  - UL symbols indicated by TDD-UL-DL-Configcommon are determined as available for Msg3 repetition.

  - FFS whether and how to use flexible symbols indicated by TDD-UL-DL-Configcommon.

Working assumption:

- Using an information field from the existing information fields in RAR UL grant for indication of the number of repetition of Msg3 initial transmission

  - Down-select only one from the following information fields in RAR UL grant for indication of the number of repetition of Msg3 initial transmission.

    - TDRA information field with introducing a new TDRA table including the repetition factors.

    - MCS information field

    - TPC information field

    - CSI request information field

    - FDRA information field

- The total size of RAR UL grant does not change.

- Position of all fields in the bit sequence of the RAR UL grant does not change, regardless of whether they are repurposed or not.

- FFS details, e.g., TDRA table selection, or whether/how to indicate which interpretation UE should use for the repurposed information field (legacy vs repurposed interpretation) etc.

Conclusion:

- Companies are encouraged to perform additional evaluations regarding intra-slot frequency hopping for Msg 3 with repetition. Aim to conclude whether or not to support this feature in RAN1#106-e (note: if supported, the intention is to not configure intra- and inter-slot frequency hopping simultaneously)

Final summary in R1-2106247

Agreements from RAN1#104b-e

R1-2103829

Agreement: For Msg3 PUSCH repetition,  support the following modified Option 2-1.

- Option 2-1: For UE requested triggered Msg3 PUSCH repetition with gNB indicating the number of repetitions,

- A UE can request trigger RACH procedure with Msg3 PUSCH repetition via separate PRACH resources (FFS details, e.g., separate PRACH occasion or separate PRACH preamble in case of shared PRACH occasions after SSB association, etc.).

- Whether a UE would request trigger is based on some conditions, e.g., measured SS-RSRP threshold, which may or may not have spec impact.

- If Msg3 PUSCH repetition is requested triggered by UE, gNB decides whether to schedule Msg3 PUSCH repetition or not. If scheduled, gNB decides the number of repetitions for Msg3 PUSCH 3 (re)-transmission.

- FFS the UE capability of supporting Msg3 PUSCH repetition can be reported after initial access procedure as usual

- FFS details if any.

Update on 4/16

Agreements: For the determination of RV for Msg3 PUSCH repetition,

- RV of the first repetition is determined in the same way as legacy.

  - Use RV 0 for the first repetition of Msg3 PUSCH initial transmission.

  - Use a dynamically indicated RV id via DCI 0_0 with CRC scrambled by TC-RNTI for the first repetition of Msg3 PUSCH re-transmission.

- FFS determination of the RV sequence.

Agreements: For indication of the number of repetitions for Msg3 initial transmission, Option 1 (i.e., using UL grant scheduling Msg3) is adopted.

- FFS additionally using MAC RAR for indication.

Agreements: For indication of the number of repetitions for Msg3 re-transmission, Option 1 (i.e., using DCI format 0_0 with CRC scrambled by TC-RNTI) is adopted.

Working assumption: The number of repetitions is counted on the basis of available slots for Type A PUSCH repetitions for Msg3.

- FFS: the determination of available slots.

Agreements from RAN1#104-e

Agreements:

- For indication of the number of repetitions for Msg3 initial transmission, down-select one option from the options below.

  - Option1: UL grant scheduling Msg3.

    - FFS details.

    - FFS fallbackRAR UL grant.

    - Note: Optimization specific for fallbackRAR UL grant in 2-step RACH is not considered in Rel-17 CovEnh WI, if supported.

  - Option2: DCI format 1_0 with CRC scrambled by RA-RNTI

    - FFS details.

  - Option3: SIB1 only

- Any modifications of RAR UL grant or DCI format 1_0 with CRC scrambled by RA-RNTI for indicating Msg3 repetitions shall not impact the legacy UE interpretation of the RAR or DCI format 1_0 with CRC scrambled by RA-RNTI respectively

Agreements:

- For indication of the number of repetitions for Msg3 re-transmission, down-select one option from the options below.

Option1: DCI format 0_0 with CRC scrambled by TC-RNTI.

FFS details.

Any modifications of DCI format 0_0 with CRC scrambled by TC-RNTI for indicating Msg3 repetitions shall not impact the legacy UE interpretation of the DCI format 0_0 with CRC scrambled by TC-RNTI.

Option2: Can be determined based on the repetition number  for  Msg3 initial transmission

Agreements:

Support inter-slot frequency hopping for repetition of Msg3 initial and re-transmission.

FFS details, e.g., signaling etc.

Agreements:

For Msg3 PUSCH repetition,  the following options are considered, aiming for down-selection in RAN1#104b-e:

- Option 1-1: For gNB scheduled Msg3 PUSCH repetition without UE request,

- A UE indicates to support of Msg3 PUSCH repetition via separate PRACH occasion or separate PRACH preamble in case of shared PRACH occasions.

- For a UE supporting Msg3 PUSCH repetition, gNB decides whether to schedule Msg3 PUSCH repetition or not. If scheduled, gNB decides the number of repetitions.

- FFS details if any.

- Option 1-2: For gNB scheduled Msg3 PUSCH repetition without UE request,

- gNB decides whether to schedule Msg3 PUSCH repetition or not. If scheduled, gNB decides the number of repetitions.

- For UE does not support Msg3 PUSCH repetition, UE transmits Msg3 PUSCH without repetition

- For UE does support Msg3 PUSCH repetition, UE transmits Msg3 PUSCH with repetition as indicated by gNB and UE uses, e.g., separate DMRS configuration or UCI multiplexing with Msg3 PUSCH (or other ways)

- Note: e.g., this can be for differentiation between UEs not supporting Msg3 PUSCH repetition and Rel-17 CE UEs supporting Msg3 PUSCH repetition or between RACH procedure with Msg3 PUSCH repetition and Msg3 PUSCH without repetition, etc.

- gNB blindly decodes Msg3 PUSCH with two different assumptions, w/ and w/o repetition.

- FFS details if any.

- Option 2-1: For UE triggered Msg3 PUSCH repetition with gNB indicating the number of repetitions,

- A UE can trigger RACH procedure with Msg3 PUSCH repetition via separate PRACH occasion or separate PRACH preamble in case of shared PRACH occasions.

- Whether a UE would trigger is based on some conditions, e.g., measured SS-RSRP threshold, which may or may not have spec impact.

- If Msg3 PUSCH repetition is triggered by UE, gNB decides the number of repetitions for Msg3 PUSCH 3 (re)-transmission.

- FFS details if any.

- Option 2-2: For UE triggered Msg3 PUSCH repetition with gNB indicating the number of repetitions,

  - gNB decides whether to schedule Msg3 PUSCH repetition or not. If scheduled, gNB decides the number of repetitions.

  - If Msg3 PUSCH repetition is scheduled, UE transmits Msg3 PUSCH with or without repetition. If UE transmits Msg3 PUSCH repetition, the number of repetition follows the indication of gNB and UE uses e.g., separate DMRS configuration or UCI multiplexing with Msg3 PUSCH (or other ways)

- Whether a UE would trigger is based on some conditions, e.g., measured SS-RSRP threshold, which may or may not have spec impact.

  - FFS details if any.

- Other options are not precluded.