3GPP TSG RAN WG1 #107-e	R1-2111828

e-Meeting, November 11th – November 19th, 2021

Agenda item:      8.14.1

Source:	Nokia, Nokia Shanghai Bell

Title:	Performance results in indoor hotspot and dense urban deployments of CG and VR/AR applications

Document for:     Discussion

# Introduction

NOTE: The main new results (for RAN1#107-e) are presented for mobility evaluation in Section 6. Other results are mostly in line with those submitted prior to RAN1#106b-e.

RAN#91 approved a revised SID on XR Evaluations for NR [1]. The XR SI is now ongoing with focus on related system-level performance evaluation to assess how well the currently standardized NR system can support such services. In this document, we present our preliminary results of XR and CG services in 5G NR. Two deployment scenarios are considered for evaluating the performance of XR and CG services, as agreed in [2]: (1) Indoor Hotspot (InH) and (2) Dense Urban (UMa). In these two deployments, we evaluate DL and UL system capacity of Cloud Gaming (CG) and Virtual Reality/Augmented Reality (VR/AR) applications according to the DL traffic models discussed in [3].

# Statistical Models and KPI for XR Traffic

In this section, we discuss the parameters we used in our performance evaluation for the statistical model for XR traffic. The parameters have been set according to the discussion in [2,4,5]. Table 1 illustrates the parameters of the traffic models used for generating DL and UL traffic. Table 2 and Table 3 illustrate the traffic models of XR applications as well as their QoS constraints used to evaluate user satisfaction and system capacity for DL and UL, respectively. Note that the maximum packet size of the transport layer is fixed to the maximum frame size to avoid segmentation at transport layer.

Table 1 – Parameters of DL and UL traffic models for XR applications.

| Parameter | Value |  |  |  |
| --- | --- | --- | --- | --- |
| Type of content | Video (DL) | Video (DL) | Video (UL) | Pose (UL) |
| Traffic model | Single stream for dual-eye buffer | Single stream for dual-eye buffer | Single stream for dual-eye buffer | Single stream |
| Number of streams | 1 | 1 | 1 | 1 |
| Packet size distribution | Truncated Gaussian |  |  | Constant |
| Bitrate | 30 Mbps | 45 Mbps | 10 Mbps | 200 kbps |
| Packet size (avg) | 62500 Bytes | 93750 Bytes | 20833 Bytes | 100 Bytes |
| Packet size (std) | 6563 Bytes (10.5% of mean value) | 9844 Bytes (10.5% of mean value) | 2188 Bytes (10.5% of mean value) | 0 |
| Packet size (max) | 93750 Bytes (150% of mean value) | 140625 Bytes (150% of mean value) | 31250 Bytes (150% of mean value) | 100 Bytes |
| Packet size (min) | 31250 Bytes (50% of mean value) | 46875 Bytes (50% of mean value) | 10417 Bytes (50% of mean value) | 100 Bytes |
| Transport protocol | UDP |  |  |  |
| Maximum transport packet size | 93750 Bytes 1 packet = 1 PDU | 140625Bytes 1 packet = 1 PDU | 31250 Bytes 1 packet = 1 PDU | 100 Bytes 1 packet = 1 PDU |
| Packet Rate | 60 fps | 60 fps | 60 fps | 250 pps |
| Inter-arrival time distribution | Truncated Gaussian |  |  | Periodic |
| Interarrival time (avg) | 16.67 ms |  |  | 4 ms |
| Interarrival time (std) | 2 ms |  |  | 0 |
| Interarrival time (max) | 20.67 ms |  |  | 4 ms |
| Interarrival time (min) | 12.67 ms |  |  | 4 ms |

Table 2 – Traffic models and QoS constraints used to evaluate XR applications in DL direction.

| Application | CG | VR | AR |
| --- | --- | --- | --- |
| Traffic model | Video single-stream | Video single-stream | Video single-stream |
| Bitrate | 30 Mbps | {30, 45} Mbps | {30, 45} Mbps |
| Packet rate | 60 fps | 60 fps | 60 fps |
| Packet Delay Budget (PDB) | 15 ms | 10 ms | 10 ms |
| Number of streams | 1 | 1 | 1 |

Table 3 – Traffic models and QoS constraints used to evaluate XR applications in UL direction.

| Application | CG | VR | AR | AR |
| --- | --- | --- | --- | --- |
| Traffic model | Pose single-stream | Pose single-stream | Video single-stream | Video+Pose dual-stream |
| Bitrate | 200 kbps | 200 kbps | 10 Mbps | 10Mbps + 200kbps |
| Packet rate | 250 pps | 250 pps | 60 fps | 60 fps + 250 pps |
| Packet Delay Budget (PDB) | 10 ms | 10 ms | 30 ms | 30 ms + 10ms |
| Number of streams | 1 | 1 | 1 | 1 |

As detailed in [2,4,5], the system capacity is measured in terms on “number of satisfied UEs”. More specifically, a UE is marked as “satisfied UE” if more than X% of packets are successfully transmitted over the air interface within the PDB. The system capacity is defined as the maximum number of UEs per cell with at least Y% of UEs marked as satisfied UEs. In our evaluation, we adopt X=99% as the percentage of frames to be successfully received within the PDB to identify satisfied UEs, while Y=90% as the percentage of satisfied UEs to measure the system capacity.

# Capacity Evaluation

In the following, we illustrate the results for capacity evaluation obtained with our system level simulator in Indoor Hotspot and Dense Urban deployments. Results are illustrated in Section 3.1 and 3.2, respectively. System configurations used for the two deployment scenario are described in Appendix. In our evaluation, we independently evaluate the DL and UL capacity.  For both deployment scenarios, we first illustrate the DL system capacity for “Video Single-Stream” traffic model with parameters for CG and VR/AR applications. Then, we illustrate the UL system capacity for “Pose Single Stream” traffic model for CG/VR and AR “Video Single-Stream”. We selected the even load random placement of UEs in the simulation area to be sure that all cells get the same number of connected UEs. Specifically, UEs are randomly dropped in the simulated area one by one according to a spatial uniform distribution. This process is repeated for all UEs until each cell served exactly the same number of UEs. Note that cell selection for a UE is based on RSRP.

## Indoor Hotspot (InH) scenario

In the following, we illustrate the results for capacity evaluation obtained with our system level simulator in InH scenario.

### DL System Capacity: Cloud Gaming (Video Single-Stream)

Figure 1 shows the ratio of satisfied UEs for CG computed with 99% of frames received within the PDB of 15ms as a function of the average cell load, which is measured in number of UEs per cell. Results correspond to a system bandwidth of 100MHz for both FR1 and FR2, and for data rate of 30Mbps. The dashed black line indicates the 90% limit on the number of satisfied UEs to measure the system capacity. According to the 90% limit, we conclude that in the Indoor Hotspot deployment we can support up to 5 UEs per cell (i.e., 60 UEs in the network) in FR1, with the capacity showing a significant increase in FR2 with up to 11 satisfied UEs per cell (i.e., 132 UEs in the network). The higher capacity for FR2 is mainly due to the use of the gNB GoB and the multi panel UE.

| (a) CG in FR1 at 30Mbps with X=99%. | (b) CG in FR2 at 30Mbps with X=99%. |
| --- | --- |

Figure 1 – DL capacity evaluation of CG traffic in Indoor Hotspot in FR1 and FR2 with X=99% of frames received within PDB, 100MHz system bandwidth and cells evenly loaded. The lower bound on the percentage of satisfied user is Y=90% (dashed line).

Observation 1: When we consider CG traffic model at 30Mbps for an InH deployment, we can support the number of UEs per cell in DL indicated in Table 4.

Table 4 – System capacity in DL for CG (PDB=15ms) at 30Mbps [#UEs/cell] in an InH deployment

|  | 30Mbps |
| --- | --- |
| FR1 | 5 |
| FR2 | 11 |

### DL System Capacity: Virtual/Augmented Reality (Video Single-Stream)

In this section, we illustrate the results of the DL system capacity obtained for VR/AR applications with PDB=10ms. Figure 2 shows the ratio of satisfied UEs computed with 99% of frames received within the PDB as a function of the average cell load, which is measured in number of UEs; results correspond to a system bandwidth of 100MHz for both FR1 and FR2, and data rates of 30Mbps and 45Mbps. The dashed black line indicates the 90% limit on the number of satisfied UEs to measure the system capacity. According to the 90% limit, we can conclude that in the Indoor Hotspot deployment in FR1 we can support 5 UEs per cell running VR/AR applications at 30Mbps (i.e., 60 UEs in the network), with such capacity increasing to 10 UEs per cell (i.e., 120 UEs in the network) for the same data rate when using FR2. For a 45Mbps data rate, in FR1 we can support 3 UEs per cell (i.e., 36 UEs in the network) whereas in FR2 up to 5 UEs per cell (i.e., 60 UEs in the network) can be supported.

| (a) VR/AR in FR1 at 30Mbps with X=99%. | (b) VR/AR in FR2 at 30Mbps with X=99%. |
| --- | --- |
| (c) VR/AR in FR1 at 45Mbps with X=99%. | (d) VR/AR in FR2 at 45Mbps with X=99%. |

Figure 2 –  DL capacity evaluation of VR/AR traffic in Indoor Hotspot in FR1 and FR2 with X=99% of frames received within PDB, 100MHz system bandwidth and cells evenly loaded. The lower bound on the percentage of satisfied user is Y=90% (dashed line).

Observation 2: When we consider VR/AR traffic model at 30Mbps and 45Mbps for an InH deployment, we can support the number of UEs per cell in DL indicated in Table 5.

Table 5 – System capacity in DL for VR/AR (PDB=10ms) at 30Mbps [#UEs/cell] in an InH deployment

|  | Rate: 30Mbps | Rate: 45Mbps |
| --- | --- | --- |
| FR1 | 5 | 3 |
| FR2 | 10 | 6 |

### UL System Capacity: Cloud Gaming and Virtual Reality (Pose Single-Stream)

In this section, we illustrate the results of the UL system capacity obtained considering the “Pose Single-Stream” traffic model for CG and VR applications at 200kbps, with PDB=10ms. Figure 3 shows the ratio of satisfied UEs when cells are evenly loaded. The ratio of satisfied UEs has been computed with 99% of packets containing pose information received within the PDB for an FR1 configurations as a function of the average cell load, which is measured in number of satisfied UEs. The dashed black line indicates the 90% limit on the number of satisfied UEs to measure the system capacity. We can observe that up to 54 UEs per cell running XR services and generating pose information can be supported in UL in FR1.

Figure 3 – UL capacity evaluation of CG and VR traffic (Pose Single-Stream) in FR1 with X=99% of frames received within PDB and cells evenly loaded. The lower bound on the percentage of satisfied user is Y=90% (dashed line).

Observation 3: When we consider baseline CG and VR traffic model for UL (Pose Single-Stream) for Indoor Hotspot deployment, we can support a number of UEs per cell indicated in Table 6.

Table 6 – System capacity in UL for CG and VR (Pose Single-Stream) at 200kbps [#UEs/cell]

|  | Rate: 200 Kbps |
| --- | --- |
| FR1 | 54 |

### UL System Capacity: Augmented Reality (Video Single-Stream and Video+Pose Dual-Stream)

In this section, we illustrate the results of the UL system capacity obtained considering the “Video Single-Stream” and “Video+Pose Dual-Stream” traffic models for AR applications. Video stream generates 10Mbps and its packets have a PDB=30ms. Pose information generates a bitrate of at 200kbps and its packets have PDB=10ms. Figure 3 shows the ratio of satisfied UEs when cells are evenly loaded. The ratio of satisfied UEs has been computed with 99% of packets containing pose information received within the PDB=10ms and 99% of video frame received withing PDB=10ms as a function of the average cell load, which is measured in number of satisfied UEs. The dashed black line indicates the 90% limit on the number of satisfied UEs to measure the system capacity. We can observe that up to 4 UEs per cell running AR services can be supported in UL in FR1 for both traffic models.

| (a) AR Single-Stream in FR1 at 10Mbps with X=99%. | (b) AR Dual-Stream in FR1 at 10Mbps and 200kbps with X=99%. |
| --- | --- |

Figure 4 – UL capacity evaluation of AR traffic models (Video Single-Stream and Video+Pose Dual-Stream) in FR1 with X=99% of frames received within PDB and cells evenly loaded. The lower bound on the percentage of satisfied user is Y=90% (dashed line).

Observation 4: When we consider baseline AR traffic models for UL (Pose Single-Stream and Video+Pose Dual-Stream) for Indoor Hotspot deployment, we can support a number of UEs per cell indicated in Table 7.

Table 7 – System capacity in UL for AR traffic models (PDB=10ms for video and PDB=10ms for pose) at 10Mbps (Video stream) and 200kbps (Pose stream) [#UEs/cell] in an InH deployment

|  | Single-Stream at 10Mbps | Dual-Stream at 10Mbps and 200kbps |
| --- | --- | --- |
| FR1 | 4 | 4 |

## Dense Urban (DU) scenario

In the following, we illustrate the results for capacity evaluation obtained with our system level simulator in DU scenario.

### DL System Capacity: Cloud Gaming (Video Single-Stream)

In this section, we illustrate the results on the DL system capacity obtained considering the “Video Single-Stream” traffic model for CG application at 30Mbps, with PDB=15ms. Figure 5 shows the ratio of satisfied UEs when cells are evenly loaded. The ratio of satisfied UEs has been computed with 99% of frames received within the PDB both in FR1 and FR2 configurations as a function of the average cell load, which is measured in number of UEs. The dashed black line indicates the 90% limit on the number of satisfied UEs to measure the system capacity. We can observe that up to 8 UEs per cell running CG services can be supported in DL in both FR1 and FR2.

| (a) CG in FR1 at 30Mbps with X=99%. | (b) CG in FR2 at 30Mbps with X=99%. |
| --- | --- |

Figure 5 – DL Capacity evaluation of CG traffic (Video Single-Stream) for Dense Urban deployment in FR1 and FR2 with X=99% of frames received within PDB and cells evenly loaded. The lower bound on the percentage of satisfied user is Y=90% (dashed line).

Observation 5: When we consider baseline CG traffic model for DL (Video Single-Stream) for Dense Urban deployment, we can support the number of UEs per cell indicated in Table 8.

Table 8 – System capacity in DL for CG (PDB=15ms) at 30Mbps [#UEs/cell]

|  | Rate: 30Mbps |
| --- | --- |
| FR1 | 8 |
| FR2 | 8 |

### DL System Capacity: Virtual/Augmented Reality (Video Single-Stream)

In this section, we illustrate the results on the DL system capacity obtained considering the “Video Single-Stream” traffic model for VR/AR application, namely with PDB=10ms. As in the previous analysis, UE are placed in the simulation area to obtain evenly loaded cells in terms of connected UEs. Figure 6 shows the ratio of satisfied UEs when cells are evenly loaded. The ratio of satisfied UEs has been computed with 99% of frames received within the PDB both in FR1 and FR2 configurations as a function of the average cell load, which is measured in number of UEs. The dashed black line indicates the 90% limit on the number of satisfied UEs to measure the system capacity. Given the smaller PDB, we can observe that up to 6 UEs per cell running VR/AR services at 30Mbps can be supported in DL in FR1 and FR2. Furthermore, we can observe that up to 4 and 3 UEs per cell running VR/AR services at 45Mbps can be supported in DL in FR1 and FR2, respectively.

| (a) VR/AR in FR1 at 30Mbps with X=99%. | (b) VR/AR in FR2 at 30Mbps with X=99%. |
| --- | --- |
| (c) VR/AR in FR1 at 45Mbps with X=99%. | (d) VR/AR in FR2 at 45Mbps with X=99%. |

Figure 6 – DL capacity evaluation of VR/AR traffic (Video Single-Stream) for Dense Urban deployment in FR1 and FR2 with X=99% of frames received within PDB and cells evenly loaded. The lower bound on the percentage of satisfied user is Y=90% (dashed line).

Observation 6: When we consider baseline VR/AR traffic model for DL (Video Single-Stream) for Dense Urban deployment, we can support a number of UEs per cell indicated in Table 9.

Table 9 – System capacity in DL for VR/AR (PDB=10ms) at 30Mbps [#UEs/cell]

|  | Rate: 30Mbps | Rate: 45Mbps |
| --- | --- | --- |
| FR1 | 6 | 4 |
| FR2 | 6 | 3 |

### UL System Capacity: Cloud Gaming and Virtual Reality (Pose Single-Stream)

In this section, we illustrate the results of the UL system capacity obtained considering the “Pose Single-Stream” traffic model for CG and VR applications in FR1 at 200kbps, with PDB=10ms. Figure 7 shows the ratio of satisfied UEs when cells are evenly loaded. The ratio of satisfied UEs has been computed with 99% of packets containing pose information received within the PDB as a function of the average cell load, which is measured in number of UEs. The dashed black line indicates the 90% limit on the number of satisfied UEs to measure the system capacity. We can observe between 45 and 50 UEs per cell running XR services generating Pose information can be supported in UL in FR1.

Figure 7 – UL capacity evaluation of CG and VR traffic (Pose Single-Stream) for Dense Urban deployment in FR1 with X=99% of frames received within PDB and cells evenly loaded. The lower bound on the percentage of satisfied user is Y=90% (dashed line).

Observation 7: When we consider baseline CG and VR traffic model for UL (Pose Single-Stream) for Dense Urban deployment, we can support a number of UEs per cell indicated in Table 10.

Table 10 – System capacity in UL for CG and VR (Pose Single-Stream) at 200kbps [#UEs/cell]

|  | Rate: 200Kbps |
| --- | --- |
| FR1 | 45-50 |

### UL System Capacity: Augmented Reality (Video Single-Stream)

In this section, we illustrate the results of the UL system capacity obtained considering the “Video Single-Stream” traffic model for AR applications in FR1 at 10Mbps, with PDB=30ms. Figure 8 shows the ratio of satisfied UEs when cells are evenly loaded. The ratio of satisfied UEs has been computed with 99% of packets containing pose information received within the PDB as a function of the average cell load, which is measured in number of UEs. The dashed black line indicates the 90% limit on the number of satisfied UEs to measure the system capacity. We can observe between  4 UEs per cell running XR services generating Pose information can be supported in UL in FR1.

Figure 8 – UL capacity evaluation of AR traffic (Video Single-Stream) for Dense Urban deployment in FR1 with X=99% of frames received within PDB and cells evenly loaded. The lower bound on the percentage of satisfied user is Y=90% (dashed line).

Observation 8: When we consider baseline AR traffic model for UL (Video Single-Stream) for Dense Urban deployment, we can support a number of UEs per cell indicated in Table 11.

Table 11 – System capacity in UL for AR (Video Single-Stream) at 10Mbps [#UEs/cell]

|  | Rate: 10Mbps |
| --- | --- |
| FR1 | 4 |

# Power Saving Evaluation

In this section, we illustrate the results for power saving evaluation obtained with our system level simulator in Indoor Hotspot and Dense Urban deployments. Results are illustrated in Section 4.1 and 4.2, respectively. System configurations used for the two deployment scenarios are described in Appendix. In our evaluation, we independently evaluate the DL and UL capacity. In our evaluation we consider CDRX as power saving scheme and analyzed its performance in term of power saving gain and capacity loss as agreed in [2]. We consider the following five CDRX configurations to evaluate the trade-off between power saving and achievable capacity with respect to the baseline scheme (UE “Always ON”):

- CDRX 1: (DRX long cycle, inactivity timer value, On duration timer value)=(4, 2, 2)

- CDRX 2: (DRX long cycle, inactivity timer value, On duration timer value)=(8, 4, 4)

- CDRX 3: (DRX long cycle, inactivity timer value, On duration timer value)=(16, 8, 8)

- CDRX 4: (DRX long cycle, inactivity timer value, On duration timer value)=(10, 8, 2)

- CDRX 5: (DRX long cycle, inactivity timer value, On duration timer value)=(10, 5, 5)

We observe that the first three CDRX configurations are not fully Rel-15/16 compliant, since 4ms, 8ms, and 16ms are not supported as value for long cycle. However, these configurations can be obtained by configuring the short and long cycle parameters as follows:

- CDRX 1:  (long DRX cycle, inactivity timer value, On duration timer value, short DRX cycle, short DRX cycle timer)= (32,2,2,4,16)

- CDRX 2:  (long DRX cycle, inactivity timer value, On duration timer value, short DRX cycle, short DRX cycle timer)= (32,4,4,8,16)

- CDRX 3:  (long DRX cycle, inactivity timer value, On duration timer value, short DRX cycle, short DRX cycle timer)= (32,8,8,16,16)

The DRX Start Offset is selected randomly across all UEs and across all simulation drops in order to evaluate time misalignment/desynchronization between XR traffic and long DRX cycle.

We compare tested PS schemes using two lexicographic ordering schemes:

- Best system-level PS scheme: the PS scheme with the highest number of satisfied UEs per cell with PS scheme enabled. If two or more PS schemes have the same number of satisfied UEs per cell, then order PS schemes in descending order of mean PS gain and pick the first scheme of the ordered set. Mean PS gain is computed considering all UEs (option 1).

- Best UE-level PS scheme: the PS scheme with the highest mean PS gain provided that at least 1 UE per cell is satisfied. If two or more PS schemes have the same mean PS gain, then order PS schemes in descending order of number of satisfied UEs per cell and pick the first scheme of the ordered set. Mean PS gain is computed considering only satisfied UEs (option 2).

## Indoor Hotspot (InH) scenario

In the following, we illustrate the measured power saving and capacity in Indoor Hotspot deployment when CDRX is enabled. The proposed configurations are compared to the results with DRX disabled shown in Section 3 (labeled “Always ON” in this section) in terms of average power saving gain and capacity loss.

### DL Power Evaluation in FR1

#### Cloud Gaming

In this section, we present the power consumption evaluation obtained considering the CG application at 30Mbps in FR1, with PDB=15ms. Table 12 and Table 13 report the PS gain and system capacity loss of the five CDRX configurations with respect to Always ON policy measured for CG application. PS gain has been evaluated considering both options. Table 12  shows the PS gain considering all UEs (option 1), whereas Table 13 shows the PS gain considering only satisfied UEs (option 2).

Table 12 – Evaluation of UE power saving schemes for {Indoor Hotspot, CG, DL Only, 30Mbps, FR1}. Option 1: PS gain computed with All UEs.

| Power Saving Scheme | Power Saving Gain (PSG) compared to ‘Always ON’ |  |  |  | #satisfied UEs per cell with PS / #satisfied UEs per cell w/o PS | Capacity with PS [#satisfied UEs/cell with PS] | Percentage of satisfied UEs per cell with PS at #satisfied UEs cell w/o PS |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Baseline | Optional |  |  |  |  |  |
|  | Mean PS gain | PS gain of 5%-tile UE in PSG CDF | PS gain of 50%-tile UE in PSG CDF | PS gain of 95%-tile UE in PSG CDF |  |  |  |
| Always ON | - | - | - | - | - / 5 | - | - |
| (4, 2, 2) | 20.9 | 22 | 21.7 | 16.6 | 4.84 / 5 | 5 | 96.8 |
| (8, 4, 4) | 18.2 | 20 | 18.9 | 13.9 | 4.84 / 5 | 5 | 96.7 |
| (16, 8, 8) | 16.2 | 18.3 | 16.7 | 11.6 | 4.75 / 5 | 5 | 95 |
| (10, 8, 2) | 9.3 | 10.4 | 9.3 | 7.5 | 4.93 / 5 | 5 | 98.5 |
| (10, 5, 5) | 17.3 | 20.3 | 17.5 | 13 | 4.82 / 5 | 5 | 96.3 |

Table 13 – Evaluation of UE power saving schemes for {Indoor Hotspot, CG, DL Only, 30Mbps, FR1}. Option 2: PS gain computed with Satisfied UEs.

| Power Saving Scheme | Power Saving Gain (PSG) compared to ‘Always ON’ |  |  |  | #satisfied UEs per cell with PS / #satisfied UEs per cell w/o PS | Capacity with PS [#satisfied UEs/cell with PS] | Percentage of satisfied UEs per cell with PS at #satisfied UEs cell w/o PS |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Baseline | Optional |  |  |  |  |  |
|  | Mean PS gain | PS gain of 5%-tile UE in PSG CDF | PS gain of 50%-tile UE in PSG CDF | PS gain of 95%-tile UE in PSG CDF |  |  |  |
| Always ON | - | - | - | - | - / 5 | - | - |
| (4, 2, 2) | 21.5 | 22 | 21.7 | 21 | 4.84 / 5 | 5 | 96.8 |
| (8, 4, 4) | 18.9 | 20 | 18.9 | 18.7 | 4.84 / 5 | 5 | 96.7 |
| (16, 8, 8) | 17.1 | 18.4 | 16.8 | 18.4 | 4.75 / 5 | 5 | 95 |
| (10, 8, 2) | 9.5 | 10.4 | 9.3 | 11 | 4.93 / 5 | 5 | 98.5 |
| (10, 5, 5) | 17.9 | 20.3 | 17.6 | 16.5 | 4.82 / 5 | 5 | 96.3 |

Observation 9: Power saving gain and DL capacity loss with different CDRX configurations for CG applications in Indoor Hotspot deployment in FR1 are indicated in Table 12 and Table 13 considering all UEs and satisfied UEs, respectively.

Observation 10: CDRX reduces UE mean power consumption between 9% and 22% across all UEs, and although it seems not to affect the DL system capacity much for CG applications for any of the considered DRX configurations, the fractional number of supported UEs is slightly different in each case.

Observation 11: The CDRX configuration (DRX long cycle, inactivity timer value, On duration timer value)=(4, 2, 2) is the best system-level and UE-level PS scheme, among the modelled ones, for CG applications in DL in Indoor Hotspot deployment in FR1.

#### Virtual/Augmented Reality

##### Virtual/Augmented Reality at 30Mbps

In this section, we illustrate the results for the power consumption evaluation obtained considering AR/VR applications at 30Mbps and 45Mbps in FR1, with PDB=10ms. Table 14 and Table 15 report the PS gain and system capacity loss of the five CDRX configurations for the case of AR/VR application at 30Mbps with respect to Always ON policy. PS gain has been evaluated considering both options. Table 14 shows the PS gain considering only satisfied UEs (option 1), whereas Table 15  shows the PS gain considering all UEs (option 2).

Table 14 – Evaluation of UE power saving schemes for {Indoor Hotspot, VR/AR, DL Only, 30Mbps, FR1}. Option 1: PS gain computed with All UEs

| Power Saving Scheme | Power Saving Gain (PSG) compared to ‘Always ON’ |  |  |  | #satisfied UEs per cell with PS / #satisfied UEs per cell w/o PS | Capacity with PS [#satisfied UEs/cell with PS] | Percentage of satisfied UEs per cell with PS at #satisfied UEs cell w/o PS |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Baseline | Optional |  |  |  |  |  |
|  | Mean PS gain | PS gain of 5%-tile UE in PSG CDF | PS gain of 50%-tile UE in PSG CDF | PS gain of 95%-tile UE in PSG CDF |  |  |  |
| Always ON | - | - | - | - | - / 5 | - | - |
| (4, 2, 2) | 20.9 | 22 | 21.7 | 16.6 | 4.5 / 5 | 5 | 90 |
| (8, 4, 4) | 18.2 | 20 | 18.9 | 13.9 | 4.15 / 5 | 4 | 83 |
| (16, 8, 8) | 16.2 | 18.3 | 16.7 | 11.6 | 0 / 5 | 0 | 0 |
| (10, 8, 2) | 9.3 | 10.4 | 9.3 | 7.5 | 4.65 / 5 | 5 | 93 |
| (10, 5, 5) | 17.3 | 20.3 | 17.5 | 13 | 3.7 / 5 | 4 | 74 |

Table 15 – Evaluation of UE power saving schemes for {Indoor Hotspot, VR/AR, DL Only, 30Mbps, FR1}. Option 2: PS gain computed with Satisfied UEs.

| Power Saving Scheme | Power Saving Gain (PSG) compared to ‘Always ON’ |  |  |  | #satisfied UEs per cell with PS / #satisfied UEs per cell w/o PS | Capacity with PS [#satisfied UEs/cell with PS] | Percentage of satisfied UEs per cell with PS at #satisfied UEs cell w/o PS |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Baseline | Optional |  |  |  |  |  |
|  | Mean PS gain | PS gain of 5%-tile UE in PSG CDF | PS gain of 50%-tile UE in PSG CDF | PS gain of 95%-tile UE in PSG CDF |  |  |  |
| Always ON | - | - | - | - | - / 5 | - | - |
| (4, 2, 2) | 22.2 | 22 | 21.7 | 25.6 | 4.5 / 5 | 5 | 90 |
| (8, 4, 4) | 19.8 | 20 | 19 | 24 | 4.15 / 5 | 4 | 83 |
| (16, 8, 8) | 0 | 0 | 0 | 0 | 0 / 5 | 0 | 0 |
| (10, 8, 2) | 10 | 10 | 9.4 | 13.5 | 4.65 / 5 | 5 | 93 |
| (10, 5, 5) | 19.2 | 20.6 | 18.4 | 22.5 | 3.7 / 5 | 4 | 74 |

##### Virtual/Augmented Reality at 45Mbps

Table 16 and Table 17 report the PS gain and system capacity loss of the five CDRX configurations for the case of AR/VR application at 45Mbps with respect to Always ON policy. PS gain has been evaluated considering both options. Table 16 shows the PS gain considering only satisfied UEs (option 1), whereas Table 17 shows the PS gain considering all UEs (option 2).

Table 16 – Evaluation of UE power saving schemes for {Indoor Hotspot, VR/AR, DL Only, 45Mbps, FR1}. Option 1: PS gain computed with All UEs.

| Power Saving Scheme | Power Saving Gain (PSG) compared to ‘Always ON’ |  |  |  | #satisfied UEs per cell with PS / #satisfied UEs per cell w/o PS | Capacity with PS [#satisfied UEs/cell with PS] | Percentage of satisfied UEs per cell with PS at #satisfied UEs cell w/o PS |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Baseline | Optional |  |  |  |  |  |
|  | Mean PS gain | PS gain of 5%-tile UE in PSG CDF | PS gain of 50%-tile UE in PSG CDF | PS gain of 95%-tile UE in PSG CDF |  |  |  |
| Always ON | - | - | - | - | - / 3 | - | - |
| (4, 2, 2) | 15.7 | 16.8 | 16.4 | 11.5 | 2.85 / 3 | 3 | 95 |
| (8, 4, 4) | 12.1 | 14 | 12.8 | 6.9 | 2.54 / 3 | 2 | 84.7 |
| (16, 8, 8) | 9.4 | 12 | 10.2 | 3.3 | 0 / 3 | 0 | 0 |
| (10, 8, 2) | 4 | 5.3 | 4.5 | 0.4 | 2.91 / 3 | 3 | 97 |
| (10, 5, 5) | 10.8 | 14.2 | 11.5 | 4.8 | 1.89 / 3 | 2 | 63 |

Table 17 - Evaluation of UE power saving schemes for {Indoor Hotspot, VR/AR, DL Only, 45Mbps, FR1}. Option 2: PS gain computed with Satisfied UEs.

| Power Saving Scheme | Power Saving Gain (PSG) compared to ‘Always ON’ |  |  |  | #satisfied UEs per cell with PS / #satisfied UEs per cell w/o PS | Capacity with PS [#satisfied UEs/cell with PS] | Percentage of satisfied UEs per cell with PS at #satisfied UEs cell w/o PS |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Baseline | Optional |  |  |  |  |  |
|  | Mean PS gain | PS gain of 5%-tile UE in PSG CDF | PS gain of 50%-tile UE in PSG CDF | PS gain of 95%-tile UE in PSG CDF |  |  |  |
| Always ON | - | - | - | - | - / 3 | - | - |
| (4, 2, 2) | 16.2 | 16.8 | 16.4 | 14.6 | 2.85 / 3 | 3 | 95 |
| (8, 4, 4) | 13 | 14 | 13 | 11.6 | 2.54 / 3 | 2 | 84.7 |
| (16, 8, 8) | 0 | 0 | 0 | 0 | 0 / 3 | 0 | 0 |
| (10, 8, 2) | 4.3 | 5.3 | 4.5 | 2.7 | 2.91 / 3 | 3 | 97 |
| (10, 5, 5) | 12.4 | 14.5 | 12.3 | 10.7 | 1.89 / 3 | 2 | 63 |

Observation 12: Power saving gain and DL capacity loss with different CDRX configurations for VR/AR applications at 30Mbps in Indoor Hotspot deployment in FR1 are indicated in Table 14 and Table 15 considering all UEs and satisfied UEs, respectively.

Observation 13: Power saving gain and DL capacity loss with different CDRX configurations for VR/AR applications at 45Mbps in Indoor Hotspot deployment in FR1 are indicated in Table 16 and Table 17 considering all UEs and satisfied UEs, respectively.

Observation 14: CDRX reduces UE mean power consumption  for VR/AR applications between 0% and 22% for 30Mbps and between 0% and 16.4% for 45Mbps but it also strongly affects the DL system capacity, reducing it to 0 in the worst case due to the low delay budget with respect to the simulated retransmission delay.

Observation 15: The CDRX configuration (DRX long cycle, inactivity timer value, On duration timer value)=(4, 2, 2) is the best system-level and UE-level PS scheme, among the modelled ones, in DL in Indoor Hotspot deployment in FR1 for AR/VR applications both at 30Mbps and 45Mbps.

Observation 16: CDRX configuration (DRX long cycle, inactivity timer value, On duration timer value)=(16, 8, 8) archives zero capacity due to the misalignment between the start of the DRX cycle and the AR/VR traffic for both 30Mbps and 45Mbps. Therefore, an improper configuration of the DRX start offset of the CDRX configuration negatively affects system capacity for AR/VR services. A shorter duration of the long DRX cycle helps to reduce the effect of the improper configuration of the DRX start offset.

### UL Power Evaluation in FR1

#### Virtual/Augmented Reality

In this section, we present the power consumption evaluation for UL considering the “Video Single Stream” traffic model for AR applications at 10Mbps in FR1, with PDB=30ms. Table 18 reports the PS gain and system capacity loss considering all UEs (option 1) for five CDRX configurations with respect to Always ON policy.

Table 18 – Evaluation of UE power saving schemes for {Indoor Hotspot, AR “Video Single Stream”, UL Only, 10Mbps, FR1}. Option 1: PS gain computed with All UEs.

| Power Saving Scheme | Power Saving Gain (PSG) compared to ‘Always ON’ |  |  |  | #satisfied UEs per cell with PS / #satisfied UEs per cell w/o PS | Capacity with PS [#satisfied UEs/cell with PS] | Percentage of satisfied UEs per cell with PS at #satisfied UEs cell w/o PS |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Baseline | Optional |  |  |  |  |  |
|  | Mean PS gain | PS gain of 5%-tile UE in PSG CDF | PS gain of 50%-tile UE in PSG CDF | PS gain of 95%-tile UE in PSG CDF |  |  |  |
| Always ON | - | - | - | - | - / 4 | - | - |
| (4, 2, 2) | 21.64 | 22.06 | 21.85 | 21.38 | 3.9 / 4 | 4 | 99% |
| (8, 4, 4) | 18.27 | 18.66 | 18.44 | 17.8 | 3.9 / 4 | 4 | 99% |
| (16, 8, 8) | 13.5 | 14.42 | 13.59 | 13 | 3.9 / 4 | 4 | 99% |
| (10, 8, 2) | 8.67 | 8.83 | 8.77 | 8.31 | 3.9 / 4 | 4 | 99% |
| (10, 5, 5) | 16.67 | 17.08 | 16.82 | 16.08 | 3.9 / 4 | 4 | 99% |

Observation 17: Power saving gain and UL capacity loss with different CDRX configurations for AR “Video Single-Stream” applications in Indoor Hotspot deployment in FR1 are indicated in Table 18  considering all UEs.

Observation 18: CDRX reduces UE mean power consumption between 8.67% and 21.64% across all UEs, and although it seems not to affect the UL system capacity for AR “Video Single-Stream” applications for any of the considered DRX configurations, the fractional number of supported UEs is slightly different in each case.

Observation 19: The CDRX configuration (DRX long cycle, inactivity timer value, On duration timer value)=(4, 2, 2) is the best system-level and UE-level  PS scheme, among the modelled ones,  for AR “Video Single-Stream” applications in UL in Indoor Hotspot deployment in FR1, since it achieves the same capacity as Always ON and the highest PS gain.

### DL Power Evaluation in FR2

#### Cloud Gaming

In this section, we present the power consumption evaluation obtained considering the CG application at 30Mbps in FR2, with PDB=15ms. Table 19 and Table 20 report the PS gain and system capacity loss of the five CDRX configurations with respect to Always ON policy measured for CG application. PS gain has been evaluated considering both options. Table 19 shows the PS gain considering all UEs (option 1), whereas Table 20 shows the PS gain considering only satisfied UEs (option 2).

Table 19 – Evaluation of UE power saving schemes for {Indoor Hotspot, CG, DL Only, 30Mbps, FR2}. Option 1: PS gain computed with All UEs.

| Power Saving Scheme | Power Saving Gain (PSG) compared to ‘Always ON’ |  |  |  | #satisfied UEs per cell with PS / #satisfied UEs per cell w/o PS | Capacity with PS [#satisfied UEs/cell with PS] | Percentage of satisfied UEs per cell with PS at #satisfied UEs cell w/o PS |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Baseline | Optional |  |  |  |  |  |
|  | Mean PS gain | PS gain of 5%-tile UE in PSG CDF | PS gain of 50%-tile UE in PSG CDF | PS gain of 95%-tile UE in PSG CDF |  |  |  |
| Always ON | - | - | - | - | - / 11 | - | - |
| (4, 2, 2) | 18.5 | 19.2 | 18.8 | 16.9 | 10.81 / 11 | 11 | 98.33 |
| (8, 4, 4) | 15.4 | 16.3 | 15.6 | 14 | 10.77 / 11 | 11 | 98 |
| (16, 8, 8) | 11.6 | 13.7 | 11.8 | 9.5 | 8.59 / 11 | 10 | 78.1 |
| (10, 8, 2) | 7.6 | 8.1 | 7.9 | 7.2 | 10.8 / 11 | 11 | 98.2 |
| (10, 5, 5) | 13.7 | 15.1 | 13.9 | 12.2 | 10.66 / 11 | 11 | 96 |

Table 20 – Evaluation of UE power saving schemes for {Indoor Hotspot, CG, DL Only, 30Mbps, FR2}. Option 2: PS gain computed with Satisfied UEs.

| Power Saving Scheme | Power Saving Gain (PSG) compared to ‘Always ON’ |  |  |  | #satisfied UEs per cell with PS / #satisfied UEs per cell w/o PS | Capacity with PS [#satisfied UEs/cell with PS] | Percentage of satisfied UEs per cell with PS at #satisfied UEs cell w/o PS |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Baseline | Optional |  |  |  |  |  |
|  | Mean PS gain | PS gain of 5%-tile UE in PSG CDF | PS gain of 50%-tile UE in PSG CDF | PS gain of 95%-tile UE in PSG CDF |  |  |  |
| Always ON | - | - | - | - | - / 11 | - | - |
| (4, 2, 2) | 18.8 | 19.2 | 18.8 | 17.5 | 10.81 / 11 | 11 | 98.33 |
| (8, 4, 4) | 15.6 | 16.3 | 15.6 | 14.6 | 10.77 / 11 | 11 | 98 |
| (16, 8, 8) | 12.2 | 13.8 | 12 | 10.9 | 8.59 / 11 | 10 | 78.1 |
| (10, 8, 2) | 7.8 | 8 | 7.9 | 7.2 | 10.8 / 11 | 11 | 98.2 |
| (10, 5, 5) | 14 | 15.1 | 13.9 | 12.7 | 10.66 / 11 | 11 | 96 |

Observation 20: Power saving gain and DL capacity loss with different CDRX configurations for CG applications in Indoor Hotspot deployment in FR2 are indicated in Table 19 and Table 20 considering all UEs and satisfied UEs, respectively.

Observation 21: CDRX reduces UE mean power consumption between 7.6% and 18.8% across all UEs, but, at the same time, it also affects the system capacity. A non-optimal configuration of CDX parameters can cause a significant drop in the number of satisfied users per cell.

Observation 22: The CDRX configuration (DRX long cycle, inactivity timer value, On duration timer value)=(4, 2, 2) is the best system-level and UE-level PS, among the modelled ones, scheme for CG applications in DL in Indoor Hotspot deployment in FR2, since it achieves the same capacity as Always ON and the highest PS gain.

#### Virtual/Augmented Reality

##### Virtual/Augmented Reality at 30Mbps

In this section, we present the power consumption evaluation obtained considering the VR/AR application at 30Mbps in FR2, with PDB=10ms.  Table 21and Table 22 report the PS gain and system capacity loss of the five CDRX configurations with respect to Always ON policy measured for VR/AR application. PS gain has been evaluated considering both options. Table 21 shows the PS gain considering all UEs (option 1), whereas Table 22 shows the PS gain considering only satisfied UEs (option 2).

Table 21 – Evaluation of UE power saving schemes for {Indoor Hotspot, VR/AR, DL Only, 30Mbps, FR2}. Option 1: PS gain computed with All UEs.

| Power Saving Scheme | Power Saving Gain (PSG) compared to ‘Always ON’ |  |  |  | #satisfied UEs per cell with PS / #satisfied UEs per cell w/o PS | Capacity with PS [#satisfied UEs/cell with PS] | Percentage of satisfied UEs per cell with PS at #satisfied UEs cell w/o PS |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Baseline | Optional |  |  |  |  |  |
|  | Mean PS gain | PS gain of 5%-tile UE in PSG CDF | PS gain of 50%-tile UE in PSG CDF | PS gain of 95%-tile UE in PSG CDF |  |  |  |
| Always ON | - | - | - | - | - / 10 | - | - |
| (4, 2, 2) | 19.58 | 20 | 19.66 | 19.12 | 8.5 / 10 | 9 | 85.58 |
| (8, 4, 4) | 16.41 | 17.27 | 16.45 | 15.78 | 2 / 10 | 8 | 20.66 |
| (16, 8, 8) | 13.16 | 15 | 13.22 | 11.58 | 0 | 0 | 0 |
| (10, 8, 2) | 8.21 | 8.43 | 8.26 | 8.03 | 9.24 / 10 | 10 | 92.41 |
| (10, 5, 5) | 14.92 | 16.29 | 14.96 | 13.76 | 0.7 / 10 | 7 | 7.16 |

Table 22 – Evaluation of UE power saving schemes for {Indoor Hotspot, VR/AR, DL Only, 30Mbps, FR2}. Option 2: PS gain computed with Satisfied UEs.

| Power Saving Scheme | Power Saving Gain (PSG) compared to ‘Always ON’ |  |  |  | #satisfied UEs per cell with PS / #satisfied UEs per cell w/o PS | Capacity with PS [#satisfied UEs/cell with PS] | Percentage of satisfied UEs per cell with PS at #satisfied UEs cell w/o PS |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Baseline | Optional |  |  |  |  |  |
|  | Mean PS gain | PS gain of 5%-tile UE in PSG CDF | PS gain of 50%-tile UE in PSG CDF | PS gain of 95%-tile UE in PSG CDF |  |  |  |
| Always ON | - | - | - | - | - / 10 | - | - |
| (4, 2, 2) | 19.68 | 20 | 19.68 | 19.25 | 8.5 / 10 | 9 | 85.58 |
| (8, 4, 4) | 16.92 | 17.42 | 16.9 | 16.41 | 2 / 10 | 8 | 20.66 |
| (16, 8, 8) | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| (10, 8, 2) | 8.26 | 8.43 | 8.27 | 7.98 | 9.24 / 10 | 10 | 92.41 |
| (10, 5, 5) | 16.22 | 16.66 | 16.23 | 15.79 | 0.7 / 10 | 7 | 7.16 |

##### Virtual/Augmented Reality at 45Mbps

In this section, we present the power consumption evaluation obtained considering the VR/AR application at 45Mbps in FR2, with PDB=10ms. Table 23 and Table 24 report the PS gain and system capacity loss of the five CDRX configurations with respect to Always ON policy measured for VR/AR application. PS gain has been evaluated considering both options. Table 23 shows the PS gain considering all UEs (option 1), whereas Table 24 shows the PS gain considering only satisfied UEs (option 2).

Table 23 – Evaluation of UE power saving schemes for {Indoor Hotspot, VR/AR, DL Only, 45Mbps, FR2}. Option 1: PS gain computed with All UEs.

| Power Saving Scheme | Power Saving Gain (PSG) compared to ‘Always ON’ |  |  |  | #satisfied UEs per cell with PS / #satisfied UEs per cell w/o PS | Capacity with PS [#satisfied UEs/cell with PS] | Percentage of satisfied UEs per cell with PS at #satisfied UEs cell w/o PS |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Baseline | Optional |  |  |  |  |  |
|  | Mean PS gain | PS gain of 5%-tile UE in PSG CDF | PS gain of 50%-tile UE in PSG CDF | PS gain of 95%-tile UE in PSG CDF |  |  |  |
| Always ON | - | - | - | - | - / 6 | - | - |
| (4, 2, 2) | 18 | 18.3 | 18.1 | 17.6 | 4.5 / 6 | 5 | 75.56 |
| (8, 4, 4) | 15 | 15.7 | 15 | 14.5 | 0.56 / 6 | 4 | 9.4 |
| (16, 8, 8) | 11.6 | 13.1 | 11.7 | 10.1 | 0 / 6 | 0 | 0 |
| (10, 8, 2) | 7.5 | 7.7 | 7.6 | 7.3 | 5.4 / 6 | 6 | 90 |
| (10, 5, 5) | 13.5 | 14.6 | 13.5 | 12.6 | 0.19 / 6 | 1 | 3.33 |

Table 24 – Evaluation of UE power saving schemes for {Indoor Hotspot, VR/AR, DL Only, 45Mbps, FR2}. Option 2: PS gain computed with Satisfied UEs.

| Power Saving Scheme | Power Saving Gain (PSG) compared to ‘Always ON’ |  |  |  | #satisfied UEs per cell with PS / #satisfied UEs per cell w/o PS | Capacity with PS [#satisfied UEs/cell with PS] | Percentage of satisfied UEs per cell with PS at #satisfied UEs cell w/o PS |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Baseline | Optional |  |  |  |  |  |
|  | Mean PS gain | PS gain of 5%-tile UE in PSG CDF | PS gain of 50%-tile UE in PSG CDF | PS gain of 95%-tile UE in PSG CDF |  |  |  |
| Always ON | - | - | - | - | - / 6 | - | - |
| (4, 2, 2) | 18.1 | 18.3 | 18.1 | 17.9 | 4.5 / 6 | 5 | 75.56 |
| (8, 4, 4) | 15.5 | 15.8 | 15.3 | 15.3 | 0.56 / 6 | 4 | 94.44 |
| (16, 8, 8) | 0 | 0 | 0 | 0 | 0 / 6 | 0 | 0 |
| (10, 8, 2) | 7.6 | 7.7 | 7.6 | 7.4 | 5.4 / 6 | 6 | 90 |
| (10, 5, 5) | 13.4 | 14.6 | 13.5 | 12.6 | 0.19 / 6 | 1 | 3.33 |

Observation 23: Power saving gain and DL capacity loss with different CDRX configurations for VR/AR applications at 30Mbps in Indoor Hotspot deployment in FR2 are indicated in Table 21 and Table 22 considering all UEs and satisfied UEs, respectively.

Observation 24: Power saving gain and DL capacity loss with different CDRX configurations for VR/AR applications at 45Mbps in Indoor Hotspot deployment in FR2 are indicated in Table 23 and Table 24 considering all UEs and satisfied UEs, respectively.

Observation 25: CDRX reduces UE mean power consumption for VR/AR applications between 0% and 19.58% for 30Mbps and between 0% and 18.1% for 45Mbps, but it also strongly affects the DL system capacity, reducing it to 0 in the worst case due to the low delay budget with respect to the simulated retransmission delay.

Observation 26: The CDRX configuration (DRX long cycle, inactivity timer value, On duration timer value)=(10, 8, 2) is the best system-level PS, among the modelled ones, scheme for AR/VR applications at 30Mbps and 45Mbps in DL in Indoor Hotspot deployment in FR2, since it achieves the same capacity as Always ON.

Observation 27: The CDRX configuration (DRX long cycle, inactivity timer value, On duration timer value)=(4, 2, 2) is the best UE-level PS scheme, among the modelled ones, for AR/VR applications at 30Mbps and 45Mbps in DL in Indoor Hotspot deployment in FR2, since it achieves the highest mean PS gain with respect to Always ON.

## Dense Urban (DU) scenario

In the following, we illustrate the measured power saving and capacity for DL in Dense Urban deployment when CDRX is enabled. The proposed configurations are compared to the results with DRX disabled shown in Section 3 (labeled “Always ON” in this section) in terms of average power saving gain and capacity loss.

### DL Power Evaluation in FR1

#### Cloud Gaming

In this section, we present the power consumption evaluation obtained considering the CG application at 30Mbps in FR1, with PDB=15ms.  Table 25 and Table 26 report the PS gain and system capacity loss of the five CDRX configurations with respect to Always ON policy measured for CG application. PS gain has been evaluated considering both options. Table 25 shows the PS gain considering all UEs (option 1), whereas Table 26 shows the PS gain considering only satisfied UEs (option 2).

Table 25 – Evaluation of UE power saving schemes for {Dense Urban, CG, DL Only, 30Mbps, FR1}. Option 1: PS gain computed with All UEs.

| Power Saving Scheme | Power Saving Gain (PSG) compared to ‘Always ON’ |  |  |  | #satisfied UEs per cell with PS / #satisfied UEs per cell w/o PS | Capacity with PS [#satisfied UEs/cell with PS] | Percentage of satisfied UEs per cell with PS at #satisfied UEs cell w/o PS |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Baseline | Optional |  |  |  |  |  |
|  | Mean PS gain | PS gain of 5%-tile UE in PSG CDF | PS gain of 50%-tile UE in PSG CDF | PS gain of 95%-tile UE in PSG CDF |  |  |  |
| Always ON | - | - | - | - | - / 8 | - | - |
| (4, 2, 2) | 20 | 21.6 | 21 | 15.5 | 7.04 / 8 | 7 | 88 |
| (8, 4, 4) | 16.7 | 18.9 | 17.6 | 11.9 | 6.72 / 8 | 7 | 84 |
| (16, 8, 8) | 13.6 | 16.8 | 14.4 | 8.3 | 5.6 / 8 | 6 | 70 |
| (10, 8, 2) | 8.8 | 9.9 | 9 | 7 | 7.44 / 8 | 8 | 93 |
| (10, 5, 5) | 15.4 | 19.4 | 15.7 | 10 | 6.08 / 8 | 7 | 76 |

Table 26 – Evaluation of UE power saving schemes for {Dense Urban, CG, DL Only, 30Mbps, FR1}. Option 2: PS gain computed with Satisfied UEs.

| Power Saving Scheme | Power Saving Gain (PSG) compared to ‘Always ON’ |  |  |  | #satisfied UEs per cell with PS / #satisfied UEs per cell w/o PS | Capacity with PS [#satisfied UEs/cell with PS] | Percentage of satisfied UEs per cell with PS at #satisfied UEs cell w/o PS |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Baseline | Optional |  |  |  |  |  |
|  | Mean PS gain | PS gain of 5%-tile UE in PSG CDF | PS gain of 50%-tile UE in PSG CDF | PS gain of 95%-tile UE in PSG CDF |  |  |  |
| Always ON | - | - | - | - | - / 8 | - | - |
| (4, 2, 2) | 21.6 | 21.6 | 21.2 | 22.4 | 7.04 / 8 | 7 | 88 |
| (8, 4, 4) | 18.6 | 19 | 17.9 | 19.5 | 6.72 / 8 | 7 | 84 |
| (16, 8, 8) | 16.6 | 17 | 15.3 | 20 | 5.6 / 8 | 6 | 67 |
| (10, 8, 2) | 9.7 | 9.9 | 9.1 | 10.5 | 7.44 / 8 | 8 | 93 |
| (10, 5, 5) | 17.8 | 19.6 | 16.8 | 19.5 | 6.08 / 8 | 7 | 76 |

Observation 28: Power saving gain and DL capacity loss with different CDRX configurations for CG applications in Indoor Hotspot deployment in FR1 are indicated in Table 25 and Table 26 considering all UEs and satisfied UEs, respectively.

Observation 29: CDRX reduces UE mean power consumption between 9% and 22% across all UEs, but, at the same time, it also affects the system capacity. A non-optimal configuration of CDX parameters can cause a significant drop in the number of satisfied users per cell.

Observation 30: The CDRX configuration (DRX long cycle, inactivity timer value, On duration timer value)=(10, 8, 2) is the best system-level PS scheme, among the modelled ones, for CG applications in DL in Dense Urban deployment in FR1, since it achieves the same capacity as Always ON.

Observation 31: The CDRX configuration (DRX long cycle, inactivity timer value, On duration timer value)=(4, 2, 2) is the best UE-level PS scheme, among the modelled ones, for CG applications in DL in Dense Urban deployment in FR1, since it achieves the highest mean PS gain with respect to Always ON.

#### Virtual/Augmented Reality

##### Virtual/Augmented Reality at 30Mbps

In this section, we illustrate the results for the power consumption evaluation obtained considering VR/AR applications at 30Mbps in FR1, with PDB=10ms. Table 27 and Table 28 report the PS gain and system capacity loss of the five CDRX configurations with respect to Always ON policy measured for CG application. PS gain has been evaluated considering both options. Table 27 shows the PS gain considering all UEs (option 1), whereas Table 28 shows the PS gain considering only satisfied UEs (option 2).

Table 27 – Evaluation of UE power saving schemes for {Dense Urban, AR/VR, DL Only, 30Mbps, FR1}. Option 1: PS gain computed with All UEs.

| Power Saving Scheme | Power Saving Gain (PSG) compared to ‘Always ON’ |  |  |  | #satisfied UEs per cell with PS / #satisfied UEs per cell w/o PS | Capacity with PS [#satisfied UEs/cell with PS] | Percentage of satisfied UEs per cell with PS at #satisfied UEs cell w/o PS |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Baseline | Optional |  |  |  |  |  |
|  | Mean PS gain | PS gain of 5%-tile UE in PSG CDF | PS gain of 50%-tile UE in PSG CDF | PS gain of 95%-tile UE in PSG CDF |  |  |  |
| Always ON | - | - | - | - | - / 6 | - | - |
| (4, 2, 2) | 21 | 21.9 | 21.5 | 17.8 | 4.98 / 6 | 5 | 83 |
| (8, 4, 4) | 18 | 19.7 | 18.5 | 14.3 | 3.66 / 6 | 5 | 61 |
| (16, 8, 8) | 15.8 | 18 | 16.2 | 11.4 | 0 / 6 | 0 | 0 |
| (10, 8, 2) | 9.2 | 10 | 9.3 | 8 | 5.58 / 6 | 6 | 93 |
| (10, 5, 5) | 17 | 20.4 | 17.1 | 12.8 | 3.12 / 6 | 4 | 52 |

Table 28 – Evaluation of UE power saving schemes for {Dense Urban, AR/VR, DL Only, 30Mbps, FR1}. Option 2: PS gain computed with Satisfied UEs.

| Power Saving Scheme | Power Saving Gain (PSG) compared to ‘Always ON’ |  |  |  | #satisfied UEs per cell with PS / #satisfied UEs per cell w/o PS | Capacity with PS [#satisfied UEs/cell with PS] | Percentage of satisfied UEs per cell with PS at #satisfied UEs cell w/o PS |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Baseline | Optional |  |  |  |  |  |
|  | Mean PS gain | PS gain of 5%-tile UE in PSG CDF | PS gain of 50%-tile UE in PSG CDF | PS gain of 95%-tile UE in PSG CDF |  |  |  |
| Always ON | - | - | - | - | - / 6 | - | - |
| (4, 2, 2) | 22.1 | 21.9 | 21.7 | 24.5 | 4.98 / 6 | 5 | 83 |
| (8, 4, 4) | 19.6 | 19.9 | 18.9 | 22.8 | 3.66 / 6 | 5 | 61 |
| (16, 8, 8) | 0 | 0 | 0 | 0 | 0 / 6 | 0 | 0 |
| (10, 8, 2) | 9.9 | 9.9 | 9.4 | 11.8 | 5.58 / 6 | 6 | 93 |
| (10, 5, 5) | 19.2 | 20.7 | 18.5 | 21.2 | 3.12 / 6 | 4 | 52 |

##### Virtual/Augmented Reality at 45Mbps

In this section, we illustrate the results for the power consumption evaluation obtained considering VR/AR applications at 45Mbps in FR1, with PDB=10ms. Table 29 and Table 30 report the PS gain and system capacity loss of the five CDRX configurations with respect to Always ON policy measured for CG application. PS gain has been evaluated considering both options. Table 29 shows the PS gain considering all UEs (option 1), whereas Table 30 shows the PS gain considering only satisfied UEs (option 2).

Table 29 – Evaluation of UE power saving schemes for {Dense Urban, AR/VR, DL Only, 45Mbps, FR1}. Option 1: PS gain computed with All UEs.

| Power Saving Scheme | Power Saving Gain (PSG) compared to ‘Always ON’ |  |  |  | #satisfied UEs per cell with PS / #satisfied UEs per cell w/o PS | Capacity with PS [#satisfied UEs/cell with PS] | Percentage of satisfied UEs per cell with PS at #satisfied UEs cell w/o PS |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Baseline | Optional |  |  |  |  |  |
|  | Mean PS gain | PS gain of 5%-tile UE in PSG CDF | PS gain of 50%-tile UE in PSG CDF | PS gain of 95%-tile UE in PSG CDF |  |  |  |
| Always ON | - | - | - | - | - / 4 | - | - |
| (4, 2, 2) | 14.5 | 16.4 | 15.9 | 5.8 | 2.76 / 4 | 3 | 69 |
| (8, 4, 4) | 10.8 | 13.3 | 12 | 1.3 | 1.6 / 4 | 3 | 40 |
| (16, 8, 8) | 7.9 | 11.1 | 9.2 | 0 | 0 / 4 | 0 | 0 |
| (10, 8, 2) | 3.2 | 5 | 4.2 | 0 | 3.52 / 4 | 3 | 88 |
| (10, 5, 5) | 9.5 | 13.2 | 10.5 | 0.3 | 0.96 / 4 | 2 | 24 |

Table 30 – Evaluation of UE power saving schemes for {Dense Urban, AR/VR, DL Only, 45Mbps, FR1}. Option 1: PS gain computed with Satisfied UEs

| Power Saving Scheme | Power Saving Gain (PSG) compared to ‘Always ON’ |  |  |  | #satisfied UEs per cell with PS / #satisfied UEs per cell w/o PS | Capacity with PS [#satisfied UEs/cell with PS] | Percentage of satisfied UEs per cell with PS at #satisfied UEs cell w/o PS |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Baseline | Optional |  |  |  |  |  |
|  | Mean PS gain | PS gain of 5%-tile UE in PSG CDF | PS gain of 50%-tile UE in PSG CDF | PS gain of 95%-tile UE in PSG CDF |  |  |  |
| Always ON | - | - | - | - | - / 4 | - | - |
| (4, 2, 2) | 16.1 | 16.4 | 16 | 16.1 | 2.76 / 4 | 3 | 69 |
| (8, 4, 4) | 12.8 | 13.6 | 12.6 | 12.8 | 1.6 / 4 | 3 | 40 |
| (16, 8, 8) | 0 | 0 | 0 | 0 | 0 / 4 | 0 | 0 |
| (10, 8, 2) | 4.3 | 5 | 4.3 | 3.4 | 3.52 / 4 | 3 | 88 |
| (10, 5, 5) | 12.3 | 13.6 | 12.1 | 11.9 | 0.96 / 4 | 2 | 24 |

Observation 32: Power saving gain and DL capacity loss with different CDRX configurations for VR/AR applications at 30Mbps in Dense Urban deployment in FR1 are indicated in Table 27 and Table 28 considering all UEs and satisfied UEs, respectively.

Observation 33: Power saving gain and DL capacity loss with different CDRX configurations for VR/AR applications at 45Mbps in Dense Urban deployment in FR1 are indicated in Table 29 and Table 30 considering all UEs and satisfied UEs, respectively.

Observation 34: CDRX reduces UE mean power consumption for VR/AR applications between 0% and 22% for 30Mbps and between 0% and 16.1% for 45Mbps, but it also strongly affects the DL system capacity, reducing it to 0 in the worst case due to the low delay budget with respect to the simulated retransmission delay.

Observation 35: The CDRX configuration (DRX long cycle, inactivity timer value, On duration timer value)=(10, 8, 2) is the best system-level PS scheme, among the modelled ones, for AR/VR applications at 30Mbps in DL in Dense Urban deployment in FR1, since it achieves the same capacity as Always ON.

Observation 36: The CDRX configuration (DRX long cycle, inactivity timer value, On duration timer value)=(4, 2, 2) is the best UE-level PS scheme, among the modelled ones, for AR/VR applications at 30Mbps in DL in Dense Urban deployment in FR1, since it achieves the highest mean PS gain with respect to Always ON.

Observation 37: The CDRX configuration (DRX long cycle, inactivity timer value, On duration timer value)=(4, 2, 2) is the best system-level and UE-level PS scheme, among the modelled ones, for AR/VR applications at 45Mbps in DL in Dense Urban deployment in FR1, since it achieves the same capacity and the highest mean PS gain with respect to Always ON.

Observation 38: CDRX configuration (DRX long cycle, inactivity timer value, On duration timer value)=(16, 8, 8) archives zero capacity due to the misalignment between the start of the DRX cycle and the AR/VR traffic for both 30Mbps and 45Mbps. Therefore, an improper configuration of the DRX start offset of the CDRX configuration negatively affects system capacity for AR/VR services. A shorter duration of the long DRX cycle helps to reduce the effect of the improper configuration of the DRX start offset.

### UL Power Evaluation in FR1

#### Augmented Reality (Video Single-Stream)

In this section, we present the power consumption evaluation for UL considering the “Video Single Stream” traffic model for AR applications at 10Mbps in FR1, with PDB=30ms. Table 31 reports the PS gain and system capacity loss considering all UEs (option 1) for five CDRX configurations with respect to Always ON policy.

Table 31 – Evaluation of UE power saving schemes for {Dense Urban, AR “Video Single Stream”, UL Only, 10Mbps, FR1}. Option 1: PS gain computed with All UEs.

| Power Saving Scheme | Power Saving Gain (PSG) compared to ‘Always ON’ |  |  |  | #satisfied UEs per cell with PS / #satisfied UEs per cell w/o PS | Capacity with PS [#satisfied UEs/cell with PS] | Percentage of satisfied UEs per cell with PS at #satisfied UEs cell w/o PS |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Baseline | Optional |  |  |  |  |  |
|  | Mean PS gain | PS gain of 5%-tile UE in PSG CDF | PS gain of 50%-tile UE in PSG CDF | PS gain of 95%-tile UE in PSG CDF |  |  |  |
| Always ON | - | - | - | - | - / 4 | - | - |
| (4, 2, 2) | 14.6 | 22 | 18.7 | 3.7 | 3.65 / 4 | 4 | 91.2 |
| (8, 4, 4) | 10.8 | 18.4 | 14.7 | 0.1 | 3.66 / 4 | 4 | 91.4 |
| (16, 8, 8) | 7.5 | 13.8 | 10.7 | 0 | 3.66 / 4 | 4 | 91.4 |
| (10, 8, 2) | 5.8 | 8.6 | 7.5 | 1.5 | 3.65 / 4 | 4 | 91.3 |
| (10, 5, 5) | 9.7 | 16.8 | 13.5 | 0 | 3.66 / 4 | 4 | 91.4 |

Observation 39: Power saving gain and UL capacity loss with different CDRX configurations for AR “Video Single-Stream” applications in Dense Urban deployment in FR1 are indicated in Table 31 considering all UEs.

Observation 40: CDRX reduces UE mean power consumption between 5.8% and 14.6% across all UEs, and although it seems not to affect the UL system capacity for AR “Video Single-Stream” applications for any of the considered DRX configurations, the fractional number of supported UEs is slightly different in each case.

Observation 41: The CDRX configuration (DRX long cycle, inactivity timer value, On duration timer value)=(8, 4, 4) is the best system-level PS scheme, among the modelled ones, for AR “Video Single-Stream” applications in UL in Dense Urban deployment in FR1, since it achieves the same capacity as Always ON.

Observation 42: The CDRX configuration (DRX long cycle, inactivity timer value, On duration timer value)=(4, 2, 2) is the best UE-level PS scheme, among the modelled ones, for AR “Video Single-Stream” applications in UL in Dense Urban deployment in FR1, since it achieves the highest mean PS gain with respect to Always ON.

# Coverage Evaluation

In this section, we illustrate the results for coverage evaluation obtained with our system level simulator in Indoor Hotspot and Dense Urban deployments. Results are illustrated in Section 5.1 and 5.2, respectively. System configurations used for the two deployment scenarios are described in Appendix. In our evaluation, we evaluate coverage measuring the coupling gain with B UEs/cell (B=1 and B=”system capacity”).

## Indoor Hotspot (InH) scenario

Figure 9 shows the empirical CDF of the coupling gain measured in the InH deployment scenario in FR1, while Table 32 shows the 5%-tile of the distribution.

Figure 9 – Empirical CDF of Coupling Gain [dB] in InH deployment in FR1.

Table 32 – 5%-tile of coupling gain [dB] in InH deployment measured with DL traffic model.

|  | 5%-tile of coupling gain |  |  |  |
| --- | --- | --- | --- | --- |
|  | B=1 CG/AR/VR | B=5 CG 30Mbps | B=5 AR/VR 30Mbps | B=3 AR/VR 45Mbps |
| FR1 | -72 | -71 | -71 | -70 |

Observation 43: The 5%-tile of coupling gain in InH deployment scenario computed at B UEs/cell (B=1 and B=“system capacity”) is indicated in Figure 9.

## Dense Urban (DU) scenario

Figure 10 shows the empirical CDF of the coupling gain measured in the DU deployment scenario in FR1, while Table 33 shows the 5%-tile of the distribution.

Figure 10  – Empirical CDF of Coupling Gain [dB] in DU deployment in FR1.

Table 33 – 5%-tile of coupling gain [dB] in DU deployment.

|  | 5%-tile of coupling gain |  |  |  |
| --- | --- | --- | --- | --- |
|  | B=1 CG/AR/VR | B=8 CG 30Mbps | B=8 AR/VR 30Mbps | B=6 AR/VR 45Mbps |
| FR1 | -123 | -126 | -126 | -126 |

Observation 44: The 5%-tile of DL coupling gain in DU deployment scenario computed at B UEs/cell (B=1 and B=“system capacity”) is indicated in Figure 10.

Note: For B=1 (mean of -123 dB), the CDF is not very flat (likely due to insufficient number of points), the range in the SLS tool is [-131;-115] and the variance is quite high. In contrast, for B=capacity, the range is smaller [-121;-128] and most of the dots are concentrated around the mean value of -126 dB.

# Mobility Evaluation

Mobility and, especially, handover events may crucially affect the performance and reliability of the XR services in certain setups. To estimate the potential impact of handover events on the characteristics of XR services, a simple still credible analytical study is utilized in this section. First, the realistic handover durations are identified for existing handover techniques by directly following the current specifications. Then, the average number of consecutive XR frames to be lost during the handover are calculated from the obtained handover time durations. Finally, the impact of different handover interruption times on the agreed XR UE satisfactory metrics is evaluated.

RAN1#106-e agreed on the following baseline definition and evaluation methodology for mobility KPI for the XR [6]:

Alternative 1 (Modified Option 3): For XR/Cloud Gaming mobility evaluation, the metric is defined to be {N,T} where N is the number of consecutive XR packets lost due to a HO event and T is the minimum target time interval between HO events, which are obtained by the following steps Step 1. HO interruption time is calculated for existing HO techniques by directly following the requirements given in 3GPP TS 38.133. Step 2. For a HO interruption time Y (calculated in Step 1) and the XR traffic pattern characterized by the packet arrival rate in average R and the packet delay budget PDB: Number of consecutive XR packets lost due to a HO event, N is estimated as: N = (Y – PDB) * R, Y >= PDB Minimum target time interval between HO events, T is estimated as: where PE,OP is packet error rate during time outside of handover procedure. Companies can report the value of PE,OP used in the evaluation and assumptions. X is the UE satisfactory requirement (baseline: X = 99%, other X value(s) can be also evaluated). Company can optionally evaluate the case of Y < PDB. E.g. N = max {(Y – PDB) * R, 0}, and , when Y < PDB; Or N = Y * R, and , when Y < PDB. Note 1: how to draw the observations/conclusion based on the simplified assumption will be discussed in RAN1 #107e. Note 2: mobility evaluation is performed in dense Urban and UMA Note 3: T maybe affected by system load, interference, etc.

## HO interruption times for existing HO techniques

This subsection illustrates Step 1 of the agreed mobility evaluation methodology presented above.

According to TS 38.133 [7] Clause 6.1.1.2.2, the baseline HO interruption time can be estimated as:

Tinterrupt = Tsearch + TIU + Tprocessing  + T∆ + Tmargin ms

In this formula, we set:

- Tsearch = 0 ms, reasonably assuming that typically a HO is performed to a target cell that is known (as per Clause 6.1.1.2.2) [7]

- TIU = 20 ms (the interruption uncertainty in acquiring the first available PRACH occasion in the new cell), as per Clause 6.1.1.2.2 [7]. “SSB to PRACH occasion associated period is defined in the table 8.1-1 of TS 38.213 [8].” In practice, TIU can be slightly lower, as the node doesn’t have to wait for the entire duration to get the first PRACH occasion. As the HO decisions are independent on the actual locations of SSB/PRACH occasions, the average can be assumed to be around half of 20 ms = 10 ms

- Tprocessing = 20 ms, as per Clause 6.1.1.2.2 [7].

- T∆ = Trs = 20 ms (default value of SMTC period), as per Clause 6.1.1.2.2 [7]. The minimal reasonable value here is 10 ms as the shortest feasible SMTC periodicity.

- Tmargin = 2 ms (time for SSB post-processing), as per Clause 6.1.1.2.2 [7].

Hence, the baseline HO would lead to an interruption time in the order of 42-62 ms (both for FR1-FR1 and FR2-FR2, as per Clause 6.1.1.4.2 [7]).

Repeating the same calculations for Conditional HO (CHO), we observe that (as per Clause 6.1.4.2.4 and Clause 6.1.4.4.4 [TS 38.133]) the interruption time is also in the order of 42-62 ms.

For DAPS, as per Clause 6.1.3.2.2 [7], the interruption time can be in the order of 2 ms.

These HO interruption times are used later in Section 6.2 and Section 6.3.

Observation 45: Appropriate interruption times incorporated in the HO, CHO, and DAPS, as detailed in TS 38.133, are computed as 42-62 ms for baseline HO and CHO, and 2 ms for DAPS.

## Number of consecutive lost XR packets

Following step 2 of the agreed methodology for mobility evaluation, the number of consecutive XR packets lost due to a HO event, N is estimated as: N = (Y – PDB) * R, Y >= PDB. To be consistent with this main formula, we also estimate N for Y<PDB as N = max {(Y – PDB) * R, 0}, hence, we set N = 0, when  Y < PDB

First, we present the results of HO interruption time, max(Y-PDB, 0), in Table 34 below:

Table 34 – Summary of XR traffic interruption time [ms] for different HO interruption times.

| Use case |  |  |  | HO interruption times [ms] |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | 2 ms (typical for DAPS) | 42 ms (typical optimistic value for baseline HO and CHO) | 62ms (typical pessimistic for baseline HO and CHO | 10 ms | 20 ms | 40 ms | 60 ms | 80 ms | 100 ms |
| DL | CG |  | Singe-stream video (60fps), 16.6ms periodicity, PDB=15ms | 0 ms | 27 ms | 47 ms | 0 ms | 5 ms | 25 ms | 45 ms | 65 ms | 85 ms |
|  | VR |  | Singe-stream video (60fps), 16.6ms periodicity, PDB=10ms | 0 ms | 32 ms | 52 ms | 0 ms | 10 ms | 30 ms | 50 ms | 70 ms | 90 ms |
|  | AR |  | Singe-stream video (60fps), 16.6ms periodicity, PDB=10ms | 0 ms | 32 ms | 52 ms | 0 ms | 10 ms | 30 ms | 50 ms | 70 ms | 90 ms |
|  |  | ======================================================= |  |  |  |  |  |  |  |  |  |  |
| UL | CG |  | Single-stream pose/control information, 4ms periodicity, PDB = 10ms | 0 ms | 32 ms | 52 ms | 0 ms | 10 ms | 30 ms | 50 ms | 70 ms | 90 ms |
|  | VR |  | Single-stream pose/control information, 4ms periodicity, PDB = 10ms | 0 ms | 32 ms | 52 ms | 0 ms | 10 ms | 30 ms | 50 ms | 70 ms | 90 ms |
|  | AR |  | Single-stream video (60 fps), 16.6ms periodicity, PDB = 30ms | 0 ms | 12 ms | 32 ms | 0 ms | 0 ms | 10 ms | 30 ms | 50 ms | 70 ms |

We then proceed with deriving the number of consecutive lost XR packets, N, as illustrated in Table 35.

Table 35 – Number of consecutive lost XR packets, N [packets], for different HO interruption times.

| Use case |  |  | HO interruption times [ms] |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 2 ms (typical for DAPS) | 42 ms (typical optimistic for baseline HO and CHO) | 62ms (typical pessimistic for baseline HO and CHO | 10 ms | 20 ms | 40 ms | 60 ms | 80 ms | 100 ms |
| DL | CG | Singe-stream video (60fps), 16.6ms periodicity, PDB=15ms | 0 packets | 1.63 packets | 2.83 packets | 0 packets | 0.3 packets | 1.51 packets | 2.71 packets | 3.92 packets | 5.12 packets |
|  | VR | Singe-stream video (60fps), 16.6ms periodicity, PDB=10ms | 0 packets | 1.93 packets | 3.13 packets | 0 packets | 0.6 packets | 1.81 packets | 3.01 packets | 4.22 packets | 5.42 packets |
|  | AR | Singe-stream video (60fps), 16.6ms periodicity, PDB=10ms | 0 packets | 1.93 packets | 3.13 packets | 0 packets | 0.6 packets | 1.81 packets | 3.01 packets | 4.22 packets | 5.42 packets |
|  |  | ======================================================= |  |  |  |  |  |  |  |  |  |
| UL | CG | Single-stream pose/control information, 4ms periodicity, PDB = 10ms | 0 packets | 8 packets | 13 packets | 0 packets | 2.5 packets | 7.5 packets | 12.5 packets | 17.5 packets | 22.5 packets |
|  | VR | Single-stream pose/control information, 4ms periodicity, PDB = 10ms | 0 packets | 8 packets | 13 packets | 0 packets | 2.5 packets | 7.5 packets | 12.5 packets | 17.5 packets | 22.5 packets |
|  | AR | Single-stream video (60 fps), 16.6ms periodicity, PDB = 30ms | 0 packets | 0.72 packets | 1.93 packets | 0 packets | 0 packets | 0.6 packets | 1.81 packets | 3.01 packets | 4.22 packets |

Observation 46: The values reported in 35 are of a particular interest, recalling that they present the number of consecutive frames to be lost. Loosing several frames in a row is more crucial than losing the same number of frames over a long period of time, as the former may cause severe drops in QoS and user experience, including video “freezing”, video “blinking”, among other negative effects.

Observation 47: When HO interruption time (Y) grows, the number of consecutive lost XR packets (N) grows as well.

Observation 48: When the packet rate (R) grows, the number of consecutive lost XR packets (N) grows as well.

Observation 49: When the packet delay budget (PDB) grows, the number of consecutive lost XR packets (N) decreases.

## Minimum target time interval between HO events, T

Following step 2 of the agreed methodology for mobility evaluation, minimum target time interval between HO events, T, is computed as

Following the same logic as in Section 6.2, we set T = 0 s for Y < PDB.

Below, we presented the values of T computed for different values of PE,OP, ranging from extremely optimistic 0% up to pessimistic 90%. For the considered XR applications, the calculated values are listed in Tables 36, 37, and 38.

Table 36 – Minimum target time interval between HO events, T [sec], for different HO interruption times;                                  VR DL singe-stream video (60fps), 16.6ms periodicity, PDB=10ms;                                                                                    AR DL singe-stream video (60fps), 16.6ms periodicity, PDB=10ms;                                                                                            CG UL Single-stream pose/control information, 4ms periodicity, PDB = 10ms;                                                                 VR UL Single-stream pose/control information, 4ms periodicity, PDB = 10ms;

| PE,OP | HO interruption times [ms] |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 2 ms (typical for DAPS) | 42 ms (typical optimistic for baseline HO and CHO) | 62 ms (typical pessimistic for baseline HO and CHO) | 10 ms | 20 ms | 40 ms | 60 ms | 80 ms | 100 ms |
| 0% | 0 s | 3.2 s | 5.2s | 0 s | 1 s | 3 s | 5 s | 7 s | 9 s |
| 0.2% | 0 s | 4 s | 6.49 s | 0 s | 1.25 s | 3.74 s | 6.24 s | 8.73 s | 11.23 s |
| 0.4% | 0 s | 5.31 s | 8.63 s | 0 s | 1.66 s | 4.98 s | 8.3 s | 11.62 s | 14.94 s |
| 0.6% | 0 s | 7.95 s | 12.92 s | 0 s | 2.49 s | 7.46 s | 12.45 s | 17.4 s | 22.37 s |
| 0.8% | 0 s | 15.87 s | 25.8 s | 0 s | 4.96 s | 14.88 s | 24.8 s | 34.72 s | 44.64 s |
| 0.9% | 0 s | 31.71 s | 51.53 s | 0 s | 9.91 s | 29.73 s | 49.55 s | 69.37 s | 89.19 s |

Table 37 – Minimum target time interval between HO events, T [sec], for different HO interruption times;                                  CG DL singe-stream video (60fps), 16.6ms periodicity, PDB=15ms.

| PE,OP | HO interruption times [ms] |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 2 ms (typical for DAPS) | 42 ms (typical optimistic for baseline HO and CHO) | 62 ms (typical pessimistic for baseline HO and CHO) | 10 ms | 20 ms | 40 ms | 60 ms | 80 ms | 100 ms |
| 0% | 0 s | 2.7 s | 4.7 s | 0 s | 0.5 s | 2.5 s | 4.5 s | 6.5 s | 8.5 s |
| 0.2% | 0 s | 3.37 s | 5.86 s | 0 s | 0.6 s | 3.12 s | 5.61 s | 8.11 s | 10.6 s |
| 0.4% | 0 s | 4.48 s | 7.8 s | 0 s | 0.83 s | 4.15 s | 7.47 s | 10.79 s | 14.11 s |
| 0.6% | 0 s | 6.71 s | 11.68 s | 0 s | 1.24 s | 6.21 s | 11.18 s | 16.15 s | 21.12 s |
| 0.8% | 0 s | 13.39 s | 23.31 s | 0 s | 2.48 s | 12.4 s | 22.32 s | 32.24 s | 42.16 s |
| 0.9% | 0 s | 26.76 s | 46.58 s | 0 s | 4.96 s | 24.77 s | 44.6 s | 64.41 s | 84.23 s |

Table 38 – Minimum target time interval between HO events, T [sec], for different HO interruption times;                                  AR UL Single-stream video (60 fps), 16.6ms periodicity, PDB = 30ms.

| PE,OP | HO interruption times [ms] |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 2 ms (typical for DAPS) | 42 ms (typical optimistic for baseline HO and CHO) | 62 ms (typical pessimistic for baseline HO and CHO) | 10 ms | 20 ms | 40 ms | 60 ms | 80 ms | 100 ms |
| 0% | 0 s | 1.2 s | 3.2 s | 0 s | 0 s | 1 s | 3 s | 5 s | 7 s |
| 0.2% | 0 s | 1.5 s | 4 s | 0 s | 0 s | 1.25 s | 3.74 s | 6.24 s | 8.73 s |
| 0.4% | 0 s | 2 s | 5.31 s | 0 s | 0 s | 1.66 s | 4.98 s | 8.3 s | 11.62 s |
| 0.6% | 0 s | 3 s | 7.95 s | 0 s | 0 s | 2.49 s | 7.46 s | 12.45 s | 17.4 s |
| 0.8% | 0 s | 5.95 s | 15.87 s | 0 s | 0 s | 4.96 s | 14.88 s | 24.8 s | 34.72 s |
| 0.9% | 0 s | 11.9 s | 31.71 s | 0 s | 0 s | 9.91 s | 29.73 s | 49.55 s | 69.37 s |

We also observe that the minimal PDB between UL and DL for GC is min(15 ms, 10 ms) = 10ms, while for VR it is min(10ms, 10ms) = 10 ms and for AR it is min(10 ms, 30 ms) = 10 ms. Hence, we observe:

Observation 50: The minimal PDB between DL and UL for ALL the considered XR/CG services is 10 ms. Therefore, recalling that the traffic is bi-directional in all the considered services, the actual values for the minimum target time interval between HO events T are given in Table 39.

Table 39 – Minimum target time interval between HO events, T [sec], for different HO interruption times for CG, VR, and AR services with minimal PDB=10ms.

| PE,OP |  | HO interruption times [ms] |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 2 ms (typical for DAPS) | 42 ms (typical optimistic for baseline HO and CHO) | 62 ms (typical pessimistic for baseline HO and CHO) | 10 ms | 20 ms | 40 ms | 60 ms | 80 ms | 100 ms |
| 0% | 0 s | 3.2 s | 5.2s | 0 s | 1 s | 3 s | 5 s | 7 s | 9 s |
| 0.2% | 0 s | 4 s | 6.49 s | 0 s | 1.25 s | 3.74 s | 6.24 s | 8.73 s | 11.23 s |
| 0.4% | 0 s | 5.31 s | 8.63 s | 0 s | 1.66 s | 4.98 s | 8.3 s | 11.62 s | 14.94 s |
| 0.6% | 0 s | 7.95 s | 12.92 s | 0 s | 2.49 s | 7.46 s | 12.45 s | 17.4 s | 22.37 s |
| 0.8% | 0 s | 15.87 s | 25.8 s | 0 s | 4.96 s | 14.88 s | 24.8 s | 34.72 s | 44.64 s |
| 0.9% | 0 s | 31.71 s | 51.53 s | 0 s | 9.91 s | 29.73 s | 49.55 s | 69.37 s | 89.19 s |

When analysing Tables 36-39 and prior results, the following observations can be made:

Observation 51: When HO interruption time (Y) grows, the minimum target time interval between HO events (T) grows as well.

Observation 52: When the packet rate (R) grows, the minimum target time interval between HO events (T) grows as well.

Observation 53: When the error rate (PE,OP) grows, the minimum target time interval between HO events (T) grows as well.

Observation 54: When the packet delay budget (PDB) grows, the minimum target time interval between HO events (T) decreases.

Hence, the following main observations can be made from the performed mobility study for XR.

Observation 55: The results in tables 36-39 utilization of a HO technique leading to HO interruption time of less than 10 ms (i.e., DAPS) allows to notably improve the performance of XR services when comparing to baseline HO technique. The HO techniques leading to less than 10ms interruption time offers the lowest handover interruption times where the number of impacted XR frames from handovers is also minimized.

As the analysis above is applicable to both FR1 and FR2 HO events, it is important to also note the following:

Observation 56: While there is a solution for FR1-FR1 HO offering less than 10 ms HO interruption time (DAPS), such a solution is not yet available for FR2-FR2 case.

# Summary

In this contribution we have discussed performance results in indoor and outdoor deployments and bottlenecks that could be improved to increase 5G NR system capacity and UE power saving. Those naturally require further studies and discussions. In summary, following observations have been made for system capacity evaluation:

The following observations have been made for system capacity evaluation in Indoor Hotspot:

Observation 1: When we consider CG traffic model at 30Mbps for an InH deployment, we can support the number of UEs per cell in DL indicated in Table 4.

Observation 2: When we consider VR/AR traffic model at 30Mbps and 45Mbps for an InH deployment, we can support the number of UEs per cell in DL indicated in Table 5.

Observation 3: When we consider baseline CG and VR traffic model for UL (Pose Single-Stream) for Indoor Hotspot deployment, we can support a number of UEs per cell indicated in Table 6.

Observation 4: When we consider baseline AR traffic models for UL (Pose Single-Stream and Video+Pose Dual-Stream) for Indoor Hotspot deployment, we can support a number of UEs per cell indicated in Table 7.

The following observations have been made for system capacity evaluation in Dense Urban:

Observation 5: When we consider baseline CG traffic model for DL (Video Single-Stream) for Dense Urban deployment, we can support the number of UEs per cell indicated in Table 8.

Observation 6: When we consider baseline VR/AR traffic model for DL (Video Single-Stream) for Dense Urban deployment, we can support a number of UEs per cell indicated in Table 9.

Observation 7: When we consider baseline CG and VR traffic model for UL (Pose Single-Stream) for Dense Urban deployment, we can support a number of UEs per cell indicated in Table 10.

Observation 8: When we consider baseline AR traffic model for UL (Video Single-Stream) for Dense Urban deployment, we can support a number of UEs per cell indicated in Table 11.

The following observations have been made for UE power saving evaluation in Indoor Hotspot:

Observation 9: Power saving gain and DL capacity loss with different CDRX configurations for CG applications in Indoor Hotspot deployment in FR1 are indicated in Table 12 and Table 13 considering all UEs and satisfied UEs, respectively.

Observation 10: CDRX reduces UE mean power consumption between 9% and 22% across all UEs, and although it seems not to affect the DL system capacity much for CG applications for any of the considered DRX configurations, the fractional number of supported UEs is slightly different in each case.

Observation 11: The CDRX configuration (DRX long cycle, inactivity timer value, On duration timer value)=(4, 2, 2) is the best system-level and UE-level PS scheme, among the modelled ones, for CG applications in DL in Indoor Hotspot deployment in FR1.

Observation 12: Power saving gain and DL capacity loss with different CDRX configurations for VR/AR applications at 30Mbps in Indoor Hotspot deployment in FR1 are indicated in Table 14 and Table 15 considering all UEs and satisfied UEs, respectively.

Observation 13: Power saving gain and DL capacity loss with different CDRX configurations for VR/AR applications at 45Mbps in Indoor Hotspot deployment in FR1 are indicated in Table 16 and Table 17 considering all UEs and satisfied UEs, respectively.

Observation 14: CDRX reduces UE mean power consumption for VR/AR applications between 0% and 22% for 30Mbps and between 0% and 16.4% for 45Mbps but it also strongly affects the DL system capacity, reducing it to 0 in the worst case due to the low delay budget with respect to the simulated retransmission delay.

Observation 15: The CDRX configuration (DRX long cycle, inactivity timer value, On duration timer value)=(4, 2, 2) is the best system-level and UE-level PS scheme, among the modelled ones, in DL in Indoor Hotspot deployment in FR1 for AR/VR applications both at 30Mbps and 45Mbps.

Observation 16: CDRX configuration (DRX long cycle, inactivity timer value, On duration timer value)=(16, 8, 8) archives zero capacity due to the misalignment between the start of the DRX cycle and the AR/VR traffic for both 30Mbps and 45Mbps. Therefore, an improper configuration of the DRX start offset of the CDRX configuration negatively affects system capacity for AR/VR services. A shorter duration of the long DRX cycle helps to reduce the effect of the improper configuration of the DRX start offset.

Observation 17: Power saving gain and UL capacity loss with different CDRX configurations for AR “Video Single-Stream” applications in Indoor Hotspot deployment in FR1 are indicated in Table 18  considering all UEs.

Observation 18: CDRX reduces UE mean power consumption between 8.67% and 21.64% across all UEs, and although it seems not to affect the UL system capacity for AR “Video Single-Stream” applications for any of the considered DRX configurations, the fractional number of supported UEs is slightly different in each case.

Observation 19: The CDRX configuration (DRX long cycle, inactivity timer value, On duration timer value)=(4, 2, 2) is the best system-level and UE-level PS scheme, among the modelled ones, for AR “Video Single-Stream” applications in UL in Indoor Hotspot deployment in FR1, since it achieves the same capacity as Always ON and the highest PS gain.

Observation 20: Power saving gain and DL capacity loss with different CDRX configurations for CG applications in Indoor Hotspot deployment in FR2 are indicated in Table 19 and Table 20  considering all UEs and satisfied UEs, respectively.

Observation 21: CDRX reduces UE mean power consumption between 7.6% and 18.8% across all UEs, but, at the same time, it also affects the system capacity. A non-optimal configuration of CDX parameters can cause a significant drop in the number of satisfied users per cell.

Observation 22: The CDRX configuration (DRX long cycle, inactivity timer value, On duration timer value)=(4, 2, 2) is the best system-level and UE-level PS scheme, among the modelled ones, for CG applications in DL in Indoor Hotspot deployment in FR2, since it achieves the same capacity as Always ON and the highest PS gain.

Observation 23: Power saving gain and DL capacity loss with different CDRX configurations for VR/AR applications at 30Mbps in Indoor Hotspot deployment in FR2 are indicated in Table 21 and Table 22 considering all UEs and satisfied UEs, respectively.

Observation 24: Power saving gain and DL capacity loss with different CDRX configurations for VR/AR applications at 45Mbps in Indoor Hotspot deployment in FR2 are indicated in Table 23 and Table 24 considering all UEs and satisfied UEs, respectively.

Observation 25: CDRX reduces UE mean power consumption for VR/AR applications between 0% and 19.58% for 30Mbps and between 0% and 18.1% for 45Mbps, but it also strongly affects the DL system capacity, reducing it to 0 in the worst case due to the low delay budget with respect to the simulated retransmission delay.

Observation 26: The CDRX configuration (DRX long cycle, inactivity timer value, On duration timer value)=(10, 8, 2) is the best system-level PS scheme, among the modelled ones, for AR/VR applications at 30Mbps and 45Mbps in DL in Indoor Hotspot deployment in FR2, since it achieves the same capacity as Always ON.

Observation 27: The CDRX configuration (DRX long cycle, inactivity timer value, On duration timer value)=(4, 2, 2) is the best UE-level PS scheme, among the modelled ones, for AR/VR applications at 30Mbps and 45Mbps in DL in Indoor Hotspot deployment in FR2, since it achieves the highest mean PS gain with respect to Always ON.

The following observations have been made for UE power saving evaluation in Dense Urban:

Observation 28: Power saving gain and DL capacity loss with different CDRX configurations for CG applications in Indoor Hotspot deployment in FR1 are indicated in Table 25 and Table 26 considering all UEs and satisfied UEs, respectively.

Observation 29: CDRX reduces UE mean power consumption between 9% and 22% across all UEs, but, at the same time, it also affects the system capacity. A non-optimal configuration of CDX parameters can cause a significant drop in the number of satisfied users per cell.

Observation 30: The CDRX configuration (DRX long cycle, inactivity timer value, On duration timer value)=(10, 8, 2) is the best system-level PS scheme, among the modelled ones, for CG applications in DL in Dense Urban deployment in FR1, since it achieves the same capacity as Always ON.

Observation 31: The CDRX configuration (DRX long cycle, inactivity timer value, On duration timer value)=(4, 2, 2) is the best UE-level PS scheme, among the modelled ones, for CG applications in DL in Dense Urban deployment in FR1, since it achieves the highest mean PS gain with respect to Always ON.

Observation 32: Power saving gain and DL capacity loss with different CDRX configurations for VR/AR applications at 30Mbps in Dense Urban deployment in FR1 are indicated in Table 27 and Table 28 considering all UEs and satisfied UEs, respectively.

Observation 33: Power saving gain and DL capacity loss with different CDRX configurations for VR/AR applications at 45Mbps in Dense Urban deployment in FR1 are indicated in Table 29 and Table 30 considering all UEs and satisfied UEs, respectively.

Observation 34: CDRX reduces UE mean power consumption for VR/AR applications between 0% and 22% for 30Mbps and between 0% and 16.1% for 45Mbps, but it also strongly affects the DL system capacity, reducing it to 0 in the worst case due to the low delay budget with respect to the simulated retransmission delay.

Observation 35: The CDRX configuration (DRX long cycle, inactivity timer value, On duration timer value)=(10, 8, 2) is the best system-level PS scheme, among the modelled ones, for AR/VR applications at 30Mbps in DL in Dense Urban deployment in FR1, since it achieves the same capacity as Always ON.

Observation 36: The CDRX configuration (DRX long cycle, inactivity timer value, On duration timer value)=(4, 2, 2) is the best UE-level PS scheme, among the modelled ones, for AR/VR applications at 30Mbps in DL in Dense Urban deployment in FR1, since it achieves the highest mean PS gain with respect to Always ON.

Observation 37: The CDRX configuration (DRX long cycle, inactivity timer value, On duration timer value)=(4, 2, 2) is the best system-level and UE-level PS scheme, among the modelled ones, for AR/VR applications at 45Mbps in DL in Dense Urban deployment in FR1, since it achieves the same capacity and the highest mean PS gain with respect to Always ON.

Observation 38: CDRX configuration (DRX long cycle, inactivity timer value, On duration timer value)=(16, 8, 8) archives zero capacity due to the misalignment between the start of the DRX cycle and the AR/VR traffic for both 30Mbps and 45Mbps. Therefore, an improper configuration of the DRX start offset of the CDRX configuration negatively affects system capacity for AR/VR services. A shorter duration of the long DRX cycle helps to reduce the effect of the improper configuration of the DRX start offset.

Observation 39: Power saving gain and UL capacity loss with different CDRX configurations for AR “Video Single-Stream” applications in Dense Urban deployment in FR1 are indicated in Table 31 considering all UEs.

Observation 40: CDRX reduces UE mean power consumption between 5.8% and 14.6% across all UEs, and although it seems not to affect the UL system capacity for AR “Video Single-Stream” applications for any of the considered DRX configurations, the fractional number of supported UEs is slightly different in each case.

Observation 41: The CDRX configuration (DRX long cycle, inactivity timer value, On duration timer value)=(8, 4, 4) is the best system-level PS scheme, among the modelled ones, for AR “Video Single-Stream” applications in UL in Dense Urban deployment in FR1, since it achieves the same capacity as Always ON.

Observation 42: The CDRX configuration (DRX long cycle, inactivity timer value, On duration timer value)=(4, 2, 2) is the best UE-level PS scheme, among the modelled ones, for AR “Video Single-Stream” applications in UL in Dense Urban deployment in FR1, since it achieves the highest mean PS gain with respect to Always ON.

The following observations have been made for the coverage evaluation:

Observation 43: The 5%-tile of coupling gain in InH deployment scenario computed at B UEs/cell (B=1 and B=“system capacity”) is indicated in Figure 9.

Observation 44: The 5%-tile of DL coupling gain in DU deployment scenario computed at B UEs/cell (B=1 and B=“system capacity”) is indicated in Figure 10.

The following observations have been made for the mobility evaluation:

Observation 45: Appropriate interruption times incorporated in the HO, CHO, and DAPS, as detailed in TS 38.133, are computed as 42-62 ms for baseline HO and CHO, and 2 ms for DAPS.

Observation 46: The values reported in 35 are of a particular interest, recalling that they present the number of consecutive frames to be lost. Loosing several frames in a row is more crucial than losing the same number of frames over a long period of time, as the former may cause severe drops in QoS and user experience, including video “freezing”, video “blinking”, among other negative effects.

Observation 47: When HO interruption time (Y) grows, the number of consecutive lost XR packets (N) grows as well.

Observation 48: When the packet rate (R) grows, the number of consecutive lost XR packets (N) grows as well.

Observation 49: When the packet delay budget (PDB) grows, the number of consecutive lost XR packets (N) decreases.

Observation 50: The minimal PDB between DL and UL for ALL the considered XR/CG services is 10 ms. Therefore, recalling that the traffic is bi-directional in all the considered services, the actual values for the minimum target time interval between HO events T are given in Table 39.

Observation 51: When HO interruption time (Y) grows, the minimum target time interval between HO events (T) grows as well.

Observation 52: When the packet rate (R) grows, the minimum target time interval between HO events (T) grows as well.

Observation 53: When the error rate (PE,OP) grows, the minimum target time interval between HO events (T) grows as well.

Observation 54: When the packet delay budget (PDB) grows, the minimum target time interval between HO events (T) decreases.

Observation 55: The results in tables 36-39 utilization of a HO technique leading to HO interruption time of less than 10 ms (i.e., DAPS) allows to notably improve the performance of XR services when comparing to baseline HO technique. The HO techniques leading to less than 10ms interruption time offers the lowest handover interruption times where the number of impacted XR frames from handovers is also minimized.

Observation 56: While there is a solution for FR1-FR1 HO offering less than 10 ms HO interruption time (DAPS), such a solution is not yet available for FR2-FR2 case.

All previous observations on system capacity and DL UE power saving gains (option 1) are captured and summarized by the following tables:

|  |  | CG (DL) | VR/AR (DL) |  | CG / VR (UL – Pose) | AR (UL – Video) | AR (UL – Video+Pose) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Deployment | Frequency Range | Rate: 30Mbps | Rate: 30Mbps | Rate: 45Mbps | Rate: 200Kbps | Rate: 10Mbps | Rate: 10Mbps+200Kbps |
| Indoor Hotspot (DL) | FR1 | 5 | 5 | 3 | 54 | 4 | 4 |
|  | FR2 | 11 | 10 | 6 | - | - | - |
| Dense Urban (DL) | FR1 | 8 | 6 | 4 | 45-50 | 4 | - |
|  | FR2 | 8 | 6 | 3 | - | - | - |

|  |  | Cloud Gaming (30Mbps) |  | Virtual/Augmented Reality (30Mbps) |  | Virtual/Augmented Reality (45Mbps) |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Deployment | PS Scheme | Mean PS gain | Capacity | Mean PS gain | Capacity | Mean PS gain | Capacity |
| Indoor Hotspot (FR1) | Always ON | - | - / 5 | - | - / 5 | - | - / 3 |
|  | CDRX (4, 2, 2) | 20.9 | 4.84 / 5 | 20.9 | 4.5 / 5 | 15.7 | 2.85 / 3 |
|  | CDRX (8, 4, 4) | 18.2 | 4.84 / 5 | 18.2 | 4.15 / 5 | 12.1 | 2.54 / 3 |
|  | CDRX (16, 8, 8) | 16.2 | 4.75 / 5 | 16.2 | 0 / 5 | 9.4 | 0 / 3 |
|  | CDRX(10,8,2) | 9.3 | 4.93 / 5 | 9.3 | 4.65 / 5 | 4 | 2.91 / 3 |
|  | CDRX(10,5,5) | 17.3 | 4.82 / 5 | 17.3 | 3.7 / 5 | 10.8 | 1.89 / 3 |
| Indoor Hotspot (FR2) | Always ON | - | - / 11 | - | - / 10 | - | - / 6 |
|  | CDRX (4, 2, 2) | 18.5 | 10.81 / 11 | 18.5 | 8.5 / 10 | 18 | 4.5 / 6 |
|  | CDRX (8, 4, 4) | 15.4 | 10.77 / 11 | 15.4 | 2 / 10 | 15 | 0.56 / 6 |
|  | CDRX (16, 8, 8) | 11.6 | 8.59 / 11 | 11.6 | 0 | 11.6 | 0 / 6 |
|  | CDRX(10,8,2) | 7.6 | 10.8 / 11 | 7.6 | 9.24 / 10 | 7.5 | 5.4 / 6 |
|  | CDRX(10,5,5) | 13.7 | 10.66 / 11 | 13.7 | 0.7 / 10 | 13.5 | 0.19 / 6 |
| Dense Urban (FR1) | Always ON | - | - / 8 | - | - / 6 | - | - / 4 |
|  | CDRX (4, 2, 2) | 20 | 7.04 / 8 | 21 | 4.98 / 6 | 14.5 | 2.76 / 4 |
|  | CDRX (8, 4, 4) | 16.7 | 6.72 / 8 | 18 | 3.66 / 6 | 10.8 | 1.6 / 4 |
|  | CDRX (16, 8, 8) | 13.6 | 5.6 / 8 | 15.8 | 0 / 6 | 7.9 | 0 / 4 |
|  | CDRX(10,8,2) | 8.8 | 7.44 / 8 | 9.2 | 5.58 / 6 | 3.2 | 3.52 / 4 |
|  | CDRX(10,5,5) | 15.4 | 6.08 / 8 | 17 | 3.12 / 6 | 9.5 | 0.96 / 4 |

# References

- RP-210460 Revised SID on XR Evaluations for NR, RAN#91

- R1-2009812 FL Summary of RAN1 103-e agreements and email discussions on Rel-17 SI on XR evaluations for NR

- RAN1 Chairman’s Notes (v018) 3GPP TSG RAN WG1 Meeting #104-e e-Meeting, January 25th – February 5th, 2021

- R1-2101866 Summary of [104-e-NR-XR-02]_Other Eval Methodology.

- Chair notes RAN1, 104-bis-e Chair_notes_104b-e

- R1- 2110675 Summary on [106bis-e-NR-XR-02] Email discussion/approval on remaining issues on XR evaluation methodology

- 3GPP, TS 38.133 "NR; Requirements for support of radio resource management”

- 3GPP, TS 38.213 "NR; Physical layer procedures for control”

# Appendix A – Simulation settings

## Indoor Hotspot (InH) scenario

The carrier frequency is set to 4 GHz and 30 GHz for FR1 and FR2, respectively. System bandwidth is assumed to be 100 MHz for FR1 and for FR2 to compare achievable system capacity when propagation and antenna configuration change. Time division duplexing (TDD) is configured according to the first option, thus using “DDDSU” as radio frame. The smallest schedulable radio resource is the physical radio blocks (PRBs) of 12 subcarriers, each is of 30 kHz and 120 kHz for FR1 and FR2, respectively. The TTI size is set to 14 OFDM symbols, with one control symbol, always placed at the start of each TTI. The asynchronous HARQ Chase combing is adopted with maximum 3 HARQ retransmission before a packet is dropped (i.e., marked with an infinite radio latency). The transmit power of gNBs is set as follows: 31dBm with 100MHz (24dBm per 20MHz) in FR1, and 24dBm with (23dBm per 80MHz) in FR2. Table 37 lists the main parameters of the Indoor Hotspot deployment that are considered in this study.

Table 45 – Main parameters for Indoor Hotspot (InH) deployment

| Parameter | Value |  |
| --- | --- | --- |
| Layout | - 120m x 50m, Single layer (indoor floor, open office) - 12 cells/TRPs - ISD: 20m |  |
| Channel model | InH |  |
| Carrier frequency | FR1: 4 GHz | FR2: 30 GHz |
| Subcarrier spacing | FR1: 30 kHz | FR2: 120 kHz |
| System bandwidth | FR1: 100 MHz | FR2: 100 MHz |
| BS height | 3 m |  |
| UE height | 1.5 m |  |
| BS noise figure | FR1: 5 dB | FR2: 7 dB |
| UE noise figure | FR1: 9 dB | FR2: 13 dB |
| BS receiver | LMMSE-IRC |  |
| UE receiver | LMMSE-IRC |  |
| Channel estimation | Realistic (with ideal CSI) |  |
| UE speed | 3 km/h |  |
| MCS | Up to 256QAM |  |
| BS Tx power | FR1: 31dBm (24dBm per 20MHz) | FR2: 24 dBm (23dBm per 80MHz) |
| UE Tx max power | 23 dBm |  |
| TDD Frame structure | DDDSU |  |
| Cell Selection | RSRP Slow Fading |  |
| BS antenna configuration | - Pattern: Ceiling-mount antenna radiation pattern - Gain: 5 dBi - Downtilt: 90° |  |
|  | Configuration in FR1: - 32 TxRU - (M, N, P, Mg, Ng) = (4,4,2,1,1) - (dH, dV) = (0.5, 0.5)λ - (Mp, Np) = (4,4) | Configuration in FR2: - 2 TxRU - (M, N, P, Mg, Ng) = (16, 8, 2,1,1) - (dH, dV) = (0.5, 0.5) λ - (Mp, Np) = (1,1) Grid of Beams: - Azimuth angles (degrees): {90, 90, 90, 112.5, 112.5, 112.5, 67.5, 67.5, 67.5, 140, 140, 140, 40, 40, 40} - Elevation angles (degrees): {-30, 0, 30, -30, 0, 30, -30, 0, 30, -30, 0, 30, -30, 0, 30} |
| UE antenna configuration | - Pattern : Omni-directional, - Gain : 0 dBi, - Configuration : 2T/4R (M, N, P, Mg, Ng) = (1,2,2,1,1) (dH, dV) = (0.5, 0)λ (Mp, Np) = (1,2) | - Pattern: UE radiation pattern model 1 (TR 38.901) - Gain: 5 dBi - Configuration (Option 1): (M, N, P) = (1, 4, 2), (dH, dV) = (0.5, 0)λ (Mp, Np) = (1,1) 3 panels (left, right, top) |
| Scheduler | SU-MIMO, Proportional Fairness |  |
| CSI acquisition | Periodic CQI on 2 ms period |  |
| PHY processing delay | PDSCH decoding: 6 OFDM symbols |  |
| PDCCH overhead | Modelled |  |
| Target BLER | 10% for first transmission |  |
| Max HARQ transmission | 3 |  |
| HARQ scheme | Chase combining |  |

## Dense Urban (DU) Scenario

The carrier frequency is set to 4 GHz and 30GHz for FR1 and FR2, respectively. System bandwidth has been fixed to 100MHz for both FR1 and FR2 to compare achievable system capacity when propagation and antenna configuration change. Time division duplexing (TDD) is configured according to the first option, thus using “DDDSU” as radio frame. The smallest schedulable radio resource is the physical radio blocks (PRB) of 12 subcarriers, each of 30kHz and 120kHz for FR1 and FR2, respectively. The TTI size is set to 14 OFDM symbols, with one control symbol, always placed at the start of each TTI. The asynchronous HARQ Chase combing is adopted with maximum 3 HARQ retransmissions before a packet is dropped (i.e., marked with an infinite radio latency). The transmit power of gNBs is set to 51 dBm (i.e., 44dBm for 20MHz). Table 38 lists the main parameters of the Dense Urban deployment that are considered in this study.

Table 46– Main parameters for Dense Urban (UMa) deployment

| Parameter | Value |  |  |
| --- | --- | --- | --- |
| Layout | 21 cells with wraparound (ISD: 200m) |  |  |
| Channel model | Uma |  |  |
| Carrier frequency | FR1: 4 GHz | FR2: 30 GHz |  |
| Subcarrier spacing | FR1: 30 kHz | FR2: 120 kHz |  |
| System bandwidth | FR1: Option 1: 100 MHz | FR2: Option 1: 100 MHz |  |
| BS height | 25m |  |  |
| UE height | hUT = 3(nfl – 1) + 1.5 |  |  |
|  | Outdoor: nfl = 1 | Indoor: - nfl ~ uniform(1,Nfl) - Nfl ~ uniform(4,8) |  |
| BS noise figure | FR1: 5 dB |  | FR2: 7 dB |
| UE noise figure | FR1: 9 dB |  | FR2: 13 dB |
| BS receiver | MMSE-IRC |  |  |
| UE receiver | MMSE-IRC |  |  |
| Channel estimation | Realistic (with ideal CSI) |  |  |
| UE speed | 3 km/h |  |  |
| MCS | Up to 256QAM |  |  |
| BS Tx power | 44 dBm per 20 MHz 51 dBm per 100 MHz |  |  |
| UE Tx max power | FR1: 23 dBm |  | FR2: 23 dBm, maximum EIRP 43 dBm |
| TDD Frame structure | Option 1: DDDSU |  |  |
| Mechanical Downtilt | Baseline: 12 degrees |  |  |
| Cell Selection | RSRP Slow Fading |  |  |
| BS antenna configuration | FR1: 32TxRUs (M, N, P, Mg, Ng; Mp, Np) = (8,2,2,1,1,8,2), (dH, dV) = (0.5λ, 0.5λ) | FR2: 2TxRUs (M, N, P, Mg, Ng; Mp, Np) = (4,8,2,2,2;1,1), (dH, dV) = (0.5λ, 0.5λ) Grid of Beams - Azimuth angles: {33.75, 56.25, 78.75, 101.25, 123.75, 146.25, 33.75, 56.25, 78.75, 101.25, 123.75, 146.25} degrees - Elevation angles: {-12.5, -12.5, -12.5, -12.5, -12.5, -12.5, -57.5, -57.5, -57.5, -57.5, -57.5, -57.5} degrees |  |
| UE antenna configuration | FR1: 2T/4R, (M, N, P, Mg, Ng; Mp, Np) = (1,2,2,1,1;1,2), (dH, dV) = (0.5λ, -N/Aλ) | FR2: (M, N, P)=(1, 4, 2), 3 panels (left, right, top) (Mp, Np)=(1, 1) |  |
| Power control parameter | Open loop, Alpha = 1, P0 = -106 dBm |  |  |
| Scheduler | SU-MIMO, Proportional Fairness |  |  |
| CSI acquisition | Periodic CQI on 2 ms period |  |  |
| PHY processing delay | PDSCH decoding: 6 OFDM symbols |  |  |
| PDCCH overhead | Modelled |  |  |
| Target BLER | 10% for first transmission |  |  |
| Max HARQ transmission | 3 |  |  |
| HARQ scheme | Chase combining |  |  |