3GPP TSG RAN WG1 #107-e		R1-2111880

e-Meeting, November 11th – 19th, 2021

Source: 	Apple Inc.

Title:                     Reduced maximum UE bandwidth for Redcap

Agenda item:	8.6.1.1

Document for:	Discussion and Decision

# 1 Introduction

One objective of Rel-17 Redcap WI is to reduce maximum UE bandwidth as follows [1]:

- Specify support for the following UE complexity reduction features [RAN1, RAN2, RAN4]: - Reduced maximum UE bandwidth: - Maximum bandwidth of an FR1 RedCap UE during and after initial access is 20 MHz. - Maximum bandwidth of an FR2 RedCap UE during and after initial access is 100 MHz.

In the RAN1 #106bis e-meeting, the following working assumption was confirmed and new agreements were made as follows:

Agreement: Confirm the working assumption: - In case a separate initial UL BWP is configured for RedCap UEs, it is supported that the network can enable/disable intra-slot PUCCH frequency hopping within the separate initial UL BWP in the PUCCH resource for HARQ feedback for Msg4/MsgB for RedCap UEs. - The frequency hopping is enabled/disabled at least via SIB. Agreement - For a cell that allows a RedCap UE to access, network can configure a separate initial UL BWP for RedCap UEs in SIB - It can be used both during and after initial access. - It is no wider than the maximum RedCap UE bandwidth. - It is always configured if the initial UL BWP for non-RedCap UEs is wider than the maximum RedCap UE bandwidth - This applies to both TDD and FDD (including FD FDD and HD FDD) cases Working Assumption - For a cell that allows a RedCap UE to access, network can configure a separate initial DL BWP for RedCap UEs in SIB. - Working assumption: It can be used during initial access - It can be used after initial access. - It is no wider than the maximum RedCap UE bandwidth. - FFS: It is always configured if the initial DL BWP for non-RedCap UEs is wider than the maximum RedCap UE bandwidth. - This applies to both TDD and FDD (including FD FDD and HD FDD) cases. - Working assumption: It applies at least after initial access for FR1 when MIB configured CORESET#0 is included Agreement - Send an LS to RAN2 and RAN4 to ask about using NCD-SSB instead of CD-SSB for idle/inactive/connected mode procedures for serving and non-serving cells for a Rel-17 RedCap UE operating with an initial or non-initial DL BWP not containing CD-SSB. - Draft the LS until Tuesday 19th October. - Indicate in the LS that a response is needed before RAN1#107-e. - Indicate in the LS both option 1 and option 2 Agreement - FFS: What specification changes (if any) are needed to support that the network can enable/disable intra-slot PUCCH frequency hopping (FH) within the separate initial UL BWP in the PUCCH resource for HARQ feedback for Msg4/MsgB for RedCap - FFS: Whether any specification changes are needed and desired in order to support multiplexing of non-FH and FH PUCCH transmissions in PUCCH resources. Agreement With below revision, draft R1-2110599 is endorsed in principle - [RAN2/4] whether it is feasible for a RedCap UE to retune to a CD-SSB rather than use an NCD-SSB of larger periodicity - Remove the blue part of questions - [RAN2/4] if CD-SSB is not transmitted in the non-initial DL BWP of RedCap UE, whether it is feasible to transmit periodic CSI-RS for UE to use as an alternative of SSB in the non-initial BWP of RedCap UE or rely on UE performing RF retuning as in measurement gap outside active BWP for BWP without SSB nor CORESET#0 operation Final LS R1-2110600 is endorsed Agreement For FR1, - For TDD, center frequencies are assumed to be the same for the initial DL (FFS: if it does not include CD-SSB and the entire CORESET#0) and UL BWPs used during random access for RedCap UEs. - FFS: For Option 1 and Option 2, whether the case that the center frequencies are different is also supported, and whether RedCap UE can expect CD-SSB and CORESET#0 in this case - For TDD, center frequencies are assumed to be the same for non-initial DL and UL BWPs with the same BWP id for a RedCap UE.

In this contribution, we discuss the remaining issues of bandwidth reduction for Redcap devices according to the RAN1 106bis-e agreements.

# 2. Discussions

## 2.1 UL/DL central frequency alignment for TDD

In the RAN1#106 bis-e, the following was made w.r.t. UL/DL central frequency alignment for TDD:

Agreement For FR1, - For TDD, center frequencies are assumed to be the same for the initial DL (FFS: if it does not include CD-SSB and the entire CORESET#0) and UL BWPs used during random access for RedCap UEs. - FFS: For Option 1 and Option 2, whether the case that the center frequencies are different is also supported, and whether RedCap UE can expect CD-SSB and CORESET#0 in this case - For TDD, center frequencies are assumed to be the same for non-initial DL and UL BWPs with the same BWP id for a RedCap UE.

Table 1 provides a summary in accordance with the agreement made in RAN1 106 bis-e meeting:

Table 1: DL/UL BWPs for Redcap UEs in TDD System

| Index | Description | During random access (RRC_IDLE mode) | After random access (RRC_CONNECTED mode) |
| --- | --- | --- | --- |
| Case 1-1: | Initial DL/UL BWP with Opt.1 Configuration | If it includes CD-SSB and entire CORESET#0, center frequencies are aligned. FFS: it does not include CD-SSB and entire CORESET#0 | FFS |
| Case 1-2 | Initial DL/UL BWP with Opt.2 Configuration | If it includes CD-SSB and entire CORESET#0, center frequencies are aligned. FFS: it does not include CD-SSB and entire CORESET#0 |  |
| Case 3 | Non-initial DL and UL BWPs | - | Center frequencies are aligned |

Per Rel-17 Redcap WID description, it is a core design target for Redcap UEs to ensure a lower complexity and minimize power consumption compared to non-Redcap UEs, including both TDD and FDD mode. Therefore, it is desirable to minimize the number of RF switching/retuning attempts or even avoiding retuning if possible. Support of different centre frequencies for DL and UL BWP in TDD system should not be allowed as it potentially requires a Redcap UE to implement two local oscillators (LOs)/PLLs and therefore increases the UE complexity and cost, which is contradicted with Redcap design target. In addition, it causes frequent RF retuning between DL and UL BWP and results in significantly increased latency (e.g., 3ms for BWP switching as illustrated in FIG.1 assuming 15kHz BWP SCS), huge power consumption and difficult to support latency-sensitive traffic for wearable devices.

For BWPs used in RRC_CONNECTED mode, including both Case 1-1/1-2/2 initial and non-initial DL/UL BWP, the switching can occur very frequent on per slot level e.g., every PDSCH reception and the associated HARQ-ACK feedback. It is therefore critical to keep same central frequencies such that frequent RF retuning and power consumption/complexity are mitigated for Redcap devices.

Proposal 1: For TDD system, NOT support the case that the centre frequencies are different for initial DL/UL BWP that are used by a Redcap UE in RRC_CONNECTED mode (i.e., Case 1-1 and Case 1-2 in Table 1).

For initial DL/UL BWPs during initial access procedure, the RF-retuning latency and power consumption maybe acceptable from UE complexity perspective due to the less frequent operation and relaxed processing time requirement. In one example, initial UL BWP for Redcap UEs maybe located at a CC edge to minimize the PUSCH resource fragmentation. In this case, it may be beneficial from overhead perspective to allow the initial DL BWP for Redcap UE is in the middle of CC to include the CD-SSB and CORESET#0 with different central frequencies with initial UL BWP, as depicted in FIG.1 below. Although RF retuning is needed in this case, it is doable in our opinion due to less frequent events. However, if separate initial DL is configured for Redcap UE without covering CD-SSB as illustrated in the middle of FIG.1 below, the central frequencies should be aligned between initial DL/UL BWPs to avoid excessive RF retuning as NCD-SSB has to be transmitted even central frequencies are not aligned.

Figure 1: Initial DL/UL BWP during RACH procedure

We therefore proposed the following:

Proposal 2: For TDD System,

- If separate initial DL BWP does NOT include CD-SSB, the centre frequencies should be aligned for separate initial DL/UL BWP during random access

- Different central frequencies of separate initial DL/UL BWP during random access can be considered if separate initial DL BWP for Redcap includes CD-SSB and CORESET#0

## 2.2 Initial DL BWP

### 2.2.1 Bandwidth of initial DL BWP

In Rel-15/16, CORESET#0 provided by MIB serves as default initial DL BWP, which has the BW below maximum BW of Redcap UEs. In addition, Rel-15 supports to configure an initial DL BWP such that the entire CC bandwidth can be used by initial DL BWP after RRC connection setup. The initial DL BWP is configured by SIB1 by providing a starting common RB and a set of consecutive RBs with a given numerology. The Rel-15 signaling framework can be used to configure separate initial DL BWP for Redcap. Furthermore, to minimize signaling overhead in SIB1, CORESET#0 can be inferred by Redcap UE as initial DL BWP if the SIB1-configured initial DL BWP has a wider bandwidth than the maximum Redcap UE bandwidth and no smaller initial DL BWP is configured in SIB1 for Redcap.

Proposal 3: Separate or additional initial DL BWP for Redcap UEs can be configured by SIB1, if supported.

Proposal 4: If SIB1-configured initial DL BWP has a wider bandwidth than the maximum Redcap UE bandwidth and additional initial DL for Redcap UEs is not configured, a Redcap UE derives initial DL BWP corresponding to CORESET #0.

### 2.2.2 Signals/Channels in separate initial DL BWP

RAN1#105-e agreed the following working assumption related to separate initial DL BWP:

Working assumption: - At least for TDD, an initial DL BWP for RedCap UEs (which is not expected to exceed the maximum RedCap UE bandwidth) can be optionally configured/defined separately from the initial DL BWP for non-RedCap UEs at least after initial access - FFS the details of the configuration/definition - The configuration for a separately configured initial DL BWP for RedCap UEs is signaled in SIB. - whether to support that separate initial DL BWP for RedCap UEs can include a configuration of CORESET and CSS(s) - whether part of the configuration can be defined instead of signaled - If a separate initial DL BWP for RedCap UEs is configured/defined, this separate initial DL BWP for RedCap UEs can be used at least after initial access (i.e., at least after RRC Setup, RRC Resume, or RRC Reestablishment). - FFS during the initial access - FFS: whether a separately configured initial DL BWP for RedCap UEs needs to contain the entire CORESET #0, and, if not, the Redcap UE behaviour for CORESET #0 monitoring - FFS: supported bandwidths in the separate initial DL BWP - FFS: whether additional SSB is transmitted in the separately configured initial DL BWP for RedCap UEs - FFS: FDD case

In Rel-15/16 NR, the SIB-configured initial DL BWP always includes the CORESET #0 and SSB for PCell and PSCell. For UE-specific BWPs, the CORESET #0 and SSB are required to be included unless UE indicates supporting FG 6-1A, as captured in Table 2.

Table 2: UE feature list for BWP operation [4].

| Index | Feature group | Components | Prerequisite feature groups | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- |
| 6-1 | Basic BWP operation with restriction | 1) 1 UE-specific RRC configured DL BWP per carrier 2) 1 UE-specific RRC configured UL BWP per carrier 3) RRC reconfiguration of any parameters related to BWP 4) BW of a UE-specific RRC configured BWP includes BW of CORESET #0 (if CORESET #0 is present) and SSB for PCell/PSCell (if configured) and BW of the UE-specific RRC configured BWP includes SSB for SCell if there is SSB on SCell |  | This feature should be mandatory without capability signaling for at least BWPs which is the same as the set of specified channel BW UE-specific RRC configured DL/UL BWP can have the same or different numerology from the initial active DL/UL BWP | Mandatory without capability signaling |
| 6-1a | BWP operation without restriction on BW of BWP(s) | BW of UE-specific RRC configured BWP may not include BW of the CORESET #0 (if CORESET #0 is present) and SSB for PCell/PSCell (if configured) and BW of the UE-specific RRC configured BWP may not include SSB for SCell | 6-1, 6-2, 6-3, or 6-4 | 6-1a is applicable to 6-1, 6-2, 6-3, or 6-4. | Optional with capability signaling |

CORESET #0 in separate initial DL BWP and RRC-configured BWP for Redcap

For UE-specific BWP that does not include CORESET #0 (i.e., a UE supports FG 6-1a), NR supports different ways to deliver SIB information

- Scheme #1: a gNB may configure common CORESET for Type0-CSS monitoring as part of UE-specific BWP configuration.

- Scheme #2: gNB may not configure common CORESET and Type0-CSS monitoring. In this case, the network provides system information through dedicated signaling using the RRCReconfiguration message, e.g., if the UE has an active BWP with no common search space configured to monitor SI or paging.

We believe the existing Rel-15 procedure described above should be reused for CORESET #0 configuration in the separate initial DL BWP for Rel-17 Redcap UE. On the other hand, mandating CORESET#0 to be always included in separate Redcap-specific initial DL BWP maybe too restriction for some use cases in TDD. For instance, separate initial DL BWP maybe configured to align its central frequency with an initial UL BWP, which is arranged at the CC edge to mitigate PUSCH resource fragmentation effect. Hence, the separate initial DL BWP for Redcap is allowed to not include COREST #0 to provide flexibility for network in TDD system.

On the other hand, like Rel-15/16 NR UEs, if an active DL BWP, including both separate initial DL BWP and non-initial DL BWP, does not contain the CORESET#0, a Redcap UE in RRC_CONNECTED mode is not required to periodically switch to CORESET #0 for SIB acquisition if it is outside of the active BWP. Instead, the system information should be provided by gNB through dedicated RRC signaling as in Rel-15/16. This is also applied for paging/RAR reception.

Proposal 5: The separate initial DL BWP and RRC-configured DL BWP for Redcap may not include CORESET #0.

Proposal 6: If a Redcap UE in RRC_CONNECTED mode has an active BWP with no Type0/0A/1/2 configured, the UE is not required to monitor these CSS outside of active BWP for SI/RAR/paging acquisition.

  - The system information should be provided by gNB through dedicated RRC signaling as in Rel-15.

Additional SSB in separate initial DL BWP and RRC-configured BWP for Redcap

One of key motivation of supporting a separate initial DL BWP is to align a same central frequency TDD UL/DL BWPs when the UL BWP is placed at the cell-edge to avoid PUSCH resource fragmentation. Figure 2 provides one example where a separate initial DL BWP is pushed to the CC-edge and may not include CD-SSB. whether or not additional SSB should be transmitted in the DL BWP (including both initial and RRC-configured BWPs) was extensively discussed and no progress was made.

Different cases can be considered for SSB transmission in a BWP for Redcap

- Case 1: RRC_CONNECTED Mode

In Rel-15/16, BWP configuration without SSB/COREST#0 (i.e., FG 6-1A) is optionally supported due to additional complexities for intra-frequency RRM measurement, RLM and Time/Frequency tracking. Requesting the Redcap UE to operate in an initial DL BWP without SSB essentially makes FG 6-1A to be mandatory for Redcap UEs, which obviously causes additional power consumption and throughput degradation and goes to the opposite direction of reduced cost and saving power. One concern raised by some companies is the SSB overhead. It should be noted that additional SSB is caused due to the separate initial DL BWP, which offers scheduling flexibility for network to place initial UL BWP to the CC-edge and solve PUSCH resource fragmentation. If the overhead is indeed concern, gNB always have the full freedom to share CD-SSB for Redcap UEs by ensuring the initial DL BWP includes the CD-SSB. In addition, SSB overhead depends on the periodicity configuration and a sparse SSB burst with period larger than that of CD-SSB can be discussed for the separate SSB. As one example, assuming 30kHz SCS, 40ms SSB burst periodicity, 8 SSBs in a burst and half-half split between DL and UL, the SSB overhead is approximately ~0.4% (i.e. (20*12*4*8)/(275*12*14*80*50%)), which should not be a real and concern, especially considering the associated benefit.

Figure 2: Separate Initial DL BWP for Redcap UEs with additional SSB

Proposal 7: Introducing a new UE feature for Redcap to indicate whether it supports an active BWP configured with UE-specific search space (USS) without SSB, denoting as Feature-X

- A UE not supporting Feature-X expects SSB transmission in the active DL BWP configured with USS.

- Case 2: RRC_IDLE and RRC_INACTIVE Mode

In RRC_IDLE and RRC_INACTIVE modes, a UE wakes up periodically according to configured DRX cycle to monitor for paging message from network. If a separate initial DL BWP is configured with paging monitoring for Redcap UE in RRC_IDLE and RRC_INACTIVE modes e.g., for paging offloading purpose, a SSB should also be transmitted within this initial DL BWP to avoid RF retuning for potential paging PDSCH reception.

Proposal 8: For a separate initial DL BWP configured with paging monitoring, a Redcap UE in RRC_IDLE/INACTIVE state expects SSB transmission in the separate initial BWP if the UE monitors for paging on it.

One more aspect needs to be discussed is a separate initial DL BWP without paging monitoring occasion but is configured with Type-1 CSS to achieve RACH offloading purpose. In this case, the RedCap UE is camping and monitoring paging, e.g., in the initial DL BWP that includes CORESET #0 and CD-SSB, same as non-RedCap UEs. Once random-access procedure is triggered by paging message, a Redcap UE may need to switch between initial DL BWP with CD-SSB and initial UL BWP during RACH procedure if there is no additional SSB in the separate initial DL BWP that has same central frequency with initial UL BWP. Given the fact that RACH procedure is a short period and less frequent event, it is acceptable that SSB is not transmitted in this initial DL BWP to reduce system overhead.

Proposal 9: For a separate initial DL BWP configured with Type-1 CSS without paging monitoring, SSB may not be configured for the separate initial BWP

## 2.3 PUCCH Frequency Hopping

The following was agreed in RAN1 106 bis-e meeting regarding PUCCH frequency hopping disabling [3]:

Agreement: Confirm the working assumption: - In case a separate initial UL BWP is configured for RedCap UEs, it is supported that the network can enable/disable intra-slot PUCCH frequency hopping within the separate initial UL BWP in the PUCCH resource for HARQ feedback for Msg4/MsgB for RedCap UEs. - The frequency hopping is enabled/disabled at least via SIB.

One potential issue was pointed out in last RAN1 meeting that the orthogonality between PUCCH resource with FH for non-Redcap UE and PUCCH resource without FH cannot be maintained if they are in a shared PRB to minimize the PUCCH overhead.

In Rel-15, the base sequence of PUCCH format 1 in initial access can be varied across two hops in a slot, which is controlled by a ‘pucch-GroupHopping’ IE in SIB1 with three configurable values (neither, enable, disable). The description of each value for a ‘pucch-GroupHopping’ IE is provided in Table 2. When the value is configured as one of <enable, disable> values, the base sequence <u, v> for two hops of a single PUCCH resource are different in accordance with the sequence generation in clause 6.3.2.2.1 of [5].

Table 2: Base sequence determination for PUCCH format 1

| Value for a ‘pucch-GroupHopping’ IE | Group Hopping | Sequence Hopping | Group index ‘u’ | Sequence index ‘v’ | Comments |
| --- | --- | --- | --- | --- | --- |
| ‘Neither’ | Disable | Disable | Depending on cell ID | 0 | No problem to share a single RB with Redcap UE |
| ‘Enable’ | Enabled | Disable | Depending on cell ID and Hop index nhop | 0 | Different Group index for two hops. |
| ‘Disable’ | Disable | Enable | Depending on cell ID | Depending on Hop index nhop | Different sequence index for two hops. |

To solve this problem and maintain the orthogonality between PUCCH resource with FH for non-Redcap UE and PUCCH resource without FH in a shared PRB, the group and sequence hopping for PUCCH format 1 configured for Redcap UE can be determined based on the value of ‘pucch-GroupHopping’ IE configured for non-Redcap UE if frequency hopping is disabled for it.

Proposal 10: If intra-slot FH is disabled for Redcap and overlapped RBs are used for non-FH and FH PUCCH transmissions, the base sequence group/sequence hopping for PUCCH format 1 is determined based on value of ‘pucch-GroupHopping’ IE configured for non-Redcap UE.

# 3. Conclusion

In this contribution, we have presented our views on the reduced maximum UE bandwidth for Redcap. Based on the discussions, we proposed the following:

Proposal 1: For TDD system, NOT support the case that the centre frequencies are different for initial DL/UL BWP that are used by a Redcap UE in RRC_CONNECTED mode (i.e., Case 1-1 and Case 1-2 in Table 1).

Proposal 2: For TDD System,

- If separate initial DL BWP does NOT include CD-SSB, the centre frequencies should be aligned for separate initial DL/UL BWP during random access

- Different central frequencies of separate initial DL/UL BWP during random access can be considered if separate initial DL BWP for Redcap includes CD-SSB and CORESET#0

Proposal 3: Separate or additional initial DL BWP for Redcap UEs can be configured by SIB1, if supported.

Proposal 4: If SIB1-configured initial DL BWP has a wider bandwidth than

Proposal 5: The separate initial DL BWP and RRC-configured DL BWP for Redcap may not include CORESET #0.

Proposal 6: If a Redcap UE in RRC_CONNECTED mode has an active BWP with no Type0/0A/1/2 configured, the UE is not required to monitor these CSS outside of active BWP for SI/RAR/paging acquisition.

  - The system information should be provided by gNB through dedicated RRC signaling as in Rel-15.

Proposal 7: Introducing a new UE feature for Redcap to indicate whether it supports an active BWP configured with UE-specific search space (USS) without SSB, denoting as Feature-X

- A UE not supporting Feature-X expects SSB transmission in the active DL BWP configured with USS.

Proposal 8: For a separate initial DL BWP configured with paging monitoring, a Redcap UE in RRC_IDLE/INACTIVE state expects SSB transmission in the separate initial BWP if the UE monitors for paging on it.

Proposal 9: For a separate initial DL BWP configured with Type-1 CSS without paging monitoring, SSB may not be configured for the separate initial BWP

Proposal 10: If intra-slot FH is disabled for Redcap and overlapped RBs are used for non-FH and FH PUCCH transmissions, the base sequence group/sequence hopping for PUCCH format 1 is determined based on value of ‘pucch-GroupHopping’ IE configured for non-Redcap UE.

# References

- RP-211574, “Revised WID on support of reduced capability NR devices”, Ericsson, 3GPP TSG RAN #92e, June 2021.

- RAN1 106 e-meeting Chairman notes

- RAN1 106bis e-meeting Chairman notes

- TR 38.822 V15.0.1, “User Equipment (UE) feature list”, July 2019.

[5].	TS 38.211, “5G NR: Physical channels and modulation”