3GPP TSG RAN WG1 #107-e		R1-2111885

e-Meeting, November 11th – 19th, 2021

Agenda Item:	8.7.1.2

Source:	Apple Inc.

Title:	Indication of TRS configurations for idle/inactive-mode UE power saving

Document for:	Discussion/Decision

# Introduction

In this contribution, we discuss the remaining details for the indication of TRS/CSI-RS to idle/inactive-mode UEs [1], including the configuration signaling, the availability signaling, and some overhead reduction enhancements. (Past agreements are included in Appendix B.)

# TRS Configurations

For the TRS configuration signaling, the following has been agreed:

Agreement

Configuration of TRS/CSI-RS occasion(s) for idle/inactive UEs include a list of one or more TRS resource sets, where:

        a TRS resource set can be configured to include

o   a set of TRS resources up to two consecutive slots,

  Note: a TRS resource is same as Rel-15/16, i.e. a CSI-RS in a symbol.

o   at least common configuration parameters:

  a QCL reference

  firstOFDMSymbolInTimeDomain,

  ‘frequencyDomainAllocation for row1’, ‘startingRB’ ,‘nrofRBs’,’powerControlOffsetSS’, periodicityAndOffset’

  FFS

        scramblingID,

        a TRS resource set ID, number of slots {1, 2} or number of symbols {2, 4} if supported

        Note: the ‘TRS resource set’ configuration is not (necessarily) identical to ‘NZP-CSI-RS-ResourceSet’ configuration for TRS in R15/16.

What remains open is how to handle scramblingID and how to indicate different CSI-RS symbols in a TRS resource set.

The field sizes for the TRS resource set parameters are listed in Table 1. The agreed common parameters require 42 bits, while one scramblingID needs 10 bits. Rel-15/16 specifications provide the flexibility to configure scramblingID independently for each CSI-RS symbol in a TRS resource set. However, if we have one scramblingID field per CSI-RS symbol, 40 bits are needed for a TRS resource set with 4 CSI-RS symbols. This takes a significant portion of overall overhead. Technically speaking, it does not provide much benefit for the gNB to configure different scramblingID for different symbols. Therefore, we think a more typical case is for the gNB to configure the same scramblingID for all the symbols within a TRS resource set, and it makes sense to define a common parameter for scramblingID for a TRS resource set to be able to save overhead for typical cases. In addition, to provide full configuration flexibility provided by Rel-15/16, scramblingID can still be provided per CSI-RS resource if needed.

Proposal 1: A common parameter of scramblingID is supported for a TRS resource set. In addition, scramblingID can also be configured per CSI-RS symbol if needed.

As a TRS resource set consists of 2 or 4 symbols in 1 or 2 slots, respectively, it is straightforward to introduce a new parameter for the number of slots, to avoid duplicating the common parameters for different CSI-RS symbols within a TRS resource set.

Proposal 2: For a TRS resource set, introduce a new parameter for the number of slots, which indicates 1 or 2 slots for the TRS configuration.

Table 1 TRS configuration parameters

| TRS configuration parameters | Value Range | Field Size (bits) |
| --- | --- | --- |
| Frequency domain allocation | {0, 1, 2, 3} | 2 |
| firstOFDMSymbolInTimeDomain | 0 to 9 | 4 |
| startingRB | Can reuse the existing range: 0 to 274 | 9 |
| nrofRBs | Can reuse the existing range: 24 to 276 | 9 |
| powerControlOffsetSS | Reuse the existing range: {-3, 0, 3, 6} dB | 2 |
| periodicityAndOffset | Can reuse the existing structure of CSI-ResourcePeriodicityAndOffset, with periodicity limited to {10, 20, 40, 80} ms. | 10 |
| QCL information | 0 to 63, indicating the QCLed SSB index | 6 |
| FFS: Number of slots for TRS | {1, 2} | 1 |
| FFS: scramblingID | Reuse the existing range: 0 to 1023 | 10 |
| FFS: TRS resource set ID | 0 to (max # of TRS resource set – 1) |  |

Assuming the most optimistic case with one scramblingID per TRS resource set, each TRS resource set takes 53 bits (ignoring any additional overhead in the signaling), and 64 TRS resource sets (one per beam for 64 beams) takes 53 * 64 = 3392 bits. However, the current limit for the SI message size is 2976 bits, as defined in TS 38.214: “The UE is not expected to receive a PDSCH assigned by a PDCCH with CRC scrambled by SI-RNTI with a TBS exceeding 2976 bits.” Moreover, SIB segmentation is not supported at the moment. RAN2 is considering the possibility of supporting SIB segmentation.

If we want to further reduce the signaling overhead and/or fit it into a SIB without segmentation, more optimization can be done. For example, it may make sense to support the configuration of some common parameters for some or all TRS resource set configurations. One good candidate is the RB allocation (starting RB and nrofRBs), which is more likely to be the same across the TRS resource sets and it also brings significant overhead saving given its size.

Proposal 3: Further consider introducing common parameters (e.g., starting RB and nrofRBs) for multiple/all TRS resource sets to reduce the signaling overhead.

# Availability indication of TRS occasion(s)

In terms of what is used to carry the availability indication of TRS occasion(s), the following has been agreed in RAN1#105-e:

Agreement:

Confirm the following working assumption:

Support at least L1 based signaling for the availability indication of TRS/CSI-RS at the configured occasion(s) to the idle/inactive UEs.

- FFS details, including paging DCI and/or PEI for L1 based signaling

- FFS SIB-based signaling/configuration

  - Note: It is RAN1 understanding that existing SI update procedure is used for SIB based signalling

Working assumption:

Support paging PDCCH based availability indication of TRS/CSI-RS occasions for idle/inactive UEs.

Support PEI based availability indication of TRS/CSI-RS occasions for idle/inactive UEs at least if PDCCH-based PEI is down-selected.

- FFS whether and how to enable/disable L1 based availability indication configurable by SIB

In this section, we discuss the details of the availability indication in paging PDCCH and PEI.

## Availability indication

In terms of the exact content of the availability indication, the following has been agreed in RAN1#104b-e/105-e/106-e/106b-e:

Agreement:

For the information provided by a physical layer availability indication of TRS/CSI-RS at the configured occasion(s) to the idle/inactive UEs, one or more alternatives from the following can be supported:

- Alt1: Availability/unavailability information for all or some of configured RS resources using a bitmap or codepoint

- e.g. using bitmap, where each bit from a bitmap or a codepoint is associated with at least one resource/configuration or a set/group of resources

- e.g. a codepoint to indicate a state of availability/unavailability for all or some of configured RS resources

- Alt2: value or codepoint to indicate one or more resource/configuration indices that correspond to the available RS resources

- FFS whether and how to indicate the ‘availability’ in beam selective manner.

- Other alternatives are not precluded

Agreement:

For the information provided by a physical layer availability indication of TRS/CSI-RS at the configured occasion(s) to the idle/inactive UEs, support availability/unavailability information for configured RS resources using a bitmap or codepoint

- e.g. using bitmap, where each bit is associated with at least one resource/configuration or a set/group of resources

- e.g. a codepoint to indicate a state of availability/unavailability for all or some of configured RS resources

- FFS maximum number of configured RS resources per physical layer availability indication to support.

- FFS whether availability/unavailability information is for all or some of configured RS resources

Agreement

Support at least one of the following alternatives

- Alt1: L1 availability indication at an occasion provides availability/unavailability information only for RS resources with the same QCL reference as the L1 availability indication occasion.

- Alt2: L1 availability indication at an occasion can provide availability/unavailability information for RS resources with QCL references not confined to be the same as for the L1 availability indication occasion

Note:  The occasion mentioned above refers to a signal/channel monitoring occasion (e.g. a paging PDCCH or PEI monitoring occasion) to provide the L1 availability indication.

Note: a RS resource is a RS from configured TRS/CSI-RS occasion(s) for idle/inactive UEs., where the configuration for TRS/CSI-RS occasion(s) for idle/inactive UEs is based on periodic TRS only.

Agreement

For L1 based availability indication of TRS/CSI-RS at the configured occasion(s) to the idle/inactive UEs, support availability information for configured RS resources using a bitmap. where each bit indicates whether associated TRS resource(s) are available.

- support L1 availability indication at an occasion can provide availability information RS resources with QCL references not confined to be the same as for the L1 availability indication occasion

  - FFS associated TRS resource(s) per bit, e.g. a bit is associated with a TRS resource set

  - Bitmap size is up to X bits

    - X = [6] for paging PDCCH based L1 availability indication.

    - FFS X for PEI DCI based L1 availability indication

    - FFS details about how to configure the DCI field: e.g. start and length of bitmap (e.g. explicitly/implicitly configured)

- for paging PDCCH based L1 availability indication, support L1 availability indication at an occasion can provide availability information for all configured RS resources

  - FFS whether this needs to be supported regardless of the number of beams or for some configured RS resources

- FFS: PEI DCI provides L1 availability indication information only for RS resources with QCL references to be the same as for the L1 availability indication occasion

- FFS: indication of unavailability

For paging PDCCH based approach, we have agreed to support L1 availability indication for all configured RS resources. This is especially suitable when the number of TRS resource set configurations is small, for which one bit per TRS resource set can be used. Since there are 6 reserved bits available in paging DCI, this can be used to support up to 6 TRS resource sets.

Proposal 4: For paging PDCCH based availability indication, support bitmap-based indication for all the configured TRS resource sets, with one bit per TRS resource set, when there are up to 6 TRS resource sets configured.

However, the overhead becomes an issue if there are a large number of beams/TRS resource sets. For example, if there are a total of 64 TRS configurations and one bit per TRS configuration is used for availability indication, we need 64 bits, which is certainly too large for a PDCCH.

There are mainly two options to reduce the overhead:

- Option 1: one bit is used to indicate a group of TRS resource sets

  - There are different flavors, e.g. how the groups are defined, whether one DCI should carry the indication for all the TRS resource sets or whether different beams can carry the indication for different TRS resource sets. All these details need to be defined.

- Option 2: the DCI only carries the availability information for the TRS resource sets that correspond to the same beam as the DCI.

  - One bit per TRS resource set should be sufficient in this case.

One concern that has been raised for the availability indication in paging PDCCH is that the indication is only useful for future paging cycles but the UE does not exactly know the strongest beam when waking up in the future paging cycle. In case TRS is available for some beams but not some other beams (or there is no availability information for some beams), the UE will most likely need to still wake up in advance to handle the worst case scenario (i.e. assuming no TRS is available). But it is a common issue for Option 1 and Option 2. The difference only exists in the case when the TRS resource sets in all the beams are indicated as available in Option 1.

Another difference between the two options is that Option 1 provides the indication for more beams, and if the UE wakes up in a different beam, it may still have the indication for that beam, while such information is not available with Option 2. However, the probability of changing the beams should not be large given a typical paging cycle of 1.28 seconds.

But the drawback of Option 1 compared to Option 2 is that the granularity of the indication is more coarse. E.g. if one bit corresponds to 6 TRS resource sets, it can be indicated as available only if all the 6 TRS resource sets are available. Generally speaking, there is no strong correlation between the TRS availability in different beams, so there is no good rule for gNB to do the grouping. This greatly reduces the probability that TRS can be used by the UE.

Whether Option 1 or Option 2 is more efficient/better depends on the particular cases. If there are a small number of TRS resource sets per group, Option 1 may be better; otherwise Option 2 may be better.

For simplicity, we propose Option 2.

Proposal 5: For paging PDCCH based availability indication, support bitmap-based indication for only the configured TRS resource sets that correspond to the same beam as the paging PDCCH, with one bit per TRS resource set.

The consideration for PEI-based availability indication is different compared to PDCCH-based availability indication. PEI should be configured in advance in time compared to the PO, to allow sufficient time for fine synchronization if there is paging. When PEI is received, the UE already knows the beam for PO reception, and only the TRS availability indication for this particular beam is needed, which can be used for the reception of the current PO. In addition, there is no need to indicate TRS availability for future paging cycle, because another PEI with availability indication will be received by the UE before decoding the PO.

Proposal 6: For PEI based availability indication, support bitmap-based indication for only the configured TRS resource sets that correspond to the same beam as the paging PDCCH, with one bit per TRS resource set.

## Validity time for availability indication

Regarding how long the availability indication stays valid, the following alternatives have been considered:

Agreement

L1 based availability indication of TRS/CSI-RS at the configured occasion(s) to the idle/inactive UEs is valid for a time duration starting from a reference point, where

- the time duration can be determined based on at least one from the following (to be down-selected):

  - Alt-1: configured by higher layer

  - Alt-2: a predefined/configured window

  - Alt-3: value indicated by the availability indication, where the value is one of multiple configured time duration(s)

  - Alt-4: until when the UE receives another availability indication

  - A combination of alternatives or other alternatives is not precluded.

- the reference point can be determined as at least one from the following (to be down-selected):

  - Alt-1: start of next PO or DRX cycle

  - Alt-2: time location where UE receives the indication

    - Note: the time location is subject to application delay if agreed

  - Alt-3: start of current PO or DRX cycle where UE receive the indication

  - Alt-4: a time location which is configured by higher layer

  - A combination of alternatives or other alternatives is not precluded.

Agreement

At least for paging PDCCH based L1 availability indication of TRS/CSI-RS at the configured occasion(s) to the idle/inactive UEs, the L1 availability indication is valid for a time duration starting from a reference point, where

- the time duration is a validity duration configured by higher layer,

  - FFS applicable values, e.g. # of DRX cycles, or multiple of default paging cycle duration (i.e. modification period)

  - FFS UE doesn’t expect inconsistent L1 based indication during the time duration

- the reference point for start of the validity duration is one of the following alternatives:

  - Alt1: SFN of the first PF from the next DRX cycle

  - Alt2: SFN of the first PF from the current DRX cycle where UE receives the indication

  - Alt3: based on SFN configured by higher layer, i.e. modification period configured as multiple of default paging cycle duration

  - Alt4: start of the PF for the PO where UE receives the indication

  - Note: the DRX cycle in Alt1 and Alt2 is the default paging cycle broadcast in SIB

  - Note: The SFN for the first PF is for (UE mod N) = 0, and can be calculated by (SFN + PF_offset) mod T = 0

- the time duration can be optionally configured by gNB

  - when the time duration is not configured, one of the following alternatives can be considered:

    - Alt1: the availability indication is valid until when the UE receives another availability indication.

    - Alt2: the availability indication is valid until L1 availability indication is changed by network

    - Alt3: default time duration e.g. default paging cycle

- FFS whether and how to handle the miss detection issue of L1 signaling

For paging PDCCH based availability indication, the UE can only use the TRS for the next paging cycle, therefore it is important that the availability indication is valid before the next PO. For this reason, we prefer Alt 4 among the alternatives for the reference point. It is not considered as essential that a common modification period is assumed for all the UEs.

On the other hand, it is important that all the UEs in the same PO, receiving the same availability indication, assume the same validity time duration. In this sense, the default paging cycle duration can be used as the unit for the validity duration.

Proposal 7: For paging PDCCH based availability indication of TRS occasions, the validity time duration is configured as multiple of default paging cycle. The reference point for the validity time duration is start of the PF for the PO where UE receives the indication (Alt 4).

For PEI based availability indication of TRS occasions, it can be used to indicate the availability in the current paging cycle. Given that the PEI should have payload reserved for the TRS availability information, whenever the UE needs to wake up to decode paging PDCCH/PDSCH, there is TRS availability information in the corresponding PEI PDCCH. Therefore, it is not necessary for PEI to indicate the availability for multiple upcoming paging cycles.

Proposal 8: For PEI based availability indication of TRS occasions, the availability indication is valid until the end of the current PO, starting from when the UE receives the indication.

# Overhead Reduction

If the TRS indicated to idle/inactive UEs is used by the connected UEs, there is no additional overhead to transmit TRS from network perspective. However, if the TRS is no longer used by the connected UEs, it may take some time for the gNB to notify all the idle/inactive UEs about the availability change. Before the gNB notifies all the UEs about the changes, the TRS transmission is additional overhead. In some other cases, the gNB may choose to provide TRS to idle/inactive UEs even when it is not used by the connected UEs. For these cases, it is also important to minimize the TRS overhead.

One way to reduce the TRS overhead is to limit the TRS occasion(s) that is assumed by the idle/inactive UEs to be available. As shown in Fig. 1, if a TRS configuration is indicated as available, the UE may assume that only the N TRS occasions before the PO are available, instead of all the TRS occasions. Given that the idle/inactive UEs only need the TRS before the PO, having such restrictions should not affect anything for these UEs.

So instead of transmitting in all the TRS occasions, the gNB can choose to transmit only the TRS occasions that are assumed to be available by the idle/inactive UEs.

Proposal 9: When a TRS configuration is indicated as available, the idle/inactive UEs assumes that only a certain number of TRS occasion(s) before a PO is available, with the number being configurable.

Figure 1 Potential TRS overhead reduction

# Conclusion

In this contribution, we discussed the remaining issues for TRS indication to idle/inactive UEs and made the following proposals:

Proposal 1: A common parameter of scramblingID is supported for a TRS resource set. In addition, scramblingID can also be configured per CSI-RS symbol if needed.

Proposal 2: For a TRS resource set, introduce a new parameter for the number of slots, which indicates 1 or 2 slots for the TRS configuration.

Proposal 3: Further consider introducing common parameters (e.g., starting RB and nrofRBs) for multiple/all TRS resource sets to reduce the signaling overhead.

Proposal 4: For paging PDCCH based availability indication, support bitmap-based indication for all the configured TRS resource sets, with one bit per TRS resource set, when there are up to 6 TRS resource sets configured.

Proposal 5: For paging PDCCH based availability indication, support bitmap-based indication for only the configured TRS resource sets that correspond to the same beam as the paging PDCCH, with one bit per TRS resource set.

Proposal 6: For PEI based availability indication, support bitmap-based indication for only the configured TRS resource sets that correspond to the same beam as the paging PDCCH, with one bit per TRS resource set.

Proposal 7: For paging PDCCH based availability indication of TRS occasions, the validity time duration is configured as multiple of default paging cycle. The reference point for the validity time duration is start of the PF for the PO where UE receives the indication (Alt 4).

Proposal 8: For PEI based availability indication of TRS occasions, the availability indication is valid until the end of the current PO, starting from when the UE receives the indication.

Proposal 9: When a TRS configuration is indicated as available, the idle/inactive UEs assumes that only a certain number of TRS occasion(s) before a PO is available, with the number being configurable.

# Reference

- RP-200938, Revised WID UE Power Saving Enhancements for NR, MediaTek Inc., RAN#88e, June 2020.

# Appendix A: NZP-CSI-RS-ResourceSet and NZP-CSI-RS-Resource

NZP-CSI-RS-ResourceSet ::= SEQUENCE { nzp-CSI-ResourceSetId NZP-CSI-RS-ResourceSetId, nzp-CSI-RS-Resources SEQUENCE (SIZE (1..maxNrofNZP-CSI-RS-ResourcesPerSet)) OF NZP-CSI-RS-ResourceId, repetition ENUMERATED { on, off } OPTIONAL, -- Need S aperiodicTriggeringOffset INTEGER(0..6) OPTIONAL, -- Need S trs-Info ENUMERATED {true} OPTIONAL, -- Need R ..., [[ aperiodicTriggeringOffset-r16 INTEGER(0..31) OPTIONAL -- Need S ]] } NZP-CSI-RS-Resource ::= SEQUENCE { nzp-CSI-RS-ResourceId NZP-CSI-RS-ResourceId, resourceMapping CSI-RS-ResourceMapping, powerControlOffset INTEGER (-8..15), powerControlOffsetSS ENUMERATED{db-3, db0, db3, db6} OPTIONAL, -- Need R scramblingID ScramblingId, periodicityAndOffset CSI-ResourcePeriodicityAndOffset OPTIONAL, -- Cond PeriodicOrSemiPersistent qcl-InfoPeriodicCSI-RS TCI-StateId OPTIONAL, -- Cond Periodic ... } CSI-RS-ResourceMapping ::= SEQUENCE { frequencyDomainAllocation CHOICE { row1 BIT STRING (SIZE (4)), row2 BIT STRING (SIZE (12)), row4 BIT STRING (SIZE (3)), other BIT STRING (SIZE (6)) }, nrofPorts ENUMERATED {p1,p2,p4,p8,p12,p16,p24,p32}, firstOFDMSymbolInTimeDomain INTEGER (0..13), firstOFDMSymbolInTimeDomain2 INTEGER (2..12) OPTIONAL, -- Need R cdm-Type ENUMERATED {noCDM, fd-CDM2, cdm4-FD2-TD2, cdm8-FD2-TD4}, density CHOICE { dot5 ENUMERATED {evenPRBs, oddPRBs}, one NULL, three NULL, spare NULL }, freqBand CSI-FrequencyOccupation, ... } CSI-FrequencyOccupation ::= SEQUENCE { startingRB INTEGER (0..maxNrofPhysicalResourceBlocks-1), nrofRBs INTEGER (24..maxNrofPhysicalResourceBlocksPlus1), ... } maxNrofPhysicalResourceBlocks = 275

# Appendix B: Related Agreements

## RAN1#102-e

Agreements:

- New types/patterns of TRS/CSI-RS are not introduced specifically for idle/inactive mode UE.

Agreements:

The TRS/CSI-RS occasion(s) that may be for connected mode UEs can be shared to idle/inactive mode UEs.

-  Note: It is understood that gNB can potentially share the occasions to idle/inactive (which would just mean it up to NW whether to share or not share).

-  Note: It is understood that TRS/CSI-RS in the TRS/CSI-RS occasion(s) may or may not be transmitted.

-  Note: Always-on TRS/CSI-RS transmission by gNodeB is not required

-  At least TRS/CSI-RS occasion(s) corresponding to periodic TRS is supported

- FFS for other RS types

-  FFS: Whether UE blind detection is required or not.

Agreements:

Idle/inactive UE may use the TRS/CSI-RS occasion(s) that are shared to it for functionalities such as:

-           AGC, time/frequency tracking

-           FFS: RRM measurement for serving cell, RRM measurement for neighbor cell, paging reception indication

Observation:

It is up to gNB implementation whether or not to transmit a TRS/CSI-RS to idle/inactive UEs even when the TRS/CSI-RS is not needed by connected UEs (e.g., when there is a connected mode UE in a cell but the UE is no longer using the TRS/CSI-RS, or when there is no longer connected mode UE in a cell, etc.)

Agreements:

The configuration of TRS/CSI-RS occasion(s) for idle/inactive mode UE(s) is provided by higher layer signalling

-           FFS higher layer signalling candidates (e.g., SIB, dedicated RRC, RRC release message, etc.)

-           FFS for other signalling candidates (e.g., pre-configuration, etc.)

-           FFS for detailed configuration parameters (e.g., whether and how to reduce the signalling overhead for configuration, etc.)

Agreements:

Further study whether and how to inform the availability of TRS/CSI-RS to idle/inactive mode UE (implicitly or explicitly).

- Note: Availability corresponds to the information for whether TRS/CSI-RS is actually transmitted or not.

## RAN1#103-e

Agreements:

- Functionality of RRM measurement for neighbour cell is not supported for TRS/CSI-RS for idle/inactive UE(s).

Agreements:

- SIB signalling provides the configuration of TRS/CSI-RS occasion(s) for idle/inactive UE(s).

  - Up to RAN2 to decide which SIB is to be used.

  - Whether or not to additionally support other high-layer signalling methods (e.g., dedicated RRC, RRC release message, etc.) is up to RAN2

Send an LS to RAN2 informing the above agreements, and

- To further add that RAN1 is working on the detailed physical layer design

Draft LS is endorsed, with final LS in R1-2009791.

Agreements:

- Aperiodic TRS and semi-persistent/aperiodic CSI-RS are not used as TRS/CSI-RS occasion(s) for idle/inactive UEs.

Agreements:

- Target sending an LS to RAN2 and RAN4 to ask whether it is feasible to allow a UE to use the potential TRS/CSI-RS occasion to enhance the SSB based IDLE/Inactive mode evaluations of the serving cell. (to also include agreements from last meeting)

* Further discussion whether any additional information needs to be included in the LS or not, including potential re-wording of the leading sentence

Agreements:

- Discuss further based on the following alternatives and down-select at RAN1#104-e:

  - Alt 1: The availability of TRS/CSI-RS at the configured occasion(s) is NOT informed to the UE.

  - Alt 2: The availability of TRS/CSI-RS at the configured occasion(s) is informed to the UE.

  - Alt 3. The conditional availability of TRS/CSI-RS at the configured occasion(s) is informed to the UE.

    - The condition can be, e.g., existence of paging.

  - Alt 4. Combination of the above alternatives.

  - FFS for details

  - FFS for UE behavior when the availability is not informed.

  - Other techniques are not precluded.

  - Companies encourage to provide sufficient information for the proposal, e.g.,

    - how to achieve power saving gain

    - how to minimize impact on NW

    - how to minimize extra UE implementation complexity

    - feasibility check on sharing the TRS/CSI-RS between connected UEs and idle/inactive UEs

  - Proposals should be consistent with the WID objective.

Conclusion:

- TRS/CSI-RS based PEI is discussed in AI 8.7.1.1.

- PEI functionality is not further discussed under AI 8.7.1.2.

- Note: This does not prevent to potentially use PEI to carry the indication for TRS/CSI-RS presence.

## RAN1#104-e

Agreements:

Configuration of TRS/CSI-RS occasion(s) for idle/inactive Ues include at least:

- powerControlOffsetSS,

- scramblingID

- firstOFDMSymbolInTimeDomain,

- startingRB.

- nrofRBs,

- FFS other parameters

- FFS applicable values

Agreements:

The SCS configuration of TRS/CSI-RS occasion(s) for idle/inactive UEs can be discussed and down-selected from following alternatives at RAN1#104b-eRAN1#105-e:

- Alt1: same as initial BWP

- Alt2: configurable parameter

Agreements:

Multiple RS resources can be configured for TRS/CSI-RS occasion(s) for idle/inactive UEs.

- FFS details (including whether or not to restrict the RS to be TRS only)

Agreements:

For a cell with TRS/CSI-RS occasions configured for IDLE/Inactive UEs, IDLE/Inactive UE’s assumption on the availability of TRS/CSI-RS at the configured occasion(s) is informed to the idle/inactive UE based on explicit indication.

- FFS details (e.g., the signalling, detailed information for the TRS/CSI-RS, etc.)

- There is no intended blind detection of the presence/absence of TRS/CSI-RS at the UE side in this feature. That is, the UE assumes TRS/CSI-RS is not present if the network does not indicate it is available (or indicates it is unavailable).

Conclusion

From RAN1 perspective, there is no consensus on supporting RRM measurement for serving cell functionality for TRS/CSI-RS occasion(s) for idles/inactive UEs.

Agreements:

The configuration of the frequency location of TRS/CSI-RS occasion(s) for idle/inactive UEs are discussed and down-selected from following alternatives at RAN1#104bis-e:

- Alt-1: within initial DL BWP

- Alt-2: is not restricted by initial BWP

  - IDLE/INACTIVE mode UE is not expected to receive TRS/CSI-RS outside the initial DL BWP.

Agreements:

To study QCL information of TRS/CSI-RS occasion(s) for idle/inactive UEs from following alternatives:

- Alt-1: TCI state from higher layer configuration, e.g. qcl-InfoPeriodicCSI-RS

- Alt-2: QCL assumptions associated with transmitted SSBs implicitly, e.g. similar to PDCCH monitoring in PO

  - FFS details

- FFS details

- Other alternatives are not precluded

Conclusion:

Decide at RAN1#104b-e, whether or not to support periodic CSI-RS in addition to periodic TRS for TRS/CSI-RS occasion(s) for idle/inactive UEs.

## RAN1#104bis-e

Agreement:

SCS of TRS/CSI-RS occasion(s) for idle/inactive UEs is same as SCS of CORESET#0.

Agreement:

Support higher layer configuration of the QCL information of TRS/CSI-RS occasion(s) for idle/inactive UEs.

- FFS details of the QCL information, e.g. associated SSB index

Agreement:

IDLE/INACTIVE mode UE is not expected to receive TRS/CSI-RS outside the initial DL BWP.

- Configuration of the frequency location of TRS/CSI-RS occasion(s) for idle/inactive UEs is not restricted by initial BWP.

Working assumption:

Support at least L1 based signaling for the availability indication of TRS/CSI-RS at the configured occasion(s) to the idle/inactive UEs.

- FFS details, including paging DCI and/or PEI for L1 based signaling

- FFS SIB-based signaling/configuration

  - Note: It is RAN1 understanding that existing SI update procedure is used for SIB based signaling

Agreement:

Configuration for TRS/CSI-RS occasion(s) for idle/inactive UEs is based on periodic TRS only, including following limitations

- Configuration parameters that are necessary to provide configuration of periodic TRS for idle/inactive UEs

- Applicable values that are necessary to provide configuration of periodic TRS for idle/inactive UEs

- If the configuration is provided, idle/inactive UEs can always implicitly assume that trs-info is configured.

  - The parameter trs-info does not need to be provided in the configuration

Agreement:

For the information provided by a physical layer availability indication of TRS/CSI-RS at the configured occasion(s) to the idle/inactive UEs, one or more alternatives from the following can be supported:

- Alt1: Availability/unavailability information for all or some of configured RS resources using a bitmap or codepoint

- e.g. using bitmap, where each bit from a bitmap or a codepoint is associated with at least one resource/configuration or a set/group of resources

- e.g. a codepoint to indicate a state of availability/unavailability for all or some of configured RS resources

- Alt2: value or codepoint to indicate one or more resource/configuration indices that correspond to the available RS resources

- FFS whether and how to indicate the ‘availability’ in beam selective manner.

- Other alternatives are not precluded

## RAN1#105-e

Agreement:

Confirm the following working assumption:

Support at least L1 based signaling for the availability indication of TRS/CSI-RS at the configured occasion(s) to the idle/inactive UEs.

- FFS details, including paging DCI and/or PEI for L1 based signaling

- FFS SIB-based signaling/configuration

  - Note: It is RAN1 understanding that existing SI update procedure is used for SIB based signalling

Agreement:

For the information provided by a physical layer availability indication of TRS/CSI-RS at the configured occasion(s) to the idle/inactive UEs, support availability/unavailability information for configured RS resources using a bitmap or codepoint

- e.g. using bitmap, where each bit is associated with at least one resource/configuration or a set/group of resources

- e.g. a codepoint to indicate a state of availability/unavailability for all or some of configured RS resources

- FFS maximum number of configured RS resources per physical layer availability indication to support.

- FFS whether availability/unavailability information is for all or some of configured RS resources

Agreement:

Support applicable values for the following configuration parameters as below.

- powerControlOffsetSS: {-3, 0, 3, 6}dB

- scramblingID: 0 to 1023

- firstOFDMSymbolInTimeDomain: 0 to 9

  - firstOFDMSymbolInTimeDomain indicates first symbol in a slot, a second symbol in the same slot can be derived implicitly with symbol index as firstOFDMSymbolInTimeDomain+4

- startingRB: 0 to 274

- nrofRBs: 24 to 276

Agreement:

The QCL information of TRS/CSI-RS occasion(s) for idle/inactive UEs is indicated as a SSB index in range of 0 to 63.

- FFS: how the QCL information can be configured, e.g. per RS resource set or per configuration

- FFS: QCL type, which is predetermined

Working assumption:

Support paging PDCCH based availability indication of TRS/CSI-RS occasions for idle/inactive UEs.

Support PEI based availability indication of TRS/CSI-RS occasions for idle/inactive UEs at least if PDCCH-based PEI is down-selected.

- FFS whether and how to enable/disable L1 based availability indication configurable by SIB

Agreement:

Configuration of TRS/CSI-RS occasion(s) for idle/inactive UEs include:

- periodicityAndOffset {10, 20, 40, 80} ms

- frequencyDomainAllocation for row1 with applicable values from {0, 1, 2, 3} to indicate the offset of the first RE to RE#0 in a RB

- FFS Configuration index

  - details,

    - E.g. Per resource or resource set or group of resource sets

    - E.g. explicit or implicit indication based on QCL source

Agreement:

Further study supporting SIB based signaling for availability information of TRS/CSI-RS occasions for idle/inactive UEs at least based on the presence/absence of the configuration of the TRS/CSI-RS occasion in SIB_X in case L1 based availability indication is not configured.

- FFS whether and how SIB based signaling and L1 based signaling can be configured simultaneously

## RAN1#106-e

Agreement

Support at least one of the following alternatives

- Alt1: L1 availability indication at an occasion provides availability/unavailability information only for RS resources with the same QCL reference as the L1 availability indication occasion.

- Alt2: L1 availability indication at an occasion can provide availability/unavailability information for RS resources with QCL references not confined to be the same as for the L1 availability indication occasion

Note:  The occasion mentioned above refers to a signal/channel monitoring occasion (e.g. a paging PDCCH or PEI monitoring occasion) to provide the L1 availability indication.

Note: a RS resource is a RS from configured TRS/CSI-RS occasion(s) for idle/inactive UEs., where the configuration for TRS/CSI-RS occasion(s) for idle/inactive UEs is based on periodic TRS only.

Agreement

L1 based availability indication of TRS/CSI-RS at the configured occasion(s) to the idle/inactive UEs is valid for a time duration starting from a reference point, where

- the time duration can be determined based on at least one from the following (to be down-selected):

  - Alt-1: configured by higher layer

  - Alt-2: a predefined/configured window

  - Alt-3: value indicated by the availability indication, where the value is one of multiple configured time duration(s)

  - Alt-4: until when the UE receives another availability indication

  - A combination of alternatives or other alternatives is not precluded.

- the reference point can be determined as at least one from the following (to be down-selected):

  - Alt-1: start of next PO or DRX cycle

  - Alt-2: time location where UE receives the indication

    - Note: the time location is subject to application delay if agreed

  - Alt-3: start of current PO or DRX cycle where UE receive the indication

  - Alt-4: a time location which is configured by higher layer

  - A combination of alternatives or other alternatives is not precluded.

Agreement

For a RS resource configured for TRS/CSI-RS occasion(s) for idle/inactive UEs, a quasi co-location type can be determined as

  - ‘typeC’ with an SS/PBCH block and, when applicable, ‘typeD’ with the same SS/PBCH block

## RAN1#106bis-e

Conclusion

No consensus to support SIB based signaling for availability information of TRS/CSI-RS occasions for idle/inactive UEs

Working Assumption

If TRS resource is configured in SIB, L1 based availability indication is always enabled based on the configuration.

Agreement

For L1 based availability indication of TRS/CSI-RS at the configured occasion(s) to the idle/inactive UEs, support availability information for configured RS resources using a bitmap. where each bit indicates whether associated TRS resource(s) are available.

- support L1 availability indication at an occasion can provide availability information RS resources with QCL references not confined to be the same as for the L1 availability indication occasion

  - FFS associated TRS resource(s) per bit, e.g. a bit is associated with a TRS resource set

  - Bitmap size is up to X bits

    - X = [6] for paging PDCCH based L1 availability indication.

    - FFS X for PEI DCI based L1 availability indication

    - FFS details about how to configure the DCI field: e.g. start and length of bitmap (e.g. explicitly/implicitly configured)

- for paging PDCCH based L1 availability indication, support L1 availability indication at an occasion can provide availability information for all configured RS resources

  - FFS whether this needs to be supported regardless of the number of beams or for some configured RS resources

- FFS: PEI DCI provides L1 availability indication information only for RS resources with QCL references to be the same as for the L1 availability indication occasion

- FFS: indication of unavailability

Agreement

At least for paging PDCCH based L1 availability indication of TRS/CSI-RS at the configured occasion(s) to the idle/inactive UEs, the L1 availability indication is valid for a time duration starting from a reference point, where

- the time duration is a validity duration configured by higher layer,

  - FFS applicable values, e.g. # of DRX cycles, or multiple of default paging cycle duration (i.e. modification period)

  - FFS UE doesn’t expect inconsistent L1 based indication during the time duration

- the reference point for start of the validity duration is one of the following alternatives:

  - Alt1: SFN of the first PF from the next DRX cycle

  - Alt2: SFN of the first PF from the current DRX cycle where UE receives the indication

  - Alt3: based on SFN configured by higher layer, i.e. modification period configured as multiple of default paging cycle duration

  - Alt4: start of the PF for the PO where UE receives the indication

  - Note: the DRX cycle in Alt1 and Alt2 is the default paging cycle broadcast in SIB

  - Note: The SFN for the first PF is for (UE mod N) = 0, and can be calculated by (SFN + PF_offset) mod T = 0

- the time duration can be optionally configured by gNB

  - when the time duration is not configured, one of the following alternatives can be considered:

    - Alt1: the availability indication is valid until when the UE receives another availability indication.

    - Alt2: the availability indication is valid until L1 availability indication is changed by network

    - Alt3: default time duration e.g. default paging cycle

- FFS whether and how to handle the miss detection issue of L1 signaling

Agreement

Configuration of TRS/CSI-RS occasion(s) for idle/inactive UEs include a list of one or more TRS resource sets, where:

        a TRS resource set can be configured to include

o   a set of TRS resources up to two consecutive slots,

  Note: a TRS resource is same as Rel-15/16, i.e. a CSI-RS in a symbol.

o   at least common configuration parameters:

  a QCL reference

  firstOFDMSymbolInTimeDomain,

  ‘frequencyDomainAllocation for row1’, ‘startingRB’ ,‘nrofRBs’,’powerControlOffsetSS’, periodicityAndOffset’

  FFS

        scramblingID,

        a TRS resource set ID, number of slots {1, 2} or number of symbols {2, 4} if supported

        Note: the ‘TRS resource set’ configuration is not (necessarily) identical to ‘NZP-CSI-RS-ResourceSet’ configuration for TRS in R15/16.