3GPP TSG RAN WG1 #107-e	                                         	      R1-2111906

e-Meeting, November 11th – 19th, 2021

Agenda item:	8.16.1

Source: 	Apple Inc.

Title: 	Views on Rel-17 FeMIMO UE features

Document for:	Discussion/Decision

- Views on Rel-17 FeMIMO UE features

Based on the updated UE feature list for Rel-17 NR provided by the moderators in [1], we provided our views on the Rel-17 FeMIMO UE features as below

- Enhancement on Multi-beam operation (FG23-1x)

  - FG23-1-1: We think this should be a general FG on R17 unified TCI state, which is a per UE FG.

    - We propose to introduce FG23-1-1a for beam indication scheme (TCI or MAC CE) as well as the action delay (Y symbols) based on the previous agreement, this can be a per band FG, since number of active TCI states could be different for different bands. If UE only supports 1 active TCI state, it is not necessary to support DCI.

    - We propose to introduce FG23-1-1b for the number of TCI states related UE capability for separate TCI indication, addressing both UE memory and complexity.

    - We propose to introduce FG23-1-1c for the number of TCI states related UE capability for joint TCI indication, addressing both UE memory and complexity.

    - We propose to introduce FG23-1-1d for beam misalignment for power control based on the previous agreement, and the association between power control setting and TCI should be included in this FG.

    - We propose to introduce FG23-1-1e for the maximum number of CCs configured with BFR with unified TCI

  - FG23-1-2: We support all the 7 components. We think component 5/6 should be counted across CCs and serving cell PCIs should be counted. Besides, we propose to add another component on maximum number of overlapped SSBs for L1-RSRP measurement. We also modify some candidate values.

  - FG23-1-3: We support all the components, and we modify some candidate values.

- Enhancement on Multi-TRP PDCCH (FG23-2x)

  - We think BD{2+3} should be removed as it was concluded no RRC configuration of BD is supported in RAN1 #106b. Besides, we suggest we add another component for additional processing delay for mTRP PDCCH decoding.

- Enhancement on Multi-TRP PUCCH/PUSCH (FG23-3x)

  - We support all components.

- Enhancement on Multi-TRP inter-cell operation (FG23-4x)

  - We support the components in principle, and we suggest some revision based on the agreements in RAN1 #106b. In addition, similar to inter-cell BM, we suggest adding one component on number of overlapped SSBs for L1-RSRP measurement.

- Enhancement on beam management for Multi-TRP (FG23-5x)

  - FG23-5-1: We support all components except for component 5.

  - FG23-5-2: We removed some invalid components. Besides, we add one additional component based on the agreement in RAN1 #106b.

- Enhancement on HST-SFN deployment (FG23-6x)

  - FG23-6-4: We propose to split the default beam related UE capability into two FGs, one for DL and one for UL, following the same approach in Rel-16

  - We propose to introduce FG23-6-5 for support of implicit/explicit configuration of RS(s) with two TCI states for beam failure detection

  - We propose to introduce FG23-6-6 for HST-SFN scheme 1 PDCCH scheduling Single-TRP PDSCH based on the agreement in the last RAN1 meeting

- Enhancement on Multi-TRP CSI (FG23-7x)

  - FG23-7-1: we propose to add two components for the maximum total number of ports and the X related to the joint Single-TRP and Multi-TRP measurement reporting

  - We propose to introduce FG23-7-6 for the support of CSI-IMR

- Enhancement on SRS (FG23-8x)

  - FG23-8-3: The components are modified to match the current antenna switching related UE capability. We also need to further discuss the allowed combination of fall back configuration or allow UE to report it freely as proposed bitmap

  - FG23-8-9: Based on the agreement from the last RAN1 meeting, we propose to introduce FG23-8-9 for the support of N=4 for 1T4R and N=2 for 1T2R/2T4R for aperiodic antenna switching SRS configurations (N is the number of aperiodic SRS resource sets configured for antenna switching)

- Enhancement on port selection codebook (FG23-9x)

  - FG23-9-4: Based on the agreement from the last RAN1 meeting, we propose to introduce FG23-9-4 for the support of R=2 when Mv=2 for Rel-17 Port Selection codebook. We can further discuss whether and how to reduce the triplet related UE capability reporting

The updated UE feature list with our proposal and with track change on is provided in Appendix in this contribution.

- Reference

[1] R1-2110587, Updated RAN1 UE features list for Rel-17 NR after RAN1 #106bis-e, 3GPP TSG RAN WG1 #106bis-e e-Meeting, October 11th – 19th, 2021

- Appendix

- NR_FeMIMO

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (Sidelink WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 23. NR_FeMIMO | 23-1-1 | Unified TCI for [intra- and inter-cell] beam management | [For both intra- and inter-cell beam management:] 1. Support of joint DL/UL TCI update [and separate DL/UL TCI update] with their components (configuration mechanism, QCL rules, applicable source and target signals) [and Common cross-CC TCI update and activation (involving RRC common TCI state pool)] |  |  |  | UE does not support Rel-17 unified TCI | per UE | N | Y |  | {Support, Not support} [4. candidate values {1, …, 32} 5. candidate values {64, 72, 80, 96, 128, 192, 256, 128*4*32} 6. candidate values {8, 16, 32, 64, 128, 256} 63. candidate values FFS {no, with DA, w/o DA, both} 7. candidate values {1, 2, 3, 4, 5, 6, 7, 8} 8. candidate values {1, 2, 3, 4,5,6,7, 8} Note 1: for component 7 & 8, if two TCI states are in one codepoint, it should be counted as 2.] | Optional with capability signalling |
| 23. NR_FeMIMO | 23-1-1a | TCI indication | 1. Minimum action delay (Y symbols) for unified TCI indication in DCI 2. Supported mode of [MAC-CE/MAC-CE+DCI]-based beam TCI state indication [(including TCI state activation, use of DCI formats 1_1/1_2 with and without DL assignment)] | 23-1-1 | Y | N/A | UE does not support DCI based unified TCI indication | Per band | N | N |  | Component 1: 15kHz SCS: {14, 28, 42} symbols 30kHz SCS: {14, 28, 42} symbols 60kHz SCS: {14, 28, 42} symbols 120kHz SCS: {14, 28, 42} symbols Component 2: {MAC CE, MAC CE + DCI with data, MAC CE + DCI with and without data} | Optional |
| 23. NR_FeMIMO | 23-1-1b | Maximum number of unified TCI states for separate TCI indication | 1. Maximum number of active DL TCI states 2. Maximum number of active UL TCI states 3. Maximum number of active DL and UL TCI states 4. Maximum number of configured DL TCI states per TCI states pool 5. Maximum number of configured UL TCI states per TCI states pool 6. Maximum number of configured DL and UL TCI states per TCI states pool 7. Maximum number of DL/UL TCI states pools | 23-1-1 | Y | N/A | UE does not support separate TCI indication | Per band | N | N |  | Component 1: {1, 2, 4, 8} Component 2: {1, 2, 4, 8} Component 3: {1, 2, 4, 8}, if a UL TCI and a DL TCI share the same QCL property as defined in FG 2-62, the UL and DL TCI states are counted as 1. Component 4: {8, 16, 32, 64, 128, X} Component 5: {8, 16, 32, 64, X} Component 6: {8, 16, 32, 64, 128, X} Component 7: {1, 2, 3, 4} | Optional |
| 23. NR_FeMIMO | 23-1-1c | Maximum number of TCI states for joint TCI indication | 1. Maximum number of active joint TCI states 2. Maximum number of configured joint TCI states per TCI states pool 3. Maximum number of joint TCI states pools | 23-1-1 | Y | N/A | UE does not support joint TCI indication | Per band | N | N |  | Component 1: {1, 2, 4, 8} Component 2: {8, 16, 32, 64, 128, X} Component 3: {1, 2, 3, 4} | Optional |
| 23. NR_FeMIMO | 23-1-1d | Uplink power control for unified TCI | 1. Support beam misalignment between PL-RS and spatial relation RS 2. Support of association between TCI state and UL PC settings [for PUCCH, PUSCH, and SRS] [(PLRS and other, including handling of beam [alignment /misalignment] for PLRS)] | 23-1-1 | Y | N/A | UE does not support power control parameters to be associated with unified TCI | Per band | N | N | FR2 only | Component 1: {Support, not support} Component 2: {Support, not support} | Optional |
| 23. NR_FeMIMO | 23-1-1e | BFR with unified TCI | 1. Maximum number of CCs configured with BFR | 23-1-1 | N | N/A |  | Per band | N | N |  | Component 1: {1, 2, 3, 4} | Optional |
| 23. NR_FeMIMO | 23-1-2 | Inter-cell measurement and reporting (for inter-cell BM [and mTRP]) | 1. Support of [L1-RSRP/beam] measurement and report on SSB(s) with PCI(s) different from serving cell PCI 2. Support of [L1-RSRP/beam] reporting of up to Kmax=4 SSBRI-RSRP[,SS-RSRP] [pairs/beams], with at least one [pair/beam] associated with a PCI different from serving cell PCI [in one report instance] 3. The number of [RRC-configured/ MAC-CE activated] PCI(s) different from serving cell PCI for beam measurement] [in FR1] 4. The number of [RRC- configured/ MAC-CE activated] PCI(s) different from serving cell PCI for beam measurement] [in FR2] 5. The max number of SSB resources configured to measure L1-RSRP within a slot across all PCI(s) across all CCs 6. The max number of SSB resources configured to measure L1-RSRP across all PCI(s) across all CCs 7. Support on that SSB(s) with PCI(s) different from serving cell PCI configured for L1 beam measurement and report are not included in SSBs with PCIs configured for L3 mobility measurement 8. Maximum number of overlapped SSBs in one SSB resource for L1-RSRP measurement | [2-24, 2-29] |  |  |  |  |  |  |  | {Support, Not support} 2. Candidate value of {1,2,3,4} 3. candidate values {1, 2, 3, 7} 4. candidate values {1, 2, 3, 7} 5. candidate values: {2, 4, 8, FFS} 6. candidate values: {8, 16, 32, 64, FFS} 7. candidate values {not support} 8. candidate values {1, 2, 3, 4, 5, 6, 7, 8} | Optional with capability signalling |
| 23. NR_FeMIMO | 23-1-3 | MPE mitigation | 1. Support of enhanced reporting for MPE mitigation (with SSBRI/CRI and [associated] P-MPR in PHR reporting) 2. Support of reporting up to Nmax P-MPR and SSBRI/CRI pairs 3. Maximum number of candidate RS(s) configured in a RRC pool for MPE mitigation |  |  |  |  |  |  |  |  | {Support, Not support} [2. Candidate value of {1,2 3,,4}] [3. Candidate value at least includes {4}] | Optional with capability signalling |
| 23. NR_FeMIMO | 23-1-4 | MPUE support for UL | [depending on agreement] |  |  |  |  |  |  |  |  | {Support, Not support} | Optional with capability signalling |
| 23. NR_FeMIMO | 23-2-1 | [Multi-TRP] PDCCH repetition | 1. Support of PDCCH repetition (based on two linked SS sets associated with corresponding CORESETs) [with non-SFN scheme TDM and FDM] [including PDCCH repetition for Type 3 CSS] 2. Support of reporting one number as required number of BDs for the two PDCCH candidates 3. support of whether the individual candidate is monitored or not when one of the linked PDCCH candidates uses the same set of CCEs as an individual (unlinked) PDCCH candidate, and they both are associated with the same DCI size, scrambling, and CORESET 4. Support max number of overlaps when one of the linked PDCCH candidates uses the same set of CCEs as an individual (unlinked) PDCCH candidate per scheduling scheduled component carrier 5. Additional delay (number of symbols) for mTRP PDCCH processing |  |  |  |  |  |  |  |  | Component1: {Support, Not support} Component2: {2, 3} Component3: {monitored, not monitored} Component4: {0, 1, 2, 3} Component5: {7, 14} | Optional with capability signalling |
| 23. NR_FeMIMO | 23-2-2 | Two QCL TypeD for [CORESET monitoring in] PDCCH [repetition] | Support of determining two QCL-TypeD for overlapping CORESETs in the same CC or for intra-band CA [when UE is configured with PDCCH repetition] | [23-2-1, 23-6-1, 23-6-2] |  |  |  |  |  |  | FR2 only | {Support, Not support} | Optional with capability signalling |
| 23. NR_FeMIMO | 23-2-3 | PDCCH repetition for Type3 CSS | Support of PDCCH repetition for Type3 CSS |  |  |  |  |  |  |  |  | {Support, Not support} | Optional with capability signalling |
| 23. NR_FeMIMO | 23-3-1 | Multi-TRP PUSCH repetition (type A) | 1. Support of multi-TRP PUSCH repetition (based on PUSCH repetition type A and B) 2. Support of cyclic mapping when the number of repetitions is larger than 2 for single DCI based M-TRP PUSCH repetition Type A and B 3. Support of second TPC field for per TRP closed-loop power control for PUSCH with DCI formats 0_1 / 0_2 4. Support of PHR reporting related to M-TRP PUSCH repetition (calculate two PHRs (at least corresponding to the CC that applies m-TRP PUSCH repetitions), each associated with a first PUSCH occasion to each TRP, and report two PHRs.) 5. Support of CG PUSCH transmission towards M-TRPs using a single CG configuration (Use same beam mapping principals as dynamic grant PUSCH repetition scheme.) 6. support of sequential mapping for single for single DCI based M-TRP PUSCH repetition Type A 7. The maximum number of PHR reporting across all CCs (including those related to M-TRP PUSCH repetition and the legacy Rel-15/16 PUSCH transmission) 8. Support of A-CSI report on two PUSCH repetitions 9. Support of SP-CSI report on two PUSCH repetitions 10: Support dynamic switching between multi-TRP PUSCH scheme and single-TRP PUSCH transmission 11. Support PUSCH operations: CB based and NCB based and corresponding parameters including number of SRS resources |  |  |  |  |  |  |  |  | {Support, Not support} | Optional with capability signalling |
| 23. NR_FeMIMO | 23-3-1-1 | Multi-TRP PUSCH repetition (type B) | 1. Support of multi-TRP PUSCH repetition (based on PUSCH repetition type B) FFS: discuss details (to be discussed after type A is stable_ |  |  |  |  |  |  |  |  |  | Optional with capability signalling |
| 23. NR_FeMIMO | 23-3-2 | Multi-TRP PUCCH repetition | 1. Support of PUCCH repetition scheme 1 (inter-slot repetition) 2. Support of PUCCH repetition scheme 3 (intra-slot repetition) 3. Support of cyclic mapping for beam mapping/power control parameter set mapping [when the number of repetitions is larger than 2] for PUCCH repetitions scheme 1 and 3 when the number of repetitions is larger than 2 4. Support of second TPC field for per TRP closed-loop power control for PUCCH with DCI formats 1_1 / 1_2 |  |  |  |  |  |  |  |  | {Support, Not support} | Optional with capability signalling |
| 23. NR_FeMIMO | 23-4 | IntCell-mTRP | 1. Support of RRC configuration of additional PCI different from serving cell associated with the TCI state and/or QCL-info including other cell info in RRC TCI state config 2. Supported max number of RRC-configured PCIs different from serving cell PCI for (X1) SSB time domain positions or periodicity of additional PCIs is exactly the same as serving cell PCI) 3. Supported max number of RRC-configured PCIs different from serving cell PCI for (X2) SSB time domain positions and periodicity is not exactly the same among the additional PCIs and the same as serving cell PCI 4. Maximum number of overlapped SSBs in one SSB resource for L1-RSRP measurement |  |  |  |  |  |  |  |  | 1. candidate values {Support, Not support} 2. candidate values {0, 1, 2, 3, 7} 3. candidate values {0, 1, 2, 3, 7} 4. candidate values {1, 2, 3, 4, 5, 6, 7, 8} | Optional with capability signalling |
| 23. NR_FeMIMO | 23-5-1 | Group based L1-RSRP Beam reporting enhancements | - Support a single L1-RSRP reporting instance consisting of N of the maximum number of beam groups and M=2 beams per [pair/group] [in one report instance] - Support of enhanced group-based reporting for Rel-17 M-TRP - Maximum number of SSB and CSI-RS resources for measurement in both CMR sets within a slot across all CCs - Maximum number of configured SSB and CSI-RS resources for measurement in both CMR sets across all CCs - Support of enhanced group-based reporting for Rel-17 intra-cell and inter-cell mTRP |  |  |  |  |  |  |  |  | 1. Candidate values: {1,2,3,4} 2. Candidate values: {Support, not support} 3. Candidate values: {1,2,3,4} 4. Candidate values: {8, 16, 24, 32, 48, 64} 5. Candidate values: {intra-cell mTRP, inter-cell mTRP, intra-cell+inter-cell mTRP} | Optional with capability signalling |
| 23. NR_FeMIMO | 23-5-2 | Maximum number of BFD-RS resources per set for MTRP BFR enhancements | - Support of the maximum number of BFD-RS resources per set - Maximum number of CSI-RS resources of both BFD-RS sets across all CCs - Maximal number of different CSI-RS and/or SSB resources of both NBI-RS sets across all CCs - Support number of PUCCH-SR resources for MTRP BFRQ - Supported maximum number of BFD-RS resources across two BFD-RS sets per BWP - Support of [single DCI/multi-DCI] based BFR enhancement - Support of association between PUCCH-SR and BFD RS set |  |  |  |  |  |  |  |  | 1. Candidate values: {1, 2} 2. Candidate values: {1, 2, 3, 4} 3. Candidate values: {8, 16, 24, 32, 48, 64} 4. Candidate values: {0, 1, 2} 5. Candidate values: {1, 2, 3, 4} 6. Candidate values: {single-DCI, multi-DCI, both} 7. Candidate values: {Support, not support} | Optional with capability signalling |
| 23. NR_FeMIMO | 23-6-1 | SFN scheme A (scheme 1) | 1. [Support of scheme A for PDCCH] [and default QCL assumption with two TCI states for PDCCH] 2. Support of scheme A for PDSCH [scheduled by [single TRP/ Scheme A] PDCCH] [and default QCL assumption with two TCI states for PDSCH] |  |  |  |  |  |  |  |  | Component 1: {Support, Not support} Component 2: {Support, Not support} | Optional with capability signalling |
| 23. NR_FeMIMO | 23-6-1a | Dynamic switching - scheme A | Support of dynamic switching between single-TRP and SFN PDSCH scheme A by TCI state field in DCI formats 1_1, 1_2 |  |  |  |  |  |  |  |  |  |  |
| 23. NR_FeMIMO | 23-6-2 | SFN scheme B (TRP based pre-compensation) | - [Support of scheme B for PDCCH] and PDSCH - Support of scheme B for PDSCH [scheduled by [single TRP/Scheme B] PDCCH] | [23-6-1] |  |  |  |  |  | [FR1 only] |  | Component 1: {Support, Not support} Component 2: {Support, Not support} | Optional with capability signalling |
| 23. NR_FeMIMO | 23-6-2a | Dynamic switching - scheme B | Support of dynamic switching between single-TRP and SFN PDSCH scheme B by TCI state field in DCI formats 1_1, 1_2 |  |  |  |  |  |  |  |  |  |  |
| 23. NR_FeMIMO | 23-6-3 | Simultaneous activation of two TCI states for PDCCH across multiple CCs [(HST/URLLC)] | Support of simultaneous activation of two TCI states [for CORESETs with the same CORESET ID] in all BWPs across a set of configured component carriers [by single MAC-CE] |  |  |  |  |  |  |  |  | {Support, Not support} | Optional with capability signalling |
| 23. NR_FeMIMO | 23-6-4-1 | DL Default beam setup for HST | 1. Support of PDSCH reception using default beam for Rel-17 enhanced SFN scheme when PDSCH is scheduled with offset less than threshold 2. Support PDSCH reception using default beam for Rel-17 enhanced SFN scheme when TCI field is not present in DCI [when PDSCH is scheduled with offset equal or larger than the threshold] 3. Support aperiodic CSI-RS reception using default beam for Rel-17 enhanced SFN scheme when scheduling offset is less than threshold |  |  |  |  |  |  |  |  | {Support, Not support} | Optional with capability signalling |
| 23. NR_FeMIMO | 23-6-4-2 | UL Default beam setup for HST | 1. Support of single-TRP PUCCH transmission using default beam when enhanced SFN PDCCH transmission scheme is configured 2. Support of single-TRP PUSCH transmission using default beam when enhanced SFN PDCCH transmission scheme is configured 3. Support of single-TRP SRS resource transmission using default beam when enhanced SFN PDCCH transmission scheme is configured |  |  |  |  |  |  |  |  | {Support, Not support} | Optional with capability signalling |
| 23. NR_FeMIMO | 23-6-5 | Support implicit/explicit configuration of RS(s) with two TCI states for beam failure detection | Support RS(s) with two TCI states configured, either implicitly or explicitly, for beam failure detection enhancement for SFN scheme A for HST |  |  |  |  |  |  |  |  | {Support, Not support} | Optional with capability signalling |
| 23. NR_FeMIMO | 23-6-6 | Support of SFN scheme A PDCCH scheduling Single-TRP PDSCH | Support of SFN scheme A PDCCH scheduling Single-TRP PDSCH |  |  |  |  |  |  |  |  | {Support, Not support} | Optional with capability signalling |
| 23. NR_FeMIMO | 23-7-1 | Basic Features of CSI Enhancement for Multi-TRP | - Support of Maximum number of NZP CSI-RS resource pairs used as CMR (channel measurement resource) pairs for NCJT measurement hypothesis: Nmax=1 - FFS: Support of Maximum number of NZP CSI-RS resources per CMR group: Ks,max=[2,4] - FFS others (supported options; values for X, codebook types, number of ports of CMRs, CMR sharing among NCJT and sTRP measurement hypotheses for FR1, two CMR groups with Ks=K1+K2 CMRs in CSI-RS resource set, reporting of 2 PMIs, 2 RIs and 2 Lis for NCJT measurement hypothesis …) - Support of maximum total number of ports across two CMRs in a pair of CMRs for Multi-TRP CSI measurement - Support of X CSI associated with single-TRP measurement hypotheses and one CSI associated with NCJT measurement hypothesis | FFS |  |  |  |  |  |  |  | {Support, Not support} Component 4: {4,8,12,16,24,32,48,64} Component 5: bitmap {0, 1, 2} | Optional with capability signalling |
| 23. NR_FeMIMO | 23-7-2 | Support of max # of Tx ports [per source/across two CMRs] [in a resource set for Multi-TRP CSI] [and max # resources] | [A list of supported combinations, each combination is {max # of Tx ports per source in a resource set for Multi-TRP CSI, max # resources in a resource set for Multi-TRP CSI}] {24 ports, 32 ports} [Note: same number of ports among CMRs] |  |  |  |  |  |  |  |  | {Support, Not support} [{4, 8, 12, 16, 24, 32}] | Optional with capability signalling |
| 23. NR_FeMIMO | 23-7-3 | Support of max # More than two resources in a resource set for Multi-TRP CSI | FFS exact candidate values, Ks,max is up to 8 |  |  |  |  |  |  |  |  | {Support, Not support} [candidate values are {2, 3, 4, 5, 6, 7, 8}] | Optional with capability signalling |
| 23. NR_FeMIMO | 23-7-4 | Support of Nmax=2 for Multi-TRP CSI | [Support of maximum number 2 pairs of CMR pairs Nmax=2 resources configured in a CSI-ReportConfig for Multi-TRP CSI measurement for CSI Enhancement for Multi-TRP] |  |  |  |  |  |  |  |  | {Support, Not support} | Optional with capability signalling |
| 23. NR_FeMIMO | 23-7-5 | Whether two CMRs from a CMR pair configured for a NCJT measurement hypothesis can be used for Single-TRP measurement hypotheses CMR sharing | [Support a NZP CSI-RS resource referred by both a CMR pair configured for NCJT measurement hypothesis and a CMR configured for Single-TRP measurement hypothesis] [FR2 only] |  |  |  |  |  |  |  |  | {Support, Not support} | Optional with capability signalling |
| 23. NR_FeMIMO | 23-7-6 | Support of CSI-IM for CSI enhancement for Multi-TRP | Support CSI-IM for CSI enhancement for Multi-TRP |  |  |  |  |  |  |  |  | {Support, Not support} | Optional with capability signalling |
| 23. NR_FeMIMO | 23-8-1 | SRS triggering offset enhancement | Support of determining aperiodic SRS location based on available slot [and dynamically based on Rel-17 introduced new field in DCI Format 0_1/0_2/1_1/1_2] |  |  |  |  |  |  |  |  | {Support, Not support} | Optional with capability signalling |
| 23. NR_FeMIMO | 23-8-2 | Triggering SRS only in DCI 0_1/0_2 | Support of triggering SRS in DCI 0_1/0_2 without data and without CSI |  |  |  |  |  |  |  |  | {Support, Not support} | Optional with capability signalling |
| 23. NR_FeMIMO | 23-8-3 | SRS Antenna switching for >4Rx | 1. Support of SRS antenna switching xTyR with y>4 [2. Report whether the uplink TX switching impact to downlink receiving in a band] [3. Report whether the UL Tx is switched together with UL Tx in another band] |  |  |  |  |  |  |  |  | Component 1: a list of TRx pairs, candidates are bitmap {"Not supported", “1T1R”, “1T2R”,”1T4R”, “1T6R”, “1T8R”, “2T2R”, “2T4R”, “2T6R”, “2T8R”, “4T4R”, “4T6R”, “4T8R”} Component 2: Candidate value set: {yes, no} Component 3: Candidate value set: {yes, no} | Optional with capability signalling |
| 23. NR_FeMIMO | 23-8-4 | Maximum 2 SP [and 1 periodic] SRS sets for antenna switching | Support of maximum [2] [SP] SRS resource sets [and maximum 1 periodic SRS resource set] for antenna switching |  |  |  |  |  |  |  |  | {Support, Not support} [Note1: - Applies for all supported xTyR where y<=8 - For xTyR where y>4, if UE does NOT support this feature, support maximum one SRS resource set for periodic SRS and maximum one SRS resource set for semi-persistent SRS - For xTyR where y<=4, if UEs that do not support this feature, follow Rel-15 on the number of resource sets for periodic and semi-persistent SRS] | Optional with capability signalling |
| 23. NR_FeMIMO | 23-8-5 | Increased repetition for SRS | Support of increased repetition patterns (8, 10, 12, 14 symbols) for SRS resource |  |  |  |  |  |  |  |  | {Support, Not support} [(N_symbol, R) = {(8, 1), (8, 2), (8, 4), (8, 8), (12, 1), (12, 2), (12, 3), (12, 4), (12, 6), (12, 12), (10, 1), (10, 2), (10, 5), (10,10), (14, 1), (14, 2), (14, 7), (14, 14)}] | Optional with capability signalling |
| 23. NR_FeMIMO | 23-8-6 | Partial frequency sounding of SRS | Support of partial [(RB level)] frequency sounding for SRS |  |  |  |  |  |  |  |  | {Support, Not support} | Optional with capability signalling |
| 23. NR_FeMIMO | 23-8-7 | Start RB location hopping for partial frequency SRS | Support of start RB location hopping in partial [(RB level)] frequency sounding SRS transmission [across different SRS frequency hopping periods for periodic/semi-persistent SRS] | [23-8-6] |  |  |  |  |  |  |  | {Support, Not support} | Optional with capability signalling |
| 23. NR_FeMIMO | 23-8-8 | Comb-8 SRS | Support of comb-8 for MIMO SRS other than for positioning |  |  |  |  |  |  |  |  | {Support, Not support} | Optional with capability signalling |
| 23. NR_FeMIMO | 23-8-9 | Extension of aperiodic antenna switching SRS configurations for <=4Rx | Support N=4 for 1T4R and N=2 for 1T2R/2T4R for aperiodic antenna switching SRS configurations (N is the number of aperiodic SRS resource sets configured for antenna switching) |  |  |  |  |  |  |  |  |  | Optional with capability signalling |
| 23. NR_FeMIMO | 23-9-1 | Basic Features of Further Enhanced Port-Selection Type II Codebook (FeType-II) | - [{Max # of Tx ports in one resource, Max # of resources and total # of Tx ports}] to support Port-selection FeType-II with Mv=1 and R=1 - Support rank 1,2 - Support parameter combinations with Mv=1 - FFS others - Support R=1 | FFS |  |  |  |  |  |  |  | Component 1: - Maximum 16 triplets - Max # of Tx ports in one resource: {4,8,12,16,24,32} - Max # resources: {1 to 64} - Max # total ports: {4 to 256} | Optional with capability signalling |
| 23. NR_FeMIMO | 23-9-1a | Codebook concurrency of Type I codebook and Rel-17 FeType II codebook | Supported mixed codebook types | [23-9-1] |  |  |  |  |  |  |  | [Codebook 1 = {Type I SP, Type I MP} {Codebook 2, Codebook 3} = {{FeType II PS, NULL}, {Type II, FeType II PS },{eType II R=1, FeType II PS }}] | Optional with capability signalling |
| 23. NR_FeMIMO | 23-9-2 | Support of Mv=2 for FeType-II | [1. Support rank 1,2] 2. Support parameter combinations with Mv=2 [3. {Max # of Tx ports in one resource, Max # of resources and total # of Tx ports} to support Port-selection FeType-II with Mv=2 and R=1] 4. Support R=1 | [23-9-1] |  |  |  |  |  |  |  | Component 1: - Maximum 16 triplets - Max # of Tx ports in one resource: {4,8,12,16,24,32} - Max # resources: {1 to 64} - Max # total ports: {4 to 256} | Optional with capability signalling |
| 23. NR_FeMIMO | 23-9-3 | Support of rank 3, 4 for FeType-II | Support of rank 3, 4 for FeType-II | [23-9-1, 23-9-2] |  |  |  |  |  |  |  | {Support, Not support} | Optional with capability signalling |
| 23. NR_FeMIMO | 23-9-4 | Support of R=2 when Mv = 2 for FeType-II | - {Max # of Tx ports in one resource, Max # of resources and total # of Tx ports}] to support Port-selection FeType-II with Mv=2 and R=2 - Support of R=2 when Mv=2 for FeType-II Port Selection Codebook |  |  |  |  |  |  |  |  | Component 1: - Maximum 16 triplets - Max # of Tx ports in one resource: {4,8,12,16,24,32} - Max # resources: {1 to 64} - Max # total ports: {4 to 256} | Optional with capability signalling |