3GPP TSG RAN WG1 #107-e		R1-2111948

e-meeting, November 11th – 19th, 2021

Source:	Lenovo, Motorola Mobility

Title: 	Enhancements on PUSCH repetition type A

Agenda Item:   8.8.1.1

Document for:	Discussion

# 1	Introduction

In RAN#91-e, revised work item for NR coverage enhancements in Rel-17 has been approved with following objectives for in RP-210855 [1]:

- Specification of PUSCH enhancements [RAN1, RAN4]

  - Specify the following mechanisms for enhancements on PUSCH repetition type A [RAN1]

    - Increasing the maximum number of repetitions up to a number to be determined during the course of the work.

    - The number of repetitions counted on the basis of available UL slots.

  - Specify mechanism(s) to support TB processing over multi-slot PUSCH [RAN1]

    - TBS determined based on multiple slots and transmitted over multiple slots.

  - Specify mechanism(s) to enable joint channel estimation [RAN1, RAN4]

    - Mechanism(s) to enable joint channel estimation over multiple PUSCH transmissions, based on the conditions to keep power consistency and phase continuity to be investigated and specified if necessary by RAN4 [RAN1, RAN4]

      - Potential optimization of DMRS location/granularity in time domain is not precluded

    - Inter-slot frequency hopping with inter-slot bundling to enable joint channel estimation [RAN1]

- Specification of PUCCH enhancements [RAN1, RAN4]

  - Specify signaling mechanism to support dynamic PUCCH repetition factor indication [RAN1]

  - Specify mechanism to support DMRS bundling across PUCCH repetitions [RAN1, RAN4]

    - When applicable, based on similar mechanism(s) for enabling joint channel estimation for PUSCH

- Specify mechanism(s) to support Type A PUSCH repetitions for Msg3 [RAN1]

In this contribution, we provide our views on following enhancements needed for PUSCH repetition type A for improved coverage:

- Repetition factor configuration/indication

- Limitation of duration of PUSCH repetitions

- PUSCH repetition mode configuration/indication

# 2	Repetition factor configuration/indication

In RAN1#106bis-e, following discussion has been captured in the moderator’s summary related to support and indication of increased number of repetitions [2]:

- Rel-17 does not support up to 32 repetitions for DG-PUSCH scheduled by DCI format 0_0 and for Type 2 CG-PUSCH activated by DCI format 0_0.

- Rel-17 does not support up to 32 repetitions configured by Rel-17 pusch-AggregationFactor for DG-PUSCH scheduled by DCI format 0_1/0_2.

- For Type 1 CG-PUSCH and Type 2 CG-PUSCH activated by DCI format 0_1/0_2, select one from the following two alternatives:

  - Alt 2:

    - Rel-17 supports up-to-32 repetitions configured by Rel-17 repK for Type 1 CG-PUSCH. Rel-17 does not support up-to-32 repetitions configured by Rel-17 numberOfRepetitions for Type 1 CG-PUSCH.

      - Note: The TDRA list for Type 1 CG-PUSCH is kept un changed (i.e., use the TDRA list for DCI format 0_0).

      - Note: Need to introduce repK-r17.

    - Rel-17 supports up-to-32 repetitions configured by Rel-17 repK for Type 2 CG-PUSCH activated by DCI format 0_1/0_2.

  - Alt 3-b:

    - Rel-17 does not support up-to-32 repetitions configured by Rel-17 repK for Type 1 CG-PUSCH and for Type 2 CG-PUSCH activated by DCI format 0_1/0_2.

    - Rel-17 supports up-to-32 repetitions configured by Rel-17 numberOfRepetitions for Type 1 CG-PUSCH, where the TDRA list for DCI format 0_1 or 0_2 is reused.

      - Note: DCI format 0_0 field bit widths are kept unchanged.

      - Note: Need a mechanism to change the TDRA list for Type 1 CG-PUSCH, from the one for DCI format 0_0 to the one for DCI format 0_1 or 0_2.

Based on the above discussion, supporting and indication of up to 32 repetitions for Type-1 and Type-2 CG is still open. In our view, indicating the increased number of repetitions up to 32 for both Type-1 and Type-2 CG-PUSCH with both repK and numberOfRepetitons can be supported. As discussed, for both Alt 2 and Alt 3-b, the specification impact is not much for both the cases. Therefore, a combined option of Alt 2 and Alt 3-b can be supported for indicating maximum number of repetitions up to 32 for Type-1 and Type-2 CG-PUSCH. Based on this, following proposal can be made.

Proposal 1: For enhance PUSCH repetition type-A in NR Rel-17, support following:

- Rel-17 does not support up to 32 repetitions for DG-PUSCH scheduled by DCI format 0_0 and for Type 2 CG-PUSCH activated by DCI format 0_0.

- Rel-17 does not support up to 32 repetitions configured by Rel-17 pusch-AggregationFactor for DG-PUSCH scheduled by DCI format 0_1/0_2.

- For Type 1 CG-PUSCH and Type 2 CG-PUSCH activated by DCI format 0_1/0_2, select both the alternatives:

  - Alt 2:

    - Rel-17 supports up-to-32 repetitions configured by Rel-17 repK for Type 1 CG-PUSCH. Rel-17 does not support up-to-32 repetitions configured by Rel-17 numberOfRepetitions for Type 1 CG-PUSCH.

      - Note: The TDRA list for Type 1 CG-PUSCH is kept un changed (i.e., use the TDRA list for DCI format 0_0).

      - Note: Need to introduce repK-r17.

    - Rel-17 supports up-to-32 repetitions configured by Rel-17 repK for Type 2 CG-PUSCH activated by DCI format 0_1/0_2.

  - Alt 3-b:

    - Rel-17 does not support up-to-32 repetitions configured by Rel-17 repK for Type 1 CG-PUSCH and for Type 2 CG-PUSCH activated by DCI format 0_1/0_2.

    - Rel-17 supports up-to-32 repetitions configured by Rel-17 numberOfRepetitions for Type 1 CG-PUSCH, where the TDRA list for DCI format 0_1 or 0_2 is reused.

      - Note: DCI format 0_0 field bit widths are kept unchanged.

      - Note: Need a mechanism to change the TDRA list for Type 1 CG-PUSCH, from the one for DCI format 0_0 to the one for DCI format 0_1 or 0_2.

# 3	Conclusion

In this section, we summarize the observation/proposals from above section:

Proposal 1: For enhance PUSCH repetition type-A in NR Rel-17, support following:

- Rel-17 does not support up to 32 repetitions for DG-PUSCH scheduled by DCI format 0_0 and for Type 2 CG-PUSCH activated by DCI format 0_0.

- Rel-17 does not support up to 32 repetitions configured by Rel-17 pusch-AggregationFactor for DG-PUSCH scheduled by DCI format 0_1/0_2.

- For Type 1 CG-PUSCH and Type 2 CG-PUSCH activated by DCI format 0_1/0_2, select both the alternatives:

  - Alt 2:

    - Rel-17 supports up-to-32 repetitions configured by Rel-17 repK for Type 1 CG-PUSCH. Rel-17 does not support up-to-32 repetitions configured by Rel-17 numberOfRepetitions for Type 1 CG-PUSCH.

      - Note: The TDRA list for Type 1 CG-PUSCH is kept un changed (i.e., use the TDRA list for DCI format 0_0).

      - Note: Need to introduce repK-r17.

    - Rel-17 supports up-to-32 repetitions configured by Rel-17 repK for Type 2 CG-PUSCH activated by DCI format 0_1/0_2.

  - Alt 3-b:

    - Rel-17 does not support up-to-32 repetitions configured by Rel-17 repK for Type 1 CG-PUSCH and for Type 2 CG-PUSCH activated by DCI format 0_1/0_2.

    - Rel-17 supports up-to-32 repetitions configured by Rel-17 numberOfRepetitions for Type 1 CG-PUSCH, where the TDRA list for DCI format 0_1 or 0_2 is reused.

      - Note: DCI format 0_0 field bit widths are kept unchanged.

      - Note: Need a mechanism to change the TDRA list for Type 1 CG-PUSCH, from the one for DCI format 0_0 to the one for DCI format 0_1 or 0_2.

# 4	References

[1]	3GPP RP-210855, “Revised WID on NR coverage enhancements”, China Telecom

[2] 	R1-2110596, “FL Summary #4 on Enhancements on PUSCH repetition type A”, Moderator (Sharp)