3GPP TSG RAN WG1 #107-e		                                    R1-2111964

e-Meeting, November 11th – 19th, 2021

Agenda Item:	8.6.1.2

Source:	InterDigital, Inc.

Title:	Duplex operation for RedCap UEs

Document for:	Discussion

# Introduction

In RAN1#106bis-e, the following agreements were taken regarding duplex operation for RedCap UEs [1]:

Agreement

For Case 1, the existing timeline in Rel-15/16 NR for operation on a single carrier /single cell in unpaired spectrum is reused for HD-FDD

Agreement

- For HD-FDD switching time, reuse existing switching times for UE not capable of full duplex in TS 38.211, Table 4.3.2-3.

Note (from Oct 15th GTW session): With this agreement, no need to confirm below Working Assumption from RAN1#104e

- For HD-FDD switching time, reuse existing switching times for UE not capable of full duplex in TS 38.211, Table 4.3.2-3.

  - FFS: whether to define the guard times in symbol units

  - FFS: the switching positions

Conclusion:

- No consensus on defining a guard time in symbol units for HD-FDD Type A operation in Rel-17

Agreement

Revise the RAN1#104bis-e agreement for Case 3 as the following

- For Case 3, semi-statically configured DL reception vs. semi-statically configured UL transmission

  - A HD-FDD UE does not expect to receive both dedicated higher layer parameters configuring transmission from the UE in the set of symbols of the slot and dedicated higher layer parameters configuring reception in the set of symbols of the slot

  - A HD-FDD UE does not expect to receive both dedicated higher layer parameters configuring transmission from the UE in the set of symbols of the slot and cell specific higher layer parameters configuring reception in the set of symbols of the slot

  - Cell-specifically configured DL reception refers to PDCCH in Type-0/0A/1/2 CSS set

  - A HD-FDD UE does not expect to receive both cell specific higher layer parameters configuring transmission from the UE in the set of symbols of the slot and dedicated higher layer parameters configuring reception in the set of symbols of the slot

  - FFS on cell-specifically configured DL reception vs. cell-specifically configured UL transmission

  - FFS: whether or not there are conditions that need to be considered

Agreement

- For Type-A HD-FDD, no additional UE behaviour for UL/DL collision handling based on a priority indicator is specified as compared to the existing specification.

Agreement

- Whether or not to account for the Tx/Rx switching time before and after the set of SSB symbols can be further discussed under Case 9.

Agreement

- For Case 8 of valid RO overlapping with dynamically scheduled DL reception, leave it to UE implementation whether to receive the dynamically scheduled DL or transmit PRACH.

Agreement

- The same validation rules of MsgA PUSCH occasions and RO/Preamble-to-PRU mapping rules for FDD can be reused for HD-FDD.

Agreement

- For HD-FDD, reuse the same principle as Rel-15/16 UE not capable of full-duplex communication

  - A HD-FDD UE is not expected to transmit in the uplink earlier than NRX-TX Tc after the end of the last received downlink symbol in the same cell

  - A HD-FDD UE is not expected to receive in the downlink earlier than NTX-RX Tc after the end of the last transmitted uplink symbol in the same cell

  - NRX-TX Tc and NTX-RX Tc are the same as the transition time for FR1 in Table 4.3.2-3, TS 38.211 for a UE not capable of full-duplex communication

- (Working Assumption) The “back-to-back” non-overlapping UL/DL without sufficient gap between RRC configured UL and DL may happen, i.e., are allowed for HD-FDD UEs.

  - RRC configured DL/UL includes at least cell specific higher layer parameters configured DL/UL

  - Discuss further whether to specify a clear UE behavior, or leave it to UE implementation to ensure that the switching time is satisfied

  - Note: This does not mean a HD-FDD UE is required to support the back-to-back UL/DL switching without sufficient gap

In this contribution, a remaining collision scenario in Case 5 is discussed.

# Remaining collision cases

The remaining issue in Case 5 is collision handling for the case of SSB overlapping with dynamically scheduled UL transmission. The two options to handle this case are:

- Option 1: Dynamically scheduled UL transmission is prioritized over SSB.

- Option 2: Reuse the existing collision handling principles of Rel-15/16 for NR TDD that SSB is prioritized over dynamically scheduled UL transmission.

In connected mode, it is preferable to adopt Option 2 since dropping SSB may impact T/F tracking and RRM measurements. In addition, the gNB can prevent the collision by proper scheduling of the UL transmission. On the other hand, during initial access, Option 1 may be beneficial because by the time dynamic uplink transmission is scheduled, further SSB reception is not needed, and it would be detrimental for the UE to skip UL transmission. Therefore, the following is proposed:

Proposal 1: During initial access, support Option 1; in connected mode support Option 2.

# Conclusion

In this contribution, a remaining collision scenario in Case 5 has been discussed and the following is proposed:

Proposal 1: During initial access, support Option 1; in connected mode support Option 2 where

- Option 1: Dynamically scheduled UL transmission is prioritized over SSB.

- Option 2: Reuse the existing collision handling principles of Rel-15/16 for NR TDD that SSB is prioritized over dynamically scheduled UL transmission.

# References

- Chairman’s Notes, 3GPP WG1 #106bis-e