3GPP TSG RAN WG1 Meeting #107                                                                   	     R1- 2112001

e-Meeting, November 11th – 19th, 2021

Agenda Item:	8.9.1

Source:	Lenovo, Motorola Mobility

Title:	Support 16QAM for NBIoT

Document for:	Discussion and decision

Introduction

In the RAN#88 plenary meeting, an updated work item for Rel.17 eMTC/NBIoT was approved. The objective is to specify the following enhancements to NB-IoT.

- Specify 16-QAM for unicast in UL and DL, including necessary changes to DL power allocation for NPDSCH and DL TBS. This is to be specified without a new NB-IoT UE category. For DL, increase in maximum TBS of e.g. 2x the Rel-16 maximum, and soft buffer size will be specified by modifying at least existing Category NB2. For UL, the maximum TBS is not increased. [NB-IoT] [RAN1, RAN4]

  - Extend the NB-IoT channel quality reporting based on the framework of Rel-14—16, to support 16-QAM in DL. [NB-IoT] [RAN2, RAN1, RAN4]

The following agreements on support 16QAM for NBIoT were achieved:

Working Assumption

For the new term ∆TF,c introduced for power control of NPUSCH,

- Reuse the LTE definition simplified for NB-IoT: ∆TF,ci=10log102BPRE∙Ks-1 for Ks=1.25 and ∆TF,ci=0 for Ks=0, where Ks is given by higher layer parameter deltaMCS-Enabled, and BPRE=KNRE where K is the code block size.

- FFS: whether the new term applies to QPSK when configured with 16QAM, if it does not, whether an additional term is introduced to avoid jump between QPSK and 16QAM

Agreement

Support 16-QAM for NPDSCH in PUR procedure

- CSI report is not supported/expected during PUR procedure.

Agreement

To support 16-QAM for NPDSCH and NPUSCH in PUR procedure,

- 16-QAM can be enabled/disabled by UE specific RRC signaling for NPDSCH and NPUSCH separately

  - The corresponding configurations and signaling details are up to RAN2

Agreement

The reserved state to indicate the use of 16QAM in DCI format N0 and DCI format N1 should be “1111”.

Agreement

Confirm the following working assumption:

Working Assumption

For downlink power allocation to support 16QAM:

- For inband deployments, a power ratio is signaled in addition to the signalling for standalone and guard-band deployments which in this case applies to “symbols with NRS” and “symbols without NRS nor CRS”.

o   the power ratio between NPDSCH EPRE and NRS EPRE in symbols with CRS is signalled

o   the signalling is UE specific

Note: “symbols with NRS” and “symbols without NRS nor CRS” have the same power.

Agreement

For the UE configured with 16-QAM for NPDSCH, the deployment of the carrier is signaled by operationModeInfo in MIB or inbandCarrierInfo in SIB/UE specific signaling.

Note: Existing agreement from RAN1#106e is "For the UE configured with 16-QAM for NPDSCH, the deployment of the carrier is signaled by operationModeInfo in MIB or inbandCarrierInfo in SIB", which is replaced by the updated agreement above from RAN1#106bis-e.

In this contribution, detailed considerations of 16QAM support for NPDSCH and NPUSCH, especially new TBS table and related DL power allocation design are presented.

Discussion

CQI Reporting to support 16-QAM in DL

Option 1: Reusing legacy CQI reporting table and extend CQI entries to 16QAM

For NBIoT CQI reporting in Msg3, it is defined as the NPDCCH repetition level of hypothetical NPDCCH BLER of 1%. The reported values for NB-IoT are based on mapping table in TS 36.133 section 9.1.22.15 with the NPDCCH repetition levels (1, 2, 4, 8, 16, 32, 64, 128, 256, 512, 1024, 2048). It is agreed that if 16-QAM is configured for NPDSCH, the channel quality report for 16-QAM is based on NPDSCH transport block that achieves an error probability not exceeding 10% BLER in previous RAN1 meeting

To extend the NB-IoT channel quality reporting based on the framework of Rel-14—16 as noted in WID, there are three unused fields utilized to incorporate the reporting for 16-QAM in DL in addition to the existing NPDCCH repetitions level if the legacy CQI reporting value and 4bit CQI reporting should be kept. However, it seems not sufficient to reporting only three states of CQI reporting adopted for 16QAM, especially there are up to 8 TBS support to 16QAM.

Therefore, we can consider 1) removing some of the legacy CQI reporting values for QPSK and add more than 3 CQI reporting values for support 16QAM in DL or 2)keeping all legacy CQI entries with more than  3 CQI reporting values for support 16QAM in DL and selecting the subset of CQI reporting values determined by higher layer parameter or channel condition (e.g., first 16 or last 16 entries out of [23] CQI reporting value is selected based on 1-bit higher layer parameter or MPDCCH repetition number Rmax).

Table 1 gives an example of CQI table for NBIoT(Inband). For standalone and guardband, higher code rate (TBS) is expected for more available resource elements, new CQI table (or new CQI entries in addition to Table 1) is needed.

Table 1 CQI table for NBIoT (Inband)

|  | NPDCCH repetition level | NPDSCH |  |
| --- | --- | --- | --- |
|  |  | Modulation | Code rate x 1024 |
| noMeasurement | No measurement reporting | N/A | N/A |
| candidateRep-A | 1 | N/A | N/A |
| candidateRep-B | 2 | N/A | N/A |
| candidateRep-C | 4 | N/A | N/A |
| candidateRep-D | 8 | N/A | N/A |
| candidateRep-E | 16 | N/A | N/A |
| candidateRep-F | 32 | N/A | N/A |
| candidateRep-G | 64 | N/A | N/A |
| candidateRep-H | 128 | N/A | N/A |
| candidateRep-I | 256 | N/A | N/A |
| candidateRep-J | 512 | N/A | N/A |
| candidateRep-K | 1024 | N/A | N/A |
| candidateRep-L | 2048 | N/A | N/A |
| candidate-M | N/A | 16QAM | [429] |
| candidate-N | N/A | 16QAM | [499] |
| candidate-O | N/A | 16QAM | [549] |
| candidate-P | N/A | 16QAM | [617] |
| candidate-Q | N/A | 16QAM | [667] |
| candidate-R | N/A | 16QAM | [712] |
| candidate-S | N/A | 16QAM | [781] |

Option 2: Follow legacy eMTC CQI table

A new CQI table is defined for 16-QAM based on the eMTC table (CQI Tables in 36.213) as a starting point. Some companies proposed the eMTC table based on TS36.213 Table 7.2.3-3. However, for NBIoT, we should give more CQI entries for low SNR region, especially for CQI=0 with large repetition number. So we propose to add the repetition number factor to the CQI table similar as TS36.213 Table 7.2.3-6 for stand-alone / in-band deployment.

Table 7.2.3-6: 4-bit CQI Table 6

| CQI index | modulation | code rate x 1024 | repetition |
| --- | --- | --- | --- |
| 0 | out of range |  |  |
| 1 | QPSK | [56] | 32 |
| 2 | QPSK | [207] | 16 |
| 3 | QPSK | [266] | 4 |
| 4 | QPSK | [195] | 2 |
| 5 | QPSK | [142] | 1 |
| 6 | QPSK | [266] | 1 |
| 7 | QPSK | [453] | 1 |
| 8 | QPSK | [637] | 1 |
| 9 | 16QAM | [423] | 1 |
| 10 | 16QAM | [557] | 1 |
| 11 | 16QAM | [696] | 1 |
| 12 | 16QAM | [845] | 1 |
| 13 | 64QAM | 651 | 1 |
| 14 | 64QAM | 780 | 1 |
| 15 | 64QAM | 888 | 1 |

Option 1 reuses the legacy NBIoT CQI entries as much as possible and aligns with WID scope better, so option 1 is our first preference. Option 2 seems easier for UE to report (e.g., only based on NPDSCH BLER) and is our second preference.

Proposal 1: Down select the follow options to support CQI reporting for 16QAM:

- Option 1: Reusing legacy CQI reporting table and extend CQI entries to 16QAM

- Option 2: Follow legacy eMTC CQI table

Uplink power control

For the additional power control parameter for 16-QAM (e.g. similar to ΔTF), RRC parameter is configured to increase the power of higher modulation in LTE. Similar as defined in LTE, ∆TF is determined as:

∆TF(i)=10log102BPER∙Ks-1 for Ks=1.25 and ∆TF,c(i)=0 for Ks=0, where Ks is given by higher layers parameter.

- BPER=Kc/NRE where Kc is the code block size and NRE is the number of resource elements determined as NRE=(NsymbUL-1)NslotsULNscRUNRU where NsymbUL, NslotsUL,  NscRU are defined in TS36.211, and NRU is defined in section 16.5.1.1 in TS36.213.

For the remaining issue of a large “jump between QPSK and 16-QAM”, the following solutions were proposed in the last meeting

- Option 1: The new term ∆TF,c can also be applied to QPSK when 16-QAM is configured

- Option 2: The new term ∆TF,c is not applied to QPSK when 16-QAM is configured, and an offset is applied on ∆TF,c to reduce the power difference between QPSK and 16-QAM. FFS: The details on the offset.

- Option 3: For NPUSCH with 16-QAM, the PO_NOMINAL_NPUSCH,c is configured by a new parameter p0-NominalNPUSCH

For option 1, it is more straightforward without further standard impact. Since we agreed to follow the legacy behavior, we should follow all procedures (e.g., adopting the term regardless of the modulation).

For option 2, we don’t think we have clear mind to give the exact offset value. It needs further evaluation and check.

For option 3, we don’t think we should give network an artificial barrier, which makes the network confused and set the initial power level with great care.

Proposal 2: The new term ∆TF,c introduced for power control of NPUSCH applies to QPSK and 16QAM when configured with 16QAM.

Conclusions

In this contribution, considerations of 16QAM support for NPDSCH and NPUSCH are provided. The following proposals are given.

Proposal 1: Down select the follow options to support CQI reporting for 16QAM:

- Option 1: Reusing the legacy CQI reporting table and extend CQI entries to 16QAM

- Option 2: Follow legacy eMTC CQI table

Proposal 2: The new term ∆TF,c introduced for power control of NPUSCH applies to QPSK and 16QAM when configured with 16QAM.

- References

RP-201306, “WID revision: Additional enhancements for NB-IoT and LTE-MTC”, Huawei, HiSilicon, RAN#88e