3GPP TSG RAN WG1 Meeting #107-e	R1-2112009

e-Meeting, November 11th – 19th, 2021

Source:	Sharp

Title:	Clarification on the condition to trigger re-evaluation in mode 2

Agenda Item:	7.2.4

Document for:	Discussion and Decision

Introduction

In this document, we share our views on one remaining issue in sidelink resource allocation mode 2.

Discussion

Clarification on the condition for re-evaluation

For sensing in Rel-16 NR V2X, the resource selection window is defined as [n+T1, n+T2] and selection of T1 is up to UE implementation under the condition 0≤T1≤Tproc,1SL. After performing Step 1)-Step 7) as specified in 8.1.4 of TS38.214, the remaining set SA is reported to MAC layer for resource selection, shown as follows.

1) A candidate single-slot resource for transmission Rx,y is defined as a set of LsubCH contiguous sub-channels with sub-channel x+j in slot t'ySL where j=0,...,LsubCH-1. The UE shall assume that any set of LsubCH contiguous sub-channels included in the corresponding resource pool within the time interval [n+T1,n+T2] correspond to one candidate single-slot resource, where - selection of T1 is up to UE implementation under 0 ≤ T1≤ Tproc,1SL , where Tproc,1SL is defined in slots in Table 8.1.4-2 where μSL is the SCS configuration of the SL BWP; - if T2min is shorter than the remaining packet delay budget (in slots) then T2 is up to UE implementation subject to T2min ≤ T2 ≤ remaining packet delay budget (in slots); otherwise T2 is set to the remaining packet delay budget (in slots). The total number of candidate single-slot resources is denoted by Mtotal. [… …] 7) If the number of candidate single-slot resources remaining in the set SA is smaller than X⋅Mtotal, then Th(pi,pj) is increased by 3 dB for each priority value Th(pi,pj) and the procedure continues with step 4. The UE shall report set SA to higher layers.

It is noticed that MAC layer would randomly select the resources from SA which is indicated by PHY, shown as follows. For the cases when T1<Tproc,1SL, the set SA includes some single-slot resources within the interval [n+T1, n+Tproc,1SL] and MAC layer may select among these resources for the selected sidelink grant, denoted as {ri}.

4> randomly select the time and frequency resources for one transmission opportunity from the resources indicated by the physical layer as specified in clause 8.1.4 of TS 38.214 [7], according to the amount of selected frequency resources and the remaining PDB of SL data available in the logical channel(s) allowed on the carrier.

According to 5.22.1.2a of TS38.321 for re-evaluation and pre-emption, any resource(s) of the selected sidelink grant is re-evaluated at the moment of T3(i.e. Tproc,1SL) before the slot where the SCI first indicates the resource(s), shown as follows.

- 5.22.1.2a Re-evaluation and Pre-emption A resource(s) of the selected sidelink grant for a MAC PDU to transmit from multiplexing and assembly entity is re-evaluated by physical layer at T3 before the slot where the SCI indicating the resource(s) is signalled at first time as specified in clause 8.1.4 of TS 38.214 [7].

For resource(s) {ri}, the moment to trigger re-evaluation as mentioned above is slotri-Tproc,1SL. Since slotri is within the time interval [n+T1, n+Tproc,1SL], it would result in the moment of triggering re-evaluation, i.e. slotri-Tproc,1SL comes before slot n which is the slot MAC triggers for resource selection. Thus, for the candidate resources within the time interval [n+T1, n+Tproc,1SL], it is naturally not needed to trigger the re-evaluation.Also, considering the re-evaluation procedure is captured in RAN2 specification, an LS is needed to inform of the constraint.

Proposal 1: A resource(s) of the selected sidelink grant is re-evaluated at slot r which is T3 before the slot m where it will be first signalled, only if the slot n when PHY is triggered to determine the selected sidelink grant is before slot r.

Proposal 2: Send an LS to RAN2 to inform of the condition.

Conclusion

In this contribution, we discuss one remaining issue in mode 2 and make the following proposals:

Proposal 1: A resource(s) of the selected sidelink grant is re-evaluated at slot r which is T3 before the slot m where it will be first signalled, only if the slot n when PHY is triggered to determine the selected sidelink grant is before slot r.

Proposal 2: Send an LS to RAN2 to inform of the condition.