3GPP TSG RAN WG1 Meeting #107-e	R1-2112066

Online, 11 – 19 November, 2021

Agenda Item:	8.12.4

Source: 	LG Electronics Inc.

Title: 	Other aspects for MBS

Document for:	Discussion and decision

# 1.	Introduction

The WID on NR Multicast and Broadcast Services was revised in RAN#88e [1]. In this contribution, we discuss other aspects for MBS.

# 2.	Discussion

RAN1 has discussed the following two options for the RedCap WI objective on reduced maximum UE bandwidth [1]:

- For FR1, following options: - Option 1: - For a separate initial DL BWP (if it does not include CD-SSB and the entire CORESET#0), - RedCap UE does NOT expect it to contain SSB/CORESET#0/SIB. - For an RRC-configured active DL BWP (if it does not include CD-SSB and the entire CORESET#0), - RedCap UE does NOT expect it to contain SSB/CORESET#0/SIB. - Option 2: - For a separate initial DL BWP (if it does not include CD-SSB and the entire CORESET#0), - If it is configured for random access while not for paging in idle/inactive mode, RedCap UE does NOT expect it to contain SSB/CORESET#0/SIB. - FFS: For BWP#0 configuration option 1, whether the UE can expect SSB transmission in the separate initial DL BWP when it is used in connected mode. - If it is configured for paging, RedCap UE expects it to contain NCD-SSB for serving cell but not CORESET#0/SIB. - For an RRC-configured active DL BWP in connected mode (if it does not include CD-SSB and the entire CORESET#0), - RedCap UE expects it to contain NCD-SSB for serving cell [FFS: or CSI-RS or measurement gap configuration] but not CORESET#0/SIB. - Note: if a separate initial/RRC configured DL BWP is configured to contain the entire CORESET#0, CD-SSB is expected by RedCap UE. - Note: The network may choose to configure SSB or MIB-configured CORESET#0 or SIB1 to be within the respective DL BWP.

Considering that both options in the above LS support a separate initial DL BWP for RedCap UE, it seems beneficial to clarify whether RedCap UE can support broadcast and/or multicast transmission in Rel-17. If RedCap UE can support broadcast and/or multicast transmission in Rel-17, we could discuss whether broadcast CFR can be configured in the separate initial DL BWP for RedCap UE and whether broadcast CFR can be configured to contain the separate initial DL BWP for RedCap UE.

Proposal 1: Clarify whether RedCap UE can support broadcast and/or multicast transmission in Rel-17.

Proposal 2: Discuss whether broadcast CFR can be configured in the separate initial DL BWP for RedCap UE and whether broadcast CFR can be configured to contain the separate initial DL BWP for RedCap UE, if RedCap UE can support broadcast and/or multicast transmission in Rel-17.

# 3.	Conclusion

In conclusion, we propose to discuss the following other aspects for MBS:

Proposal 1: Clarify whether RedCap UE can support broadcast and/or multicast transmission in Rel-17.

Proposal 2: Discuss whether broadcast CFR can be associated with the separate initial DL BWP for RedCap UE and whether broadcast CFR can be configured to contain the separate initial DL BWP for RedCap UE, if RedCap UE can support broadcast and/or multicast transmission in Rel-17.

# Reference

[1] R1-2110600	LS on use of NCD-SSB instead of CD-SSB for RedCap UE,	TSG-RAN1