3GPP TSG-RAN WG1 Meeting #107			R1-2112133

E-Meeting, November 11th – 19th, 2021

Agenda item:	8.16.2

Title:	Views on UE features for supporting NR from 52.6 GHz to 71 GHz

Source:	NTT DOCOMO, INC.

Document for:	Discussion and Decision

- Introduction

This contribution describes our views on UE features for supporting NR from 52.6 GHz to 71 GHz in Rel-17, captured in section 6 of R1-2109913 [1].

- Discussion

2.1. General views

A general issue would be how to consider FR-related differentiation. In this WI, companies discuss on various types of new features and enhancements of the existing NR functionalities to extend NR to 52.6 – 71 GHz frequency range. Any UE feature to be supported in this WI will then be applicable at least for 52.6 – 71 GHz. However, companies may or may not argue that some of the UE features to be specified in this WI could be technically beneficial even in another frequency range, and then desire to discuss whether/how to expand the applicability of such UE features to other frequency ranges. It may consume quite a lot of time in RAN1 to discuss the range of applicability one by one. Note that 52.6 – 71 GHz frequency range at least include unlicensed bands, while licensed bands may also be identified in the future. The WI also target the support for both licensed and unlicensed band in this frequency range. This fact may also make the discussion more complex.

Also, frequency range notation itself should be carefully considered. In the latest WID for supporting NR from 52.6 GHz to 71 GHz [2], two important aspects on frequency range definition regarding beyond 24 GHz are captured; one is to extend the definition of FR2 up to 71GHz, and the other is to introduce new FR sub-labels, FR2-1 and FR2-2, to be used for differentiating 24.25 – 52.6 GHz and 52.6 – 71 GHz if needed. Therefore, as well as FR1/2 differentiation, FR2-1/2-2 differentiation may also need to be considered.

With the consideration above, how to have FR-related differentiation would depend on each UE feature in our view. We see the following alternatives at this stage.

- One potential approach to easily solve this issue could be to define all the UE features to be specified in this WI per-band (or per BC). With this, UE can report its capability regarding functionalities for 52.6 – 71 GHz operation per band that the UE supports, which means vendors have a freedom for its own implementation. However, it may increase the overhead for UE capability signalling depending on the number of bands/band combinations to be specified.

- For UE features which can be applied regardless of licensed or unlicensed band, extending per-FR capability signalling may be another possibility. For example, by enabling per-FR capability signalling to differentiate FR2-1 and FR2-2, it would be possible to indicate a certain UE feature is applicable for FR2-2 only if needed. Or, if a UE feature is applicable to both FR2-1 and FR2-2 without any difference, just to use the existing per-FR capability signalling would also be possible. By defining in this manner, vendors still have a freedom to implement a certain feature for a certain frequency range, while overhead for capability signalling can be suppressed.

- To decrease signalling overhead more, per-UE signalling with some Notes can also be considered. This approach, however, may be applicable to particular UE features only, for which the targeted FR is crystal clear. For example, if a UE feature is clearly applicable for FR2-2 unlicensed band only, it could be possible to define it as a UE feature with per-UE capability signalling with a Note saying i.e., “this is applicable only for unlicensed band in FR2-2”. While this approach achieves much less overhead on UE capability signalling, an issue may be less implementation flexibility.

Table 1. Comparison of FR differentiation approaches

| FR differentiation | Flexibility for implementation | UE capability signalling overhead |
| --- | --- | --- |
| Per-band | Very flexible | Heavy |
| Per FR | Less flexible | Relatively light |
| Per UE | Much less flexible | Light |

Proposal 1: For the discussion on Rel-17 UE features at least regarding 52.6 – 71 GHz WI, the following alternatives can be considered in case-by-case manner, in terms of FR differentiation.

- Alt 1: define as per-band

- Alt 2: define as per-FR

  - Differentiation of FR2-1/2-2 may or may not be needed

- Alt 3: define as per-UE

  - A fixed limitation (e.g., as a Note) on applicable frequency range may be needed

2.2. Views on the latest Rel-17 UE feature list

During RAN1#106bis-e, there were some discussions based on the preliminary UE feature list although only a few UE features were captured since a number of discussions are still on-going in RAN1. One is whether to have DL/UL separation especially for FG24-1 (i.e., Basic FR2-2 support). There are six components to be potentially included (see below), while some of them may be separately defined.

Table 2.1-1: The latest status of FG24-1

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (Sidelink WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 24. NR_ext_to_71GHz | 24-1 | General Basic FR2-2 [DL] support | 1. Support 120KHz SCS [transmission and] reception for [initial/non-initial access] [2. Support multi-RB PUCCH format 0/1/4 for 120 kHz] [3. PRACH with 120KHz SCS and length 139[/571/1151]] [4. Support 120kHz subcarrier spacing for DL data and control channels and reference signals in FR2-2] [5. Support 120kHz subcarrier spacing for UL data and control channels and reference signals in FR2-2] [6. Support multi-PUSCH[/PDSCH] scheduling by single DCI for the operation with 120 kHz SCS] |  |  |  | FR2-2 is not supported | [per UE][per band] |  |  |  |  | Optional with capability signalling [A UE that supports FR2-2 must indicate this FG is supported] |
| 24. NR_ext_to_71GHz | 24-1a | FR2-2 PRACH support | 1. PRACH with 120KHz SCS and length 139[/571/1151] | 24-1 |  |  |  | [per UE][per band] |  |  |  |  | Optional with capability signalling |
| 24. NR_ext_to_71GHz | 24-1b | Basic FR2-2 UL support | 1. Support multi-RB PUCCH format 0/1/4 for 120 kHz [2. Support 120kHz subcarrier spacing for UL data and control channels and reference signals in FR2-2] | [24-1] |  |  |  | [per UE][per band] |  |  |  |  | Optional with capability signalling |
| 24. NR_ext_to_71GHz | 24-1c | Multi-PDSCH scheduling | 1. Support multi-PDSCH scheduling by single DCI for the operation with 120 kHz SCS | 24-1 |  |  |  | [per UE][per band] |  |  |  |  | Optional with capability signalling |
| 24. NR_ext_to_71GHz | 24-1d | Multi-PUSCH scheduling | 1. Support multi-PUSCH scheduling by single DCI for the operation with 120 kHz SCS | 24-1b |  |  |  | [per UE][per band] |  |  |  |  | Optional with capability signalling |
| 24. NR_ext_to_71GHz | 24-2 | 120KHz SSB based stand-alone support in FR2-2 | 1. Support 480120KHz SSB for initial access in FR2-2 | 24-1 |  |  |  |  |  |  |  |  | Optional with capability signalling |
| 24. NR_ext_to_71GHz | 24-3 | 480KHz SSB based stand-alone support for initial access in FR2-2 | 1. Support 480KHz SSB for initial access in FR2-2 | 24-1[, 24-2, 24-4] | Yes FFS |  |  | [Per UE/band] |  |  |  | From WID: - In addition to 120kHz, support 480 kHz SSB for initial access with support of CORESET#0/Type0-PDCCH configuration in the MIB with following constraints: - Note: 480 kHz is an optional SSB numerology for initial access for the UE. A UE supporting a band in 52.6-71 GHz must at least support 120 kHz SCS (for initial access and after initial access) - [only 480kHz CORESET#0/Type0-PDCCH SCS supported for 480 kHz SSB SCS] | Optional with capability signalling |
| 24. NR_ext_to_71GHz | 24-4 | 480KHz SCS support | 1. 480KHz SCS for UL data and control channels and reference signal transmission in FR2-2 2. 480KHz SCS for DL data and control channels and reference signal reception in FR2-2 3. 480KHz for SSB monitoring [for non-initial access] 4. Multiple-slot PDCCH monitoring for 480KHz with X=[4] slots [FFS: Component description to be updated once further details of multi-slot monitoring capability are known, e.g., definition of Y] 5. PRACH with 480KHz and length 139/[571] FFS: 6. Support multi-RB PUCCH format 0/1/4 for 480 kHz FFS: 7. Multi-PUSCH/PDSCH scheduling by single DCI for the operation with 480 kHz SCS | 24-1 | Yes |  |  | [Per UE/band] |  |  |  | From WID: In addition to 120kHz SCS, specify new SCS, 480kHz and 960kHz, and define maximum bandwidth(s), for operation in this frequency range for data and control channels and reference signals, only NCP supported. [Agreement: A UE supporting 480 kHz SCS supports multi-slot PDCCH monitoring for 480 kHz SCS. Agreement: Do not support PRACH length L=571, 1151 for 960kHz PRACH and at least L =1151 for 480kHz PRACH] Note: • Resolve the issues of wideband PRACH, multi-RB PUCCH format 0/1/4, and multi-PUSCH/PDSCH scheduling by single DCI, i.e., whether to have components of a single FG or separate FGs, for 120 kHz first, then use the same structure for 480 kHz • Resolve the issue of having separate capabilities for DL and UL (data and control channels as well as reference signals) for 120 kHz first, then use the same structure for 480 kHz | Optional with capability signalling |
| 24. NR_ext_to_71GHz | 24-5 | 960KHz SCS support | 1. 960KHz SCS for UL data and control channels and reference signal transmission in FR2-2 2. 960KHz SCS for DL data and control channels and reference signal reception in FR2-2 3. 960KHz for SSB monitoring 4. Multiple-slot PDCCH monitoring for 960KHz with X=[8] slots [FFS: Component description to be updated once further details of multi-slot monitoring capability are known, e.g., definition of Y] 5. PRACH with 960KHz and length 139 FFS: 6. Support multi-RB PUCCH format 0/1/4 for 960 kHz FFS: 7. Multi-PUSCH/PDSCH scheduling by single DCI for the operation with 960 kHz SCS | 24-1 | Yes |  |  | [Per UE/band] |  |  |  | From WID - In addition to 120kHz SCS, specify new SCS, 480kHz and 960kHz, and define maximum bandwidth(s), for operation in this frequency range for data and control channels and reference signals, only NCP supported. [Agreement: A UE supporting 960 kHz SCS supports multi-slot PDCCH monitoring for 960 kHz SCS. Agreement: Do not support PRACH length L=571, 1151 for 960kHz PRACH and at least L =1151 for 480kHz PRACH] Note: • Resolve the issues of wideband PRACH, multi-RB PUCCH format 0/1/4, and multi-PUSCH/PDSCH scheduling by single DCI, i.e., whether to have components of a single FG or separate FGs, for 120 kHz first, then use the same structure for 480 kHz • Resolve the issue of having separate capabilities for DL and UL (data and control channels as well as reference signals) for 120 kHz first, then use the same structure for 480 kHz | Optional with capability signalling |
| 24. NR_ext_to_71GHz | 24-?6 | Cat 3 or Cat 4 LBT support Uplink channel access procedure for FR2-2 unlicensed operation | - Cat 3 [or Cat 4] LBT support [(not agreed yet if CW is supported, so it can be either Cat 3 or Cat 4 LBT for now. Will update when we have agreement)] - [Support LBT performed per carrier/BWP bandwidth] | 24-1 |  |  |  | [Per band] |  |  |  |  | Optional with capability signalling [A UE that supports FR2-2 must indicate this FG is supported when required by regulation] |
| 24. NR_ext_to_71GHz | 24-?7 | Cat 2 LBT support for uplink channel access procedure for FR2-2 unlicensed operation | 1. Support Cat 2 LBT | 24-1[, 24-6] |  |  |  | [Per band] |  |  |  |  | Optional with capability signalling [A UE that supports FR2-2 must indicate this FG is supported when required by regulation] |
| 24. NR_ext_to_71GHz | 24-8 | 32 DL HARQ processes [for FR 2-2] | Support 32 HARQ processes in DL [for 480/960 kHz] |  |  |  |  |  |  |  |  |  | Optional with capability signalling |
| 24. NR_ext_to_71GHz | 24-9 | 32 UL HARQ processes [for FR 2-2] | Support 32 HARQ processes in UL [for 480/960 kHz] |  |  |  |  |  |  |  |  |  | Optional with capability signalling |

On this issue, we are open for either way regarding some components captured currently. For example, component #2 and #3 are the features supported in WI discussion to deal with regional regulations. Since it may not be essential in all the regions, separate FGs could be considered. However, we believe some other components should be defined as the basic feature, rather than separate ones, such as component #4 and #5. In component #4 and #5, the use of 120 kHz SCS for data and control channels and reference signals are captured for DL and UL, respectively. We do not see any need to separate DL and UL for such cases. It rather makes capability reporting more redundant without any motivation. For component#6, although we clearly agreed to support multi-PUSCH/PDSCH scheduling by single DCI with 120 kHz SCS, it may not be an essential feature in case of 120 kHz SCS as multi-slot PDCCH monitoring is not considered. Also, any feature regarding the operation in unlicensed bands will be optional as well as Cat-3 LBT support. Given the situation, we think component #6 can be defined as a separate optional feature. With those in mind, we support to include component #1, #4,　and #5 in basic feature FG24-1.

Another aspect related to FG24-1 is whether to include initial access aspect, which is also related to FG24-2. We think the current wording in FG24-1 is a bit unclear. It may be interpreted as “both initial access procedures and non-initial access procedure are supported” or “to support initial access related signals/channels”. Our understanding is this sentence is to show the support of whether initial and/or non-initial access procedures are supported. Therefore, some wordings, e.g., “procedure”, should be added. If the bracket is removed with keeping the sentence as it is, initial access procedure with 120 kHz SCS will be a basic feature, which implies that the current FG24-2 should be removed. If we keep FG24-2 as a separate one as in the current list, we have to remove “initial/” within the bracket in FG24-1 to remove the bracket, which implies initial access procedure with 120 kHz SCS will be an optional feature. On this issue, while we prefer to reduce the number of FGs in general, assuming LAA-like use case together with licensed band(s), a separate feature for initial access with 120 kHz SCS may be a proper way in our view.

Whether to have finer separation should also be discussed for FG24-4 and FG24-5, defining the support of 480 and 960 kHz SCS, respectively. We think it would be straightforward to follow the direction considered for 120 kHz SCS in FG24-1.

Proposal 2: Regarding FG24-1 (to be basic feature for FR2-2 support):

- Components #2 and #3 could be defined separately

- Components #1, #4, and #5 should be included

Proposal 3: Regarding FG24-2 (to be a feature of initial access support with 120 kHz SCS):

- It should be kept as it is, and the wording “initial/” within the bracket in component #1 of FG24-1 should be removed

For FG24-6, LBT support was captured, while whether to include Cat-4 LBT was unclear. On this issue, RAN1 reached conclusions deciding no support of CWS adjustment and CAPC, which automatically means no support of Cat-4 LBT in our understanding. Thus, Cat-4 LBT should be removed from FG24-6.

Proposal 4: Regarding FG24-6 (to be a feature of LBT support):

- Cat-4 LBT should be removed

2.3. Views on applicability of Rel-15/16 NR UE features to FR2-2

In Appendix, we show a brief set of analysis regarding Rel-15/16 UE features in terms of applicability to 52.6 – 71 GHz frequency range, based on the UE features specified in 38.822 [2], where we have focused with the principles below:

- Check mandatory UE features in Rel-15/16 if it is applicable to 52.6 – 71 GHz frequency range

- Check UE features with per-UE signalling if it is applicable to 52.6 – 71 GHz frequency range when it is reported applicable to FR2

- For UE features with per-FR capability signalling, we have not analysed yet since it may be straightforward that per-FR signalling will indicate sub-FR level applicability, although it needs further discussions

- For UE features with per-band or per-BC capability signalling, we have not analysed yet since per-band/BC capability signalling is finer granularity than FR-level differentiation

Below are some particular aspects that may require discussions

2.3.1 On mandatory UE features

Some UE features are defined as mandatory for NR in Rel-15. It has to be supported even for UEs supporting Rel-17 functionalities. However, they didn’t consider the operation in 52.6 – 71 GHz when specified, especially with larger SCSs. Therefore, some UE features, even the ones defined as mandatory in Rel-15/16, may or may not be feasible in case of the operation in 52.6 – 71 GHz frequency range.

One potential issue among the mandatory features is related to FG3-1 on basic DL control channel, with the following components:

1) One configured CORESET per BWP per cell in addition to CORESET0 - CORESET resource allocation of 6RB bit-map and duration of 1 – 3 OFDM symbols for FR1 - For type 1 CSS without dedicated RRC configuration and for type 0, 0A, and 2 CSSs, CORESET resource allocation of 6RB bit-map and duration 1-3 OFDM symbols for FR2 - For type 1 CSS with dedicated RRC configuration and for type 3 CSS, UE specific SS, CORESET resource allocation of 6RB bit-map and duration 1-2 OFDM symbols for FR2 - REG-bundle sizes of 2/3 RBs or 6 RBs - Interleaved and non-interleaved CCE-to-REG mapping - Precoder-granularity of REG-bundle size - PDCCH DMRS scrambling determination - TCI state(s) for a CORESET configuration 2) CSS and UE-SS configurations for unicast PDCCH transmission per BWP per cell - PDCCH aggregation levels 1, 2, 4, 8, 16 - UP to 3 search space sets in a slot for a scheduled SCell per BWP - This search space limit is before applying all dropping rules. - For type 1 CSS with dedicated RRC configuration, type 3 CSS, and UE-SS, the monitoring occasion is within the first 3 OFDM symbols of a slot - For type 1 CSS without dedicated RRC configuration and for type 0, 0A, and 2 CSS, the monitoring occasion can be any OFDM symbol(s) of a slot, with the monitoring occasions for any of Type 1- CSS without dedicated RRC configuration, or Types 0, 0A, or 2 CSS configurations within a single span of three consecutive OFDM symbols within a slot 3) Monitoring DCI formats 0_0, 1_0, 0_1, 1_1 4) Number of PDCCH blind decodes per slot with a given SCS follows Case 1-1 table 5) Processing one unicast DCI scheduling DL and one unicast DCI scheduling UL per slot per scheduled CC for FDD 6) Processing one unicast DCI scheduling DL and 2 unicast DCI scheduling UL per slot per scheduled CC for TDD

For UEs supporting NR from 52.6 GHz to 71 GHz, at least the operation with 120 kHz SCS is to be supported. As 120 kHz SCS is the one which was supported in Rel-15/16 NR already, it does not cause any issue to support the existing UE features, including the ones specified as mandatory, e.g., FR3-1.

However, the UEs supporting NR in 52.6 – 71 GHz may support larger SCS(s), i.e., 480 and/or 960 kHz SCS, as well in order to achieve the operation with larger absolute bandwidth per CBW. Since symbol duration is scaled based on SCS in the same manner as in FR1 and FR2, and definition of slot is same as in Rel-15/16 NR, the operation with 480/960 kHz SCS automatically means the one with shortened duration of a slot.

When operating with shortened duration of a slot by supporting 480 and/or 960 kHz SCS, some components supported as mandatory in FG3-1 may not be feasible. For example, in the second component, up to 3 search space sets in a slot for a scheduled SCell per BWP is supported. Since NR in 52.6 – 71 GHz will be operated with SCS of 120 kHz or larger, whether up to 3 SS sets in a slot is always possible may not be clear. Moreover, in the sixth component, per-slot and per-CC maximum limitation of DCI processing is described, where one unicast DCI scheduling DL and 2 unicast DCI scheduling UL are supported. The feasibility of this may also be affected by shortened duration of a slot, e.g., only smaller number of DCIs may be possible for UE to process per slot with shortened time duration.

Given above, we propose to discuss on how to interpret FG3-1 for the operation with SCS of 480 and/or 960 kHz. We see some alternatives to deal with the situation can be considered; one is to add a Note in a new UE feature to support 480 and 960 kHz SCS in 52.6 – 71 GHz frequency range such as “[a certain components of] FG 3-1 is not applicable to the SCS supported by this FG”.

Proposal 5: For UEs supporting NR in 52.6 – 71 GHz frequency range, how to treat a mandatory UE feature, FG 3-1, should be discussed at least when the UE supports the operation with 480 and/or 960 kHz SCS

2.3.2 On UE features with per-UE capability signalling

As well as mandatory UE features, UE features with per-UE capability signalling also need to be checked in terms of their applicability to the operation in 52.6 – 71 GHz. When UEs report their support of a certain UE feature with per-UE capability signalling, NW will understand that the UE supports the feature regardless of the operating band, frequency range (or even duplex). However, it may not always the case that UE features with per-UE capability signalling are applicable to 52.6 – 71 GHz when it is applicable to the existing frequency ranges.

Our brief analysis is shown on the 6th column on the tables in Appendix. We generally believe most of the UE features with per-UE capability signalling are also applicable to FR2-2 as well. On the other hand, to support such UE features in practice in 52.6 – 71 GHz frequency range, some maintenances in the specifications will be needed, especially in terms of SCS. For example, FG 12-6 is a UE feature with per-UE capability signalling to report whether the UE supports DL SPS with the periodicity shorter than 10 ms. In Rel-16, an RRC parameter periodicityExt-r16 is supported for configuring DL SPS periodicity shorter than 10 ms. However, how to use the value configured via periodicityExt-r16 has not been defined in case that larger SCS than 120 kHz is configured. In other words, even if the UE feature reporting is supported for NR in 52.6 – 71 GHz as it is, when larger SCS than 120 kHz is used, this functionality (i.e., DL SPS with shorter than 10 ms periodicity) cannot be configured in practice.

Observation 1: While most of Rel-15/16 UE features with per-UE capability signalling can be reused as they are for UE to report their support for NR in 52.6 – 71 GHz, some maintenances will be required in the specifications to support the functionalities in practice.

Proposal 6: For Rel-15/16 UE features with per-UE capability signalling, whether to be applicable to FR2-2 when they are reported as applicable should be analysed a case-by-case manner

2.3.3 On UE features with per-FR/band/BC capability signalling

There would be other types of UE features in NR in terms of FR differentiation, that is, UE features with per-FR/band/BC capability signalling. For the ones with per band/BC capability signalling, we do not see the need to check their validity since per-band/BC signalling naturally differentiate FR2-2 as well as the other FRs. Thus we do not incorporate them with the table in Appendix.

On the other hand, some UE features with per-band/BC signalling include a Note associated with FR and/or whether it is licensed or unlicensed. For example, FG22-6/7 on PUCCH grouping, there are the descriptions on carrier type, which differentiate between the existing FRs and licensed/unlicensed band for FR1. Here, whether “FR2” can include 52.6 – 71 GHz or not is unclear at this stage. Moreover, as 52.6 – 71 GHz includes unlicensed bands, differentiation between licensed/unlicensed band may be required even if the wording “FR2” is kept as it is.

Proposal 7: For Rel-15/16 UE features with per-FR capability signalling,

- If FR-related description is included in e.g., component, whether/how to consider 52.6 – 71 GHz may need to be discussed.

- Otherwise, as it can naturally differentiate FR2-2 from other FRs, there is no need to discuss in terms on FR2-2

The ones with per-FR capability signalling may not have any issue either since Rel-15/16 defines FR2 as a frequency range between 24.25 – 52.6 GHz. Also, even if FR2-2 is additionally considered, as well as FR1/2-1 differentiation which has already been done via per-FR capability signalling, FR2-2 will need to be differentiated from the other FRs in many cases. Given that, we have not analysed yet on the ones with per-FR capability signalling on the tables in Appendix.

An issue which may be lying on the ones with per-FR capability signalling would be whether to be applicable when they are reported as applicable to FR2 if no differentiation between FR2-1 and FR2-2 is considered. Some could be applicable to FR2-2 in the same manner as to FR2-1, while some others may not. This issue may also need to be checked in a case-by-case basis. We think it should also be discussed in RAN1 in the future.

Proposal 8: For Rel-15/16 UE features with per-FR capability signalling, how to treat when it is reported as applicable to FR2 should be discussed

- Option 1: Differentiation between FR2-1 and FR2-2 is introduced

- Option 2: All the UE features are treated as applicable or inapplicable to FR2-2 as well as FR2-1 when it is reported for FR2, while the ones for which such treatment cannot be appropriate are defined as exceptional cases via e.g., adding Note

- Conclusion

Our proposals/observations are summarized as follows:

Proposal 1: For the discussion on Rel-17 UE features at least regarding 52.6 – 71 GHz WI, the following alternatives can be considered in case-by-case manner, in terms of FR differentiation.

- Alt 1: define as per-band

- Alt 2: define as per-FR

  - Differentiation of FR2-1/2-2 may or may not be needed

- Alt 3: define as per-UE

  - A fixed limitation (e.g., as a Note) on applicable frequency range may be needed

Proposal 2: Regarding FG24-1 (to be basic feature for FR2-2 support):

- Components #2 and #3 could be defined separately

- Components #1, #4, #5 and #6 should be included

Proposal 3: Regarding FG24-2 (to be a feature of initial access support with 120 kHz SCS):

- It should be kept as it is, and the wording “initial/” within the bracket in component #1 of FG24-1 should be removed

Proposal 4: Regarding FG24-6 (to be a feature of LBT support):

- Cat-4 LBT should be removed

Proposal 5: For UEs supporting NR in 52.6 – 71 GHz frequency range, how to treat a mandatory UE feature, FG 3-1, should be discussed at least when the UE supports the operation with 480 and/or 960 kHz SCS

Observation 1: While most of Rel-15/16 UE features with per-UE capability signalling can be reused as they are for UE to report their support for NR in 52.6 – 71 GHz, some maintenances will be required in the specifications to support the functionalities in practice.

Proposal 6: For Rel-15/16 UE features with per-UE capability signalling, whether to be applicable to FR2-2 when they are reported as applicable should be analysed a case-by-case manner

Proposal 7: For Rel-15/16 UE features with per-FR capability signalling,

- If FR-related description is included in e.g., component, whether/how to consider 52.6 – 71 GHz may need to be discussed.

- Otherwise, as it can naturally differentiate FR2-2 from other FRs, there is no need to discuss in terms on FR2-2

Proposal 8: For Rel-15/16 UE features with per-FR capability signalling, how to treat when it is reported as applicable to FR2 should be discussed

- Option 1: Differentiation between FR2-1 and FR2-2 is introduced

- Option 2: All the UE features are treated as applicable or inapplicable to FR2-2 as well as FR2-1 when it is reported for FR2, while the ones for which such treatment cannot be appropriate are defined as exceptional cases via e.g., adding Note

- Appendix

4.1. the latest UE feature list for supporting NR from 52.6 to 71 GHz

- NR_ext_to_71GHz

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (Sidelink WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 24. NR_ext_to_71GHz | 24-1 | General Basic FR2-2 [DL] support | 1. Support 120KHz SCS [transmission and] reception for [initial/non-initial access] [2. Support multi-RB PUCCH format 0/1/4 for 120 kHz] [3. PRACH with 120KHz SCS and length 139[/571/1151]] [4. Support 120kHz subcarrier spacing for DL data and control channels and reference signals in FR2-2] [5. Support 120kHz subcarrier spacing for UL data and control channels and reference signals in FR2-2] [6. Support multi-PUSCH[/PDSCH] scheduling by single DCI for the operation with 120 kHz SCS] |  |  |  | FR2-2 is not supported | [per UE][per band] |  |  |  |  | Optional with capability signalling [A UE that supports FR2-2 must indicate this FG is supported] |
| 24. NR_ext_to_71GHz | 24-1a | FR2-2 PRACH support | 1. PRACH with 120KHz SCS and length 139[/571/1151] | 24-1 |  |  |  | [per UE][per band] |  |  |  |  | Optional with capability signalling |
| 24. NR_ext_to_71GHz | 24-1b | Basic FR2-2 UL support | 1. Support multi-RB PUCCH format 0/1/4 for 120 kHz [2. Support 120kHz subcarrier spacing for UL data and control channels and reference signals in FR2-2] | [24-1] |  |  |  | [per UE][per band] |  |  |  |  | Optional with capability signalling |
| 24. NR_ext_to_71GHz | 24-1c | Multi-PDSCH scheduling | 1. Support multi-PDSCH scheduling by single DCI for the operation with 120 kHz SCS | 24-1 |  |  |  | [per UE][per band] |  |  |  |  | Optional with capability signalling |
| 24. NR_ext_to_71GHz | 24-1d | Multi-PUSCH scheduling | 1. Support multi-PUSCH scheduling by single DCI for the operation with 120 kHz SCS | 24-1b |  |  |  | [per UE][per band] |  |  |  |  | Optional with capability signalling |
| 24. NR_ext_to_71GHz | 24-2 | 120KHz SSB based stand-alone support in FR2-2 | 1. Support 480120KHz SSB for initial access in FR2-2 | 24-1 |  |  |  |  |  |  |  |  | Optional with capability signalling |
| 24. NR_ext_to_71GHz | 24-3 | 480KHz SSB based stand-alone support for initial access in FR2-2 | 1. Support 480KHz SSB for initial access in FR2-2 | 24-1[, 24-2, 24-4] | Yes FFS |  |  | [Per UE/band] |  |  |  | From WID: - In addition to 120kHz, support 480 kHz SSB for initial access with support of CORESET#0/Type0-PDCCH configuration in the MIB with following constraints: - Note: 480 kHz is an optional SSB numerology for initial access for the UE. A UE supporting a band in 52.6-71 GHz must at least support 120 kHz SCS (for initial access and after initial access) - [only 480kHz CORESET#0/Type0-PDCCH SCS supported for 480 kHz SSB SCS] | Optional with capability signalling |
| 24. NR_ext_to_71GHz | 24-4 | 480KHz SCS support | 1. 480KHz SCS for UL data and control channels and reference signal transmission in FR2-2 2. 480KHz SCS for DL data and control channels and reference signal reception in FR2-2 3. 480KHz for SSB monitoring [for non-initial access] 4. Multiple-slot PDCCH monitoring for 480KHz with X=[4] slots [FFS: Component description to be updated once further details of multi-slot monitoring capability are known, e.g., definition of Y] 5. PRACH with 480KHz and length 139/[571] FFS: 6. Support multi-RB PUCCH format 0/1/4 for 480 kHz FFS: 7. Multi-PUSCH/PDSCH scheduling by single DCI for the operation with 480 kHz SCS | 24-1 | Yes |  |  | [Per UE/band] |  |  |  | From WID: In addition to 120kHz SCS, specify new SCS, 480kHz and 960kHz, and define maximum bandwidth(s), for operation in this frequency range for data and control channels and reference signals, only NCP supported. [Agreement: A UE supporting 480 kHz SCS supports multi-slot PDCCH monitoring for 480 kHz SCS. Agreement: Do not support PRACH length L=571, 1151 for 960kHz PRACH and at least L =1151 for 480kHz PRACH] Note: • Resolve the issues of wideband PRACH, multi-RB PUCCH format 0/1/4, and multi-PUSCH/PDSCH scheduling by single DCI, i.e., whether to have components of a single FG or separate FGs, for 120 kHz first, then use the same structure for 480 kHz • Resolve the issue of having separate capabilities for DL and UL (data and control channels as well as reference signals) for 120 kHz first, then use the same structure for 480 kHz | Optional with capability signalling |
| 24. NR_ext_to_71GHz | 24-5 | 960KHz SCS support | 1. 960KHz SCS for UL data and control channels and reference signal transmission in FR2-2 2. 960KHz SCS for DL data and control channels and reference signal reception in FR2-2 3. 960KHz for SSB monitoring 4. Multiple-slot PDCCH monitoring for 960KHz with X=[8] slots [FFS: Component description to be updated once further details of multi-slot monitoring capability are known, e.g., definition of Y] 5. PRACH with 960KHz and length 139 FFS: 6. Support multi-RB PUCCH format 0/1/4 for 960 kHz FFS: 7. Multi-PUSCH/PDSCH scheduling by single DCI for the operation with 960 kHz SCS | 24-1 | Yes |  |  | [Per UE/band] |  |  |  | From WID - In addition to 120kHz SCS, specify new SCS, 480kHz and 960kHz, and define maximum bandwidth(s), for operation in this frequency range for data and control channels and reference signals, only NCP supported. [Agreement: A UE supporting 960 kHz SCS supports multi-slot PDCCH monitoring for 960 kHz SCS. Agreement: Do not support PRACH length L=571, 1151 for 960kHz PRACH and at least L =1151 for 480kHz PRACH] Note: • Resolve the issues of wideband PRACH, multi-RB PUCCH format 0/1/4, and multi-PUSCH/PDSCH scheduling by single DCI, i.e., whether to have components of a single FG or separate FGs, for 120 kHz first, then use the same structure for 480 kHz • Resolve the issue of having separate capabilities for DL and UL (data and control channels as well as reference signals) for 120 kHz first, then use the same structure for 480 kHz | Optional with capability signalling |
| 24. NR_ext_to_71GHz | 24-?6 | Cat 3 or Cat 4 LBT support Uplink channel access procedure for FR2-2 unlicensed operation | - Cat 3 [or Cat 4] LBT support [(not agreed yet if CW is supported, so it can be either Cat 3 or Cat 4 LBT for now. Will update when we have agreement)] - [Support LBT performed per carrier/BWP bandwidth] | 24-1 |  |  |  | [Per band] |  |  |  |  | Optional with capability signalling [A UE that supports FR2-2 must indicate this FG is supported when required by regulation] |
| 24. NR_ext_to_71GHz | 24-?7 | Cat 2 LBT support for uplink channel access procedure for FR2-2 unlicensed operation | 1. Support Cat 2 LBT | 24-1[, 24-6] |  |  |  | [Per band] |  |  |  |  | Optional with capability signalling [A UE that supports FR2-2 must indicate this FG is supported when required by regulation] |
| 24. NR_ext_to_71GHz | 24-8 | 32 DL HARQ processes [for FR 2-2] | Support 32 HARQ processes in DL [for 480/960 kHz] |  |  |  |  |  |  |  |  |  | Optional with capability signalling |
| 24. NR_ext_to_71GHz | 24-9 | 32 UL HARQ processes [for FR 2-2] | Support 32 HARQ processes in UL [for 480/960 kHz] |  |  |  |  |  |  |  |  |  | Optional with capability signalling |

4.2. Rel-15 UE feature list for supporting NR from 52.6 to 71 GHz

4.1.1. Initial access and mobility

| Index | FG | Prerequisite feature groups | Field name | Parent IE | Assessment |
| --- | --- | --- | --- | --- | --- |
| 1-1 | Basic initial access channels and procedures |  | n/a | n/a | Ok with no change |
| 1-2 | SS block based SINR measurement (SS-SINR) | 1-1 | ss-SINR-Meas | MeasAndMobParametersFRX-Diff |  |
| 1-3 | SS block based RLM | 1-1 | ssb-RLM | MeasAndMobParametersCommon | Ok to keep common (for licensed band only) |
| 1-4 | CSI-RS based RRM measurement with associated SS-block | 1-1, CSI-RS | csi-RSRP-AndRSRQ-MeasWithSSB | MeasAndMobParametersFRX-Diff | (for licensed band only) |
| 1-5 | CSI-RS based RRM measurement without associated SS-block | 1-1, CSI-RS | csi-RSRP-AndRSRQ-MeasWithoutSSB | MeasAndMobParametersFRX-Diff | (for licensed band only) |
| 1-6 | CSI-RS based RS-SINR measurement | 1-1, 1-4 | csi-SINR-Meas | MeasAndMobParametersFRX-Diff | (for licensed band only) |
| 1-7 | CSI-RS based RLM | 1-1, CSI-RS | csi-RS-RLM | MeasAndMobParametersFRX-Diff | (for licensed band only) |
| 1-8 | RLM based on a mix of SS block and CSI-RS signals within active BWP | 1-3 and 1-7 | ssb-AndCSI-RS-RLM | MeasAndMobParametersCommon | Ok to keep common (for licensed band only) |
| 1-9 | CSI-RS based contention free RA for HO | 1-1, CSI-RS, 1-4 or 1-5 | csi-RS-CFRA-ForHO | Phy-ParametersCommon | Ok to keep common (for licensed band only) |
| 1-12 | E-UTRA RS-SINR measurement |  | rs-SINR-MeasEUTRA | EUTRA-ParametersCommon | Ok to keep common |
| 1-13 | Maximal number of CSI-RS resources for RRM and RS-SINR measurement across all measurement frequencies per slot | 1-4 or 1-5 or 1-6 | maxNumberCSI-RS-RRM-RS-SINR | MeasAndMobParametersCommon | Ok to keep common |
| 1-14 | Maximal number of CSI-RS resources within a slot per PCell/PSCell for CSI-RS based RLM | 1-7 or 1-8 | maxNumberResource-CSI-RS-RLM | MeasAndMobParametersFRX-Diff |  |

4.1.2. MIMO

| Index | FG | Prerequisite feature groups | Field name | Parent IE | Assessment |
| --- | --- | --- | --- | --- | --- |
| 2-1 | Basic PDSCH reception |  | n/a | n/a | Ok to keep mandatory w/o capability signaling |
| 2-5 | Basic downlink DMRS for scheduling type A | 2-1 | n/a | n/a | Ok to keep mandatory w/o capability signaling |
| 2-6 | Basic downlink DMRS for scheduling type B |  | n/a | n/a | Ok to keep mandatory w/o capability signaling |
| 2-7 | Supported 2 symbols front-loaded DMRS (downlink) | 2-5 | twoFL-DMRS (MSB) | Phy-ParametersFRX-Diff |  |
| 2-10 | Support DMRS type (downlink) |  | supportedDMRS-TypeDL | Phy-ParametersFRX-Diff |  |
| 2-11 | Downlink dynamic PRB bundling (downlink) | 2-1 | dynamicPRB-BundlingDL | Phy-ParametersCommon | Ok to keep common |
| 2-12 | Basic PUSCH transmission |  | n/a | n/a | Ok to keep mandatory w/o capability signaling |
| 2-16 | Basic uplink DMRS (uplink) for scheduling type A |  | n/a | n/a | Ok to keep mandatory w/o capability signaling |
| 2-16a | Basic uplink DMRS for scheduling type B |  | n/a | n/a | Ok to keep mandatory w/o capability signaling |
| 2-16b | Support 1+2 DMRS (uplink) | 2-16a and 2-16 | oneFL-DMRS-TwoAdditionalDMRS-UL | Phy-ParametersFRX-Diff |  |
| 2-17 | Support DMRS type (uplink) | 2-16 | supportedDMRS-TypeUL | Phy-ParametersFRX-Diff |  |
| 2-18 | Supported 2 symbols front-loaded DMRS (uplink) | 2-16 | twoFL-DMRS (LSB) | Phy-ParametersFRX-Diff |  |
| 2-18a | Supported 2 symbols front-loaded +2 symbols additional DMRS (uplink) | 2-16 | twoFL-DMRS-TwoAdditionalDMRS-UL | Phy-ParametersFRX-Diff |  |
| 2-19 | Support 1+3 uplink DMRS symbols(uplink) | 2-16 | oneFL-DMRS-ThreeAdditionalDMRS-UL | Phy-ParametersFRX-Diff |  |
| 2-32 | Basic CSI feedback |  | n/a | n/a | Ok to keep (but needs further check) |
| 2-32a | Semi-persistent CSI report on PUCCH |  | sp-CSI-ReportPUCCH | Phy-ParametersCommon | Ok to keep common |
| 2-32b | Semi-persistent CSI report on PUSCH |  | sp-CSI-ReportPUSCH | Phy-ParametersCommon | Ok to keep common |
| 2-32c | New CQI table |  | cqi-TableAlt | Phy-ParametersFRX-Diff |  |
| 2-33a | Supported PDSCH RE-mapping patterns |  | 1. pdsch-RE-MappingFR1-PerSymbol 2. pdsch-RE-MappingFR1-PerSlot | Phy-ParametersFR1 |  |
|  |  |  | 1. pdsch-RE-MappingFR2-PerSymbol 2. pdsch-RE-MappingFR2-PerSlot | Phy-ParametersFR2 | Ok to keep the whole FR2 |
| 2-33b | SP CSI-RS | 2-1 | sp-CSI-RS | Phy-ParametersFRX-Diff |  |
| 2-33c | SP CSI-IM | 2-1 | sp-CSI-IM | Phy-ParametersFRX-Diff |  |
| 2-34 | NZP-CSI-RS based interference measurement | 2-33 | nzp-CSI-RS-IntefMgmt | Phy-ParametersCommon | Ok to keep common |
| 2-37 | Support Semi-open loop CSI | 2-35 | semiOpenLoopCSI | Phy-ParametersFRX-Diff |  |
| 2-38 | CSI report without PMI | 2-35 | csi-ReportWithoutPMI | Phy-ParametersFRX-Diff |  |
| 2-39a | CSI report without CQI | 2-35 | csi-ReportWithoutCQI | Phy-ParametersFRX-Diff |  |
| 2-42 | Support Type II SP-CSI feedback on long PUCCH | 2-41 | type2-SP-CSI-Feedback-LongPUCCH | Phy-ParametersCommon | Ok to keep common |
| 2-44 | Basic DL PTRS |  | onePortsPTRS (MSB) | Phy-ParametersFRX-Diff |  |
| 2-47 | Basic UL PTRS |  | onePortsPTRS (LSB) | Phy-ParametersFRX-Diff |  |
| 2-50 | Basic TRS |  | n/a | n/a | Ok to keep as mandatory w/o signaling |
| 2-52 | Basic SRS |  | n/a | n/a | Ok to keep as mandatory w/o signaling |

4.1.3. DL control channel and procedure

| Index | FG | Prerequisite feature groups | Field name | Parent IE | Assessment |
| --- | --- | --- | --- | --- | --- |
| 3-1 | Basic DL control channel |  | n/a | n/a | How to treat Component (2) and (6) should be discussed when larger SCSs are supported |
| 3-2 | PDCCH monitoring on any span of up to 3 consecutive OFDM symbols of a slot |  | pdcchMonitoringSingleOccasion | Phy-ParametersFR1 |  |
| 3-3 | More than one CORESET configurations per BWP in addition to CORESET0 |  | multipleCORESET | Phy-ParametersFRX-Diff |  |
| 3-6 | Dynamic SFI monitoring |  | dynamicSFI | Phy-ParametersXDD-Diff Phy-ParametersFRX-Diff |  |
| 3-7 | Precoder-granularity of CORESET size |  | precoderGranularityCORESET | Phy-ParametersCommon | Ok to keep as common |
| 3-8 | Up to 10 search spaces in a SCell |  | maxNumberSearchSpaces | Phy-ParametersCommon | Ok to keep as common |

4.1.4. UL control channel and procedure

| Index | FG | Prerequisite feature groups | Field name | Parent IE | Assessment |
| --- | --- | --- | --- | --- | --- |
| 4-1 | Basic UL control channel |  | n/a | n/a | Ok to keep mandatory without signaling |
| 4-2 | 2 PUCCH of format 0 or 2 in consecutive symbols |  | twoPUCCH-F0-2-ConsecSymbols | Phy-ParametersXDD-Diff Phy-ParametersFRX-Diff |  |
| 4-3 | PUCCH format 2 over 1 – 2 OFDM symbols once per slot with frequency hopping as "enabled" |  | pucch-F2-WithFH | Phy-ParametersFRX-Diff |  |
| 4-4 | PUCCH format 3 over 4 – 14 OFDM symbols once per slot with frequency hopping as "enabled" |  | pucch-F3-WithFH | Phy-ParametersFRX-Diff |  |
| 4-5 | PUCCH format 4 over 4 – 14 OFDM symbols once per slot with frequency hopping as "enabled" |  | pucch-F4-WithFH | Phy-ParametersFRX-Diff |  |
| 4-6 | Non-frequency hopping for PUCCH formats 0 and 2 with frequency hopping as "disabled" |  | freqHoppingPUCCH-F0-2 | Phy-ParametersFRX-Diff |  |
| 4-7 | Non-frequency hopping for PUCCH format 1, 3, and 4 with frequency hopping as "disabled" |  | freqHoppingPUCCH-F1-3-4 | Phy-ParametersFRX-Diff |  |
| 4-10 | Dynamic HARQ-ACK codebook |  | dynamicHARQ-ACK-Codebook | Phy-ParametersCommon | Ok to keep common |
| 4-11 | Semi-static HARQ-ACK codebook |  | semiStaticHARQ-ACK-Codebook | Phy-ParametersCommon | Ok to keep common |
| 4-12 | HARQ-ACK spatial bundling for PUCCH or PUSCH per PUCCH group |  | spatialBundlingHARQ-ACK | Phy-ParametersCommon | Ok to keep common |
| 4-13 | More than one SR configurations per PUCCH group |  | multipleSR-Configurations | MAC-ParametersXDD-Diff |  |
| 4-19 | SR/HARQ-ACK/CSI multiplexing once per slot using a PUCCH (or HARQ-ACK/CSI piggybacked on a PUSCH) when SR/HARQ-ACK/CSI are supposed to be sent with the same starting symbol on the PUCCH resources in a slot |  | sameSymbol in mux-SR-HARQ-ACK-CSI-PUCCH-OncePerSlot | Phy-ParametersFRX-Diff |  |
| 4-19a | SR/HARQ-ACK multiplexing once per slot using a PUCCH (or HARQ-ACK piggybacked on a PUSCH) when SR/HARQ-ACK are supposed to be sent with different starting symbols in a slot | 4-19 | mux-SR-HARQ-ACK-PUCCH | Phy-ParametersFRX-Diff |  |
| 4-19b | SR/HARQ-ACK/CSI multiplexing more than once per slot using a PUCCH (or HARQ-ACK/CSI piggybacked on a PUSCH) when SR/HARQ-ACK/CSI are supposed to be sent with the same or different starting symbol in a slot | 4-19c | mux-SR-HARQ-ACK-CSI-PUCCH-MultiPerSlot | Phy-ParametersFRX-Diff |  |
| 4-19c | SR/HARQ-ACK/CSI multiplexing once per slot using a PUCCH (or HARQ-ACK/CSI piggybacked on a PUSCH) when SR/HARQ-ACK/CSI are supposed to be sent with different starting symbols in a slot | 4-19a | diffSymbol in mux-SR-HARQ-ACK-CSI-PUCCH-OncePerSlot | Phy-ParametersFRX-Diff |  |
| 4-20 | UCI code-block segmentation |  | uci-CodeBlockSegmentation | Phy-ParametersFRX-Diff |  |
| 4-21 | Dynamic beta-offset configuration and indication for HARQ-ACK and/or CSI |  | dynamicBetaOffsetInd-HARQ-ACK-CSI | Phy-ParametersCommon | Ok to keep common |
| 4-22 | 1 PUCCH format 0 or 2 and 1 PUCCH format 1, 3, or 4 in the same slot |  | onePUCCH-LongAndShortFormat | Phy-ParametersFRX-Diff |  |
| 4-22a | 2 PUCCH transmissions in the same slot which are not covered by 4-22 and 4-2 |  | twoPUCCH-AnyOthersInSlot | Phy-ParametersFRX-Diff |  |
| 4-23 | Repetitions for PUCCH format 1, 3, and 4 over multiple slots with K = 2, 4, 8 |  | pucch-Repetition-F1-3-4 | Phy-ParametersCommon | Ok to keep common |
| 4-27 | More than one group of overlapping channels for control multiplexing |  | mux-MultipleGroupCtrlCH-Overlap | Phy-ParametersFRX-Diff |  |
| 4-28 | HARQ-ACK multiplexing on PUSCH with different PUCCH/PUSCH starting OFDM symbols | 4-1 | mux-HARQ-ACK-PUSCH-DiffSymbol | Phy-ParametersFRX-Diff |  |

4.1.5. Scheduling/HARQ operation

| Index | FG | Prerequisite feature groups | Field name | Parent IE | Assessment |
| --- | --- | --- | --- | --- | --- |
| 5-1 | Basic scheduling/HARQ operation |  | n/a | n/a | Ok to keep as mandatory |
| 5-1b | More than one DL/UL switch point in a slot |  | tdd-MultiDL-UL-SwitchPerSlot | Phy-ParametersFRX-Diff |  |
| 5-2 | RA Type 0 for PUSCH |  | ra-Type0-PUSCH | Phy-ParametersCommon | Ok to keep common |
| 5-3 | Dynamic switching between RA Type 0 and RA Type 1 for PDSCH |  | dynamicSwitchRA-Type0-1-PDSCH | Phy-ParametersCommon | Ok to keep common |
| 5-4 | Dynamic switching between RA Type 0 and RA Type 1 for PUSCH | 5-2 | dynamicSwitchRA-Type0-1-PUSCH | Phy-ParametersCommon | Ok to keep common |
| 5-6 | PDSCH mapping type A with less than 7 OFDM symbols |  | pdsch-MappingTypeA | Phy-ParametersCommon | Ok to keep common |
| 5-6a | PDSCH mapping type B |  | pdsch-MappingTypeB | Phy-ParametersCommon | Ok to keep common |
| 5-7 | Interleaving for VRB-to-PRB mapping for PDSCH |  | interleavingVRB-ToPRB-PDSCH | Phy-ParametersCommon | Ok to keep common |
| 5-9 | Intra-slot frequency-hopping for PUSCH except for PUSCH scheduled by Type 1 CSS before RRC connection |  | intraSlotFreqHopping-PUSCH | Phy-ParametersFRX-Diff |  |
| 5-10 | Inter-slot frequency hopping for PUSCH |  | interSlotFreqHopping-PUSCH | Phy-ParametersCommon | Ok to keep common |
| 5-14 | Type 1 configured PUSCH repetitions over multiple slots |  | type1-PUSCH-RepetitionMultiSlots | Phy-ParametersCommon | Ok to keep common (for licensed band only) |
| 5-16 | Type 2 configured PUSCH repetitions over multiple slots |  | type2-PUSCH-RepetitionMultiSlots | Phy-ParametersCommon | Ok to keep common (for licensed band only) |
| 5-17 | PUSCH repetitions over multiple slots |  | pusch-RepetitionMultiSlots | Phy-ParametersCommon | Ok to keep common (for licensed band only) |
| 5-17a | PDSCH repetitions over multiple slots |  | pdsch-RepetitionMultiSlots | Phy-ParametersCommon | Ok to keep common (for licensed band only) |
| 5-18 | DL SPS |  | downlinkSPS | Phy-ParametersCommon | Ok to keep common (for licensed band only) |
| 5-19 | Type 1 Configured UL grant |  | configuredUL-GrantType1 | Phy-ParametersCommon | Ok to keep common (for licensed band only) |
| 5-20 | Type 2 Configured UL grant |  | configuredUL-GrantType2 | Phy-ParametersCommon | Ok to keep common (for licensed band only) |
| 5-21 | Pre-emption indication for DL |  | pre-EmptIndication-DL | Phy-ParametersCommon | Ok to keep common (for licensed band only) |
| 5-22 | CBG-based re-transmission for DL using CBGTI |  | cbg-TransIndication-DL | Phy-ParametersCommon | Ok to keep common |
| 5-23 | CBGFI for CBG-based re-transmission for DL | 5-22 | cbg-FlushIndication-DL | Phy-ParametersCommon | Ok to keep common |
| 5-24 | Dynamic HARQ-ACK codebook using sub-codebooks for CBG-based re-transmission for DL |  | dynamicHARQ-ACK-CodeB-CBG-Retx-DL | Phy-ParametersCommon | Ok to keep common |
| 5-25 | CBG-based re-transmission for UL using CBGTI |  | cbg-TransIndication-UL | Phy-ParametersCommon | Ok to keep common |
| 5-26 | Semi-static rate-matching resource set configuration for DL |  | rateMatchingResrcSetSemi-Static | Phy-ParametersCommon | Ok to keep common |
| 5-27 | Dynamic rate-matching resource set configuration for DL |  | rateMatchingResrcSetDynamic | Phy-ParametersCommon | Ok to keep common |
| 5-27a | Dynamic rate-matching control resource set for DL |  | rateMatchingCtrlResrcSetDynamic | Phy-ParametersCommon | Ok to keep common |
| 5-29 | LBRM for PUSCH |  | pusch-LBRM | Phy-ParametersFRX-Diff |  |
| 5-30 | DL scheduling slot offset greater than zero for PDSCH mapping type A |  | dl-SchedulingOffset-PDSCH-TypeA | Phy-ParametersXDD-Diff Phy-ParametersFRX-Diff |  |
| 5-30a | DL scheduling slot offset greater than zero for PDSCH mapping type B |  | dl-SchedulingOffset-PDSCH-TypeB | Phy-ParametersXDD-Diff Phy-ParametersFRX-Diff |  |
| 5-31 | UL scheduling slot offset greater than 12 |  | ul-SchedulingOffset | Phy-ParametersXDD-Diff Phy-ParametersFRX-Diff |  |
| 5-34 | New 64QAM MCS table for PDSCH |  | dl-64QAM-MCS-TableAlt | Phy-ParametersFRX-Diff |  |
| 5-34a | New 64QAM MCS table for PUSCH |  | ul-64QAM-MCS-TableAlt | Phy-ParametersFRX-Diff |  |

4.1.6. CA/DC, BWP, SUL

| Index | FG | Prerequisite feature groups | Field name | Parent IE | Assessment |
| --- | --- | --- | --- | --- | --- |
| 6-1 | Basic BWP operation with restriction |  | n/a | n/a | Ok to keep as mandatory without signaling |
| 6-5a | PDCCH blind detection capability for CA | 6-5 | pdcch-BlindDetectionCA | Phy-ParametersFRX-Diff | ok Note: FR differentiation is not supported now. |
| 6-25a | PDCCH blind detection capability for MCG and for SCG in synchronous NR-NR DC | 6-5, 6-25 | pdcch-BlindDetectionMCG-UE pdcch-BlindDetectionSCG-UE | Phy-ParametersFRX-Diff |  |

4.1.7. Channel coding

| Index | FG | Prerequisite feature groups | Field name | Parent IE | Assessment |
| --- | --- | --- | --- | --- | --- |
| 7-1 | Channel coding |  | n/a | n/a | Ok to keep as mandatory without signaling |

4.1.8. UL TPC

| Index | FG | Prerequisite feature groups | Field name | Parent IE | Assessment |
| --- | --- | --- | --- | --- | --- |
| 8-3 | Basic power control operation |  | n/a | n/a | Ok to keep as mandatory |
| 8-4 | TPC-PUSCH-RNTI |  | tpc-PUSCH-RNTI | Phy-ParametersFRX-Diff |  |
| 8-5 | TPC-PUCCH-RNTI |  | tpc-PUCCH-RNTI | Phy-ParametersFRX-Diff |  |
| 8-6 | TPC-SRS-RNTI |  | tpc-SRS-RNTI | Phy-ParametersFRX-Diff |  |
| 8-7 | Absolute TPC command mode |  | absoluteTPC-Command | Phy-ParametersFRX-Diff |  |
| 8-8 | UL power control with 2 PUSCH closed loops |  | twoDifferentTPC-Loop-PUSCH | Phy-ParametersXDD-Diff Phy-ParametersFRX-Diff |  |
| 8-9 | UL power control with 2 PUCCH closed loops |  | twoDifferentTPC-Loop-PUCCH | Phy-ParametersXDD-Diff Phy-ParametersFRX-Diff |  |

4.2. Rel-16 UE feature list for supporting NR from 52.6 to 71 GHz

4.2.1 NR_2step_RACH

| Index | FG | Prerequisite feature groups | Field name | Parent IE | Assessment |
| --- | --- | --- | --- | --- | --- |
| 9-1 | Basic channel structure and procedure of 2-step RACH |  | twoStepRACH-r16 | Phy-ParametersCommon | Ok to keep common |

4.2.2 NR_unlicensed

| Index | FG | Prerequisite feature groups | Field name | Parent IE | Assessment |
| --- | --- | --- | --- | --- | --- |
| 10-32 | SS block based SINR measurement (SS-SINR) for unlicensed spectrum |  | ss-SINR-Meas-r16 | Phy-ParametersSharedChAccess-r16 | Ok to keep applicable for the whole unlicensed band operation |
| 10-33 | Semi-persistent CSI report on PUCCH for unlicensed spectrum |  | sp-CSI-ReportPUCCH-r16 | Phy-ParametersSharedChAccess-r16 | Ok to keep applicable for the whole unlicensed band operation |
| 10-33a | Semi-persistent CSI report on PUSCH for unlicensed spectrum |  | sp-CSI-ReportPUSCH-r16 | Phy-ParametersSharedChAccess-r16 | Ok to keep applicable for the whole unlicensed band operation |
| 10-34 | Dynamic SFI monitoring for unlicensed spectrum |  | dynamicSFI-r16 | Phy-ParametersSharedChAccess-r16 | Ok to keep applicable for the whole unlicensed band operation |
| 10-35 | SR/HARQ-ACK/CSI multiplexing once per slot using a PUCCH (or HARQ-ACK/CSI piggybacked on a PUSCH) when SR/HARQ-ACK/CSI are supposed to be sent with the same starting symbol on the PUCCH resources in a slot for unlicensed spectrum |  | mux-SR-HARQ-ACK-CSI-PUCCH-OncePerSlot-r16 { sameSymbol-r16, diffSymbol-r16 } | Phy-ParametersSharedChAccess-r16 | Ok to keep applicable for the whole unlicensed band operation |
| 10-35a | SR/HARQ-ACK multiplexing once per slot using a PUCCH (or HARQ-ACK piggybacked on a PUSCH) when SR/HARQ-ACK are supposed to be sent with different starting symbols in a slot for unlicensed spectrum | 10-35 | mux-SR-HARQ-ACK-CSI-PUCCH-MultiPerSlot-r16 | Phy-ParametersSharedChAccess-r16 | Ok to keep applicable for the whole unlicensed band operation |
| 10-35b | SR/HARQ-ACK/CSI multiplexing more than once per slot using a PUCCH (or HARQ-ACK/CSI piggybacked on a PUSCH) when SR/HARQ-ACK/CSI are supposed to be sent with the same or different starting symbol in a slot for unlicensed spectrum | 10-35c | mux-SR-HARQ-ACK-CSI-PUCCH-MultiPerSlot-r16 | Phy-ParametersSharedChAccess-r16 | Ok to keep applicable for the whole unlicensed band operation |
| 10-35c | SR/HARQ-ACK/CSI multiplexing once per slot using a PUCCH (or HARQ-ACK/CSI piggybacked on a PUSCH) when SR/HARQ-ACK/CSI are supposed to be sent with different starting symbols in a slot for unlicensed spectrum | 10-35a | mux-SR-HARQ-ACK-CSI-PUCCH-OncePerSlot-r16 { sameSymbol-r16, diffSymbol-r16 } | Phy-ParametersSharedChAccess-r16 | Ok to keep applicable for the whole unlicensed band operation |
| 10-36 | HARQ-ACK multiplexing on PUSCH with different PUCCH/PUSCH starting OFDM symbols for unlicensed spectrum |  | mux-HARQ-ACK-PUSCH-DiffSymbol-r16 | Phy-ParametersSharedChAccess-r16 | Ok to keep applicable for the whole unlicensed band operation |
| 10-37 | Repetitions for PUCCH format 1, 3, and 4 over multiple slots with K = 2, 4, 8 for unlicensed spectrum |  | pucch-Repetition-F1-3-4-r16 | Phy-ParametersSharedChAccess-r16 | Ok to keep applicable for the whole unlicensed band operation |
| 10-38 | Type 1 configured PUSCH repetitions over multiple slots for unlicensed spectrum |  | type1-PUSCH-RepetitionMultiSlots-r16 | Phy-ParametersSharedChAccess-r16 | Ok to keep applicable for the whole unlicensed band operation |
| 10-39 | Type 2 configured PUSCH repetitions over multiple slots for unlicensed spectrum |  | type2-PUSCH-RepetitionMultiSlots-r16 | Phy-ParametersSharedChAccess-r16 | Ok to keep applicable for the whole unlicensed band operation |
| 10-40 | PUSCH repetitions over multiple slots for unlicensed spectrum |  | pusch-RepetitionMultiSlots-r16 | Phy-ParametersSharedChAccess-r16 | Ok to keep applicable for the whole unlicensed band operation |
| 10-40a | PDSCH repetitions over multiple slots for unlicensed spectrum |  | pdsch-RepetitionMultiSlots-r16 | Phy-ParametersSharedChAccess-r16 | Ok to keep applicable for the whole unlicensed band operation |
| 10-41 | DL SPS for unlicensed spectrum |  | downlinkSPS-r16 | Phy-ParametersSharedChAccess-r16 | Ok to keep applicable for the whole unlicensed band operation |
| 10-42 | Type 1 Configured UL grant for unlicensed spectrum |  | configuredUL-GrantType1-r16 | Phy-ParametersSharedChAccess-r16 | Ok to keep applicable for the whole unlicensed band operation |
| 10-43 | Type 2 Configured UL grant for unlicensed spectrum |  | configuredUL-GrantType2-r16 | Phy-ParametersSharedChAccess-r16 | Ok to keep applicable for the whole unlicensed band operation |
| 10-44 | Pre-emption indication for DL for unlicensed spectrum |  | pre-EmptIndication-DL-r16 | Phy-ParametersSharedChAccess-r16 | Ok to keep applicable for the whole unlicensed band operation |

4.2.3 NR_L1enh_URLLC

| Index | FG | Prerequisite feature groups | Field name | Parent IE | Assessment |
| --- | --- | --- | --- | --- | --- |
| 11-1 | Monitoring DCI format 1_2 and DCI format 0_2 |  | dci-Format1-2And0-2-r16 | Phy-ParametersCommon | Ok to keep common |
| 11-1a | Monitoring both DCI format 0_1/1_1 and DCI format 0_2/1_2 in the same search space | 11-1 | monitoringDCI-SameSearchSpace-r16 | Phy-ParametersCommon | Ok to keep common |
| 11-1b | Type 1 HARQ-ACK codebook support for relative TDRA for DL | 11-1 | type1-HARQ-ACK-Codebook-r16 | Phy-ParametersFRX-Diff |  |
| 11-4b | DL priority indication in DCI with mixed DCI formats | 11-1, 11-4 | dci-DL-PriorityIndicator-r16 | Phy-ParametersCommon | Ok to keep common |
| 11-6 | PUSCH repetition Type A | One of {5-16, 5-17] | pusch-RepetitionTypeA-r16 { sharedSpectrumChAccess-r16, non-SharedSpectrumChAccess-r16 } | Phy-ParametersCommon | Ok to keep common |
| 11-8 | Enhanced UL power control scheme |  | enhancedPowerControl-r16 | Phy-ParametersFRX-Diff |  |
| 11-10 | Type 2 configured grant release by DCI format 0_1 | 5-20 | type2-CG-ReleaseDCI-0-1-r16 | Phy-ParametersCommon | Ok to keep common |
| 11-11 | Type 2 configured grant release by DCI format 0_2 | 5-20, 11-1 | type2-CG-ReleaseDCI-0-2-r16 | Phy-ParametersCommon | Ok to keep common |
| 11-12 | CBG-based re-transmission for UL using CBGTI with only in-order CBG-based re-transmission(s) for cancelled initial PUSCH transmission |  | cbg-TransInOrderPUSCH-UL-r16 | Phy-ParametersCommon | Ok to keep common |

4.2.4 NR_IIOT

| Index | FG | Prerequisite feature groups | Field name | Parent IE | Assessment |
| --- | --- | --- | --- | --- | --- |
| 12-1a | UL priority indication in DCI with mixed DCI formats | 12-1 and 11-1 | dci-UL-PriorityIndicator-r16 | Phy-ParametersCommon | Ok to keep common |
| 12-3 | SPS release by DCI format 1_1 | 5-18 DL SPS | sps-ReleaseDCI-1-1-r16 | Phy-ParametersCommon | Ok to keep common |
| 12-3a | SPS release by DCI format 1_2 | 5-18 DL SPS and 11-1 | sps-ReleaseDCI-1-2-r16 | Phy-ParametersCommon | Ok to keep common |
| 12-5 | Configuration of aggregation factor per SPS configuration | 5-18 DL SPS | aggregationFactorSPS-DL-r16 | Phy-ParametersFRX-Diff | Ok to keep common |
| 12-6 | Support of SPS periodicity shorter than 10 ms | 5-18 DL SPS | extendedSPS-Periodicities-r16 | Phy-ParametersCommon | Seems technically possible, but the specification needs to be updated to support larger SCSs |

4.2.5 NR positioning

| Index | FG | Prerequisite feature groups | Field name | Parent IE | Assessment |
| --- | --- | --- | --- | --- | --- |
| 13-1a | Max number of positioning frequency layers UE supports across all positioning methods across all bands |  | maxSupportedFreqLayers-r16 | NR-DL-PRS-ProcessingCapability-r16 LPP | Ok to keep common |
| 13-2 | DL PRS Resources for DL AoD | 13-1 | 1 maxNrOfDL-PRS-ResourceSetPerTrpPerFrequencyLayer-r16 2 maxNrOfTRP-AcrossFreqs-r16 3 maxNrOfPosLayer-r16 | NR-DL-PRS-ResourcesCapability-r16 LPP | Ok to keep common |
| 13-3 | DL PRS Resources for DL-TDOA | 13-1 | 1 maxNrOfDL-PRS-ResourceSetPerTrpPerFrequencyLayer-r16 2 maxNrOfTRP-AcrossFreqs-r16 3 maxNrOfPosLayer-r16 | NR-DL-PRS-ResourcesCapability-r16 LPP | Ok to keep common |
| 13-4 | DL PRS Resources for Multi-RTT | 13-1 | 1 maxNrOfDL-PRS-ResourceSetPerTrpPerFrequencyLayer-r16 2 maxNrOfTRP-AcrossFreqs-r16 3 maxNrOfPosLayer-r16 | NR-DL-PRS-ResourcesCapability-r16 LPP | Ok to keep common |
| 13-5 | DL PRS Measurement Report for DL-AoD | 13-2, | maxDL-PRS-RSRP-MeasurementFR1-r16 maxDL-PRS-RSRP-MeasurementFR2-r16 | NR-DL-AoD-MeasurementCapability LPP |  |
| 13-6 | DL PRS Measurement Report for DL-TDOA | 13-3 | dl-RSTD-MeasurementPerPairOfTRP-FR1-r16 dl-RSTD-MeasurementPerPairOfTRP-FR2-r16 | NR-DL-TDOA-MeasurementCapability-r16 LPP |  |
| 13-9f | PathLoss estimate maintenance across all cells | One of {13-9, 13-9a, 13-9b, 13-9c} | LPP maxNumberSRS-PosPathLossEstimateAllServingCells-r16 RRC maxNumberSRS-PosPathLossEstimateAllServingCells-r16 | LPP NR-UL-SRS-Capability-r16 RRC Phy-ParametersCommon |  |
| 13-10f | Spatial relation maintenance | One of {13-10, 13-10a, 13-10b, 13-10d, 13-10e} | LPP maxNumberSRS-PosSpatialRelationsAllServingCells-r16 RRC maxNumberSRS-PosSpatialRelationsAllServingCells-r16 | LPP NR-UL-SRS-Capability-r16 RRC Phy-ParametersFR2 |  |
| 13-11a | Association between SRS for positioning and DL PRS for Multi-RTT | 13-4 and 13-8 | LPP srs-AssocPRS-MultiLayersFR1-r16 srs-AssocPRS-MultiLayersFR2-r16 | LPP NR-Multi-RTT-MeasurementCapability-r16 | Ok to keep common |
| 13-11 | UE Rx-Tx Measurement Report for Multi-RTT | 13-4 and 13-8 | LPP 1 maxNrOfRx-TX-MeasFR1-r16 maxNrOfRx-TX-MeasFR2-r16 2 supportOfRSRP-MeasFR1-r16 supportOfRSRP-MeasFR2-r16 | LPP NR-Multi-RTT-MeasurementCapability-r16 |  |
| 13-12 | SS-RSRP RRM measurements for NR E-CID Positioning | 1-1 | nr-ECID-MeasSupported-r16 BIT STRING { ssrsrpSup(0), ssrsrqSup(1), csirsrpSup(2), csirsrqSup(3)} (SIZE(1..8)) | NR-ECID-ProvideCapabilities-r16 LPP | Ok to keep common |
| 13-12a | SS-RSRQ RRM measurements for NR E-CID Positioning | 1-1 | nr-ECID-MeasSupported-r16 BIT STRING { ssrsrpSup(0), ssrsrqSup(1), csirsrpSup(2), csirsrqSup(3)} (SIZE(1..8)) | NR-ECID-ProvideCapabilities-r16 LPP | Ok to keep common |
| 13-12b | CSI-RSRP RRM measurements for NR E-CID Positioning | 1-4 | nr-ECID-MeasSupported-r16 BIT STRING { ssrsrpSup(0), ssrsrqSup(1), csirsrpSup(2), csirsrqSup(3)} (SIZE(1..8)) | NR-ECID-ProvideCapabilities-r16 LPP | Ok to keep common |
| 13-12c | CSI-RSRQ RRM measurements for NR E-CID Positioning | 1-4 | nr-ECID-MeasSupported-r16 BIT STRING { ssrsrpSup(0), ssrsrqSup(1), csirsrpSup(2), csirsrqSup(3)} (SIZE(1..8)) | NR-ECID-ProvideCapabilities-r16 LPP | Ok to keep common |
| 13-18 | Support of parallel processing of LTE PRS and NR PRS |  | simulLTE-NR-PRS-r16 | NR-DL-PRS-ProcessingCapability-r16 LPP | Ok to keep common |

4.2.6 NR_TEI

| Index | FG | Prerequisite feature groups | Field name | Parent IE | Assessment |
| --- | --- | --- | --- | --- | --- |
| 14-6 | New RACH configuration for FR1 TDD |  |  |  | Ok to keep mandatory without signaling |
| 14-8 | CSI trigger states containing non-active BWP |  | csi-TriggerStateNon-ActiveBWP-r16 | Phy-ParametersCommon | Ok to keep common |

4.2.7 NR_eMIMO

| Index | FG | Prerequisite feature groups | Field name | Parent IE | Assessment |
| --- | --- | --- | --- | --- | --- |
| 16-1b-1 | TCI state activation across multiple CCs | Component 1: 2-1, 2-4 | simultaneousTCI-ActMultipleCC-r16 | Phy-ParametersFRX-Diff |  |
| 16-1b-2 | Spatial relation update across multiple CCs | Component 1: 2-59, 2-60 | simultaneousSpatialRelationMultipleCC-r16, cli-RSSI-FDM-DL-r16, cli-SRS-RSRP-FDM-DL-r16 | Phy-ParametersFRX-Diff |  |
| 16-1c | Default spatial relation | 2-53, 2-59 | defaultSpatialRelationPathlossRS-r16 | Phy-ParametersFR2 | Ok to keep common in FR2 |
| 16-1d | MAC CE spatial relation update for AP-SRS | 2-53, 2-59 | spatialRelationUpdateAP-SRS-r16, maxNumberSRS-PosSpatialRelationsAllServingCells-r16 | Phy-ParametersFR2 | Ok to keep common in FR2 |
| 16-1e | Pathloss reference RS activation via MAC CE | 8-3 | maxNumberPathlossRS-Update-r16 | Phy-ParametersCommon | Ok to keep common |
| 16-1g | Resources for beam management, pathloss measurement, BFD, RLM and new beam identification | 2-24, 2-31 | maxTotalResourcesForOneFreqRange-r16 { maxNumberResWithinSlotAcrossCC-OneFR-r16, maxNumberResAcrossCC-OneFR-r16} | Phy-ParametersFRX-Diff |  |
| 16-1g-1 | Resources for beam management, pathloss measurement, BFD, RLM and new beam identification across frequency ranges | 2-24, 2-31, 16-1g | maxTotalResourcesForAcrossFreqRanges-r16 { maxNumberResWithinSlotAcrossCC-AcrossFR-r16, maxNumberResAcrossCC-AcrossFR-r16} | Phy-ParametersCommon | Ok to keep common |
| 16-1j-1 | 2 port CSI -RS for new beam identifications |  | newBeamIdentifications2PortCSI-RS-r16 | Phy-ParametersCommon | Ok to keep common |
| 16-1j-2 | 2 port CSI -RS for pathloss estimation |  | pathlossEstimation2PortCSI-RS-r16 | Phy-ParametersCommon | Ok to keep common |
| 16-2a-4 | HARQ-ACK for multi-DCI based multi-TRP - separate | 16-2a | harqACK-separateMultiDCI-MultiTRP-r16 { maxNumberLongPUCCHs-r16 } | Phy-ParametersCommon | Ok to keep common |
| 16-2a-4a | HARQ-ACK for multi-DCI based multi-TRP - joint | 16-2a | harqACK-jointMultiDCI-MultiTRP-r16 | Phy-ParametersCommon | Ok to keep common |
| 16-2a-8 | Indicates that retransmission scheduled by a different CORESETPoolIndex for multi-DCI multi-TRP is not supported. | 16-2a | supportRetx-Diff-CoresetPool-Multi-DCI-TRP-r16 | Phy-ParametersCommon | Ok to keep common |
| 16-7 | Extension of the maximum number of configured aperiodic CSI report settings | 2-32 | csi-ReportFrameworkExt-r16 | Phy-ParametersFRX-Diff AND MIMO-ParametersPerBand |  |
| 16-x RAN2 | Mulit-CC simultaneous TCI activation with multi-TRP | If the UE indicates support of 16-1b-1 for a FR and support of at least one of 16-2b-1, 16-2b-2, 16-2b-3, 16-2b-4 or 16-2b-5 for at least one band or component carrier of this FR, the UE shall indicate support of 16-x for this FR | twoTCI-Act-servingCellInCC-List-r16 | Phy-ParametersFRX-Diff |  |
| 16-y RAN2 | Slot based repetition | 16-2b-5 and maxNumberTCI-states-r16 is set to 2 for at least one band | supportRepetitionZeroOffsetRV-r16 | Phy-ParametersCommon | Ok to keep common |
| 16-z RAN2 | spCell-BFR-CBRA-r16 |  | spCell-BFR-CBRA-r16 | BeamFailureRecoveryConfig | Ok to keep common |

4.2.8 NR_CLI_RIM

| Index | FG | Prerequisite feature groups | Field name | Parent IE | Assessment |
| --- | --- | --- | --- | --- | --- |
| 17-1 | CLI-RSSI measurement |  | cli-RSSI-Meas-r16 maxNumberCLI-RSSI-r16 | MeasAndMobParametersFRX-Diff |  |
| 17-2 | SRS-RSRP measurement |  | cli-SRS-RSRP-Meas-r16 maxNumberCLI-SRS-RSRP-r16 maxNumberPerSlotCLI-SRS-RSRP-r16 | MeasAndMobParametersFRX-Diff |  |
| 17-3 | Simultaneous reception of DL signals/channels and CLI-RSSI measurement resource | 17-1 | cli-RSSI-FDM-DL-r16 | Phy-ParametersFRX-Diff |  |
| 17-4 | Simultaneous reception of DL signals/channels and SRS-RSRP measurement resource | 17-2 | cli-SRS-RSRP-FDM-DL-r16 | Phy-ParametersFRX-Diff |  |

4.2.9 MR-DC/CA enhancement

| Index | FG | Prerequisite feature groups | Field name | Parent IE | Assessment |
| --- | --- | --- | --- | --- | --- |
| 18-8 | HARQ-ACK codebook type and HARQ-ACK spatial bundling configuration per PUCCH group | 6-7 | harqACK-CB-SpatialBundlingPUCCH-Group-r16 | Phy-ParametersCommon | Ok to keep common |
| 18-9 | Type2 HARQ-ACK codebook for >1 unicast DL DCIs in same Monitoring Occasion | 3-1 | type2-HARQ-ACK-Codebook-r16 | Phy-ParametersCommon | Ok to keep common |
| 18-3a | Semi-statically configured LTE UL transmissions in all UL subframes not limited to tdm-pattern in case of FDD PCell | One of {18-2a, 18-3} | fdd-PCellUL-TX-AllUL-Subframe-r16 | Phy-ParametersMRDC | FR1 only |
| 18-3b | Semi-statically configured LTE UL transmissions in all UL subframes not limited to tdm-pattern in case of TDD PCell | 18-2 | tdd-PCellUL-TX-AllUL-Subframe-r16 | Phy-ParametersMRDC | FR1 only |

4.2.10. UE power saving

| Index | FG | Prerequisite feature groups | Field name | Parent IE | Assessment |
| --- | --- | --- | --- | --- | --- |
| 19-1 | DRX Adaptation | N/A | drx-Adaptation-r16 { non-SharedSpectrumChAccess-r16 MinTimeGap-r16, sharedSpectrumChAccess-r16 MinTimeGap-r16 } | MAC-ParametersFRX-Diff-r16 |  |
| 19-2 | Cross Slot Scheduling |  | crossSlotScheduling-r16 { non-SharedSpectrumChAccess-r16, sharedSpectrumChAccess-r16 } | Phy-ParametersCommon | ok |
| 19-3 | Maximum MIMO Layer Adaptation | See Note | maxLayersMIMO-Adaptation-r16 | Phy-ParametersFRX-Diff |  |
| 19-4a | UE assistance information | 19-2 | minSchedulingOffsetPreference-r16 | PowSav-ParametersCommon-r16 | Seems technically possible, but the specification needs to be updated to support larger SCSs |

4.2.11. NR_IAB

| Index | FG | Prerequisite feature groups | Field name | Parent IE | Assessment |
| --- | --- | --- | --- | --- | --- |
| 20-2 | Inter-IAB-node discovery and measurements: SSB reception configuration |  | seperateSMTC-InterIAB-Support-r16 | Phy-ParametersCommon | Ok to keep common |
| 20-3 | Extension of RACH occasions and periodicities for backhaul RACH resources |  | seperateRACH-IAB-Support-r16 | Phy-ParametersCommon | Ok to keep common |
| 20-5a | UL-Flexible-DL slot formats | 5-1a | ul-flexibleDL-SlotFormatSemiStatic-IAB-r16 | Phy-ParametersCommon | Ok to keep common |
| 20-5b | UL-Flexible-DL slot formats | 3-6 | ul-flexibleDL-SlotFormatDynamics-IAB-r16 | Phy-ParametersCommon | Ok to keep common |
| 20-6 | Dynamic indication of soft resource availability |  | dci-25-AI-RNTI-Support-IAB-r16 | Phy-ParametersCommon | Ok to keep common |
| 20-7 | Case 1 OTA timing alignment |  | t-DeltaReceptionSupport-IAB-r16 | t-DeltaReceptionSupport-IAB-r16 | Ok to keep common |
| 20-8 | Guard symbols |  | guardSymbolReportReception-IAB-r16 | t-DeltaReceptionSupport-IAB-r16 | Ok to keep common |

4.2.12. New FGs that are not dedicated to a specific Rel-16 work item/TEI

| Index | FG | Prerequisite feature groups | Field name | Parent IE | Assessment |
| --- | --- | --- | --- | --- | --- |
| 22-10 | Support of pdcch-MonitoringAnyOccasionsWithSpanGap in case of cross-carrier scheduling with different SCSs in the scheduling cell and the scheduled cell | 3-5b, 18-5 | pdcch-MonitoringAnyOccasionsWithSpanGapCrossCarrierSch-r16 | Phy-ParametersCommon | Depending on 8.2.2 discussion |
| 22-11 | Support of 'cri-RI-CQI' report without non-PMI-PortIndication | 2-35 | cri-RI-CQI-WithoutNon-PMI-PortInd-r16 | Phy-Parameters |  |

- References

- R1-2109913, “Summary of UE features for supporting NR from 52.6 GHz to 71 GHz”, Moderator (AT&T), 3GPP RAN1#106bis-e, October 2021

- TR 38.822 V16.1.0 (2021-09), “User Equipment (UE) feature list”, 3GPP