3GPP TSG RAN WG1 #107-e	                	                                  R1-2112138

e-Meeting, November 11th – 19th, 2021

Source:	NTT DOCOMO, INC.

Title:	Discussion on Rel.17 UE features for UE power saving enhancements

Agenda Item:	8.16.7

Document for: 	Discussion and Decision

Introduction

Updated Rel.17 UE feature list after RAN1#106bis-e including UE power saving enhancements has been prepared [1]. In this contribution, we present our views and updates regarding the UE features for UE power saving enhancements based on the list.

Rel-17 UE features for UE power saving

We itemized our suggested corrections to the existing UE features as follows.

- FG 29-1:

  - Regarding whether to support FG 29-1 as optional with capability signaling or optional without capability signaling, we think it can be left to RAN2 discussion.

  - Type should be per UE

- FG 29-2:

  - For the Components: Regarding the FFS whether to separate the capability for receiving L1 indication for TRS availability, we think no need to separate the capability since RAN1 made the conclusion in the last RAN1 meeting that there is no consensus to support SIB based signaling for availability information of TRS/CSI-RS occasions for idle/inactive UEs

  - For the Components: Regarding whether to separate the capability for receiving PEI based indication for TRS availability as another FG, If PEI based availability indication is confirmed, we think need to separate the capability.

  - Regarding whether to support FG 29-2 as optional with capability signaling or optional without capability signaling, we think it can be left to RAN2 discussion.

  - Type should be per UE

  - The sentence in “Consequence if the feature is not supported by the UE” should be revised to “UE does not support TRS occasions for idle/inactive UEs”.

| Features | Index | Feature group | Components |
| --- | --- | --- | --- |
| 29. NR_UE_pow_sav_enh | 29-2 | TRS resources for idle/inactive Ues | TRS receiving for idle/inactive Ues 1. Support reading TRS configuration from SIB 2. Support receiving L1 indication for TRS availability FFS whether to separate the capability for receiving L1 indication for TRS availability FFS whether to separate the capability for receiving PEI/Paging DCI for TRS availability |

- FG 29-3:

  - The column for “Mandatory/Optional” in FG 29-3 can be updated to “Optional with capability signaling”.

  - Type should be per UE

  - The sentence in “Consequence if the feature is not supported by the UE” should be revised to “UE monitors all configured search space sets”.

  - Regarding whether/how to separate the capabilities of FG 29-3

    - we think FG 29-3 should be split into four (or five) FGs as follows:

    - FG 29-3a: Support of PDCCH skipping

    - FG 29-3b: Support of  2 SSSG switching

    - FG 29-3c: Support of  3 SSSG switching

    - FG 29-3d: Support of  2 SSSG switching with PDCCH skipping

    - [FG 29-3e: Support of  3 SSSG switching with PDCCH skipping]

| Features | Index | Feature group | Components |
| --- | --- | --- | --- |
| 29. NR_UE_pow_sav_enh | FG 29-3a | Support of PDCCH monitoring adaptation behaviour 1/1A Support of PDCCH skipping (Case1) | Support of PDCCH monitoring adaptation behaviour 1/1A |
| 29. NR_UE_pow_sav_enh | 29-3b | Support of PDCCH monitoring adaptation behaviour 2/2A Support of 2 SSSG switching (Case2) | Support of PDCCH monitoring adaptation behaviour 2/2A |
| 29. NR_UE_pow_sav_enh | 29-3c | [Support of PDCCH monitoring adaptation behaviour 1/1A] Support of 2 SSSG switching (Case3) | Support of PDCCH monitoring adaptation behaviour 2/2A/2B |
| 29. NR_UE_pow_sav_enh | 29-3d | [Support of PDCCH monitoring adaptation behaviour 1/1A] Support of 2 SSSG switching with PDCCH skipping (Case4) | Support of PDCCH monitoring adaptation behaviour 1/1A/2/2A |
| 29. NR_UE_pow_sav_enh | [29-3e] | [Support of 3 SSSG switching with PDCCH skipping (Case5)] | [Support of PDCCH monitoring adaptation behaviour 1/1A/2/2A] |

According to the above comments, the following revision is proposed for UE power saving.

Proposal 1: Adopt the following changes on UE power saving feature groups:

- FG 29-2:

| Features | Index | Feature group | Components |
| --- | --- | --- | --- |
| 29. NR_UE_pow_sav_enh | 29-2 | TRS resources for idle/inactive Ues | TRS receiving for idle/inactive Ues 1. Support reading TRS configuration from SIB 2. Support receiving L1 indication for TRS availability FFS whether to separate the capability for receiving L1 indication for TRS availability FFS whether to separate the capability for receiving PEI/Paging DCI for TRS availability |

- FG 29-3:

| Features | Index | Feature group | Components |
| --- | --- | --- | --- |
| 29. NR_UE_pow_sav_enh | FG 29-3a | Support of PDCCH monitoring adaptation behaviour 1/1A Support of PDCCH skipping (Case1) | Support of PDCCH monitoring adaptation behaviour 1/1A |
| 29. NR_UE_pow_sav_enh | 29-3b | Support of PDCCH monitoring adaptation behaviour 2/2A Support of 2 SSSG switching (Case2) | Support of PDCCH monitoring adaptation behaviour 2/2A |
| 29. NR_UE_pow_sav_enh | 29-3c | [Support of PDCCH monitoring adaptation behaviour 1/1A] Support of 2 SSSG switching (Case3) | Support of PDCCH monitoring adaptation behaviour 2/2A/2B |
| 29. NR_UE_pow_sav_enh | 29-3d | [Support of PDCCH monitoring adaptation behaviour 1/1A] Support of 2 SSSG switching with PDCCH skipping (Case4) | Support of PDCCH monitoring adaptation behaviour 1/1A/2/2A |
| 29. NR_UE_pow_sav_enh | [29-3e] | [Support of 3 SSSG switching with PDCCH skipping (Case5)] | [Support of PDCCH monitoring adaptation behaviour 1/1A/2/2A] |

Summary of proposals

The proposals in this paper are summarized as follows.

Proposal 1: Adopt the following changes on UE power saving feature groups:

- FG 29-1:

| Features | Index | Feature group | Components |
| --- | --- | --- | --- |
| 29. NR_UE_pow_sav_enh | 29-2 | TRS resources for idle/inactive Ues | TRS receiving for idle/inactive Ues 1. Support reading TRS configuration from SIB 2. Support receiving L1 indication for TRS availability FFS whether to separate the capability for receiving L1 indication for TRS availability FFS whether to separate the capability for receiving PEI/Paging DCI for TRS availability |

- FG 29-3:

| Features | Index | Feature group | Components |
| --- | --- | --- | --- |
| 29. NR_UE_pow_sav_enh | FG 29-3a | Support of PDCCH monitoring adaptation behaviour 1/1A Support of PDCCH skipping (Case1) | Support of PDCCH monitoring adaptation behaviour 1/1A |
| 29. NR_UE_pow_sav_enh | 29-3b | Support of PDCCH monitoring adaptation behaviour 2/2A Support of 2 SSSG switching (Case2) | Support of PDCCH monitoring adaptation behaviour 2/2A |
| 29. NR_UE_pow_sav_enh | 29-3c | [Support of PDCCH monitoring adaptation behaviour 1/1A] Support of 2 SSSG switching (Case3) | Support of PDCCH monitoring adaptation behaviour 2/2A/2B |
| 29. NR_UE_pow_sav_enh | 29-3d | [Support of PDCCH monitoring adaptation behaviour 1/1A] Support of 2 SSSG switching with PDCCH skipping (Case4) | Support of PDCCH monitoring adaptation behaviour 1/1A/2/2A |
| 29. NR_UE_pow_sav_enh | [29-3e] | [Support of 3 SSSG switching with PDCCH skipping (Case5)] | [Support of PDCCH monitoring adaptation behaviour 1/1A/2/2A] |

Reference

[1]	Moderators (AT&T, NTT DOCOMO, INC.), R1-2110587, Updated RAN1 UE features list for Rel-17 NR after RAN1 #106bis-e, Oct. 2021.