3GPP TSG RAN WG1 Meeting #107-e		R1-2112182

e-Meeting, Nov 11th – 19th, 2021

Agenda item:       8.1.4

Source: 	Nokia, Nokia Shanghai Bell

Title: 	Enhancement on CSI measurement and reporting

Document for:     Discussion and Decision

# 1	Introduction

In this contribution we discuss the remaining aspects of the design for reciprocity-aided FDD CSI enhancement, and Multi-TRP CSI enhancement identified by the agreements in RAN1#106bis-e [2].

# 2	FDD CSI reporting with partial reciprocity

## 2.1	Wf reporting for N3=3

In RAN1#106bis-e, it was agreed to support the additional value of N=4 for the window size when M=2. One remaining detail is how to handle the case in which the parameter N3, which corresponds to the number of PMI subbands configured in the reporting band, is smaller than N, in which case the codebook indices available for the selection of Wf are fewer than the window size.

Agreement

In addition to N=2, N=4 is supported when M=2 for rank 1/2

- For rank 3/4, when M=2, N = 2 or 4 is supported and same with the value of N configured for rank 1/2

  - FFS how to handle N3=3 case

The minimum number of CQI subbands is 3, for a BWP of 24 PRBs and subband size 8 PRBs. Besides, in general, csi-ReportingBand can be configured with a small number of subbands. Therefore, for candidate values N=4,5, the number of PMI subbands, N3 may be smaller than the window size. Although this case is unlikely to occur in practice, a simple solution is to add the condition n3(f)<N3 such that, in case N3=3, the nonzero offset between n3(0) and n3(1)  can only be 1 or 2. Changing the bitwidth of the index i1,6 to log2(minN,N3-1) to save 1 bit in the special case when N3=3, N=4, M=2, seems unnecessary.

- Regarding the case N3=3, the general condition n3(f)<N3 can be added in the definition of the FD basis vector n3, which is applicable for any value of N3, M and N. There is no need to reduce the bitwidth of the index i1,6 by one bit for the special case of N3=3,  when M=2, and N=4, as this small number of subbands is a very infrequent case.

## 2.2	UCI mapping and CSI omission

After RAN1#106bis-e, a couple of details regarding the UCI mapping and omission rules are left to be decided. The first one is related to the mapping of CSI fields in the three reporting priority levels or Groups. The following options were identified, which are not mutually exclusive.

Agreement

For UCI part II of Rel-17 PS codebook, study the following alternatives and down-select one or more alternatives in RAN1 107

- Alt 1: Report Port indicator, SCI, and FD indicator in Group 0

- Alt 2: Report bitmap in Group 0 or Group 1 without bitmap partition

- Alt 3: Three groups of UCI Part 2 for Rel-16 PS codebook is reused for Rel-17 PS codebook enhancement except that the starting position of the FD basis window is not needed

Note that other solutions of UCI part II design are not excluded.

Alt 1 has the advantage of making the FD indicator available, if reported, also when Group 1 and 2 are omitted. In Rel-17 this indicator is only reported when N=4 and M=2 and is 2-bit long, hence including it in Group 0 does not increase its size significantly. Besides, unlike in Rel-16, where the FD bases could not be used for precoder reconstruction without the bitmap and nonzero coefficients, and the strongest coefficient was always located in FD component 0, in Rel-17 the FD bases indication, when reported, is needed to determine the FD component of the strongest coefficient.

- In Rel-17 the FD basis indicator is needed, when reported, to determine the precoder associated with the strongest coefficient because this can be found in either of the two FD components. Conversely, in Rel-16, the strongest coefficient is always found in FD component 0. Besides the FD component indicator, when reported, is only 2 bits.

- Regarding the mapping of UCI Part 1, support Alt 1 as the FD basis indicator is needed, when reported, to determine the precoder associated with the strongest coefficient.

The second issue is related to the priority function used to map coefficients and bitmap bits in their corresponding UCI field. The following alternatives were identified in RAN1#106bis-e.

Agreement

For the priority of mapping coefficients for Rel17 PS codebook, study the following alternatives and down-select one or more alternatives in RAN1#107-e:

- Alt 1: Support mapping coefficients firstly across port indices, secondly across FD basis indices, and thirdly across layers, i.e. priority value is given by the priority value Pril,i,f=K1⋅M⋅l+K1⋅f+i

- Alt 2: Support mapping coefficients firstly across layers, secondly across port indices, and thirdly across FD basis indices, i.e., the priority value is given by Pril,i,f=v⋅K1⋅f+v⋅i+l

- Alt 3: Support mapping coefficients firstly across layers, secondly across port indices, and thirdly across FD basis indices, i.e., the priority value is given by Pril,i,f=v⋅K1⋅f+v⋅ψ(i)+l

  - FFS port permutation function ψ(i)

Note that other solutions are not excluded.

Alt 1 corresponds to a simple mapping of one layer at a time by reading coefficients firstly along ports and then along the FD components. One advantage is simplicity and absence of any interleaving of coefficients across layers, which allows to report at least one layer accurately with high probability, in the presence of omission.

Alt 2 and 3 reuse the same sequence as in Rel-16 with interleaving across layers. Note that in all alternatives the permutation function π(f) across FD components introduced in Rel-16 boils down to the identity because there are, at most, M=2 FD components to map in Rel-17.

Alt 3 proposed to apply a new permutation across ports to avoid omitting coefficients mostly from the second polarisation. This problem occurs because the number of selected ports K1 is much larger than M, hence most coefficients from the second polarisation are likely to be assigned to Group 2. The permutation of the port index i=0,…,K1-1, with L=K1/2, can be such that, for each layer, the ports are reordered starting from the index of the strongest coefficient, il*, and alternating between the two polarisations as follows:

ψi for layer l=1,…,ν: 0,1,2,…, 2L-1→il*,il*+L,il*+1,il*+L+1,il*+2,… mod K1.

The permutation function can be written as follows, where iL,∈{0,1} indicates the polarisation port index i.

ψli=2(i-il*-L)mod K1+iL-2iL-1il*L

- Regarding the mapping of coefficients in the UCI field, support Alt 3 by reordering the port index for each layer, starting from the index of the strongest coefficient, il*, and alternating between the two polarisations: 
ψli=2(i-il*-L)mod K1+iL-2iL-1il*L

## 2.3	Parameter combinations

In RAN1#106bis-e, 8 parameter combinations were agreed, as described in the following agreement.

Agreement

With regarding to parameter combinations, following 8 parameter combinations are supported in Rel-17 PS codebook:

| M | Alpha | Beta |
| --- | --- | --- |
| 1 | 1 | 1 |
| 1 | 1 | 3/4 |
| 1 | 1 | 1/2 |
| 1 | 3/4 | 1/2 |
| 2 | 1 | 3/4 |
| 2 | 1 | 1/2 |
| 2 | 3/4 | 1/2 |
| 2 | 1/2 | 1/2 |

FFS: whether further restrictions/dependences for given parameter combination(s) are needed

One remaining issue is whether combinations the combination(s) with the largest overhead for M=2 should be restricted for large number of ports.

In a typical 20MHz BWP with 13 subbands, the maximum value of K0 in Rel-16 CBs is 36 for parameter combination 8, whereas the maximum value of K0 for any BWP size is 45 for parameter combination 8 and BWP with 19 subbands. However, for Rel-17 CB, the maximum value of K0 is 48 for parameter combination M,α,β=(2,1,3/4) and PCSI-RS=32. Hence, from the point of view of limiting the maximum overhead of reporting the nonzero coefficients, it is reasonable to restrict the combination M,α,β=(2,1,3/4) to PCSI-RS<32, such that the maximum value of K0 is limited to 36, in line with the maximum value of K0 in Rel-16 CBs for a typical 13-subband BWP.

From performance perspective, we simulated all parameter combinations for PCSI-RS=8,16,32 and verified that the restriction on parameter combination M,α,β=(2,1,3/4) for PCSI-RS<32 does not impact performance and allows to keep the maximum overhead within the same range as for Rel-16 eType II regular codebook.

- Support a restriction of parameter combination M,α,β=(2,1,3/4) to PCSI-RS<32.

Figure 1 to Figure 4 show performance results for the 4 parameter combinations with M=1 for rank 1 (MR1) and max rank 4 (MR4) and for both DFT-based and eigenvector-based precoding of CSI-RS ports.

Figure 5 shows simulation results for the 4 parameter combinations with M=2 for max rank 4 and DFT-based precoding of CSI-RS ports. The window size is set to N=4 and the two values of R=1,2 are simulated.

| Figure 1. Performance/overhead trade-off for the 4 parameter combinations with M=1 and rank 1. CSI-RS precoding is DFT-based. | Figure 2. Performance/overhead trade-off for the 4 parameter combinations with M=1 and rank 1. CSI-RS precoding is eigenvector-based. |
| --- | --- |

| Figure 3. Performance/overhead trade-off for the 4 parameter combinations with M=1 and max rank 4. CSI-RS precoding is DFT-based. | Figure 4. Performance/overhead trade-off for the 4 parameter combinations with M=1 and rank 4. CSI-RS precoding is eigenvector-based. |
| --- | --- |

Figure 5. Performance/overhead trade-off for the 4 parameter combinations with M=2 and max rank 4. Window size is N=4 and the two values of R=1,2 are tested. CSI-RS precoding is DFT-based.

# 3	M-TRP CSI reporting enhancement

## 3.1	UCI mapping and CSI priority/omission/CPU occupation

In RAN1#106-e three issues were identified related to UCI mapping and CSI priority/omission/CPU occupation rules. It was also agreed to prioritise the following issue 1 and, after that, if needed to address issue 2 and 3:

- Issue 1: to confirm the order of UCI payload construction for reported CSIs

- Issue 2: to enhance CSI part 2 omission rules for prioritized CSI measurement hypotheses and associated CSIs

- Issue 3: to enhance CPU occupancy rules for prioritized CSI measurement hypotheses and associated CSIs

Regarding Issue 1, the following alternatives were agreed for down-selection in RAN1#106-e:

Agreement

To confirm the order of UCI payload construction for reported CSIs, study following Alternatives and down-select one or more Alternative(s) for required specification changes in RAN1 106bis:

- Alt 1: modify priority equation, i.e., Section 5.2.5 in 38.214.

- Alt 2: modify the table of priority reporting levels for Part 2 CSI, i.e., Table 5.2.3-1 in 38.214.

- Alt 4: modify mapping order of CSI fields of one CSI report, i.e., Table 6.3.2.1.2-3/4/5 in 38.212

In RAN1#106bis-e, the following proposals were discussed at length

Proposal 24

To confirm the order of UCI payload construction for reported CSIs,

- modify mapping order of CSI fields of one CSI report, i.e., Table 6.3.1.1.2-[7]/9/10/11 for PUCCH and Table 6.3.2.1.2-3/4/5 for PUSCH in 38.212

  - Introduce mapping order of CSI fields in the order of NCJT CSI, the first TRP CSI, and the second TRP CSI. It also implies that one CSI reporting setting for NCJT measurement reporting contains single CSI report which may corresponds multiple single-TRP and/or NCJT measurement hypotheses

New Proposal 24-1

Support CSI part2 omission priority level to prioritize single-TRP or NCJT measurement hypothesis for Option 1 with X=1 or X=2

FFS detailed specification impact

Regarding UCI mapping of the CSIs, for a UE configured with codebook-based MTRP CSI reporting, Mode 1 and X=1,2, multiple CSIs are reported with the same reportConfigID. Hence, it is not clear what order should be used to map the CSI fields of different CSIs in a single report. Proposal 24 above addresses this issue by introducing a mapping order, which is applicable to both Part 1 and Part 2 for subband reporting on PUCCH (‘periodic’ or ‘semi persistent’) and to both Part 1 and Part 2 of wideband or subband reporting on PUSCH (‘aperiodic’).

- Confirm the order of payload construction for reported CSIs by supporting Proposal 24 from RAN1#106bis-e:

- modify mapping order of CSI fields of one CSI report, i.e., Table 6.3.1.1.2-[7]/9/10/11 for PUCCH and Table 6.3.2.1.2-3/4/5 for PUSCH in 38.212

  - Introduce mapping order of CSI fields in the order of NCJT CSI, the first TRP CSI, and the second TRP CSI. It also implies that one CSI reporting setting for NCJT measurement reporting contains single CSI report which may corresponds multiple single-TRP and/or NCJT measurement hypotheses

Regarding wideband reporting of MTRP CSI on PUCCH, we think it should be supported but only with reporting Mode 1 and X=0 or reporting Mode 2, such that the report consists of a single CSI with either one or two WB PMIs. Because wideband reporting on PUCCH consists of Part 1 only, which is of fixed size, padding may be added if needed.

- Support wideband reporting of MTRP CSI on PUCCH, but only for reporting Mode 1, with X=0 or Mode 2, by modifying the mapping order of CSI fields of Table 6.3.1.1.2-7 of 38.212.

Regarding Issue 2, i.e., whether a prioritisation between the CSIs in an MTRP CSI report is needed to enhance omission rules, assigning the reporting priority levels in Part 2 as shown in Figure 6 does not require prioritising between CSIs because, if a priority level is omitted, all the information at that priority level is omitted for all the PMIs in the report.

When omitting Part 2 CSI information for a particular priority level assigned to a MTRP CSI report, the UE will omit all the information at that priority level for all the PMIs in the report. Figure 6 illustrates an example of UCI mapping of Part 2 for an MTRP CSI report with Mode 1 and X=2. In Part 2, for MTRP CSI report number n, reporting priority 0 is given to the WB components of all the PMIs conveyed in the report; priority 2n-1 is given to the even SB components of all PMIs in the report; priority 2n is assigned to the odd SB components of all PMIs in the report. The mapping order of the PMIs in Part 2 priority 0, 2n-1 and 2n is the same as introduced in 38.212. The specific order does not impact the omission rules because all the CSIs are omitted if a priority level is omitted.

Figure 6. Example of UCI mapping for Part 2 including an MTRP CSI report with Mode 1 and X=2. CSI report n contains 3 CSIs: one NCJT CSI and 2 single-TRP CSIs

- Regarding Issue 2, i.e., whether a prioritisation between the CSIs is needed to enhance omission rules, no prioritisation between CSIs is needed because the 3 reporting priority levels for WB CSI, even SB and odd SB CSI can be assigned to each CSI in the report.

Regarding Issue 3, i.e., whether any enhancement to the CPU occupation rules is needed, it is possible to improve the handling of CPU calculations for MTRP CSI reports with multiple CSIs without impacting the legacy CSI reports. One solution consists in introducing X+1 separate CPU requirements for each of the X+1 CSIs in the report alongside the legacy total CPU requirement for the CSI report. These separate CPU requirements can be used in a special case, when the first CSI report exceeding the CPU count, i.e., CSI report n=M, is an MTRP CSI report configured with Mode 1 and X=1 or 2.

The following rule can be appended to first paragraph of Sec 5.2.1.6 in 38.214. If CSI report n=M is an MTRP CSI report configured with Mode 1 and X=1 or 2, where each CSI, x=0,…,X corresponds to OCPU(M,x), and NCPU-L-n=0M-1OCPUn CPUs are unoccupied, the UE is expected to update the first Y CSIs and is not required to update the last X+1-Y CSIs, according to their UCI mapping order, where 0≤Y≤X+1 is the largest value such that x=0Y-1OCPU(M,x)≤NCPU-L-n=0M-1OCPUn holds.

- Regarding Issue 3, i.e., whether any enhancement to the CPU occupation rules is needed, support improved handling of CPU calculations for Mode 1 and X=1,2, without impacting the rules for legacy CSI reports. This can be done by

- introducing X+1 separate CPU occupations for an MTRP CSI report n configured with Mode 1 and X=1 or 2: OCPU(n,x), with x=0,…,X, alongside the legacy OCPUn such that OCPU(n,0)=2N,  OCPU(n,1)=M, for X=1 and OCPU(n,0)=2N,  OCPU(n,1)=M1, OCPU(n,2)=M2, for X=2, and OCPUn=2N+M=x=0XOCPU(n,x);

- adding a dedicated “soft” formula in Sec. 5.2.1.6 of 38.214 for the case when the first CSI report exceeding the CPU count is an MTRP CSI report configured with Mode 1 and X=1 or 2.

- Text proposal for the additional “soft” formula for CSI updates at the end of the first paragraph of Sec 5.2.1.6 in 38.214:

If CSI report M' is the first CSI report exceeding the CPU count and it is configured with Mode 1 and X=1 or 2, where each CSI occupies OCPU(M',x) CPUs, for x=0,…,X, and NCPU-L-n=0M'-1OCPUn CPUs are unoccupied, the UE is expected to update the first Y CSIs and is not required to update the last X+1-Y CSIs, where 0≤Y≤X+1 is the largest value such that x=0Y-1OCPU(M',x)≤NCPU-L-n=0M'-1OCPUn holds.

## 3.2	RI restriction and CBSR

In RAN1#106bis-e, it was agreed to support two RI restrictions, one for single-TRP and one for NCJT measurement hypotheses:

Agreement

For a CSI report associated with a Multi-TRP/panel NCJT measurement hypothesis configured by single CSI reporting setting

- Alt 4: Two RI restrictions can be configured per CodebookConfig, whereas one RI restriction is applied to all Single-TRP measurement hypotheses, and another one is applied to all NCJT measurement hypotheses.

  - If rank restriction of (X, Y) is configured, reported rank is X for all single-TRP measurement hypotheses and reported rank (1 out of 4 possible rank combinations) is Y for all NCJT measurement hypotheses.

    - FFS: Whether there can be multiple candidate values of X and Y

In Rel-15/16 both RI and CBSR restrictions are included in CodebookConfig and CodebookConfig-r16, respectively, of CSI-ReportConfig. Therefore, for single-TRP CSI reporting, these restrictions pertain to the codebook configuration and because a CSI-ReportConfig can include only a single codebook configuration, the same restrictions apply to all measurement hypotheses.

The first parameter can follow legacy design and be defined as in Clause 5.2.2.2.1 of 38.214.

The second parameter may also follow legacy design and form a four-bit sequence r3,r2,r1,r0, where r0 is the LSB and r3 is the MSB. When ri=0, i∈{0,1,2,3}, the two PMIs and RIs are not allowed to correspond to any precoder associated to the (i+1)-th rank combination, in the order: {1,1}, {1,2}, {2,1}, {2,2}, where the first rank combination is {1,1}.

- Regarding the RI restriction parameters, follow legacy design for Type I reporting and allow multiple candidate values. For the NCJT RI restriction parameter use a four-bit sequence as per legacy design.

Regarding support for CBSR, two alternatives were identified as follows.

Agreement

For a CSI report associated with a Multi-TRP/panel NCJT measurement hypothesis configured by single CSI reporting setting, down-select one alternative from the following in RAN1 107:

- Alt 1: One CBSR can be configured per CodebookConfig, whereas CBSR is applied to all CMRs regardless measurement hypotheses or CMR groups.

- Alt 2: Two CBSRs can be configured per CodebookConfig, whereas one CBSR is applied to one CMR group in a CMR resource set respectively, i.e. per TRP.

Alt 2 is slightly preferred for the extra flexibility it provides.

- Regarding CBSR, support Alt 2, i.e., two CBSRs, one for each CMR group.

## 3.3	RRC parameters and UE features

The consolidated RRC parameter list after RAN1#106bis-e is found in [11], and copied below for the CSI-mTRP part.

It appears the parameter sharedCMR is missing in the new version

- Reintroduce the parameter sharedCMR in the RRC parameter list, as defined in R1-2108676, because it seems left out from R1-2110573.

Regarding the RI restriction, we suggest clarifying to RAN2 that there are two different parameters, one for single-TRP, which may reuse the old parameter name typeISinglePanel-ri‑Restriction, and amend the description which may suggest that there is a single parameter jointly encoding the two restrictions

- Clarify that there are two RI restriction parameters, one for single-TRP, which can reuse the Rel-15/16 name typeISinglePanel-ri‑Restriction and the new one for NCJT. Change the description for the new parameter to: “RI restriction applicable to all NCJT measurement hypotheses (up to 4 rank combinations)”.

We also propose to define the new NCJT RI restriction parameter as a bitmap of size 4 to be able to describe the parameter in 38.214. Similarly, for legacy Type I RI restriction, 38.331 actually refers to the description provided in 38.214.

- For the new NCJT RI restriction parameter, add in the description or in value range that it is a bit sequence of length 4, so the relative description can be added to 38.214 as for the legacy Type I RI restriction.

# 4	Conclusion

Hereafter is a summary of observations and proposals for FDD CSI enhancement

Observation 1	In Rel-17 the FD basis indicator is needed, when reported, to determine the precoder associated with the strongest coefficient because this can be found in either of the two FD components. Conversely, in Rel-16, the strongest coefficient is always found in FD component 0. Besides the FD component indicator, when reported, is only 2 bits.

Proposal 1	Regarding the case N3=3, the general condition n3(f)<N3 can be added in the definition of the FD basis vector n3, which is applicable for any value of N3, M and N. There is no need to reduce the bitwidth of the index i1,6 by one bit for the special case of N3=3,  when M=2, and N=4, as this small number of subbands is a very infrequent case.

Proposal 2	Regarding the mapping of UCI Part 1, support Alt 1 as the FD basis indicator is needed, when reported, to determine the precoder associated with the strongest coefficient.

Proposal 3	Regarding the mapping of coefficients in the UCI field, support Alt 3 by reordering the port index for each layer, starting from the index of the strongest coefficient, il*, and alternating between the two polarisations: 
ψli=2(i-il*-L)mod K1+iL-2iL-1il*L

Proposal 4	Support a restriction of parameter combination M,α,β=(2,1,3/4) to PCSI-RS<32.

Hereafter is a summary of observations and proposals for M-TRP CSI enhancement.

Proposal 5	Confirm the order of payload construction for reported CSIs by supporting Proposal 24 from RAN1#106bis-e:

- modify mapping order of CSI fields of one CSI report, i.e., Table 6.3.1.1.2-[7]/9/10/11 for PUCCH and Table 6.3.2.1.2-3/4/5 for PUSCH in 38.212

  - Introduce mapping order of CSI fields in the order of NCJT CSI, the first TRP CSI, and the second TRP CSI. It also implies that one CSI reporting setting for NCJT measurement reporting contains single CSI report which may corresponds multiple single-TRP and/or NCJT measurement hypotheses

Proposal 6	Support wideband reporting of MTRP CSI on PUCCH, but only for reporting Mode 1, with X=0 or Mode 2, by modifying the mapping order of CSI fields of Table 6.3.1.1.2-7 of 38.212.

Proposal 7	Regarding Issue 2, i.e., whether a prioritisation between the CSIs is needed to enhance omission rules, no prioritisation between CSIs is needed because the 3 reporting priority levels for WB CSI, even SB and odd SB CSI can be assigned to each CSI in the report.

Proposal 8	Regarding Issue 3, i.e., whether any enhancement to the CPU occupation rules is needed, support improved handling of CPU calculations for Mode 1 and X=1,2, without impacting the rules for legacy CSI reports. This can be done by

- introducing X+1 separate CPU occupations for an MTRP CSI report n configured with Mode 1 and X=1 or 2: OCPU(n,x), with x=0,…,X, alongside the legacy OCPUn such that OCPU(n,0)=2N,  OCPU(n,1)=M, for X=1 and OCPU(n,0)=2N,  OCPU(n,1)=M1, OCPU(n,2)=M2, for X=2, and OCPUn=2N+M=x=0XOCPU(n,x);

- adding a dedicated “soft” formula in Sec. 5.2.1.6 of 38.214 for the case when the first CSI report exceeding the CPU count is an MTRP CSI report configured with Mode 1 and X=1 or 2.

Proposal 9	Text proposal for the additional “soft” formula for CSI updates at the end of the first paragraph of Sec 5.2.1.6 in 38.214:

If CSI report M' is the first CSI report exceeding the CPU count and it is configured with Mode 1 and X=1 or 2, where each CSI occupies OCPU(M',x) CPUs, for x=0,…,X, and NCPU-L-n=0M'-1OCPUn CPUs are unoccupied, the UE is expected to update the first Y CSIs and is not required to update the last X+1-Y CSIs, where 0≤Y≤X+1 is the largest value such that x=0Y-1OCPU(M',x)≤NCPU-L-n=0M'-1OCPUn holds.

Proposal 10	Regarding the RI restriction parameters, follow legacy design for Type I reporting and allow multiple candidate values. For the NCJT RI restriction parameter use a four-bit sequence as per legacy design.

Proposal 11	Regarding CBSR, support Alt 2, i.e., two CBSRs, one for each CMR group.

Proposal 12	Reintroduce the parameter sharedCMR in the RRC parameter list, as defined in R1-2108676, because it seems left out from R1-2110573.

Proposal 13	Clarify that there are two RI restriction parameters, one for single-TRP, which can reuse the Rel-15/16 name typeISinglePanel-ri‑Restriction and the new one for NCJT. Change the description for the new parameter to: “RI restriction applicable to all NCJT measurement hypotheses (up to 4 rank combinations)”.

Proposal 14	For the new NCJT RI restriction parameter, add in the description or in value range that it is a bit sequence of length 4, so the relative description can be added to 38.214 as for the legacy Type I RI restriction.

# References

- RP-212535, Samsung, “Revised (3rd) WID: Further enhancements on MIMO for NR,” RAN#93e.

- RAN1 Chair’s Notes, RAN1#106bis-e.

- R1-2110420, Huawei, “Summary of CSI enhancements for MTRP and FDD (Round 0)”, RAN1 #106bis-e.

- R1-2110421, Huawei, “Summary of CSI enhancements for MTRP and FDD (Round 1)”, RAN1 #106bis-e.

- R1-2110422, Huawei, “Summary of CSI enhancements for MTRP and FDD (Round 2)”, RAN1 #106bis-e.

- R1-2110506, Huawei, “Summary of CSI enhancements for MTRP and FDD (Round 3)”, RAN1 #106bis-e.

- R1-2105277, Nokia, Nokia Shanghai Bell, “Enhancement on CSI measurement and reporting”, RAN1 #105-e.

- R1-2108058, Nokia, Nokia Shanghai Bell, “Enhancement on CSI measurement and reporting”, RAN1 #106-e.

- R1-2109876, Nokia, Nokia Shanghai Bell, “Enhancement on CSI measurement and reporting”, RAN1 #106bis-e.

- R1-2108676, “Consolidated higher layers parameter list for Rel-17 NR”, post-RAN1#106-e email discussion.

- R1-2110573, “Consolidated higher layers parameter list for Rel-17 NR”, post-RAN1#106bis-e email discussion.

# Appendix

| Parameter | Value |
| --- | --- |
| Scenario | Dense Urban (Macro only) |
| Frequency Range | 2GHz with duplexing gap of 200MHz between DL and UL |
| Inter-BS distance | 200m |
| Antenna setup and port layouts at gNB | 32 ports: (8,8,2,1,1,2,8), (dH,dV) = (0.5, 0.8)λ |
| Antenna setup and port layouts at UE | 2RX: (1,1,2,1,1,1,1), (dH,dV) = (0.5, 0.5)λ for (rank 1,2) |
| BS Tx power | 44dBm for 20MHz |
| Simulation bandwidth | 20 MHz for 15kHz |
| Channel model for reciprocity | Alt1: based on Section 5.3 of TR 36.897 |
| CSI-RS overhead | All DL RS overhead is included in the DL throughput calculation |
| Traffic model | FTP model 1 with packet size 0.5 Mbytes |
| Traffic load (Resource utilization) | ~70% for SU/MU-MIMO with rank adaptation |
| UE distribution | 80% indoor (3km/h), 20% outdoor (30km/h) |
| UE receiver | MMSE-IRC as the baseline receiver |
| Feedback assumption | Realistic |
| Channel estimation | Realistic |
| SRS error model and configuration | SRS error model in Table A.1-2 in 36.897 with Δ=9 dB (SRS period, comb, # OFDM symbols, # UEs) = (5ms, 4, 4, 8) |
| Calibration error model | HUL=HUL⋅E=HUL⋅a1ejθ1⋯0⋮⋱⋮0⋯aNejθN Amplitude error (expressed in decibel of x=20log10a) and phase error have normal distribution with 0.7dB and 5 degrees standard deviation, respectively. |