3GPP TSG RAN WG1 #107-e	R1-2112195

e-Meeting, November 11th – November 19th, 2021

Agenda item:	7.2.6

Source: 	Qualcomm Incorporated

Title: 	Discussion on ambiguities in KNZ reporting and coefficients partition in eT2 CSI

Document for:	Discussion/Decision

# Introduction

Enhanced Type II codebook and Enhanced port-selection Type II codebook are introduced Rel-16. In CSI feedback based on these two codebooks, the actual number of coefficients, i.e., KNZ, is reported by UE. However, the spec is unclear about how the codepoints of KNZ reporting are mapped to the candidate values. Also, the spec is unclear how to partition the coefficients when KNZ≤2*ν. In this contribution, we will discuss these two issues and propose solutions to clarify them in the spec.

# Issue 1: codepoints mapping in KNZ reporting

In current 214 spec, section 5.2.2.2.5 and section 5.2.2.2.6, KNZ is defined as following: “KlNZ=i=02L-1f=0Mυ-1kl,i,f(3)≤K0 is the number of nonzero coefficients for layer l=1,…,υ and KNZ=l=1υKlNZ≤2K0 is the total number of nonzero coefficients”. Given this definition, it is clear that KNZ takes the value between 1 and 2K0. In current 212 spec, it further specified that the bitwidth to report KNZ is log2K0 if maximum configured rank is rank-1 or log22K0 otherwise. However, it is unclear whether UE should encode KNZ values starting from codepoint “0” or codepoint “1”. For instance, if 2K0=6, UE will use 3-bit to encode KNZ, but only 6 out of 8 codepoints are valid. The first option is mapping the KNZ candidate values to codepoint {000, 001, 010, 011, 100, 101}, the second option is mapping KNZ values directly to binary bits meaning that the KNZ candidate values are mapped to codepoint {001, 010, 011, 100, 101, 110}. In our view, this encoding procedure is similar to RI, thus it is more straightforward to reuse the scheme as RI encoding. For RI the smallest allowable rank is mapped to codepoint “0”. Based on that, we propose

Proposal 1: In Rel-16 enhanced Type II and enhanced Type II port-selection codebook, clarify that the codepoints of KNZ indicator field are mapped to KNZ indicator in increasing order where codepoint “0” is mapped to the smallest allowed KNZ indicator value.

Following spec change can be considered for TS38.212 spec:

### 6.3.2 Uplink control information non PUCSH

### 6.3.2.1.2 CSI only

>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>> unchanged text omitted <<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<

Table 6.3.2.1.2-8: RI and CQI of codebookType=typeII-r16 or typeII-PortSelection-r16

| Field | Bitwidth |
| --- | --- |
| Rank Indicator | min2,log2nRI |
| Wide-band CQI | 4 |
| Subband differential CQI | 2 |
| Indicator of the total number of non-zero coefficients summed across all layers KNZ | log2(K0) if max allowed rank is 1; log2(2K0) otherwise |

where nRI is the number of allowed rank indicator values according to Clauses 5.2.2.2.5 and 5.2.2.2.6 [6, TS 38.214], K0=2Lp1×N3Rβ, where p1, N3, R, and β are given by Clause 5.2.2.2.5 and 5.2.2.2.6 in [6, TS 38.214]. The values of the rank indicator field are mapped to allowed rank indicator values with increasing order, where '0' is mapped to the smallest allowed rank indicator value. The values of the KNZ indicator field are mapped to the allowed values of KNZ values according to Clauses 5.2.2.2.5 and 5.2.2.2.6 [6, TS 38.214] with increasing order, where ‘0’ is mapped to the smallest allowed KNZ indicator value.

>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>> unchanged text omitted <<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<

# Issue 2: Coefficient partition

In reporting of the linear combination coefficients, the total KNZ non-zero coefficients are partitioned into two groups, wherein the first KNZ/2-ν coefficients are said to have higher priority and are packed firstly (omitted secondly) in Group 1, while the remaining KNZ/2 coefficients are said to have lower priority and are packed secondly (omitted firstly) in Group 2. However, if KNZ<2*ν, the value KNZ/2-ν is negative and it causes ambiguity in partitioning. In our view, since the strongest coefficients are reported separately via strongest coefficient indicator for each layer, the intention of the grouping is to partition the remaining KNZ-ν coefficients into two groups. Hence, the most straightforward solution is the pack the first KNZ-ν2 higher priority coefficients into Group 1, while the remaining KNZ-ν2 are packed into Group 2. Based the discussion, we propose

Proposal 2: For Rel-16 Enhanced Type II and Enhanced Type II port-selection, clarify that UCI Group 1 includes the KNZ-ν2 highest priority elements of i2,4,l and the  KNZ-ν2 highest priority elements of i2,5,l (l=1,…,υ. Also, UCI Group 2 includes the KNZ-ν2 lowest priority elements of i2,4,l and the KNZ-ν2 lowest priority elements of i2,5,l (l=1,…,υ).

Following text proposal can be considered for TS38.214 spec:

### 5.2.3 CSI reporting using PUSCH

>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>> unchanged text omitted <<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<

-	For Enhanced Type II reports, for a given CSI report n, each reported element of indices  i2,4,l i2,5,l and i1,7,l, indexed by l,i and f, is associated with a priority value Pril,i,f=2⋅L⋅υ⋅πf+υ⋅i+l, with π(f)=min⁡(2⋅n3,lf,2⋅N3-n3,l(f)-1) with l=1,2,…,υ, i=0,1,…,2L-1, and f=0,1,…,Mυ-1, and where n3,l(f) is defined in Clause 5.2.2.2.5. The element with the highest priority has the lowest associated value Pril,i,f. Omission of Part 2 CSI is according to the priority order shown in Table 5.2.3-1, where

-	Group 0 includes indices i1,1, i1,2 and i1,8,l (l=1,…,υ).

-	Group 1 includes indices i1,5 (if reported), i1,6,l (if reported), the υ2LMυ-KNZ/2 highest priority elements of i1,7,l,  i2,3,l, the  KNZ-ν2 KNZ/2-υ highest priority elements of i2,4,l and the  KNZ-ν2 KNZ/2-υ highest priority elements of i2,5,l (l=1,…,υ).

-	Group 2 includes the KNZ/2 lowest priority elements of i1,7,l, the KNZ-ν2 KNZ/2 lowest priority elements of i2,4,l and the KNZ-ν2 KNZ/2 lowest priority elements of i2,5,l (l=1,…,υ).

>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>> unchanged text omitted <<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<

Following text proposal can be considered for TS38.212 spec:

### 6.3.2 Uplink control information non PUCSH

### 6.3.2.1.2 CSI only

>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>> unchanged text omitted <<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<

Table 6.3.2.1.2-5A: Mapping order of CSI fields of one CSI report, CSI part 2 of codebookType=typeII-r16 or typeII-PortSelection-r16

| CSI report number | CSI fields |
| --- | --- |
| CSI report #n CSI part 2, group 0 | PMI fields X1, from left to right as in Tables 6.3.2.1.2-1A/2A, if reported |
| CSI report #n CSI part 2, group 1 | The following PMI fields X2, from left to right, as in Tables 6.3.2.1.2-1A/2A: i2,3,l:l=1,…,υ, i1,5, i1,6,l:l=1,…,υ and KNZ-ν2(KNZ2-υ)×3 highest priority bits of i2,4,l:l=1,…,υ,KNZ-ν2(KNZ/2-υ)×4 highest priority bits of {i2,5,l:l=1,…,υ} and ν*2LMυ-KNZ/2 highest priority bits ofi1,7,l:l=1,…,υ, in decreasing order of priority based on function Pril,i,f defined in clause 5.2.3 of TS38.214, if reported |
| CSI report #n CSI part 2, group 2 | The following PMI fields X2, from left to right, as in Tables 6.3.2.1.2-1A/2A: KNZ-ν2KNZ/2×3 lowest priority bits of i2,4,l:l=1,…,υ, KNZ-ν2KNZ/2×4 lowest priority bits of i2,5,l:l=1,…,υ and KNZ/2 lowest priority bits of i1,7,l:l=1,…,υ, in decreasing order of priority based on function Pril,i,f defined in clause 5.2.3 of TS38.214, if reported |

>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>> unchanged text omitted <<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<

# Conclusion

In this contribution, we discuss issues in KNZ codepoints mapping and coefficients partition in enhanced Type II and enhanced Type II port-selection codebook. Following is the summary of proposals and we suggest the text proposal in Section 2 and 3.

Proposal 1: In Rel-16 enhanced Type II and enhanced Type II port-selection codebook, clarify that the codepoints of KNZ are mapped in increasing order to the candidate values of KNZ with the lowest codepoint mapped to KNZ=1.

Proposal 2: For Rel-16 Enhanced Type II and Enhanced Type II port-selection, clarify that UCI Group 1 includes the KNZ-ν2 highest priority elements of i2,4,l and the  KNZ-ν2 highest priority elements of i2,5,l (l=1,…,υ. Also, UCI Group 2 includes the KNZ-ν2 lowest priority elements of i2,4,l and the KNZ-ν2 lowest priority elements of i2,5,l (l=1,…,υ).