3GPP TSG RAN WG1 #107-e			  R1-2112248

e-Meeting, November 11th – 19th, 2021

Agenda item:	8.16.3

Source: 	Qualcomm Incorporated

Title: 	UE features for enhanced IIOT and URLLC

Document for:	Discussion/Decision

- Introduction

This contribution includes proposals on RAN1 UE features for Rel-17 IIOT/URLLC.

- Discussion on IIOT/URLLC UE features

Based on the “Updated RAN1 UE features list for Rel-17 NR after RAN1 #106bis-e” provided in R1-2110587 [1], some proposals are provided.

First of all, it is observed that the type of many features is “per UE” in R1-2108679. We think this is very problematic. If we make a feature per-UE, then we make it a necessity that the UE implements and tests the feature in unlicensed (unconditionally) and in NTN (conditioned on how NTN features will be handled in general).  As long as unlicensed base stations don't get this feature implemented, testing is not possible, and the feature is going to get effectively disabled across the board.

Due to the above concern, we make the following proposal.

Proposal 1: Unless otherwise stated, the type for a UE feature should be at least per band (if not with finer granularity type), given the potential UE testing differentiation among licensed, unlicensed, and NTN band.

In the following, we analyze each UE feature and make some proposals to capture the key changes on UE features for Rel-17 IIOT/URLLC.

With regards to the Feature 25-1 (SPS HARQ Deferral in TDD), the feature is not necessary at FDD. Prerequisite for this feature is SPS support - feature 5-18-whose RRC Parent IE is Phy-ParametersCommon and therefore the feature should be supported per Feature Set per Component Carrier (FSPC).

Proposal 2: Feature 25-1 (SPS HARQ Deferral in TDD collision) is for TDD only. Furthermore, it should a per Feature Set Per Component Carrier (FSPC) feature rather than a per UE feature.

With regards to 25-2 - Repetitions for PUCCH Format 0 and 2 over multiple slots-it would be useful to clarify that the HARQ Codebook is slot-based. The prerequisite for 25-2 is 4-23 – support for PUCCH repetitions- whose RRC Parent IE is Phy-ParametersCommon and therefore the feature should be supported per Feature Set per Component Carrier (FSPC).

Proposal 3: For feature 25-2 (Repetitions for PUCCH Format 0 and 2 over multiple slots) a clarification needs to be made that the feature is for slot-based codebook. Furthermore, the feature should be supported per Feature Set Per Component Carrier (FSPC) rather than per UE.

With regards to 25-3 - Repetitions for PUCCH Format 0, 1, 2, 3, 4 over multiple sub-slots-the prerequisites for 25-2 are:

- 4-23 – support for PUCCH repetitions - whose RRC Parent IE is Phy-ParametersCommon and

- feature 11-3 - support for more than 1 PUCCH HARQ within a slot - whose RRC Parent IE is a FeatureSetUplink-v1610 feature.

Therefore the feature should be supported per Feature Set per Component Carrier (FSPC).

Proposal 4: For feature 25-3 (Repetitions for PUCCH Format 0, 1, 2, 3, 4 over multiple sub-slots) the feature should be supported per Feature Set Per Component Carrier (FSPC).

With regards to 25-4 -One-shot HARQ feedback triggered by DCI 1_2-the prerequisites are:

- 10-16 (support for 1-shot HARQ-ACK feedback) and

- 11-1 (monitoring DCI 1_2)

10-16 has RRC Parent IE SharedSpectrumChAccessParamsPerBand-r16 and 11-1 Phy-ParametersCommon. Therefore the feature should be supported per Feature Set per Component Carrier (FSPC).

Proposal 5: Feature 25-4 (One-shot HARQ feedback triggered by DCI 1_2) should be supported per Feature Set Per Component Carrier (FSPC).

With regards to 25-5 - PHY layer priority for One-shot HARQ feedback - the prerequisites are:

- 10-16 (support for 1-shot HARQ-ACK feedback) and

- 11- 4 (2 HARQ-ACK codebooks with up to 1 sub-slot based HARQ-ACK codebook)

10-16 has RRC Parent IE SharedSpectrumChAccessParamsPerBand-r16 and 11-4 FeatureSetUplink-v1640.

Therefore the feature should be supported per Feature Set per Component Carrier (FSPC).

Proposal 6: Feature 25- 5 (PHY layer priority for One-shot HARQ feedback) should be supported per Feature Set Per Component Carrier (FSPC).

With regards to 25-6 - Enhanced Type 3 HARQ-ACK feedback - the prerequisites are:

- 10-16 (support for 1-shot HARQ-ACK feedback) and

10-16 has RRC Parent IE SharedSpectrumChAccessParamsPerBand-r16. Therefore the feature should be supported per Feature Set per Component Carrier (FSPC).

Proposal 7: Feature 25- 6 (Enhanced Type 3 HARQ-ACK feedback) should be supported per Feature Set Per Component Carrier (FSPC).

With regards to 25-7 - Triggered HARQ-ACK codebook retransmission - the feature should be supported per band.

Proposal 8: Feature 25-7 (Triggered HARQ-ACK codebook retransmission) should be supported per Feature Set Per band.

With regards to 25-8 - Semi-static HARQ-ACK codebook for sub-slot PUCCH - the prerequisites are:

- 4-11 (support for semi-static HARQ-ACK feedback) and

- 11-3 (support for more than 1 PUCCH HARQ within a slot)

4-11 has RRC Parent IE Phy-ParametersCommon and 11-3 FeatureSetUplink-v1610.

Therefore the feature should be supported per Feature Set per Component Carrier (FSPC).

Proposal 9: Feature 25-8 (Semi-static HARQ-ACK codebook for sub-slot PUCCH) should be supported per Feature Set Per Component Carrier (FSPC).

With regards to the Feature 25-9, the feature is not necessary at FDD. The feature should be supported, for obvious reasons, per Feature Set per Band combination.

Proposal 10: Feature 25-9 (semi-static PUCCH cell switch) and 25-10 (dynamic PUCCH cell switch) are for TDD only. Furthermore, they should be per band combination rather than per UE.

Regarding to feature 25-15, the current formulation is problematic. As we explained in R1-2110181 [2], a UE needs extra processing time to handle the cancel/dropping of a LP PUSCH before switch to transmit a HP PUSCH. One should notice that cancel/drop a PUSCH is more complicated than cancel/drop a PUCCH, which is why extra processing time is needed on top of Rel-16 cancellation time for cancel/drop a PUCCH.

Proposal 11: For feature 25-15 (PHY prioritization of overlapping high-priority DG-PUSCH and low-priority CG-PUSCH), on top of Rel-16 cancellation time (N2+d1) for PUCCH/PUCCH or PUCCH/PUSCH collision, additional time d2 is needed (which results N2+d1+d2 in total cancellation time) for LP CG-PUSCH and HP DG-PUSCH collision resolution. The additional number of OFDM symbols (d2) needed is listed in following table.

Table 1. d2 for LP CG-PUSCH and HP DG-PUSCH collision resolution

|  | d2 [symbols] |
| --- | --- |
| 0 | 1 |
| 1 | 2 |
| 2 | 4 |
| 3 | 8 |

Regarding to feature 25-16 and 25-17, each individual component under these two features should have independent capability signaling, e.g., a UE support the 1st component of 25-16 (LP HARQ-ACK and HP HARQ multiplexing on PUCCH) does not necessily have the capability to support the 3rd component of 25-16 (LP HARQ-ACK, HP HARQ-ACK, and SR multiplexing on a PUCCH). Separate capabilities are needed to allow different UE implementations.

Proposal 12: The components 1, 2, and 3 of feature 25-16 (HARQ-ACK with different priorities multiplexing using a PUCCH), should be in separate rows to have separate capability signaling. Furthermore, support a new component 4 (multiplexing a low-priority HARQ-ACK, a high priority HARQ-ACK, a high-priority/low-priority SR, and CSI into a PUCCH) with separate capability signaling.

Proposal 13: The components 1, 2, 3, and 4 of feature 25-17 (HARQ-ACK piggybacked on a PUSCH of a different priority), should be in separate rows to have separate capability signaling.

- Proposed modification for IIOT/URLLC in UE feature list table

The detailed changes according to the proposals in above section is captured in the following table. Hence we make the following proposal.

Proposal 14: Adopt the following table for Rel-17 IIOT/URLLC UE features.

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (Sidelink WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 25. NR_IIOT_URLLC_enh | 25-1 | SPS HARQ-ACK deferral in case of TDD collision | 1. Idenfify HARQ-ACK bits of active SPS configurations for deferral in the initial PUCCH slot 2. Determination of the target PUCCH slot for SPS HARQ-ACK deferral 3. Multiplexing and transmission of deferred SPS HARQ-ACK information in the target PUCCH slot FFS whether to separate capability for handling of the collision for the same HARQ process due to deferred SPS HARQ-ACK | 5-18 | Yes | N/A |  | Per UE Per FSPC | No (TDD only) | No | N/A |  | Optional with capability signaling |
| 25. NR_IIOT_URLLC_enh | 25-2 | Repetitions for PUCCH format 0, and 2 over multiple slots (for slot based codebook) with K = 2, 4, 8 | Repetitions for PUCCH format 0 and 2 over multiple slots with K = 2, 4, 8 | 4-23 | Yes | N/A |  | Per UE Per FSPC | No | No | N/A |  | Optional with capability signaling |
| 25. NR_IIOT_URLLC_enh | 25-3 | Repetitions for PUCCH format 0, 1, 2, 3 and 4 over multiple PUCCH subslots with configured K = 2, 4, 8 | Repetitions for PUCCH format 0, 1, 2, 3 and 4 over multiple PUCCH subslots with RRC configured repetition factor K = 2, 4, 8 FFS whether to separate the capability per UCI type | 4-23 11-3 | Yes | N/A |  | Per UE Per FSPC | No | No | N/A |  | Optional with capability signaling |
| 25. NR_IIOT_URLLC_enh | 25-3a | Repetitions for PUCCH format 0, 1, 2, 3 and 4 over multiple PUCCH subslots using dynamic repetition indication | Repetitions for PUCCH format 0, 1, 2, 3 and 4 over multiple PUCCH subslots based on dynamic repetition indication. FFS whether to separate the capability per UCI type | 25-3 30-5 | Yes | N/A |  | Per UE Per FSPC | No | No | N/A |  | Optional with capability signaling |
| 25. NR_IIOT_URLLC_enh | 25-4 | One-shot HARQ ACK feedback triggered by DCI format 1_2 | 1. Support feedback of type 3 HARQ-ACK codebook, triggered by a DCI 1_2 scheduling a PDSCH 2. Support feedback of type 3 HARQ-ACK codebook, triggered by a DCI 1_2 without scheduling a PDSCH using a reserved FDRA value | 10-16 11-1 | Yes | N/A |  | Per UE Per FSPC | No | No | N/A |  | Optional with capability signaling |
| 25. NR_IIOT_URLLC_enh | 25-5 | PHY priority handling for one-shot HARQ ACK feedback | Support transmission of type 3 HARQ-ACK codebook using the first or second PUCCH configuration based on PHY priority indication in the triggering DCI | 10-16 11-4 | Yes | N/A |  | Per UE Per FSPC | No | No | N/A |  | Optional with capability signaling |
| 25. NR_IIOT_URLLC_enh | 25-6 | Enhanced type 3 HARQ-ACK codebook feedback | 1. Support feedback of enhanced type 3 HARQ-ACK codebook, triggered by a DCI 1_1 and DCI format 1_2 (for a UE supporting DCI format 1_2, 11-1) 2. Support configuration of up to X enhanced type 3 HARQ-ACK codebooks. 3. Support feedback of a dynamically selected enhanced type 3 HARQ-ACK codebook based on triggering information in DCI 1_1 and DCI 1_2 (for a UE supporting DCI format 1_2, 11-1) 4. Support transmission of enhanced type 3 HARQ-ACK codebook using the first or second PUCCH configuration based on PHY priority indication in the triggering DCI (for a UE supporting two HARQ-ACK codebooks / PUCCH config in 11-4) | 10-16 | Yes | N/A |  | Per UE Per FSPC | No | No | N/A | For component 2, the UE indicates its capability in the number of enhanced type 3 HARQ-ACK codebooks: {1,...,X} For component 3, the dynamic indication is only supported if the UE for component 2 supports more than one enhanced type 3 HARQ-ACK codebook to be configured | Optional with capability signaling |
| 25. NR_IIOT_URLLC_enh | 25-7 | Triggered HARQ-ACK codebook re-transmission | 1. Support HARQ-ACK re-transmission from an earlier PUCCH slot based on the triggering information in DCI format 1_1 and DCI format 1_2 (for a UE supporting DCI format 1_2, 11-1) 2. Support the related PHY priority handling in terms of HARQ-ACK codebook selection and the applicable PUCCH configuration (for a UE supporting two HARQ-ACK codebooks / PUCCH config in 11-4) |  | Yes | N/A |  | Per UE Per Band | No | No | N/A |  | Optional with capability signaling |
| 25. NR_IIOT_URLLC_enh | 25-8 | Semi-static HARQ-ACK codebook for sub-slot PUCCH | Semi-static (Type 1) HARQ-ACK codebook for sub-slot based PUCCH configuration | 4-11 11-3 | Yes | N/A |  | Per UE Per FSPC | No | No | N/A |  | Optional with capability signaling |
| 25. NR_IIOT_URLLC_enh | 25-9 | Semi-static PUCCH cell switching | Semi-static PUCCH cell switching using configured time-domain domain pattern of applicable PUCCH cell / carrier FFS whether to separate the capability for different numerologies |  | Yes | N/A |  | Per UE Per BC | No (TDD only) | No | N/A |  | Optional with capability signaling |
| 25. NR_IIOT_URLLC_enh | 25-10 | PUCCH cell switching based on dynamic indication | PUCCH cell switching based on dynamic indication in the DCI scheduling the PUCCH FFS whether to separate the capability for different numerologies |  | Yes | N/A |  | Per UE Per BC | No (TDD only) | No | N/A |  | Optional with capability signaling |
| 25. NR_IIOT_URLLC_enh | 25-11 | 4-bits subband CQI | Subband CQI reporting with 4 bits per subband |  | Yes | N/A |  | Per UE Per Band | No | No | N/A |  | Optional with capability signaling |
| 25. NR_IIOT_URLLC_enh | 25-12 | UE initiating a semi-static channel occupancy with configurations dependent on gNB semi-static channel access configurations | Support initiating a semi-static channel access occupancy by the UE where the corresponding period is the same as, integer multiple of, or inter-factor of the period configured for a semi-static channel occupancy that can be initiated by gNB. | 10-1a | Yes | N/A |  | Per band | N/A | N/A | N/A | The signaling is per band but is only expected for a band where shared spectrum channel access must be used | Optional with capability signaling |
| 25. NR_IIOT_URLLC_enh | 25-13 | UE initiating a semi-static channel occupancy with independent configurations from gNB semi-static channel access configurations | Support initiating a semi-static channel access occupancy by the UE where the corresponding period is independently configured from the period configured for a semi-static channel occupancy that can be initiated by gNB. | 10-1a, 25-12 | Yes | N/A |  | Per band | N/A | N/A | N/A | The signaling is per band but is only expected for a band where shared spectrum channel access must be used | Optional with capability signaling |
| 25. NR_IIOT_URLLC_enh | [25-14] | PHY prioritization of overlapping low-priority DG-PUSCH and high-priority CG-PUSCH | Support PHY prioritization for the case where low-priority DG-PUSCH collides with high-priority CG-PUSCH | 12-1 | Yes | N/A |  | Per band Per FS | N/A | N/A | N/A |  | Optional with capability signaling |
| 25. NR_IIOT_URLLC_enh | [25-15] | PHY prioritization of overlapping high-priority DG-PUSCH and low-priority CG-PUSCH | Support PHY prioritization of overlapping high-priority dynamic grant PUSCH and low-priority configured grant PUSCH on a BWP of a serving cell with additional cancellation time d2 on top of N2+d1 (which is Rel-16 PUCCH/PUCCH or PUCCH/PUSCH cancellation time), which results total cancellation time N2+d1+d2. Note: the value of d2 is given in table as below. Table: d2 for LP CG-PUSCH and HP DG-PUSCH collision resolution d2 [symbols]. 0; 1. 1; 2. 2; 4. 3; 8. | 12-1 | Yes | N/A |  | Per band Per FS | N/A | N/A | N/A |  | Optional with capability signaling |
| 25. NR_IIOT_URLLC_enh | 25-16a | HARQ-ACK with different priorities multiplexing using a PUCCH | 1. Support multiplexing a high-priority HARQ-ACK and a low-priority HARQ-ACK into a PUCCH. Support separate coding for the two HARQ-ACKs. 2. [Support multiplexing a low-priority HARQ-ACK and a high-priority SR into a PUCCH for some HARQ-ACK/SR PF combinations (FFS applicable combinations).] 3. [Support multiplexing a low-priority HARQ-ACK, a high-priority HARQ-ACK and a high-priority SR into a PUCCH.] FFS whether to merge with FG 25-17 FFS whether to separate capability for different UCI type | 11-3 | Yes | N/A |  | Per UE Per FS | No | No | N/A |  | Optional with capability signaling |
| 25. NR_IIOT_URLLC_enh | 25-16b | HARQ-ACK and SR with different priorities multiplexing using a PUCCH | Support multiplexing a low-priority HARQ-ACK and a high-priority SR into a PUCCH for some HARQ-ACK/SR PF (FFS applicable combinations). | 11-3 | Yes | N/A |  | Per FS | No | No | N/A |  | Optional with capability signaling |
| 25. NR_IIOT_URLLC_enh | 25-16c | High-priority HARQ-ACK, low priority HARQ-ACK, and SR multiplexing using a PUCCH | Support multiplexing a low-priority HARQ-ACK, a high-priority HARQ-ACK and a high-priority SR into a PUCCH. | 11-3 | Yes | N/A |  | Per FS | No | No | N/A |  | Optional with capability signaling |
| 25. NR_IIOT_URLLC_enh | 25-17a | HARQ-ACK piggybacked on a PUSCH of a different priority Low-priority HARQ-ACK multiplexing on High-priority PUSCH | 1. Support multiplexing a low-priority HARQ-ACK in a high-priority PUSCH (conveying UL-SCH only). Support separate beta_offset values for this priority combination. 2. Support multiplexing a high-priority HARQ-ACK in a low-priority PUSCH (conveying UL-SCH only). Support separate beta_offset values for this priority combination. 3. Support multiplexing a low-priority HARQ-ACK, a high-priority PUSCH conveying UL-SCH, a high-priority HARQ-ACK and/or CSI. 4. Support multiplexing a high-priority HARQ-ACK, a low-priority PUSCH conveying UL-SCH, a low-priority HARQ-ACK and/or CSI. FFS whether to merge into FG 25-16 FFS whether to separate capability for different UCI type | 11-3 12-1 | Yes | N/A |  | Per UE Per FS | No | No | N/A |  | Optional with capability signaling |
| 25. NR_IIOT_URLLC_enh | 25-17b | High-priority HARQ-ACK multiplexing on low-priority PUSCH | Support multiplexing a high-priority HARQ-ACK in a low-priority PUSCH (conveying UL-SCH only). Support separate beta_offset values for this priority combination. | 11-3 12-1 | Yes | N/A |  | Per FS | No | No | N/A |  | Optional with capability signaling |
| 25. NR_IIOT_URLLC_enh | 25-17c | multiplexing a low-priority HARQ-ACK, a high-priority PUSCH conveying UL-SCH, a high-priority HARQ-ACK and/or CSI | Support multiplexing a low-priority HARQ-ACK, a high-priority PUSCH conveying UL-SCH, a high-priority HARQ-ACK and/or CSI | 11-3 12-1 | Yes | N/A |  | Per FS | No | No | N/A |  | Optional with capability signaling |
| 25. NR_IIOT_URLLC_enh | 25-17d | multiplexing a high-priority HARQ-ACK, a low-priority PUSCH conveying UL-SCH, a low-priority HARQ-ACK and/or CSI | Support multiplexing a high-priority HARQ-ACK, a low-priority PUSCH conveying UL-SCH, a low-priority HARQ-ACK and/or CSI | 11-3 12-1 | Yes | N/A |  | Per FS | No | No | N/A |  | Optional with capability signaling |
| 25. NR_IIOT_URLLC_enh | 25-18 | Parallel PUCCH and PUSCH transmission across CCs in inter-band CA | Support simultaneous PUCCH/PUSCH transmissions on different cells [at least] for inter-band CA. |  | Yes | N/A |  | Per UE Per BC | No | No | N/A |  | Optional with capability signaling |

- References

- 3GPP TSA RAN1 Meeting #106bis-e, R1-2110587, “Updated RAN1 UE features list for Rel-17 NR after RAN1 #106bis-e,” e-Meeting, October 11th – 19th, 2021

- 3GPP TSA RAN1 Meeting #106bis-e, R1-2110181, “Intra-UE multiplexing and prioritization for IOT and URLLC,” e-Meeting, Oct 11th – 19th, 2021