3GPP TSG RAN WG1 #107-e			R1-2112257

e-Meeting, November 11th – 19th, 2021

Agenda item:	8.16.12

Source: 	Qualcomm Incorporated

Title: 	UE features for MBS

Document for:	Discussion and Decision

- 1. Introduction

In last RAN1 meeting, we have the following agreements on UE features for Rel-17 NR MBS.

This contribution includes our views on each FG and the tracking changes are used with respect the version of the agreed FGs.

Given the potential UE testing differentiation among licensed, unlicensed, and NTN band, the type for a UE feature should be ‘per Band’ for broadcast and ‘per FSPC’ for multicast in Rel-17 NR MBS.

Agreement

FG 33-1 is kept as “Broadcast” as follows.

| 33. NR_MBS | 33-1 | Broadcast | - Support of group-common PDCCH/PDSCH with CRC scrambled by MCCH-RNTI. - Support of group-common PDCCH/PDSCH with CRC scrambled by G-RNTI. - Support of CFR configuration for broadcast. - Support of CORESET and common search space for broadcast. - Support of DCI format 1_0 with CRC scrambled with G-RNTI/MCCH-RNTI for broadcast. - Support of inter-slot TDM between unicast PDSCH and group-common PDSCH in different slots. - Support MCCH change notification indication via DCI. |  | Yes |  |  | Per UE | No | No |  |  | Optional without capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Note that yellow highlight means FFS and to be discussed further. These parts are provides as placeholders.

Agreement

FG 33-2-x is added as “Multiple G-RNTIs for group-common PDSCHs” as follows.

| 33. NR_MBS | 33-2-x | Multiple G-RNTIs for group-common PDSCHs | Capability on number of G-RNTI for groupcast FFS details. | 33-2 | Yes |  |  | Per UE | No | No |  |  | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Note that yellow highlight means FFS and to be discussed further. These parts are provides as placeholders.

Agreement

FG 33-3-1 is kept as “Slot-level repetition for group-common PDSCH” as follows.

| 33. NR_MBS | 33-3-1 | Slot-level repetition for group-common PDSCH | - Support slot-level repetition for group-common PDSCH for multicast. FFS whether to split FG 33-1-1 into two FGs for semi-static slot-level repetition and dynamic slot-level repetition | 33-2 | Yes |  |  | Per UE | No | No |  |  | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Note that yellow highlight means FFS and to be discussed further. These parts are provides as placeholders.

Agreement

FG 33-3-4 is merged into FG 33-3-3 as follows.

| 33. NR_MBS | 33-3-3 | Intra-slot TDM-ed unicast PDSCH and group-common PDSCH | - Support TDM between one unicast PDSCH and one group-common PDSCH in a slot. - Support TDM between M (M>1) TDMed unicast PDSCHs and one group-common PDSCH in a slot per CC - Support TDM among N (N>1) group-common PDSCHs in a slot per CC - Support TDM between K (K>1) TDMed unicast PDSCHs and L (L>1) TDMed group-common PDSCHs in a slot per CC - The UE maximum number of TDMed PDSCH receptions capability in a slot per CC is kept as for Rel-15/Rel-16, i.e., {2/4/7} based on UE FG5-11/5-11a/5-11b. - Note: Group-common PDSCH(s) are counted as unicast PDSCH(s). - Support TDM-ed Type-1 HARQ-ACK codebook for multicast. - Support TDM-ed Type-2 HARQ-ACK codebook for multicast. FFS whether/how to separate the capability for HARQ-ACK codebook | 33-1, 33-2 | Yes |  |  | Per UE | No | No |  |  | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Note that yellow highlight means FFS and to be discussed further. These parts are provides as placeholders.

Agreement

FG 33-3-2 is kept as “FDM-ed unicast PDSCH and group-common PDSCH” as follows.

| 33. NR_MBS | 33-3-2 | FDM-ed unicast PDSCH and group-common PDSCH | - Support FDM between one unicast PDSCH and one group-common PDSCH in a slot. - Support FDM-ed Type-1 HARQ-ACK codebook for multicast. - Support FDM-ed Type-2 HARQ-ACK codebook for multicast. FFS whether/how to separate the capability for HARQ-ACK codebook | 33-1, 33-2 | Yes |  |  | Per UE | No | No |  |  | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Note that yellow highlight yellow means FFS and to be discussed further. These parts are provides as placeholders.

Agreement

FG 33-6-1 is kept as “DL priority indication for multicast in DCI” as follows.

| 33. NR_MBS | 33-6-1 | DL priority indication for multicast in DCI | 1. Support of priority indicator field configured in DCI formats 1_1 with CRC scrambled with G-RNTI for multicast. FFS whether/how to separate the above capability from FG 33-6-1 | 33-2 | Yes |  |  | Per UE | No | No |  |  | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Note that yellow highlight means FFS and to be discussed further. These parts are provides as placeholders.

Agreement

FG 33-4 is kept as “NACK-only based HARQ-ACK feedback for multicast” as follows.

| 33. NR_MBS | 33-4 | NACK-only based HARQ-ACK feedback for multicast | - Support NACK-only based HARQ-ACK feedback. | 33-2 | Yes |  |  | Per UE | No | No |  |  | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Note that yellow highlight means FFS and to be discussed further. These parts are provides as placeholders.

Agreement

FG 33-2 is kept as “Dynamic scheduling for multicast” as follows

| 33. NR_MBS | 33-2 | Dynamic scheduling for multicast | - Support of group-common PDCCH/PDSCH with CRC scrambled by G-RNTI. - Support of CFR configuration for multicast. - Support of CORESET and common search space configuration for multicast. - Support of DCI format 1_0 / 1_1 with CRC scrambled with G-RNTI for multicast. - Support of inter-slot TDM between unicast PDSCH and group-common PDSCH in different slots. - Support ACK/NACK based HARQ-ACK feedback, and support enabling/disabling ACK/NACK based HARQ-ACK feedback configured by RRC signaling. - Support PTM retransmission for multicast. - Support PTP retransmission for multicast. FFS whether/how to separate the above capabilities from FG 33-2 |  | Yes |  |  | Per UE | No | No |  |  | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Note that yellow highlight means FFS and to be discussed further. These parts are provides as placeholders.

Agreement

FG 33-5-1 is kept as “SPS group-common PDSCH for multicast” as follows

| 33. NR_MBS | 33-5-1 | SPS group-common PDSCH for multicast | - Support one SPS group-common PDSCH configuration for multicast - Support ACK/NACK based HARQ-ACK feedback for SPS group-common PDSCH without PDCCH scheduling. - Support NACK-only based HARQ-ACK feedback for SPS group-common PDSCH without PDCCH scheduling. - FFS: HARQ-ACK feedback for SPS group-common with PDCCH scheduling and SPS release PDCCH. - Support slot-level repetition for SPS group-common PDSCH FFS whether/how to separate the above capabilities from FG 33-5-1 | 33-2 | Yes |  |  | Per UE | No | No |  |  | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Note that yellow highlight means FFS and to be discussed further. These parts are provides as placeholders.

Agreement

FG 33-5-2 is kept as “Multiple SPS group-common PDSCH configuration” as follows

| 33. NR_MBS |  |  | 33-5-2 | Multiple SPS group-common PDSCH configuration | - Support [N>1] SPS group-common PDSCH configuration per CFR for multicast - Support [M>=1] activated SPS group-common PDSCH configurations among the N SPS group-common PDSCH configurations per CFR for multicast | 33-2 | Yes |  |  | Per UE | No | No |  |  | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Note that yellow highlight means FFS and to be discussed further. These parts are provides as placeholders.

Agreement

FG 33-7 is kept as “Supporting group-common DCI indicating the enabling/disabling [ACK/NACK based] HARQ-ACK feedback” as follows

| 33. NR_MBS | 33-7 | Supporting group-common DCI indicating the enabling/disabling [ACK/NACK based] HARQ-ACK feedback | 1. Supports the function of group-common DCI indicating the enabling/disabling [ACK/NACK based] HARQ-ACK feedback | 33-2 | Yes |  |  | Per UE | No | No |  |  | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Note that yellow highlight means FFS and to be discussed further. These parts are provides as placeholders.

- FG33-1

| 33. NR_MBS | 33-1 | Broadcast | - Support of group-common PDCCH/PDSCH with CRC scrambled by MCCH-RNTI. - Support of [Nbroadcast>=1 (FFS: value of Nbroadcast)] group-common PDCCH/PDSCH with CRC scrambled by G-RNTI(s) for MTCH. - Support of CFR configuration for broadcast. - Support of CORESET and common search space in a CFR for broadcast. - Support of DCI format 1_0 with CRC scrambled with G-RNTI/MCCH-RNTI for broadcast. - Support of inter-slot TDM between other PDSCH and group-common PDSCH for broadcast in different slots. - Support MCCH change notification indication via DCI. - Support of slot-level repetition for group-common PDSCH scheduled by using DCI format 1_0 - FFS: Support of TRS for GC-PDCCH/PDSCH configured in a CFR for broadcast |  | Yes |  |  | Per Band | N/A | N/A |  |  | Optional without capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

- FG33-2

| 33. NR_MBS | 33-2 | Dynamic scheduling for multicast | - Support of group-common PDCCH/PDSCH with CRC scrambled by G-RNTI(s) for multicast. - Support of CFR configuration for multicast. - Support of CORESET and common search space configuration in a CFR for multicast. - Support of DCI format 1_0 with CRC scrambled with G-RNTI for multicast. - Support of inter-slot TDM between unicast PDSCH and group-common PDSCH for multicast in different slots. - Support of slot-level repetition for group-common PDSCH scheduled by using DCI format 1_0 |  | Yes |  |  | Per FSPC | N/A | N/A |  |  | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 33. NR_MBS | 33-2a | Dynamic multicast with DCI format 1_1 | - Support of DCI format 1_1 with CRC scrambled with G-RNTI for multicast | 33-2 | Yes |  |  | Per FSPC | N/A | N/A |  |  | Optional with capability signalling |
| 33. NR_MBS | 33-2b | ACK/NACK-based feedback for Dynamic scheduling for multicast and PTM retransmission for Dynamic scheduling for multicast | - Support of enabling/disabling ACK/NACK-based HARQ-ACK feedback per G-RNTI for multicast by RRC signaling - Support of PTM retransmission for dynamically scheduled multicast based on G-RNTI | 33-2 | Yes |  |  | Per FSPC | N/A | N/A |  |  | Optional with capability signalling |
| 33. NR_MBS | 33-2c | DCI-based ACK/NACK-based feedback for Dynamic scheduling for multicast and PTM retransmission for Dynamic scheduling for multicast | - Support DCI-based enabling/disabling ACK/NACK based HARQ-ACK feedback configured per G-RNTI by RRC signaling | 33-2b | Yes |  |  | Per FSPC | N/A | N/A |  |  | Optional with capability signalling |
| 33. NR_MBS | 33-2d | PTP retransmission for Dynamic scheduling for multicast | - Support of PTP retransmission for dynamically scheduled multicast based on C-RNTI | 33-2b | Yes |  |  | Per FSPC | N/A | N/A |  |  | Optional with capability signalling |
| 33. NR_MBS | 33-2e | HARQ-ACK feedback mode selection for dynamically scheduled group-common PDSCH for multicast | - Support of selection of ACK/NACK-based or NACK-only-based HARQ-ACK feedback for multicast per G-RNTI. | 33-2b, 33-4 | Yes |  |  | Per FSPC | N/A | N/A |  |  | Optional with capability signalling |

- FG33-2-x

| 33. NR_MBS | 33-2-x | Multiple G-RNTIs for group-common PDSCHs | Capability on max number of G-RNTIs for multicast FFS details. | 33-2 | Yes |  |  | Per FSPC | N/A | N/A |  | FFS: Values = {2, …} | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 33. NR_MBS | 33-2-y | Multiple G-CS-RNTIs for SPS group-common PDSCHs | Capability on max number of G-CS-RNTIs for multicast FFS details. | 33-5-1 | Yes |  |  | Per FSPC | N/A | N/A |  | FFS: Values = {2, …} | Optional with capability signalling |

- FG33-3-1

| 33. NR_MBS | 33-3-1 | Slot-level repetition for group-common PDSCH | - Support slot-level repetition for group-common PDSCH for multicast by using DCI format 1_1. FFS whether to split FG 33-1-1 into two FGs for semi-static slot-level repetition and dynamic slot-level repetition | 33-2, 33-2a | Yes |  |  | Per FSPC | N/A | N/A |  |  | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

- FG33-3-2

| 33. NR_MBS | 33-3-2 | FDM-ed unicast PDSCH and group-common PDSCH | - Support of FDM between one unicast PDSCH and one group-common PDSCH in a slot. | 33-1, 33-2 | Yes |  |  | Per FSPC | N/A | N/A |  | FFS: maximum data rate of (GC-)PDSCHs in a slot per CC. | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 33. NR_MBS | 33-3-2a | Feedback multiplexing for FDM-ed unicast PDSCH and group-common PDSCH | 1. Support of FDM-ed ACK/NACK-based HARQ-ACK codebook for one unicast and one multicast in a slot. | 33-2b, 33-3-2 | Yes |  |  | Per FSPC | N/A | N/A |  | Values = {Type-1 HARQ-ACK codebook only, Type-2 HARQ-ACK codebook only, both} | Optional with capability signalling |

- FG33-3-3

| 33. NR_MBS | 33-3-3 | Intra-slot TDM-ed unicast PDSCH and group-common PDSCH for multicast | - Support TDM between one unicast PDSCH and one group-common PDSCH in a slot. - Support TDM between M (M>1) TDMed unicast PDSCHs and one group-common PDSCH in a slot per CC - Support TDM among N (N>1) group-common PDSCHs in a slot per CC - Support TDM between K (K>1) TDMed unicast PDSCHs and L (L>1) TDMed group-common PDSCHs in a slot per CC - The UE maximum number of TDMed PDSCH receptions capability in a slot per CC is kept as for Rel-15/Rel-16, i.e., {2/4/7} based on UE FG5-11/5-11a/5-11b. - Note: Group-common PDSCH(s) are counted as unicast PDSCH(s). | 33-1, 33-2 | Yes |  |  | Per FSPC | N/A | N/A |  | FFS: maximum data rate of (GC-)PDSCHs in a slot per CC. | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 33. NR_MBS | 33-3-3a | Feedback multiplexing for intra-slot TDM-ed unicast PDSCH and group-common PDSCH for multicast | 1. Support of TDM-ed ACK/NACK-based HARQ-ACK codebook for M>= 1 unicast and N>=1 multicast in a slot. | 33-2b, 33-3-3 | Yes |  |  | Per FSPC | N/A | N/A |  | Values = {Type-1 HARQ-ACK codebook only, Type-2 HARQ-ACK codebook only, both} | Optional with capability signalling |

- FG33-4

| 33. NR_MBS | 33-4 | NACK-only based HARQ-ACK feedback for multicast | - Support of NACK-only based HARQ-ACK feedback. - Support PTM retransmission for dynamically scheduled multicast based on G-RNTI | 33-2 | Yes |  |  | Per FSPC | N/A | N/A |  |  | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 33. NR_MBS | 33-4a | More than one NACK-only based HARQ-ACK feedback for multicast in a PUCCH resource | 1. Support of more than one NACK-only based HARQ-ACK feedback for dynamically scheduled multicast in a PUCCH resource | 33-4 | Yes |  |  | Per FSPC | N/A | N/A |  |  | Optional with capability signalling |

- FG33-5-1

| 33. NR_MBS | 33-5-1 | SPS group-common PDSCH for multicast | - Support of one SPS group-common PDSCH configuration for multicast - Support of [Nmulticast_SPS >=1 (FFS: value of Nmulticast_SPS)] group-common PDCCH/PDSCH with CRC scrambled by G-CS-RNTI(s) for multicast - Support of DCI format 1_0 with CRC scrambled with G-CS-RNTI for multicast. - Support of slot-level repetition for SPS group-common PDSCH by using DCI format 1_0 - FFS: HARQ-ACK feedback for SPS group-common PDCCH activation and SPS release PDCCH. | 33-2 | Yes |  |  | Per FSPC | N/A | N/A |  |  | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 33. NR_MBS | 33-5-1a | SPS multicast using DCI format 1_1 | - Support of DCI format 1_1 with CRC scrambled with G-CS-RNTI for multicast SPS transmission. | 33-5-1 | Yes |  |  | Per FSPC | N/A | N/A |  |  | Optional with capability signalling |
| 33. NR_MBS | 33-5-1b | ACK/NACK-based HARQ ACK feedback for SPS multicast | - Support of enabling/disabling ACK/NACK-based HARQ-ACK feedback for SPS group-common PDSCH without PDCCH scheduling. - Support of PTM retransmission for SPS group-common PDSCH based on G-CS-RNTI | 33-5-1 | Yes |  |  | Per FSPC | N/A | N/A |  |  | Optional with capability signalling |
| 33. NR_MBS | 33-5-1c |  | - Support PTP retransmission for SPS group-common PDSCH based on CS-RNTI | 33-5-1b | Yes |  |  | Per FSPC | N/A | N/A |  |  | Optional with capability signalling |
| 33. NR_MBS | 33-5-1d | NACK-only-based HARQ ACK feedback for SPS multicast | - Support of NACK-only-based HARQ-ACK feedback for SPS group-common PDSCH without PDCCH scheduling. - Support of PTM retransmission for SPS group-common PDSCH based on G-CS-RNTI | 33-5-1 | Yes |  |  | Per FSPC | N/A | N/A |  |  | Optional with capability signalling |
| 33. NR_MBS | 33-5-1e | Slot-level repetition of SPS group-common PDSCH for multicast by using DCI format 1_1 | - Support of slot-level repetition for SPS group-common PDSCH by using DCI format 1_1 FFS whether to split FG 33-5-1e into two FGs for semi-static slot-level repetition and dynamic slot-level repetition | 33-5-1, 33-5-1a | Yes |  |  | Per FSPC | N/A | N/A |  |  | Optional with capability signalling |
| 33. NR_MBS | 33-5-1f | HARQ-ACK feedback mode selection for SPS group-common PDSCH without PDCCH scheduling for multicast | - Support of selection of ACK/NACK-based or NACK-only-based HARQ-ACK feedback for SPS group-common PDSCH without PDCCH scheduling [FFS per G-CS-RNTI]. | 33-5-1b, 33-5-1d | Yes |  |  | Per FSPC | N/A | N/A |  |  | Optional with capability signalling |

- FG33-5-2

| 33. NR_MBS | 33-5-2 | Multiple SPS group-common PDSCH configuration | - Support [N>1] SPS group-common PDSCH configurations per CFR for multicast - Support [N>=M>=1] activated SPS group-common PDSCH configurations among the N SPS group-common PDSCH configurations per CFR for multicast | 33-2 | Yes |  |  | Per FSPC | N/A | N/A |  |  | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

- FG33-6-1

| 33. NR_MBS | 33-6-1a | DL priority of multicast ACK/NACK-based feedback for dynamically scheuled multicast | 1. Support of priority configured for multicast ACK/NACK-based HARQ-ACK feedback of dynamically scheuled multicast Notes: - Two priority indexes are introduced for multicast, with index 0 meaning low priority and index 1 meaning high priority. - The priority of multicast is the same as the priority of unicast for the same priority index of HARQ-ACK at least for ACK/NACK based feedback. | 33-2b | Yes |  |  | Per FSPC | N/A | N/A |  |  | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 33. NR_MBS | 33-6-1b | DL priority of multicast NACK-only-based feedback for dynamically scheuled multicast | 1. Support of priority configured for multicast NACK-only-based HARQ-ACK feedback of dynamically scheuled multicast Notes: - Two priority indexes are introduced for multicast, with index 0 meaning low priority and index 1 meaning high priority. - The priority of multicast for NACK-only based feedback is the same as the priority of unicast for the same priority index of HARQ-ACK. | 33-4 | Yes |  |  | Per FSPC | N/A | N/A |  |  | Optional with capability signalling |
| 33. NR_MBS | 33-6-1c | DL priority indication for dynamically scheduled multicast in DCI | 1. Support of priority indicator field configured in DCI formats 1_1 with CRC scrambled with G-RNTI for multicast. | 33-2a, 33-6-1a or 33-6-1b | Yes |  |  | Per FSPC | N/A | N/A |  |  | Optional with capability signalling |
| 33. NR_MBS | 33-6-1d | DL priority of multicast ACK/NACK-based feedback SPS multicast | 1. Support of priority configured for multicast ACK/NACK-based HARQ-ACK feedback of SPS multicast | 33-5-1b | Yes |  |  | Per FSPC | N/A | N/A |  |  | Optional with capability signalling |
| 33. NR_MBS | 33-6-1e | DL priority of multicast NACK-only-based feedback SPS multicast | 1. Support of priority configured for multicast NACK-only-based HARQ-ACK feedback of SPS multicast | 33-5-1d | Yes |  |  | Per FSPC | N/A | N/A |  |  | Optional with capability signalling |
| 33. NR_MBS | 33-6-1f | DL priority indication for SPS multicast with PDCCH scheduling | 1. Support of priority indicator field configured in DCI format 1_1 with CRC scrambled with G-CS-RNTI for SPS multicast. | 33-5-1a, 33-6-1d or 33-6-1e | Yes |  |  | Per FSPC | N/A | N/A |  |  | Optional with capability signalling |

- FG33-6-2 and 33-6-3 in R1-2108679

| 33. NR_MBS | 33-6-2 | Two ACK/NACK-based HARQ-ACK codebooks simultaneously constructed for supporting HARQ-ACK codebooks with: different priorities for multicast or for unicast and multicast at a UE. | 1. Supports of two ACK/NACK-based HARQ-ACK codebooks with different priorities to be simultaneously constructed different priorities for multicast or for unicast and multicast at a UE. | 33-2b, 33-6-1c | Yes |  |  | Per FSPC | No | No |  |  | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 33. NR_MBS | 33-6-3 | More than one PUCCH for ACK/NACK-based HARQ-ACK transmission for multicast or for unicast and multicast within a slot | 1. Support of two non-overlapping slot-based PUCCHs for ACK/NACK based HARQ-ACK feedback for multicast or for unicast and multicast with different priorities in a slot. | 33-6-2 | Yes |  |  | Per FSPC | No | No |  |  | Optional with capability signalling |

- FG33-7

| 33. NR_MBS | 33-7 | Supporting group-common DCI indicating the enabling/disabling [ACK/NACK based] HARQ-ACK feedback for dynamically scheduled group-common PDSCH | 1. Supports the function of group-common DCI indicating the enabling/disabling [ACK/NACK based] HARQ-ACK feedback for dynamically scheduled group-common PDSCH | 33-2b | Yes |  |  | Per FSPC | N/A | N/A |  |  | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

- New FG33-8 for multicast PUCCH resource configration

| 33. NR_MBS | 33-8-1 | PUCCH resource configuration for ACK/NACK-based multicast feedback for dynamically scheduled multicast | 1. Support of a PUCCH-Config for multicast ACK/NACK-based HARQ-ACK feedback, separate from that of unicast configurations | 33-2b | Yes |  |  | Per FSPC | N/A | N/A |  |  | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 33. NR_MBS | 33-8-2 | Up to 2 PUCCH resources configuration for ACK/NACK-based multicast feedback for dynamically scheduled multicast | 1. Support of a PUCCH-ConfigurationList for multicast ACK/NACK-based HARQ-ACK feedback, separate from that of unicast configurations | 33-2b, 33-6-1a | Yes |  |  | Per FSPC | N/A | N/A |  |  | Optional with capability signalling |
| 33. NR_MBS | 33-8-3 | PUCCH resource configuration for NACK-only-based multicast feedback for dynamically scheduled multicast | 1. Support of a PUCCH-Config for multicast NACK-only-based HARQ-ACK feedback, separate from that of multicast ACK/NACK-based configurations if configured | 33-4 | Yes |  |  | Per FSPC | N/A | N/A |  |  | Optional with capability signalling |
| 33. NR_MBS | 33-8-4 | Up to 2 PUCCH resources configuration for NACK-only-based multicast feedback for dynamically scheduled multicast | 1. Support of a seperate PUCCH-ConfigurationList for multicast NACK-only-based HARQ-ACK feedback, separate from that of multicast ACK/NACK-based configurations if configured | 33-4, 33-6-1b | Yes |  |  | Per FSPC | N/A | N/A |  |  | Optional with capability signalling |
| 33. NR_MBS | 33-8-5 | PUCCH resource configuration for ACK/NACK-based multicast feedback for SPS GC-PDSCH | 1. Support of a SPS-PUCCH-AN-List for multicast ACK/NACK-based HARQ-ACK feedback of all SPS configuration(s), separate from that of SPS unicast configurations | 33-5-1b | Yes |  |  | Per FSPC | N/A | N/A |  |  | Optional with capability signalling |