3GPP TSG RAN WG1 Meeting #107-e		R1-2112288

e-Meeting, November 11th – 19th, 2021

Agenda Item: 8.16.3

Source: MediaTek Inc.

Title: Views on UE features for enhanced IIoT and URLLC

Document for: Discussion and Decision

# Introduction

This paper provides our views on UE features for enhanced IIoT/URLLC [1]. Detailed proposals are described in the following section.

# Discussion

In R15/R16, the UE reports the maximum number of PUCCHs per slot that is supported by the UE. The R17 UELLC/IIoT features listed below will be used to report the capability of supporting new PUCCH repetitions schemes. However, reporting these features for PUCCH repetitions shouldn’t imply an increase of the number of PUCCHs per slot that supported by the UE.

| Index | Feature group | Components | Prerequisite FG | Type | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- |
| 25-2 | Repetitions for PUCCH format 0, and 2 over multiple slots with K = 2, 4, 8 | Repetitions for PUCCH format 0 and 2 over multiple slots with K = 2, 4, 8 | 4-23 | Per UE |  | Optional |
| 25-3 | Repetitions for PUCCH format 0, 1, 2, 3 and 4 over multiple PUCCH subslots with configured K = 2, 4, 8 | Repetitions for PUCCH format 0, 1, 2, 3 and 4 over multiple PUCCH subslots with RRC configured repetition factor K = 2, 4, 8 | 4-23 11-3 | Per UE |  | Optional |
| 25-3a | Repetitions for PUCCH format 0, 1, 2, 3 and 4 over multiple PUCCH subslots using dynamic repetition indication | Repetitions for PUCCH format 0, 1, 2, 3 and 4 over multiple PUCCH subslots based on dynamic repetition indication. | 25-3 30-X | Per UE |  | Optional |

- Reporting FG25-2, 25-3 and 25-3a doesn’t imply an increase in the maximum number of PUCCHs per slot that supported by the UE.

Regarding FG25-18, the support of parallel PUCCH and PUSCH transmission across CCs in inter-band CA should be reported per band combination.

| Index | Feature group | Components | Prerequisite FG | Type | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- |
| 25-18 | Parallel PUCCH and PUSCH transmission across CCs in inter-band CA | Support simultaneous PUCCH/PUSCH transmissions on different cells [at least] for inter-band CA. |  | Per BC |  | Optional |

- Change the Type of FG25-18 from “Per UE” to “Per BC”.

PUCCH cell switching (FG25-9 & FG25-10): PUCCH cell switching feature should have UL CA as Prerequisite FG. Also, the type for FG25-9 & FG25-10 should be per band combination.

| Index | Feature group | Components | Prerequisite FG | Type | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- |
| 25-9 | Semi-static PUCCH cell switching |  | FG6-6 | Per BC |  | Optional |
| 25-10 | PUCCH cell switching based on dynamic indication |  | FG6-6 | Per BC |  | Optional |

- Change the Type of FG25-9 & FG25-10 from “Per UE” to “Per BC”, and add FG6-6 as “Prerequisite FG” for these two FGs.

# Conclusion

In this contribution, we provided our views on UE features for enhanced IIoT/URLLC and we have the following proposals:

- Reporting FG25-2, 25-3 and 25-3a doesn’t imply an increase in the maximum number of PUCCHs per slot that supported by the UE.

- Change the Type of FG25-18 from “Per UE” to “Per BC”.

- Change the Type of FG25-9 & FG25-10 from “Per UE” to “Per BC”, and add FG6-6 as “Prerequisite FG” for these two FGs.

# References

- R1-2110587, “Updated RAN1 UE features list for Rel-17 NR after RAN1 #106bis-e”, Moderators (AT&T, DOCOMO), RAN1#106bis-e, October, 2021.