3GPP TSG-RAN WG1 Meeting #107-e	R1-2112363

e-Meeting, November 11th – 19th, 2021

Agenda Item:	8.9.3

Source:	Ericsson

Title:	On the support of 16-QAM for unicast in UL and DL in TDD NB-IoT

Document for:	Discussion

# 1	Introduction

In the Work Item (WI) on “Rel-17 enhancements for NB-IoT and LTE-MTC” [1], one of the objectives is to specify the following enhancement for NB-IoT:

- Specify 16-QAM for unicast in UL and DL, including necessary changes to DL power allocation for NPDSCH and DL TBS. This is to be specified without a new NB-IoT UE category. For DL, increase in maximum TBS of e.g. 2x the Rel-16 maximum, and soft buffer size will be specified by modifying at least existing Category NB2. For UL, the maximum TBS is not increased. [NB-IoT] [RAN1, RAN4] - Extend the NB-IoT channel quality reporting based on the framework of Rel-14—16, to support 16-QAM in DL. [NB-IoT] [RAN2, RAN1, RAN4]

One open issue for 16-QAM in NB-IoT is whether it will be supported only for FDD or for both FDD and TDD. In this contribution we provide our views on it.

# 2	On the applicability of 16-QAM only for “FDD” or both “FDD and TDD”

Envisioning the finalization of Rel-17, discussions on the UE parameter list have been started. The “UE Feature List” contains a question on the “Need of FDD/TDD differentiation” which still under FFS.

Recently it has been found that supporting 16-QAM for TDD won’t be transparent from a RAN1 perspective nor from a RAN4 perspective.

The foreseen RAN1 impacts from supporting 16-QAM for TDD NB-IoT are:

- In legacy TDD NB-IoT, NPDSCH can be transmitted on DwPTS.

- For NPDSCH without repetition, rate matching is used for the Resource Element (RE) mapping into the special subframe.

- The RE mapping on special subframes including rate matching aspects would have to be discussed for supporting 16-QAM in TDD NB-IoT.

The foreseen RAN4 impacts from supporting 16-QAM for TDD NB-IoT are:

- Define dedicated UE demodulation requirements for 16QAM in TDD NB-IoT in TS 36.101.

- Define a BS conformance test (Test Model) for 16-QAM in TDD NB-IoT in TS 36.141.

Thus, we have the following proposals:

Conclusion: In Rel-17, 16-QAM for unicast in UL and DL for NB-IoT is only supported for FDD operation.

In the “UE Feature list”, the field “Need of FDD/TDD differentiation” is filled-in as “FDD only”.

# 6	Conclusion

Based on the discussion in the previous section we made the following observations:

Observation 1	Recently it has been found that supporting 16-QAM for TDD won’t be transparent from a RAN1 perspective nor from a RAN4 perspective.

Observation 2	The foreseen RAN1 impacts from supporting 16-QAM for TDD NB-IoT are:

	In legacy TDD NB-IoT, NPDSCH can be transmitted on DwPTS.

	For NPDSCH without repetition, rate matching is used for the Resource Element (RE) mapping into the special subframe.

	The RE mapping on special subframes including rate matching aspects would have to be discussed for supporting 16-QAM in TDD NB-IoT.

Observation 3	The foreseen RAN4 impacts from supporting 16-QAM for TDD NB-IoT are:

	Define dedicated UE demodulation requirements for 16QAM in TDD NB-IoT in TS 36.101.

	Define a BS conformance test (Test Model) for 16-QAM in TDD NB-IoT in TS 36.141.

Based on the discussion in the previous sections we propose the following:

Proposal 1	Conclusion: In Rel-17, 16-QAM for unicast in UL and DL for NB-IoT is only supported for FDD operation.

Proposal 2	In the “UE Feature list”, the field “Need of FDD/TDD differentiation” is filled-in as “FDD only”.

# References

RP-201306, “WID revision: Additional enhancements for NB-IoT and LTE-MTC”, RAN #88e, Electronic Meeting, June 29th – July 3rd, 2020.